| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B081 Filename: JVT-B081.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | Adaptive Use of Double Scan for Intra Performance |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul-Woo Kim McubeWorks 14 F, SK Telecom Bldg, Sunae-Dong, Bundang-Gu, Sungnam Seoul, 463-784, Korea Sang Woo Rhie SK Telecom 99 Seorin-dong, Jongro-gu, Seoul, 110-110, Korea | Tel: Email: Tel: Email: | +82-31-601-5875 charlie@mcubeworks.com +82-2-2121-3463 rainism@sktelecom.com |
| Source: | McubeWorks & SK Telecom |  |  |

- Introduction

The use of double / simple scan in the current JM is separated by the QP value in Intra frames. Depending on this QP value of 24 in Intra frames, transformed luminance coefficients are collected either according to the simple or double scan order. Though the separation of double and simple scan at the QP value of 24 seems to be effective in general, it eventually loses optimality due to abrupt change of statistical characteristics caused by transition from simple to double scan. This kind of property is not considered to be desirable for a standard video codec. We propose to adaptively use double / simple scan to show full performance at the vicinity of QP 24 by providing a method of adaptive decision of double / simple scan to encoder. Detailed description of the proposed method and syntax are shown in section 2, 3. Computer simulation results follow in section 4.

- Adaptive Use of Double Scan in Intra Frames

Current JM specifies that the double scan is used for Intra coding of luma with QP < 24. That is, luma coefficients with QP equal to or greater than 24 shall be coded using the simple scan table and QP value of less than 24 shall be coded using double scan. As it’s already pointed out that separation of simple / double scan at the QP 24 seems to work effectively. But since the statistical characteristics of video sequences differ from one another, a fixed threshold QP value of 24 is not always expected to be the optimal threshold of the decision between simple and double scan. Direct transition from simple to double scan at the fixed value of QP 24 eventually leads to abrupt change of VLC characteristics which also leads to some ‘performance loss at the vicinity of QP 24’.

This is due to fixing the threshold value to QP 24 for the decision between simple and double scan in handling video sequences with various statistical characteristics. We propose a method to adjust the threshold value of decision between simple and double scan. DELTAthr is proposed to follow every slice header to adjust the threshold value of simple and double scan, which is currently fixed as 24. According to the adjusted threshold values THRadjusted, luma coefficients in the slice are coded using simple or double scan. When QP value of a macroblock is QPmacroblock, then the luma coefficients in the macroblock are encoded using double scan if

QPmacroblock < THRadjusted

Proposed syntax to generated THRadjusted are shown in the following section.

- Proposed Syntax for Adjusting the Threshold

We propose that there should be provided a syntax by which a encoder can make optimal decision in choosing between simple and double scan. Since the given condition for the double scan, luma with QP < 24, shows fairly effective performance, a DELTAthr value is suggested to adjust the current fixed threshold value of 24. Each slice in the Intra frame is proposed to have THRadjusted to specify new threshold that separates simple / double scan. THRadjusted is calculated in the similar manner as in the case that QPmacroblokc  is adjusted by DQUANT which is known as ‘arithmetic wrap’.

THRadjusted = modulo32 (24 + DELTAthr + 32)

And the DELTAthr may range from –16 to 16 which enables THRadjusted to be changed to any value in the range 0-31. DELTAthr is encoded exactly in the same manner that DQUANT or MVD is encoded.

As is shown in Fig. 1, DELTAthr value is proposed to follow the slice header of Intra frames. It is implemented that DELTAthr  to follow the slice header in the currently uploaded software (JVT-B081.zip).

Fig. 1. Proposed syntax for DELTAthr

| SLICE HEADER | DELTAthr | MB_TYPE |
| --- | --- | --- |

- Simulation Results

Totally 9 cases were tested including 7 sequences specified in the VCEG-N81. The common condition recommends computer simulation at QP values of (16, 20, 24, 28). However, because the focus of this contribution is to provide optimal use of simple and double scan at the vicinity of QP 24, QP value of 18, 22, 26, 28 were also tested.

Table 1. Simulation Sequences

| Sequnence | Format | Frame Rate (fps) | Frames |
| --- | --- | --- | --- |
| Foreman Container News Silent Paris Mobile Tempete Stephan Weather | QCIF QCIF QCIF QCIF CIF CIF CIF QCIF QCIF | 10 10 10 15 15 30 30 10 10 | 100 100 100 150 150 250 240 100 100 |

Simulation results show that the proposed method shows up to 5 % of bit savings as shown in Table. 2. This is because that current JM reveals the ‘performance loss area’ at near the QP value of 24 which can be clearly identified in Fig. 2. BD-PSNR and BD-BRS are also shown in Table 2. As is already mentioned, the main interest of this contribution is to provide an optimal selection of double and simple scan near QP 24, bit rate saving at the QP value of 24 is also provided in the table. Software implementation using TML 9.4 is also uploaded (JVT-B081.zip). In the software simulation, the DELTAthr is allocated the same codeword and value as DQUANT and it is located right after the slice header. Excel files providing the whole results are also provided. (JVT-B081.xls)

Table 2. Performance Comparison with TML 9.4

| Sequence | BD-SNR (dB) | BD-BRS (%) | Bit Saving at QP 24 (%) |
| --- | --- | --- | --- |
| Container | 0.142 | -1.77 | 3.73 |
| Foreman | 0.018 | -0.25 | 0.52 |
| News | 0.184 | -1.83 | 3.76 |
| Silent | -0.002 | 0.04 | -0.006 |
| Paris | 0.133 | -1.41 | 2.81 |
| Tempete | 0.073 | -0.84 | 1.54 |
| Mobile | 0.222 | -2.13 | 4.14 |
| Stephan | 0.217 | -2.15 | 3.79 |
| Weather | 0.317 | -2.64 | 5.49 |

- ‘-‘ in the BD-BRS represents ‘bit rate gain’ and ‘+’ in the BD-SNR ‘SNR gain’

- Conclusion

The separation of double and simple scan by a fixed QP value of 24 is not considered to be  desirable for a general purpose standard video codec. DELTAthr is proposed to adjust the current fixed threshold for the decision between simple and double scan. With the DELTAthr, optimal encoder can improve the Intra performance at the vicinity of QP 24. Simulation results show that there exists ‘performance loss area’ at the current JM and it is shown that it can be improved by providing DELTAthr.

Fig. 2. Bit rate savings depending on OP values

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | McubeWorks Inc. & SK Telecom |  |  |
| Mailing address | 14F SK Telecom Bldg. 9-1 Sunae-dong, Bundang-gu, Sungnam City, 463-784 |  |  |
| Country | Korea |  |  |
| Contact person | Chul-Woo Kim |  |  |
| Telephone | +1 82 31 601 5875 |  |  |
| Fax | +1 82 31 601 5878 |  |  |
| Email | charlie@mcubeworks.com |  |  |
| Place and date of submission | 23/01/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | ### JVT |  |
| Title |  | Adaptive use of double scan for intra performance |  |
| Contribution number |  | JVT-B081 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: