| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B086 Filename: JVT-B086.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | Results on core experiment of sub-picture coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-shik Kim P.O.Box 111 Suwon 440-600 Korea Dae-Sung Cho P.O.Box 111 Suwon 440-600 Korea | Tel: Email: Tel: Email: | +82 31 280 9214 wskim@sait.samsung.co.kr +82 31 280 9211 dscho@sait.samsung.co.kr |
| Source: | Samsung AIT |  |  |

_____________________________

# Introduction

In the Pattaya meeting, VCEG/JVT decided to initiate a core experiment about the sub-picture coding technique presented in VCEG-O57, titled “New Image Segmentation Method”. We (Samsung AIT) have verified Nokia proposal of the sub-picture coding under the simulation environment as described in the document. Section 2 describes the core experiment results

# Simulation results

Table 1. Results of Carphone

| Codec & Parameters | PLR (%) | Y PSNR (dB) |  |  | Cb PSNR (dB) |  |  | Cr PSNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg | Max | Min | Avg | Max | Min | Avg | Max | Min |
| RSP codec Target bitrate = 64.00/56.06 kbps 3 GOBs/slice for Foreground 1 slices/picture for Background IGUf = 1 GOB/frame, IGUb = 1 GOB/frame QPf = 21, 22, 23, QPb = 24 Bit rate = 52.06 kbps | 0 | 27.74/28.78 |  |  | 34.88/35.41 |  |  | 36.73/35.72 |  |  |
|  | 3 | 27.11/28.75 | 27.74/28.78 | 26.02/28.43 | 34.76/35.41 | 34.88/35.41 | 34.43/35.32 | 36.69/35.72 | 36.73/35.72 | 36.49/35.69 |
|  | 5 | 26.17/28.60 | 26.9/28.78 | 25.02/27.93 | 34.67/35.41 | 34.83/35.41 | 34.29/35.37 | 36.64/35.72 | 36.72/35.75 | 36.55/35.66 |
|  | 10 | 25.38/28.08 | 26.08/28.78 | 24.75/27.20 | 34.42/35.34 | 34.74/35.42 | 34.13/35.17 | 36.56/35.64 | 36.66/35.72 | 36.4/35.40 |
|  | 20 | 24.04/27.44 | 25.41/28.76 | 22.94/25.86 | 34.03/35.26 | 34.53/35.41 | 33.52/35.01 | 36.4/35.58 | 36.64/35.72 | 36.17/35.27 |
| Conv. Codec + ROI Target bitrate = 64.00/54.40 kbps Slice size = 3 GOBs/slice IGU = 1 GOB/frame QPf = 23, 24, 25, QPb = 26 Bit rate = 53.75 kbps | 0 | 27.15/28.09 |  |  | 34.49/34.91 |  |  | 36.44/35.37 |  |  |
|  | 3 | 27.06/28.04 | 27.15/28.09 | 26.36/27.61 | 34.49/34.91 | 34.49/34.91 | 34.47/34.91 | 36.44/35.36 | 36.44/35.37 | 36.42/35.34 |
|  | 5 | 26.91/27.86 | 27.15/28.09 | 26.01/27.11 | 34.45/34.89 | 34.49/34.91 | 34.27/34.83 | 36.42/35.34 | 36.44/35.37 | 36.34/35.27 |
|  | 10 | 26.51/27.46 | 27.05/27.99 | 25.83/26.58 | 34.41/34.86 | 34.5/34.91 | 34.29/34.76 | 36.38/35.33 | 36.44/35.36 | 36.3/35.30 |
|  | 20 | 24.86/26.23 | 26.37/27.22 | 22.64/24.49 | 34.22/34.78 | 34.43/34.89 | 33.61/34.65 | 36.28/35.23 | 36.42/35.34 | 36.05/35.01 |
| Conv. Codec Target bitrate = 64.00/54.40 kbps Slice size = 3 GOBs/slice IGU = 1 GOB/frame QP = 26 Bit rate = 49.14 kbps | 0 | 26.95/27.51 |  |  | 34.35/34.48 |  |  | 36.17/34.86 |  |  |
|  | 3 | 26.87/27.44 | 26.95/27.51 | 26.26/26.80 | 34.34/34.48 | 34.35/34.48 | 34.31/34.46 | 36.17/34.86 | 36.17/34.87 | 36.15/34.86 |
|  | 5 | 26.69/27.31 | 26.95/27.51 | 25.8/26.65 | 34.31/34.46 | 34.35/34.48 | 34.19/34.37 | 36.16/34.86 | 36.17/34.88 | 36.08/34.82 |
|  | 10 | 26.26/26.91 | 26.79/27.30 | 25.52/26.08 | 34.27/34.43 | 34.34/34.46 | 34.13/34.38 | 36.12/34.85 | 36.17/34.88 | 36.05/34.81 |
|  | 20 | 24.93/25.71 | 25.97/26.70 | 23.33/24.03 | 34.07/34.35 | 34.28/34.47 | 33.63/34.25 | 36.04/34.78 | 36.17/34.85 | 35.96/34.69 |

Table 2. Results of Coastguard

| Codec & Parameters | PLR (%) | Y PSNR (dB) |  |  | Cb PSNR (dB) |  |  | Cr PSNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg | Max | Min | Avg | Max | Min | Avg | Max | Min |
| RSP codec Target bitrate = 64.00 / 56.00 kbps 1 GOBs/slice for FG, 1 GOBs/slice for BG IGUf = 1 GOB/frame, IGUb = 1 GOB/frame QPf = 24, 25, 26, QPb = 27 Bit rates = 55.41 kbps | 0 | 26.01/24.97 |  |  | 38.75/39.75 |  |  | 41.39/39.75 |  |  |
|  | 3 | 25.67/24.91 | 26.01/24.97 | 25.05/24.4 | 38.71/39.75 | 38.75/39.75 | 38.66/39.74 | 41.38/39.75 | 41.4/39.75 | 41.3/39.72 |
|  | 5 | 25.48/24.87 | 25.78/24.97 | 24.98/24.49 | 38.67/39.75 | 38.72/39.76 | 38.54/39.74 | 41.4/39.74 | 41.66/39.77 | 41.29/39.56 |
|  | 10 | 24.73/24.64 | 25.57/24.97 | 23.93/24.11 | 38.52/39.74 | 38.68/39.77 | 38.05/39.73 | 41.4/39.73 | 41.71/39.78 | 41.26/39.66 |
|  | 20 | 24.1/24.09 | 24.7/24.96 | 23.58/23.28 | 38.41/39.70 | 38.6/39.76 | 38.31/39.64 | 41.4/39.73 | 41.47/39.82 | 41.28/39.59 |
| Conv. Codec + ROI Target bitrate = 64.00/54.40 kbps Slice size = 1 GOBs/slice IGU = 1 GOB/frame QPf = 25, 26, 27, QPb = 28 Bit rates = 50.99 kbps | 0 | 25.66/24.52 |  |  | 38.83/40.32 |  |  | 40.83/39.42 |  |  |
|  | 3 | 25.62/24.46 | 25.66/24.52 | 25.47/24.12 | 38.84/40.31 | 38.84/40.32 | 38.83/40.30 | 40.82/39.41 | 40.83/39.42 | 40.78/39.30 |
|  | 5 | 25.37/24.28 | 25.66/24.52 | 24.91/23.76 | 38.8/40.29 | 38.84/40.32 | 38.73/40.22 | 40.83/39.43 | 40.87/39.51 | 40.82/39.41 |
|  | 10 | 25.1/23.88 | 25.45/24.34 | 24.31/23.14 | 38.79/40.27 | 38.84/40.37 | 38.67/40.19 | 40.81/39.39 | 40.89/39.48 | 40.68/39.20 |
|  | 20 | 24.15/22.90 | 25.15/23.78 | 23.18/22.11 | 38.65/40.25 | 38.85/40.40 | 38.41/40.12 | 40.82/39.41 | 41.06/39.76 | 40.7/39.26 |
| Conv. Codec Target bitrate = 64.00/54.40 kbps Slice size = 1 GOBs/slice IGU = 1 GOB/frame QP = 27 Bit rates = 52.08 kbps | 0 | 25.67/24.31 |  |  | 38.76/39.82 |  |  | 41.61/39.81 |  |  |
|  | 3 | 25.64/24.26 | 25.67/24.31 | 25.5/23.96 | 38.7/39.82 | 38.76/39.83 | 38.7/39.82 | 41.61/39.80 | 41.61/39.81 | 41.54/39.73 |
|  | 5 | 25.36/24.06 | 25.67/24.31 | 24.96/23.72 | 38.75/39.84 | 38.77/39.91 | 38.67/39.81 | 41.6/39.79 | 41.68/39.89 | 41.54/39.67 |
|  | 10 | 25.07/23.73 | 25.46/24.11 | 24.22/23.05 | 38.72/39.82 | 38.76/39.90 | 38.63/39.73 | 41.59/39.78 | 41.64/39.84 | 41.52/39.63 |
|  | 20 | 24.21/22.83 | 25.05/23.47 | 23.28/21.99 | 38.64/39.82 | 38.78/39.90 | 38.41/39.67 | 41.53/39.66 | 41.66/39.76 | 41.37/39.52 |

Table 3. Results of Forman (64kbps)

| Codec & Parameters | PLR (%) | Y PSNR (dB) |  |  | Cb PSNR (dB) |  |  | Cr PSNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg | Max | Min | Avg | Max | Min | Avg | Max | Min |
| RSP codec Target bitrate = 64.00/58.00 kbps 3 GOBs/slice for FG 1 GOBs/slice for BG IGUf = 1 GOB/frame, IGUb = 1 GOB/frame QPf = 21, 22, 23, QPb = 24 Bit rates = 54.87 kbps | 0 | 26.31/26.71 |  |  | 36.2/36.37 |  |  | 35.93/35.20 |  |  |
|  | 3 | 25.36/26.42 | 26.17/26.71 | 22.56/23.84 | 35.97/36.22 | 36.22/36.37 | 34.55/34.84 | 35.52/35.03 | 35.88/35.20 | 33.77/33.56 |
|  | 5 | 24.57/26.49 | 26.07/26.71 | 22.12/25.76 | 35.92/36.33 | 36.24/36.37 | 35.43/36.22 | 35.39/35.15 | 35.9/35.20 | 34.39/35.01 |
|  | 10 | 24.23/26.25 | 25.28/26.71 | 22.67/24.71 | 35.82/36.31 | 36.15/36.37 | 35.34/36.16 | 35.24/35.12 | 35.65/35.22 | 34.55/34.88 |
|  | 20 | 21.99/25.40 | 24.41/26.71 | 19.39/23.18 | 35.06/36.15 | 35.88/36.37 | 33.84/35.04 | 34.68/34.90 | 35.22/35.20 | 33.71/33.64 |
| Conv. Codec + ROI Target bitrate = 64.00/58.80 kbps Slice size = 1 GOBs/slice IGU = 1/2 GOB/frame QPf = 23, 24, 25, QPb = 26 Bit Rates = 53.42 kbps | 0 | 25.67/26.12 |  |  | 36.12/36.27 |  |  | 35.71/35.00 |  |  |
|  | 3 | 25.64/26.12 | 25.67/26.12 | 25.38/26.08 | 36.11/36.27 | 36.12/36.27 | 36.08/36.27 | 35.67/35.00 | 35.71/35.00 | 35.32/35.00 |
|  | 5 | 24.97/25.54 | 25.67/26.12 | 20.88/21.14 | 36/36.20 | 36.12/36.27 | 35.22/35.67 | 35.53/34.89 | 35.71/35.00 | 34.72/34.07 |
|  | 10 | 24.48/24.92 | 25.67/26.12 | 21.78/21.56 | 35.96/36.15 | 36.12/36.27 | 35.55/35.88 | 35.48/34.84 | 35.71/35.02 | 35/34.44 |
|  | 20 | 23.8/24.44 | 25.25/25.70 | 18.97/19.72 | 35.7/35.84 | 36.08/36.23 | 34.04/33.97 | 35.12/34.45 | 35.68/34.96 | 32.76/31.67 |
| Conv. Codec Target bitrate = 64.00/56.80 kbps Slice size = 3 GOBs/slice IGU = 1 GOB/frame QP = 25 Bit rate = 53.52 kbps | 0 | 25.89/26.02 |  |  | 36.19/36.29 |  |  | 35.74/34.87 |  |  |
|  | 3 | 25.87/26.02 | 25.89/26.02 | 25.66/26.00 | 36.19/36.29 | 36.19/36.29 | 36.18/36.29 | 35.71/34.87 | 35.74/34.87 | 35.39/34.87 |
|  | 5 | 25.31/25.45 | 25.89/26.02 | 22.01/21.86 | 36.1/36.21 | 36.19/36.29 | 35.59/35.69 | 35.63/34.77 | 35.74/34.87 | 35.05/34.21 |
|  | 10 | 24.63/24.82 | 25.89/26.02 | 22.2/22.08 | 36.02/36.17 | 36.19/36.29 | 35.52/35.90 | 35.48/34.72 | 35.74/34.87 | 34.65/34.23 |
|  | 20 | 24.07/24.22 | 25.59/25.74 | 21.12/21.09 | 35.87/35.91 | 36.18/36.26 | 34.75/34.41 | 35.32/34.43 | 35.72/34.84 | 34.18/32.71 |

Table 4. Results of Forman (114kbps)

| Codec & Parameters | PLR (%) | Y PSNR (dB) |  |  | Cb PSNR (dB) |  |  | Cr PSNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg | Max | Min | Avg | Max | Min | Avg | Max | Min |
| RSP codec Target bitrate = 144.00/138.00 kbps 1 GOBs/slice for FG 1 GOBs/slice for BG IGUf = 1 GOB/frame IGUb = 1 GOB/frame QPf = 12, 13, 14, QPb = 15 Bit rate = 136.38 kbps | 0 | 28.83/29.06 |  |  | 38.28/38.62 |  |  | 38.58/38.10 |  |  |
|  | 3 | 27.01/28.46 | 28.67/29.06 | 23.4/23.03 | 37.95/38.39 | 38.28/38.62 | 36.21/36.30 | 37.95/37.82 | 38.58/38.10 | 35.3/35.30 |
|  | 5 | 26.02/28.55 | 28.63/29.06 | 22.42/25.42 | 37.82/38.56 | 38.27/38.62 | 36.82/38.38 | 37.89/38.01 | 38.53/38.10 | 35.94/37.69 |
|  | 10 | 25.14/28.51 | 27.11/29.06 | 23.19/26.21 | 37.66/38.55 | 38/38.62 | 36.95/38.42 | 37.16/37.98 | 38.32/38.10 | 35.78/37.65 |
|  | 20 | 22.46/27.24 | 25.86/29.06 | 19.51/23.20 | 36.7/38.13 | 37.74/38.62 | 34.71/35.61 | 36.4/37.51 | 37.53/38.10 | 34.48/34.97 |
| Conv. Codec + ROI Target bitrate = 144.00/136.80 kbps Slice size = 1 GOBs/slice IGU = 1 GOB/frame QPf = 15, 16, 17, QPb = 18 Bit rate = 126.33 kbps | 0 | 28.06/28.35 |  |  | 37.57/37.76 |  |  | 37.55/36.92 |  |  |
|  | 3 | 28.05/28.35 | 28.06/28.35 | 28.04/28.33 | 37.57/37.76 | 37.57/37.76 | 37.57/37.76 | 37.55/36.92 | 37.55/36.92 | 37.55/36.92 |
|  | 5 | 27.23/27.75 | 28.06/28.35 | 22.88/23.52 | 37.43/37.65 | 37.57/37.76 | 36.64/37.01 | 37.32/36.78 | 37.55/36.92 | 36.53/35.82 |
|  | 10 | 26.29/26.75 | 28.06/28.35 | 22.4/22.44 | 37.39/37.62 | 37.57/37.76 | 36.97/37.34 | 37.23/36.68 | 37.55/36.92 | 36.47/35.82 |
|  | 20 | 25.58/26.03 | 27.59/27.93 | 20.8/20.34 | 36.99/37.22 | 37.51/37.68 | 34.38/34.84 | 36.88/36.23 | 37.49/36.85 | 34.52/33.18 |
| Conv. Codec Target bitrate = 144.00/136.80 kbps Slice size = 1 GOBs/slice IGU = 1 GOB/frame QP = 17 Bit rate = 129.09 kbps | 0 | 28.17/28.23 |  |  | 37.5/37.63 |  |  | 37.46/36.58 |  |  |
|  | 3 | 28.16/28.23 | 28.17/28.23 | 28.08/28.20 | 37.5/37.63 | 37.5/37.63 | 37.49/37.62 | 37.45/36.58 | 37.46/36.58 | 37.32/36.58 |
|  | 5 | 27.15/27.66 | 28.17/28.23 | 22.77/24.10 | 37.37/37.55 | 37.5/37.63 | 36.69/37.10 | 37.17/36.48 | 37.46/36.58 | 36.41/35.89 |
|  | 10 | 26.2/26.61 | 28.17/28.23 | 22.74/22.63 | 37.31/37.48 | 37.5/37.63 | 36.97/37.20 | 37.05/36.39 | 37.46/36.58 | 35.88/35.71 |
|  | 20 | 25.49/25.98 | 27.75/27.81 | 21.14/21.39 | 37.05/37.22 | 37.47/37.57 | 35.55/35.71 | 36.8/36.07 | 37.41/36.52 | 35.09/34.25 |

Table 5. Results of Hall

| Codec & Parameters | PLR (%) | Y PSNR (dB) |  |  | Cb PSNR (dB) |  |  | Cr PSNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg | Max | Min | Avg | Max | Min | Avg | Max | Min |
| RSP codec Target bitrate = 32.00/24.00 kbps 1 GOBs/slice for FG 3 GOBs/slice for BG IGUf = 1 GOB/frame IGUb = 1/3 GOB/frame QPf = 28, 29, 30, QPb = 31 Bit rate = 24.25 kbps | 0 | 26.35/23.92 |  |  | 35.11/34.06 |  |  | 37.73/37.47 |  |  |
|  | 3 | 26.3/23.89 | 26.36/23.92 | 26.13/23.61 | 35.09/34.05 | 35.12/34.06 | 35.04/33.97 | 37.74/37.47 | 37.85/37.47 | 37.72/37.47 |
|  | 5 | 26.22/23.82 | 26.3/23.92 | 26.14/23.58 | 35.09/34.04 | 35.14/34.06 | 35.03/34.01 | 37.72/37.47 | 37.84/37.47 | 37.66/37.44 |
|  | 10 | 26.07/23.72 | 26.19/23.92 | 25.97/23.38 | 35.03/33.99 | 35.12/34.06 | 34.69/33.84 | 37.73/37.46 | 37.83/37.48 | 37.65/37.44 |
|  | 20 | 25.85/23.42 | 26.15/23.92 | 25.64/23.17 | 34.99/33.94 | 35.19/34.06 | 34.57/33.77 | 37.69/37.42 | 37.8/37.47 | 37.56/37.33 |
| Conv. Codec + ROI Target bitrate = 32.00/22.40 kbps Slice size = 1 GOBs/slice IGU = 1 GOB/frame QPf = 28, 29, 30, QPb = 31 Bit rate = 32.05 kbps | 0 | 26.29/23.88 |  |  | 35.31/34.19 |  |  | 37.75/37.50 |  |  |
|  | 3 | 26.25/23.83 | 26.29/23.88 | 26.09/23.61 | 35.3/34.18 | 35.31/34.19 | 35.25/34.10 | 37.74/37.49 | 37.75/37.50 | 37.73/37.46 |
|  | 5 | 26.22/23.80 | 26.29/23.88 | 26.09/23.58 | 35.29/34.18 | 35.31/34.20 | 35.22/34.12 | 37.74/37.49 | 37.81/37.68 | 37.66/37.33 |
|  | 10 | 26.1/23.58 | 26.26/23.86 | 25.72/23.19 | 35.28/34.17 | 35.31/34.20 | 35.15/34.10 | 37.74/37.50 | 37.77/37.57 | 37.71/37.41 |
|  | 20 | 25.65/22.97 | 26.08/23.66 | 25.15/22.18 | 35.19/34.08 | 35.28/34.16 | 34.99/33.96 | 37.7/37.44 | 37.78/37.62 | 37.65/37.31 |
| Conv. Codec Target bitrate = 32.00/22.40 kbps Slice size = 1 GOBs/slice IGU = 1 GOB/frame QP = 31 Bit rate = 29.10 kbps | 0 | 25.75/22.83 |  |  | 35.23/33.99 |  |  | 37.74/37.43 |  |  |
|  | 3 | 25.72/22.80 | 25.75/22.83 | 25.57/22.59 | 35.22/33.98 | 35.23/33.99 | 35.18/33.92 | 37.74/37.43 | 37.74/37.43 | 37.73/37.41 |
|  | 5 | 25.69/22.76 | 25.75/22.83 | 25.6/22.61 | 35.22/33.98 | 35.23/33.99 | 35.16/33.95 | 37.74/37.42 | 37.75/37.45 | 37.69/37.36 |
|  | 10 | 25.6/22.63 | 25.72/22.80 | 25.26/22.32 | 35.2/33.97 | 35.25/34.00 | 35.08/33.92 | 37.73/37.40 | 37.77/37.44 | 37.7/37.32 |
|  | 20 | 25.23/22.18 | 25.56/22.63 | 24.88/21.70 | 35.12/33.90 | 35.22/33.97 | 34.93/33.76 | 37.7/37.39 | 37.75/37.54 | 37.63/37.28 |

Table 6. Results of News

| Codec & Parameters | PLR (%) | Y PSNR (dB) |  |  | Cb PSNR (dB) |  |  | Cr PSNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg | Max | Min | Avg | Max | Min | Avg | Max | Min |
| RSP codec Target bitrate = 144.00 /132.00 kbps 3 GOBs/slice for FG 1 slice/picture for BG IGUf = 1 GOB/frame IGUb = 1/2 GOB/frame QPf = 15, 16, 17, QPb = 18 bitrate = 127.25 kbps | 0 | 33.52/32.26 |  |  | 38.49/37.33 |  |  | 39.09/37.91 |  |  |
|  | 3 | 33.26/32.10 | 33.51/32.26 | 32.68/31.56 | 38.44/37.32 | 38.5/37.33 | 38.3/37.23 | 39.05/37.90 | 39.1/37.91 | 38.99/37.83 |
|  | 5 | 32.99/31.94 | 33.38/32.26 | 32.18/31.42 | 38.38/37.27 | 38.46/37.33 | 38.13/36.91 | 39.01/37.87 | 39.09/37.91 | 38.87/37.65 |
|  | 10 | 32.5/31.42 | 33.15/32.26 | 31.27/29.62 | 38.3/37.24 | 38.45/37.33 | 38.12/37.07 | 38.94/37.85 | 39.02/37.91 | 38.85/37.73 |
|  | 20 | 30.9/29.90 | 31.86/31.22 | 29.21/27.94 | 38.12/37.06 | 38.23/37.16 | 37.95/36.86 | 38.81/37.72 | 38.92/37.79 | 38.72/37.61 |
| Conv. Codec + ROI Target bitrate = 144.00/129.60 kbps Slice size = 1 slice/picture IGU = 1 GOB/frame QPf = 19, 20, 21, QPb = 22 bitrate = 128.56 kbps | 0 | 31.46/30.4 |  |  | 37.04/35.88 |  |  | 37.61/36.14 |  |  |
|  | 3 | 31.41/30.34 | 31.46/30.4 | 31.22/30.13 | 37.03/35.87 | 37.04/35.88 | 37.02/35.85 | 37.61/36.14 | 37.62/36.15 | 37.6/36.13 |
|  | 5 | 31.16/30.01 | 31.46/30.4 | 30.21/28.75 | 36.84/35.67 | 37.04/35.88 | 35.5/34.01 | 37.51/36.04 | 37.61/36.12 | 36.81/35.17 |
|  | 10 | 30.72/29.43 | 31.33/30.21 | 29.45/27.82 | 36.92/35.79 | 37.01/35.86 | 36.82/35.72 | 37.53/36.07 | 37.59/36.12 | 37.47/36.01 |
|  | 20 | 29.28/27.61 | 30.29/28.88 | 27.55/25.55 | 35.86/34.55 | 36.79/35.71 | 33.97/32.2 | 36.98/35.43 | 37.44/36.04 | 36/34.29 |
| Conv. Codec Target bitrate = 144.00/129.60 kbps Slice size = 1 slice/picture IGU = 1 GOB/frame QP=21 bitrate = 119.96 kbps | 0 | 31.5/30.07 |  |  | 36.76/35.51 |  |  | 37.35/35.88 |  |  |
|  | 3 | 31.45/30.01 | 31.5/30.07 | 31.28/29.83 | 36.76/35.51 | 36.76/35.51 | 36.75/35.49 | 37.35/35.88 | 37.36/35.89 | 37.34/35.88 |
|  | 5 | 31.2/29.70 | 31.5/30.07 | 30.29/28.54 | 36.5/35.21 | 36.77/35.51 | 34.52/32.71 | 37.24/35.75 | 37.35/35.88 | 36.44/34.71 |
|  | 10 | 30.8/29.19 | 31.37/29.91 | 29.64/27.70 | 36.67/35.43 | 36.74/35.50 | 36.58/35.31 | 37.29/35.81 | 37.34/35.87 | 37.21/35.72 |
|  | 20 | 29.45/27.53 | 30.35/28.66 | 28.06/25.83 | 35.46/33.97 | 36.61/35.36 | 32.95/30.97 | 36.74/35.14 | 37.25/35.79 | 35.45/33.59 |

Table 7. Results of Irene

| Codec & Parameters | PLR (%) | Y PSNR (dB) |  |  | Cb PSNR (dB) |  |  | Cr PSNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Avg | Max | Min | Avg | Max | Min | Avg | Max | Min |
| RSP codec Target bitrate = 384.00/360.00 kbps 3 GOBs/slice for FG 3 GOBs/slice for BG IGUf = 1 GOB/frame IGUb = 1/4 GOB/frame QPf = 19, 20, 21, QPb = 22 bitrate = 359.00 kbps | 0 | 35.59/33.74 |  |  | 38.71/37.59 |  |  | 37.95/37.04 |  |  |
|  | 3 | 35.2/33.60 | 35.59/33.74 | 34.5/32.35 | 38.68/37.58 | 38.71/37.59 | 38.6/37.51 | 37.9/37.04 | 37.95/37.04 | 37.77/36.97 |
|  | 5 | 34.52/33.05 | 35.23/33.74 | 33.14/30.87 | 38.57/37.51 | 38.7/37.59 | 38.12/37.26 | 37.81/36.96 | 37.92/37.04 | 37.65/36.67 |
|  | 10 | 33.73/32.48 | 34.69/33.74 | 32.24/31.39 | 38.53/37.45 | 38.65/37.59 | 38.14/37.21 | 37.73/36.87 | 37.85/37.04 | 37.55/36.64 |
|  | 20 | 32/30.56 | 33.2/32.78 | 31.01/29.12 | 38.37/37.18 | 38.57/37.54 | 38/36.71 | 37.4/36.56 | 37.66/36.99 | 37.07/36.11 |
| Conv. Codec + ROI Target bitrate = 384.00/355.20 kbps Slice size = 3 GOBs/slice IGU = 1 GOB/frame QPf = 22, 23, 24, QPb = 25 bitrate = 318.86 kbps | 0 | 33.81/31.88 |  |  | 37.39/36.72 |  |  | 36.61/36.10 |  |  |
|  | 3 | 33.78/31.83 | 33.81/31.88 | 33.59/31.55 | 37.38/36.70 | 37.39/36.72 | 37.29/36.49 | 36.61/36.09 | 36.61/36.10 | 36.56/35.96 |
|  | 5 | 33.22/31.09 | 33.81/31.88 | 32.2/29.73 | 37.35/36.66 | 37.39/36.72 | 37.24/36.57 | 36.56/36.02 | 36.61/36.10 | 36.5/35.90 |
|  | 10 | 32.54/30.19 | 33.45/31.47 | 31.33/28.6 | 37.3/36.56 | 37.38/36.71 | 37.16/36.37 | 36.49/35.89 | 36.58/36.07 | 36.39/35.68 |
|  | 20 | 31.03/28.32 | 32.44/30.06 | 29.35/26.38 | 37.02/36.19 | 37.25/36.46 | 36.64/35.6 | 36.23/35.49 | 36.45/35.85 | 35.86/34.86 |
| Conv. Codec Target bitrate = 384.00/355.20 kbps Slice size = 3 GOBs/slice IGU = 1 GOB/frame QP = 23 bitrate = 338.49 kbps | 0 | 34.38/32.06 |  |  | 37.76/36.96 |  |  | 37.14/36.29 |  |  |
|  | 3 | 34.34/32.01 | 34.38/32.06 | 34.09/31.65 | 37.75/36.95 | 37.76/36.96 | 37.71/36.89 | 37.14/36.29 | 37.14/36.29 | 37.13/36.27 |
|  | 5 | 33.72/31.24 | 34.38/32.06 | 32.73/30.03 | 37.7/36.88 | 37.76/36.96 | 37.63/36.76 | 37.07/36.19 | 37.14/36.29 | 36.99/36.05 |
|  | 10 | 32.94/30.31 | 33.88/31.64 | 31.6/28.57 | 37.65/36.76 | 37.78/36.94 | 37.54/36.55 | 36.99/36.05 | 37.1/36.25 | 36.87/35.87 |
|  | 20 | 31.36/28.44 | 32.93/30.32 | 29.58/26.51 | 37.32/36.25 | 37.64/36.76 | 36.65/34.88 | 36.67/35.59 | 36.95/36.05 | 36.3/35.13 |