| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B088 Filename: JVT-B088.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | MPEG-2 Systems NAL |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author: | Miska M. Hannuksela Nokia Mobile Phones P.O. Box 68 33721 Tampere Finland | Tel: Email: | +358 40 5212845 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Introduction

Video access units, such as slices, have to be identified in MPEG-2 Program and Transport Streams for synchronization purposes. During the reflector discussion between the Pattaya and Geneva meetings, at least two solutions were mentioned: introduction of unique start codes and aligning video access units with Program Elementary Stream (PES) headers.

When unique start codes are in use, start code emulations in the coded video data have to be prevented. The JVT codec has not been designed for such a purpose, and prevention of start code emulation might break its elegant entropy coding design. One possibility to overcome the problem is to insert and remove start code emulation prevention bits separately from the core coding and decoding processes. In other words, emulation prevention bits could be inserted into encoded slice data before transmission and removed before decoding the slice data. However, this is a bit-based operation and requires relatively many computations and duplicated buffering space for coded data.

Aligning video access units with PES headers inherits bit-rate redundancy, as only one PES packet can reside in one transport packet. Thus, this solution is not recommended.

This paper proposes a simple transport packet format that provides seamless interoperability with the planned RTP payload format for JVT. Moreover, the paper proposes a PES packet format that follows the current JVT design.

# Proposal

## MPEG-2 Transport Packet for the JVT Codec

Figure 1 shows a block diagram of the proposed transport packet syntax for the JVT codec. The transport packet header follows the MPEG-2 Systems standard. Packet IDs (PIDs) are used to distinguish between different streams. In addition, a PID can indicate presence of Program Specific Indication (PSI). If PSI is signaled, the syntax following a transport packet header is defined in the MPEG-2 Systems standard and is not reprinted in Figure 1. The transport packet header contains a payload_unit_start_indicator that indicates the presence of a PES header, which is coded according to MPEG-2 Systems. PES headers provide compatibility between the proposed design and existing MPEG-2 systems. Moreover, PES headers include some indications, such as time-stamps, that can be useful for applications receiving Transport Streams. If the payload_unit_start_indicator does not indicate the presence of a PES header, an 8-bit header_pointer is present. If header_pointer equals to 0xFF, there are no new video payloads (access units) in this transport packet, but rather the rest of the transport packet contains a continuation of the previous payload. If header_pointer is less than 0xFF, the transport packet contains at least one new payload. Next, a number of bytes equal to header_pointer follow and contain a continuation of the previous payload. header_pointer shall not have a value that would cause a transport packet overrun. After that, a payload, whose syntax is the same as the proposed RTP payload syntax (VCEG-N72r1), follows. The payload starts with a byte telling the payload type (single slice, data partition A/B/C, compound packet, supplemental enhancement information, or parameter update packet). Compound packets should be avoided, as they provide a “packet assembly” function, which is obsolete in MPEG-2 Transport Streams. Then, the payload data follows, and it can include a slice header and macroblock data, for example. If the payload fits into the transport packet (and if there are unused bytes left in the transport packet), the next payload can be started. If the payload does not fit into the transport packet totally, it is continued in the next packet.

Figure 1. MPEG-2 Transport Packet Syntax for the JVT codec.

Synchronization to the bitstream after loss or corruption is easy: First, the sync_byte in the header of each Transport Packet is used to recover synchronization to the transport packets. Then, the receiver has to check the first byte (header_pointer) of a JVT video transport packet. If header_pointer is smaller than 0xFF, the transport packet contains a new video access unit, and bitstream parsing can be continued from the indicated unit.

When generating a transport packet, location of the next access unit or payload start position has to be known, if it is within the limits of the transport packet. In some coding systems, this may cause some delay. In order to overcome the packet generation delay, the header_pointer can be located as the last byte of the transport packet and point backwards to the first video access unit of the packet. In this case, when the receiver is out of synchronization, it has to buffer at least a whole transport packet before it can identify a video access unit.

The proposed syntax differs from the conventional MPEG-2 design, as no PES packets are inserted to the transport packets as such. Instead, the proposed PES packet syntax can be easily converted to transport packet syntax and vice versa. This decision helps to avoid use of start codes and provides additional flexibility in the PES packet design (see the next sub-section).

The proposed packetization scheme has at least the following benefits:

- Its bit-rate redundancy is likely to be smaller compared to the previous solutions.

- There are no bit-based operations.

- The syntax is aligned with the RTP payload syntax, which makes implementation of video gateways straightforward.

- The proposal does not require any changes in the Video Coding Layer.

## MPEG-2 PES Packet and Program Stream for the JVT Codec

A PES packet contains a standard PES packet header according to MPEG-2 Systems. The packet data follows the interim JVT file format (VCEG-O58). If a PES packet begins with a Clump, the data_alignment_indicator of the PES packet header shall be set. A PES packet may contain any number of Clumps and PES packet boundaries need not be aligned with Clump boundaries (although it is recommended).

System specifications, such as the next generation of DVD, may specify restrictions to the interim JVT file format. For example, having alternate tracks may be forbidden.

The proposed solution has at least the following benefits:

- It is aligned with the interim JVT file format, the RTP payload format, and the MPEG-2 Transport Packet format for JVT. Conversions between these formats are straightforward.

- The interim JVT file format is extensible and follows the fundamentals of the ISO media file format.

# IPR Statement

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission | Geneva, Switzerland, January 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | MPEG-2 Systems NAL |  |
| Contribution number |  | JVT-B088 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: