| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B090 Filename: JVT-B090.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | Core Experiment Results on Improved Macroblock Prediction Modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S.Adachi, S.Kato, M.Kobayashi, and M.Etoh 3-5, Hikari-no-oka, Yokosuka, Japan | Tel: Email: | +81-468-40-3519 adachi@spg.yrp.nttdocomo.co.jp |
| Source: | NTT DoCoMo, Inc. |  |  |

_____________________________

1. Introduction

This document reports the core experiment (CE) results of improved macroblock (MB) prediction modes. The CE has been set up in the Pattaya meeting to evaluate two proposals of improved MB prediction modes, VCEG-O17 [1] and VCEG-O22 [2]. We have conducted the entire CE plans defined in the CE description [3] and show the results in this document. We also describe the B-frame syntax for VCEG-O22, which are newly defined in the CE description to evaluate the proposals in various conditions.

The results show that the both proposals can improve coding efficiency of TML in all of the tested conditions specified in the CE description. VCEG-O22 shows better improvement than that of VCEG-O17.

2. B-frame syntax for VCEG-O22

The CE description defines some additional specifications of the proposals, which have not been described in the original proposal documents, to conduct the evaluations by each participant. Especially, B-frame syntax for VCEG-O22 has been newly defined in the CE description to conduct evaluations on B-frames. Since VCEG-O17 has reported its results on B-frames, the evaluation of both of the proposals should be conducted also on B-frames.

The proposed B-frame syntax, as same as the P-frame syntax described in VCEG-O22, replaces UVLC table for the MB prediction modes (MB_type) and changes the semantics and syntax for reference frame (Ref_frame) so that it can be attached to each motion segment in a MB. Blk_size specified in the TML is not used in this syntax. Other syntax elements are not changed at all from the TML9.0 specification. Figure 1 illustrates the MB syntax diagram.

Figure 1. MB-level syntax of the proposed coding method for B-frames

In this syntax, the prediction modes having two motion vectors in a MB are allowed to use different reference frame and different prediction direction for each motion segment. To enable this function, semantics of Ref_frame is changed so that it also signals the direction of prediction, while in case of the TML, MB_type signals the direction of prediction.

Note that the multiple reference frames and multiple prediction direction in a MB is not allowed for the modes having more than 4 motion vectors, due to inefficiency caused by its multiple segmentation overhead.

Table 1 shows the UVLC table and CABAC binarization table to represent the MB_type for B-frames. Considering the overhead due to the number of motion vectors required, the proposed modes using two motion vectors are inserted just after the 16x16 MC mode (Mode1 – 14).

Table 2 shows the table to represent the Ref_frame for B-frames. In addition to the signaling of reference frame number, ref_frame signals the direction of prediction.

The bisectional segmentation model proposed in VCEG-O22 provides flexible adaptation to motion discontinuity caused by object boundary lies in an MB. The B-frame syntax described here enables those segmentations to select their optimal prediction scheme independently. This feature is expected to be effective for the concept proposed in VCEG-O22.

Table 2. Reference frames (Ref_frame) for B-frames of VCEG-O22

| Code_number | Reference frame | CABAC binarization |
| --- | --- | --- |
| 0 | Forward (1 frame back) | 000 |
| 1 | Backward | 001 |
| 2 | Bi-directional (1 frame back) | 01 |
| 3 | Forward (2 frames back) | 100 |
| 4 | Bi-directional (2 frames back) | 110 |
| 5 | Forward (3 frames back) | 1010 |
| 6 | Bi-directional (3 frames back) | 1110 |
| 7 | Forward (4 frames back) | 10110 |
| 8 | Bi-directional (4 frames back) | 11110 |
| 9 | Forward (5 frames back) | 101110 |
| 10 | Bi-directional (5 frames back) | 111110 |
| ... | ... | ... |

Table 1. Macroblock prediction modes (MB_type) for B-frames of VCEG-O22

| Code_number | MB_type | CABAC binarization |
| --- | --- | --- |
| 0 *1 | Skip/Direct | 0 |
| 1 | Mode0 (TML Mode1) | 10 |
| 2 | Mode1 (TML Mode2) | 110000 |
| 3 | Mode2 (TML Mode3) | 110001 |
| 4 | Mode3 | 1100100 |
| 5 | Mode4 | 1100101 |
| 6 | Mode5 | 1100110 |
| 7 | Mode6 | 1100111 |
| 8 | Mode7 | 1101000 |
| 9 | Mode8 | 1101001 |
| 10 | Mode9 | 1101010 |
| 11 | Mode10 | 1101011 |
| 12 | Mode11 | 1101100 |
| 13 | Mode12 | 1101101 |
| 14 | Mode13 | 1101110 |
| 15 | Mode14 | 1101111 |
| 16 | Mode15 (TML Mode4) | 111000 |
| 17 | Mode16 (TML Mode5) | 111001 |
| 18 | Mode17 (TML Mode6) | 111010 |
| 19 | Mode18 (TML Mode7) | 111011 |
| 20 | Intra4x4 | 11110 |
| 21 | Intra16x16 *2 | 11111 *3 |
| ... | ... | ... |

*1 UVLC for P-frame do not have the code for skip

*2 The rest of MB_type codes for Intra16x16 are the same as TML

*3 The rest of MB_type binarizations for Intra16x16 are the same as TML

3. Simulations

3.1 Softwares for the simulation

We, as the proponent of VCEG-O22, have exchanged softwares with HHI, the proponent of VCEG-O17, and use them to obtain experimental results, as defined in the CE description. The VCEG-O22 implementation is based on TML9.0, and implemented by ourselves. The VCEG-O17 implementation is based on TML8.5, and implemented by HHI. In both softwares, we have checked and confirmed that the bitstream from the encoder executable can be decoded by the decoder executable independently, and the encoder local decoded sequences are the same as the decoder output sequences.

3.2 Simulation conditions

We have conducted the entire CE plans defined in the CE description. Table 3 shows the simulation conditions. They are the same as the conditions defined in the CE description.

Table 3. Simulation conditions

| Conditions | Configuration |
| --- | --- |
| Intra Period | 0 (only the first frame) |
| QP First Frame | the same as P frames |
| QP Remaining Frame | 16, 20, 24, or 28 (P-frames) |
| MV Resolution | 1/4-pel, or 1/8-pel for CIF of B-frames (IBBP...) |
| Use Hadamard | used |
| Search Range | 32 pixel |
| Number Referece Frames | 1, or 5 |
| Number B Frames | 0 (P-frames, IPPP...), or 2 (B-frames, IBBP...) |
| QP B Picture | the same as P frames |
| Symbol Mode | UVLC, or CABAC |
| Restrict Search Range | no restrictions |
| RD Optimization | on |
| _FAST_FULL_ME_ | defined (no adaptive search window) |

Note that the adaptive search window for motion vector search is disabled in the condition. This function adjusts the MV search window of the spiral motion search, by the predicted MV for every block and every prediction mode of a MB, to get. Due to the lack of this functionality on the tested implementation, the evaluation was conducted without this function. This can be achieved for TML by enabling _FAST_FULL_ME_ option in the TML software.

4. Results

The CE description defines two categories of experiments. CE1 is for verifications on simple P-frame encoding, and CE2 is for verifications on higher-complexity B-frame encoding.

We conducted both of CE1 and CE2, with two reference frame settings (i.e. Ref_frame=1 or 5) and two entropy coding settings (i.e. UVLC or CABAC).

The experimental results are compared with the results of TML9.0 or TML8.5 respectively, depending on the base software of each implementation of the proposals.

4.1 CE1: Performance Verification on P-frame Coding

Table 4 to 7 show the results of CE1. BD-PSNR (Bjøntegaard delta PSNR) [4] values between TML9.0 and VCEG-O22 (so-called VPM1 in the CE description), TML8.5 and VCEG-O17 (VPM2), and VCEG-O22 and VCEG-O17 are shown. R-D plots and detailed results can be seen in the accompanied Excel file, JVT-B090_xxx.xls.

The results show that the proposed prediction modes can improve coding efficiency of TML in all of the tested conditions. VCEG-O22 provides up to 6.5% bitrate reduction or up to 0.30dB PSNR improvement from TML9.0. VCEG-O17 provides up to 2.9% bitrate reduction or up to 0.14dB PSNR improvement from TML8.5. In both methods, better improvement can be achieved with larger Ref_frame and CABAC. This can be interpreted as an advantage of multiple reference frame selection in a MB, and multi-mode strategy on the arithmetic coding.

Note that there are differences of coding efficiency between original TML8.5 and TML9.0. Because of that, the comparison between VCEG-O22 and VCEG-O17 shows some reversal results. For example, some of the results of “mobile and calendar” show that VCEG-O17 surpasses VCEG-O22, although the VCEG-O22 always surpasses VCEG-O17 at improvement from TML.

Table 4. CE results for CE1 (P-frame), UVLC, Ref_frame=1

|  | TML9.0->VCEG-O22 |  | TML8.5->VCEG-O17 |  | VCEG-O22->VCEG-O17 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_dif f[dB] | Bitrate_diff [%] |
| foreman | 0.261 | -5.418 | 0.103 | -2.123 | -0.155 | 3.348 |
| news | 0.167 | -2.951 | 0.066 | -1.115 | -0.138 | 2.446 |
| container | 0.159 | -3.363 | 0.023 | -0.461 | -0.237 | 5.215 |
| silent | 0.209 | -4.282 | 0.062 | -1.255 | -0.135 | 2.895 |
| paris | 0.159 | -3.144 | 0.032 | -0.633 | -0.111 | 2.232 |
| mobile | 0.136 | -2.983 | 0.026 | -0.568 | 0.031 | -0.682 |
| tempete | 0.143 | -3.494 | 0.041 | -1.008 | -0.023 | 0.565 |
| avrage | 0.176 | -3.662 | 0.050 | -1.023 | -0.110 | 2.289 |

Table 5. CE results for CE1 (P-frames), UVLC, Ref_frame=5

|  | TML9.0->VCEG-O22 |  | TML8.5->VCEG-O17 |  | VCEG-O22->VCEG-O17 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] |
| foreman | 0.300 | -5.964 | 0.141 | -2.789 | -0.156 | 3.201 |
| news | 0.208 | -3.598 | 0.105 | -1.793 | -0.122 | 2.132 |
| container | 0.095 | -1.981 | 0.002 | -0.047 | -0.231 | 4.798 |
| silent | 0.251 | -5.087 | 0.116 | -2.337 | -0.135 | 2.841 |
| paris | 0.209 | -4.105 | 0.104 | -2.011 | -0.094 | 1.879 |
| mobile | 0.309 | -6.486 | 0.120 | -2.567 | -0.098 | 2.146 |
| tempete | 0.284 | -6.576 | 0.111 | -2.620 | -0.095 | 2.314 |
| avrage | 0.237 | -4.828 | 0.100 | -2.023 | -0.133 | 2.759 |

Table 6. CE results for CE1 (P-frames), CABAC, Ref_frame=1

|  | TML9.0->VCEG-O22 |  | TML8.5->VCEG-O17 |  | VCEG-O22->VCEG-O17 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] |
| foreman | 0.283 | -5.868 | 0.141 | -2.938 | -0.117 | 2.535 |
| news | 0.174 | -3.074 | 0.118 | -2.052 | -0.075 | 1.343 |
| container | 0.130 | -2.685 | 0.007 | -0.179 | -0.217 | 4.677 |
| silent | 0.225 | -4.668 | 0.093 | -1.908 | -0.111 | 2.406 |
| paris | 0.178 | -3.421 | 0.088 | -1.713 | -0.076 | 1.465 |
| mobile | 0.132 | -2.939 | 0.050 | -1.119 | 0.083 | -1.833 |
| tempete | 0.146 | -3.570 | 0.067 | -1.667 | 0.022 | -0.532 |
| avrage | 0.181 | -3.747 | 0.081 | -1.654 | -0.070 | 1.437 |

Table 7. CE results for CE1 (P-frames), CABAC, Ref_frame=5

|  | TML9.0->VCEG-O22 |  | TML8.5->VCEG-O17 |  | VCEG-O22->VCEG-O17 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] |
| foreman | 0.362 | -7.311 | 0.187 | -3.818 | -0.140 | 2.937 |
| news | 0.252 | -4.351 | 0.140 | -2.354 | -0.113 | 2.010 |
| container | 0.181 | -3.562 | 0.038 | -0.794 | -0.219 | 4.476 |
| silent | 0.266 | -5.345 | 0.131 | -2.671 | -0.143 | 2.954 |
| paris | 0.260 | -5.009 | 0.138 | -2.686 | -0.109 | 2.122 |
| mobile | 0.317 | -6.777 | 0.149 | -3.255 | -0.064 | 1.471 |
| tempete | 0.282 | -6.612 | 0.140 | -3.371 | -0.049 | 1.270 |
| avrage | 0.274 | -5.567 | 0.132 | -2.707 | -0.120 | 2.463 |

4.2 CE2: Performance Verification on Higher Complexity Coding

Table 8 to 11 show the results of CE2. BD-PSNR values are shown as the same way as CE1. R-D plots and detailed results can be seen in JVT-B090_xxx.xls.

The results show that the proposed prediction modes can be more effective for B-frames. Improvement of both of VCEG-O22 and VCEG-O17 exceeds their improvement shown in CE1. Up to 8.5% bitrate reduction or 0.41dB PSNR improvement can be seen by VCEG-O22, and up to 5.2% or up to 0.25dB improvement can be seen by VCEG-O17.

An advantage of multiple reference frame selection in a MB, and multi-mode strategy on the arithmetic coding provides more improvement for larger Ref_frame and CABAC, as seen in CE1. In addition to that, multiple prediction direction in a MB can contribute in B-frames.

Table 8. CE results for CE2 (B-frames), UVLC, Ref_frame=1

|  | TML9.0->VCEG-O22 |  | TML8.5->VCEG-O17 |  | VCEG-O22->VCEG-O17 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] |
| foreman | 0.180 | -3.650 | 0.077 | -1.576 | -0.115 | 2.454 |
| news | 0.152 | -2.686 | 0.123 | -2.132 | -0.071 | 1.269 |
| container | 0.187 | -3.699 | 0.065 | -1.382 | -0.203 | 4.032 |
| silent | 0.177 | -3.867 | 0.097 | -2.072 | -0.093 | 2.020 |
| paris | 0.222 | -4.183 | 0.167 | -3.143 | -0.077 | 1.479 |
| mobile | 0.172 | -4.071 | 0.099 | -2.333 | -0.096 | 2.330 |
| tempete | 0.184 | -4.723 | 0.108 | -2.742 | -0.181 | 4.856 |
| avrage | 0.182 | -3.840 | 0.105 | -2.197 | -0.119 | 2.634 |

Table 9. CE results for CE2 (B-frames), UVLC, Ref_frame=5

|  | TML9.0->VCEG-O22 |  | TML8.5->VCEG-O17 |  | VCEG-O22->VCEG-O17 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] |
| foreman | 0.247 | -4.873 | 0.099 | -1.959 | -0.144 | 2.992 |
| news | 0.214 | -3.721 | 0.178 | -3.084 | -0.073 | 1.272 |
| container | 0.207 | -3.910 | 0.048 | -1.017 | -0.258 | 4.951 |
| silent | 0.262 | -5.490 | 0.104 | -2.197 | -0.166 | 3.612 |
| paris | 0.335 | -6.325 | 0.205 | -3.861 | -0.149 | 2.900 |
| mobile | 0.285 | -6.303 | 0.141 | -3.137 | -0.129 | 2.985 |
| tempete | 0.247 | -6.215 | 0.135 | -3.389 | -0.240 | 6.377 |
| avrage | 0.257 | -5.263 | 0.130 | -2.664 | -0.166 | 3.584 |

Table 10. CE results for CE2 (B-frames), CABAC, Ref_frame=1

|  | TML9.0->VCEG-O22 |  | TML8.5->VCEG-O17 |  | VCEG-O22->VCEG-O17 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] |
| foreman | 0.278 | -5.868 | 0.179 | -3.778 | -0.102 | 2.228 |
| news | 0.214 | -3.749 | 0.186 | -3.183 | -0.062 | 1.101 |
| container | 0.175 | -3.455 | 0.057 | -1.077 | -0.190 | 3.797 |
| silent | 0.287 | -6.144 | 0.146 | -3.080 | -0.118 | 2.599 |
| paris | 0.336 | -6.313 | 0.223 | -4.172 | -0.143 | 2.759 |
| mobile | 0.229 | -5.363 | 0.148 | -3.500 | -0.085 | 2.065 |
| tempete | 0.235 | -6.045 | 0.164 | -4.181 | -0.177 | 4.773 |
| avrage | 0.251 | -5.277 | 0.157 | -3.282 | -0.125 | 2.760 |

Table 11. CE results for CE2 (B-frames), CABAC, Ref_frame=5

|  | TML9.0->VCEG-O22 |  | TML8.5->VCEG-O17 |  | VCEG-O22->VCEG-O17 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] | PSNR_diff [dB] | Bitrate_diff [%] |
| foreman | 0.416 | -8.580 | 0.180 | -3.742 | -0.222 | 4.898 |
| news | 0.302 | -5.235 | 0.218 | -3.710 | -0.114 | 2.018 |
| container | 0.166 | -3.223 | 0.034 | -0.744 | -0.222 | 4.279 |
| silent | 0.347 | -7.204 | 0.165 | -3.424 | -0.183 | 3.997 |
| paris | 0.416 | -7.840 | 0.258 | -4.888 | -0.177 | 3.491 |
| mobile | 0.326 | -7.208 | 0.201 | -4.485 | -0.112 | 2.594 |
| tempete | 0.287 | -7.321 | 0.206 | -5.230 | -0.212 | 5.738 |
| avrage | 0.323 | -6.659 | 0.180 | -3.746 | -0.177 | 3.859 |

5. Conclusion

This document shows the CE results on improved MB prediction modes. The results show that the improved MB prediction modes can improve coding efficiency of the TML in all of the tested conditions. Up to 8.5% bitrate reduction or 0.41dB PSNR improvement can be seen by the method of VCEG-O22, and up to 5.2% or up to 0.25dB improvement can be seen by the method of VCEG-O17.

The results shown here justify that the improved MB prediction modes definitely improve the coding efficiency. Both methods defined in the CE description provide improvement, but the method proposed in VCEG-O22 provides better improvement. Therefore, we propose to adopt it into the TML, with incorporation of any effective functions proposed in VCEG-O17, such like Intra predicted segment in a MB.

## IPR Policy

We argue 2.2. See attached policy statement.

# References

[1] Heiko Schwarz and Thomas Wiegand, “Tree-structured Macroblock Partition”, document VCEG-O17, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, TH, 4-6 December 2001.

[2] Satoru Adachi et.al, “Refined Results on the Low-overhead Prediction Modes”, document VCEG-O22, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, TH, 4-6 December 2001.

[3] Satoru Adachi, Heiko Schwarz, et.al, “Core Experiments Description on Improved Macroblock Prediction Modes”, document VCEG-O61, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, TH, 4-6 December 2001.

[4] Gary Sullivan and Gisle Bjontegaard, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan source Material,” document VCEG-N81, ITU- T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

[5] Gisle Bjontegaard, “Calculation of average PSNR differences between RD-curves”, document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, USA, April, 2001

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | NTT DoCoMo, Inc. |  |  |
| Mailing address | 3-5, Hikari-no-oka, Yokosuka, Kanagawa |  |  |
| Country | Japan |  |  |
| Contact person | Minoru Etoh |  |  |
| Telephone | +81 468 40 3515 |  |  |
| Fax | +81 468 40 3788 |  |  |
| Email | etoh@mml.yrp.nttdocomo.co.jp |  |  |
| Place and date of submission | Geneva, Jan.29-Feb.1, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Core Experiment Results on Improved Macroblock Prediction Modes |  |
| Contribution number |  | JVT-B090 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  | S.Adachi, S.Kato, M.Kobayashi, and M.Etoh |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: