| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B091 Filename: JVT-B091.doc Generated: 2002-01-23 |
| --- | --- |

| Title: | Proposed Requirements on CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minoru Etoh Satoru Adachi Mitsuru Kobayashi Frank Bossen(1) | Tel: Email: | +81-468-40-3515 etoh@computer.org |
| Source: | NTT DoCoMo, Inc. (1) DoCoMo Communications Laboratories USA, Inc. |  |  |

_____________________________

We request that the following requirements for the Joint Video Project be considered to the JVT targeted goals.

# Proposed Requirements

It is stated in the JVT requirements document[1] that JVT Codecs shall achieve 50% or greater bit rate savings over H.263v2 (with Annexes DFIJ&T) or MPEG-4 Advanced Simple Profile at all bit rates. To achieve this goal, we may use CABAC[2] that is expected to attain 10% improvement to UVLC in entropy coding. Here we have two concerns: (1) its feasibility in small mobile devices that require low-power consumption (with a considerably low LSI clock frequency) and (2) coding efficiency degradation sacrificed by generality of Unified Binarization Scheme[3].

1. Requirements on arithmetic coding core:

1.1 Coding efficiency shall be as high as possible and at least comparable to the current

test model.

1.2 The average processing time per symbol shall be as small as possible.

1.2 The size and the number of registers shall be as small as possible.

1.3 The delay in the encoder shall be bounded.

2. Requirements on context modeling:

2.1 Context design shall be flexible such as to satisfy the high coding efficiency

requirement.

2.2 Binarization shall be designed such as to minimize the number of binary arithmetic

coding operations.

# Discussion

With regard to implementation, adaptive arithmetic coding like CABAC has a drawback in that the decoding process must be serialized, that is, it can not be pipelined or parallelized. The decoding process thus requires high-frequency systems LSIs if implemented in a non-PC environment. For small mobile appliances in particular, low power consumption is essential.  We thus request JVT to consider light-weight implementations while retaining maximal coding efficiency.

With regard to the context design requirement, the unified binarization scheme which was proposed in [3] provides a single binarization scheme. That is in good accordance with the current UVLC design as far as context-modeling is singleton. In other words, the unified binarization is a better way to encode a generic integer value with one-state or two-state context. To achieve higher coding efficiency however, creating more contexts may be needed.  Generality of binarization sometimes prevents the design of a specific context-sensitive binarization such as MB_type context modeling.

Below we show a comparison between two CABAC results, namely VCEG-018 (unified binarization) and VCEG-023 (dedicated binarization).

Table 1.  Performance Comparison between Unified binarization and Specific binarization for coding MB prediction modes in CABAC (foreman).

| VCEG-O18 | (unified binarization) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| QP | PSNR | all_bit |  | mode_bit |  |
| 2 | 46.43 | 4672904 |  | 31090 |  |
| 6 | 43.24 | 2896592 |  | 33375 |  |
| 10 | 40.18 | 1750808 |  | 33163 |  |
| 14 | 37.28 | 1009728 |  | 31798 |  |
| 16 | 35.98 | 772584 |  | 30871 |  |
| 20 | 33.46 | 451176 |  | 28018 |  |
| 24 | 31.1 | 270688 |  | 25069 |  |
| 28 | 28.74 | 173640 |  | 23261 |  |
| VCEG-O23 | (dedicated binarization) |  |  |  |  |
| QP | PSNR | all_bit |  | mode_bit |  |
| 2 | 46.43 | 4666968 | -0.13% | 25074 | -19.35% |
| 6 | 43.24 | 2892456 | -0.14% | 29137 | -12.70% |
| 10 | 40.18 | 1748376 | -0.14% | 30663 | -7.54% |
| 14 | 37.28 | 1008960 | -0.08% | 30827 | -3.05% |
| 16 | 35.98 | 771984 | -0.08% | 30199 | -2.18% |
| 20 | 33.46 | 451008 | -0.04% | 27875 | -0.51% |
| 24 | 31.1 | 270312 | -0.14% | 24885 | -0.73% |
| 28 | 28.74 | 172952 | -0.40% | 22606 | -2.82% |

As a matter of course, the dedicated binarization with sophisticated context design outperforms the unified binarization, with which a context design is restricted to the structure of binary bins. The bit-saving in mode bits are up to 20%. We thus request JVT to consider the co-habitation of two binarization schemes: one for generic integer values and the other for symbol sets consisting of a limited number of symbols (e.g., MB modes).

# IPR Policy

We argue 2.2. See attached policy statement.

# References

- [1]	JVT Requirements Activity (Dave Lindbergh and Ajay Luthra, “Requirements ofr JVT Codec”, Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG,  document JVT-004, Thailand. Dec., 2001.

- [2]	Detlev Marpe et al., "Improving coding efficiency of H.26L using CABAC", document D.151, ITU-T Q6/16, WP 3/16 ,Porto Seguro, 28 May – 8 June 2001.

- [3]	Detlev Marpe et al., "Improved CABAC” Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG ,  document VCEG-O18r1, Thailand. Dec., 2001.

- [4]	Shun-ichi Sekiguchi et al., "Improvements of CABAC in Inter-frame Coding” Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG ,  document VCEG-O23, Thailand. Dec., 2001.

_________________

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | NTT DoCoMo, Inc. |  |  |
| Mailing address | 3-5, Hikari-no-oka, Yokosuka, Kanagawa |  |  |
| Country | Japan |  |  |
| Contact person | Minoru Etoh |  |  |
| Telephone | +81 468 40 3515 |  |  |
| Fax | +81 468 40 3788 |  |  |
| Email | etoh@mml.yrp.nttdocomo.co.jp |  |  |
| Place and date of submission | Geneva, Jan.29-Feb.1, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Core Experiment Results on Improved Macroblock Prediction Modes |  |
| Contribution number |  | JVT-B090 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  | S.Adachi, S.Kato, M.Kobayashi, and M.Etoh |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: