| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B100 Filename: JVT-B100.doc Generated: 2002-01-24 |
| --- | --- |

| Title: | Performance of CABAC for Interlaced Video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Detlev Marpe, Heiko Schwarz, and Thomas Wiegand Heinrich-Hertz-Institute (HHI) Einsteinufer 37 10587 Berlin, Germany | Tel: Fax Email: | +49-(0)30-31002 619 +40-(0)30-392 72 00 marpe@hhi.de hschwarz@hhi.de wiegand@hhi.de |
| Source: | Heinrich-Hertz-Institute (HHI) |  |  |

# Summary

In this contribution, we evaluate the improved CABAC entropy coding method with regard to interlaced source material. Our simulation results show that CABAC significantly improves the rate-distortion performance for the specified set of test sequences. We observed average BD bit-rate savings of 11%, or equivalently, BD PSNR gains of 0.45 dB in comparison to the UVLC entropy coding method.

# Experimental Results

The performance of the improved CABAC entropy coding method [1] was analyzed for the set of interlaced sequences specified in VCEG-N85 [2]. Only the sequences with half horizontal resolution have been used. The improved CABAC entropy coding was integrated into the TML-9.0 software. Comparisons are made against the UVLC entropy coding.

The common test conditions as specified in VCEG-N85 [2] and VCEG-N81 [3] are followed with some exceptions that will be explained below. The interlaced sequences have been coded using field pictures. (Each interlaced picture has been decomposed into two field pictures.) The following sequence structures have been adopted:

25 Hz sequences:		IB|BB|BB|PB|BB|BB|PB|BB|BB|PB|BB|BB|   IB|BB|BB|PB|BB|BB|PB|…

30 Hz sequences:		IB|BB|BB|PB|BB|BB|PB|BB|BB|PB|BB|BB|IB|BB|BB|   IB|BB|BB|PB|…

An Intra field is inserted every 500 milliseconds. Multi-frame prediction has not been used for the experiment. Thus, just one field picture is used as reference for motion compensation. This encoding option is not realistic; but if B-pictures are inserted, the TML-9.0 software cannot be configured in a way that both fields of an interlaced frame can be used as reference for motion compensation. Nevertheless, the obtained coding gains for the improved CABAC entropy coding method should be realistic for the regarded type of sequences. Motion compensation is always performed with ¼ pixel accuracy. The motion search is conducted in a range of [-32…32]x[-32…32] pixels. RD-optimisation is always enabled.

The quantizer value QP is held constant for all I- and P-pictures of a sequence. Quantizer values of QP=28, 24, 20, and 16 are used. The quantization parameter for B-pictures is set to QP(B)=QP(P)+2, where QP(P) is the quantization parameter for I- and P-pictures. Rate-distortion curves for all sequences are plotted in the accompanying Excel document. In Fig. 1-5, the obtained bit-rate savings are depicted over the quantization parameter QP. In addition table 1 shows the average PSNR gains and the corresponding average bit-rate savings [4] that are obtained if the improved CABAC is used as entropy coding method instead of the UVLC.

Table 1: Average PSNR gains and bit-rate savings

| Sequence | Format | # of frames | PSNR gain | Rate saving |
| --- | --- | --- | --- | --- |
| Canoe | 352x576 interlaced, 25 Hz | 220 | 0,44 dB | 10,24 % |
| Formula 1 | 352x576 interlaced, 25 Hz | 220 | 0,43 dB | 12,18 % |
| Rugby | 352x576 interlaced, 25 Hz | 220 | 0,40 dB | 9,35 % |
| Mobile | 352x480 interlaced, 30 Hz | 259 | 0,55 dB | 13,35 % |
| Football | 352x480 interlaced, 30 Hz | 259 | 0,41 dB | 9,95 % |
| Average |  |  | 0,45 dB | 11,01 % |

The presented results show that CABAC significantly improves the rate-distortion efficiency for the specified set of interlaced sequences. On average, about 11% of the bit-rate can be saved.

Figure 1: Bit-rate saving of the improved CABAC entropy coding in comparison with UVLC entropy coding for the sequence “Canoe” in half horizontal resoltuion

Figure 2: Bit-rate saving of the improved CABAC entropy coding in comparison with UVLC entropy coding for the sequence “Formula 1” in half horizontal resoltuion

Figure 3: Bit-rate saving of the improved CABAC entropy coding in comparison with UVLC entropy coding for the sequence “Rugby” in half horizontal resoltuion

Figure 4: Bit-rate saving of the improved CABAC entropy coding in comparison with UVLC entropy coding for the sequence “Mobile” in half horizontal resoltuion

Figure 5: Bit-rate saving of the improved CABAC entropy coding in comparison with UVLC entropy coding for the sequence “Football” in half horizontal resoltuion

# References

- Marpe et al., “New Results on Improved CABAC”, JVT-B101, Jan. 2002.

- Peter Borgwardt, “Core Experiment on Interlaced Video Coding”, VCEG-N85, October, 2001.

- Gary Sullivan and Gisle Bjontegaard, "Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material", VCEG-N81, September 2001.

- G. Bjontegaard, “Calculation of average PSNR differences between RD-Curves”, Doc. VCEG-M33, Mar. 2001.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Heinrich-Hertz-Institute |  |  |
| Mailing address | Einsteinstraße 37 10587 Berlin |  |  |
| Country | Germany |  |  |
| Contact person | Detlev Marpe |  |  |
| Telephone | +49-30-31 002-619 |  |  |
| Fax | +49-30-392 72 00 |  |  |
| Email | marpe@hhi.de |  |  |
| Place and date of submission | Berlin, Jan. 24, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Performance of CABAC for Interlaced Video |  |
| Contribution number |  | JVT-B100 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: