| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 2nd Meeting: Geneva, CH, Jan. 29 - Feb. 1, 2002 | Document: JVT-B101 Filename: JVT-B101.doc Generated: 2002-01-26 |
| --- | --- |

| Title: | New Results on Improved CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Detlev Marpe, Gabi Blättermann, Thomas Wiegand Image Processing Department Heinrich-Hertz-Institute (HHI) Einsteinufer 37 D-10587 Berlin, Germany Ragip Kurceren, Marta Karczewicz, Jani Lainema Nokia Research Center 6000 Connection Drive, 2-700 Irving, TX, 75039 | Tel: Email: Tel: Email: | +49 30 31 002 619 marpe@hhi.de +1 972 894 6907 ragip.kurceren@nokia.com |
| Source: | HHI, Nokia |  |  |

_____________________________

# Summary

In this document, we propose new coding elements and new context models for improving coding efficiency of CABAC, both for inter and intra frame coding. This proposal combines ideas of earlier contributions. Simulation results show performance gains of up to 5.2% BD bit-rate savings relative to the current CABAC specification for intra coding. In the case of inter coding (IPPP) improvements of up to 2.4% BD bit-rate reduction relative to the original CABAC method have been obtained.

# Background

At the Pattaya meeting, four contributions for improving the coding efficiency of CABAC entropy coding were presented [2]-[5]. As a common element, all these proposals were addressing the issue of improving the coding efficiency of transform coefficients. This proposal contains the essential ideas of [2] and [4] together with some further enhancements. However, one major change in comparison to the prior design given in [2] consists in the underlying binarization scheme. Instead of proposing the usage of the exp-Golomb code tree as a unifying binarization scheme, we recombined the new coding methods for transform coefficients as presented in [2] with the unary binarization as defined in the current Joint Model (JM1) [1]. The choice of an optimal binarization is still an open issue, which (hopefully) will be addressed in a forthcoming proposal.

# Description of Proposed Methods

In this proposal a new coding scheme for transform coefficients within the CABAC entropy coding framework is presented, which is based on two central ideas. First, an additional coding element, called COEFF_COUNT is introduced, which denotes the number of non-zero coefficients in a given block, and the second main idea is to exploit the given inter-symbol redundancies in transform coding in a more efficient way by using appropriately defined context models. In addition, we propose a minor modification for the scan mode in intra coding such that for all QP values the double scan is used.

Figure 1: Proposed new coding scheme for transform coefficients

## Coding of Coefficients by Using the Number of Significant Coefficients

An additional symbol COEFF_COUNT indicating the number of significant coefficients in a given block, serves as a substitute for the EOB symbol in the following way: First, the number of non-zero coefficients COEFF_COUNT is encoded. Then, in a second step, RUN, LEVEL and SIGN of all non-zero coefficients are encoded. Figure 1 illustrates our new encoding scheme. Since the initially encoded COEFF_COUNT provides some additional information of a given block in advance, we can exploit this information in the subsequent coding steps, as described in more detail in the following sections.

## Context Modeling for Coding of COEFF_COUNT

For capturing the correlations between COEFF_COUNTs of neighboring blocks we use appropriately designed context models to further increase coding efficiency of the transform coding unit. More specifically, if A and B denote the COEFF_COUNTs of the corresponding blocks to the left and on top of a given block, respectively, then the context model ctx_coeff_count(C) of the current COEFF_COUNT symbol C is defined by ctx_coeff_count(C) = ((A==0)?0:1) + 2*((B==0)?0:1). Note that this context model is only applied to the first binary decision (bin) in the unary binarization tree associated to the COEFF_COUNT symbol.

## Truncated Binarization for RUN

For encoding the RUN, we exploit the information of the initially encoded COEFF_COUNT and all previously encoded RUN elements, as shown in the pseudo-C code of Figure 1. Here, we make use of the fact that, if the maximum number of a given syntax element is known in advance, we can restrict the binarization to a truncated code tree. Suppose, we have given 10 non-zero coefficients in a 4x4 single scan block. Then, at the beginning of the loop, it is known that the maximum RUN (MaxRun) is given by the 6, since in this particular case the maximum number of coefficients MaxCoeff is given by 16. Consequently, in the (very unlikely) event of RUN=6 at the beginning of the block, we can omit the terminating “1” in the unary binarization of RUN=6 (cp. Table 4, [1]).  However, by using the information given by previously encoded RUNs we can further adapt the MaxRun counter, which may lead to potentially shorter binarized codewords of subsequent RUN elements to encode. In some cases, where, for instance, coefficients are aggregated at the end of the scan path, signaling of zero-valued RUN elements is completely omitted.

## Context Modeling for Coding of RUN

Context models for coding of RUN information depend on a threshold decision involving the coefficient counter, e.g. in 4x4 single scan inter coding mode, the context model ctx_run for luma is given by ctx_run = ((COEFF_COUNT) >= 4) ? 1 : 0. The idea behind this context design is that the COEFF_COUNT represents the activity of the given block, and that the probability distribution of the RUN symbol somehow depends on the activity of the block.

## Context Modeling  for Coding of LEVEL

Context models assigned to a bin representing a given LEVEL depend on the previously encoded or decoded LEVEL within the block. More specifically, similar to the current design, the symbols are first classified based on coefficient/coding mode, and then the following context model is applied for bins of LEVELs:

Proposed Context Model for LEVEL

if (bin_nr>3)

bin_nr=3;

end

if (Prev_level>MAX_LEVEL)

prev_level=MAX_LEVEL;

end

context_nr = (bin_nr-1)*MAX_LEVEL+Prev_level,

where the Prev_level is initialized to zero at the beginning of each block and MAX_LEVEL=3. Note that for double-scan, this variable is initialized at the beginning of each scan, i.e., twice per block. In contrast to earlier proposals, this context model is also used for coding of LEVEL in inter frame mode.

## Double Scan Always for CABAC Intra Mode

In the current Joint Model [1], the scan mode for intra coding depends on the QP value: for QP < 24, the double scan is used, whereas for the high QP24 the single scan mode is employed. We examined both scan modes in the high QP range with respect to coding efficiency, and our results documented in the next section revealed that it is more beneficial for CABAC entropy coding to use double scan mode always, i.e., independently of the given QP value.

# Experimental Results

The results are obtained by a modified version of TML 9.0 software that implements the proposed schemes. In the following we provide a summary of the results of the experiments while further results can be found in the Excel-document JVT-B101.xls. Below, the simulation results are summarized into three sets of results: Test A) and B) All the frames are encoded as I-frames. Test C) All frames except the first frame, which is an I-frame, are encoded as P-frames. The test conditions from VCEG-N81 were followed.

| QP | Container | Foreman | News | Silent | Paris | Mobile | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -3.97 | -4.50 | -3.57 | -5.17 | -4.26 | -5.22 | -5.06 | -4.53 |
| High | -2.54 | -2.19 | -1.88 | -2.70 | -2.52 | -3.32 | -2.87 | -2.57 |

Table 1: Performance improvement (Bjontegaard Delta bitrate savings in %) w.r.t. CABAC (TML9.0) of new context models in CABAC Intra coding. QP = 2, 6, 10, 14 is used for low QP and QP = 16, 20, 24, 28 for high QP.

Table 1 illustrates the results of Test A, i.e., intra-coding. Here, the results are reported as improvements in Bjontegaard Delta bitrate savings in % w.r.t. TML 9.0 CABAC performance for low and high QP ranges. As can be seen from Table 1, the average performance improvements are between 3.57-5.22% and 1.88-3.32% for low and high QP ranges, respectively. The improvements are higher for smaller QP values.

| QP | Container | Foreman | News | Silent | Tempete | Mobile | Paris | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Very High | -1.49 | -0.71 | -0.31 | -0.32 | -0.85 | -1.51 | -0.69 | -0.84 |

Table 2: Performance improvement (Bjontegaard Delta bitrate savings in %) for using double scan mode always in CABAC Intra coding. Performance gains are relative to our improved CABAC operated in single scan mode. QP = 24, 26, 28, 31 is used for the very high QP range.

Table 2 contains results for Test B, where the usage of double scan for the whole QP range was evaluated. As can be seen from Table 2, the improvements are in the range from 0.31-1.51% BD bitrate savings with an average of 0.84%. Note that these gains are relative to the improved CABAC operating in single mode as evaluated in Test A.

| QP | Container | Foreman | News | Silent | Tempete | Mobile | Paris | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -2.16 | -2.11 | -1.60 | -1.59 | -2.36 | -2.14 | -2.43 | -2.06 |
| High | -1.31 | -1.00 | -0.92 | -0.23 | -0.92 | -1.14 | -0.98 | -0.93 |

Table 3: Performance improvement (Bjontegaard Delta bitrate savings in %) w.r.t. CABAC (TML9.0) of new context models in CABAC Inter coding. QP = 2, 6, 10, 14 is used for low QP and QP = 16, 20, 24, 28 for high QP.

Table 3 finally illustrates results of Test C, i.e., inter-coding, similar to the results illustrated in Table 1. As can be observed from , for inter frame coding, the improvements are between 1.59-2.43% for low QP and between 0.23-1.31% for high QP ranges. Again the improvements are higher for smaller QP values. It can also be noted that the gains for inter coding are lower than the ones observed in intra-coding case.

Figure 2: CABAC intra coding performance relative to UVLC: bitrate savings [%] versus QP obtained by original CABAC (JM1) and our proposed improved CABAC for “Mobile”.

Figure 3: CABAC inter coding performance relative to UVLC: bitrate savings [%] versus QP obtained by original CABAC (JM1) and our proposed improved CABAC for “Mobile”.

#### References

- Wiegand, “Joint Model Number 1 (JM-1)”, JVT-A003.

- Marpe et al., “Improved CABAC”, VCEG-O18.

- Sekiguchi et al., “Improvements of CABAC Inter-frame Coding”, VCEG-O23.

- Kurceren et al., “Improvements of CABAC”, VCEG-O24.

- Bäse et al., “Enhanced Coding of Coefficients”, VCEG-34.

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Heinrich-Hertz-Institute |  |  |
| Mailing address | Einsteinstraße 37 10587 Berlin |  |  |
| Country | Germany |  |  |
| Contact person | Detlev Marpe |  |  |
| Telephone | +49-30-31 002-619 |  |  |
| Fax | +49-30-392 72 00 |  |  |
| Email | marpe@hhi.de |  |  |
| Place and date of submission | Berlin, Jan. 25, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | New Results on Improved CABAC |  |
| Contribution number |  | JVT-B101 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Inc. |  |  |
| Mailing address | 6000 Connection Drive, Irving, TX-75039 |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 972 8944188 |  |  |
| Fax | +1 972 8944589 |  |  |
| Email | Marta.Karczewicz@nokia.com |  |  |
| Place and date of submission | 1/2/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | New Results on Improved CABAC |  |
| Contribution number |  | JVT-B101 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: