| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C022 Filename: JVT-C022.doc |
| --- | --- |

| Title: | Additional “Zero-motion” Sub-block Mode for “Motion Copy” |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | 360-817-7531 ssun@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

In Geneva meeting, an AHG is formed to evaluate GMVC [1], GMC [2], and “Motion Copy” [3]. This contribution provides results of a study for “Motion Copy” with an additional “zero-motion” sub-block mode. It will hopefully provide useful information for the AHG and JVT to reach a final decision.

# 1. Introduction

The “Motion Copy”, as suggested by Nokia, replaces the conventional copy (or skip) mode with a “motion copy” MB mode. The motion vector for the “motion copy” mode is derived as the following: if one of the four block neighbors (Up, Left, Up-Left, Up-Right) has a zero motion vector, the motion vector for the SKIP mode Macroblock is zero; otherwise the median predicted motion vector for Inter 16x16 mode is used.

Questions have been raised during the AHG studies whether the original skip mode should somehow be still kept in use. Therefore, an additional “zero-motion” sub-partition mode was proposed for the 8x8 (split) MB mode as a modification to the “Motion Copy”. The modified version was referred as Mcopy_r1 or MC_r1 in the AHG. A difference of the “zero-motion” mode from the “skip” concept is that residuals can be coded for the subblock if it is signaled by CBP. However, when CBP signals zero residual for a “zero-motion” subblock, the decoding of the subblock will be “skipped”, similar to the conventional “skip” mode, but as a smaller block. The suggested change is shown in Table 1.

Table 1. Block Modes for 8x8 sub-Partition in P frames.

Experiments have been conducted to compare the “Motion Copy” and modified “Motion Copy”, and the results provided in this document have already been released to the AHG.

# 2. Experiments

The Experiments were based on JM1.7. The “Motion Copy” codec was kindly provided by Nokia to the AHG. The modified “Motion Copy” codec was also released to the AHG. For all the following experiments, RDOPT is on, search range is 32, number of reference frames is 5, all MB modes are enabled, and the symbol mode is UVLC.

Table 1. Comparison between “MC_r1” and “Motion Copy” for P-frame Coding

Table 2. Comparison between “MC_r1” and “Motion Copy” for B-frame Coding

2.1 P-frame coding

Thirteen sequences were tested for P-frame coding. Table 1 shows the results. The MC_r1 achieves slightly better coding gain (2.31% saving) than “Motion Copy” (2.13% saving).

2.2 B-frame coding

Seven sequences were tested for B-frame coding. Two B frames were inserted before each P frame. The results are shown in Table 2. On average, the modified “Motion Copy” performs better (0.58% saving) than “Motion Copy” (0.34% saving) when B frames are considered.

2.3 Slice-mode coding

Six sequences were selected by the AHG for this part of experiments. Slice mode is set to 1. Slice argument is 22 for QCIF, and 44 for CIF sequences. Results are available in Table 3. The modified version performs on average also better (2.00% saving) for this case than “Motion Copy” (1.74% saving).

Table 3. Comparison between “MC_r1” and “Motion Copy” for Slice-mode Coding

3. Experimental Results of the Revised “Motion Copy”

During the AHG studies, the “Motion Copy” method has been revised [4]. The revised rule is the following: if one of the two block neighbors (Up, Left) has a zero motion vector, the motion vector for the SKIP mode Macroblock is zero; otherwise the median predicted motion vector for Inter 16x16 mode is used. The revised “Motion Copy” is referred as “MC_r2”. And let us call the “zero motion” update as “MC_r1+2”. Tables 4, 5, and 6 summarized the experiments based on the revision. The relative improvement by the “zero motion” sub-partition mode is in general consistent with the result in Section 2. However, MC_r2 has less coding gain in the slice model experiment comparing to the original “Motion Copy.” The results show that among all the choices, MC_r1 described in Sections 1 and 2 so far has given the best performance.

# Additional Comment

The relative improvement by the “zero motion” sub-partition mode is also consistent with the experiments conducted by other Expert in the AHG [5].

# Summary

Since there is no clear consensus from the AHG effort, as suggested by the AHG, it is proposed that JVT consider the “Zero Motion” sub partition as necessary addition to the “Motion Copy” method if it is to be included in the JM, and it is further suggest that JVT consider the integration of GMVC, GMC, “Motion Copy”, and “Zero Motion” sub partition as presented in [5] and [6].

Table 4. Comparison between “MC_r1+2” and “MC_r2” for P-frame Coding

Table 5. Comparison between “MC_r1+2” and “MC_r2” for B-frame Coding

Table 6. Comparison between “MC_r1+2” and “MC_r2” for Slice-mode Coding

# References

[1] Shijun Sun and Shawmin Lei, " Global Motion Vector Coding (GMVC)," document JVT-B019, JVT Meeting, Geneva, January-February 2002.

[2] Hideaki Kimata, “GMVC and GMC Switched by MV,” document JVT-B046, JVT meeting, Geneva, January-February 2002.

[3] Gary Sullivan, Thomas Wiegand, and Ajay Luthra, “JVT 2nd Meeting Report,” document JVT-B001d4, JVT Meeting, Geneva, January-February 2002.

[4] Jani Lainema, “Skip Mode Motion Compensation,” document JVT-C027, JVT Meeting, Fairfax, May 2002.

[5] Hideaki Kimata, “GMVC and GMC with B-picture,” document JVT-C043, JVT Meeting, Fairfax, May 2002.

[6] Shijun Sun and Shawmin Lei, “Experimental results of GMVC and ‘Motion Copy’,” document JVT-C021, JVT Meeting, Fairfax, May 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sharp Labs of America |  |  |
| Mailing address | 5750 Pacific Rim Blvd, Camas, WA 98607 |  |  |
| Country | USA |  |  |
| Contact person | Shijun Sun |  |  |
| Telephone | 360 817 7531 |  |  |
| Fax | 360 817 8436 |  |  |
| Email | ssun@sharplabs.com |  |  |
| Place and date of submission | Fairfax, May 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | ## Additional “Zero-motion” Sub-block Mode for “Motion Copy” |  |
| Contribution number |  | JVT-C022 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Provisional |  |
| Inventor(s)/Assignee(s) |  | Shijun Sun |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | To support a royalty-free baseline we follow ITU-T 2.2.1 for Baseline Profile applications agreeing to a free license on condition that all other patent holders do the same. |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: