| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C023r1 Filename: JVT-C023r1.doc |
| --- | --- |

| Title: | Lossless Coding and QP Range Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information/Proposal |  |  |
| Author(s) or Contact(s): | Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | 360-817-7531 ssun@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

Since the Geneva meeting [1], there have been intensive discussions on QP range selection, especially on the negative QP’s for high fidelity video coding. This contribution provides results of a simple lossless coding scheme based on the existing JM framework. The information can hopefully be considered as a guideline for the QP selection.

# 1. Introduction

Since the current transforms and quantization could cause data loss by rounding or shifting operation, the transform and quantization steps are skipped in order to achieve a simple lossless coding. To make this easy to fit into the current codec, the existing CABAC is used to code the image residuals after Intra or Inter predictions.

# 2. Experimental results

2.1 Lossless Coding

The Experiments were based on JM1.7. For the lossless coding, RDOPT is on, search range is 32, number of reference frames is 5, all MB modes are enabled, and the symbol mode is CABAC. Since the lossless coding approach still uses all the possible Intra and Inter predictions, it fits naturally into I, P, and B frames. However, in this experiment, each sequence is coded as one I frame followed by all P frames; no B frame is inserted. The loopfilter is turned off for the lossless coding.

The results are listed in Table 1. Among thirteen tested sequences, the compression ratio ranges from 2.06 to 4.69; the average is 2.80. The ratio is comparable with or might be even better than the reported results for Digital Cinema archival applications.

Table 1. Compression Ratio of the Lossless Video Coding

2.2 Comparison with JM Lossy Coding

Lossy coding results of JM1.7 were obtained to compare with the lossless coding. The coding condition is same as stated in Section 2.1 with RDOPT, CABAC, 5 reference frames, search range of 32, all MB modes, and IPPP. QP values of (–6), (-7), and (–8) were chosen for the comparison. The loopfilter is kept as it is in JM1.7. The compression ratios are shown in Table 2. Results of QP of (–10) are based on an extension of JM1.7 [2].

Table 2. Comparison of the Compression Ratio of JM1.7 coding and Lossless Coding

2.3 Comments on QP Range Selection

Although data for QP less than (-10) are not available at this point, it is expected that QP less than (-10) or even (-8) would not be necessary if a lossless coding can be achieved with less complexity and similar compression ratio.

3. Discussion

3.1 Signaling Lossless Coding Mode

There are several ways of signaling the lossless coding. One is to signal it in the slice or picture level, then the whole slice will be coded as lossless, and QP and Dquant might become redundancies that could also be skipped.

Another way is to define a special QP, such as (-9) or (-10), for this lossless coding purpose, the decision of lossy or lossless coding can then be made per MB, the possible modification needed for this option will be to protect the losslessly coded pixels from loopfilter so that they can be used for deblocking of neighboring lossy MB’s, but with their own values kept the same.

An idea in between will be to signal lossless coding by a special QP (-9), and in the meanwhile skip Dquant automatically, similar to proposal JVT-C052, so the switchback to lossy coding is prevented and loopfilter is naturally switched off.

3.2 Possible Improvement

The results presented were based on an approach staying as close as possible to the existing JM working draft. Since the characteristic of the image residual is different from that of the transform coefficients, the CABAC models can be further improved or special model can be introduced if the lossless coding option is desired for the JVT.

In the meanwhile, experiments showed the average compression ratio for lossless coding is only around 2.0 for the 13-sequence test set when UVLC is used. For example, when QP is (-10), JM1.7 gives average compression ratio of 1.95. While with several minor modifications, the lossless compression ratio is 2.04 on average. The changes for the lossless coding are: no double scan (enabled by JVT-C102), separate VLC for level and run (no 2D VLC), alternative horizontal zigzag scan (less than 1% saving, not proposed). There should be more room for new UVLC-based code design if it is also desired.

# Summary

We have demonstrated that a decent lossless coding performance can be achieved based on the existing JM framework by simply skipping transform and quantization steps. It is proposed that QP range be selected based upon this information with a lower limit of (-8), as implemented in JM1.7. It is further proposed that JVT consider this lossless coding option for the standard with a special QP (-9) to signal lossless coding and in the meanwhile skipping Dquant, as proposed in JVT-C052, removing double scan as proposed in JVT-C102 for UVLC, and coding level/run in separate UVLC codes.

# References

[1] Gary Sullivan, Thomas Wiegand, and Ajay Luthra, “JVT 2nd Meeting Report,” document JVT-B001d4, JVT Meeting, Geneva, January-February 2002.

[2] Louie Kerofsky, “Modifications to JVT IDCT” document JVT-C025, JVT Meeting, Fairfax, May 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sharp Labs of America |  |  |
| Mailing address | 5750 Pacific Rim Blvd, Camas, WA 98607 |  |  |
| Country | USA |  |  |
| Contact person | Shijun Sun |  |  |
| Telephone | 360 817 7531 |  |  |
| Fax | 360 817 8436 |  |  |
| Email | ssun@sharplabs.com |  |  |
| Place and date of submission | Fairfax, May 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | ## Lossless Coding and QP Range Selection |  |
| Contribution number |  | JVT-C023 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Provisional |  |
| Inventor(s)/Assignee(s) |  | Shijun Sun |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | To support a royalty-free baseline we follow ITU-T 2.2.1 for Baseline Profile applications agreeing to a free license on condition that all other patent holders do the same. |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: