| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C028 Filename: JVT-C028r1.doc |
| --- | --- |

| Title: | Context-adaptive VLC (CVLC) coding of coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gisle Bjøntegaard and Karl Lillevold RealNetworks, Inc. 2601 Elliott Ave Seattle, WA 98121, USA | Tel: Email: | +206 892 6286 gisle.bjontegaard@telenor.com karll@real.com |
| Source: | RealNetworks |  |  |

_____________________________

# Overview

CVLC is a method of coding transform coefficients.  It replaces the UVLC coding of transform coefficients (Tcoeff) for luma as well as chroma.  All other elements are identical to the description of UVLC coding.

The motivation for using CVLC can be summarized in the following way:

- Highly context adaptive coding result in very good coding efficiency over the whole quality range of the standard.

- Zig-zag scanning is still used.  Separation of the coding of Run and Level allows for better adaptivity and thereby better coding efficiency.

- Separation of Run and Level also result in low complexity with low demand on memory.

The following coding elements are used to replace the Tcoeff elements of the UVLC coding method:

  - If there are non-zero coefficients, it is typically observed that there is a string of coefficients at the highest frequencies that are 1.  Hence, a common parameter Num-Trail is used that contains the number of coefficients as well as the number of "Trailing 1s" (from now referred to as T1s).  For T1s only the sign has to be coded.

  - For coefficients other than the T1s, Level information is coded.  Since this is a one dimensional parameter, coding is simplified and well structured VLCs are used.

  - Lastly, the Run information is coded.  Since the number of coefficients is already known, this sets a limit to the Run, which is used for additional compression.  Run is split into Total-run for all coefficients and Run before each non-zero coefficient.

Zig-zag scanning is used, but in the transmission of coefficient data, both levels and runs, the scanning is done in reverse order.  Therefore, in the Level information, the signs of T1s appear first (in reverse order), then the Level information of the last coefficient where this is needed, and so on.  Run information is coded similarly.  First Total number of zeros in Runs is coded, followed by Run before the last nonzero coefficient, and so on.

Coding flow

Luma:

{ [ NumTrail →  SignTrail →  Levels(16) →  TotalZeros →  Runs ] * 4  } * 4

Chroma DC:

[  NumTrail →  SignTrail →  Levels(4) →  TotalZeros →  Runs ] * 2

Chroma AC:

{ [ NumTrail →  SignTrail →  Levels(15) →  TotalZeros →  Runs ] * 4 } * 2  }

Intra 16x16 DC:

NumTrail →  SignTrail →  Levels(16) →  TotalZeros →  Runs

Intra 16x16 AC:

[ NumTrail →  SignTrail →  Levels(15) →  TotalZeros →  Runs ] * 16 (4x4 blocks)

# Num-Trail

Three luma and one chroma DC VLC tables are used for combined coding of number of coefficients and T1s, i.e. one codeword to signal both parameters.  VLCs are listed in the tables below.  T1s is clipped to 3.  Any remaining trailing 1s are coded as normal levels.

Table 1: Num-VLC0

| NumCoef\T1s | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| 0 | 1 | - | - | - |
| 1 | 000011 | 01 | - | - |
| 2 | 00000111 | 0001001 | 001 | - |
| 3 | 000001001 | 00000110 | 0001000 | 00011 |
| 4 | 000001000 | 000001011 | 000000101 | 000010 |
| 5 | 0000000111 | 000001010 | 000000100 | 0001011 |
| 6 | 00000000111 | 0000000110 | 0000001101 | 00010101 |
| 7 | 000000001001 | 00000000110 | 0000001100 | 00010100 |
| 8 | 000000001000 | 00000001001 | 000000001010 | 000000111 |
| 9 | 0000000000111 | 000000001011 | 000000000101 | 0000000101 |
| 10 | 0000000000110 | 0000000001101 | 0000000001111 | 00000001000 |
| 11 | 00000000000011 | 0000000001100 | 0000000001110 | 000000000100 |
| 12 | 00000000000010 | 00000000000100 | 00000000000110 | 0000000000101 |
| 13 | 00000000000101 | 00000000000111 | 000000000010001 | 00000000001001 |
| 14 | 000000000000011 | 000000000000010 | 000000000010000 | 0000000000000011 |
| 15 | 00000000000000001 | 00000000000000011 | 00000000000000010 | 00000000000000101 |
| 16 | 00000000000000000 | 000000000000001001 | 0000000000000010001 | 0000000000000010000 |

Table 2: Num-VLC1

| NumCoeff\T1s | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| 0 | 11 | - | - | - |
| 1 | 000011 | 011 | - | - |
| 2 | 000010 | 00011 | 010 | - |
| 3 | 001001 | 001000 | 001010 | 101 |
| 4 | 1000001 | 001011 | 100101 | 0011 |
| 5 | 00000111 | 1000000 | 1000010 | 00010 |
| 6 | 00000110 | 1000011 | 1001101 | 10001 |
| 7 | 000001001 | 10011101 | 10011100 | 100100 |
| 8 | 000001000 | 000001011 | 000000101 | 1001100 |
| 9 | 0000000111 | 000001010 | 000000100 | 10011111 |
| 10 | 0000000110 | 0000001101 | 0000001100 | 10011110 |
| 11 | 00000000101 | 00000000111 | 00000001001 | 000000111 |
| 12 | 00000000100 | 00000000110 | 00000001000 | 0000000101 |
| 13 | 000000000011 | 000000000010 | 000000000100 | 000000000111 |
| 14 | 0000000000011 | 000000000101 | 0000000000010 | 0000000001101 |
| 15 | 00000000000001 | 00000000000000 | 000000000000111 | 0000000001100 |
| 16 | 000000000000101 | 000000000000100 | 0000000000001101 | 0000000000001100 |

Table 3: Num-VLC2

| NumCoeff\T1s | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| 0 | 0011 | - | - | - |
| 1 | 0000011 | 0010 | - | - |
| 2 | 0000010 | 101110 | 1101 | - |
| 3 | 000011 | 101001 | 010110 | 1100 |
| 4 | 000010 | 101000 | 010001 | 1111 |
| 5 | 101101 | 101011 | 010000 | 1110 |
| 6 | 101100 | 101010 | 010011 | 1001 |
| 7 | 101111 | 010101 | 010010 | 1000 |
| 8 | 0110101 | 010100 | 011101 | 00011 |
| 9 | 0110100 | 010111 | 011100 | 00010 |
| 10 | 0110111 | 0110110 | 0110000 | 011111 |
| 11 | 01111001 | 0110001 | 01111010 | 0110011 |
| 12 | 01111000 | 01111011 | 01100101 | 01100100 |
| 13 | 000000011 | 000000010 | 000000100 | 000000111 |
| 14 | 0000000011 | 000000101 | 0000001101 | 0000001100 |
| 15 | 0000000010 | 00000000011 | 00000000010 | 00000000001 |
| 16 | 0000000000001 | 000000000001 | 00000000000001 | 00000000000000 |

Table 4: Num-VLC_Chroma_DC

| NumCoeff\T1s | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| 1 | 0001 | 1 | - | - |
| 2 | 00001 | 00111 | 01 | - |
| 3 | 00110 | 000001 | 001010 | 00100 |
| 4 | 0000001 | 00000000 | 00000001 | 001011 |

## Table Selection

For all elements, except chroma DC, a choice between three tables and one FLC must be made.    Selection is done as follows:  N is calculated based on the number of coefficients in the block above and to the left of the current block: NU and NL. In the table below, X means that the block is available in the same slice:

Table 5: Calculation of N for Num-VLCN

| Upper block (NU) | Left block (NL) | N |
| --- | --- | --- |
| X | X | (Nl+Nu)/2 |
| X |  | Nu |
|  | X | Nl |
|  |  | 0 |

0 <= N < 2 : Num-VLC0
2 <= N < 4 : Num-VLC1
5 <= N < 8 : Num-VLC2

N >= 8 : 6 bit FLC xxxxyy, as follows:

NumCoeff–1 is transmitted in the first 4 bits (xxxx).  The last 2 bits (yy) are used for T1.  Since NumCoeff=0 also has to be represented, the codeword 000011 is used for this.

For chroma DC, Num-VLC_Chroma_DC is used.

# Coding of Level information

First, the sign of T1s are coded with 1 bit each. For the remaining level information, four structured VLCs are used to code level.  The level tables are listed in .

Table 6: Level tables

Lev-VLC0

1

0 1

0 0 1

..

0 0 0 0 0 0 0 0 0 0 0 0 0 1

0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x x x

0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x x x x x x x x x x x

Lev-VLC1

1 x

0 1 x

0 0 1 x

..

0 0 0 0 0 0 0 0 0 0 0 0 0 1 x

0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x x x

0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x x x x x x x x x x x

Lev-VLC2

1 x x

0 1 x x

0 0 1 x x

..

0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x

0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x x x

0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x x x x x x x x x x x

Lev-VLC3

1 x x x

0 1 x x x

0 0 1 x x x

..

0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x x

0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x x x

0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x x x x x x x x x x x

Lev-VLC4

1 x x x x

0 1 x x x x

0 0 1 x x x x

..

0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x x x

0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 x x x x x x x x x x x x

The code numbers in each VLC increase according to the number of leading zeros and the natural interpretation of the string of xxx.  Normally levels to be coded take values  1,  2 etc. However, for the first coefficient to be coded (after T1s) and if T1s < 3 or Number_of_coefficients = T1s, levels to be coded take values  2,  3 etc (Level’).  Since the first coefficient is always coded with Lev-VLC0 or Lev-VLC1 (see section 3.1), these levels starting at  2 are only applicable to Lev-VLC0 and Lev-VLC1.

Levels are assigned according to ascending code numbers.  Positive values receive the lowest code number and negative values receive the next code number.

The last two entries in each table can be considered escape codes. The first escape code, with four “x”’s, is used to code the 8 levels above the last regularly coded level.  The next escape code, with 12 “x”’s, is used to code all remaining levels.

To see how this works out, see Table 7, where a selection of code words for each Lev-VLC table are listed.

Table 7: Level table examples

| Lev-VLC0 |  |  |  |
| --- | --- | --- | --- |
| Code no. | Code | Level (1, 2..) | Level’ (2, 3..) |
| 0 | 1 | 1 | 2 |
| 1 | 01 | -1 | -2 |
| 2 | 001 | 2 | 3 |
| 3 | 0001 | -2 | -3 |
| 4 | 00001 | 3 | 4 |
| .. | .. | .. | .. |
| 13 | 00000000000001 | -7 | -8 |
| 14 | 000000000000001xxxx | 8 to 15 | 9 to 16 |
| 15 | 000000000000001xxxxxxxxxxxx | 16 -> | 17 -> |
| Lev-VLC1 |  |  |  |
| Code no. | Code | Level (1, 2..) | Level’ (2, 3..) |
| 0 | 10 | 1 | 2 |
| 1 | 11 | -1 | -2 |
| 2 | 010 | 2 | 3 |
| 3 | 011 | -2 | -3 |
| 4 | 0010 | 3 | 4 |
| 5 | 0011 | -3 | -4 |
| .. | .. | .. | .. |
|  | 000000000000010 | 14 | 15 |
|  | 000000000000011 | -14 | -15 |
|  | 000000000000001xxxx | 15 to 22 | 16 to 23 |
|  | 000000000000001xxxxxxxxxxxx | 23 -> | 24 -> |
| Lev-VLC2 |  |  |  |
| Code no. | Code | Level (1, 2..) |  |
| 0 | 100 | 1 |  |
| 1 | 101 | -1 |  |
| 2 | 110 | 2 |  |
| 3 | 111 | -2 |  |
| 4 | 0100 | 3 |  |
| 5 | 0101 | -3 |  |
| 6 | 0110 | 4 |  |
| 7 | 0111 | -4 |  |
| 8 | 00100 | 5 |  |
| .. | .. | .. |  |
|  | 0000000000000110 | 28 |  |
|  | 0000000000000111 | -28 |  |
|  | 000000000000001xxxx | 29 to 36 |  |
|  | 000000000000001xxxxxxxxxxxx | 37 -> |  |
| Lev-VLC3 |  |  |  |
| Code no. | Code | Level (1, 2..) |  |
| 0 | 1000 | 1 |  |
| 1 | 1001 | -1 |  |
| 2 | 1010 | 2 |  |
| 3 | 1011 | -2 |  |
| 4 | 1100 | 3 |  |
| 5 | 1101 | -3 |  |
| 6 | 1110 | 4 |  |
| 7 | 1111 | -4 |  |
| 8 | 01000 | 5 |  |
| .. | .. | .. |  |
|  | 00000000000001110 | 56 |  |
|  | 00000000000001111 | -56 |  |
|  | 000000000000001xxxx | 57 to 64 |  |
|  | 000000000000001xxxxxxxxxxxx | 65 -> |  |
| Lev-VLC4 |  |  |  |
| Code no. | Code | Level (1, 2..) |  |
| 0 | 10000 | 1 |  |
| 1 | 10001 | -1 |  |
| 2 | 10010 | 2 |  |
| 3 | 10011 | -2 |  |
| .. | .. | .. |  |
|  | 11110 | 8 |  |
|  | 11111 | -8 |  |
|  | 010000 | 9 |  |
| .. | .. | .. |  |
|  | 0000000000000011110 | 120 |  |
|  | 0000000000000011111 | -120 |  |
|  | 000000000000001xxxxxxxxxxxx | 121 -> |  |

## Table Selection

For improved coding efficiency, the tables are changed along with the coding process based on QP value, number of coefficients, and the size of the previously coded level value.

After each Level is encoded, the VLC number is updated according to the following method, where Level is the absolute value of the previously coded level.

### Inter and intra with QP >= 9

First coefficient with VLC0. Next VLC1. 
Increase VLC by one (up to 2) if Level> 3

### Intra with QP < 9

if (number of coefficients > 10)
	First coefficient with VLC1.  Next VLC2.  
else
	First coefficient with VLC0.  Next VLC1.

if (vlc = VLC1) change to VLC2 if Level> 3.  
if (vlc >=VLC2) increase vlc by one (up to 4) if Level > 5

The same procedure is used for chroma AC and DC coefficient levels.

# Coding of Run information

Run coding is separated in total number of Zeros (i.e. the number of zeros located before the last non-zero coefficient) and Run (of zeros) before each coefficient.  This works especially well at high bitrates where there are typically several coefficients with no Run (of zeros) at the beginning of the forward scan.

## TotalZeros

The parameter TotalZeros is the sum of all zeros located before the last non-zero coefficient in a forward scan.  For example, given the string of coefficients 0 0 3 0 0 4 0 0 0 0 2 0 1 0 0 0, TotalZeros will be 2+2+4+1=9.  Since NumCoeff is already known, it determines the maximum possible value of TotalZeros.  One out of 15 VLC tables is chosen based on NumCoeff.

If NumCoeff indicates that all coefficients are non-zero, TotalZeros is not coded since it is known to be zero.

Table 8: TotalZeros tables for all 4x4 blocks

| NumCoeff TotZeros | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 111 | 0010 | 111101 | 01000 | 101100 | 111000 |  |
| 1 | 011 | 101 | 1101 | 1110 | 01010 | 101101 | 111001 |  |
| 2 | 010 | 011 | 000 | 0110 | 01011 | 1010 | 11101 |  |
| 3 | 0011 | 001 | 010 | 1010 | 1110 | 001 | 1001 |  |
| 4 | 0010 | 000 | 1011 | 000 | 011 | 010 | 1111 |  |
| 5 | 00011 | 1000 | 1111 | 100 | 100 | 000 | 00 |  |
| 6 | 00010 | 0101 | 011 | 110 | 1111 | 110 | 01 |  |
| 7 | 000011 | 1001 | 100 | 1011 | 110 | 111 | 101 |  |
| 8 | 000010 | 1100 | 0011 | 010 | 101 | 100 | 110 |  |
| 9 | 0000011 | 01000 | 1110 | 001 | 001 | 011 | 100 |  |
| 10 | 0000010 | 11011 | 1010 | 0111 | 000 | 10111 | - |  |
| 11 | 00000001 | 11010 | 11000 | 1111 | 01001 | - | - |  |
| 12 | 00000000 | 010010 | 110011 | 111100 | - | - | - |  |
| 13 | 00000011 | 0100111 | 110010 | - | - | - | - |  |
| 14 | 000000101 | 0100110 | - | - | - | - | - |  |
| 15 | 000000100 | - | - | - | - | - | - |  |
| NumCoeff TotZeros | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 | 101000 | 111000 | 10000 | 11000 | 1000 | 100 | 00 | 0 |
| 1 | 101001 | 111001 | 10001 | 11001 | 1001 | 101 | 01 | 1 |
| 2 | 10101 | 11101 | 1001 | 1101 | 101 | 11 | 1 | - |
| 3 | 1011 | 1111 | 101 | 111 | 0 | 0 | - | - |
| 4 | 110 | 00 | 01 | 0 | 11 | - | - | - |
| 5 | 00 | 01 | 11 | 10 |  | - | - | - |
| 6 | 111 | 10 | 00 | - | - | - | - | - |
| 7 | 01 | 110 | - | - | - | - | - | - |
| 8 | 100 | - | - | - | - | - | - | - |
| 9 | - | - | - | - | - | - | - | - |
| 10 | - | - | - | - | - | - | - | - |
| 11 | - | - | - | - | - | - | - | - |
| 12 | - | - | - | - | - | - | - | - |
| 13 | - | - | - | - | - | - | - | - |
| 14 | - | - | - | - | - | - | - | - |
| 15 | - | - | - | - | - | - | - | - |

Table 9: TotalZeros table for chroma DC 2x2 blocks

| NumCoeff TotZeros | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 1 |
| 1 | 01 | 01 | 0 |
| 2 | 001 | 00 | - |
| 3 | 000 | - | - |

## Run before each coefficient

At this stage it is known how many zeros are left to distribute (call this ZerosLeft).  When encoding or decoding a non-zero coefficient for the first time, ZerosLeft begins at TotalZero, and decreases as more non-zero coefficients are encoded or decoded.

For example, if there is only 1 zero left, the run before the next coefficient must be either of length 0 or 1, and only one bit is needed.

The number of preceding zeros before each non-zero coefficient (called RunBefore) needs to be coded to properly locate that coefficient. Before coding the next RunBefore, ZerosLeft is updated and used to select one out of 7 tables.  RunBefore does not need to be coded in the following two situations:

- If the total number of zeros has been reached (ZerosLeft = 0)

- For the last coefficient in the backward scan. Then the value is known to be ZerosLeft.  This also means that the maximum value to be coded is 14.

Table 10 Tables for Run before each coefficient

| Runsleft Run Before | 1 | 2 | 3 | 4 | 5 | 6 | >6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 01 | 01 | 01 | 01 | 000 |
| 1 | 0 | 01 | 00 | 00 | 00 | 00 | 010 |
| 2 | - | 00 | 11 | 11 | 11 | 101 | 101 |
| 3 | - | - | 10 | 101 | 101 | 100 | 100 |
| 4 | - | - | - | 100 | 1001 | 111 | 111 |
| 5 | - | - | - | - | 1000 | 1101 | 110 |
| 6 | - | - | - | - | - | 1100 | 0011 |
| 7 | - | - | - | - | - | - | 0010 |
| 8 |  | - | - | - | - | - | 00011 |
| 9 | - | - | - | - | - | - | 00010 |
| 10 | - | - | - | - | - | - | 00001 |
| 11 | - | - | - | - | - | - | 0000011 |
| 12 | - | - | - | - | - | - | 0000010 |
| 13 | - | - | - | - | - | - | 0000001 |
| 14 | - | - | - | - | - | - | 00000001 |

# Results

The proposed method is implemented in JM 1.1 and the results of the simulations are summarized in the tables below in terms of Bjontegaard Delta bitrate savings in % from JM-1.1. We followed the test conditions from VCEG-N81.  The results below were obtained with the software uploaded to the JVT ftp site on May 1 (JVT-C028-software.zip), which was a minor update (~0.3%) over the software released on April 25.

Table 11: Performance improvement (Bjontegaard Delta bitrate savings in %) of the proposed coefficient coding in Intra coding. QP = 0, 4, 8, 12 is used for low QP and QP = 16, 20, 24, 28 for high QP.

| QP | Contnr | Foremn | News | Silent | Tempt | Mobile | Paris | Flower |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -16.70 | -12.45 | -16.95 | -14.95 | -19.21 | -23.11 | -17.66 | -25.25 |
| High | -6.61 | -3.16 | -8.03 | -2.14 | -6.53 | -9.93 | -6.92 | -11.78 |

Table 12: Performance improvement (Bjontegaard Delta bitrate savings in %) of the proposed coefficient coding in Inter coding. QP = 0, 4, 8, 12 is used for low QP and QP = 16, 20, 24, 28 for high QP.

| QP | Contnr | Foremn | News | Silent | Tempt | Mobile | Paris | Flower |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -7.67 | -9.92 | -12.59 | -10.36 | -15.33 | -16.05 | -14.27 | -17.24 |
| High | -2.55 | -0.85 | -3.53 | -1.13 | -2.34 | -3.28 | -3.15 | -3.43 |

The detailed results can be found in the spreadsheet JVT-C028r1.xls.  From these we can see that the bitrate reductions are always higher for lower QP values.  For Mobile CIF it reaches about 31% for Intra and 24% for Inter and QP 0.  For a sequence not included among the standard test sequences, Flowergarden CIF, the maximum bitrate reductions are 33% and 26% for Intra and Inter, respectively.

The differences between CVLC and other coefficient coding proposals are listed in the tables below, where a positive number is in favor of CVLC.  The Nokia results are obtained running the software provided by Nokia in  JVT-C088-software.zip, using the same config files as for CVLC, and as originally provided by Nokia.  The TI results are from the JVT-C086 document.  We have also included a few alternative sequences, i.e. sequences not used for the common test conditions, but frequently used by the ITU and MPEG in the past.  The details are included in JVT-C028r1_xls.zip.

Table 13: Performance relative to Nokia JVT-C088, for Inter

| QP | Contnr | Foremn | News | Silent | Tempt | Mobile | Paris | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | 2.75 | 2.24 | 1.35 | 1.88 | 2.13 | 1.94 | 1.44 | 1.96 |
| High | 1.92 | 0.31 | 0.42 | -0.17 | 1.2 | 2.7 | 1.26 | 1.09 |

Table 14: Performance relative to Nokia JVT-C088, for Intra

| QP | Contnr | Foremn | News | Silent | Tempt | Mobile | Paris | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | 0.91 | 0.61 | -0.28 | 0.89 | 0.88 | 0.29 | -0.16 | 0.45 |
| High | 1.09 | 0.52 | 0.39 | -0.83 | 0.7 | 1.61 | 1.16 | 0.66 |

Table 15: Performance relative to Nokia JVT-C088, for Inter, alternative sequences

| QP | Bus | Flower | Coastg | Grama | M&D | Hall | Stefan | Table | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | 2.48 | 1.53 | 2.32 | 2.70 | 2.45 | 2.77 | 2.14 | 2.83 | 2.40 |
| High | 1.14 | 3.08 | 1.49 | 0.34 | 0.95 | 0.67 | 3.68 | 1.24 | 1.57 |

Table 16: Performance relative to Nokia JVT-C088, for Inter, alternative sequences

| QP | Bus | Flower | Coastg | Grama | M&D | Hall | Stefan | Table | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | 1.17 | 0.71 | 1.88 | 0.36 | 0.52 | 1.14 | 0.95 | 1.00 | 0.97 |
| High | 1.23 | 2.10 | 2.26 | -0.98 | -0.24 | 0.26 | 1.37 | 2.42 | 1.05 |

Table 17: Performance relative to TI JVT-C086, for Intra

| QP | Contnr | Foremn | News | Silent | Tempt | Mobile | Paris | Flower | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | 1.08 | 1.80 | 1.41 | 2.38 | 2.05 | 0.78 | 0.51 | 1.08 | 1.43 |
| High | 0.98 | 0.82 | 0.80 | 0.64 | 0.86 | 1.32 | 0.97 | 0.98 | 0.91 |

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | RealNetworks, Inc. |  |  |
| Mailing address | RealNetworks, Inc. 2601 Elliott Ave Seattle, WA 98121 |  |  |
| Country | USA |  |  |
| Contact person | Gary Greenbaum |  |  |
| Telephone | +1 206 674 2339 |  |  |
| Fax | +1 206 674 3580 |  |  |
| Email | garyg@real.com |  |  |
| Place and date of submission | 3rd JVT meeting May 6 – 11, Fairfax |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Context-adaptive VLC (CVLC) coding of coefficients |  |
| Contribution number |  | JVT-C028 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: For purposes of this contribution, the terms of 2.2.1 above [and as detailed in document VCEG-O54 submitted by Gary Sullivan] are agreed to only as necessary to comply with the required portions of a “baseline” JVT video profile.