| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 March, 2002 | Document: JVT-C033 Filename: JVT-C033.doc |
| --- | --- |

| Title: | Scalable Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Incorporated 12500 TI Boulevard, MS 8649 Dallas, Texas 75243, USA | Tel: Email: | +1 214 480-3816 zhou@ti.com |
| Source: | Texas Instruments |  |  |

# Introduction

In the current H.26L [1] a nine-mode based intra prediction scheme is adopted (see section 2 for details). Compared to the previous six-mode based scheme, the newly adopted scheme is more efficient but increases both memory requirement and computational complexity by a factor about four. While the coding efficiency increase is of top priority on the high-end devices, the complexity increase is not favorable for the low-end H.26L applications. Therefore, it is desired to have a complexity-scalable intra prediction that can serve the needs of both the low- and high-end applications.

In contribution we proposed a complexity-scalable intra prediction scheme for H.26L video coding to satisfy the different needs of low- and high-end H.26L devices. The scheme is based on a design of a memory scalable probability table that allows the scheme to operate in low-complexity or high-complexity mode. In the low-complexity mode, the scheme only needs to store a sub-table of the probability table and the entropy table used for encoding the prediction modes. While in the high-complexity mode the entire probability and the entropy table has to be stored. The high-complexity mode is fully backward compatible to the low-complexity mode. The experiments showed that the proposed scheme provides the comparable coding efficiency with the current nine-mode based and previous six-mode based intra prediction scheme when operating in the high- and low- complexity mode, respectively.

- Problem with Intra Prediction H.26L

The intra prediction method described in H.26L section 4.4.4 [1]  requires the storage of two tables (i.e. table 1 and table 2) on the decoder side. The probability table (Table 3 in [1]) requires 10x10x9/2 = 450 bytes memory for ASIC design.  For DSPs this number is higher if each number in Table 3 is stored as byte (8-bit), which leads to memory size of 900 bytes. On top of that, the UVLC table (table 2) requires 9x9 = 81 bytes for storage. Considering that the memory requirement for storing the rest of UVLC tables in H.26L is less than 338 bytes, the memory requirement of the intra prediction is extremely high, and can be easily critical for ASIC and DSP implementation.

In the low-complexity H.26L codecs, it is desired to use only a subset of the total nine intra prediction modes in order to reduce the memory requirement and computational complexity. However, the current intra prediction method is lack of the complexity scalability. In table 3 [1] prediction mode 0, 1, 2, …8 spread over all the nine probability positions in each string. Therefore, even if a subset of the prediction modes is used, the decoder has to store a portion of Table 1 and entire table 2. For example, if only 6 modes are used, the memory size for DSP implementation is 7x7x9 (portion of table 3) + 81 (UVLC table) = 522 bytes. What is more, experimental results revealed that the use of six modes out of nine modes leads to worse performance than the previous 6-mode based algorithm in JVT WD1.

- Complexity Scalable Intra Prediction

The complexity scalable intra prediction method is aimed to satisfy the needs of low- and high-complexity H.26L codecs. The memory and computational resource is highly limited on low-end devices, so a low-complexity intra prediction scheme is desired. While on high-end devices (such as PCs) where memory and computational complexity is not a concern, coding efficiency is of high priority and a complex and but highly efficient intra prediction scheme can be used. In addition, the backward compatibility between the low- and high-complexity schemes should be considered during the design. It is desired that the high-end decoder can decode the bitstreams generated by the low-end devices, which inevitably requires that the low-complexity intra-prediction scheme be a subset of the high-complexity scheme.

Table 3 and table 4 are  used to explain the concept of the complexity-scalable intra prediction described above. In this particular case two complexity layers are established (i.e. M = 9, N =2, M0 = 6, M1 = 9 ). There are in total 9 prediction modes (0,1,2,…8), only six modes (0,1,2,3,4,5) are supported in the low-complexity layer. So the 9 modes are divided into two sub-groups, layer 0 with mode 0,1, 2, 3, 4, 5, layer 1 with mode 0, 1, 2, 3, 4, 5, 6, 7,8. In order to ensure the highest possible coding efficiency in the low-complexity scheme, any mode in layer 0 has higher probability than mode 6, 7, 8.

A limitation then is imposed on construction of the probability table. As shown in Table 3, for B\A less than 6 the strings are divided into two groups, “red” and “black”. Mode 0, 1, ..5 can only appear in “red” part (position from 0 to 5), ordered by their probabilities, while mode 6, 7, 8 can only spread in the “black” part (position from 6 to 8). For B\A larger than 5, no limitation is imposed, nine modes can spread all over the nine positions in each string, depending on their probabilities.

In this way, the low-complexity intra prediction scheme which uses mode 0 to 5 only needs to store the sub-tables marked “red” in table 3 and 4, while the high-complexity scheme stores entire table 3 and table4. Since the low-complexity scheme is strictly a subset of the high-complexity scheme, the backward compatibility is guaranteed.

The low-complexity scheme has the least amount of memory requirement. In this particular case, only 7x7x6 (probability sub-table) + 36 (UVLC sub-table) = 310 bytes are needed for storing the tables in the low-complexity scheme, which is about 40% saving compared to 522 bytes needed in the H.26L intra prediction described in previous sections when six modes used.

- TABLE 3

- Prediction mode as a function of ordering signaled in the bitstream in a complexity scalable scheme

| B\A | outside 0 1 2 3 4 5 6 7 8 |
| --- | --- |
| outside 0 1 2 3 4 5 6 7 8 | 0-------- 021------ 102------ 201------ 012------ 012------ 012------ 012------ 012------ 012------ 045------ 041325687 104325678 230415687 304215867 043512876 045132786 603247158 014675238 084351267 045------ 014325867 102435867 201345687 032154678 041325768 014532867 263105784 106527438 038214765 045------ 024315867 120345867 210345687 320145678 042351768 021534768 263147580 261748035 230481675 045------ 304152768 310425678 231054678 304215867 403512687 304512786 360218457 013568742 380452716 405------ 403512876 401352867 245031867 430512867 405312876 405312876 436521087 405138762 480531267 504------ 504312876 015432786 204513768 530412687 450312876 504132786 560328417 451038276 584362107 504------ 016348527 261357048 261038754 360251784 401362587 146805732 632104785 016538724 038654721 504------ 015863724 105742386 210538647 035124678 045687321 501364278 310625847 103547862 053812476 504------ 840317652 038142567 231408756 384156702 480357261 048357261 306482157 043817526 843051672 |

Table 4, the proposed UVLC table used for (prob0, prob1) coding

UVLC_Prob[prob0=0,1,…8][prob1=0,1,…8]

= {

{ 0,  2,  3,  6,  8, 13,  9, 18, 15, },

{ 1,  5, 12, 17, 23, 31, 27, 35, 34, },

{ 4, 11, 19, 24, 30, 43, 37, 47, 42, },

{ 7, 16, 22, 28, 36, 50, 45, 54, 52, },

{10, 20, 29, 38, 51, 59, 61, 66, 63, },

{21, 26, 40, 53, 60, 71, 72, 76, 70, },

{48, 33, 44, 57, 68, 77, 78, 80, 79, },

{25, 39, 49, 56, 62, 73, 74, 75, 64, },

{14, 32, 41, 46, 55, 65, 67, 69, 58, },

};

- Proposed Complexity Scalable Intra Prediction Scheme to H.26L

we proposed use the following complexity scalabale intra prediction scheme to replace the current H.26L one. The scheme contains nine prediction modes defined in this section together with the table 3 and table 4 described in the last section. The (prob0, prob1) coding is unchanged expect for the UVLC table (table 4) is different. Note that the modes defined below already existed in the previous H.26L versions.

Compared to the H.26L intra prediction, the proposed one has the following advantages:

- simplicity: only nine boundary pixels from the adjacent blocks (A, B, C, D, E, F, G, H, I in figure 4, see figure 1 also) are used in prediction, mode 0 to mode 5 are much simpler than those in the current H.26L

- complexity-scalability: allows two layers of complexity scalability, a low-complexity codec can support the first six prediction mode, while a high-complexity codec can use all the nine modes. Full backward compatibility between the high- and low-complexity codecs is guaranteed.

- FIGURE 4

Syntax diagram for the Picture header.

For the luminance signal, there are 9 intra prediction modes labeled 0 to 8.  Mode 0 is ‘DC-prediction’ (see below).  The other modes represent directions of predictions as indicated below.

2

4

3

1

6

7

5

8

- FIGURE 5

Mode 0: DC prediction

Generally all pixels are predicted by (A+B+C+D+E+F+G+H)//8.  If four of the pixels are outside the picture, the average of the remaining four is used for prediction.  If all 8 pixels are outside the picture the prediction for all pixels in the block is 128.  A block may therefore always be predicted in this mode.

Mode 1: Vertical/Diagonal Prediction

This mode is used only if all A,B,C,D are inside the picture.

a is predicted by: 			(A+B)/2

e is predicted by 			B

b,i are predicted by 		(B+C)/2

f,m are predicted by 		C

c,j are predicted by 		(C+D)/2

d,g,h,k,l,n,o,p are predicted by 	D

Mode 2: Vertical prediction

If A,B,C,D are inside the picture,  a,e,i,m are predicted by A,  b,f,j,n by B etc.

Mode 3: Diagonal prediction

This mode is used only if all A,B,C,D,E,F,G,H,I are inside the picture.  This is a 'diagonal' prediction.

m is predicted by: 			(H+2G+F)//4

i,n are predicted by 		(G+2F+E)//4

e,j,o are predicted by 		(F+2E+I)//4

a,f,k,p are predicted by 		(E+2I+A)//4

b,g,l are predicted by 		(I+2A+B)//4

c,h are predicted by 		(A+2B+C)//4

d is predicted by 			(B+2C+D)//4

Mode 4: Horizontal prediction

If E,F,G,H are inside the picture,  a,b,c,d are predicted by E,  e,f,g,h by F etc.

Mode 5: Horizontal/Diagonal prediction

This mode is used only if all E,F,G,H are inside the picture.

a is predicted by: 			(E+F)/2

b is predicted by 			F

c,e are predicted by 		(F+G)/2

f,d are predicted by 		G

i,g are predicted by 		(G+H)/2

h,j,k,l,m,n,o,p are predicted by 	H

Mode 6: Vertical-Left prediction

This mode is used only if all A,B,C,D,E,F,G,H,I are inside the picture.  This is a 'diagonal' prediction.

a, j are predicted by 	(I + A + 1)>>1;

b, k are predicted by 	 (A + B + 1)>>1;

c, l are predicted by 	 (B + C + 1)>>1;

d  is predicted by 	 (C + D + 1)>>1;

e, n are predicted by 	 (E + 2*I + A + 2)>>2;

f, o are predicted by 	 (I + 2*A + B + 2)>>2;

g, p are predicted by 	 (A + 2*B + C + 2)>>2;

h  is predicted by 	 (B + 2*C + D + 2)>>2;

I is predicted by	 	 (I + 2*E + F + 2)>>2;

m is predicted by 		 (E + 2*F + G + 2)>>2;

Mode 7: Vertical-Right prediction

This mode is used only if all A,B,C,D,E,F,G,H,I are inside the picture.  This is a 'diagonal' prediction.

a  is predicted by 	 ( 2*(A+B+G)+F+H+4)>>3;

b, i are predicted by 	 (B + C + 1)>>1;

c, j are predicted by 	 (C + D + 1)>>1;

d, k are predicted by 	 (D + E + 1)>>1;

l  is predicted by 	 (E + F + 1)>>1;

e  is predicted by 	 (2*(B+H) + A + C + G + H + 4)>>3;

f, m are predicted by 	 (B + 2*C + D + 2)>>2;

g, n are predicted by 	 (C + 2*D + E + 2)>>2;

h, o are predicted by 	 (D + 2*E + F + 2)>>2;

p is  predicted by 	 (E + 2*F + G + 2)>>2;

Mode 8: Horizontal-Down prediction

This mode is used only if all A,B,C,D,E,F,G,H,I are inside the picture.  This is a 'diagonal' prediction.

a, g are predicted by 	 (I + E + 1)>>1;

b, h are predicted by 	 (E + 2*I + A + 2)>>2;

c is predicted by 	 (I + 2*A + B +2)>>2;

d is predicted by 	 (A + 2*B + C +2)>>2;

e, k are predicted by 	 (E + F + 1)>>1;

f, l are predicted by 	 (I + 2*E + F + 2)>>2;

i, o are predicted by 	 (F + G + 1)>>1;

j, p are predicted by 	 (E + 2*F + G + 2)>>2;

m is predicted by 	 (G + H + 1)>>1;

n is predicted by 	 (F + 2*G + H +2)>>2;

- Experimental Results

Simulation was carried out on the seven H.26L test sequences. H.26L reference software JM1.9 was used. The simulation conditions follow the recommendation specified in [2], except for that all the frames are coded as I-frames. QP =16, 20, 24, 28 are used. Table 5, 6, 7, 8 list the simulation results. TML6_JVTWD1 denotes the previous version of H.26L intra prediction which used mode 0 to 5 defined in section 4. TML9_JVTWD2 standards for the current H.26L intra prediction based on the nine modes defined. TML6_JVTWD2 represents the current 9-mode based intra prediction but with mode 5, 6, 8 disabled. Low-complexity uses only the six modes in the proposed intra prediction scheme, while high-complexity uses all the nine prediction modes defined in the previous section. At the end of each table, the average result over the seven test sequences is listed, followed by “Diff” which shows the PSNY difference and bit-rate reduction with respect to TML6_JVTWD1.

| Sequence | Intra Prediction with 6 modes |  |  |  |  |  | Intra Prediction with 9 modes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TML6_JVTWD1 |  | TML6_JVTWD2 |  | low_complexity |  | TML9_JVTWD2 |  | high_complexity |  |
|  | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] |
| container | 37.12 | 251.12 | 37.11 | 254.51 | 37.11 | 251.14 | 37.15 | 249.75 | 37.14 | 248.43 |
| foreman | 36.79 | 244.96 | 36.82 | 256.64 | 36.79 | 245.12 | 36.87 | 242.72 | 36.83 | 238.64 |
| mobile | 35.32 | 7171.01 | 35.31 | 7236.69 | 35.31 | 7175.97 | 35.38 | 7092.07 | 35.37 | 7114.38 |
| news | 37.68 | 273.70 | 37.65 | 278.76 | 37.68 | 274.10 | 37.72 | 269.99 | 37.71 | 270.33 |
| paris | 36.61 | 2137.31 | 36.60 | 2152.90 | 36.61 | 2136.45 | 36.63 | 2099.83 | 36.63 | 2110.59 |
| silent | 36.38 | 406.83 | 36.36 | 410.86 | 36.38 | 407.57 | 36.43 | 396.56 | 36.43 | 403.91 |
| tempete | 36.12 | 4598.66 | 36.10 | 4646.08 | 36.11 | 4602.17 | 36.17 | 4545.71 | 36.17 | 4574.45 |
| average | 36.57 | 2154.80 | 36.56 | 2176.63 | 36.57 | 2156.07 | 36.62 | 2128.09 | 36.61 | 2137.25 |
| Diff | 0.00 | 0.00% | -0.01 | 1.01% | -0.00 | 0.06% | 0.05 | -1.24% | 0.04 | -0.81% |

Table 5, simulation result for QP = 16

| Sequence | Intra Prediction with 6 modes |  |  |  |  |  | Intra Prediction with 9 modes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TML6_JVTWD1 |  | TML6_JVTWD2 |  | low_complexity |  | TML9_JVTWD2 |  | high_complexity |  |
|  | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] |
| container | 34.36 | 171.62 | 34.34 | 174.15 | 34.37 | 171.70 | 34.39 | 170.08 | 34.40 | 169.32 |
| foreman | 34.03 | 162.03 | 34.04 | 173.37 | 34.03 | 162.34 | 34.10 | 162.46 | 34.09 | 157.60 |
| mobile | 31.84 | 5223.63 | 31.83 | 5281.66 | 31.83 | 5228.01 | 31.89 | 5150.99 | 31.88 | 5171.85 |
| news | 34.60 | 197.61 | 34.61 | 201.72 | 34.59 | 197.97 | 34.63 | 194.13 | 34.62 | 194.96 |
| paris | 33.56 | 1512.34 | 33.55 | 1526.83 | 33.55 | 1511.19 | 33.57 | 1477.80 | 33.57 | 1488.13 |
| silent | 33.56 | 261.38 | 33.54 | 265.80 | 33.55 | 262.33 | 33.59 | 255.03 | 33.57 | 259.79 |
| tempete | 32.90 | 3190.58 | 32.88 | 3234.16 | 32.89 | 3192.04 | 32.93 | 3142.01 | 32.93 | 3170.03 |
| average | 33.55 | 1531.31 | 33.54 | 1551.10 | 33.54 | 1532.23 | 33.59 | 1507.50 | 33.58 | 1515.95 |
| Diff | 0.00 | 0.00% | -0.01 | 1.29% | -0.01 | 0.06% | 0.04 | -1.56% | 0.03 | -1.00% |

Table 6, simulation result for QP = 20

| Sequence | Intra Prediction with 6 modes |  |  |  |  |  | Intra Prediction with 9 modes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TML6_JVTWD1 |  | TML6_JVTWD2 |  | low_complexity |  | TML9_JVTWD2 |  | high_complexity |  |
|  | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] |
| container | 31.56 | 113.51 | 31.55 | 115.31 | 31.56 | 113.30 | 31.58 | 111.95 | 31.59 | 111.44 |
| foreman | 31.31 | 101.09 | 31.31 | 111.85 | 31.30 | 101.43 | 31.40 | 104.99 | 31.36 | 98.87 |
| mobile | 28.55 | 3557.63 | 28.54 | 3607.02 | 28.54 | 3560.68 | 28.59 | 3494.05 | 28.58 | 3514.95 |
| news | 31.62 | 136.02 | 31.57 | 139.19 | 31.62 | 136.32 | 31.62 | 132.48 | 31.65 | 133.64 |
| paris | 30.55 | 1021.23 | 30.54 | 1032.71 | 30.54 | 1019.64 | 30.56 | 991.49 | 30.56 | 999.62 |
| silent | 31.05 | 161.21 | 31.02 | 163.78 | 31.06 | 161.54 | 31.07 | 159.10 | 31.06 | 159.69 |
| tempete | 29.88 | 2070.06 | 29.85 | 2106.48 | 29.87 | 2071.29 | 29.89 | 2031.69 | 29.89 | 2055.66 |
| average | 30.65 | 1022.96 | 30.63 | 1039.48 | 30.64 | 1023.46 | 30.67 | 1003.68 | 30.67 | 1010.55 |
| Diff | 0.00 | 0.00% | -0.02 | 1.61% | -0.00 | 0.05% | 0.03 | -1.89% | 0.02 | -1.21% |

Table 7, simulation result for QP = 24

| Sequence | Intra Prediction with 6 modes |  |  |  |  |  | Intra Prediction with 9 modes |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TML6_JVTWD1 |  | TML6_JVTWD2 |  | low_complexity |  | TML9_JVTWD2 |  | high_complexity |  |
|  | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] | PSNRY[dB] | Bitrate [Kbps] |
| container | 28.83 | 72.43 | 28.83 | 74.40 | 28.83 | 72.57 | 28.85 | 71.90 | 28.86 | 71.33 |
| foreman | 28.60 | 60.60 | 28.64 | 70.52 | 28.60 | 61.10 | 28.73 | 66.93 | 28.64 | 59.98 |
| mobile | 25.43 | 2314.34 | 25.42 | 2350.36 | 25.42 | 2316.17 | 25.48 | 2272.49 | 25.46 | 2286.83 |
| news | 28.62 | 89.36 | 28.60 | 90.71 | 28.63 | 89.52 | 28.62 | 86.99 | 28.66 | 87.88 |
| paris | 27.58 | 667.80 | 27.58 | 673.06 | 27.58 | 666.76 | 27.60 | 643.16 | 27.60 | 651.26 |
| silent | 28.50 | 94.56 | 28.49 | 95.78 | 28.50 | 94.73 | 28.49 | 94.65 | 28.49 | 94.24 |
| tempete | 27.01 | 1262.16 | 26.98 | 1279.23 | 27.01 | 1261.33 | 27.00 | 1236.81 | 27.02 | 1254.32 |
| average | 27.80 | 651.61 | 27.79 | 662.01 | 27.80 | 651.74 | 27.82 | 638.99 | 27.82 | 643.69 |
| Diff | 0.00 | 0.00% | -0.00 | 1.60% | 0.00 | 0.02% | 0.03 | -1.94% | 0.02 | -1.21% |

Table 8, simulation result for QP = 28

As shown Table 5 to 8, the current 9-mode based intra prediction scheme provides up to about 2% gain compared to the previous six-mode based intra prediction scheme (TML6_JVTWD1). However, it underperfoms the previous 6-mode based intra prediction scheme by up to 1.6% in average if only the six corresponding predction modes are turned on. With the proposed complexity-scalable intra prediction scheme, it can provide roughly the same coding efficiency as the previous six-mode based intra prediction scheme (TML6_JVTWD1) when operating in low-complexity mode. When operating in high-complexity mode, the proposed schemes underperforms the current 9-mode based one about 0.7% in average.

- Conclusions

The proposed complexity-scalable intra prediction scheme provides the comparable coding efficiency with the current nine-mode based and previous six-mode based intra prediction scheme when operating in the high- and low- complexity mode, respectively. Compared to the current intra prediction scheme, the proposed one is much simpler in terms of computational complexity and satisfies the needs of both the low- and high-end H.26L applications due to its nature of complexity scalability. Also, it guarantees the backward compatibility between the high- and low-complexity modes.

- Reference

[1]  H.26L Joint Video Team Working Draft (JVT-WD2), 2nd JVT meeting, Geneva, Switzerland, Jan.29 – Feb.2, 2002

[2] Recommended Simulation Conditions for H.26L, ITUT-T VCEG-M75

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Texas Instruments |  |  |
| Mailing address | 12500 TI Blvd., Dallas, TX-75243 |  |  |
| Country | USA |  |  |
| Contact person | Minhua Zhou |  |  |
| Telephone | +1 408 3816 |  |  |
| Fax | +1 972 6969 |  |  |
| Email | zhou@ti.com |  |  |
| Place and date of submission | Fairfax, VA, USA May 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Scalable Intra Predication |  |
| Contribution number |  | JVT-C033 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| x | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: