| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C034 Filename: JVT-C034.doc |
| --- | --- |

| Title: | A novel fast fractional-pel motion estimation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhibo Chen, Yun He Dept. of Electronic Engineering, Tsinghua University, Beijing, China Yidong Chen SVA Group, Co.Ltd, Shanghai, China | Tel: Email: | 86-10-62781413 chenzb@video.mdc.tsinghua.edu.cn hey@tsinghua.edu.cn |
| Source: | Tsinghua University, China and SVA Group, China |  |  |

_____________________________

(Begin text of document here: 11-point font is suggested for short documents, 10 for long ones)

1. Introduction

Hybrid coding framework has been successfully adopted in various moving picture coding standards, such as H.261, MPEG-1, MPEG-2, H.263, and MPEG-4. Comparing with H.263, some significant changes have been adopted in H.26L: the residue coding is 4x4 block based, the DCT is replaced by integer transform, universal VLC table is used, 1/4 and 1/8 pixel accuracy motion estimation is proposed, etc. A PSNR analysis shows the superiority of the H.26L results of averages more than 2 dB better in luma PSNR across all test cases, relative to ISO-MPEG’s chosen MPEG-4 “Advanced Simple” profile anchor performance. However, H.26L is essentially still based on hybrid coding framework, motion estimation and compensation take even more important parts in achieving the high coding efficiency.

Generally motion estimation is conducted into two steps: the first step is integer pixel motion vector estimation; and the second is fractional pixel motion vector estimation. For fractional pixel motion estimation, 1/2-pel accuracy is frequently used (H.263, MPEG-1, MPEG-2, MPEG-4), higher resolution motion vector are adopted recently in MPEG-4 (1/4-pel accuracy)[1] and H.26L (1/4, 1/8-pel accuracy)[2].

Algorithms on fast motion estimation are always hot research spot. Especially fast integer pixel motion estimation has achieved much more attention because traditional fractional pixel motion estimation (such as 1/2-pel) only takes a very little proportion in the computation load of motion estimation. However, with the development of integer fast motion estimation algorithm and the decreasing of integer motion search points, computation load of fractional pixel motion estimation become more and more comparable to that of integer case. For instance, some center-biased fast integer motion vector estimation algorithms have reduced the checking pixel positions averagely down to 10[3][4], while 8 1/2-pel positions are needed to be checked around the best integer pixel matching position in full 1/2-pel motion estimation, and more positions should be searched if higher resolution motion estimation is adopted, such as 16 positions in 1/4-pel case and 24 positions in 1/8-pel case, even possesses a much higher proportion than integer pixel motion estimation. While from our knowledge, there may not have relate work investigating fast fractional pixel motion estimation algorithms in the proposals, hence, this proposal presents test results and shows both necessary and significant on this topic.

In any cases (1/2 pixel, 1/4 pixel or 1/8 pixel resolution) a roughly computation reduction of 62.5% in fractional pixel motion estimation can be achieved compared with the full fractional pixel motion search algorithm. And experimental results show that the strategy keeps a better performance in preserving image quality and makes little influence on the bit rate.

Experimental results are given in the next section.

2. Experiment Results:

In following table I, ‘original’ represents the original fractional motion estimation algorithm adopted in H.26L, and ‘fast’ represents our proposed fast algorithm. All test sequences are coded in Cif format, 100 frames, 30f/s, with a constant Qp=10. If it is not marked specifically, only one reference frame and one Inter block search Mode (16x16) is chosen. “Stefan (7Mode)” represents the case when seven Inter block search Modes are chosen and “Stefan (5)” represents the case when 5 reference frames are chosen. Therefore our fast algorithm can keep a better performance in preserving image quality in different mode.

Table I Average PSNR of P frames

| Sequence | PSNR/dB |  |
| --- | --- | --- |
|  | Original | Fast |
| News | 41.621 | 41.608 |
| Car Phone | 42.381 | 42.373 |
| Coast Guard | 38.832 | 38.824 |
| Akyio | 44.860 | 44.840 |
| Stefan | 39.818 | 39.821 |
| Stefan (7Mode) | 39.866 | 39.820 |
| Stefan (5) | 41.452 | 41.466 |

[1] MPEG-4 Video Verification Model version 16.0, ISO/IEC JTC1/SC29/WG11 N3312, Noordwijkerhout , March,  2000

[2] “H.26L Test Model Long Term Number 8 (TML-8) draft0”, ITU VCEG, June, 2001

[3] P.I.Hosur and K.K.Ma, “Motion Vector Field Adaptive Fast Motion Estimation”, Second Inter. Conf. On Information Communications and Signal Processing(ICICS’99). Singapore. Dec’99.

[4] Tourapis, A.M.; Au, O.C.; Liou, M.L. “New results on zonal based motion estimation algorithms-advanced predictive diamond zonal search”, Circuits and Systems, 2001. ISCAS 2001. The 2001 IEEE International Symposium on , Volume: 5 , 2001 Page(s): 183 -186 vol. 5

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address | 100084 Dept.E&E of Tsinghua University Beijing, China, P.R |  |  |
| Country | China.P.R |  |  |
| Contact person | Zhibo Chen, Yun He |  |  |
| Telephone | See contribution header |  |  |
| Fax | 86-10-62770317 |  |  |
| Email | chenzb@video.mdc.tsinghua.edu.cn |  |  |
| Place and date of submission | 3rd JVT Meeting, Fairfax VA USA, May 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  | JVT-C034 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| # X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| # X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: