| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C038 Filename: JVT-C038.doc |
| --- | --- |

| Title: | Bounding the complexity of arithmetic decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen DoCoMo Communications Labs USA 181 Metro Drive Suite 300 San Jose CA 95110 USA | Tel: Email: | +1 408 451 4712 bossen@docomolabs-usa.com |
| Source: | DoCoMo Communications Laboratories USA, Inc. |  |  |

_____________________________

# Introduction

In the Joint Working Draft 2.0 arithmetic coding is used to increase coding efficiency. Each syntax element is converted into a sequence of binary events (bins) that are compressed by the binary arithmetic coding engine. Compression is achieved by generating less than one output bit per binary event on average. The more skewed the probability distribution of an event, the less bits will be output by the arithmetic coder, on average. When the probability distribution becomes extremely skewed the arithmetic coder can compress approximately 64 binary events into a single bit (considering the current arithmetic coding engine and a probability estimation varying between 64/65 and 126/127). The downside is that for every bit in a bit stream a high number of arithmetic decoding operations may be required. To verify this hypothesis a test pattern was generated and encoded using the available reference software. The test pattern and the encoding parameters were selected such as to maximize the number of binary events. For a test pattern of CIF size and at 30fps, a 0.911Mbps stream was generated. When decoding the obtained bit stream, the arithmetic decoder clearly became the bottleneck in the decoder. To decode 1 second of video from that stream, approximately 1.87 seconds of processing time were needed by the arithmetic decoding core on a 800MHz Mobile Pentium 3 processor. Clearly real-time decoding is not possible under these conditions. This document addresses the issue of bounding the average number of events that are compressed into a single bit in a worst case scenario such as to always guarantee real-time decoding capability. A modification to the arithmetic coder is proposed such as to limit the maximum average number of events per bit. This modification does not impact the coding efficiency in any significant way but does dramatically (by an order of magnitude) reduce the average number of events per bit in worst case scenarios.

# Worst case analysis

In this section we analyze worst case situations for JWD2. We start by analyzing the case of a single macroblock, then extend it to an entire frame.

We define a context with extreme probability, a context in which the value of each coded event may always be the same (i.e. all ‘0’ or all ‘1’). On average such a context may require no more than 0.015652 bits per event. Note that this number of bits, as all number quoted below, was obtained by processing 100 million events with the arithmetic coding engine and averaging the number of renormalization operations over the number of events.

The table below summarizes the average number of bits per event for different maximum count values (as defined in JWD2).

| Rescale counts on | Bits per event | Events per bit |
| --- | --- | --- |
| 50 | 0.040410 | 24.75 |
| 64 | 0.031483 | 31.76 |
| 100 | 0.020071 | 49.82 |
| 128 | 0.015652 | 63.89 |

## Macroblock level

In this section we construct a scenario that is meant to maximize the number of events per bit in a macroblock. From this case we show how to modify the parameters of the macroblock such as to meet other bit rates.

This scenario is not necessarily the true worst case, but about as bad as it can get. This analysis applies to I-frames only, but similar behavior is expected for P- and B-frames.

Luminance block level:

| Syntax element | # events (50/100) | # events (64/128) | Comment |
| --- | --- | --- | --- |
| IPRED | 8 |  | Always select the least probable (a priori) prediction mode |
| COEFF_COUNT |  | 16 | Always 16 coefficients per block |
| COEFF |  | 16*4 | All coefficients have value -3 (or alternatively +3) |
| Sum | 8 | 80 |  |

Luminance macroblock level:

| Syntax element | # events (50/100) | # events (64/128) | Comment |
| --- | --- | --- | --- |
| MBMODE | 1 |  | Always Intra 4x4 mode |
| DQUANT | 1 |  | Always same QP |
| CBPY | 4 |  | All coefficients have value -3 (or alternatively +3) |
| Blocks | 16*8 | 16*80 |  |
| Sum | 134 | 1280 |  |

Chrominance AC block level:

| Syntax element | # events (50/100) | # events (64/128) | Comment |
| --- | --- | --- | --- |
| COEFF_COUNT |  | 15 | Always 15 coefficients per block |
| COEFF |  | 15*4 | All coefficients have value -3 (or alternatively +3) |
| Sum | 0 | 75 |  |

Chrominance DC block level:

| Syntax element | # events (50/100) | # events (64/128) | Comment |
| --- | --- | --- | --- |
| COEFF_COUNT |  | 4 | Always 4 coefficients per block |
| COEFF |  | 4*4 | All coefficients have value -3 (or alternatively +3) |
| Sum | 0 | 20 |  |

Chrominance macroblock level:

| Syntax element | # events (50/100) | # events (64/128) | Comment |
| --- | --- | --- | --- |
| CBPC | 2 |  | All blocks coded |
| DC blocks |  | 2*20 |  |
| AC blocks |  | 8*75 |  |
| Sum | 2 | 640 |  |

At the macroblock level luminance and chrominance are summed for a total of 136+1920 events and an average bit cost of 65.943120 for QCIF and 32.781496 for CIF.

To obtain figures for higher bit rates a slope of 26.788106 events per bit is used for QCIF and 44.454323 for CIF. Indeed any +/-3 coefficient could be replaced by a +/- 1024 coefficient.

## Frame level

The table below summarizes – for a range of frame sizes, frames rates, and bit rates – the worst case average number of events per bit.

| Frame size | Frame rate | Bit rate | E/B JWD2 |
| --- | --- | --- | --- |
| QCIF | 10 | 64kbps | 31.27 |
| QCIF | 15 | 128kbps | 30.15 |
| CIF | 15 | 384kbps | 53.72 |
| CIF | 30 | 1Mbps | 51.57 |
| SDTV | 30 | 2Mbps | 56.58 |
| SDTV | 30 | 4Mbps | 50.52 |
| HDTV | 30 | 16Mbps | 53.61 |

Coming back to the example described in the introduction, the 0.911Mbps stream is the result of 34.066 million binary events, averaging 37.39 events per bit. This number is not so different from the estimation given above (that did not assume a real image source).

# Constraining the arithmetic coder

In order to constrain the number of events per bit, a counter is used to keep track of the number of events and bits. The counter is initialized to -64 when the arithmetic coder is started. Then the counter is incremented every time a binary event is processed. Whenever renormalization occurs a constant value is subtracted from the counter. The range is scaled only if the counter is negative. If the counter is positive stuffing bits are generated by the arithmetic coder until the counter is negative again. The value by which the counter is decremented determines the maximum number of events per bit. In our experiments we have set this value to 4 (in general the number of events per bit tends to be below 2, so this seems pretty safe in terms of affecting coding efficiency).

Figures 1-4 show flow diagrams of a modified arithmetic decoder.

# Results

Simulations were run using the release 1.7 of the software to detect any coding efficiency penalty resulting from the proposed changes. Results show that there is no significant difference in coding efficiency between JWD2 and this proposal.

## Container QCIF, 300 frames:

Proposal:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 36.19 | 40.36 | 40.08 | 27.22 |
| 20 | 33.61 | 38.97 | 38.38 | 16.15 |
| 24 | 30.94 | 37.25 | 36.69 | 10.78 |
| 28 | 28.13 | 36.80 | 35.60 | 7.33 |

JM1.7:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 36.20 | 40.38 | 40.08 | 27.54 |
| 20 | 33.61 | 38.96 | 38.42 | 15.58 |
| 24 | 30.93 | 37.30 | 36.63 | 9.68 |
| 28 | 28.11 | 36.81 | 35.61 | 5.78 |

## Foreman QCIF, 400 frames

Proposal:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 35.64 | 38.87 | 39.99 | 110.93 |
| 20 | 33.12 | 37.82 | 38.55 | 64.03 |
| 24 | 30.71 | 36.61 | 37.08 | 37.37 |
| 28 | 28.30 | 35.94 | 36.23 | 22.20 |

JM1.7:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 35.63 | 38.85 | 40.01 | 110.72 |
| 20 | 33.12 | 37.86 | 38.57 | 64.24 |
| 24 | 30.75 | 36.65 | 37.13 | 37.33 |
| 28 | 28.29 | 35.92 | 36.34 | 21.79 |

## News QCIF, 300 frames

Proposal:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 36.94 | 39.24 | 39.69 | 68.48 |
| 20 | 33.99 | 37.56 | 38.32 | 42.91 |
| 24 | 31.15 | 36.02 | 36.79 | 25.79 |
| 28 | 28.24 | 34.76 | 35.51 | 15.08 |

JM1.7:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 36.94 | 39.18 | 39.62 | 68.30 |
| 20 | 34.01 | 37.62 | 38.27 | 42.95 |
| 24 | 31.13 | 35.96 | 36.80 | 25.75 |
| 28 | 28.25 | 34.74 | 35.48 | 14.83 |

## Silent QCIF, 300 frames

Proposal:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 36.06 | 38.37 | 39.08 | 76.91 |
| 20 | 33.38 | 36.62 | 37.97 | 46.28 |
| 24 | 30.88 | 35.07 | 36.86 | 26.22 |
| 28 | 28.34 | 34.26 | 36.25 | 14.12 |

JM1.7:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 36.06 | 38.37 | 39.08 | 76.89 |
| 20 | 33.38 | 36.62 | 37.97 | 46.25 |
| 24 | 30.88 | 35.07 | 36.86 | 25.98 |
| 28 | 28.35 | 34.26 | 36.27 | 13.64 |

## Mobile CIF, 300 frames

Proposal:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 34.07 | 35.21 | 35.11 | 1064.83 |
| 20 | 30.96 | 33.33 | 33.19 | 545.21 |
| 24 | 28.05 | 31.42 | 31.18 | 267.22 |
| 28 | 25.19 | 30.30 | 29.95 | 141.55 |

JM1.7:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 34.06 | 35.22 | 35.12 | 1063.97 |
| 20 | 30.97 | 33.32 | 33.19 | 544.37 |
| 24 | 28.05 | 31.41 | 31.16 | 267.89 |
| 28 | 25.19 | 30.30 | 29.95 | 141.27 |

## Paris CIF, 1000 frames

Proposal:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 35.65 | 38.34 | 38.43 | 343.68 |
| 20 | 32.69 | 36.48 | 36.81 | 198.36 |
| 24 | 29.81 | 34.81 | 34.96 | 111.35 |
| 28 | 26.87 | 33.30 | 33.70 | 63.63 |

JM1.7:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 35.65 | 38.35 | 38.45 | 343.56 |
| 20 | 32.69 | 36.48 | 36.81 | 198.20 |
| 24 | 29.81 | 34.81 | 34.96 | 110.84 |
| 28 | 26.87 | 33.30 | 33.70 | 63.16 |

## Tempete CIF, 260 frames

Proposal:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 34.99 | 36.65 | 38.62 | 905.44 |
| 20 | 32.20 | 35.28 | 37.38 | 448.82 |
| 24 | 29.60 | 33.75 | 36.03 | 222.12 |
| 28 | 27.03 | 32.80 | 35.19 | 115.63 |

JM1.7:

| QP | SNR Y | SNR U | SNR V | kb/s |
| --- | --- | --- | --- | --- |
| 16 | 35.00 | 36.67 | 38.61 | 904.85 |
| 20 | 32.20 | 35.28 | 37.37 | 449.24 |
| 24 | 29.61 | 33.77 | 36.05 | 221.86 |
| 28 | 27.01 | 32.81 | 35.18 | 114.94 |

# Conclusion

We recommend the adoption of this proposal such as to reduce the number of arithmetic decoding operations in worst case scenarios and hence implementation complexity. In these worst case scenarios, implementation complexity remains significant, and could be further reduced by considering simpler arithmetic coding engines.

It is noted that in some cases performance is degraded by the implementation of this proposal. This however is due to poor binarization of data. Better binarization would alleviate this problem. One prime example is coding of the direct mode in B-frames. JM1.7 always codes CBP information for these macroblocks and does not seem to support the skipped mode. This has signification impact since at very low bit rates 6 events per bit are coded instead of just one.

Figure 1: Start arithmetic decode

Figure 2: Decode symbol

- Figure 3: Renormalize

- Figure 4: Update counts

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | DoCoMo Communications Laboratories USA, Inc. |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  |  |  |
| Contribution number |  | JVT-B038 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: