| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C040 Filename: JVT-C040.doc |
| --- | --- |

| Title: | Adaptive motion interpolation on MB basis |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei-ichi Chono and Yoshihiro Miyamoto NEC Multimedia Laboratories, 4-1-1 Miyazaki Miyamae Kawasaki, 216-8555 JAPAN | Tel: Email: | +81 44 856 8141 chono@dsp.cl.nec.co.jp miyamoto@dsp.cl.nec.co.jp |
| Source: | NEC |  |  |

_____________________________

1. Overview

Many experts have shown an interest in the adaptive interpolation method, estimating　and transmitting 6-tap filter-coefficients per picture [1][2]. To consider non-stationary statistical properties of video signals, this adaptive interpolation scheme seems to be beneficial.

However, the adaptive interpolation method requires a large amount of computation: estimating the filter-coefficients at encoder, 32-bit accuracy interpolation at decoder [1].

In this contribution, MB basis Adaptive Interpolation Filter(MBAIF) and its coding efficiency are presented. MBAIF is a given set selection in pre-calculated filter-coefficients with 16-bit accuracy.  The additional complexity, by switching the filter-coefficients in MBAIF, is low enough compared to JM14 decoder’s.

2. MB basis Adaptive Interpolation Filter (MBAIF)

In this section, the filter-selection in pre-calculated filter-coefficients, the transmission of the filter-coefficients and the change of determining the boundary strength (BS) in loop-filter are shown.

2.1 Filter-selection in pre-calculated filter-coefficients

JM14 interpolates fractional-pixels by two successive interpolations with two different interpolation filters. First interpolation filter is a symmetric 6-tap non-adaptive filter for producing 2:1 resolution. Second one is a symmetric non-adaptive bilinear-filter for producing 4:1 resolution (Fig 1).

Fig 1:	Interpolation process for 1/4-pixcel resolution

In MBAIF, non-symmetric 6-tap interpolation filters are used for Filter 1. Filter 2 is the exactly same as the filter used in JM14.  Non-symmetric 6-tap interpolation filters are shown in Table 1. NSAIF(i) denotes a non-symmetric filter that is adapted on MB.

Table 1: Filter 1 of the JM14 and MBAIF.  NSAIF(i) denotes a non-symmetric filter that is adapted on MB basis

In our experiments, the following steps are used to select the best NSAIF and its vector for current coding MB. However, it is an encoder issue to find the best NSAIF and its vector set.

- Best full-pixel vectors are estimated.

- Then, prediction errors and fractional pixel vectors are calculated in each NSAIF.

- NSAIF(i) and its vector, with minimum Cost described in (1), are used for current MB prediction.

Costi=k,l16,16|o(k,l)−ri(k−vxi, l−vyi)|+∂×(FF(i)+VF(vxi−px,vyi−py))     (1)

, where i is a number corresponding to the coefficients NSAIF(i), o(k,l) is original video signal,  ri(k,l) is reconstructed video signal with filter-coefficients i, (vxi,vyi) is vector set with filter number i, (px,py) is the predicted vector, FF(i) is a function represents the number of the bits of filter i, VF(x,y) is a function represents the number of the bits of vector set (x,y) and ∂ is a parameter weighting the number of bits.

The vector prediction does not consider NSAIF’s difference because the calculated vector difference between NSAIF(i) and NSAIF(j) is less than Integer. Therefore, MBAIF do not need a change for vector prediction.

The best i is transmitted as described in next sub-section.

2.2 Transmission of the filter-coefficients

We think that the coding efficiency of Adaptive Interpolation Filter(AIF) depends on the property of video signal. Therefore, the usage of MBAIF is signaled in PTYPE.  The additional PTYPE 7 and 8 are shown:

# PTYPE

Code_number =7:	AIF picture with prediction from the most recent decoded picture only. For this mode information of selected interpolation filter_number must be signalled for each macroblock

Code_number =8:	AIF picture with possibility of prediction from more than one previous decoded picture.  For this mode information reference picture for prediction and information of selected interpolation filter_number must be signaled for each macroblock.

When AIF is signaled in PTYPE, filter_number is also signaled in each MB to interpolate fractional pixels with suitable filter-coefficients at decoder. The coding rule for filter_number is same as the ref_frame’s. Code_number corresponding to NSAIF(i) is transmitted as shown in Fig2.

# Code to NSAIF filter_number

Code		Interpolation filter-coefficients

0		NSAIF(1)

1		NSAIF(2)

2		NSAIF(3)

3		NSAIF(4)

4		NSAIF(5)

Fig2: MB syntax diagram for proposed method

In our experiment, filter-coefficients NSAIF(i) has fixed value between encoder and decoder as shown in Table 1. However, it is possible to use filter-coefficients with unfixed value when filter-coefficients values are also signaled per picture or GOP.

2.3 Change of determining Boundary Strength(BS) in loop-filter

The determination of loop-filter's BS is changed by MBAIF because the NSAIF difference over block boundary affects the prediction error’s continuity. The change is not complicated as described in Fig 3.

Fig 3: Modified flow chart for determining the boundary strength (Bs), for the block boundary between two neighboring blocks p and q, where Fil(p) means the interpolation filter of p, Ref(p) means the reference frame of p and V(p) means motion vector of p.

3.Experimental result

3.1 Test sequences and coding conditions

Test sequences are shown in Table 1.

Table 1: Test sequence

The coding conditions are: 16 pixel search range, RestrictSearchRange set to two, Use Hadamard set to on, quarter pixel-motion, UVLC in entropy coding, RD optimization set to on, no B-picture. The number of multiple reference frame set to 1 and/or 5.

And JM14 is improved to support the loop filter described in WD2.0.

  - Results

MBAIF’s coding gain compared to JM14 is following:

1 reference:

- MBAIF leads to average gain 0.22[dB], and up to 0.46[dB], at QP=8-14

- MBAIF leads to average gain 0.06[dB], and up to 0.33[dB], at QP=16-24

5reference:

- MBAIF leads to average gain 0.24[dB], and up to 0.53[dB], at QP=8-14

- MBAIF leads to average gain 0.04[dB], and up to 0.27[dB], at QP=16-24

The avsnr4-results, rate-distortion curves and decoding time comparison are attached.

4. Conclusion

Experimental results show three facts: First fact is that MBAIF’s gain leads up to 0.53[dB] at high bit-rate, second one is that MBAIF with 1 reference leads to a gain compare to JM14 with 5 reference in Container and News, Third one is that the complexity of the interpolation is not significantly increased and half of 1/8 pixel MC’s.

Considering these results, we propose:

- Adoption of the MBAIF coding in JM and syntax changes to obtain further more coding gain

- Syntax changes described in 2.2 to obtain further more coding gain with low complexity increase

Reference

[1]Tom-Ivar Johansen, ” AHG Report: Adaptive Motion Interpolation”, ISO/IEC JTC1/SC29/WG11 and ITU-T , JVT-B018.doc, Geneva, CH, Jan. 29 - Feb. 1, 2002

[2]Thomas Wedi, “More Results on Adaptive Interpolation Filter for H.26L”, ITU-T SG16, doc. VCEG-O16, Pattaya, Thailand, 4-6 Dec., 2001

# Avsnr4-results

1-reference with QP=8-14

5-reference with QP=8-14

1-reference with QP=16-28

5-reference with QP=16-28

Rate-distrotion curves

## Complexity comparison

Evaluation condition is: Pentium4 1.7GHz, 512MB memory, Microsoft Windows2000,

and Intel VTune5 with 1msec sampling rate.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | NEC |  |  |
| Mailing address | 4-1-1 Miyazaki Miyamae-ku Kawasaki 216-8555 |  |  |
| Country | JAPAN |  |  |
| Contact person | Yoshihiro Miyamoto |  |  |
| Telephone | +81 44 855 8141 |  |  |
| Fax | +81 44 855 2232 |  |  |
| Email | miyamoto@dsp.cl.nec.co.jp |  |  |
| Place and date of submission | 3rd JVT meeting May 6 – 11, Fairfax |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Adaptive motion interpolation on MB basis |  |
| Contribution number |  | JVT-C040 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: