| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C042 Filename: JVT-C042.doc |
| --- | --- |

| Title: | Core Experiment Results on Interlace Chroma Phase Shift |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshinori Suzuki 1-280, Higashi-koigakubo Kokubunji-shi, Tokyo 185-8601, Japan | Tel: Email: | +81-42-323-1111 yosinori@crl.hitahit.co.jp |
| Source: | Hitachi, Ltd. |  |  |

_____________________________

Summary

This document provides a report on the results of core experiment for interlace chroma phase shift. The proposed tool provides improvements up to 10%, while the JM 1.7 with proposed tool shows better rate-distortion performance than the original JM 1.7 for all the test cases.

1. Introduction

This document provides a report on the results of core experiment for interlace chroma phase shift.

2. Simulation Conditions

The experiment was performed based on the description of core experiment [1]. The test tool was integrated in the JM 1.7 software. The test sequences and test conditions are listed in Table 1 and Table 2 respectively.

Table 1. Test Sequences

|  | Group 1 | Group 2 |
| --- | --- | --- |
| Sequences | Popple, 720x480 Src6, 720x576 | Src9, 720x576 Mobile, 720x480 Flower, 720x480 |

Table 2. Test Condition

| MV resolution | 1/4 pel |
| --- | --- |
| Hadamard | ON |
| RD optimization | ON |
| Search Range | 16 |
| Restrict Search Range | 2 |
| Reference Frames | 1 or 5 |
| Symbol Mode | UVLC or CABAC |
| GOP structure | IPP or IBBP |
| QP | QPI=QPP=16,20,24,28 QPB= QPP +2 |

3. Results

The tables 3-10 show the average PSNR gains and bitrate savings as calculated using the Bjontegaard measurement method [3]. And rate-distortion curves for test cases are provided in the excel sheet. Furthermore, The statistical data, averaged PSNR and coding bitrate, for each bitstream are shown in the tables 11-18.

4. Conclusions

The following results were obtained by this experiment:

 	The averaged bit-rate savings of 8.46% - 10.39% are obtained for “Src6”.

 	The averaged bit-rate savings of 2.74% - 5.00% are obtained for “Popple”.

 	The JM 1.7 with proposed tool shows better rate-distortion performance than the original JM 1.7 for all the test cases.

# Reference

[1] “Core Experiment on Interlace Chroma Phase Shift”, JVT-B116, February 2002.

[2] G. Bjontegaard, “Calculation of average PSNR differences between RD-curves”, Doc. VCEG-M33, ITU-T Q6/16, Austin, TX, USA, 2-4 April 2001.

Table 3: Results of configuration with P-frames only, UVLC, 1 reference frame

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -3.12 | 0.11 | -3.75 | 0.13 | -4.13 | 0.15 |
| Src6 | -8.46 | 0.36 | -14.06 | 0.40 | -15.34 | 0.36 |
| Src9 | -0.40 | 0.02 | -0.30 | 0.00 | -1.15 | 0.02 |
| Mobile | -0.23 | 0.01 | -0.64 | 0.01 | -1.12 | 0.02 |
| Flower | -0.43 | 0.02 | -0.52 | 0.01 | -0.69 | 0.01 |

Table 4: Results of configuration with P-frames only, UVLC, 5 reference frame

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -4.62 | 0.16 | -5.76 | 0.21 | -5.89 | 0.20 |
| Src6 | -9.97 | 0.44 | -17.31 | 0.52 | -18.48 | 0.46 |
| Src9 | -0.37 | 0.02 | -0.94 | 0.02 | -1.60 | 0.04 |
| Mobile | -0.74 | 0.03 | -1.19 | 0.02 | -2.96 | 0.06 |
| Flower | -0.16 | 0.01 | -1.07 | 0.03 | -0.26 | 0.00 |

Table 5: Results of configuration with 2 B-frames, UVLC, 1 reference frame

(a) Results for the whole sequence

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -2.92 | 0.10 | -4.36 | 0.15 | -4.03 | 0.13 |
| Src6 | -9.84 | 0.40 | -24.03 | 0.70 | -22.03 | 0.54 |
| Src9 | -0.31 | 0.01 | -1.22 | 0.02 | -2.22 | 0.05 |
| Mobile | -0.32 | 0.01 | -0.27 | 0.00 | -2.04 | 0.03 |
| Flower | -0.63 | 0.03 | -1.43 | 0.03 | -1.72 | 0.03 |

(b) Results for P-frames

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -2.95 | 0.11 | -3.98 | 0.15 | -3.27 | 0.12 |
| Src6 | -6.83 | 0.32 | -17.47 | 0.55 | -17.10 | 0.45 |
| Src9 | 0.04 | 0.00 | -0.47 | 0.01 | -0.19 | 0.01 |
| Mobile | -0.24 | 0.01 | -0.10 | 0.00 | -2.44 | 0.05 |
| Flower | -0.48 | 0.02 | -1.01 | 0.02 | -1.53 | 0.03 |

(c) Results for B-frames

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | Average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -2.72 | 0.08 | -4.72 | 0.15 | -3.27 | 0.12 |
| Src6 | -15.65 | 0.54 | -32.77 | 0.85 | -30.48 | 0.65 |
| Src9 | -0.66 | 0.02 | -2.44 | 0.04 | -4.35 | 0.08 |
| Mobile | -0.12 | 0.00 | -0.41 | 0.00 | -3.06 | 0.04 |
| Flower | -0.64 | 0.02 | -1.96 | 0.03 | -2.65 | 0.03 |

Table 6: Results of configuration with 2 B-frames, UVLC, 5 reference frame

(a) Results for the whole sequence

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -2.74 | 0.09 | -5.27 | 0.18 | -4.96 | 0.16 |
| Src6 | -10.39 | 0.44 | -24.44 | 0.74 | -22.30 | 0.56 |
| Src9 | -0.19 | 0.01 | -0.93 | 0.02 | -2.28 | 0.05 |
| Mobile | -0.50 | 0.02 | -2.88 | 0.05 | -4.56 | 0.08 |
| Flower | -0.89 | 0.04 | -3.48 | 0.08 | -2.17 | 0.03 |

(b) Results for P-frames

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -3.01 | 0.11 | -5.31 | 0.20 | -5.02 | 0.19 |
| Src6 | -6.88 | 0.33 | -16.90 | 0.55 | -16.28 | 0.44 |
| Src9 | 0.16 | -0.01 | -0.51 | 0.01 | -0.07 | 0.00 |
| Mobile | -0.62 | 0.03 | -2.96 | 0.06 | -4.07 | 0.09 |
| Flower | -1.00 | 0.05 | -3.49 | 0.10 | -1.98 | 0.03 |

(c) Results for B-frames

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | Average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -2.30 | 0.07 | -5.24 | 0.16 | -5.21 | 0.16 |
| Src6 | -17.04 | 0.61 | -34.61 | 0.93 | -31.50 | 0.70 |
| Src9 | -0.63 | 0.02 | -1.90 | 0.03 | -4.34 | 0.08 |
| Mobile | -0.34 | 0.01 | -3.76 | 0.05 | -5.49 | 0.07 |
| Flower | -0.43 | 0.01 | -4.10 | 0.08 | -2.27 | 0.03 |

Table 7: Results of configuration with P-frames only, CABAC, 1 reference frame

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -3.02 | 0.10 | -4.16 | 0.15 | -3.94 | 0.14 |
| Src6 | -8.63 | 0.36 | -13.72 | 0.38 | -16.23 | 0.37 |
| Src9 | -0.36 | 0.02 | -0.40 | 0.01 | -1.14 | 0.03 |
| Mobile | -0.16 | 0.00 | -0.66 | 0.01 | -0.80 | 0.02 |
| Flower | -0.25 | 0.01 | -0.47 | 0.00 | -0.46 | 0.00 |

Table 8: Results of configuration with P-frames only, CABAC, 5 reference frame

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -5.00 | 0.16 | -6.31 | 0.23 | -6.04 | 0.21 |
| Src6 | -10.29 | 0.44 | -17.64 | 0.51 | -19.42 | 0.47 |
| Src9 | -0.34 | 0.02 | -1.02 | 0.02 | -1.22 | 0.03 |
| Mobile | -0.47 | 0.02 | -1.50 | 0.03 | -2.59 | 0.05 |
| Flower | -0.25 | 0.01 | -1.11 | 0.03 | -1.41 | 0.02 |

Table 9: Results of configuration with 2 B-frames, CABAC, 1 reference frame

(a) Results for the whole sequence

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -2.90 | 0.09 | -4.29 | 0.14 | -4.26 | 0.13 |
| Src6 | -9.24 | 0.37 | -23.09 | 0.67 | -21.44 | 0.51 |
| Src9 | -0.26 | 0.01 | -0.54 | 0.01 | -2.44 | 0.05 |
| Mobile | -0.30 | 0.01 | -1.18 | 0.01 | -2.74 | 0.04 |
| Flower | -0.61 | 0.03 | -1.48 | 0.03 | -1.05 | 0.01 |

(b) Results for P-frames

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -4.74 | 0.19 | -5.55 | 0.23 | -5.03 | 0.21 |
| Src6 | -5.65 | 0.26 | -16.14 | 0.51 | -15.23 | 0.40 |
| Src9 | -0.35 | 0.02 | -0.10 | 0.01 | -0.68 | 0.02 |
| Mobile | -0.37 | 0.01 | -0.95 | 0.01 | -2.67 | 0.05 |
| Flower | -0.62 | 0.03 | -1.29 | 0.03 | -0.53 | 0.00 |

(c) Results for B-frames

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | Average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -2.82 | 0.08 | -4.67 | 0.14 | -4.90 | 0.14 |
| Src6 | -14.65 | 0.46 | -31.52 | 0.77 | -30.08 | 0.61 |
| Src9 | -0.88 | 0.03 | -1.16 | 0.02 | -4.63 | 0.09 |
| Mobile | -0.35 | 0.01 | -0.99 | 0.01 | -3.10 | 0.04 |
| Flower | -0.21 | 0.01 | -1.87 | 0.03 | -1.52 | 0.02 |

Table 10: Results of configuration with 2 B-frames, CABAC, 5 reference frame

(a) Results for the whole sequence

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -3.27 | 0.10 | -5.44 | 0.18 | -4.63 | 0.15 |
| Src6 | -9.84 | 0.40 | -23.43 | 0.69 | -21.01 | 0.51 |
| Src9 | -0.43 | 0.02 | -0.28 | 0.01 | -2.67 | 0.06 |
| Mobile | -0.75 | 0.03 | -3.34 | 0.06 | -4.11 | 0.08 |
| Flower | -1.09 | 0.05 | -2.90 | 0.66 | -2.33 | 0.03 |

(b) Results for P-frames

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -3.15 | 0.11 | -5.37 | 0.20 | -4.19 | 0.15 |
| Src6 | -5.65 | 0.27 | -15.32 | 0.50 | -14.07 | 0.38 |
| Src9 | -0.05 | 0.00 | -0.13 | 0.01 | -0.09 | 0.01 |
| Mobile | -0.75 | 0.03 | -3.14 | 0.06 | -3.66 | 0.08 |
| Flower | -1.21 | 0.06 | -3.09 | 0.08 | -2.27 | 0.03 |

(c) Results for B-frames

| Test Sequences | Y |  | U |  | V |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | average bit savings [%] | average PSNR gain [dB] | Average bit savings [%] | average dB PSNR gain [dB] | average bit savings [%] | average dB PSNR gain [dB] |
| Popple | -3.48 | 0.10 | -5.61 | 0.17 | -5.20 | 0.15 |
| Src6 | -18.39 | 0.61 | -35.49 | 0.91 | -32.01 | 0.67 |
| Src9 | -0.91 | 0.03 | -1.09 | 0.02 | -5.19 | 0.10 |
| Mobile | -0.70 | 0.02 | -4.08 | 0.05 | -5.28 | 0.08 |
| Flower | -0.74 | 0.02 | -4.12 | 0.07 | -2.49 | 0.03 |

Table 11: P frames only, UVLC, Reference frame 1

| Sequence |  | JM1.7 |  | Chroma Phase Shift |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Kbsp | dB (Y, U, V) | kbsp | dB (Y, U, V) |
| Popple | 28 | 1452.83 (P) | 28.23, 28.93, 29.03 (P) | 1442.18 (P) | 28.24, 28.95, 29.04 (P) |
|  | 24 | 2987.69 (P) | 30.36, 30.62, 30.68 (P) | 2912.41 (P) | 30.38, 30.64, 30.73 (P) |
|  | 20 | 5813.28 (P) | 32.49, 33.32, 33.29 (P) | 5611.23 (P) | 32.49, 33.37, 33.34 (P) |
|  | 16 | 9787.10 (P) | 34.59, 35.72, 35.51 (P) | 9432.12 (P) | 34.59, 35.66, 35.56 (P) |
| Src6 | 28 | 652.16 (P) | 28.39, 33.52, 35.71 (P) | 598.33 (P) | 28.42, 33.82, 35.82 (P) |
|  | 24 | 1286.70 (P) | 31.00, 34.70, 36.75 (P) | 1184.13 (P) | 31.02, 34.92, 36.89 (P) |
|  | 20 | 2499.58 (P) | 33.59, 36.80, 38.26 (P) | 2300.58 (P) | 33.61, 36.94, 38.47 (P) |
|  | 16 | 4614.83 (P) | 36.32, 38.58, 39.77 (P) | 4248.14 (P) | 36.33, 38.64, 40.06 (P) |
| Src9 | 28 | 1434.94 (P) | 27.35, 35.81, 36.05 (P) | 1430.45 (P) | 27.34, 35.83, 36.07 (P) |
|  | 24 | 2744.22 (P) | 29.96, 36.55, 37.09 (P) | 2734.37 (P) | 29.96, 36.54, 37.09 (P) |
|  | 20 | 4728.09 (P) | 32.59, 37.73, 38.68 (P) | 4695.86 (P) | 32.58, 37.72, 38.70 (P) |
|  | 16 | 7769.39 (P) | 35.27, 38.87, 39.92 (P) | 7722.98 (P) | 35.27, 38.86, 39.96 (P) |
| Mobile | 28 | 989.17 (P) | 25.11, 32.08, 31.64 (P) | 985.24 (P) | 25.11, 32.09, 31.64 (P) |
|  | 24 | 2618.33 (P) | 28.07, 32.97, 32.67 (P) | 2617.83 (P) | 28.07, 32.97, 32.69 (P) |
|  | 20 | 5288.26 (P) | 31.12, 34.45, 34.29 (P) | 5283.34 (P) | 31.13, 34.46, 34.33 (P) |
|  | 16 | 9213.07 (P) | 34.18, 35.94, 35.98 (P) | 9181.85 (P) | 34.19, 35.97, 35.99 (P) |
| Flower CIF 15Hz | 28 | 954.51 (P) | 25.48, 31.58, 33.16 (P) | 950.22 (P) | 25.48, 31.59, 33.17 (P) |
|  | 24 | 2026.86 (P) | 28.43, 32.62, 33.73 (P) | 2020.65 (P) | 28.43, 32.62, 33.73 (P) |
|  | 20 | 3808.69 (P) | 31.48, 34.43, 34.91 (P) | 3794.14 (P) | 31.48, 34.42, 34.90 (P) |
|  | 16 | 6585.47 (P) | 34.59, 36.24, 36.22 (P) | 6525.30 (P) | 34.59, 36.26, 36.24 (P) |

Table 12: P frames only, UVLC, Reference frame 5

| Sequence |  | JM1.7 |  | Chroma Phase Shift |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Kbsp | dB (Y, U, V) | kbsp | dB (Y, U, V) |
| Popple | 28 | 1287.85 (P) | 28.33, 29.03, 29.14 (P) | 1268.08 (P) | 28.33, 29.03, 29.19 (P) |
|  | 24 | 2513.31 (P) | 30.37, 30.65, 30.70 (P) | 2430.44 (P) | 30.38, 30.68, 30.74 (P) |
|  | 20 | 4859.86 (P) | 32.44, 33.24, 33.22 (P) | 4577.86 (P) | 32.45, 33.30, 33.26 (P) |
|  | 16 | 8432.18 (P) | 34.54, 35.57, 35.44 (P) | 7933.06 (P) | 34.54, 35.66, 35.51 (P) |
| Src6 | 28 | 644.79 (P) | 28.51, 33.53, 35.73 (P) | 588.78 (P) | 28.51, 33.86, 35.83 (P) |
|  | 24 | 1230.90 (P) | 31.10, 34.69, 36.78 (P) | 1118.92 (P) | 31.12, 35.01, 36.96 (P) |
|  | 20 | 2353.70 (P) | 33.68, 36.81, 38.28 (P) | 2114.05 (P) | 33.70, 37.00, 38.57 (P) |
|  | 16 | 4306.34 (P) | 36.39, 38.62, 39.79 (P) | 3863.24 (P) | 36.40, 38.68, 40.14 (P) |
| Src9 | 28 | 1428.40 (P) | 27.43, 35.85, 36.08 (P) | 1425.08 (P) | 27.42, 35.85, 36.12 (P) |
|  | 24 | 2695.38 (P) | 30.04, 36.61, 37.09 (P) | 2686.92 (P) | 30.04, 36.62, 37.14 (P) |
|  | 20 | 4604.91 (P) | 32.67, 37.81, 38.74 (P) | 4573.95 (P) | 32.66, 37.82, 38.74 (P) |
|  | 16 | 7518.62 (P) | 35.34, 38.96, 39.99 (P) | 7474.71 (P) | 35.34, 38.97, 39.99 (P) |
| Mobile | 28 | 800.08 (P) | 25.48, 32.28, 31.91 (P) | 795.68 (P) | 25.47, 32.29, 31.91 (P) |
|  | 24 | 1685.38 (P) | 28.32, 33.26, 33.00 (P) | 1675.58 (P) | 28.32, 32.26, 33.04 (P) |
|  | 20 | 3447.90 (P) | 31.30, 34.72, 34.58 (P) | 3425.59 (P) | 31.30, 34.73, 34.65 (P) |
|  | 16 | 6449.66 (P) | 34.34, 36.15, 36.21 (P) | 6355.70 (P) | 34.35, 36.18, 36.28 (P) |
| Flower CIF 15Hz | 28 | 953.45 (P) | 25.57, 31.63, 33.16 (P) | 953.61 (P) | 25.56, 31.64, 33.17 (P) |
|  | 24 | 1948.12 (P) | 28.53, 32.66, 33.77 (P) | 1948.09 (P) | 28.54, 32.67, 33.76 (P) |
|  | 20 | 3592.76 (P) | 31.56, 34.45, 34.96 (P) | 3580.22 (P) | 31.56, 34.49, 34.96 (P) |
|  | 16 | 6198.54 (P) | 34.63, 36.27, 36.25 (P) | 6143.65 (P) | 34.63, 36.30, 36.26 (P) |

Table 13: 2-B frames, UVLC, Reference frame 1

| Sequence |  | JM1.7 |  | Chroma Phase Shift |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Kbsp | dB (Y, U, V) | kbsp | dB (Y, U, V) |
| Popple | 28 | 599.01 (P) 564.87 (B) | 28.16, 29.32, 29.44 (P) 26.96, 28.34, 28.52 (B) | 593.94 (P) 560.69 (B) | 28.16, 29.38, 29.48 (P) 26.97, 28.37, 28.54 (B) |
|  | 24 | 1131.26 (P) 1038.58 (B) | 30.32, 30.86, 30.83 (P) 28.95, 29.36, 29.49 (B) | 1099.95 (P) 1024.44 (B) | 30.34, 30.93, 30.87 (P) 28.97, 29,45, 29.55 (B) |
|  | 20 | 2066.17 (P) 2373.50 (B) | 32.43, 33.40, 33.32 (P) 30.98, 32.01, 32.01 (B) | 2006.59 (P) 2310.47 (B) | 32.44, 33.42, 33.30 (P) 31.00, 32.08, 32.08 (B) |
|  | 16 | 3449.66 (P) 4569.10 (B) | 34.56, 35.70, 35.53 (P) 33.19, 34.58, 34.48 (B) | 3334.99 (P) 4401.34 (B) | 34.56, 35.74, 35.56 (P) 33.20, 34.67, 34.57 (B) |
| Src6 | 28 | 342.50 (P) 150.00 (B) | 28.33, 33.56, 35.65 (P) 27.53, 33.30, 35.56 (B) | 323.89 (P) 123.36 (B) | 28.35, 33.97, 35.79 (P) 27.57, 33.87, 35.73 (B) |
|  | 24 | 638.02 (P) 299.22 (B) | 30.98, 34.65, 36.65 (P) 30.05, 34.36, 36.52 (B) | 598.02 (P) 248.44 (B) | 30.97, 35.08, 36.87 (P) 30.06, 34.97, 36.79 (B) |
|  | 20 | 1187.11 (P) 659.79 (B) | 33.59, 36.78, 38.24 (P) 32.50, 36.44, 38.04 (B) | 1094.75 (P) 563.81 (B) | 33.59, 37.07, 38.58 (P) 32.50, 36.90, 38.46 (B) |
|  | 16 | 2072.78 (P) 1499.62 (B) | 36.32, 38.56, 39.71 (P) 35.09, 38.18, 39.42 (B) | 1929.18 (P) 1313.05 (B) | 36.33, 38.69, 40.14 (P) 35.10, 38.44, 39.98 (B) |
| Src9 | 28 | 782.77 (P) 422.39 (B) | 27.54, 35.66, 35.85 (P) 26.02, 35.71, 35.80 (B) | 785.77 (P) 419.22 (B) | 27.54, 35.65, 35.86 (P) 26.02, 35.70, 35.86 (B) |
|  | 24 | 1405.82 (P) 920.72 (B) | 30.12, 36.32, 36.88 (P) 28.52, 36.39, 36.69 (B) | 1405.33 (P) 918.28 (B) | 30.12, 36.34, 36.89 (P) 28.52, 36.42, 36.77 (B) |
|  | 20 | 2244.18 (P) 1838.86 (B) | 32.69, 37.48, 38.54 (P) 31.14, 37.52, 38.19 (B) | 2244.93 (P) 1822.07 (B) | 32.69, 37.50, 38.54 (P) 31.14, 37.55, 38.26 (B) |
|  | 16 | 3459.95 (P) 3388.55 (B) | 35.31, 38.68, 39.75 (P) 33.83, 38.61, 39.51 (B) | 3458.85 (P) 3350.96 (B) | 35.31, 38.68, 39.75 (P) 33.82, 38.64, 39.54 (B) |
| Mobile | 28 | 378.88 (P) 121.66 (B) | 25.28, 32.37, 31.99 (P) 24.58, 32.33, 31.96 (B) | 378.49 (P) 122.38 (B) | 25.28, 32.38, 32.00 (P) 24.58, 32.33, 31.96 (B) |
|  | 24 | 906.92 (P) 441.30 (B) | 28.16, 33.25, 32.94 (P) 27.00, 33.18, 32.86 (B) | 906.16 (P) 440.78 (B) | 28.16, 33.23, 32.97 (P) 26.18, 33.18, 32.89 (B) |
|  | 20 | 1811.58 (P) 1296.63 (B) | 31.18, 34.68, 34.52 (P) 29.67, 34.55, 34.34 (B) | 1807.73 (P) 1294.86 (B) | 31.18, 34.68, 34.60 (P) 29.67, 34.56, 34.39 (B) |
|  | 16 | 3176.35 (P) 3114.00 (B) | 34.21, 36.10, 36.15 (P) 32.64, 35.91, 35.81 (B) | 3145.74 (P) 3092.30 (B) | 34.21, 36.12, 36.21 (P) 32.64, 35.91, 35.89 (B) |
| Flower CIF 15Hz | 28 | 581.50 (P) 143.01 (B) | 25.48, 31.51, 33.01 (P) 24.96, 31.59, 33.10 (B) | 579.06 (P) 142.24 (B) | 25.48, 31.54, 33.03 (P) 24.95, 31.61, 33.11 (B) |
|  | 24 | 1191.59 (P) 338.59 (B) | 28.41, 32.58, 33.59 (P) 27.67, 32.64, 33.68 (B) | 1181.78 (P) 337.68 (B) | 28.39, 32.57, 33.59 (P) 27.68, 32.63, 33.68 (B) |
|  | 20 | 2151.00 (P) 773.72 (B) | 31.38, 34.44, 34.83 (P) 30.46, 34.38, 34.89 (B) | 2140.89 (P) 769.02 (B) | 31.38, 34.45, 34.85 (P) 30.46, 34.42, 34.93 (B) |
|  | 16 | 3578.46 (P) 1744.84 (B) | 34.43, 36.25, 36.17 (P) 33.35, 36.07, 36.16 (B) | 3549.66 (P) 1726.14 (B) | 34.43, 36.29, 36.21 (P) 33.35, 36.15, 36.23 (B) |

Table 14: 2-B frames, UVLC, Reference frame 5

| Sequence |  | JM1.7 |  | Chroma Phase Shift |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Kbsp | dB (Y, U, V) | kbsp | dB (Y, U, V) |
| Popple | 28 | 562.26 (P) 564.64 (B) | 28.24, 29.35, 29.44 (P) 27.02, 28.34, 28.52 (B) | 551.99 (P) 560.93 (B) | 28.24, 29.41, 29.45 (P) 27.02, 28.39, 28.53 (B) |
|  | 24 | 1060.90 (P) 1030.42 (B) | 30.38, 30.84, 30.82 (P) 29.00, 29.38, 29.48 (B) | 1027.79 (P) 1016.04 (B) | 30.33, 30.92, 30.89 (P) 28.98, 29,46, 29.57 (B) |
|  | 20 | 1971.16 (P) 2323.47 (B) | 32.44, 33.38, 33.26 (P) 31.01, 31.99, 31.98 (B) | 1882.90 (P) 2248.66 (B) | 32.44, 33.42, 33.30 (P) 31.02, 32.08, 32.06 (B) |
|  | 16 | 3299.95 (P) 4458.78 (B) | 34.54, 35.69, 35.49 (P) 33.19, 34.56, 34.45 (B) | 3178.11 (P) 4280.00 (B) | 34.55, 35.74, 35.53 (P) 33.21, 34.65, 34.55 (B) |
| Src6 | 28 | 363.25 (P) 163.39 (B) | 28.37, 33.56, 35.65 (P) 27.52, 33.28, 35.55 (B) | 345.18 (P) 131.01 (B) | 28.38, 33.85, 35.78 (P) 27.54, 33.77, 35.72 (B) |
|  | 24 | 641.66 (P) 311.94 (B) | 31.02, 34.67, 36.72 (P) 30.05, 34.37, 36.56 (B) | 599.83 (P) 256.30 (B) | 31.02, 35.13, 36.89 (P) 30.07, 35.02, 36.81 (B) |
|  | 20 | 1176.05 (P) 679.08 (B) | 33.62, 36.79, 38.27 (P) 32.50, 36.40, 38.04 (B) | 1085.60 (P) 571.06 (B) | 33.62, 37.08, 38.63 (P) 32.50, 36.92, 38.49 (B) |
|  | 16 | 2048.74 (P) 1514.95 (B) | 36.38, 38.60, 39.75 (P) 35.10, 38.17, 39.42 (B) | 1909.59 (P) 1314.46 (B) | 36.35, 38.71, 40.14 (P) 35.11, 38.44, 39.98 (B) |
| Src9 | 28 | 762.70 (P) 433.74 (B) | 27.57, 35.71, 35.90 (P) 26.05, 35.75, 35.87 (B) | 766.15 (P) 430.45 (B) | 27.58, 35.72, 35.90 (P) 26.05, 35.76, 35.91 (B) |
|  | 24 | 1244.42 (P) 930.69 (B) | 30.15, 36.38, 36.94 (P) 28.55, 36.45, 36.74 (B) | 1346.73 (P) 926.78 (B) | 30.15, 36.40, 36.94 (P) 28.55, 36.46, 36.83 (B) |
|  | 20 | 2157.85 (P) 1835.96 (B) | 32.73, 37.54, 38.58 (P) 31.18, 37.59, 38.22 (B) | 2162.36 (P) 1821.05 (B) | 32.73, 37.56, 38.60 (P) 31.17, 37.61, 38.29 (B) |
|  | 16 | 3323.85 (P) 3370.82 (B) | 35.35, 38.76, 39.79 (P) 33.88, 38.67, 39.55 (B) | 3327.59 (P) 3334.79 (B) | 35.36, 38.76, 39.80 (P) 33.89, 38.71, 39.58 (B) |
| Mobile | 28 | 377.20 (P) 129.05 (B) | 25.41, 32.44, 32.04 (P) 24.62, 32.41, 32.02 (B) | 378.29 (P) 128.41 (B) | 25.42, 32.45, 32.08 (P) 24.63, 32.41, 32.04 (B) |
|  | 24 | 795.82 (P) 416.20 (B) | 28.29, 33.37, 33.07 (P) 27.16, 33.30, 33.01 (B) | 792.53 (P) 417.42 (B) | 28.29, 33.42, 33.11 (P) 27.16, 33.35, 33.06 (B) |
|  | 20 | 1543.20 (P) 1076.20 (B) | 31.28, 34.84, 34.65 (P) 29.84, 34.73, 34.50 (B) | 1531.74 (P) 1071.48 (B) | 31.28, 34.90, 34.78 (P) 29.84, 34.78, 34.63 (B) |
|  | 16 | 2737.44 (P) 2588.26 (B) | 34.32, 36.24, 36.27 (P) 32.76, 36.08, 35.98 (B) | 2698.12 (P) 2562.00 (B) | 34.32, 36.29, 36.36 (P) 32.76, 36.13, 36.11 (B) |
| Flower CIF 15Hz | 28 | 556.05 (P) 152.46 (B) | 25.49, 31.54, 33.04 (P) 24.97, 31.62, 33.12 (B) | 554.30 (P) 151.62 (B) | 25.49, 31.57, 33.05 (P) 24.96, 31.65, 33.13 (B) |
|  | 24 | 1120.58 (P) 348.52 (B) | 28.44, 32.59, 33.62 (P) 27.72, 32.68, 33.71 (B) | 1110.80 (P) 347.73 (B) | 28.45, 32.67, 33.62 (P) 27.71, 32.73, 33.71 (B) |
|  | 20 | 1999.43 (P) 779.08 (B) | 31.44, 34.42, 34.85 (P) 30.50, 34.41, 34.92 (B) | 1975.86 (P) 774.00 (B) | 31.44, 34.50, 34.87 (P) 30.50, 34.49, 34.95 (B) |
|  | 16 | 3318.55 (P) 1746.92 (B) | 34.47, 36.26, 36.16 (P) 33.38, 36.11, 36.19 (B) | 3288.60 (P) 1723.15 (B) | 34.47, 36.33, 36.22 (P) 33.38, 36.23, 36.26 (B) |

Table 15: P frames only, CABAC, Reference frame 1

| Sequence |  | JM1.7 |  | Chroma Phase Shift |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Kbsp | dB (Y, U, V) | kbsp | dB (Y, U, V) |
| Popple | 28 | 1268.78 (P) | 28.28, 29.02, 29.10 (P) | 1253.31 (P) | 28.28, 29.02, 29.11 (P) |
|  | 24 | 2694.03 (P) | 30.43, 30.74, 30.77 (P) | 2627.97 (P) | 30.44, 30.78, 30.82 (P) |
|  | 20 | 5203.87 (P) | 32.55, 33.38, 33.35 (P) | 5017.46 (P) | 32.55, 33.45, 33.38 (P) |
|  | 16 | 8713.14 (P) | 34.66, 35.65, 35.53 (P) | 8388.13 (P) | 34.66, 35.69, 35.60 (P) |
| Src6 | 28 | 568.27 (P) | 28.45, 33.56, 35.76 (P) | 516.98 (P) | 28.46, 33.85, 35.85 (P) |
|  | 24 | 1164.64 (P) | 31.09, 34.71, 36.70 (P) | 1073.98 (P) | 31.12, 34.93, 36.88 (P) |
|  | 20 | 2272.71 (P) | 33.68, 36.85, 38.27 (P) | 2086.25 (P) | 33.70, 36.95, 38.49 (P) |
|  | 16 | 4136.89 (P) | 36.40, 38.61, 39.74 (P) | 3812.88 (P) | 36.42, 38.65, 40.04 (P) |
| Src9 | 28 | 1327.70 (P) | 27.48, 35.78, 36.04 (P) | 1330.22 (P) | 27.48, 35.78, 36.07 (P) |
|  | 24 | 2544.50 (P) | 30.08, 36.52, 37.07 (P) | 2529.45 (P) | 30.07, 36.53, 37.09 (P) |
|  | 20 | 4352.59 (P) | 32.66, 37.74, 38.66 (P) | 4324.10 (P) | 32.65, 37.71, 38.66 (P) |
|  | 16 | 7078.67 (P) | 35.34, 38.87, 39.93 (P) | 7036.54 (P) | 35.34, 38.89, 39.95 (P) |
| Mobile | 28 | 900.48 (P) | 25.16, 32.08, 31.65 (P) | 900.55 (P) | 25.17, 32.10, 31.66 (P) |
|  | 24 | 2398.10 (P) | 28.17, 32.96, 32.69 (P) | 2399.94 (P) | 28.17, 32.96, 32.70 (P) |
|  | 20 | 4731.87 (P) | 31.19, 34.44, 34.31 (P) | 4728.34 (P) | 31.20, 34.46, 34.32 (P) |
|  | 16 | 8001.19 (P) | 34.24, 35.95, 35.97 (P) | 7977.39 (P) | 34.24, 35.96, 36.00 (P) |
| Flower CIF 15Hz | 28 | 860.42 (P) | 25.55, 31.58, 33.16 (P) | 863.53 (P) | 25.56, 31.60, 33.16 (P) |
|  | 24 | 1861.39 (P) | 28.53, 32.64, 33.72 (P) | 1867.11 (P) | 28.54, 32.61, 33.71 (P) |
|  | 20 | 3487.38 (P) | 31.58, 34.42, 34.90 (P) | 3471.82 (P) | 31.58, 34.45, 34.92 (P) |
|  | 16 | 5919.57 (P) | 34.67, 36.24, 36.22 (P) | 5864.32 (P) | 34.67, 36.27, 36.24 (P) |

Table 16: P frames only, CABAC, Reference frame 5

| Sequence |  | JM1.7 |  | Chroma Phase Shift |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Kbsp | dB (Y, U, V) | kbsp | dB (Y, U, V) |
| Popple | 28 | 1116.62 (P) | 28.36, 29.12, 29.22 (P) | 1097.27 (P) | 28.39, 29.12, 29.22 (P) |
|  | 24 | 2257.01 (P) | 30.43, 30.72, 30.76 (P) | 2163.48 (P) | 30.43, 30.76, 30.79 (P) |
|  | 20 | 4380.90 (P) | 32.48, 33.31, 33.26 (P) | 4123.18 (P) | 32.49, 33.38, 33.31 (P) |
|  | 16 | 7543.33 (P) | 34.57, 35.61, 35.46 (P) | 7078.48 (P) | 34.57, 35.70, 35.54 (P) |
| Src6 | 28 | 548.94 (P) | 28.55, 33.64, 35.75 (P) | 498.34 (P) | 28.55, 33.93, 35.89 (P) |
|  | 24 | 1097.04 (P) | 31.17, 34.74, 36.77 (P) | 994.42 (P) | 31.21, 35.04, 36.97 (P) |
|  | 20 | 2111.73 (P) | 33.76, 36.84, 38.29 (P) | 1893.70 (P) | 33.76, 37.04, 38.58 (P) |
|  | 16 | 3845.89 (P) | 36.47, 38.64, 39.78 (P) | 3452.30 (P) | 36.49, 38.72, 40.15 (P) |
| Src9 | 28 | 1313.31 (P) | 27.54, 35.87, 36.08 (P) | 1311.22 (P) | 27.53, 35.89, 36.11 (P) |
|  | 24 | 2484.08 (P) | 30.14, 36.59, 37.11 (P) | 2478.25 (P) | 30.14, 36.60, 37.14 (P) |
|  | 20 | 4225.43 (P) | 32.73, 37.81, 38.72 (P) | 4201.27 (P) | 32.72, 37.82, 38.71 (P) |
|  | 16 | 6849.60 (P) | 35.40, 38.96, 39.99 (P) | 6803.44 (P) | 35.40, 38.99, 40.03 (P) |
| Mobile | 28 | 699.34 (P) | 25.52, 32.32, 31.97 (P) | 699.26 (P) | 25.51, 32.32, 31.97 (P) |
|  | 24 | 1534.57 (P) | 28.43, 33.29, 33.04 (P) | 1528.80 (P) | 28.42, 32.30, 33.06 (P) |
|  | 20 | 3107.62 (P) | 31.40, 34.72, 34.59 (P) | 3086.37 (P) | 31.40, 34.75, 34.67 (P) |
|  | 16 | 5692.00 (P) | 34.41, 36.16, 36.22 (P) | 5605.44 (P) | 34.41, 36.18, 36.27 (P) |
| Flower CIF 15Hz | 28 | 838.77 (P) | 25.60, 31.62, 33.17 (P) | 837.25 (P) | 25.60, 31.62, 33.18 (P) |
|  | 24 | 1771.40 (P) | 28.63, 32.66, 33.75 (P) | 1769.82 (P) | 28.62, 32.68, 33.77 (P) |
|  | 20 | 3267.54 (P) | 31.65, 34.46, 34.95 (P) | 3255.19 (P) | 31.65, 34.48, 34.96 (P) |
|  | 16 | 5562.61 (P) | 34.71, 36.26, 36.24 (P) | 5504.55 (P) | 34.71, 36.30, 36.27 (P) |

Table 17: 2-B frames, CABAC, Reference frame 1

| Sequence |  | JM1.7 |  | Chroma Phase Shift |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Kbsp | dB (Y, U, V) | kbsp | dB (Y, U, V) |
| Popple | 28 | 521.00 (P) 446.79 (B) | 28.21, 29.38, 29.49 (P) 26.98, 28.36, 28.55 (B) | 514.33 (P) 449.29 (B) | 28.22, 29.43, 29.50 (P) 27.00, 28.40, 28.57 (B) |
|  | 24 | 1017.62 (P) 887.62 (B) | 30.37, 30.97, 30.89 (P) 28.98, 29.41, 29.50 (B) | 993.82 (P) 877.02 (B) | 30.38, 30.99, 30.93 (P) 28.99, 29,46, 29.56 (B) |
|  | 20 | 1883.57 (P) 2088.42 (B) | 32.52, 33.45, 33.37 (P) 31.01, 32.03, 32.01 (B) | 1813.68 (P) 2014.57 (B) | 32.50, 33.47, 33.35 (P) 31.03, 32.09, 32.08 (B) |
|  | 16 | 3121.82 (P) 4024.09 (B) | 34.65, 35.73, 35.54 (P) 33.20, 34.59, 34.50 (B) | 3005.92 (P) 3862.54 (B) | 34.65, 35.81, 35.55 (P) 33.21, 34.68, 34.59 (B) |
| Src6 | 28 | 308.31 (P) 124.02 (B) | 28.43, 33.56, 35.68 (P) 27.70, 33.28, 35.58 (B) | 297.55 (P) 95.94 (B) | 28.42, 33.95, 35.79 (P) 27.72, 33.84, 35.72 (B) |
|  | 24 | 590.82 (P) 250.41 (B) | 31.06, 34.64, 36.62 (P) 30.17, 34.37, 36.49 (B) | 558.85 (P) 206.29 (B) | 31.05, 35.06, 36.84 (P) 30.14, 34.95, 36.77 (B) |
|  | 20 | 1085.01 (P) 584.46 (B) | 33.66, 36.83, 38.26 (P) 32.56, 36.46, 38.02 (B) | 1014.35 (P) 494.09 (B) | 33.66, 37.10, 38.56 (P) 32.56, 36.89, 38.44 (B) |
|  | 16 | 1846.15 (P) 1346.34 (B) | 36.41, 38.57, 39.71 (P) 35.16, 38.18, 39.40 (B) | 1727.16 (P) 1161.47 (B) | 36.41, 38.71, 40.12 (P) 35.17, 38.45, 39.95 (B) |
| Src9 | 28 | 722.14 (P) 374.11 (B) | 27.70, 35.66, 35.86 (P) 26.18, 35.73, 35.83 (B) | 720.05 (P) 372.20 (B) | 27.70, 35.64, 35.83 (P) 26.18, 35.70, 35.86 (B) |
|  | 24 | 1278.02 (P) 826.95 (B) | 30.24, 36.29, 36.86 (P) 28.63, 36.38, 36.68 (B) | 1286.15 (P) 821.13 (B) | 30.24, 36.29, 36.87 (P) 28.63, 36.40, 36.76 (B) |
|  | 20 | 2066.26 (P) 1644.16 (B) | 32.77, 37.49, 38.52 (P) 31.18, 37.53, 38.18 (B) | 2061.30 (P) 1631.28 (B) | 32.76, 37.49, 38.54 (P) 31.19, 37.54, 38.27 (B) |
|  | 16 | 31819.96 (P) 3015.87 (B) | 35.41, 38.69, 39.76 (P) 33.86, 38.61, 39.50 (B) | 3179.48 (P) 2985.33 (B) | 35.41, 38.68, 39.77 (P) 33.87, 38.64, 39.55 (B) |
| Mobile | 28 | 344.54 (P) 110.53 (B) | 25.32, 32.34, 32.02 (P) 24.60, 32.30, 31.98 (B) | 344.57 (P) 109.98 (B) | 25.33, 32.37, 32.03 (P) 24.60, 32.33, 31.99 (B) |
|  | 24 | 835.31 (P) 398.93 (B) | 28.25, 33.25, 32.96 (P) 27.04, 33.19, 32.89 (B) | 836.13 (P) 400.01 (B) | 28.25, 33.24, 33.00 (P) 27.05, 33.19, 32.91 (B) |
|  | 20 | 1636.60 (P) 1161.13 (B) | 31.26, 34.68, 34.53 (P) 29.72, 34.57, 34.35 (B) | 1624.86 (P) 1153.71 (B) | 31.25, 34.69, 34.61 (P) 29.71, 34.57, 34.41 (B) |
|  | 16 | 2785.80 (P) 2710.64 (B) | 34.27, 36.11, 36.17 (P) 32.67, 35.91, 35.84 (B) | 2759.48 (P) 2689.50 (B) | 34.27, 36.14, 36.22 (P) 32.67, 35.92, 35.89 (B) |
| Flower CIF 15Hz | 28 | 532.84 (P) 122.76 (B) | 25.54, 31.55, 33.02 (P) 25.03, 31.62, 33.11 (B) | 528.95 (P) 124.01 (B) | 25.54, 31.57, 33.03 (P) 25.03, 31.63, 33.11 (B) |
|  | 24 | 1101.67 (P) 300.58 (B) | 28.49, 32.57, 33.59 (P) 27.78, 32.64, 33.67 (B) | 1093.97 (P) 300.60 (B) | 28.50, 32.58, 33.56 (P) 27.79, 32.64, 33.66 (B) |
|  | 20 | 1961.36 (P) 687.43 (B) | 31.46, 34.46, 34.84 (P) 30.52, 34.39, 34.89 (B) | 1955.42 (P) 686.54 (B) | 31.46, 34.49, 34.85 (P) 30.52, 34.45, 34.92 (B) |
|  | 16 | 3190.53 (P) 1557.08 (B) | 34.49, 36.26, 36.16 (P) 33.38, 36.08, 36.15 (B) | 3171.49 (P) 1540.74 (B) | 34.49, 36.30, 36.21 (P) 33.38, 36.15, 36.22 (B) |

Table 18: 2-B frames, CABAC, Reference frame 5

| Sequence |  | JM1.7 |  | Chroma Phase Shift |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Kbsp | dB (Y, U, V) | kbsp | dB (Y, U, V) |
| Popple | 28 | 487.29 (P) 449.13 (B) | 28.26, 29.39, 29.47 (P) 27.02, 28.36, 28.52 (B) | 478.59 (P) 445.65 (B) | 28.28, 29.44, 29.48 (P) 27.04, 28.40, 28.56 (B) |
|  | 24 | 941.30 (P) 874.23 (B) | 30.40, 30.89, 30.91 (P) 29.01, 29.40, 29.54 (B) | 909.26 (P) 858.04 (B) | 30.37, 30.99, 30.93 (P) 29.02, 29,50, 29.59 (B) |
|  | 20 | 1761.83 (P) 2033.86 (B) | 32.49, 33.43, 33.32 (P) 31.02, 32.05, 32.01 (B) | 1700.78 (P) 1957.99 (B) | 32.50, 33.49, 33.36 (P) 31.05, 32.11, 32.08 (B) |
|  | 16 | 2958.53 (P) 3914.51 (B) | 34.61, 35.71, 35.52 (P) 33.20, 34.58, 34.48 (B) | 2837.30 (P) 3756.13 (B) | 34.62, 35.77, 35.57 (P) 33.22, 34.67, 34.57 (B) |
| Src6 | 28 | 310.88 (P) 124.39 (B) | 28.44, 33.57, 35.61 (P) 27.73, 33.29, 35.54 (B) | 288.74 (P) 96.66 (B) | 28.47, 34.01, 35.80 (P) 27.76, 33.92, 35.74 (B) |
|  | 24 | 582.34 (P) 260.06 (B) | 31.09, 34.65, 36.63 (P) 30.17, 34.34, 36.50 (B) | 542.26 (P) 204.82 (B) | 31.09, 35.10, 36.85 (P) 30.17, 34.99, 36.78 (B) |
|  | 20 | 1062.62 (P) 575.22 (B) | 33.69, 36.82, 38.29 (P) 32.59, 36.45, 38.06 (B) | 1026.83 (P) 490.74 (B) | 33.70, 37.06, 38.53 (P) 32.58, 36.88, 38.39 (B) |
|  | 16 | 1803.18 (P) 1355.34 (B) | 36.42, 38.60, 39.73 (P) 35.17, 38.18, 39.39 (B) | 1691.38 (P) 1126.93 (B) | 36.42, 38.71, 40.15 (P) 35.18, 38.47, 40.02 (B) |
| Src9 | 28 | 694.26 (P) 375.22 (B) | 27.71, 35.71, 35.90 (P) 26.20, 35.77, 35.86 (B) | 695.52 (P) 373.75 (B) | 27.70, 35.69, 35.90 (P) 26.20, 35.76, 35.92 (B) |
|  | 24 | 1237.40 (P) 823.75 (B) | 30.28, 36.36, 36.89 (P) 28.67, 36.44, 36.72 (B) | 1232.02 (P) 817.74 (B) | 30.27, 36.36, 36.90 (P) 28.67, 36.44, 36.83 (B) |
|  | 20 | 1981.26 (P) 1632.42 (B) | 32.81, 37.57, 38.59 (P) 31.23, 37.61, 38.22 (B) | 1981.66 (P) 1616.46 (B) | 32.81, 37.58, 38.57 (P) 31.24, 37.61, 38.29 (B) |
|  | 16 | 3045.39 (P) 2983.82 (B) | 35.44, 38.77, 39.82 (P) 33.92, 38.68, 39.55 (B) | 3040.02 (P) 2950.23 (B) | 35.44, 38.78, 39.82 (P) 33.91, 38.73, 39.59 (B) |
| Mobile | 28 | 336.22 (P) 115.38 (B) | 25.45, 32.42, 32.09 (P) 24.65, 32.39, 32.05 (B) | 334.57 (P) 114.62 (B) | 25.46, 32.45, 32.10 (P) 24.65, 32.42, 32.05 (B) |
|  | 24 | 729.46 (P) 371.15 (B) | 28.38 33.38, 33.10 (P) 27.23, 33.32, 33.03 (B) | 725.10 (P) 368.42 (B) | 28.38, 33.42, 33.15 (P) 27.23, 33.36, 33.08 (B) |
|  | 20 | 1392.84 (P) 952.31 (B) | 31.36, 34.82, 34.68 (P) 29.91, 34.72, 34.53 (B) | 1383.60 (P) 943.44 (B) | 31.37, 34.88, 34.78 (P) 29.90, 34.77, 34.62 (B) |
|  | 16 | 2410.58 (P) 2254.41 (B) | 34.37, 36.21, 36.27 (P) 32.80, 36.07, 36.01 (B) | 2375.82 (P) 2233.14 (B) | 34.37, 36.28, 36.38 (P) 32.80, 36.11, 36.14 (B) |
| Flower CIF 15Hz | 28 | 503.15 (P) 128.11 (B) | 25.56, 31.59, 33.04 (P) 25.05, 31.68, 33.12 (B) | 502.08 (P) 126.56 (B) | 25.56, 31.63, 33.06 (P) 25.03, 31.70, 33.14 (B) |
|  | 24 | 1028.87 (P) 299.66 (B) | 28.54, 32.59, 33.59 (P) 27.82, 32.68, 33.68 (B) | 1024.14 (P) 300.07 (B) | 28.55, 32.65, 33.58 (P) 27.83, 32.72, 33.67 (B) |
|  | 20 | 1836.82 (P) 685.50 (B) | 31.51, 34.47, 34.83 (P) 30.55, 34.44, 34.90 (B) | 1802.82 (P) 680.48 (B) | 31.52, 34.50, 34.86 (P) 30.56, 34.49, 34.94 (B) |
|  | 16 | 2973.48 (P) 1543.51 (B) | 34.55, 36.28, 36.16 (P) 33.42, 36.13, 36.20 (B) | 2929.74 (P) 1518.66 (B) | 34.54, 36.35, 36.21 (P) 33.42, 36.24, 36.26 (B) |

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Hitachi, Ltd. |  |  |
| Mailing address | 1-280, Higashi-koigakubo, Kokubunji-shi, Tokyo 185-8601, Japan |  |  |
| Country | Japan |  |  |
| Contact person | Yoshinori Suzuki |  |  |
| Telephone | +81-42-323-1111 |  |  |
| Fax | +81-42-327-7776 |  |  |
| Email | yosinori@crl.hitachi.co.jp |  |  |
| Place and date of submission | Fairfax, Virginia, USA, 6-10 May, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Core Experiment Results on Interlace Chroma Phase Shift |  |
| Contribution number |  | JVT-B042 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: