| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 March, 2002 | Document: JVT-C044 Filename: JVT-C044.doc |
| --- | --- |

| Title: | Introducing Direct Mode P-picture (DP) to reduce coding complexity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Qunshan Gu, Qi Wang, Wei Qi, Sho-Long Chen 5300 Stevens Creek, #320 San Jose, CA 95129 USA | Tel: Email: | (408)615-1888 qgu@vwebcorp.com, qiwang@vwebcorp.com, wqi@vwebcorp.com, slchen@vwebcorp.com |
| Source: | Vweb Corporation |  |  |

_____________________________

# I. Introduction

Although H.26L could bring much more benefit in coding efficiency than MPEG-4 Simple Profile (nearly 50% bits saving [1]), the high complexity issue seems significant for codec implementation. find ways to reduce

The proposed Direct Mode P picture (DP) could be used to reduce the coding complexity significantly by eliminating the “Motion Estimation + Mode Decision” process for the whole picture.

# II. Proposed Method

The DP picture is very similar with P picture except that there are no Motion Vector and Macro-block Mode information for the whole picture in the bitstream. When decoding a DP picture, derived from the co-located Macro-blocks of the he Motion Vector infon couldderived the neighbored Macro-blocks around Macro-block fros frame as shownure 1. h motion vf A-E in Figure 2 represents a block. E is th neighbored blocks around E in previous frameby the folA is the block from the left of block E, and closest to the top-left corner of block E;

block frotop ofk E, closso top-lfornro E;C is the block from the right of block E, and closest to the top-right corner of block E;

So the motion vector of current block in DP picture could be calculated as the median value of motion vector A-E from previous P picture.

Since the Macro-block mode and Motion Vector information of the Macro-block from DP picture could be derived directly from co-located Macro-block of previous frame,  there is no need to do Motion Estimation and Mode Decision for DP picture, and there is no need to code the motion vector and mode information in the DP picture. So for the DP picture, the coding complexity could be significantly reduced.

## Figure 1 Median prediction of motion vector

Based on this definition, the modifications in syntax are as follows:

At picture level, a new picture type will be added to indicate DP picture;

At macro-block level, the macro-block syntax for DP picture is defined in Figure .

Two special cases should be consider when doing DP picture coding:

Note 1: if a macro-block from previous picture is intra mode or skipped, the co-located macro-block in current frame is assigned as 16x16 inter type with zero motion vector;

Note 2: if the CBP of the macro-block from current picture is zero, then the RUN will be added by 1 and the current macro-block information will not be coded. For the decoder, it is slightly different from the normal “skipped” concept. The decoder still has to copy previous co-located macro-block’s motion vector and mode to do motion compensation. But the CBP could be considered as zero (actually no CBP will be coded when they are all zero in the DP picture macro-block).

The feasibility for DP picture comes from the temporal redundancy between two consecutive pictures, especially for scene with less motion or translational global motion. In these two cases, the motion vector fields between two consecutive pictures are very similar. So the motion vectors for the second picture could be approximated by that of the first picture, and the motion estimation and mode decision for the second picture could be omitted.

It is obvious that this method is not a universal solution. But definitely the introduced new picture type DP picture could provide much more flexibility for the encoder implementation. For example, in some application, picture skipping is not allowed, while the computation power is not enough to do 30 frames coding per second. Then the DP picture could be used to meet the requirement.

It is likely that with using DP picture, the picture quality may decrease. In next section the simulation results will show how big the impact is and in what situation it could fit.

Figure  Syntax diagram of coded macro-block in DP picture

# III. Simulation Results

Reference software JM17 is used for the simulation. The testing conditions are as follows:

- Search Range: 39

- Motion Estimation Method: _FAST_FULL_ME

- Reference Frame Number: 1

- No B Frame

- 1/4th-pel motion compensation

- UVLC

- With RD Optimization

The testing sequences used:

Akiyo (CIF), Coastguard (CIF), Container (CIF), Foreman (CIF), Hallmonitor (CIF), Silence (CIF)

For each sequence, 30 frames are used and only the first frame is coded as I picture.

The every other normal P pictures are selected as DP pictures.

For each sequence, five testing points are got by setting QP = 12, 16, 20, 24, 28. And the resulting RD curves are shown from Figure  to Figure 8

We could see from the results that for sequences with less motion (such as Akiyo, Container, Hallmonitor) or translational global motion (such as Coastguard with panning), the quality loss is less than 0.5 dB for low-medium bitrate. Those are the preferred cases for DP picture. For sequences with complex global motions (such as Forman, with some zooming), the quality loss is bigger. We could utilize the above information in real encoder implementation. The encoder could make decision of a DP picture based on the knowledge from previous P picture. For example, the motion vector and mode information of previous P picture could be analyzed and classified. If either of the two conditions (less motion or translational global motion) could be satisfied, then the current picture is possible to be coded as DP picture without big quality loss.

Since we force the DP picture selection as every other P pictures in our simulation, the total computation time cost is nearly half of the method not using DP picture.

The introduction of DP picture is just to provide a more flexible way for encoder to reduce its complexity.

Figure  Simulation results for Akiyo

Figure  Simulation results for Coastguard

Figure  Simulation results for Container

Figure  Simulation results for Foreman

Figure  Simulation results for Hallmonitor

Figure  Simulation results for Silence

# References

[1] Evaluation and Simplification of H.26L Baseline Coding Tools, Minhua Zhou, JVT-B030.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | VWEB Corporation |  |  |
| Mailing address | 5300 Stevens Creek, #320 San Jose, CA 95129 |  |  |
| Country | USA |  |  |
| Contact person | Qunshan Gu |  |  |
| Telephone | (408)615-1888 ext 244 |  |  |
| Fax | (408)615-1893 |  |  |
| Email | Qgu@vwebcorp.com |  |  |
| Place and date of submission | San Jose, CA, USA, 2002-05-01 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | ### Introducing Direct Mode P-picture (DP) to reduce coding complexity |  |
| Contribution number |  | C044 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| x | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: