| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C051r1 Filename: JVT-C051r1.doc |
| --- | --- |

| Title: | Result of Core Experiment on Adaptive MV Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report of Core Experiment |  |  |
| Author(s) or Contact(s): | Kazushi Sato, Teruhiko Suzuki and Yoichi Yagasaki Gate City Osaki East Tower 20 F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan | Tel: Fax: Email: | +81-3-5435-3669 +81-3-5435-3891 kazushi@av.crl.sony.co.jp teruhiko@av.crl.sony.co.jp yagasaki@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

- Introduction

Improvement of motion vector coding is one of the key issues for realizing higher coding efficiency in JVT coding. In Geneva meeting, adaptive motion vector coding was proposed in JVT-B061 [2] and later it was modified [3] so that it can be applied to the improved macroblock partition modes [4].

The purpose of this contribution is to verify the adaptive motion vector coding scheme proposed in Geneva meeting.

- Simulation Condition

In this experiment, the following two coders are used to evaluate the new entropy coding for motion vector data:

Anchor TML: the previous or latest TML on and after the version 1.7, and

AMVC SW: the anchor TML with the adaptive MV coding described in JVT-B115.

2.1 Test sequences

The seven sequences described in the common conditions document [4] and additional two sequences, “Carphone” and “Funfair”, are used for this test.

Table 2.1 Test sequences

| Frame Skip | 2 | 1 | 0 |
| --- | --- | --- | --- |
| Sequences | Container, QCIF,300 News, QCIF,300 Foreman, QCIF,300 | Paris, CIF,300 Silent Voice, QCIF,300 Carphone, CIF, 300 | Mobile CIF 300 Tempete CIF 260 Funfair, CIF, 300 |

2.2 Test conditions

The experiments shall be conducted according to the guideline for higher capability profile described in the common conditions document [4]. The actual test conditions are shown in Table 7.

Table 2.2 Test conditions

| MV resolution | 1/4 pel or 1/8 pel |
| --- | --- |
| UseHadamard | ON |
| Reference Frames | 1 or 5 |
| I-Picture | First frame only |
| B-picture | P-pictures only or 2 B-pictures |
| Search Range | 32 |
| Symbol Mode | UVLC or CABAC |
| Restrict Search Range | 2 |
| RD optimization | ON |
| QP | 16,20,24,28 (whole sequence) |

2.3 Experiments

2.3.1 higher capability test

The experiments of 1) - 4) shall be conducted:

- 2 B-frames, 1/4 pel MC (QCIF) and 1/8 pel MC (CIF), UVLC, 1 reference frames,

- 2 B-frames, 1/4 pel MC (QCIF) and 1/8 pel MC (CIF), UVLC, 5 reference frames,

- 2 B-frames, 1/4 pel MC (QCIF) and 1/8 pel MC (CIF), CABAC, 1 reference frames, and

- 2 B-frames, 1/4 pel MC (QCIF) and 1/8 pel MC (CIF), CABAC, 5 reference frames.

2.3.2 baseline test

The experiments of 5) - 6) are optional:

- P-frames only, 1/4 pel MC, UVLC, 1 reference frames,

P-frames only, 1/4 pel MC, UVLC, 5 reference frames.

- Simulation Result

The simulation results are shown in the file “JVT-C051r1.xls”. We can see that for the most cases the proposed algorithm outperforms JM1.7 by up to 5%.

- Conclusion

We have verified adaptive motion vector coding scheme proposed in JVT-B061. Our simulation results show that the proposed algorithm will contribute to improvement in coding efficiency by up to 5%.

References

[1] “Working Draft Number 2, Revision 4 (WD-2 rev 5)”, T. Wiegand, JVT-B118r7, DRAFT ISO/IEC 14496-10, Geneva, CH, Feb 2002

[2] “Adaptive Motion Vector Coding”, Y. Suzuki, JVT-B061r1, Geneva, CH, Feb 2002

[3] “Core Experiment Results On Improved Macroblock Prediction Modes”, H. Schwarz and T. Wiegand, JVT-B054, Geneva, CH, Feb 2002

[4] “Core Experiment Description on Adaptive MV Coding”, Y. Suzuki, JVT-B115, Geneva, CH, Feb 2002

[5] “Calculation of average PSNR difference between RD-curves”, G. Bjontegaard, VCEG-M33, March 2001

[6] “Recommended Simulation Conditions for H.26L”, G. Bjontegaard, VCEG-M75, May 2001