| ### Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C054 Filename: JVT-C054.doc |
| --- | --- |

| Title: | Study of Random Access for JVT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki and Yoichi Yagasaki Gate City Osaki East Tower 20 F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan | Tel: Email: | +81-3-5435-3669 teruhiko@av.crl.sony.co.jp yagasaki@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

Random access for JVT codec was discussed in Geneva meeting, and it was agreed to support two types of random access, instantaneous decoder refresh and gradual decoder refresh. In case of instantaneous decoder refresh, decoder is initialized at random access point and pictures after random access point do not refer to the pictures beyond random access point. Gradual decoder refresh provides the flexibility to tradeoff random and coding efficiency. But several issues, e.g. syntax, are still opened. In this document, further study on random access for JVT is reported.

# Gradual Decoder Refresh

Since JVT codec supports multiple reference frame prediction and long term frame buffer, random access is more difficult than that of MPEG-2/4. In case of MPEG-2/4, only B-pictures before the first I frame in GOP may refer to the pictures beyond random access point. All pictures after the first I-picture in display order can be decoded correctly. However, it is not applicable for JVT. P-picture may not be able to decoded correctly as well due to the multiple reference frame prediction. Closed GOP is one approach to support clear random access, but it reduces the coding efficiency. Gradual decoder refresh was proposed in JVT-B063. It proposed

- Pre-roll delay (pictures)

- Init delay (time)

- Use both

- Move random access point to any picture type

## Signaling to identify the random access point

The syntax to support the instantaneous decoder refresh for RTP NAL is proposed in JVT-B109. The definition of the gradual decoder refresh is tentatively defined in JVT-B109. In this proposal, dedicated NAL packet type is defined for instantaneous decoder refresh. IDERP (instantaneous decoder refresh packet) indicates the start of clean random access point. It is used for all slices of a randomly accessible picture. However specific packet types are not defined for gradual decoder refresh random access. Random access point must be identified easily and identification of random access point at higer syntax level is required.

Parameters for gradual decoder refresh are signaled in Supplemental Enhancement Information. The example of syntax for RTP NAL is described in Sec. 4.2.2.

# Splicing and Bitstream Editing

## Seamless and Non-seamless Splicing

Splicing is one of the important functionality of codec and JVT codec should support it. It was studied for MPEG-2 to insert the advertisement during broadcasting. For example, SMPTE standardized the splicing for MPEG-2 TS in SMPTE PT20.02, which supports both seamless and non-seamless splicing. JVT codec should support both of splicing. However the current specification doesn’t clearly support it, because 1) there are no clear syntax to carry buffer fullness, and 2) gradual decoder refresh random access is not defined yet. In this section, the extension to support splicing is described.

There are two types of splicing, seamless splicing and non-seamless splicing. Seamless splicing allows clean instantaneous switching of streams. The video stream to be spliced must be created to have matching VBV buffer characteristics at the splice. The time between when the old streams end and the last old picture is decoded must be exactly one frame less than the startup delay of the new stream.

Non-seamless splicing avoids decoder buffer overflow by inserting short dead time between two streams. This assures that new stream begins with an empty buffer. The splicing device waits before inserting the new stream to assure that the decoder’s buffer is empty, thus avoiding the chance of overflow. The decoder’s picture should freeze during the startup delay of the new stream. VBV behavior of seamless and non-seamless splicing is shown in Figure 1.

(a) Seamless splicing                                   (b) Non-seamless splicing

Figure 1: Seamless and non-seamless splicing VBV buffer behavior

The example of seamless splicing is shown in Figure 2. New stream begins the moment the old stream ends as shown in Figure 1 (a). The last old picture is decoded exactly one frame before the first new picture. The buffer occupancy at the end of old stream and occupancy just before removing bits of the first picture of new stream must be exactly the same as shown in Figure 2. In case of MPEG-2, vbv_delay of the last frame in the old stream must be the same as that of the first frame in the new stream. The splice point should be a random access point and new stream follows random access point header (GOP header). For seamless splicing, random access point should be clean random access point (closed_GOP).

Figure 2: Example of Seamless Splicing

The example of non-seamless splicing is shown in Figure 3. Splicing device holds up new stream at least drain time. The picture is frozen or turned to be black at splicing point while decoder initializes the buffer. The splice point should be a random access point and new stream follows random access point header (GOP header). For non-seamless splicing, gradual decoder refresh random access point (open_GOP) should be allowed. The pictures in the new stream before the frame indicated by pre-roll count are discarded, because it can’t be decoded correctly. The picture is frozen or turned to be block until decoder reconstructs correct pictures. In order to identify the availability of reference frame, indication of discontinuity of video stream is necessary.

Figure 3: Example of Non-seamless splicing

In order to support the above functionalities, the following extension is needed.

- Transmit buffer occupancy at the random access point

- Definition of buffer initialization at splicing point and random access point

- Definition of displaying pictures during decoder initialization

- Indication of discontinuity of stream at random access point

## Generalized HRD

The generalized hypothetical reference decoder (HRD) including multiple leaky bucket concept is defined. The parameters of leaky buckets, (R1,B1,F1),,,,(Rn,Bn,Fn) are encoded at the header of each bitstream. It intend to allow decoders at various bitrate and buffersize. Minimum bitrate and buffer size that bitstream is contained in leaky bucket is defined in sec. 13.7. The decoder can decode bitstream if its bitrate and buffer size is larger than Rmin and Bmin. The example of the characteristic of multiple leaky bucket (R, B, F) is shown in Figure 4.

In order to guaranteed the continuity of buffer characteristics at splicing point, the leaky bucket parameters(R, B, F) of new stream must be exactly the same or must be contained in those of old stream. Therefore, the following conditions must be satisfied.

Rmin_old >= Rmin_new

Bmin_old >= Bmin_new

The set of leaky bucket parameters of new stream is encoded at splicing point (random access point).

Figure 4: Illustration of leaky bucket concept

# Proposal

## Random Access Point and Buffering

In order to support the above functionalities, the following extensions are proposed.

- Transmit buffer occupancy at the random access point

- Definition of buffer initialization at splicing point and random access point

- Definition of displaying pictures during decoder initialization

- Indication of discontinuity of stream at random access point

- Carriage of leaky bucket parameters at random access point

- Leaky bucket parameters condition at random access point

## Syntax

The syntax for RTP NAL is defined in this section. At the moment, the Supplemental Enhancement Information (SEI) mechanism is used, as proposed in JVT-B109, to carry information of buffer occupancy and discontinuity of stream.

### NALP type

Regarding NAL packet type, dedicated packet type for gradual decoder refresh is also needed to identify the gradual decoder refresh random access point effectively. An gradual decoder refresh packet (GDERP) type should be added to NALP type. New NALP type is as follows,

NALP type		EI-Flag		NALP First Byte

GDERP		0			0x82

GDERP		1			0x83

### Syntax elements

The syntax for MPEG-2 NAL is described in JVT-C055. In case of MPEG-2 NAL, the following information is encoded at random access point header (GOP header).

aligned(8) class VBVBufferOccupancy aligned(8) extends SEIMessage(SEI_VBVBufferOccupancy) {

unsigned int(16) vbvBufferOccupancy;

}

aligned(8) class discontinuity aligned(8) extends SEIMessage(SEI_discontinuity) {

unsigned int(1) discontinuity;

}

vbvBufferOccupancy indicates the buffer occupancy just before removing the bits associated with the frame.

discontinuity indicates the availability of reference frame of multiple reference picture prediction. discontinuity = 1 means that the reference picture is not available due to the editing or splicing. Decoder may not be reconstruct pictures by the frame indicated by pre-roll count.

### Examples

The example of random access point with instantaneous decoder refresh is as follows,

first_byte = “compound packet”

sub_packet = “SEI”

SEI_message = PresentationTimeStamp

SEI_message = DecodingTimeStamp

SEI_message = VBVBufferOccupancy

SEI_message = dicsontinuity

SEI_message = HRD_parameters

sub_pecket = “IDERP”

The example of random access with gradual decoder refresh is as follows,

first_byte = “compound packet”

sub_packet = “SEI”

SEI_message = PresentationTimeStamp

SEI_message = GradualDecoderRefreshPreRollCount

SEI_message = VBVBufferOccupancy

SEI_message = dicsontinuity

SEI_message = HRD_parameters

sub_pecket = “GDERP”

The example of random access point header for MPEG-2 NAL is described in JVT-C055.

# Summaries

In this contribution, further study on random access for JVT codec is reported. Splicing and bitstream editing for JVT codec is studied. JVT codec should support splicing and bitstream editing at video layer as it is supported by MPEG-2. The syntax and semantics extensions are proposed to support splicing.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20 F 1-11-1 Osaki Shinagawa-ku, Tokyo, 141-0032 |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5435-3669 |  |  |
| Fax | +81-3-5435-3891 |  |  |
| Email | teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission | Tokyo, Japan on Jan. 23rd in 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Study of random access for JVT |  |
| Contribution number |  | C054 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X |  |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: