| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 March, 2002 | Document: JVT-C067 Filename: JVT-C067.doc |
| --- | --- |

| Title: | Experiment result on multi-frame interpolative prediction with default coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yoshihiro Kikuchi Takeshi Chujoh | Tel: Email: | +81 44 549 2288 yoshihiro.kikuchi@toshiba.co.jp |
| Source: | Toshiba |  |  |

_____________________________

Summary

The purpose of this contribution is to report the evaluation result of the multi-frame interpolative prediction (MFIP) originally proposed in JVT-B075[1]. The detailed description and the B-picture based modified syntax are described in JVT-C066[2]. For the precise evaluation purpose, several fade types, fade from black and white with linear, quadratic-curve and S-curve, and cross-fade are used for the simulation. The MFIP coefficients used in this contribution are default ones, (1/2,1/2,0) and (2,-1,0), where the prediction signal is calculated only by shift and addition operations. Simulation results show the coding performance of MFIP for overall fade patterns as well as normal sequences even with the simple default coefficients; MFIP outperforms JM up to 1.7dB for fade sequences and 0.3 dB for normal sequences.

# Test condition

Computer simulations were conducted to evaluate the MFIP following the common test condition (VCEG-N81)[3]. The following tables summarize the simulation condition.

Common test condition

| Base program | JM1.7 |
| --- | --- |
| R-D optimization | used |
| MV search range | 32 |
| Bi-directional/Backward prediction | not used(forward prediction only) |
| CABAC | not use(UVLC) |
| 1/8 pel MC | not used(up to 1/4 pel accuracy) |
| Slice structure, Data partitioning | not used |
| Hadamard | used |
| Number of reference pictures | 5(fade), 5/2(normal) |
| QP | 4, 8, 12, 16, 20, 24, 28 |

The algorithm specific parameters are summarized in the following table.

Algorithm specific parameters

| Number of interpolated frames | 2 |
| --- | --- |
| MFIP coefficient | (1/2,1/2,0) and (2,-1,0) |
| dmv search range | 0 |

Though MFIP is a general frame interpolative prediction framework, MFIP coefficients used in this contribution are only default ones, (1/2,1/2,0) and (2,-1,0). In this case, the prediction signal calculation is simplified as the following table (see JVT-C066). It is accomplished by one shift and one addition/subtraction operations; the same calculation amount as bi-directional interpolation of B-picture. Combined with the B-picture based syntax in JVT-C066, no overhead information is necessary to switch between two coefficients.

Default MFIP coefficients

| MFIP coefficient | prediction signal |
| --- | --- |
| (1/2, 1/2, 0) | (ref1 + ref2)/2 |
| (2, -1, 0) | 2*ref1 - ref2 |

To evaluate the performance for fade sequences, additional video sequences with fade were generated. The following table lists the fade patterns and test sequences used for the simulation. Only linear fade-in from black was used in JVT-B057. This time, other several fade patterns (quadratic curve, sigmoid curve, cross-fade, fade-in from white) are additionally used. The detail for fade sequence generation is explained in Annex A.

Fade patterns and test sequences

| Fade type | Video sequence | Fade duration |
| --- | --- | --- |
| Linear fade from black | Container(QCIF 10Hz) News(QCIF 10Hz) Foreman(QCIF 10Hz) | 2 sec. |
| Quadratic curve fade from black |  |  |
| Sigmoid (S-curve) fade from black |  |  |
| Linear fade from white |  |  |
| Quadratic curve fade from white |  |  |
| Sigmoid (S-curve) fade from white |  |  |
| Cross fade | Container to News (QCIF 10Hz) | 2 sec. |

# Simulation result and consideration

The simulation results are shown in the attached excel file (JVT-C067.xls). Average SNR improvements and bitrate savings are summarized in the table bellow.

There are coding gains for overall fade sequences. The average SNR improvements are 0.50 dB to 1.74 dB. The coding gains for fade-from-white are almost the same as those for fade-from-black. The improvement is also observed for cross-fade sequence; the SNR improvement is about 0.6dB. These verify the effectiveness of the extrapolation prediction with MFIP coefficient (2,-1,0) not only for simple fading like fade-from-black but also for general fading like fade from non-black colors or cross-fade.

There are also improvements for normal sequences especially for small QP values (high bitrate). The SNR improvement and bitrate saving are up to 0.32dB and 11.93%, respectively. 2 reference frames plus MFIP outperforms JM with 5 reference frames in case of Hall monitor.

Average SNR improvement and bitrate saving

| Sequence | ### Effect |  | avsnr4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | QP=16,20,24,28 |  | QP=4,8,12,16 |  |
|  |  |  | ΔPSNR(dB) | Δbitrate(%) | ΔPSNR(dB) | Δbitrate(%) |
| Container | Fade-in from black | Linear | 1.62 | -24.54 | 0.84 | -14.46 |
|  |  | Quadratic | 1.67 | -26.71 | 1.07 | -20.48 |
|  |  | S-curve | 1.53 | -23.94 | 0.97 | -17.81 |
| News |  | Linear | 0.99 | -17.35 | 0.91 | -13.57 |
|  |  | Quadratic | 1.17 | -21.18 | 0.96 | -17.23 |
|  |  | S-curve | 1.16 | -19.70 | 1.05 | -16.71 |
| Foreman |  | Linear | 0.62 | -11.28 | 0.25 | -4.65 |
|  |  | Quadratic | 0.78 | -15.64 | 0.38 | -8.14 |
|  |  | S-curve | 0.61 | -10.99 | 0.27 | -5.39 |
| Container | Fade-in white | Linear | 1.45 | -23.50 | 0.87 | -15.13 |
|  |  | Quadratic | 1.62 | -26.95 | 1.05 | -20.30 |
|  |  | S-curve | 1.50 | -24.41 | 0.99 | -18.25 |
| News |  | Linear | 1.74 | -25.73 | 1.22 | -17.20 |
|  |  | Quadratic | 1.60 | -26.74 | 1.32 | -21.60 |
|  |  | S-curve | 1.56 | -22.95 | 1.14 | -17.47 |
| Foreman |  | Linear | 0.50 | -9.49 | 0.18 | -3.39 |
|  |  | Quadratic | 0.62 | -12.82 | 0.33 | -7.26 |
|  |  | S-curve | 0.56 | -10.55 | 0.25 | -5.05 |
| ### Container -> News | Cross-fade | Linear | 0.63 | -10.56 | 0.66 | -8.94 |
| Mobile(ref. 2frames) | None |  | 0.14 | -2.96 | 0.29 | -4.22 |
| Hall(ref. 2frames) |  |  | -0.02 | 0.31 | 0.32 | -11.93 |
| Mobile(ref. 5frames) |  |  | 0.04 | -0.77 | 0.15 | -2.30 |
| Hall(ref. 5frames) |  |  | 0.04 | -1.22 | 0.28 | -10.77 |

# Conclusion

Multi-frame interpolative prediction, which is originally proposed in JVT-B075, was evaluated based on the current JM software and with additional fading patterns. Experimental results show that there are coding gains for overall fade sequences of up to 1.7dB, and 0.3 dB for normal sequences, even with the simple default coefficients (1/2,1/2,0) and (2,-1,0).

# References

[1] Y. Kikuchi, T. Chujoh, “Improved multiple frame motion compensation using frame interpolation”, JVT-B075, Jan. 2002.

[2] Y. Kikuchi, T. Chujoh, S. Kadono, “Multi-frame interpolative prediction with modified syntax”, JVT-C066, May 2002.

[3] G. Sullivan, G. Bjontegaard, "Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material", VCEG-N81, Sep. 2001.

Annex A

The fade curve functionf(t) is represented as follows,

- Linear	f(t)=tT

- Quadratic	f(t)=tT2

- S-curve	f(t)=2tT2,0≤t≤T2

where T is the fading duration. Fading image g(t) is created by the following formula,

g(t)=a(t)(1−f(t))+b(t)f(t)

a(t) and b(t) are set as the following table.

|  | a(t) | b(t) |
| --- | --- | --- |
| fade-in from black/white | black/white | original image |
| fade-out to black/white | original image | black/white |
| cross-fade | original image A | original image B |

Black and white images are created by adding random value 0 or 1 toY,Cb,Cr=16,128,128 and Y,Cb,Cr=239,128,128, respectively.