| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C079 Filename: JVT-C079.doc |
| --- | --- |

| Title: | Improved Coding of Slice Headers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Authors: | Miska M. Hannuksela Nokia Mobile Software P.O. Box 68 33721 Tampere Finland Jani Lainema Nokia Research Center 6000 Connection Drive Irving, TX 75039, U.S.A. | Tel: Email: | +358 40 521 2845 miska.hannuksela@nokia.com +1 972 894 4045 jani.lainema@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Proposal

We have understood that the primary reason for using VLC in all codewords has been extensibility of the codeword space. However, in the case of certain slice header parameters, the maximum codeword value is limited by the parameter set in use. Thus, an extension of the range of these parameters actually requires a change in the corresponding values in the parameter set. Consequently, a fixed length code whose length depends on the maximum values given in the parameter set could be used. We analyzed the bit-rate savings of fixed-length coding of certain parameters in Section 2. Also bit-rate savings obtained by introducing a sequence level quantization parameter and coding the slice quantization parameters differentially with respect to that are analyzed. Based on the analysis, we propose changes and additions into the definitions of the parameter set and the slice header.

## Proposed Changes to the Parameter Set

Following changes are proposed to the definition of the parameter set:

- Legal values for the maximum picture number (MAX_PN) are 2MAX_PN_LEN – 1, MAX_PN_LEN = 2, 3, 4, ...

- Slice size granularity (SSG) signals which is the minimum size of a slice in number of macroblocks. A slice can contain one or many sets of SSG macroblocks in raster scan order. Every SSG’th macroblock is a possible slice start position. Possible slice start positions are enumerated in raster scan order starting from zero.

- Maximum slice size (MAX_SS) signals the maximum size of a slice in number of sets of SSG macroblocks.

- Sequence Quantization Parameter (SeQP) signals the reference quantization parameter that is used to decode the slice quantization parameters.

Maximum slice start position (MAX_SSP) is calculated based on SSG and the number of macroblocks in a picture (NUM_MBS): 
MAX_SSP = NUM_MBS / SSG + (NUM_MBS % SSG > 0) - 1, 
where / donates division by truncation, % donates the modulo of the division, and > is a “greater than” comparison operator that results into 1 if the condition is true and 0 otherwise.

JVT-C078 proposes four parameter set types, namely independent GOP, picture, slice, and presentation parameter set. If JVT-C078 gets adopted, SSG and MAX_SS belong to the picture parameter set, as their value must not be changed during a picture. SeQP belongs to the slice parameter set, as the value of the reference quantization parameter can be changed slice by slice.

## Proposed Changes to the Slice Header

The slice header is modified as follows:

- Picture Number (PN) is a fixed-length code, whose size is MAX_PN_LEN bits.

- Slice start position (SSP) replaces “First MB in slice”. SSP is a fixed-length code, whose size is ceil(log2(MAX_SSP)) bits. The macroblock number (in raster scan order) of the first macroblock of the slice is equal to SSP x SSG.

- Slice size (SS) replaces “Number of macroblocks in a slice”. SS is present only if MAX_SS is greater than one. SS is a fixed-length code, whose size is ceil(log2(MAX_SS – 1) bits. The number of macroblocks in a slice is equal to (SS + 1) x SSG.

- Slice Difference Quantization Parameter (SDQP) is transmitted instead of the Slice Quantization Parameter (SQP). The value of the SQP is obtained by adding the values of SeQP and SDQP.

Note that if the on-line calculation of MAX_SSP, ceil(log2(MAX_SSP)), and ceil(log2(MAX_SS – 1) is considered too cumbersome for the decoder, their values could be added to the parameter set as well.

A single UVLC codeword is used to represent the SDQP. Codeword indexes for the SDQP are mapped starting from zero and increasing as the absolute value of the differences increases, not allocating codewords for difference values leading to overflowing or underflowing SQP. The SeQP is coded in the same way using the maximum allowed QP value 39 as prediction. The SPQP parameter is proposed to be coded using the same approach and SQP as the prediction. The same mapping of codeword indexes is proposed also for Macroblock Dquant values for consistency, compression efficiency (no wrapping of high probability codewords to the other end of the parameter space) and complexity (no modulo operations) reasons.

Annex A of the contribution contains a change-bar version of section 8 of JVT WD2r7 that contains the proposed slice header modifications.

# Analysis of Bit-Rate Savings

## Picture Number

Picture Number (PN) shall be incremented by 1 for each coded and transmitted picture, in modulo MAX_PN operation, relative to the PN of the previous stored picture.  For non-stored pictures, PN shall be incremented from the value in the most temporally recent stored picture, which precedes the non-stored picture in bitstream order.

Next, we take some examples how many bits could be saved when coding PN in FLC:

- Assuming MAX_PN of 255, the average length of PN is about 13 bits, whereas the corresponding fixed-length code would be 8 bits long.

- Assuming MAX_PN of 1023, the average length of PN is about 17 bits, whereas the corresponding fixed-length code would be 10 bits long.

In order to make the bit-rate savings more concrete, we considered an example case of video communication over a mobile channel. We assumed QCIF picture size, bit-rate of 64000 bits/sec, an average of 100 bytes per slice, and 8-bit picture numbers. The bit-rate saving caused by fixed-length coding of PN was 400 bits/sec  0.6 %.

## Slice Start Position and Size

Two ways to signal the position of a slice have been proposed in the JVT standardization previously:

- The number of the first macroblock of the slice. Macroblocks are numbered in raster scan order. The top-left macroblock of the picture has index 0.

- The horizontal position (column) and the vertical position (row) of the first macroblock in the slice are signaled in separate codewords. Columns are enumerated from left to right starting from 0. Rows are enumerated from top to bottom starting from 0.

Next, we take some examples how many bits could be saved when coding slice start position as a fixed-length code:

- Let us assume QCIF picture size and arbitrary slice start position. If macroblock positions are enumerated in raster scan order and coded with VLC, the expected codeword length is about 10.6 bits. A similar fixed length code could be signaled with 7 bits. If column and row are signaled separately with VLC, their expected lengths are 5 and 4.6 bits respectively.

- Let us assume CIF picture size and arbitrary slice start position. If macroblock positions are enumerated in raster scan order and coded with VLC, the expected codeword length is about 14.5 bits. A similar fixed length code could be signaled with 9 bits. If column and row are signaled separately with VLC, their expected lengths are 6.6 and 6.1 bits respectively.

In order to make the bit-rate savings more concrete, we considered an example case of video communication over a mobile channel. We assumed QCIF picture size, bit-rate of 64000 bits/sec, and an average of 100 bytes per slice. The bit-rate saving caused by fixed-length coding of the slice start position was about 204 bits/sec  0.3 %.

A fixed number of macroblocks per slice, such as a row of macroblocks, is likely to be used in many video communication systems. To reflect these coding scenarios, we propose that a new parameter called slice size granularity (SSG) is added to the parameter set. SSG signals which is the minimum size of a slice in number of macroblocks. A slice can contain one or many sets of SSG macroblocks in raster scan order. Every SSG’th macroblock is a possible slice start position. Possible slice start positions are enumerated in raster scan order starting from zero. For example, if we code a QCIF picture and want to have one slice per macroblock row, slice size granularity is 11. Moreover, the left-most macroblock in each macroblock row is a possible slice start position, and the slice start positions run from 0 to 8. Consequently, the slice start position requires a 4-bit fixed-length code. Assuming QCIF picture size and 15 Hz frame rate, the bit-rate saving compared to fixed-length coding of a complete macroblock address (7 bits per slice) is 405 bits/sec.

For CABAC entropy coding, the number of macroblocks contained in the slice is transmitted in the slice header. The maximum slice size (MAX_SS) could be signaled in the parameter set in units defined by SSG. For example, if one slice is always a macroblock row, no slice size needs to be transmitted in the slice header. Assuming QCIF picture size, 15 Hz frame rate, and one slice per macroblock row, 945 bits/sec would be saved.

## Slice Quantization Parameter

The JM spends significant overhead to code the quantization parameters in every slice header. The slice QP in JM 2.0 is coded as difference to the maximum possible QP value 39. For example, 9 bits are needed to code a QP between 9 and 24. In a 15 Hz QCIF case with one row of Macroblocks in each slice, this corresponds to more than 1.2 kbps overhead.

Attached spreadsheet JVT-C079.xls contains examples of bitrate savings achieved with the proposed change. One to five percent of the total bitrate is saved in QCIF test sequences, when slices containing one row of Macroblocks are used.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name |  |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: