| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C083 Filename: JVT-C083.doc |
| --- | --- |

| Title: | Signaling of “Clean” Random Access Positions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Mobile Software P.O. Box 68 33721 Tampere Finland | Tel: Email: | +358 40 521 2845 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Introduction

Instantaneous decoder refresh refers to “clean” random access, where no data prior to the random access position is referred to in the decoding process. JVT-B041 analyzed the requirements for instantaneous decoder refresh, and appropriate syntax and semantics were published in JVT-B109 and reviewed during the Geneva meeting. There was no disagreement on the contents of JVT-B109, but as the review was done in such a late phase of the meeting, the final decision of the adoption was deferred to the Fairfax meeting. This contribution recaps the main points of JVT-B041 and JVT-B109 regarding instantaneous decoder refresh and includes a WD text proposal. No technical change compared to JVT-B041 and JVT-B109 has been done.

The gradual decoder refresh aspect of JVT-B109 has been developed further and is presented in JVT-C074. The timestamp signaling aspect of JVT-B109 is reconsidered and presented in JVT-C070.

Section 2 presents the proposal and Annex A of the contribution presents the proposed changes to the standard text.

# Proposal

## Overview

This section recaps the proposal in JVT-B109. There were two aspects in the proposal: identification of random access positions using a specific NAL packet type and a specific picture number update rule in the instantaneous decoder refresh positions.

## Instantaneous Decoder Refresh NAL Packet

An Instantaneous Decoder Refresh NAL Packet (IDERP) indicates a “clean” random access position, from which decoding and displaying can be re-started without reception of any prior coded slices or data partitions. An IDERP contains a single I- or SI-slice. Once IDERP is used for one slice of a picture, all other slices of the picture are also encapsulated in IDERPs. An IDERP implies that all pictures in the multi-picture buffer are marked as “unused” except the current picture. Moreover, the maximum long-term index is reset to zero.

We propose that the group of pictures starting from an instantaneous decoder refresh position (inclusive) and including all the pictures until the next instantaneous decoder refresh position (exclusive) is called an independent GOP or an independently decodable GOP.

Since the Geneva meeting, the structure of the first byte of the NAL packet was reconsidered. The structure of the first byte, that indicates the NALP type and the status of the error indication flag (EI), could not be chosen linearly because of design issues in byte-stream-oriented networks. In particular, this byte, preceded with a fixed two-byte start code prefix, is going to be used as a start code in the MPEG-2 transport environment. To avoid start code emulations, values 0x00 and 0xb9 to 0xff were reserved. Thus, the NALP type according to the table below is proposed:

NALP type     EI-Flag     NALP First Byte

SSP           0           0x10

SSP           1           0x11

DPA           0           0x20

DPA           1           0x21

DPB           0           0x30

DPB           1           0x31

DPC           0           0x40

DPC           1           0x41

SEIP          0           0x50

SEIP          1           0x51

PUP           0           0x60

PUP           1           0x61

CP            0           0x70

CP            1           0x71

IDERP         0           0x80

IDERP         1           0x81

An instantaneous decoder refresh packet (IDERP) implies that all pictures in the multi-picture buffer are marked as “unused” except the current picture. Moreover, the maximum long-term index is reset to zero. The payload of the packet contains a single I or SI slice, as if it was coded in the single slice mode. The “Instantaneous Decoder Refresh” packet type shall be used for all slices of a randomly accessible picture. (Note: As I or SI slices are always used with instantaneous decoder refresh, no need for data partitioning was foreseen. Therefore, the “Instantaneous Decoder Refresh” packet type implies the single slice mode.)

See the most up-to-date definition of NALP types in the Internet Draft for RTP payload Format for JVT Video (ftp://ftp.ietf.org/internet-drafts/draft-wenger-avt-rtp-jvt-00.txt).

## Picture Number Update Behavior

JVT-B109 proposed that the picture number shall be reset to zero in a picture corresponding to an instantaneous decoder refresh position. The proposal is justified by improved error resiliency, which results from two aspects:

- The first picture of an independent GOP resets the multi-picture buffer. Thus, if the first picture of an independent GOP is lost, multi-picture buffer is not reset and the maximum long-term index might be greater than zero. Consequently, too few pictures might be buffered as short-term pictures after a loss of the first picture of an independent GOP. When the picture number is reset to zero in the beginning of an independent GOP, the decoder can deduce a probable loss of the first picture of an independent GOP, when it suddenly receives picture number 1 (or any other small number) after previous relative large received picture number. Then, it can reset the multi-picture buffer.

- Many times random access positions are aligned with scene cuts. Let us assume the following example:

Figure 1. An example of an independent GOP without picture number reset.

Boxes represent coded pictures and arrows represent motion compensation. A picture coding type (P or I) and a picture number is included in each box. Frame I37 starts a new scene and is coded as an intra frame and transmitted as IDERPs. Let us assume that frame I37 is lost during transmission. Because no information on the scene cut was received, a typical decoder would conceal the lost frame I37 by repeating frame P36. Frames P38 and P39 are then reconstructed normally but using a totally incorrect reference picture, which would cause temporal propagation of the error for the whole independent GOP. When the picture number is reset to zero, as illustrated in Figure 2, the decoder can deduce that an intra-frame was lost and avoid the use of temporal error concealment.

Figure 2. An example of an independent GOP with picture number reset.

Notice that contribution JVT-C082 proposes signaling of spare reference pictures. The mechanism allows an encoder to signal if the first frame of an independent GOP can be replaced with an earlier frame in case of a picture loss.

In Geneva, JVT concluded picture number resetting is an appropriate action.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Corporation |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Signaling of “Clean” Random Access Positions |  |
| Contribution number |  | JVT-C083 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: