| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C088 Filename: JVT-C088.doc |
| --- | --- |

| Title: | Adaptive Transform Coefficient Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz and Ragip Kurceren Nokia Research Center, 6000 Connection Drive Irving, Texas 75039, USA | Tel: Email: | +972 894 4188 marta.karczewicz@nokia.com |
| Source: | Nokia Research Center |  |  |

## Method overview

New method of transform coefficients' coding is proposed. Firstly, the number of non-zero coefficients within the block denoted as N is determined and encoded. Later, each of the non-zero coefficients in the block in question is encoded as (run,level) pair. Single scan is used for Inter and Intra blocks. Since the number of non-zero coefficients is sent, there is no need to transmit an EOB symbol.

## Number of non-zero coefficients coding

Four VLCs are used to code the number of non-zero coefficients in the block (codeword number is equal to non-zero number). 3 of the VLCs are Rice-Golomb codes for k=0,1,2 and are constructed by concatenation of a unary and a binary part. The unary term is j+1 bits in length where j=0,1,…, and is followed by k bits of the binary part.

Code No. 	VLC(0) (k=0)		VLC(1)(k=1)		VLC(2) (k=2)

0		1			10			100

1		01			11			101

2		001			010			110

3		0001			011			111

4		00001			0010			0100

5		000001			0011			0101

…		…			…			…

VLC(3) consists of codewords with length equal to 4 for code numbers 0,1, …,14 and codewords with length equal to 5 for code numbers 15 and 16. The 4-bit codewords are the binary representation of the corresponding codenumbers whereas codewords for 15 and 16 are given by 11110 and 11111. Choice of the table VLC(i) is done as follows:

i=vlcNo(indN), where

vlcNo=[0, 0, 1, 1, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 3, 3] for macroblocks in Intra mode,

vlcNo=[0, 0, 0, 1, 1, 1, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 3] otherwise.

Index indN is calculated based on the nonzero values Nu and Nl of the upper and left 4x4 blocks as given in the table below. By X we denote that the block is available and coded in the same mode, i.e., Inter, Intra-4x4 and Intra-16x16, as the current block.

| Upper block (Nu) | Left block (Nl) | IndN |
| --- | --- | --- |
| X | X | (Nl+Nu+1)/2 |
| X |  | Nu |
|  | X | Nl |
|  |  | 0 |

For 4x4 DC luma blocks, i is always equal to 0. For 2x2 DC chroma U blocks, i is equal to 0 while for V block, it is equal to 0 if N of the U block is smaller or equal to 2 and 1 otherwise.

Run-Level Coding

(run level) pairs are sent in the reverse order, i.e., starting from the last (run, level) pair within the block. 5 different tables Tab(i), i=0,1,2,3,4 , given in Table 1, are used to assign codeword numbers to (run, level) pairs. Since run values for Tab(0) are always equal to 0, this table need not to be kept in the memory.

Table 1: The correspondence between (run,level) pairs and the codeword numbers. The remaining level/run combinations are assigned to a code number according to the same priority (equation-based formula) as in JM with the difference that the maximum run value depends on index indR defined below. The maximum run value is equal to 0, 1, 3, 7 and 15 for indR equal to 0, 1, 2, 3 and 4, respectively.

| Codeword Number | Tab(0) |  | Tab(1) |  | Tab(2) |  | Tab(3) |  | Tab(4) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | level (+,-) | run | level (+,-) | run | level (+,-) | run | level (+,-) | run | Level (+,-) | run |
| 0, 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 2, 3 | 2 | 0 | 2 | 0 | 2 | 0 | 1 | 1 | 1 | 1 |
| 4, 5 | 3 | 0 | 3 | 0 | 1 | 1 | 2 | 0 | 1 | 2 |
| 6, 7 | 4 | 0 | 4 | 0 | 3 | 0 | 1 | 2 | 1 | 3 |
| 8, 9 | 5 | 0 | 5 | 0 | 4 | 0 | 3 | 0 | 1 | 4 |
| 10, 11 | 6 | 0 | 6 | 0 | 2 | 1 | 1 | 3 | 2 | 0 |
| 12, 13 | 7 | 0 | 1 | 1 | 1 | 2 | 2 | 1 | 1 | 5 |
| 14, 15 | 8 | 0 | 7 | 0 | 5 | 0 | 1 | 4 | 1 | 6 |
| 16, 17 | 9 | 0 | 8 | 0 | 3 | 1 | 4 | 0 | 1 | 7 |
| 18, 19 | 10 | 0 | 2 | 1 | 6 | 0 | 2 | 2 | 3 | 0 |
| 20, 21 | 11 | 0 | 9 | 0 | 2 | 2 | 3 | 1 | 1 | 8 |
| 22, 23 | 12 | 0 | 3 | 1 | 1 | 3 | 1 | 5 | 2 | 1 |
| 24, 25 | 13 | 0 | 10 | 0 | 7 | 0 | 2 | 3 | 1 | 9 |
| 26, 27 | 14 | 0 | 11 | 0 | 4 | 1 | 5 | 0 | 2 | 2 |
| 28, 29 | 15 | 0 | 12 | 0 | 8 | 0 | 6 | 0 | 4 | 0 |
| 30, 31 | 16 | 0 | 4 | 1 | 3 | 2 | 3 | 2 | 1 | 10 |

Choice of the table number i for each (run,level) pair is based on the maximum possible run maxRun and the maximum magnitude lPrev of the previously coded levels in the block. maxRun is initialized at the beginning of each block as 16-N with the exception of Intra-16x16 and AC/DC chroma blocks for which it is initialized as 4-N for DC chroma and 15-N otherwise. After a (run, level) pair is coded, the index maxRun  is updated as maxRun=maxRun-run. The table number i is calculated as follows:

i=min(indR, indL) where

indR=indRTab(maxRun),  indRTab=[0, 1, 2, 2, 3, 3, 3, 3, 4, 4, 4, 4, 4, 4, 4, 4],

and value of indL is given in the table below:

|  | Intra macroblocks |  |  |  |  |  |  |  |  | Inter macroblocks |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | lPrev |  |  |  |  |  |  |  |  | lPrev |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | >=6 |  |  | 0 | 1 | 2 | 3 | 4 | 5 | >=6 |
| all QPs | 4 | 3 | 2 | 2 | 1 | 1 | 0 |  | QP<8 | 4 | 4 | 3 | 2 | 2 | 2 | 1 |
|  |  |  |  |  |  |  |  |  | 24>QP>=8 | 4 | 4 | 3 | 2 | 2 | 1 | 1 |
|  |  |  |  |  |  |  |  |  | 28>QP>=24 | 4 | 4 | 2 | 1 | 1 | 0 | 0 |
|  |  |  |  |  |  |  |  |  | QP>=28 | 4 | 3 | 2 | 1 | 0 | 0 | 0 |

3 different VLCs are used to code codeword numbers: exp-Golomb codes for parameters k=1, 2 and 3:

| VLC1 (k=1) |  |  |  | VLC2 (k=2) |  |  |  | VLC3 (k=3) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Unary part | Binary part |  | No. | Unary part | Binary part |  | No. | Unary part | Binary part |
| 0 | 1 | 0 |  | 0 | 1 | 00 |  | 0 | 1 | 000 |
| 1 | 1 | 1 |  | 1 | 1 | 01 |  | 1 | 1 | 001 |
| 2 | 01 | 00 |  | 2 | 1 | 10 |  | 2 | 1 | 010 |
| 3 | 01 | 01 |  | 3 | 1 | 11 |  | 3 | 1 | 011 |
| 4 | 01 | 10 |  | 4 | 01 | 000 |  | 4 | 1 | 100 |
| 5 | 01 | 11 |  | 5 | 01 | 001 |  | 5 | 1 | 101 |
| 6 | 001 | 000 |  | 6 | 01 | 010 |  | 6 | 1 | 110 |
| 7 | 001 | 001 |  | 7 | 01 | 011 |  | 7 | 1 | 111 |
| … | … | … |  | … | … | … |  | … | … | … |

Exp-Golomb code with parameter k has 2k codewords of shortest length, 2k+1 codewords of next shortest length, etc. Each codeword is constructed by concatenation of a unary and a binary part. The unary term is j+1 bits in length where j=0,1,…, and is followed by k+j bits of the binary term.

Choice of the parameter k depends on the values of QP and lPrev:

|  | Intra macroblocks |  |  |  |  |  |  |  |  | Inter macroblocks |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | lPrev |  |  |  |  |  |  |  |  | LPrev |  |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | >=6 |  |  | 0 | 1 | 2 | 3 | 4 | 5 | >=6 |
| QP<8 | 1 | 1 | 2 | 2 | 2 | 3 | 3 |  | QP<8 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |
| 8<=QP<16 | 1 | 1 | 2 | 2 | 2 | 2 | 3 |  | QP>=8 | 1 | 1 | 1 | 2 | 2 | 2 | 3 |
| QP>=16 | 1 | 1 | 1 | 2 | 2 | 2 | 2 |  |  |  |  |  |  |  |  |  |

## Results

The proposed method is implemented in JM 1.1 and the results of the simulations are summarized in Table 3 (Intra Coding) and Table 4 (Inter Coding) in terms of Bjontegaard Delta bitrate savings in % from JM1.1. The test conditions from VCEG-N81 were followed and the detailed results can be found in the attached excell-spreadsheet. We also include in the spreadsheet a comparison with Real Networks' results that were sent to the email reflector on 1st of May, 2002.

Table 3: Performance improvement (Bjontegaard Delta bitrate savings in %) of the proposed coefficient coding in Intra coding. QP = 0, 4, 8, 12 is used for low QP and QP = 16, 20, 24, 28 for high QP.

| QP | Container | Foreman | News | Silent | Tempete | Mobile | Paris |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -15.79 | -11.84 | -17.23 | -14.06 | -18.31 | -22.83 | -17.83 |
| High | -5.52 | -2.64 | -7.64 | -2.97 | -5.84 | -8.34 | -5.76 |

Table 4: Performance improvement (Bjontegaard Delta bitrate savings in %) of the proposed coefficient coding in Inter coding. QP = 0, 4, 8, 12 is used for low QP and QP = 16, 20, 24, 28 for high QP.

| QP | Container | Foreman | News | Silent | Tempete | Mobile | Paris |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -4.92 | -8.01 | -11.19 | -8.29 | -13.21 | -14.12 | -12.92 |
| High | -0.91 | -1.28 | -3.3 | -1.56 | -1.15 | -0.58 | -1.97 |

## Conclusions

The proposed method significantly improves overall performance of the JVT coder, the improvements are reaching 23% for inter and almost 30% for intra. Meanwhile, the increase in the memory consumption and computational complexity is marginal. For example, three tables that are used to assign VLC index to run level pairs in the current JM have been replaced by four tables with the same size. Furthermore, VLC codes are from the family of Golomb codes.

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Recommendation/Standard/Draft)

Please send to:

Q.6/16 Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Redmond WA 98052-6399, USA

Email (preferred): garysull@microsoft.com  Fax: +1 425 936 7329

This form provides the Video Coding Experts Group (VCEG) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation/Standard.  VCEG requires that all technical contributions be accompanied with this form.  Anyone with knowledge of any patent affecting the use of VCEG work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by VCEG during the progress of their work.  If a given technical proposal is not incorporated in a Recommendation/Standard, the relevant patent information will be removed from the “living list”.  The intent is that the VCEG experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to VCEG for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU-T Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the TSB Director before final approval of any Recommendation/Standard.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia |  |  |
| Mailing address | 6000 Connection Drive Irving, TX 75039 |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 214 763 1314 |  |  |
| Fax |  |  |  |
| Email | marta.karczewicz@nokia.com |  |  |
| Place and date of submission | 05/01/02 |  |  |
| Relevant Recommendation/Standard/Draft/Contribution: |  |  |  |
| Number (ex: “H.26L”) |  | JVT |  |
| Title |  | ## Adaptive Transform coefficient coding |  |
| Contribution number |  | JVT-C088 |  |

(Form continues on next page)

| ## Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of any granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above ITU-T Recommendation ¦ ISO/IEC International Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above ITU-T Recommendation ¦ ISO/IEC International Standard. Such negotiations are left to the parties concerned and are performed outside the ITU-T ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1 or 2.2 above. In this case, the following information should be provided as part of this disclosure: - patent registration/application number; - an indication of which portions of the ITU-T Recommendation ¦ ISO/IEC International Standard are affected. - a description of the patent claims covering the ITU-T Recommendation ¦ ISO/IEC International Standard; |  |  |
| In the case of box 2.1, 2.2, or 2.3 above, please provide the following: |  |  |  |
| Patent number/status |  |  |  |
| Inventor/Assignee |  |  |  |
| Relevance to VCEG |  |  |  |

(form continues on next page)

Third party patent information – please fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| ## Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation/Standard/Draft/Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - VCEG will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to VCEG |  |  |  |  |

Any other comments or remarks: