| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C093 Filename: JVT-C093.doc |
| --- | --- |

| Title: | WD2 rev. 8 with changes for FMO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger Teles AG / TU Berlin Franklinstr. 28-29 D-10587 Berlin Germany Michael Horowitz Polycom Incorporated 5000 Plaza on the Lake Austin, Texas 78746 USA | Tel: Email: Tel: Email: | +49-172-3000813 stewe@cs.tu-berlin.de +1 512.372.7091 mhorowitz@austin.polycom.com |
| Source: | Teles AG and Polycom Inc., ITU-T Sector Members |  |  |

_____________________________

# Summary

This document contains a change-bared version of JVT-B118r8 to reflect necessary modifications of the WD to support FMO, as defined in JVT-C089.  Before making those changes, the author used the “Accept All Changes” tool of WinWord.  Hence, all highlighted changes in this document are relevant to FMO.

Three main areas were identified that require changes:

- Section 3, definitions (to add the new keywords introduced in JVT-C089,

- All areas where Slices are defined (as it turns out, placeholders were deleted and generally useful text added.

- Section 9.4 (Deblocking filter), to reflect the potential unavailability of the upper and left neighbours.  Note that this change does not at all change the operation of the loopfilter – it only defines at what time the filtering process is applied.  However, this change needs careful review by those with an in-depth deblocking-filter understanding.

| Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG Geneva, Switzerland, January 29-February 1, 2002 |  | Document JVT-B118r8 File: JVT-B118r8.doc Generated: 2002-04-23 |
| --- | --- | --- |
| Title: | Working Draft Number 2, Revision 8 (WD-2 rev 8) |  |
| Status: | Approved Output Document |  |
| Contact: | Thomas Wiegand Heinrich Hertz Institute (HHI), Einsteinufer 37, D-10587 Berlin, Germany Tel: +49 - 30 - 31002 617, Fax: +49 - 030 - 392 72 00, wiegand@hhi.de Revisions 3 through 7 generated by interim substitute editor: Gary Sullivan One Microsoft Way Redmond, WA 98052 USA Tel: +1 (425) 703-5308, Fax: +1 (425) 706-7329, garysull@microsoft.com |  |
| Purpose: | Working Draft of JVT standard |  |

Ed. Notes:

The following recently-adopted features are included relative to WD-1:

–	Deblocking filter JVT-B011 (general usefulness)

–	CACM+ CABAC JVT-B036 (general usefulness with CABAC)

–	SI Frames JVT-B055 (use for streaming, random access, error recovery)

–	Intra Prediction JVT-B080 (put in software as configurable feature, use it in common conditions, seek complexity analysis)

–	Interlace frame/field switch at picture level from JVT-B071 (with field coding as the candidate baseline interlace-handling design)

–	MB partition alteration VCEG-O17 (general usefulness)

–	CABAC eff. improve JVT-B101 (general usefulness with CABAC)

–	Normative picture number update behavior from JVT-B042 (error resilience)

–	Enhanced GOP concept description in interim file format non-normative appendix JVT-B042

–	Exp-Golomb VLC JVT-B029 (general usefulness without CABAC)

–	Bitstream NAL structure with start code and emulation prevention part of JVT-B063 (use for bitstream environments)

–	Transform JVT-B038 (general usefulness)

–	Extension of quant range (general usefulness)

Additionally, the formula in the encoder test model for the determination of the Lagrange parameter was adapted by the editor to obtain a working description.

Non-editorial changes between JVT-B118r1 and JVT-B118r2:

–	Another correction of table for mapping of codewords to parameter values

–	Picture type (Ptype) extended for signaling S-frames

Revision 3 changes: The entire document was reformatted into a consistent standard template form for the creation of JVT-B118r3.  This has caused some glitches in the content, which we will need to work to correct ASAP.  Further work to achieve greater consistency is still needed.  (Hint: Look for “QQ”, “Ed. Note”, and look at equations and special symbols).

Revision 4 changes:

–	Addition of syntax tables (driven esp. by José Alvarez)

–	Deblocking filter section modifications (driven esp. by Peter List)

–	Motion compensation interpolation description modifications (driven esp. by Jani Lainema)

–	General editorial improvements (driven esp. by Jani Lainema)

–	Some steps toward "generalized ERPS" for B pictures (driven esp. by discussions on multi-frame ad hoc reflector, still needs further work) – more needed on this

–	Restructuring of the transform/inverse transform/quantisation/inverse quantisation/coefficient scan descriptions to move encoder-only issues to the non-normative annex and present the sequence of processing in decoding order rather than encoding order

Revision 5 through 7 changes:

–	Primarily provided to fix up inverse-transform and inverse quantisation part of spec.

–	Also minor fixes to intra prediction, syntax diagrams, and various minor editorial aspects.

Revision 8 changes:

–	Refinement of range limits on inverse transform and inverse quantisation

- Syntax description fix to reflect that in intra pictures, MB’s can’t be skipped

–	Encoding Motion Search Range JVT-B022 (lower complexity method not for common conditions testing)

–	Robust reference frame selection JVT-B102

DRAFT INTERNATIONAL  STANDARD

DRAFT ISO/IEC 14496-10 : 2002 (E)

DRAFT ITU-T Rec. H.264 (2002 E)

DRAFT ITU-T  RECOMMENDATION

Table OF CONTENTS

Foreword	ix

Introduction	ix

1	Scope	1

2	Normative References	1

3	Definitions	1

4	Abbreviations	6

5	Conventions	6

6	Source Coder	6

6.1	Picture Formats	6

6.2	Subdivision of a picture into macroblocks	8

6.3	Order of the bitstream within a macroblock	9

7	Syntax	10

7.1	Method of describing the syntax in graphical form	10

7.2	Syntax diagrams	11

7.2.1	Macroblock syntax diagrams	11

7.1	Method of describing the syntax in tabular form	12

7.2	Definitions of Functions and Mnemonics	14

7.3	Syntax in tabular form	15

7.3.1	Slice layer syntax	15

7.3.2	Slice header syntax	15

7.3.3	Slice video data content syntax	15

7.3.4	Coded macroblock layer syntax	16

7.3.5	Prediction data for 8x8 modes	17

7.3.6	Prediction data for 16x16 modes	18

7.3.7	Residual data syntax	19

8	Semantics	19

8.1	Organization of syntax elements (NAL concept)	19

8.1.1	Raw byte sequence payload (RBSP)	20

8.1.2	Encapsulated byte sequence payload (EBSP)	20

8.1.3	Payload type indicator byte (PTIB)	21

8.1.4	Slice mode	21

8.1.5	Data partitioning mode	21

8.2	Slice Layer	22

8.2.1	Temporal reference (TRType/TR)	22

8.2.2	Picture type (Ptype)	22

8.2.3	Picture structure (PSTRUCT)	22

8.2.4	Size information	22

8.2.5	Reference Picture ID	23

8.2.6	First MB in slice	23

8.2.7	Slice QP (SQP)	23

8.2.8	SP Slice QP	23

8.2.9	Number of  macroblocks in slice	23

8.2.10	Picture Number (PN)	23

8.2.11	Reference picture selection layer indicator (RPSLI)	23

8.2.12	Re-Mapping of Picture Numbers Indicator (RMPNI)	23

8.2.12.1	Default index order for P pictures	24

8.2.12.2	Default index order for B pictures	24

8.2.12.2.1	Forward prediction in B pictures	24

8.2.12.2.2	Backward prediction in B pictures	24

8.2.12.3	Reordering of forward and backward reference sets	25

8.2.12.1	Absolute Difference of Picture Numbers (ADPN)	25

8.2.12.2	Long-term Picture Index for Re-Mapping (LPIR)	26

8.2.13	Reference Picture Buffering Type (RPBT)	26

8.2.13.1	Memory Management Control Operation (MMCO)	26

8.2.13.2	Difference of Picture Numbers (DPN)	27

8.2.13.3	Long-term Picture Index (LPIN)	27

8.2.13.4	Maximum Long-Term Picture Index Plus 1 (MLIP1)	27

8.2.14	Supplemental Enhancement Information	27

8.3	Macroblock layer	28

8.3.1	Number of Skipped Macroblocks (mb_skip_run)	28

8.4.2	Macro block type (MB_Type)	28

8.4.2.1	Intra Macroblock modes	28

8.4.2.2	Inter Macroblock Modes	28

8.4.3	8x8 sub-partition modes	29

8.4.4	Intra Coding	29

8.4.4.1	Prediction for Intra-4x4 mode for luminance blocks	29

8.4.4.1.1	Mode 0: DC prediction	30

8.4.4.1.2	Mode 1: Vertical Prediction	30

8.4.4.1.3	Mode 2: Horizontal prediction	30

8.4.4.1.4	Mode 3: Diagonal Down/Right prediction	30

8.4.4.1.5	Mode 4: Diagonal Down/Left prediction	30

8.4.4.1.6	Mode 5: Vertical-Left prediction	30

8.4.4.1.7	Mode 6: Vertical-Right prediction	31

8.4.4.1.8	Mode 7: Horizontal-Up prediction	31

8.4.4.1.9	Mode 8: Horizontal-Down prediction	31

8.4.4.2	Coding of Intra 4x4 prediction modes	31

8.4.4.3	Intra-16x16 mode for luminance	33

8.4.4.3.1	Mode 0 (vertical)	33

8.4.4.3.2	Mode 1 (horizontal)	33

8.4.4.3.3	Mode 2 (DC prediction)	33

8.4.4.3.4	Mode 3 (Plane prediction)	33

8.4.4.4	Prediction in intra coding of chrominance blocks	34

8.4.5	Reference picture (ref_idx_fwd)	35

8.4.6	Motion Vector Data (MVD)	36

8.4.6.1	Prediction of vector components	36

8.4.6.1.1	Median prediction	37

8.4.6.1.2	Directional segmentation prediction	37

8.4.6.2	Chrominance vectors	38

8.4.7	Coded Block Pattern (CBP)	38

8.4.8	Change of Quantiser Value (Dquant)	38

9	Decoding Process	39

9.1	Slice Decoding	39

9.1.1	Multi-Picture Decoder Process	39

9.1.1.1	Decoder Process for Short/Long-term Picture Management	39

9.1.1.2	Decoder Process for Reference Picture Buffer Mapping	40

9.1.1.3	Decoder Process for Multi-Picture Motion Compensation	40

9.1.1.4	Decoder Process for Reference Picture Buffering	40

9.2	Motion compensation	41

9.2.1	Fractional sample accuracy	41

9.2.1.1	Quarter sample luminance interpolation	41

9.2.1.2	One eighth sample luminance interpolation	42

9.2.1.3 Chrominance interpolation	43

9.3	Transform Coefficient Decoding	44

9.3.1	Use of 2-dimensional model for coefficient coding.	44

9.3.2	Coefficient scanning order	44

9.3.2.1	Simple zig-zag scan	44

9.3.2.2	Double zig-zag scan	45

9.3.3	Inverse quantisation and inverse transformation	45

9.3.3.1	Luminance DC coefficients in Intra 16x16 macroblock	46

9.3.3.2	Chrominance DC coefficients	46

9.3.3.3	Residual 4x4 blocks	47

9.4	Deblocking Filter	48

9.4.1	Content dependent boundary filtering strength	49

9.4.2	Thresholds for each block boundary	50

9.4.3	Default filtering	51

9.4.4	Stronger filtering for some intra coded blocks	52

9.5	Entropy Coding	53

10	Context-based Adaptive Binary Arithmetic Coding (CABAC)	56

10.1	Overview	56

10.2	Context Modeling for Coding of Motion and Mode Information	57

10.2.1	Context Models for Macroblock Type	57

10.2.1.1	Intra Pictures	57

10.2.1.2	P and B pictures	57

10.2.2	Context Models for Motion Vector Data	58

10.2.3	Context Models for Reference Frame Parameter	58

10.3	Context Modeling for Coding of Texture Information	58

10.3.1	Context Models for Coded Block Pattern	59

10.3.2	Context Models for Intra Prediction Mode	59

10.3.3	Context Models for Run/Level and Coeff_count	59

10.3.3.1	Context-based Coding of COEFF_COUNT Information	60

10.3.3.2	Context-based Coding of RUN Information	60

10.3.3.3	Context-based Coding of LEVEL Information	61

10.4	Double Scan Always for CABAC Intra Mode	61

10.5	Context Modeling for Coding of Dquant	61

10.6	Binarization of Non-Binary Valued Symbols	62

10.6.1	Truncated Binarization for RUN, COEFF_COUNT and Intra Prediction Mode	62

10.7	Adaptive Binary Arithmetic Coding	62

11	B-pictures	63

11.1	Introduction	63

11.2	Macroblock modes and 8x8 sub-partition modes	63

11.3	B-Picture Syntax	65

11.3.1	Picture type (Ptype) ), Picture Structure (PSTRUCT) and RUN	66

11.3.2	Macroblock type (MB_type) and 8x8 sub-partition type	66

11.3.3	Intra prediction mode (Intra_pred_mode)	66

11.3.4	Reference picture parameters (ref_idx_fwd and ref_idx_bwd)	67

11.3.5	Motion vector data (MVDFW, MVDBW)	67

11.4	Decoder Process for motion vector	67

11.4.1	Differential motion vectors	67

11.4.2	Motion vectors in direct mode	67

12	SP-pictures	72

12.1	Syntax	72

12.1.1	Picture type (Ptype) and RUN	72

12.1.2	Macro block type (MB_Type)	72

12.1.3	Macroblock modes for SI-pictures	72

12.1.4	Macroblock Modes for SP-pictures	73

12.1.5	Intra Prediction Mode (Intra_pred_mode)	73

12.2	S-frame decoding	74

12.2.1	Decoding of DC values of Chrominance	75

12.2.2	Deblocking Filter	75

13	Hypothetical Reference Decoder	75

13.1	Purpose	75

13.2	Operation of the HRD	75

13.3	Decoding Time of a Picture	76

13.4	Schedule of a Bitstream	76

13.5	Containment in a Leaky Bucket	76

13.6	Bitstream Syntax	77

13.7	Minimum Buffer Size and Minimum Peak Rate	77

13.8	Encoder Considerations (informative)	78

Annex A  Byte Stream and Packet Stream Network Adaptation Layer Definitions	79

A.1	Byte Stream NAL	79

A.2	IP NAL Format	79

A.2.1	Assumptions	79

A.2.2	Combining of Partitions according to Priorities	79

A.2.3	Packet Structure	80

A.2.4	Packetization Process	80

A.2.5	De-packetization	81

A.2.6	Repair and Error Concealment	81

Annex B  Non-normative Example Encoding Methods	82

B.1	Motion Estimation and Mode Decision	82

B.1.1	Low-complexity mode	82

B.1.1.1	Finding optimum prediction mode	82

B.1.1.1.1	SA(T)D0	82

B.1.1.1.2	Block_difference	83

B.1.1.1.3	Hadamard transform	83

B.1.1.1.4	Mode decision	84

B.1.1.2	Encoding on macroblock level	84

B.1.1.2.1	Intra coding	84

B.1.1.2.2	Table for intra prediction modes to be used at the encoder side	84

B.1.1.2.3	Inter mode selection	84

B.1.1.2.4	Integer sample search	84

B.1.1.2.5	Fractional sample search	86

B.1.1.2.6	Decision between intra and inter	86

B.1.2	High-complexity mode	86

B.1.2.1	 Motion Estimation	86

B.1.2.1.1	Integer-sample search	86

B.1.2.1.2	Fractional sample search	86

B.1.2.1.3	Finding the best motion vector	86

B.1.2.1.4	Finding the best reference frame	87

B.1.2.1.5	Finding the best prediction direction for B frames	87

B.1.2.2	Mode decision	87

B.1.2.2.1	Macroblock mode decision	87

B.1.2.2.2	8x8 mode decision	88

B.1.2.2.3	INTER 16x16 mode decision	88

B.1.2.2.4	INTER 4x4 mode decision	88

B.1.2.3	Algorithm for motion estimation and mode decision	89

B.2	Transform and quantisation	90

B.2.1	Quantization tables	90

B.2.1	4x4 spatial block processing	90

B.2.2	DC luminance coefficients in 16x16 intra mode	91

B.2.3	DC chrominance coefficients	91

B.3	Elimination of single coefficients in inter macroblocks [Ed. Note: Do we still use this?]	92

B.3.1	Luminance	92

B.3.2	Chrominance	92

B.4	S-Pictures	92

B.4.1	Encoding of secondary SP-pictures	92

B.4.2	Encoding of SI-pictures	92

B.5	Encoding with Anticipation of Slice Losses	92

Annex C  Interim File Format	95

C.1	General	95

C.2	File Identification	95

C.3	Box	95

C.3.1	Definition	95

C.3.1.1	Syntax	95

C.3.1.2	Semantics	95

C.4	Box Order	96

C.5	Box Definitions	96

C.5.1	File Type Box	96

C.5.1.1	Definition	96

C.5.1.2	Syntax	96

C.5.1.3	Semantics	97

C.5.2	File Header Box	97

C.5.2.1	Definition	97

C.5.2.2	Syntax	97

C.5.2.3	Semantics	98

C.5.3	Content Info Box	98

C.5.3.1	Definition	98

C.5.3.2	Syntax	98

C.5.3.3	Semantics	99

C.5.4	Alternate Track Info Box	99

C.5.4.1	Definition	99

C.5.4.2	Syntax	100

C.5.4.3	Semantics	100

C.5.5	Parameter Set Box	100

C.5.5.1	Definition	100

C.5.5.2	Syntax	100

C.5.5.3	Semantics	101

C.5.6	Segment Box	102

C.5.6.1	Definition	102

C.5.6.2	Syntax	102

C.5.6.3	Semantics	102

C.5.7	Alternate Track Header Box	103

C.5.7.1	Definition	103

C.5.7.2	Syntax	103

C.5.7.3	Semantics	103

C.5.8	Picture Information Box	103

C.5.8.1	Definition	103

C.5.8.2	Syntax	103

C.5.8.3	Semantics	105

C.5.9	Layer Box	105

C.5.9.1	Definition	105

C.5.9.2	Syntax	105

C.5.9.3	Semantics	105

C.5.10	Sub-Sequence Box	106

C.5.10.1	Definition	106

C.5.10.2	Syntax	106

C.5.10.3	Semantics	106

C.5.11	Alternate Track Media Box	107

C.5.11.1	Definition	107

C.5.11.2	Syntax	107

C.5.12	Switch Picture Box	107

C.5.12.1	Definition	107

C.5.12.2	Syntax	107

C.5.12.3	Semantics	108

Annex D Non-Normative Error Concealment	109

D.1	Introduction	109

D.2	Intra Frame Concealment	109

D.3	Inter and SP Frame Concealment	110

D.3.1	General	110

D.3.2	Concealment using motion vector prediction	110

D.3.3	Handling of Multiple reference frames	111

D.4	B Frame Concealment	111

D.5	Handling of Entire Frame Losses	112

list of figures

Figure 6-1 – Vertical and temporal sampling positions of samples in 4:2:0 interlaced frames	8

Figure 6-2 – Subdivision of a QCIF picture into 16x16 macroblocks	9

Figure 6-3 – Numbering of the vectors for the different blocks depending on the inter mode.  For each block the horizontal component comes first followed by the vertical component	10

Figure 6-4 – Ordering of blocks for CBPY and residual coding of 4x4 blocks	10

Figure 7-1 – Syntax diagram for all the elements in the macroblock layer	21

Figure 7-2 – Identification of samples used for intra spatial prediction	27

Figure 7-3 – Intra prediction directions	28

Figure 7-4 – a) Prediction mode of block C shall be established.  A and B are adjacent blocks. b) order of intra prediction information in the bitstream [Ed. Note: Lousy Figure title]	30

Figure 7-5 – Prediction of chrominance blocks	33

Figure 7-6 - Reference picture number assignment when the current picture is the first field coded in a frame. Solid-line is for frames and dotted-line for fields. f1 stands for field 1 and f2 for field 2.  [Ed. Note: Cumbersome title]	34

Figure 7-7 - Reference picture number assignment when the current picture is the second field coded in a frame. Solid-line is for frames and dotted-line for fields. f1 stands for field 1 and f2 for field 2.  [Ed. Note: Cumbersome title]	34

Figure 7-8 -Median prediction of motion vectors	35

Figure 7-9 – Directional segmentation prediction	36

Figure 8-1 – Diagonal interpolation for 1/8 pel interpolation	41

Figure 8-2 – Simple zig-zag scan	44

Figure 8-3 – Double zig-zag scan	44

Figure 8-4 – Flow chart for determining boundary strength (Bs), for the block boundary between two neighbouring blocks j and k	48

Figure 9-1 – Generic block diagram of CABAC entropy coding scheme	54

Figure 9-2 – Neighbouring symbols A and B used for conditional coding of a current symbol C	55

Figure 9-3 – Illustration of the encoding process for a given residual motion vector component mvdk(C) of a block C: (a) Context selection rule. (b) Separation of mvdk(C) into sign and magnitude, binarization of the magnitude and assignment of context models to bin_nos.	56

Figure 9-4 – Coding scheme for transform coefficients	58

Figure 10-1 – Illustration of B picture concept	61

Figure 10-2 – Syntax diagram for B pictures [Ed. Note: Add PSTRUCT]	63

Figure 10-3 – Both the current MB in B and its collocated MB in the future reference of I or P are in frame mode. Solid-line is for frames and dot-line for fields. f1 stands for field 1 and f2 for field 2 [Ed. Note: Strange title for a Figure]	65

Figure 10-4 – Both the current MB in B and its collocated MB in the future reference of I or P are in field mode. Solid-line is for frames and dot-line for fields. f1 stands for field 1 and f2 for field 2. The forward MV of the collocated MB in field 1 of the future reference always points to one of the previously coded I or P fields.	66

Figure 10-5 – Both the current MB in B and its collocated MB in the future reference of I or P are in field mode. Solid-line is for frames and dot-line for fields. f1 stands for field 1 and f2 for field 2.  The forward MV of the collocated MB in field 2 of the future reference frame may point to field 1 of the same frame.	67

Figure 10-6 – The current MB in B is in field mode while its collocated MB in the future reference of I or P in frame mode. Solid-line is for frames and dot-line for fields. f1 stands for field 1 and f2 for field 2	68

Figure 10-7 – The current MB in B is in frame mode while its collocated MB in the future reference of I or P is in field mode. Solid-line is for frames and dot-line for fields. f1 stands for field 1 and f2 for field 2.	69

Figure 11-1 – A generic block diagram of S-picture decoder	71

Figure 12-1 – Illustration of the leaky bucket concept	73

Figure 12-2 – Illustration of the leaky bucket concept [Ed. Note: Wasn’t that the name of the last Figure?]	75

Figure B-1 – Loop for prediction mode decision	79

Figure C-1 – Relation of display window and rectangle attributes	96

Figure D-1 – MB status map at the decoder	103

Figure D-2 – Spatial concealment based on weighted pixel averaging	104

Figure D-3 – Selecting the motion vector for prediction	105

list of TAbles

Table 7-1 – Interpretation of PTIB	20

Table 7-2 – RMPNI operations for re-mapping of reference pictures	23

Table 7-3 – Memory management control operation (MMCO) values	25

Table 7-3 – Prediction mode as a function of ordering signaled in the bitstream (see text)	30

Table 7-3 (concluded)	31

Table 8-1 – QP dependent threshold parameters  and 	49

Table 8-2 - QP and Bs dependent Clipping threshold	50

Table 8-3 – UVLC code number and codewords in explicit form	51

Table 8-4 – Connection between codeword number and parameter values	51

Table 8-5 – Connection between codeword number and Intra Prediction Mode Probability	53

Table 9-1 –  Numbering of the different block types used for coding of RUN, LEVEL and COEFF_COUNT	57

Table 9-2 – Description of the context formation for the first bin for coding of COEFF_COUNT (CC denotes the COEFF_COUNT of the corresponding blocks)	58

Table 9-2 – Description of the context formation for RUNs (the variable i denotes the coefficient counter (0 ≤ i ≤COEFF_COUNT-1)	59

Table 9-3 – Binarization by means of the unary code tree	59

Table 9-4 – (a) Binarization for P-frame MB_type and (b) for B-frame MB_type	60

Table 10-1 – Macroblock modes for B-frames	61

Table 10-2 – Modes for 8x8 sub-partitions in B-frames	62

Table 10-3 – Code_number for ref_frame	Error! Bookmark not defined.

Table 11-1 – MB Type for SI-Pictures	70

Table C-1 – Box types	90

# Foreword

Foreword.

# Introduction

Introduction.

Joint Video Team (JVT)
VIDEO CODING

# 1	Scope

This document specifies ITU-T Recommendation H.264 | ISO/IEC International Standard ISO/IEC 14496-10 video coding.  The basic configuration of the algorithm is similar to H.263 and ISO/IEC 14496-2.

# 2	Normative References

Normative References.

# 3	Definitions

For the purposes of this Recommendation | International Standard, the following definitions apply.

3.1	AC coefficient: Any transform coefficient for which the frequency index in one or both dimensions is non-zero.

3.2	B-field picture: A field structure B-Picture.

3.3	B-frame picture: A frame structure B picture.

  - B picture: A predictive-coded picture; A picture that is coded in a manner in which some macroblocks may use a weighted average of two distinct motion-compensated prediction values for the prediction of the macroblock sample values.  B-pictures contain at least one B-Slice.

  - B-Slice: A Slice that contains at least one B-Macroblock

3.5	backward motion vector: A motion vector used for backward prediction.

3.6	backward prediction: Interframe prediction of the content of a B picture using a reference picture index into the backward reference set.

3.7	backward reference set: The ordered set of reference frames or fields defined for use in interframe backward prediction for a B picture.  The backward reference set is one of two ordered sets of reference pictures used by a B picture, with the other being the forward reference set.

3.8	bit rate: The rate at which the coded bitstream is delivered from the storage medium to the input of a decoder. [Ed. Note: Why assume it uses a storage medium? What about networks? Also, the bit rate could be an attribute of the stream itself, or an attribute of the way it is delivered to the decoder. A robust standard should specify the bit rate in terms of the stream itself, independently of the way it is delivered. For example, what happens if there is a non-real-time decoder? Or a non-real-time encoder, putting data on a storage medium such that the data has not even gotten to a decoder yet? This may sound like nit-picking but I really don’t think it is, there are some deep issues here.]

3.9	bitstream (stream): An ordered series of bits that forms the coded representation of the encoded data.

3.10	block: An N-row by M-column matrix of samples, or NxM transform coefficients (source, quantised, or inverse-quantised).

3.11	bottom field: One of two fields that comprise a frame. Each line of a bottom field is spatially located immediately below a corresponding line of the top field.

3.12	byte aligned: A bit in a coded bitstream is byte-aligned if its position is a multiple of 8 bits from the first bit in the stream.

3.13	byte: Sequence of 8 bits.

3.14	channel: A digital medium that stores or transports a bitstream constructed according to this Specification.

3.15	chrominance component: A matrix, block or single sample representing one of the two colour difference signals related to the primary colours in the manner defined in the bitstream. The symbols used for the chrominance signals are Cr and Cb.

3.16	chrominance format: Defines the number of chrominance blocks in a macroblock.

3.17	coded frame: A coded frame is a coded I frame, a coded P frame or a coded B frame.

3.18	coded I frame: An I-frame picture or a pair of field pictures, where the first field picture is an I picture and the second field picture is an I picture or a P picture.

3.19	coded B frame: A B-frame picture or a pair of B-field pictures.

3.20	coded order: The order in which the pictures are transmitted and decoded. This order is not necessarily the same as the display order.

3.21	coded P frame: A P-frame picture or a pair of P-field pictures.

3.19	coded picture: A coded picture is made of a picture header, the optional extensions immediately following it, and the following picture data. [Ed. Note: Questionable definition] A coded picture may be a coded frame or a coded field.

3.20	coded representation: A data element as represented in its encoded form.

3.21	coded video bitstream: A coded representation of a series of one or more pictures as defined in this Specification.

3.23	component: A matrix, block or single sample from one of the three matrices (luminance and two chrominance) that make up a picture.

3.24	compression: Reduction in the number of bits used to represent an item of data.

3.25	constant bit rate coded video: A coded video bitstream with a constant average bit rate. [Ed. Note: “Average” needs to be defined someplace in order to be truly meaningful. For video streams that can have arbitrary and upbounded duration, I don’t believe this is meaningful except in the context of a buffer model, such as a VBV (I see it is referred to below).]

3.26	constant bit rate: Operation where the bit rate is constant from start to finish of the coded bitstream. [Ed. Note: What if there is not start, nor finish? The comment above applies here too.]

3.28	data partitioning: A method for dividing a bitstream into two or more separate bitstreams for error resilience purposes. The bitstreams need to be recombined for decoding.

3.29	DC coefficient: The transform coefficient for which the frequency index is zero in both dimensions.

3.30	decoder input buffer: The first-in first-out (FIFO) buffer specified in the video buffering verifier.

3.31	decoder: An embodiment of a decoding process.

3.32	decoding (process): The process defined in this Specification that reads an input coded bitstream and produces decoded pictures or audio samples.

3.33	digital storage media (DSM):  A digital storage or transmission device or system.

3.34	display aspect ratio: The ratio height/width (in SI units) of the intended display.

3.35	display order: The order in which the decoded pictures are intended to be displayed. Normally this is the same order in which they were presented at the input of the encoder.

3.36	display process: The (non-normative) process by which reconstructed frames are displayed.

### Encapsulated byte sequence payload (EBSP).  An EBSP contains the octets of an RBSP with start code emulation prevention information.

editing: The process by which one or more coded bitstreams are manipulated to produce a new coded bitstream. Conforming edited bitstreams must meet the requirements defined in this Specification.

3.38	encoder: An embodiment of an encoding process.

3.39	encoding (process): A process, not specified in this Specification, that reads a stream of input pictures or audio samples and produces a valid coded bitstream as defined in this Specification.

3.40	fast forward playback: The process of displaying a sequence, or parts of a sequence, of pictures in display order faster than real-time.

3.41	fast reverse playback: The process of displaying the picture sequence in the reverse of display order faster than real-time.

3.42	field (structure) picture: A field structure picture is a coded picture with picture_structure is equal to “Top field” or “Bottom field”.

3.43	field period: The reciprocal of twice the frame rate.

3.44	field: A “field” is the assembly of alternate lines of a frame. Therefore an interlaced frame is composed of two fields, a top field and a bottom field.

3.45	field-based prediction: A prediction mode using only one field of the reference frame.

3.46	flag: A variable which can take one of only the two values defined in this Specification.

3.47	forbidden: The term “forbidden” when used in the clauses defining the coded bitstream indicates that the value shall never be used. This is usually to avoid emulation of start codes. [Ed. Note: Since we will not have start code emulation problems in JVT video, this definition may be either unnecessary or incorrect.]

3.48	forward motion vector: A motion vector used for forward prediction.

3.49	forward prediction: Interframe prediction of the content of a P, B, or SP picture using a reference picture index into the forward reference set.  All interframe prediction used for P and SP pictures is considered forward prediction.  The forward reference set is one of two ordered sets of reference pictures used by a B picture, with the other being the backward reference set.

3.50	forward reference set: The ordered set of reference frames or fields defined for use in interframe forward prediction for a P, B, or SP picture.

3.51	forward transform: A part of a (non-normative) example encoding process by which blocks of spatial-domain samples are converted into frequency-domain transform coefficients for subsequent quantisation.

3.52	frame: A frame contains the lines of spatial information of a video signal. For progressive video, these lines contain samples starting from one time instant and continuing through successive lines to the bottom of the frame. For interlaced video a frame consists of two fields, a top field and a bottom field. One of these fields will commence one field period later than the other.

3.53	frame (structure) picture: A frame structure picture is a coded picture with picture_structure is equal to “Frame”.

3.54	frame period: The reciprocal of the frame rate.

3.55	frame rate: The rate at which frames are to be output from the decoding process.

3.56	frame reordering: The process of reordering the reconstructed frames when the coded order is different from the display order.

3.57	frame-based prediction: A prediction mode using both fields of the reference frame.

3.58	future reference frame (field): A future reference frame (field) is a reference frame (field) that occurs at a later time than the current picture in display order.

3.59	group of pictures: tbd (to be defined ;o)

3.60	header: A block of data in the coded bitstream containing the coded representation of a number of data elements pertaining to the coded data that follow the header in the bitstream.

3.61	I-field picture: A field structure I-Picture.

3.62	I-frame picture: A frame structure I picture.

3.63	interlace: The property of conventional television frames where alternating lines of the frame represent different instances in time. In an interlaced frame, one of the fields is meant to be displayed first. This field is called the first field. The first field can be the top field or the bottom field of the frame.

3.64	intra coding: Coding of a macroblock or picture that uses information only from that macroblock or picture.

3.65	I picture, intra-coded picture: A picture coded using information only from itself.

3.66	inverse quantisation: The process of rescaling the quantised transform coefficients after their representation in the bitstream has been decoded.

3.67	inverse transform: A part of the decoding process by which a block of inverse-quantised transform coefficient values is converted into a block of spatial-domain samples.

3.68	level: A defined set of constraints on the values which may be taken by the parameters of this Specification within a particular profile. A profile may contain one or more levels. In a different context, level is the absolute value of a non-zero coefficient (see “run”).

3.69	luminance component: A matrix, block or single sample representing a monochrome representation of the signal and related to the primary colours in the manner defined in the bitstream. The symbol used for luminance is Y.

  - macroblock: The four 8x8 blocks of luminance data and the two (for 4:2:0 chrominance format), four (for 4:2:2 chrominance format) or eight (for 4:4:4 chrominance format) corresponding 8x8 blocks of chrominance data coming from a 16x16 section of the luminance component of the picture. [Ed. Note: Some of this not needed if support only 4:2:0] Macroblock is sometimes used to refer to the sample data and sometimes to the coded representation of the sample values and other data elements defined in the macroblock header of the syntax defined in this part of this Specification.

  - Macroblock Allocation Map (mba_map): a one dimensional array of integers with one entry for each macroblock in raster scan order.  Each entry identifies the slice_group_id of the corresponding macroblock.  The mba_map is part of each parameter_set.

3.71	Mbit: 1 000 000 bits.

3.75	motion compensation: The use of motion vectors to improve the efficiency of the prediction of sample values. The prediction uses motion vectors to provide offsets into the past and/or future reference frames or reference fields containing previously decoded sample values that are used to form the prediction error.

3.76	motion estimation: The process of estimating motion vectors during the encoding process.

  - motion vector: A two-dimensional vector used for motion compensation that provides an offset from the coordinate position in the current picture or field to the coordinates in a reference frame or reference field.

  - NAL: Network Adaptation Layer

  - NALU: Network Adaptation Layer Unit, a byte buffer the NAL operates on.  A NALU consists of one octed carrying the NALU type, and an EBSP octet string carrying the payload of the NALU, and the length of the EBSP in bytes.

3.78	non-intra coding: Coding of a macroblock or picture that uses information both from itself and from macroblocks and pictures occurring at other times.

3.79	opposite parity: The opposite parity of top is bottom, and vice versa.

3.80	parameter: A variable within the syntax of this Specification which may take one of a range of values. A variable which can take one of only two values is called a flag.

3.81	parity (of field): The parity of a field can be top or bottom.

3.82	past reference frame (field): A past reference frame(field) is a reference frame(field) that occurs at an earlier time than the current picture in display order.

3.83	P-field picture: A field structure P picture.

3.84	P-frame picture: A frame structure P picture.

3.87	P picture, predictive-coded picture: A picture that is coded using motion compensated prediction from previously-decoded reference fields or frames, using at most one motion vector and reference picture to predict the value of each individual region.

3.85	picture data: In the VBV operations, picture data is defined as all the bits of the coded picture, all the header(s) and user data immediately preceding it if any (including any stuffing between them) and all the stuffing following it, up to (but not including) the next start code, except in the case where the next start code is an end of sequence code, in which case it is included in the picture data. [Ed. Note: Needs refinement.  We don’t have start codes in all environments.]

3.86	picture: Source, coded or reconstructed image data. A source or reconstructed picture consists of three rectangular matrices of 8-bit numbers representing the luminance and two chrominance signals. A “coded picture” is defined in 3.21. For progressive video, a picture is identical to a frame, while for interlaced video, a picture can refer to a frame, or the top field or the bottom field of the frame depending on the context.

3.88	prediction error: The difference between the actual value of a sample or data element and its predictor.

3.89	prediction: The use of a predictor to provide an estimate of the sample value or data element currently being decoded.

3.90	predictor: A linear combination of previously decoded sample values or data elements.

3.91	profile: A defined subset of the syntax of this Specification. [Ed. Note: Definition could perhaps be improved.  A profile often does not define the syntax, but rather constrains the value of elements of the syntax (such as “OBMC_disable must be TRUE”).]

3.92	progressive: The property of video frames that indicates that all of the samples of the frame represent the same instant in time.

3.93	quantised transform coefficients: Transform coefficients before inverse quantisation and inverse transform. A lossless representation of the quantised transform coefficients is contained in the video bitstream. [Ed. Note: Do we have a bitstream?]

3.94	quantiser scale: A scale factor coded in the bitstream and used by the decoding process to scale the inverse quantisation.

3.95	random access: The process of beginning to read and decode the coded bitstream at an arbitrary point.

3.96	raster scan order: A mapping of a regular rectangular two-dimensional pattern to a one-dimensional scanning order such that the first entries in the scan order are from the first row of the pattern scanned from left to right, followed similarly by the second, third, etc. rows of the pattern each scanned from left to right.

3.97	reconstructed frame: A reconstructed frame consists of three rectangular matrices of 8-bit numbers representing the luminance and two chrominance signals. A reconstructed frame is obtained by decoding a coded frame.

3.98	reconstructed picture: A reconstructed picture is obtained by decoding a coded picture. A reconstructed picture is either a reconstructed frame (when decoding a frame picture), or one field of a reconstructed frame (when decoding a field picture). If the coded picture is a field picture, then the reconstructed picture is the top field or the bottom field of the reconstructed frame.

3.99	reference field: A reference field is one field of a reconstructed frame. Reference fields are used for forward and backward prediction when P-pictures and B-pictures are decoded. Note that when field P-pictures are decoded, prediction of the second field P picture of a coded frame uses the first reconstructed field of the same coded frame as a reference field.

3.100	reference frame: A reference frame is a reconstructed frame that was coded in the form of a coded I frame or a coded P frame [Ed. Note: Not an adequate definition]. Reference frames are used for forward and backward prediction when P-pictures and B-pictures are decoded.

3.101	reordering delay: A delay in the decoding process that is caused by frame reordering in order to display pictures in a different order than the order specified for the decoding process.

3.102	reserved: The term “reserved” when used in the clauses defining the coded bitstream indicates that some values of a particular syntax element may be used in extensions of this Specification by ITU-T | ISO/IEC, and that these values shall not be used unless so specified.

3.103	run: The number of zero coefficients preceding a non-zero coefficient, in the scan order. The absolute value of the non-zero coefficient is called “level”.

3.104	sample aspect ratio: (abbreviated to SAR). This specifies the distance between samples. It is defined (for the purposes of this Specification) as the vertical displacement of the lines of luminance samples in a frame divided by the horizontal displacement of the luminance samples. Thus its units are (metres per line) ÷ (metres per sample).

3.105	saturation: Limiting a value that exceeds a defined range by setting its value to the maximum or minimum of the range as appropriate.

3.106	side information: Information in the bitstream necessary for controlling the decoder. 16x8 prediction — A prediction mode similar to field-based prediction but where the predicted block size is 16x8 luminance samples. [Ed Note: What?]

3.107	skipped macroblock: A macroblock for which no data is encoded other than an indication that the macroblock is to be decoded as "skipped".

  - slice: One or more macroblocks belonging to the same slice_group.  Within the slice_group the macroblocs are ordered in raster scan order.  The term slice is used both for the reproduced (pixel-) representation and for the coded (bitstream) representation of the macroblocks.

  - slice_group: One or more uncoded macroblocks, not necessarily in raster scan order, that share the same slice_group_id in the mba_map.

  - slice_group_id: an integer in the range of 0 to 7 that identifies the slice group of a macroblock.

3.109	Source (input): Term used to describe the video material or some of its attributes before encoding.

3.110	spatial prediction: A prediction derived from the content of the current decoded frame.

3.111	start codes [system and video]: 32-bit codes embedded in that coded bitstream that are unique. They are used for several purposes including identifying some of the structures in the coding syntax. [Ed. Note: Only in certain NALs, and possibly different in each.]

3.112	stuffing (bits, bytes): Code-words that may be inserted into the coded bitstream that are discarded in the decoding process. One purpose of stuffing is to increase the bit rate of the stream which would otherwise be lower than the desired bit rate.  Another purpose is to align the data to specific boundaries such as byte or system packet boundaries.

3.113	temporal prediction: prediction derived from pictures other than the current decoded picture.

3.114	top field: One of two fields that comprise a frame. Each line of a top field is spatially located immediately above the corresponding line of the bottom field.

3.115	transform coefficient: A scalar quantity considered to be in a frequency domain that is associated with a particular two-dimensional frequency index in the inverse transform part of the decoding process.

3.116	transform: Either a forward transform or an inverse transform.

3.117	variable bit rate: Variable bit rate is an attribute of a stream, such that the stream’s bit rate varies with time. The bit rate is defined above.

3.118	variable length coding (VLC): A reversible procedure for coding that assigns shorter code-words to frequent events and longer code-words to less frequent events.

3.119	video buffering verifier (VBV): A hypothetical decoder that is conceptually connected to the output of the encoder. Its purpose is to provide a constraint on the variability of the data rate that an encoder or editing process may produce. [Ed. Note: May need to refine the statement of the purpose of the VBV.]

3.120	video sequence: The highest syntactic structure of coded video bitstreams. It contains a series of one or more coded frames.

3.121	XYZ profile decoder: decoder able to decode bitstreams conforming to the specifications of the XYZ profile (with XYZ being any of the defined Profile names).

3.122	zig-zag scanning order: A specific sequential ordering of transform coefficients from (approximately) the lowest spatial frequency to the highest.

# 4	Abbreviations

# 5	Conventions

The following mathematical and logical operators are defined as follows

a >> b:	Arithmetic right shift of a two's complement integer representation of a by b binary digits

a << b:	Arithmetic left shift of a two's complement integer representation of a by b binary digits

If b is negative, a << b is interpreted as a >> (–b)

a % b:	Remainder of a divided by b, defined only for a and b both positive integers

a && b:	Boolean logical "and" of a and b

a || b:	Boolean logical "or" of a and b

The following mathematical functions are defined as follows

clip3( a, b, c) = a if c<a, b if c>b, otherwise c	(5-1)

clip1( c ) = clip3( 0, 255, c )		(5-2)

# 6	Source Coder

## 6.1	Picture Formats

The image width and height of the decoded luminance memory arrays are multiples of 16. Decoder output picture sizes that are not a multiple of 16 in width or height can be specified using a cropping rectangle. At the moment only colour sequences using 4:2:0 chrominance sub-sampling are supported.

This specification describes coding of video that contains either progressive or interlaced frames, which may be mixed together in the same sequence.  The vertical and horizontal locations of luminance and chrominance samples in progressive frames are shown in Figure 6-1.

Figure 6-1 – Vertical and horizontal locations of 4:2:0 luminance and chrominance samples in progressive scan pictures

A frame of video contains two fields, the top field and the bottom field, which are interleaved. The first (i.e., top), third, fifth, etc. lines of a frame are the top field lines. The second, fourth, sixth, etc. lines of a frame are the bottom field lines. A top field picture consists of only the top field lines of a frame. A bottom field picture consists of only the bottom field lines of a frame.

The two fields of an interlaced frame are separated in time by a field period (which is half the time of a frame period). They may be coded separately as two field pictures or together as a frame picture. A progressive frame should always be coded as a single frame picture. However, a progressive frame is still considered to consist of two fields (at the same instant in time) so that other field pictures may reference the sub-fields of the frame.

The vertical and temporal sampling positions of samples in interlaced frames are shown in Figure 6-2. The vertical sampling positions of the chrominance samples in a top field of an interlaced frame are specified as shifted up by 1/4 luminance sample height relative to the field-sampling grid in order for these samples to align vertically to the usual position relative to the full-frame sampling grid. The vertical sampling positions of the chrominance samples in a bottom field of an interlaced frame are specified as shifted down by 1/4 luminance sample height relative to the field-sampling grid in order for these samples to align vertically to the usual position relative to the full-frame sampling grid. The horizontal sampling positions of the chrominance samples are specified as unaffected by the application of interlaced field coding.

Figure 6-2 – Vertical and temporal sampling positions of samples in 4:2:0 interlaced frames

## 6.2	Subdivision of a picture into slices and macroblocks

Pictures (both frame and field) are divided into macroblocks of 16x16 luminance samples each, with two associated 8x8 chrominance samples. For instance, a QCIF picture is divided into 99 macroblocks as indicated in Figure 6-3.

Each Macroblock in a picture belongs to exactly one slice.  The minimum number of slices for a picture is 1, and the maximum number of slices of a picture is the number of macroblocks in the picture (for example 99 in QCIF picture).

When Data Partitioning is not used, coded slices start with a slice header and are followed by the entropy coded symbols of the macroblock data for all the macroblocks of the slice.  The NAL makes them available to the VCL in the form of a Single Slice NALU (SSU).  When Data Partitioning is used, the macroblock data of a Slice is partitioned in up to three partitions, containing header, information, intra CBP and coeffiecients, and inter CPB and coefficients, respectively.

The order of the macroblocks in the payload of the NALU bitstream depends on the Macroblock Allocation Map and is not necessarily raster scan order.

### The Macroblock Allocation Map (mba_map)

The mba_map contains the slice_group_id of every macroblock in a picture.  It is part of the Parameter Set that is referenced by the parameter_set_id in the slice header.  All uncoded macroblocks that share the same slice_group_id are called a slice group.  Hence, an mba_map allocates every macroblock to exactly one slice group.  Each slice group is represented by one or more slices in the bitstream.  Within each slice group,  macroblocks are coded in raster scan order.

FMO Edt.: It may be adivisable to illustrate the concept of slices and FMO-slices in one or more figures

Figure 6-3 – Subdivision of a QCIF picture into 16x16 macroblocks

## 6.3	Order of the bitstream within a macroblock

Figures 6-4 and 6-5 indicate how a macroblock or 8x8 sub-block is divided and the order of the different syntax elements resulting from coding a macroblock.

Figure 6-4 – Numbering of the vectors for the different blocks depending on the inter mode.  For each block the horizontal component comes first followed by the vertical component (raster scan order)

Figure 6-5 – Ordering of blocks for CBPY and residual coding of 4x4 blocks

# 7	Syntax

## 7.1	Method of describing the syntax in graphical form

## 7.2	Syntax diagrams

### 7.2.1	Macroblock syntax diagrams

[Ed. Note: RUN should not be shown as part of the same macroblock layer as the remaining elements of this syntax diagram, as RUN may be the last element of a slice.]

Figure 7-1 – Syntax diagram for all the elements in the macroblock layer

## 7.1	Method of describing the syntax in tabular form

The syntax is described in a similar manner as the syntax in ITU-T Recommendation H.262 | ISO/IEC 13818-2. It closely follows the C-language syntactic constructs. Data items in the bitstream are represented in bold type. Each data item is described by its name, its length in bits and an indication of type and order of transmission. A decoder behaves according to the value of the decoded data element in the bitstream and on the values of previously decoded data elements.

The syntax tables describe a superset of all correct and error-free input bitstreams. An actual decoder must implement correct means for identifying the beginning of the bitstream for proper decoding and to identify and handle errors in the bitstream. The methods for identifying and handling errors and other such situations are not described here.

Following C-language conventions, a value of '0' represents a FALSE condition in a test statement. The value TRUE is usually represented by '1', but any other value different than zero is understood as TRUE.

The following table lists examples of pseudo code used to describe the syntax. When data_element appears, it indicates that a data element is read (extracted) from the bitstream and the bitstream pointer advances to the bit following the last bit of the data element extracted.

|  | Category | Mnemonic |
| --- | --- | --- |
| /* A statement can be a data element with an associated syntax category and mnemonic or can be an expression used to specify conditions for the type and quantity of data elements, as in the following two examples */ |  |  |
| data_element | 3 | ecselbf |
| conditioning statement |  |  |
| /* A group of statements enclosed in curly brackets is a compound statement and is treated functionally as a single statement. */ |  |  |
| { |  |  |
| statement |  |  |
| statement |  |  |
| … |  |  |
| } |  |  |
| /* A “while” structure indicates a test of whether a condition is true, and if true, indicates evaluation of a statement (or compound statement) repeatedly until the condition is no longer true */ |  |  |
| while (condition) |  |  |
| statement |  |  |
| /* A “do … while” structure indicates evaluation of a statement once, followed by a test of whether a condition is true, and if true, indicates repeated evaluation of the statement until the condition is no longer true */ |  |  |
| do |  |  |
| statement |  |  |
| while (condition) |  |  |
| /* An “if … else” structure indicates a test of whether a condition is true, and if the condition is true, indicates evaluation of a primary statement, otherwise indicates evaluation of an alternative statement. The “else” part of the structure and the associated alternative statement is omitted if no alternative statement evaluation is needed */ |  |  |
| if (condition) |  |  |
| primary statement |  |  |
| else |  |  |
| alternative statement |  |  |
| /* A “for” structure indicates evaluation of an initial statement, followed by a test of a condition, and if the condition is true, indicates repeated evaluation of a primary statement followed by a subsequent statement until the condition is no longer true. */ |  |  |
| for (initial statement; condition; subsequent statement) |  |  |
| primary statement |  |  |

## 7.2	Definitions of Functions and Mnemonics

The functions presented here are used to better understand the behaviour of a compliant decoder. These functions assume the existence of a bitstream pointer with an indication of the position of the next bit to be read by the decoder from the bitstream.

bytealigned()

- Returns TRUE if the current position in the bitstream is on a byte boundary, i.e., the next bit in the bitstream is the first bit in a byte. Otherwise it returns FALSE

nextbits()

- Provides the next bits in the bitstream for comparison purposes, without advancing the bitstream pointer. Provides a look at the next token in the bitstream.

more_data_in_slice()

- Returns TRUE is there is more data in a slice.  Otherwise it returns FALSE.  The method for determining whether there is more data in the slices is QQ.

Presently only a few mnemonics are used to describe the syntax.

- bslbf: bit string written (from left to right) with the left bit first

- uimsbf(n): unsigned integer using n bits with most significant bit written first

- ecselbf: entropy-coded syntax element with the left bit first

When CABAC entropy coding is applied, these mnemonics do not apply to some categories of syntax elements.

## 7.3	Syntax in tabular form

### 7.3.1	Slice layer syntax

| slice_layer( ) { | Category | Mnemonic |
| --- | --- | --- |
| slice_header() | 4 |  |
| slice_video_data_content() |  |  |
| } |  |  |

### 7.3.2	Slice header syntax

| slice_header( ) { | Category | Mnemonic |
| --- | --- | --- |
| qq |  |  |
| } |  |  |

### 7.3.3	Slice video data content syntax

| slice_video_data_content( ) { | Category | Mnemonic |
| --- | --- | --- |
| do { |  |  |
| if (PicType != Intra) |  |  |
| mb_skip_run | 4 | ecselbf |
| if ( more_data_in_slice() ) |  |  |
| coded_macroblock_layer() |  |  |
| } while ( more_data_in_slice() ) |  |  |
| } |  |  |

### 7.3.4	Coded macroblock layer syntax

| coded_macroblock_layer( ) { | Category | Mnemonic |
| --- | --- | --- |
| mb_type | 4 | ecselbf |
| if ( mb_type = = MB8x8refX ¦ ¦ mb_type = = MB8x8ref0 ) |  |  |
| for( i=0; i<4; i++ ) |  |  |
| prediction8x8( mb_type ) |  |  |
| else |  |  |
| prediction16x16( mb_type ) |  |  |
| if ( mb_type != MBintra16x16 ) |  |  |
| cbp | 4 | ecselbf |
| if ( cbp ¦¦ (mb_type = = MBintra16x16)) { |  |  |
| delta_quant | 4 | ecselbf |
| residual( ) |  |  |
| } |  |  |
| } |  |  |

### 7.3.5	Prediction data for 8x8 modes

| prediction8x8( mb_type ) { | Category | Mnemonic |
| --- | --- | --- |
| region8x8_type | 4 | ecselbf |
| if ( region8x8_type = = BlockIntra4x4 ) { |  |  |
| for ( i4x4=0; i4x4<4; i4x4++ ) |  |  |
| intra_pred_mode | 4 | ecselbf |
| } else { |  |  |
| if ( multiple_fwd_reference_pictures() && mb_type ! = MB8x8ref0 ) |  |  |
| if ( two_picture_fwd_prediction() ) |  |  |
| ref_idx_fwd | 4 | uimsbf(1) |
| else |  |  |
| ref_idx_fwd | 4 | ecselbf |
| if ( multiple_bwd_ref_pictures() && ( region8x8_type = = this_qq ¦ ¦ region8x8_type = = that_qq ) ) |  |  |
| if ( two_picture_bwd_prediction() ) |  |  |
| ref_idx_bwd | 4 | uimsbf(1) |
| else |  |  |
| ref_idx_bwd | 4 | ecselbf |
| for ( i=0; i<number_region8x8_mvectors[region8x8_type]; i++ ) { |  |  |
| if ( block_type = = this_qq ¦ ¦ block_type = = that_qq ) |  |  |
| for ( j=0; j<2; j++ ) |  |  |
| mvd_fwd[i][j] | 4 | ecselbf |
| if ( block_type = = this_qq ¦ ¦ block_type = = that_qq ) |  |  |
| for ( j=0; j<2; j++ ) |  |  |
| mvd_bwd[i][j] | 4 | ecselbf |
| } |  |  |
| } |  |  |
| } |  |  |

### 7.3.6	Prediction data for 16x16 modes

| prediction16x16( mb_type ) { | Category | Mnemonic |
| --- | --- | --- |
| if ( mb_type = = MBintra4x4 ) |  |  |
| for ( i4x4=0; i4x4<16; i4x4++ ) |  |  |
| intra_pred_mode | 4 | ecselbf |
| else if ( mb_type != MBintra16x16 ) { |  |  |
| if ( multiple_fwd_ref_pictures() && ( mb_type = = MBfwd ¦ ¦ mb_type = = MBbidir ) ) { |  |  |
| if ( two_picture_fwd_prediction() ) |  |  |
| ref_idx_fwd | 4 | uimsbf(1) |
| else |  |  |
| ref_idx_fwd | 4 | ecselbf |
| if ( multiple_bwd_ref_pictures() && ( mb_type = = MBbwd ¦ ¦ mb_type = = MBbidir ) ) |  |  |
| if ( two_picture_bwd_prediction() ) |  |  |
| ref_idx_bwd | 4 | uimsbf(1) |
| else |  |  |
| ref_idx_bwd | 4 | ecelbf |
| } |  |  |
| for(i=0; i<number_mb_mvectors[mb_type]; i++) { |  |  |
| if ( mb_type = = MBfwd ¦ ¦ mb_type = = MBbidir ) |  |  |
| for(j=0; j<2; j++) |  |  |
| mvd_fwd[j] | 4 | ecelbf |
| if ( mb_type = = MBbwd ¦ ¦ mb_type = = MBbidir ) |  |  |
| for(j=0; j<2; j++) |  |  |
| mvd_bwd[j] | 4 | ecelbf |
| } |  |  |
| } |  |  |

### 7.3.7	Residual data syntax

| residual( mb_type ) { | Category | Mnemonic |
| --- | --- | --- |
| if ( mb_type = = MBintra16x16 ) |  |  |
| do |  |  |
| luma_dc_zigzag_token | 5 ¦ 6 | ecselbf |
| while ( lum_dc_zigzag_token != EOB ) |  |  |
| for ( i8x8=0; i8x8<4; i8x8++ ) /* each luma 8x8 region */ |  |  |
| for (i4x4=0; i4x4<4; i4x4++) /* each 4x4 sub-block of region */ |  |  |
| if (cbp & (1<< i8x8) |  |  |
| if ( double_scan() ) { |  |  |
| do |  |  |
| luma_dblscan0_token | 5 ¦ 6 | ecselbf |
| while ( luma_dblscan0_token != EOB ) |  |  |
| do |  |  |
| luma_dblscan1_token | 5 ¦ 6 | ecselbf |
| while ( luma_dblscan1_token != EOB ) |  |  |
| } else |  |  |
| do |  |  |
| luma_zigzag_token | 5 ¦ 6 | ecselbf |
| while ( luma_zigzag_token != EOB ) |  |  |
| if (cbp & 0x30) { /* chroma DC residual coded */ |  |  |
| for ( iCbCr=0; iCbCr<2; iCbCr++) { |  |  |
| do |  |  |
| chroma_2x2_dc_raster_token | 5 ¦ 6 | ecselbf |
| while ( chroma_2x2_dc_raster_token != EOB ) |  |  |
| if (cbp & 0x20) { /* chroma AC residual coded */ |  |  |
| for ( iCbCr=0; iCbCr<2; iCbCr++ ) |  |  |
| for ( i4x4=0; i4x4<4; i4x4++) |  |  |
| do /* [Ed. Note: Does chroma ever use doublescan?] |  |  |
| chroma_ac_zigzag_token | 5 ¦ 6 | ecselbf |
| while ( chroma_ac_zigzag_token != EOB ) |  |  |
| } |  |  |
| } |  |  |

# 8	Semantics

## 8.1	Organization of syntax elements (NAL concept)

The Video Coding Layer (VCL) is defined to efficiently represent the content of the video data, and the Network Adaptation Layer (NAL) is defined to format that data and provide header information in a manner appropriate for conveyance by the higher level system.  The data is organized into data packets, each of which contains an integer number of bytes.  These data packets are then transmitted in a manner defined by the NAL.

Any sequence of bits can be formatted into a sequence of bytes in a manner defined as a Raw Byte Sequence Payload (RBSP), and any RBSP can be encapsulated in a manner that prevents emulation of byte stream start code prefixes in a manner defined defined as an encapsulated byte sequence payload (EBSP).  These two formats are described below in this section.  An NAL carries EBSP’s in a manner customized to some particular network environment. [Ed. Note: The last sentence added to indicate that emulation prevention applies to all NALs (so that emulation prevention will never need to be applied as part of a cross-network NAL translator), which appears to be the current consensus.]

### 8.1.1	Raw byte sequence payload (RBSP)

A raw byte sequence payload (RBSP) is defined as an ordered sequence of bytes that contains a string of data bits (SODB).  A SODB is defined as an ordered sequence of bits, in which the left-most bit is considered to be the first and most significant bit (MSB) and the right-most bit is considered to be the last and least significant bit (LSB).  The RBSP contains the SODB in the following form:

a)	If the SODB is null, the RBSP is also null.

b)	Otherwise, the RBSP shall contain the SODB in the following form:

1)	The first byte of the RBSP shall contain the (most significant, left-most) eight bits of the SODB; the next byte of the RBSP shall contain the next eight bits of the SODB, etc.; until fewer than eight bits of the SODB remain.

2)	The final byte of the RBSP shall have the following form:

i)	The first (most significant, left-most) bits of the final RBSP byte shall contain the remaining bits of the SODB, if any,

ii)	The next bit of the final RBSP byte shall consist of a single packet stop bit (PSB) having the value one ('1'), and

iii)	Any remaining bits of the final RBSP byte, if any, shall consist of byte-alignment stuffing bits (BASB’s) having the value zero ('0').

Note that the last byte of a RBSP can never have the zero (0x00), because it contains the PSB.

If the boundaries of the RBSP are known, the decoder can extract the SODB from the RBSP by concatenating the bits of the bytes of the RBSP and discarding the PSB, which is the last (least significant, right-most) bit having the value one ('1'), and discarding any following (less significant, farther to the right) bits that follow it, which have the value zero ('0').

The method of determining the boundaries of the RBSP depend on the NAL.

### 8.1.2	Encapsulated byte sequence payload (EBSP)

A encapsulated byte sequence payload (EBSP) is defined as an ordered sequence of bytes that contains the raw byte sequence payload (RBSP) in the following form:

a)	If the RBSP contains fewer than three bytes, the EBSP shall be the same as the RBSP.

b)	Otherwise, the EBSP shall contain the RBSP in the following form:

1)	The first byte of the EBSP shall contain the first byte of the RBSP,

2)	The second byte of the EBSP shall contain the second byte of the RBSP,

3)	Corresponding to each subsequent byte of the RBSP, the EBSP shall contain one or two subsequent bytes as follows:

i)	If the last two previous bytes of the EBSP are both equal to zero (0x00) and if the next byte of the RBSP is either equal to one (0x01) or equal to 255 (0xFF), the EBSP shall contain two bytes of data that correspond to the next byte of the RBSP.  The first of these two bytes shall be an emulation prevention byte (EPB) equal to 255 (0xFF), and the second of these two bytes shall be equal to the next byte of the RBSP.

ii)	Otherwise, the EBSP shall contain one next byte of data corresponding to the next byte of the RBSP.  This byte shall be equal to the next byte of the RBSP.

The format of the EBSP prevents the three-byte start code prefix (SCP) equal to 0x00, 0x00, 0x01 from occurring within an EBSP.  A decoder shall extract the RBSP from the EBSP by removing and discarding each byte having the value 255 (0xFF) which follows two bytes having the value zero (0x00) within an EBSP.  The means for determining the boundaries of an EBSP is specified by the NAL.

Note that the effect of concatenating two or more RBSP’s and then encapsulating them into an EBSP is the same as first encapsulating each individual RBPS and then concatenating the result (because the last byte of an RBSP is never zero).  This allows the association of individual EBSP’s to video data packets to depend on the NAL without affecting the content of the EBSP.  For example, high-level header data can be placed in its own EBSP and this same EBSP content can be carried in several alternative ways, depending on the particular NAL design in use, including the possibilities of:

a)	Sending it within a low-level video packet with associated lower-level data for support of independent packet decoding, or

b)	Sending it in a separate higher-level header packet, or

c)	Sending it in an external “out-of-band” reliable channel.

### 8.1.3	Payload type indicator byte (PTIB)

The type of data carried in a data packet is indicated by a payload type indicator byte (PTIB).  Table 8-1 contains the values defined for the PTIB.

Table 8-1 – Interpretation of PTIB

| Value | Re-mapping Specified |
| --- | --- |
| 0x01 | Sequence Data: Payload contains configuration information for sequence |
| 0x02 | Sequence SEI: Payload contains SEI for sequence |
| 0x03 | Random Access Point Data: Payload TBD |
| 0x04 | Random Access Point SEI: Payload contains SEI for random access point |
| 0x05 | Picture Data: Null payload |
| 0x06 | Picture SEI: Payload contains SEI for picture |
| 0x07 | Non-Partitioned Slice Data: Payload is one EBSP containing slice header data and non-data-partitioned video data for the slice |
| 0x05 | Slice Mode and MV Data: Payload is one EBSP containing slice header data and all other non-coefficient data for the slice |
| 0x06 | Slice Intra Coefficient Data: Payload is one EBSP containing slice header data and all intra coefficients for the slice |
| 0x07 | Slice Inter Coefficient Data: Payload is one EBSP containing slice header data and all non-intra coefficients for the slice |
| 0x08 | End of Sequence Data: Null payload |
| 0x09 | End of Sequence SEI: Payload contains SEI for end of sequence |

[Ed. Note: Further work needed to finalize format of PTIB and define the types of data to follow.]

### Data partitioning mode

[Ed. Note: StW: this is the data partitioning as accepted into TML2.  The version accepted in TML5 or 6 (Austin meeting) needs to be added.  Low priority because DP is not in a profile]

Data partitioning re-arranges the symbols in a way that all symbols of one data type (e.g. DC coefficients, macroblock headers, motion vectors) that belong to a single slice are collected in one VLC coded bitstream that starts byte aligned. Decoders can process such a partitioned data streams by fetching symbols from the correct partition.  The partition to fetch from is determined through the decoder’s state machine, according to the syntax diagram discussed in subclause QQ.

Data Partitioning is implemented by concatenating all VLC coded symbols of one data type and one slice (or full picture if slices are not used).  At the moment, for a few partitions as indicated below contain data of more than one data type that are so closely related that a finer diversion seems to be fruitless.  The following data types are currently defined:

[Ed. Note: This needs work.  Also reformat the below into a table if this content to be preserved]

0	TYPE_HEADER 		Picture or Slice Headers (Note 1)

1	TYPE_MBHEADER 	Macroblock header information (Note 2)

2	TYPE_MVD		Motion Vector Data

3	TYPE_CBP		Coded Block Patterm

4	TYPE_2x2DC		2x2 DC Coefficients

5	TYPE_COEFF_Y		Luminance AC Coefficients

6	TYPE_COEFF_C		Chrominance AC Coefficients

7	TYPE_EOS		End-of-Stream Symbol

NOTE 1 – TYPE_HEADER encompasses all Picture/Slice header information

NOTE 2 – TYPE_MBHEADER encompasses The MB-Type, Intra-Prediction mode and Reference Frame ID.

## 8.2	Slice Layer

[Ed. Note: The slice layer needs a significant amount of work together with the NAL concept. The Picture sync and picture type codewords are present to ease the VCL development.]

The header information sent at the slice layer is coded depending on the NAL type. The coding, the order and even the presence of some header fields may differ between the different NALs. [Ed. Note: Revisit these two statements as the NAL specification matures.]

### 8.2.1	Temporal reference (TRType/TR)

[Ed. Note: Clarify relation to NAL, parameter sets, etc.]  Two codewords. Temporal reference type (TRType) indicates how TR is transmitted

TRType = 0:		Absolute TR

TRType <> 0: 	Error

The value of TR is formed by incrementing its value in the temporally-previous reference picture header by one plus the number of skipped or non-reference pictures at the picture clock frequency since the previously transmitted one.

### 8.2.2	Picture type (Ptype)

[Ed. Note: Clarify relation to NAL, parameter sets, etc.]

Code_number =0:	Inter picture with prediction from the most recent decoded picture only.

Code_number =1:	Inter picture with possibility of prediction from more than one previous decoded picture.  For this mode information reference picture for prediction must be signalled for each macroblock.

Code_number =2:	Intra picture.

Code_number =3:	B picture with prediction from the most recent previous decoded and subsequent decoded pictures only.

Code_number =4:	B picture with possibility of prediction from more than one previous decoded picture and subsequent decoded picture.  When using this mode, information reference frame for prediction must be signalled for each macroblock.

Code_number =5:	SP picture with prediction from the most recent decoded picture only.

Code_number =6:	SP picture with possibility of prediction from more than one previous decoded picture.  For this mode information reference picture for prediction must be signalled for each macroblock.

Code_number =7:	SI picture

### 8.2.3	Picture structure (PSTRUCT)

[Ed. Note: Clarify relation to NAL, parameter sets, etc.]

Code_number =0:	Progressive frame picture.

Code_number =1:	Top field picture.

Code_number =2:	Bottom field picture.

Code_number =3:	Interlaced frame picture, whose top field precedes its bottom field in time.

Code_number =4:	Interlaced frame picture, whose bottom field precedes its top field in time.

Note that when top field and bottom field pictures are coded for a frame, the one that is decoded first is the one that occurs first in time.

### 8.2.4	Size information

[Ed. Note: Clarify relation to NAL, parameter sets, etc.]

A series of up to three codewords.  The first codeword indicates a size change.  If set to zero the size is unchaged otherwise (if set to one) it is followed by two codewords containing the new width and height.

### 8.2.5	Reference Picture ID

[Ed. Note: Clarify relation to NAL, parameter sets, etc.]

### 8.2.6	First MB in slice

The number of the first macroblock contained in this slice.

### 8.2.7	Slice QP (SQP)

Information about the quantiser QUANT to be used for luminance for the picture. (See under Quantisation concerning QUANT for chrominance).  The 6-bit representation is the natural binary representation of the value of QP+12, which range from 0 to 47 (QP ranges from -12 to +39).  QP+12 is a pointer to the actual quantisation parameter QUANT to be used.  (See below under quantisation).  The range of quantisation value is still about the same as for H.263, 1-31.  An approximate relation between the QUANT in H.263 and QP is: QUANTH.263(QP)  QP0(QP) = 2QP/6  .  QP0() will be used later for scaling purposes when selecting prediction modes. Negative values of QP correspond to even smaller step sizes, as described below under quantisation.

### 8.2.8	SP Slice QP

For SP frames the SP slice QP is transmitted, using the same scale described above.

### 8.2.9	Number of  macroblocks in slice

For CABAC entropy coding the number of macroblocks contained in the slice is transmitted.

### 8.2.10	Picture Number (PN)

[Ed. Note: Clarify relation to NAL, parameter sets, etc.]

PN shall be incremented by 1 for each coded and transmitted picture in transmission order, in modulo MAX_PN operation, relative to the PN of the previous stored picture in transmission order.  For non-stored pictures, PN shall be incremented from the value in the most temporally recent stored picture which precedes the non-stored picture in transmission order.

The PN serves as a unique ID for each picture stored in the multi-picture buffer within MAX_PN coded and stored pictures.  Therefore, a picture cannot be kept in the buffer after more than MAX_PN-1 subsequent coded and stored pictures unless it has been assigned a long-term picture index as specified below.  The encoder shall ensure that the bitstream shall not specify retaining any short-term picture after more than MAX_PN-1 subsequent stored pictures.  A decoder which encounters a picture number on a current picture having a value equal to the picture number of some other short-term stored picture in the multi-picture buffer should treat this condition as an error.

### 8.2.11	Reference picture selection layer indicator (RPSLI)

[Ed. Note: Clarify relation to NAL, parameter sets, etc.]

RPSLI indicates the presence of reference picture selection layer (RPSL).  RPSLI shall be one of the following two values:

–	Code number 0:	The RPS layer is not sent,

–	Code number 1:	The RPS layer is sent.

If RPSL is not sent, the default buffer indexing order presented in the next subsection shall be applied.  RPS layer information sent at the slice level does not affect the decoding process of any other slice.

If RPSL is sent, the buffer indexing used to decode the current slice and to manage the contents of the picture buffer is sent using the following code words.

### 8.2.12	Re-Mapping of Picture Numbers Indicator (RMPNI)

RMPNI is present in the RPS layer if the picture is a P or B picture.  It is not present if the picture is an I picture. It is present at least once in P pictures and at least twice in B pictures.  RMPNI indicates whether any default picture indices are to be re-mapped for forward or backward motion compensation of the current slice – and how the re-mapping of the relative indices into the multi-picture buffer is to be specified if indicated.  The interpretation of RMPNI is shown in Figure 8-2. If RMPNI indicates the presence of an ADPN or LPIR field, an additional RMPNI field immediately follows the ADPN or LPIR field.

A picture reference index parameter (ref_idx_fwd or ref_idx_bwd) is a relative index into an ordered set of reference pictures.  For the coding of a P picture, there is one such set of reference pictures, called the forward reference set.  For the coding of a B picture, there are two such sets of reference pictures, called the forward and backward reference sets.

The RMPNI, ADPN, and LPIR fields allow the order of that relative indexing into the multi-picture buffer to be temporarily altered from the default index order for the decoding of a particular slice. An RMPNI "end loop" indication indicates the end of a list of re-ordering commands for the forward or backward reference set.

#### 8.2.12.1	Default index order for P pictures

[Ed. Note: should this go in the "decoding process" section instead of the "semantics" section?]

The default index order for forward prediction of P pictures is for the short-term pictures (i.e., pictures which have not been given a long-term index) to precede the long-term pictures in the reference indexing order.  Within the set of short-term pictures, the default order is for the pictures to be ordered starting with the most recently-transmitted reference picture and proceeding through to the reference picture in the short-term buffer that was transmitted first (i.e., in decreasing order of picture number in the absence of wrapping of the ten-bit picture number field).  Within the set of long-term pictures, the default order is for the pictures to be ordered starting with the picture with the smallest long-term index and proceeding up to the picture with long-term index equal to the most recent value of MLIP1-1.

For example, if the buffer contains three short-term pictures with short-term picture numbers 300, 302, and 303 (which were transmitted in increasing picture-number order) and two long-term pictures with long-term picture indices 0 and 3, the default index order is:

default relative index 0 refers to the short-term picture with picture number 303,

default relative index 1 refers to the short-term picture with picture number 302,

default relative index 2 refers to the short-term picture with picture number 300,

default relative index 3 refers to the long-term picture with long-term picture index 0, and

default relative index 4 refers to the long-term picture with long-term picture index 3.

#### 8.2.12.2	Default index order for B pictures

[Ed. Note: The proper default index order should be further investigated – this should be considered only a first attempt.]

The default index order for B pictures is defined such that short-term pictures that temporally precede the B picture are distinguished from short-term pictures that temporally follow the B picture, based on the temporal reference for each reference picture.  The default order for forward prediction (subclause 8.2.13.2.1) is specified in a manner that gives a lower index order to short-term pictures that temporally precede the current picture, and the default order for backward prediction (subclause 8.2.13.2.1) is specified in a manner that ordinarily gives a lower index order to short-term pictures that temporally follow the current picture.

###### 8.2.12.2.1	Forward prediction in B pictures

Within the set of short-term pictures, the default order for B-picture forward prediction shall be for the pictures to be ordered starting with the most recently-transmitted temporally-preceding reference picture and proceeding through to the temporally-preceding reference picture in the short-term buffer that was transmitted first.  These temporally-preceding pictures shall then be followed by the temporally-following reference pictures, starting with the most recently-transmitted temporally-following reference picture in the short-term buffer and proceeding through to the temporally-following reference picture in the short-term buffer that was transmitted first.  These pictures shall then be followed by the long-term pictures, starting with the picture with the smallest long-term index and proceeding up to the picture with long-term index equal to the most recent value of MLIP1-1.

###### 8.2.12.2.2	Backward prediction in B pictures

The default order for B-picture backward prediction is defined in a similar manner as for forward prediction, but giving preference in the order for pictures that temporally follow the B picture and swapping the order of the first two pictures if this would result in an identical forward and backward default indexing order.

Within the set of short-term pictures, the ordinary default order for B-picture backward prediction shall be for the pictures to be ordered starting with the most recently-transmitted temporally-following reference picture and proceeding through to the temporally-following reference picture that has been in the short-term buffer the longest.  These temporally-following pictures shall then be followed by the temporally-preceding reference pictures, starting with the most recently-transmitted temporally-preceding reference picture and proceeding through to the temporally-preceding reference picture that has been in the short-term buffer the longest.  These pictures shall then be followed by the long-term pictures, starting with the picture with the smallest long-term index and proceeding up to the picture with long-term index equal to the most recent value of MLIP1-1.

The ordinary default order defined in the previous paragraph shall be used as the default index order for backward prediction unless there is more than one reference picture in the set and the ordinary default index order for backward prediction is the same as the default index order for forward prediction.  In this exceptional case, the default index order for backward prediction shall be the ordinary default index order with the order of the first two pictures switched.

#### 8.2.12.3	Reordering of forward and backward reference sets

The first ADPN or LPIR field that is received (if any) moves a specified picture out of the default order to the relative index of zero.  The second such field moves a specified picture to the relative index of one, etc. The set of remaining pictures not moved to the front of the relative indexing order in this manner shall retain their default order amongst themselves and shall follow the pictures that have been moved to the front of the buffer in relative indexing order.

If there is not more than one forward reference picture used, no more than one ADPN or LPIR field shall be present in the same RPS layer unless the current picture is a B picture.  If the current picture is a B picture and there is not more than one backward reference picture used, no more than two ADPN or LPIR fields shall be present in the same RPS layer.

Any re-mapping of picture numbers specified for some slice shall not affect the decoding process for any other slice.

In a P picture an RMPNI “end loop” indication is followed by RPBT.  In a B picture, the first RMPNI "end loop" indication, which concludes the remapping of the forward reference set, is followed by an additional RMPNI indicator that begins the remapping operations (if any) for the backward reference set.

Within one RPS layer, RMPNI shall not specify the placement of any individual reference picture into more than one re-mapped position in relative index order.

Table 8-2 – RMPNI operations for re-mapping of reference pictures

| Code Number | Re-mapping Specified |
| --- | --- |
| 0 | ADPN field is present and corresponds to a negative difference to add to a picture number prediction value |
| 1 | ADPN field is present and corresponds to a positive difference to add to a picture number prediction value |
| 2 | LPIR field is present and specifies the long-term index for a reference picture |
| 3 | End loop for re-mapping of reference picture set relative indexing default order |

#### 8.2.12.1	Absolute Difference of Picture Numbers (ADPN)

ADPN is present only if indicated by RMPNI. ADPN follows RMPNI when present. The code number of the UVLC corresponds to ADPN – 1. ADPN represents the absolute difference between the picture number of the currently re-mapped picture and the prediction value for that picture number. If no previous ADPN fields have been sent within the current RPS layer, the prediction value shall be the picture number of the current picture. If some previous ADPN field has been sent, the prediction value shall be the picture number of the last picture that was re-mapped using ADPN.

If the picture number prediction is denoted PNP, and the picture number in question is denoted PNQ, the decoder shall determine PNQ from PNP and ADPN in a manner mathematically equivalent to the following:

if (RMPNI == '1') {	/* a negative difference */

if (PNP – ADPN < 0)

PNQ = PNP – ADPN + MAX_PN;

else

PNQ = PNP – ADPN;

}else{					/* a positive difference */

if (PNP + ADPN > MAX_PN-1)

PNQ  = PNP + ADPN – MAX_PN;

else

PNQ  = PNP + ADPN;

}

The encoder shall control RMPNI and ADPN such that the decoded value of ADPN shall not be greater than or equal to MAX_PN.

As an example implementation, the encoder may use the following process to determine values of ADPN and RMPNI to specify a re-mapped picture number in question, PNQ:

DELTA = PNQ – PNP;

if (DELTA < 0) {

if (DELTA < –MAX_PN/2-1)

MDELTA = DELTA + MAX_PN;

else

MDELTA = DELTA;

}else{

if (DELTA > MAX_PN/2)

MDELTA = DELTA – MAX_PN;

else

MDELTA = DELTA;

}

ADPN = abs(MDELTA);

where abs() indicates an absolute value operation.  Note that the code number of the UVLC corresponds to the value of ADPN – 1, rather than the value of ADPN itself.

RMPNI would then be determined by the sign of MDELTA.

#### 8.2.12.2	Long-term Picture Index for Re-Mapping (LPIR)

LPIR is present only if indicated by RMPNI. LPIR follows RMPNI when present. LPIR is transmitted using UVLC codewords. It represents the long-term picture index to be re-mapped. The prediction value used by any subsequent ADPN re-mappings is not affected by LPIR.

### 8.2.13	Reference Picture Buffering Type (RPBT)

RPBT specifies the buffering type of the currently decoded picture. It follows an RMPNI “end loop” indication when the picture is not an I picture. It is the first element of the RPS layer if the picture is an I picture. The values for RPBT are defined as follows:

Code number 0:	Sliding Window,

Code number 1:	Adaptive Memory Control.

In the “Sliding Window” buffering type, the current decoded picture shall be added to the buffer with default relative index 0, and any marking of pictures as “unused” in the buffer is performed automatically in a first-in-first-out fashion among the set of short-term pictures. In this case, if the buffer has sufficient “unused” capacity to store the current picture, no additional pictures shall be marked as “unused” in the buffer. If the buffer does not have sufficient “unused” capacity to store the current picture, the picture with the largest default relative index among the short-term pictures in the buffer shall be marked as “unused”. In the sliding window buffering type, no additional information is transmitted to control the buffer contents.

In the "Adaptive Memory Control" buffering type, the encoder explicitly specifies any addition to the buffer or marking of data as “unused” in the buffer, and may also assign long-term indices to short-term pictures. The current picture and other pictures may be explicitly marked as “unused” in the buffer, as specified by the encoder. This buffering type requires further information that is controlled by memory management control operation (MMCO) parameters.

#### 8.2.13.1	Memory Management Control Operation (MMCO)

MMCO is present only when RPBT indicates “Adaptive Memory Control”, and may occur multiple times if present.  It specifies a control operation to be applied to manage the multi-picture buffer memory.  The MMCO parameter is followed by data necessary for the operation specified by the value of MMCO, and then an additional MMCO parameter follows – until the MMCO value indicates the end of the list of such operations.  MMCO commands do not affect the buffer contents or the decoding process for the decoding of the current picture – rather, they specify the necessary buffer status for the decoding of subsequent pictures in the bitstream.  The values and control operations associated with MMCO are defined in Table 8-3.

If MMCO is Reset, all pictures in the multi-picture buffer (but not the current picture unless specified separately) shall be marked “unused” (including both short-term and long-term pictures).

The picture height and width shall not change within the bitstream except within a picture containing a Reset MMCO command.

A “stored picture” does not contain an MMCO command in its RPS layer which marks that (entire) picture as “unused”.  If the current picture is not a stored picture, its RPS layer shall not contain any of the following types of MMCO commands:

a)	A Reset MMCO command,

b)	Any MMCO command which marks any other picture (other than the current picture) as “unused” that has not also been marked as “unused” in the ERPS layer of a prior stored picture, or

c)	Any MMCO command which assigns a long-term index to a picture that has not also been assigned the same long-term index in the ERPS layer of a prior stored picture

Table 8-3 – Memory management control operation (MMCO) values

| Code Number | Memory Management Control Operation | Associated Data Fields Following |
| --- | --- | --- |
| 0 | End MMCO Loop | None (end of ERPS layer) |
| 1 | Mark a Short-Term Picture as “Unused” | DPN |
| 2 | Mark a Long-Term Picture as “Unused” | LPIN |
| 3 | Assign a Long-Term Index to a Picture | DPN and LPIN |
| 4 | Specify the Maximum Long-Term Picture Index | MLIP1 |
| 5 | Reset | None |

#### 8.2.13.2	Difference of Picture Numbers (DPN)

DPN is present when indicated by MMCO. DPN follows MMCO if present.  DPN is transmitted using UVLC codewords and is used to calculate the PN of a picture for a memory control operation.  It is used in order to assign a long-term index to a picture, mark a short-term picture as “unused”.  If the current decoded picture number is PNC and the decoded UVLC code number is DPN, an operation mathematically equivalent to the following equations shall be used for calculation of PNQ, the specified picture number in question:

if (PNC – DPN < 0)

PNQ = PNC – DPN + MAX_PN;

else

PNQ = PNC – DPN;

Similarly, the encoder may compute the DPN value to encode using the following relation:

if (PNC – PNQ < 0)

DPN = PNC – PNQ + MAX_PN;

else

DPN = PNC – PNQ;

For example, if the decoded value of DPN is zero and MMCO indicates marking a short-term picture as “unused”, the current decoded picture shall be marked as “unused” (thus indicating that the current picture is not a stored picture).

#### 8.2.13.3	Long-term Picture Index (LPIN)

LPIN is present when indicated by MMCO.  LPIN specifies the long-term picture index of a picture. It follows DPN if the operation is to assign a long-term index to a picture. It follows MMCO if the operation is to mark a long-term picture as “unused”.

#### 8.2.13.4	Maximum Long-Term Picture Index Plus 1 (MLIP1)

MLIP1 is present if indicated by MMCO.  MLIP1 follows MMCO if present.  If present, MLIP1 is used to determine the maximum index allowed for long-term reference pictures (until receipt of another value of MLIP1).  The decoder shall initially assume MLIP1 is "0" until some other value has been received.  Upon receiving an MLIP1 parameter, the decoder shall consider all long-term pictures having indices greater than the decoded value of MLIP1 – 1 as “unused” for referencing by the decoding process for subsequent pictures.  For all other pictures in the multi-picture buffer, no change of status shall be indicated by MLIP1.

### 8.2.14	Supplemental Enhancement Information

[Ed. Note: Clarify relation to NAL, parameter sets, etc.]

Supplemental enhancement information (SEI) is encapsulated into chunks of data separate from coded slices, for example. It is up to the network adaptation layer to specify the means to transport SEI chunks. Each SEI chunck may contain one or more SEI messages. Each SEI message shall consist of a SEI header and SEI payload. The SEI header starts at a byte-aligned position from the first byte of a SEI chunk or from the first byte after the previous SEI message. The SEI header consists of two codewords, both of which consist of one or more bytes. The first codeword indicates the SEI payload type. Values from 00 to FE shall be reserved for particular payload types, whereas value FF is an escape code to extend the value range to yet another byte as follows:

payload_type = 0;

for (;;) {

payload_type += *byte_ptr_to_sei;

if (*byte_ptr_to_sei < 0xFF)

break;

byte_ptr_to_sei++;

}

The second codeword of the SEI header indicates the SEI payload size in bytes. SEI payload size shall be coded similarly to the SEI payload type.

The SEI payload may have a SEI payload header. For example, a payload header may indicate to which picture the particular data belongs. The payload header shall be defined for each payload type separately.

## 8.3	Macroblock layer

Following the syntax diagram for the macroblock elements, the various elements are described.

### 8.3.1	Number of Skipped Macroblocks (mb_skip_run)

A macroblock is called skipped if no information is sent. In that case the reconstruction of an inter macroblock is made by copying the collocated picture material from the last decoded frame.

If PSTRUCT indicates a frame, then the skipped macroblock is formed by copying the collocated picture material from the last decoded frame, which either was decoded from a frame picture or is the union of two decoded field pictures. If PSTRUCT indicates a field, then the skipped macroblock is formed by copying the collocated material from the last decoded field of the same parity (top or bottom), which was either decoded from a field picture or is part of the most recently decoded frame picture.

[Editor: this needs alignment with the RPS.]

For a B macroblock skip means direct mode without coefficients. mb_skip_run indicates the number of skipped macroblocks in an inter- or B-picture before a coded macroblock.  If a picture or slice ends with one or more skipped macroblocks, they are represented by an additional mb_skip_run which counts the number of skipped macroblocks.

### 8.4.2	Macro block type (MB_Type)

Refer to Table QQ. There are different MB-Type tables for Intra and Inter frames.

#### 8.4.2.1	Intra Macroblock modes

Intra 4x4	Intra coding as defined in sections QQ to QQ.

Imode, nc, AC	See definition in section QQ. These modes refer to 16x16 intra coding.

#### 8.4.2.2	Inter Macroblock Modes

Skip	No further information about the macroblock is transmitted.  A copy of the colocated macroblock in the most recent decoded picture is used as reconstruction for the present macroblock.

NxM (eg. 16x8)	The macroblock is predicted from a past picture with block size NxM.  For the macroblock modes 16x16, 16x8, and 8x16, a motion vector is provided for each NxM block.  If N=M=8, for each 8x8 sub-partition an additional codeword is transmitted which indicates in which mode the corresponding sub-partition is coded (see subclause 8.4.3). Depending on N,M and the 8x8 sub-partition modes there may be 1 to 16 sets of motion vector data for a macroblock.

Intra 4x4	4x4 intra coding.

Code numbers from 6 and upwards represent 16x16 intra coding.

### 8.4.3	8x8 sub-partition modes

NxM (eg. 8x4)	The corresponding 8x8 sub-partition is predicted from a past picture with block size NxM. A motion vector is transmitted for each NxM block. Depending on N and M, up to 4 motion vectors are coded for an 8x8 sub-partition, and thus up to 16 motion vectors are transmitted for a macroblock.

Intra	The 8x8 sub-partition is coded in intra 4x4 mode.

### 8.4.4	Intra Coding

Even in intra mode, prediction is always used for each sub block in a macroblock.  A 4x4 block is to be coded (samples labeled a to p below).  The samples A to Q from neighbouring blocks may already be decoded and may be used for prediction. When samples E-H are not available, whether because they have not yet been decoded, are outside the picture or outside the current slice, the sample value of D is substituted for samples E-H. When samples M-P are not available, the sample value of L is substituted for samples M-P.

[Ed. Note: For some reason, the text in the figure below disappears when I print this to paper.  Should be fixed or replaced by Visio diagram.]

Q A B C D E F G H

I a b c d

J e f g h

K i j k l

L m n o p

M

N

O

P

Figure 8-2 – Identification of samples used for intra spatial prediction

#### 8.4.4.1	Prediction for Intra-4x4 mode for luminance blocks

For the luminance signal, there are nine intra prediction modes labeled 0 to 8.  Mode 0 is ‘DC-prediction’ (see below).  The other modes represent directions of predictions as indicated below.

[Ed. Note: Something went wrong with the figure when copying.  Needs to be fixed or preferably replaced by a Visio figure.]

1

2

3

4

5

6

7

8

Figure 8-3 – Intra prediction directions

[Ed. Note: Something wrong with the location and framing of the picture.  How was this created and how can it be fixed?]

###### 8.4.4.1.1	Mode 0: DC prediction

If all samples A, B, C, D, I, J, K, L, are within the slice, all samples are predicted by (A+B+C+D+I+J+K+L+4)>>3.  If A, B, C, and D are outside the slice and I, J, K, and L are not, all samples are predicted by (I+J+K+L+2)>>2.  If I, K, K, and L are outside the slice and A, B, C, and D are not, all samples are predicted by (A+B+C+D+2)>>2.  If all eight samples are outside the slice, the prediction for all samples in the block is 128.  A block may therefore always be predicted in this mode.

###### 8.4.4.1.2	Mode 1: Vertical Prediction

If A, B, C, D are inside the slice, then

–	a, e, i, m are predicted by A,

–	b, f, j, n are predicted by B,

–	c, g, k, o are predicted by C,

–	d, h, l, p are predicted by D.

###### 8.4.4.1.3	Mode 2: Horizontal prediction

If I, J, K, L are inside the slice, then

–	a, b, c, d are predicted by I,

–	e, f, g, h are predicted by J,

–	i, j, k, l are predicted by K,

–	m, n, o, p are predicted by L.

###### 8.4.4.1.4	Mode 3: Diagonal Down/Right prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	m is predicted by: 			(J + 2K + L + 2) >> 2

–	i, n are predicted by 			(I + 2J + K + 2) >> 2

–	e, j, o are predicted by 		(Q + 2I + J + 2) >> 2

–	a, f, k, p are predicted by 		(A + 2Q + I + 2) >> 2

–	b, g, l are predicted by 		(Q + 2A + B + 2) >> 2

–	c, h are predicted by 			(A + 2B + C + 2) >> 2

–	d is predicted by 			(B + 2C + D + 2) >> 2

###### 8.4.4.1.5	Mode 4: Diagonal Down/Left prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	a is predicted by			(A + 2B + C + I + 2J + K + 4) >> 3

–	b, e are predicted by 			(B + 2C + D + J + 2K + L + 4) >> 3

–	c, f, i are predicted by 		(C + 2D + E + K + 2L + M + 4) >> 3

–	d, g, j, m are predicted by 		(D + 2E + F + L + 2M + N + 4) >> 3

–	h, k, n are predicted by 		(E + 2F + G + M + 2N + O + 4) >> 3

–	l, o are predicted by 			(F + 2G + H + N + 2O + P + 4) >> 3

–	p is predicted by 			(G + H + O + P + 2) >> 3

###### 8.4.4.1.6	Mode 5: Vertical-Left prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	a, j are predicted by 			(Q + A + 1) >> 1

–	b, k are predicted by			(A + B + 1) >> 1

–	c, l are predicted by 			(B + C + 1) >> 1

–	d is predicted by 			(C + D + 1) >> 1

–	e, n are predicted by 			(I + 2Q + A + 2) >> 2

–	f, o are predicted by 			(Q + 2A + B + 2) >> 2

–	g, p are predicted by 			(A + 2B + C + 2) >> 2

–	h is predicted by 			(B + 2C + D + 2)  >> 2

–	i is predicted by 			(Q + 2I + J + 2) >> 2

–	m is predicted by 			(I + 2J + K + 2) >> 2

###### 8.4.4.1.7	Mode 6: Vertical-Right prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	a is predicted by 			(2A + 2B + J + 2K + L + 4) >> 3

–	b, i are predicted by			(B + C + 1) >> 1

–	c, j are predicted by 			(C + D + 1) >> 1

–	d, k are predicted by 			(D + E + 1) >> 1

–	l is predicted by 			(E + F + 1) >> 1

–	e is predicted by 			(A + 2B + C + K + 2L + M + 4) >> 3

–	f, m are predicted by 		(B + 2C + D + 2) >> 2

–	g, n are predicted by 			(C + 2D + E + 2) >> 2

–	h, o are predicted by 			(D + 2E + F + 2) >> 2

–	p is predicted by 			(E + 2F + G + 2) >> 2

###### 8.4.4.1.8	Mode 7: Horizontal-Up prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	a is predicted by 			(B + 2C + D + 2I + 2J + 4) >> 3

–	b is predicted by 			(C + 2D + E + I + 2J + K + 4) >> 3

–	c, e are predicted by 			(D + 2E + F + 2J + 2K + 4) >> 3

–	d, f are predicted by 			(E + 2F + G + J + 2K + L + 4) >> 3

–	g, i are predicted by 			(F + 2G + H + 2K + 2L + 4) >> 3

–	h, j are predicted by 			(G + 3H + K + 3L + 4) >> 3

–	l, n are predicted by 			(L + 2M + N + 2) >> 3

–	k, m are predicted by 		(G + H + L + M + 2) >> 2

–	o is predicted by 			(M + N + 1) >> 1

–	p is predicted by 			(M + 2N + O + 2) >> 2

###### 8.4.4.1.9	Mode 8: Horizontal-Down prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	a, g are predicted by 			(Q + I + 1) >> 1

–	b, h are predicted by 			(I + 2Q + A+ 2) >> 2

–	c is predicted by 			(Q + 2A + B+ 2) >> 2

–	d is predicted by 			(A + 2B + C+ 2) >> 2

–	e, k are predicted by 			(I + J + 1) >> 1

–	f, l are predicted by 			(Q + 2I + J+ 2) >> 2

–	i, o are predicted by 			(J + K + 1) >> 1

–	j, p are predicted by 			(I + 2J + K+ 2) >> 2

–	m is predicted by 			(K + L + 1) >> 1

–	n is predicted by 			(J + 2K + L + 2) >> 2

#### 8.4.4.2	Coding of Intra 4x4 prediction modes

Since each of the 4x4 luminance blocks is assigned a prediction mode, this will require a considerable number of bits if coded independently.  The chosen prediction of a block is highly correlated with the prediction modes of adjacent blocks.  This is illustrated in Figure 8-4.  When the prediction modes of A and B are known (including the case that A or B or both are outside the slice) an ordering of the most probable, next most probable etc. of C is given.  When an adjacent block is coded by 16x16 intra mode, prediction mode is “mode 0: DC_prediction”; when it is coded in inter mode, prediction mode is “mode 0: DC_prediction” in the usual case and “outside” in the case of constrained intra update. This ordering is listed in Table 8-3.

For each prediction mode of A and B a list of 9 numbers is given in Table 8-3.  Example: Prediction mode for A and B is 2.  The string 2 8 7 1 0 6 4 3 5 indicates that mode 2 is also the most probable mode for block C.  Mode 8 is the next most probable one etc.  In the bitstream there will for instance be information that Prob0 = 1 (see Table 9-5) indicating that the next most probable mode shall be used for block C.  In our example this means Intra prediction mode 8.  Use of '–' in the table indicates that this instance cannot occur because A or B or both are outside the slice.

For more efficient coding, information on intra prediction of two 4x4 luminance blocks are coded in one codeword (Prob0 and Prob1 in Table 9-5).  The order of the resulting eight codewords is indicated in Table 8-3.

Figure 8-4 – a) Prediction mode of block C to be established, where A and B are adjacent blocks. b) order of intra prediction information in the bitstream

Table 8-3 – Prediction mode as a function of ordering signalled in the bitstream

| Index | B/A | Outside | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- | --- | --- |
| 0a | Outside | 0-------- | 01------- | 10------- | --------- | --------- |
| 1a | 0 | 02------- | 021648573 | 125630487 | 021876543 | 021358647 |
| 2a | 1 | --------- | 102654387 | 162530487 | 120657483 | 102536487 |
| 3a | 2 | 20------- | 280174365 | 217683504 | 287106435 | 281035764 |
| 4a | 3 | --------- | 201385476 | 125368470 | 208137546 | 325814670 |
| 5a | 4 | --------- | 201467835 | 162045873 | 204178635 | 420615837 |
| 6a | 5 | --------- | 015263847 | 152638407 | 201584673 | 531286407 |
| 7a | 6 | --------- | 016247583 | 160245738 | 206147853 | 160245837 |
| 8a | 7 | --------- | 270148635 | 217608543 | 278105463 | 270154863 |
| 9a | 8 | --------- | 280173456 | 127834560 | 287104365 | 283510764 |

Table 8-3 (concluded)

| Index | B/A | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- |
| 0b | Outside | --------- | --------- | --------- | --------- | --------- |
| 1b | 0 | 206147583 | 512368047 | 162054378 | 204761853 | 208134657 |
| 2b | 1 | 162045378 | 156320487 | 165423078 | 612047583 | 120685734 |
| 3b | 2 | 287640153 | 215368740 | 216748530 | 278016435 | 287103654 |
| 4b | 3 | 421068357 | 531268470 | 216584307 | 240831765 | 832510476 |
| 5b | 4 | 426015783 | 162458037 | 641205783 | 427061853 | 204851763 |
| 6b | 5 | 125063478 | 513620847 | 165230487 | 210856743 | 210853647 |
| 7b | 6 | 640127538 | 165204378 | 614027538 | 264170583 | 216084573 |
| 8b | 7 | 274601853 | 271650834 | 274615083 | 274086153 | 278406153 |
| 9b | 8 | 287461350 | 251368407 | 216847350 | 287410365 | 283074165 |

#### 8.4.4.3	Intra-16x16 mode for luminance

Assume that the block to be predicted has sample locations 0 to 15 horizontally and 0 to 15 vertically.  We use the notation P(i,j) where i,j = 0..15.  P(i,-1), i=0..15 are the neighbouring samples above the block and P(-1,j), j=0..15 are the neighbouring samples to the left of the block.  Pred(i,j)  i,j = 0..15 is the prediction for the whole Luminance macroblock.  We have 4 different prediction modes:

##### 8.4.4.3.1	Mode 0 (vertical)

Pred(i, j) = P(i, -1), i, j=0..15

##### 8.4.4.3.2	Mode 1 (horizontal)

Pred(i, j) = P(-1, j), i, j=0..15

##### 8.4.4.3.3	Mode 2 (DC prediction)

Pred(i, j) = ((i=015(P(−1,i)+P(i,−1)))+16)>>5    i, j=0..15,

where only the average of 16 samples are used when the other 16 samples are outside the slice. If all 32 samples are outside the slice, the prediction for all samples in the block is 128.

##### 8.4.4.3.4	Mode 3 (Plane prediction)

Pred(i,j) = clip1( (a + b·(i-7) + c·(j-7) +16) >> 5 ),

where:

–	a = 16·(P(-1,15) + P(15,-1))

–	b = (5*(H>>2)>>4) [Ed. Note: Correct? Why no rounding offset?]

–	c = (5*(V>>2)>>4)

and H and V are defined as:

H=i=18i⋅(P(7+i,−1)−P(7−i,−1))	(8-1)

V=j=18j⋅(P(−1,7+j)−P(−1,7−j))	(8-2)

Residual coding [Ed. Note: All The next few paragraphs are wrong and should be corrected or deleted.  Now a different transform is applied to DC and also we're not using these normalization factors anymore.]

The residual coding is based on 4x4 transform.  But similar to coding of chrominance coefficients, another 4x4 transform to the 16 DC coefficients in the macroblock are added.  In that way we end up with an overall DC for the whole MB which works well in flat areas.

Since we use the same integer transform to DC coefficients, we have to perform additional normalization to those coefficients, which implies a division by 676.  To avoid the division we performed normalization by 49/215 on the encoder side and 48/215 on the decoder side, which gives sufficient accuracy.

Only single scan is used for 16x16 intra coding.

To produce the bitstream, we first scan through the 16 ‘DC transform’ coefficients.  There is no ‘CBP’ information to indicate no coefficients on this level.  If AC = 1 (see below) ac coefficients of the 16 4x4 blocks are scanned.  There are 15 coefficients in each block since the DC coefficients are included in the level above.

Coding of mode information for Intra-16x16 mode

See Table 7. Three parameters have to be signalled.  They are all included in MB-type.

Imode:	0,1,2,3

AC:	0 means there are no ac coefficients in the 16x16 block. 1 means that there is at least one ac coefficient and all 16 blocks are scanned.

nc:	CBP for chrominance (see qq)

#### 8.4.4.4	Prediction in intra coding of chrominance blocks

For chrominance prediction there is only one mode.  No information is therefore needed to be transmitted.  The prediction is indicated in Figure QQ.  The 8x8 chrominance block consists of 4 4x4 blocks A,B,C,D.  S0,1,2,3 are the sums of 4 neighbouring samples.

If S0, S1, S2, S3 are all inside the frame:

–	A = (S0 + S2 + 4)/8

–	B = (S1 + 2)/4

–	C = (S3 + 2)/4

–	D = (S1 + S3 + 4)/8

If only S0 and S1 are inside the frame:

–	A = (S0 + 2)/4

–	B = (S1 + 2)/4

–	C = (S0 + 2)/4

–	D = (S1 + 2)/4

If only S2 and S3 are inside the frame:

–	A = (S2 + 2)/4

–	B = (S2 + 2)/4

–	C = (S3 + 2)/4

–	D = (S3 + 2)/4

If S0, S1, S2, S3 are all outside the frame: A = B = C = D = 128

[Ed. Note: This prediction should be considered changed.  What does that last sentence mean?]

Figure 8-5 – Prediction of chrominance blocks

### 8.4.5	Reference picture (ref_idx_fwd)

If PTYPE indicates possibility of prediction from more than one previously-decoded picture, the exact picture to be used must be signalled.  This is done according to the following tables.  If PSTRUCT indicates that the current picture is a frame picture, then the reference picture is a previous frame in the forward reference buffer that was either encoded as a single frame picture or a frame that was encoded as two field pictures and has been reconstructed as a frame.   Thus for frames the following table gives the reference frame:

Code_number	Reference frame

0	The first frame in the forward reference set

1	The second frame in the forward reference set

2	The third frame in the forward reference set

..	..

The reference index parameter is transmitted for each 16x16, 16x8, or 8x16 block. If the macroblock is coded in 8x8 mode, the reference frame parameter is coded once for each 8x8 sub-partition unless the 8x8 sub-partition is transmitted in intra mode. If the UVLC is used for entropy coding and the macroblock type is indicated by codeword 4 (8x8, ref=0), no reference frame parameters are transmitted for the whole macroblock.

If PSTRUCT indicates that the current picture is a field picture, then the reference picture is either a previous field in the reference buffer that was separately encoded as a field picture or a previous field that is half of a frame that was encoded as a frame picture.  Note that for a P field picture, forward prediction from field 1 to field 2 in the same frame is allowed.  For the purpose of determining ref_idx_fwd, a unique reference field number is assigned to each reference field in the reference field (frame) buffer according to its distance from the current field, as shown in Figures 3a and 3b, modified by the fact that smaller code numbers are given to the fields of the same field parity as the current field.  Thus for fields the following table gives the reference field:

[Ed. Note: Make this into a table?]

Code_number	Reference field

0	the last decoded previous field of the same parity (2 fields back)

1	the last decoded previous field of the opposite parity (1 field back)

2	same parity field 4 fields back

3	opposite parity field 3 fields back

..	..

Figure 8-6 - Reference picture number assignment when the current picture is the first field coded in a frame. Solid-line is for frames and dotted-line for fields. f1 stands for field 1 and f2 for field 2.  [Ed. Note: Cumbersome title]

Figure 8-7 - Reference picture number assignment when the current picture is the second field coded in a frame. Solid-line is for frames and dotted-line for fields. f1 stands for field 1 and f2 for field 2.  [Ed. Note: Cumbersome title]

### 8.4.6	Motion Vector Data (MVD)

If so indicated by MB_type, vector data for 1-16 blocks are transmitted.  For every block a prediction is formed for the horizontal and vertical components of the motion vector.  MVD signals the difference between the vector component to be used and this prediction.  The order in which vector data is sent is indicated in Figure 2. Motion vectors are allowed to point to samples outside the reference frame.  If a sample outside the reference frame is referred to in the prediction process, the nearest sample belonging to the frame (an edge or corner sample) shall be used. All fractional sample positions shall be interpolated as described in subclause 9.2. If a sample referred in the interpolation process (necessarily integer accuracy) is outside of the reference frame it shall be replaced by the nearest sample belonging to the frame (an edge or corner sample). Reconstructed motion vectors shall be clipped to ±19 integer samples outside of the frame. [Ed. Note: What does that mean?  Is the encoder required not to send such out-of-bounds vectors?  Is the decoder required to implement its decoding process by clipping motion vector values?  Are the clipped or unclipped vectors used in MV prediction of subsequent MV's?]

#### 8.4.6.1	Prediction of vector components

No vector component prediction takes place across macroblocks boundaries of MBs that do not belong to the same slice.  For the purpose of motion vector prediction, macroblocks that do not belong to the same slice are considered “outside the picture”.

With exception of the 16x8 and 8x16 block shapes, "median prediction" (see subclause 8.4.6.1.1) is used.  In case the macroblock may be classified to have directional segmentation the prediction is defined in subclause 8.4.6.1.2.

##### 8.4.6.1.1	Median prediction

In the Figure below the vector component E of the indicated block shall be predicted.  The prediction is normally formed as the median of A, B and C.  However, the prediction may be modified as described below.  Notice that it is still referred to as "median prediction"

A	The component applying to the sample to the left of the upper left sample in E

B	The component applying to the sample just above the upper left sample in E

C	The component applying to the sample above and to the right of the upper right sample in E

D	The component applying to the sample above and to the left of the upper left sample in E

Figure 8-8 -Median prediction of motion vectors

A, B, C, D and E may represent motion vectors from different reference pictures.  As an example we may be seeking prediction for a motion vector for E from the last decoded picture.  A, B, C and D may represent vectors from 2, 3, 4 and 5 pictures back.  The following substitutions may be made prior to median filtering.

–	If A and D are outside the picture, their values are assumed to be zero and they are considered to have "different reference picture than E".

–	If D, B, C are outside the picture, the prediction is equal to A (equivalent to replacing B and C with A before median filtering).

–	If C is outside the picture or still not available due to the order of vector data (see Figure 2), C is replaced by D.

If any of the blocks A, B, C, D are intra coded they count as having "different reference picture”.  If one and only one of the vector components used in the median calculation (A, B, C) refer to the same reference picture as the vector component E, this one vector component is used to predict E.

##### 8.4.6.1.2	Directional segmentation prediction

If the macroblock where the block to be predicted is coded in 16x8 or 8x16 mode, the prediction is generated as follows (refer to Figure below and the definitions of A, B, C, E above):

a)	Vector block size 8x16:

1)	Left block: A is used as prediction if it has the same reference picture as E, otherwise "median prediction" is used

2)	Right block: C is used as prediction if it has the same reference picture as E, otherwise "median prediction" is used

b)	Vector block size 16x8:

1)	Upper block: B is used as prediction if it has the same reference picture as E, otherwise "median prediction" is used

2)	Lower block: A is used as prediction if it has the same reference picture as E, otherwise "median prediction" is used

If the indicated prediction block is outside the picture, the same substitution rules are applied as in the case of median prediction.

Figure 8-9 – Directional segmentation prediction

#### 8.4.6.2	Chrominance vectors

Chrominance vectors are derived from the luminance vectors.  Since chrominance has half resolution compared to luminance, the chrominance vectors are obtained by dividing the corresponding luminance motion vectors by two.

Due to the lower resolution of the chrominance array relative to the luminance array, a chrominance vector applies to 1/4 as many samples as the Luminance vector.  For example if the Luminance vector applies to 8x16 Luminance samples, the corresponding chrominance vector applies to 4x8 chrominance samples and if the Luminance vector applies to 4x4 Luminance samples, the corresponding chrominance vector applies to 2x2 chrominance samples.

### 8.4.7	Coded Block Pattern (CBP)

The CBP contains information of which 8x8 blocks - Luminance and chrominance - contain transform coefficients.  Notice that an 8x8 block contains 4 4x4 blocks meaning that the statement '8x8 block contains coefficients' means that 'one or more of the 4 4x4 blocks contain coefficients'.  The 4 least significant bits of CBP contain information on which of 4 8x8 luminance blocks in a macroblock contains nonzero coefficients.  Let us call these 4 bits CBPY. The ordering of 8x8 blocks is indicated in Figure 3.  A 0 in position n of CBP (binary representation) means that the corresponding 8x8 block has no coefficients whereas a 1 means that the 8x8 block has one or more non-zero coefficients.

For chrominance we define 3 possibilities:

nc=0:	no chrominance coefficients at all.

nc=1	There are nonzero 2x2 transform coefficients.  All chrominance AC coefficients = 0.  Therefore we do not send any EOB for chrominance AC coefficients.

nc=2	There may be 2x2 nonzero coefficients and there is at least one nonzero chrominance AC coefficient present.  In this case we need to send 10 EOBs (2 for DC coefficients and 2x4=8 for the 8 4x4 blocks) for chrominance in a macroblock.

The total CBP for a macroblock is:  CBP = CBPY + 16xnc

The CBP is signalled with a different codeword for Inter macroblocks and Intra macroblocks since the statistics of CBP values are different in the two cases.

### 8.4.8	Change of Quantiser Value (Dquant)

Dquant contains the possibility of changing QUANT on the macroblock level.  It does not need to be present for macroblocks without nonzero transform coefficients.  More specifically Dquant is present for non-skipped macroblocks:

–	If CBP indicates that there are nonzero transform coefficients in the MB or

–	If the MB is 16x16 based intra coded

The value of Dquant may range from -26 to +26, which enables the QP to be changed to any value in the range [-12..39].

QUANTnew = -12 + modulo52(QUANTold + Dquant + 64)      (also known as "arithmetic wrap")

# 9	Decoding Process

# 9.1 Slice Decoding

A slice is decoded as follows:

If the picture number of the slice to be decoded is different from the picture number of the previously decoded slice, decoding of a new picture is started. Otherwise, decoding of the current picture is continued.  [FMO Edt: This mechanism has a serious problem in lossy environments in conjunction with disposable pictures following in sequence.]

The macroblock address of the first macroblock of the slice is used an index to the macroblock allocation map to determine which slice group the slice belongs to. Using the calculated macroblock address, the macroblock can be reconstructed directly.  Since no other macroblock belonging to this slice has been decoded (as it is the first macroblock after the slice header), all in-picture prediction mechanisms are reset before decoding and reconstruction.

When mb_allocation_map_type equals zero, the macroblock address of the next macroblock is implicitly the current macroblock address plus 1.  When mb_allocation_map_type is not zero, the next macroblock address is determined by searching the macroblock allocation map for the next macroblock in raster-scan order belonging to the slice group being decoded.  [Ed note StW: in the test model section some optimizations to this brute-force search could be added]

[Ed. Note StW: this paragraph may go to the test model: The macroblock address of the next macroblock, as found above, is used for reconstruction of that macroblock.  The main difference between FMO and the current design (with scan order slices) is that, except at the left edge of the picture, a scan-order slice system can always assume that at least the neighbor to the left is available, whereas an FMO-based system cannot rely on this property.  In-picture prediction for intra macroblocks and for the Motion Vectors operates using the available neighbors.  If no neighbors exist, then the decoding is identical to the decoding of the first macroblock of the slice.  No special considerations have to be observed when reaching the end of the slice.  The decoder simply restarts the same algorithm with the next slice.]

### 9.1.1	Multi-Picture Decoder Process

The decoder stores the reference pictures for inter-picture decoding in a multi-picture buffer.  The decoder replicates the multi-picture buffer of the encoder according to the reference picture buffering type and any memory management control operations specified in the bitstream.  The buffering scheme may also be operated when partially erroneous pictures are decoded.

Each transmitted and stored picture is assigned a Picture Number (PN) which is stored with the picture in the multi-picture buffer.  PN represents a sequential picture counting identifier for stored pictures. PN is constrained, using modulo MAX_PN arithmetic operation. For the first transmitted picture, PN should be "0". For each and every other transmitted and stored picture, PN shall be increased by 1. If the difference (modulo MAX_PN) of the PNs of two consecutively received and stored pictures is not 1, the decoder should infer a loss of pictures or corruption of data. In such a case, a back-channel message indicating the loss of pictures may be sent to the encoder.

Besides the PN, each picture stored in the multi-picture buffer has an associated index, called the default relative index. When a picture is first added to the multi-picture buffer it is given default relative index 0 – unless it is assigned to a long-term index. The default relative indices of pictures in the multi-picture buffer are modified when pictures are added to or removed from the multi-picture buffer, or when short-term pictures are assigned to long-term indices.

The pictures stored in the multi-picture buffers can also be divided into two categories: long-term pictures and short-term pictures. A long-term picture can stay in the multi-picture buffer for a long time (more than MAX_PN-1 coded and stored picture intervals). The current picture is initially considered a short-term picture. Any short-term picture can be changed to a long-term picture by assigning it a long-term index according to information in the bitstream. The PN is the unique ID for all short-term pictures in the multi-picture buffer. When a short-term picture is changed to a long-term picture, it is also assigned a long-term picture index (LPIN). A long-term picture index is assigned to a picture by associating its PN to an LPIN. Once a long-term picture index has been assigned to a picture, the only potential subsequent use of the long-term picture’s PN within the bitstream shall be in a repetition of the long-term index assignment. The PNs of the long-term pictures are unique within MAX_PN transmitted and stored pictures. Therefore, the PN of a long-term picture cannot be used for assignment of a long-term index after MAX_PN-1 transmitted subsequent stored pictures. LPIN becomes the unique ID for the life of a long-term picture.

PN (for a short-term picture) or LPIN (for a long-term picture) can be used to re-map the pictures into re-mapped relative indices for efficient reference picture addressing.

#### 9.1.1.1	Decoder Process for Short/Long-term Picture Management

The decoder may have both long-term pictures and short-term pictures in its multi-picture buffer. The MLIP1 field is used to indicate the maximum long-term picture index allowed in the buffer. If no prior value of MLIP1 has been sent, no long-term pictures shall be in use, i.e. MLIP1 shall initially have an implied value of "0". Upon receiving an MLIP1 parameter, a new MLIP1 shall take effect until another value of MLIP1 is received. Upon receiving a new MLIP1 parameter in the bitstream, all long-term pictures with associated long-term indices greater than or equal to MLIP1 shall be considered marked “unused”. The frequency of transmitting MLIP1 is out of the scope of this Recommendation. However, the encoder should send an MLIP1 parameter upon receiving an error message, such as an Intra request message.

A short-term picture can be changed to a long-term picture by using an MMCO command with an associated DPN and LPIN. The short-term picture number is derived from DPN and the long-term picture index is LPIN. Upon receiving such an MMCO command, the decoder shall change the short-term picture with PN indicated by DPN to a long-term picture and shall assign it to the long-term index indicated by LPIN. If a long-term picture with the same long-term index already exists in the buffer, the previously-existing long-term picture shall be marked “unused”. An encoder shall not assign a long-term index greater than MLIP1–1 to any picture. If LPIN is greater than MLIP1–1, this condition should be treated by the decoder as an error. For error resilience, the encoder may send the same long-term index assignment operation or MLIP1 specification message repeatedly. If the picture specified in a long-term assignment operation is already associated with the required LPIN, no action shall be taken by the decoder. An encoder shall not assign the same picture to more than one long term index value. If the picture specified in a long-term index assignment operation is already associated with a different long-term index, this condition should be treated as an error. An encoder shall only change a short-term picture to a long-term picture within MAX_PN transmitted consecutive stored pictures. In other words, a short-term picture shall not stay in the short-term buffer after more than MAX_PN-1 subsequent stored pictures have been transmitted. An encoder shall not assign a long-term index to a short-term picture that has been marked as “unused” by the decoding process prior to the first such assignment message in the bitstream. An encoder shall not assign a long-term index to a picture number that has not been sent.

#### 9.1.1.2	Decoder Process for Reference Picture Buffer Mapping

The decoder employs indices when referencing a picture for motion compensation on the macroblock layer.  In pictures other than B pictures, these indices are the default relative indices of pictures in the multi-picture buffer when the fields ADPN and LPIR are not present in the current slice layer as applicable, and are re-mapped relative indices when these fields are present.  In B pictures, the first one or two pictures (depending on BTPSM) in relative index order are used for backward prediction, and the forward picture reference parameters specify a relative index into the remaining pictures for use in forward prediction. (needs to be changed)

The indices of pictures in the multi-picture buffer can be re-mapped onto newly specified indices by transmitting the RMPNI, ADPN, and LPIR fields. RMPNI indicates whether ADPN or LPIR is present.  If ADPN is present, RMPNI specifies the sign of the difference to be added to a picture number prediction value.  The ADPN value corresponds to the absolute difference between the PN of the picture to be re-mapped and a prediction of that PN value.  The first transmitted ADPN is computed as the absolute difference between the PN of the current picture and the PN of the picture to be re-mapped.  The next transmitted ADPN field represents the difference between the PN of the previous picture that was re-mapped using ADPN and that of another picture to be re-mapped.  The process continues until all necessary re-mapping is complete.  The presence of re-mappings specified using LPIR does not affect the prediction value for subsequent re-mappings using ADPN.  If RMPNI indicates the presence of an LPIR field, the re-mapped picture corresponds to a long-term picture with a long-term index of LPIR.  If any pictures are not re-mapped to a specific order by RMPNI, these remaining pictures shall follow after any pictures having a re-mapped order in the indexing scheme, following the default order amongst these non-re-mapped pictures.

If the indicated parameter set in the latest received slice or data partition signals the required picture number update behavior, the decoder shall operate as follows. The default picture index order shall be updated as if pictures corresponding to missing picture numbers were inserted to the multi-picture buffer using the “Sliding Window” buffering type. An index corresponding to a missing picture number is called an “invalid” index. The decoder should infer an unintentional picture loss if any “invalid” index is referred to in motion compensation or if an “invalid” index is re-mapped.

If the indicated parameter set in the latest received slice or data partition does not signal the required picture number update behavior, the decoder should infer an unintentional picture loss if one or several picture numbers are missing or if a picture not stored in the multi-picture buffer is indicated in a transmitted ADPN or LPIR.

In case of an unintentional picture loss, the decoder may invoke some concealment process. If the required picture number update behavior was indicated, the decoder may replace the picture corresponding to an “invalid” index with an error-concealed one and remove the “invalid” indication. If the required picture number update behavior was not indicated, the decoder may insert an error-concealed picture into the multi-picture buffer assuming the “Sliding Window” buffering type. Concealment may be conducted by copying the closest temporally preceding picture that is available in the multi-picture buffer into the position of the missing picture. The temporal order of the short-term pictures in the multi-picture buffer can be inferred from their default relative index order and PN fields. In addition or instead, the decoder may send a forced intra update signal to the encoder by external means (for example, Recommendation H.245), or the decoder may use external means or back-channel messages (for example, Recommendation H.245) to indicate the loss of pictures to the encoder.

#### 9.1.1.3	Decoder Process for Multi-Picture Motion Compensation

Multi-picture motion compensation is applied if the use of more than one reference picture is indicated. For multi-picture motion compensation, the decoder chooses a reference picture as indicated using the reference frame fields on the macroblock layer. Once, the reference picture is specified, the decoding process for motion compensation proceeds.

#### 9.1.1.4	Decoder Process for Reference Picture Buffering

The buffering of the currently decoded picture can be specified using the reference picture buffering type (RPBT). The buffering may follow a first-in, first-out ("Sliding Window") mode. Alternatively, the buffering may follow a customized adaptive buffering ("Adaptive Memory Control") operation that is specified by the encoder in the forward channel.

The "Sliding Window" buffering type operates as follows. First, the decoder determines whether the picture can be stored into “unused” buffer capacity. If there is insufficient “unused” buffer capacity, the short-term picture with the largest default relative index (i.e. the oldest short-term picture in the buffer) shall be marked as “unused”. The current picture is stored in the buffer and assigned a default relative index of zero. The default relative index of all other short-term pictures is incremented by one. The default relative indices of all long-term pictures are incremented by one minus the number of short-term pictures removed.

In the "Adaptive Memory Control" buffering type, specified pictures may be removed from the multi-picture buffer explicitly.  The currently decoded picture, which is initially considered a short-term picture, may be inserted into the buffer with default relative index 0, may be assigned to a long-term index, or may be marked as “unused” by the encoder.  Other short-term pictures may also be assigned to long-term indices. The buffering process shall operate in a manner functionally equivalent to the following: First, the current picture is added to the multi-picture buffer with default relative index 0, and the default relative indices of all other pictures are incremented by one. Then, the MMCO commands are processed:

If MMCO indicates a reset of the buffer contents , all pictures in the buffer are marked as “unused” except the current picture (which will be the picture with default relative index 0).

If MMCO indicates a maximum long-term index using MLIP1, all long-term pictures having long-term indices greater than or equal to MLIP1 are marked as “unused” and the default relative index order of the remaining pictures are not affected.

If MMCO indicates that a picture is to be marked as “unused” in the multi-picture buffer and if that picture has not already been marked as “unused”, the specified picture is marked as “unused” in the multi-picture buffer and the default relative indices of all subsequent pictures in default order are decremented by one.

If MMCO indicates the assignment of a long-term index to a specified short-term picture and if the specified long-term index has not already been assigned to the specified short-term picture, the specified short-term picture is marked in the buffer as a long-term picture with the specified long-term index.  If another picture is already present in the buffer with the same long-term index as the specified long-term index, the other picture is marked as “unused”.  All short-term pictures that were subsequent to the specified short-term picture in default relative index order and all long-term pictures having a long-term index less than the specified long-term index have their associated default relative indices decremented by one. The specified picture is assigned to a default relative index of one plus the highest of the incremented default relative indices, or zero if there are no such incremented indices.

## 9.2	Motion compensation

The motion compensation process generates predictions for picture blocks using previously decoded reference pictures. The selected reference picture and motion vectors to be used are described in subclauses 7.4.5 and 7.4.6, respectively. If PSTRUCT indicates a field picture, only the reference field indicated by the ref_idx_fwd or ref_idx_bwd is used in the motion compensation.

### 9.2.1	Fractional sample accuracy

Fractional sample accuracy is indicated by TBDQQ. Depending on the value of the TBDQQ, either quarter sample interpolation with a 6-tap filter or one eighth sample interpolation with an 8-tap filter is performed for all the luminance samples in the block. The prediction process for chrominance samples in both cases is described in subclause 9.2.1.3.

#### 9.2.1.1	Quarter sample luminance interpolation

The positions labelled 'A' in Figure 8-1 below represent reference picture samples in integer positions. Other symbols represent interpolated values at fractional sample positions.

Figure 9-1 – Integer samples ('A') and fractional sample positions for quarter sample luminance interpolation

The prediction values at integer positions shall be obtained by using the samples of the reference picture without alteration. The prediction values at half sample positions shall be obtained by applying a 6-tap filter with tap values (1, -5, 20, 20, -5, 1). The prediction values at quarter sample positions shall be generated by averaging samples at integer and half sample positions. The process for each position is described below.

–	The samples at half sample positions labelled as 'bh' shall be obtained by first calculating intermediate value b applying the 6-tap filter to the nearest samples 'A' at integer positions in horizontal direction. The final value shall be calculated using bh = clip1(((b+16)>>5). The samples at half sample positions labelled as 'bv' shall be obtained equivalently with the filter applied in vertical direction.

–	The samples at half sample positions labelled as 'cm' shall be obtained by applying the 6-tap filter to intermediate values b of the closest half sample positions in either vertical or horizontal direction to form an intermediate result c. The final value shall be calculated using cm = clip1(((c+512)>>10).

–	The samples at quarter sample positions labelled as 'd', 'g', 'e' and 'f' shall be obtained by averaging with truncation the two nearest samples at integer or half sample position using d=(A+bh)>>1, g=(bv+c)>>1, e=(A+bv)>>1, f=(bh+cm)>>1.

–	The samples at quarter sample positions labelled as 'h' shall be obtained by averaging with truncation the closest 'bh' and 'bv' samples in diagonal direction using h = (bh+bv)>>1.

–	The samples at quarter sample positions labelled as 'i' shall be computed using the four nearest samples in integer positions using i = (A1+A2+A3+A4+2)>>2.

#### 9.2.1.2	One eighth sample luminance interpolation

The positions labelled 'A' in the Figure 9-2 represent reference picture samples in integer positions. Other symbols represent interpolated values at fractional sample positions.

Figure 8-2 – Integer samples ('A') and fractional sample locations for one eighth sample luminance interpolation

The samples at half and quarter sample positions shall be obtained by applying 8-tap filters with following coefficients:

–	1/4 position: (-3,  12,  -37,  229,    71,  -21,    6,  -1)

–	2/4 position: (-3,  12,  -39,  158,  158,  -39,  12,  -3)

–	3/4 position: (-1,    6,  -21,    71,  229,  -37,  12,  -3)

The samples at one eighth sample positions are defined as weighted averages of reference picture samples at integer, half and quarter sample positions. The process for each position is described below.

–	The samples at half and quarter sample positions labelled as 'bh' shall be obtained by first calculating an intermediate value b applying 8-tap filtering to the nearest samples 'A' at integer positions in horizontal direction. The final value of 'bh' shall be obtained using bh = clip1(((b+128)>>8). The samples at half and quarter sample positions labelled as 'bv' shall be obtained equivalently with the filter applied in vertical direction.

–	The samples at half and quarter sample positions labelled as 'cm' and 'cq' shall be obtained by 8-tap filtering of the closest intermediate values b in either horizontal or vertical direction to obtain a value c, and then the final result shall be obtained using cm = clip1(((c+32768)>>16) or cq = clip1(((c+32768)>>16).

–	The samples at one eighth sample positions labelled as 'd' shall be obtained by averaging with truncation the two closest 'A', 'b' or 'c' samples using d = (A+bh)>>1, d = (bh0+bh1)>>1, d = (A+bv)>>1, d = (bh+cq)>>1, d = (bv+cq)>>1, d = (cq0+cq1)>>1, d = (bv0+bv1)>>1, or d = (cq+cm)>>1.

–	The samples at one eighth sample positions labelled as 'e' shall be obtained by averaging with truncation the closest 'bh' and 'bv' samples in diagonal direction using e = (bh+bv)>>1.

–	The samples at one eighth sample positions labelled as 'g' shall be obtained from the closest integer samples 'A' and the 'cc' samples using g = (A+3cc+2)>>2.

–	The samples at one eighth sample positions labelled as 'fh' and  'fv' shall be calculated as fh = (3bh+ bv+2)>>2 and  fv = (3bv+ bh+2)>>2.

Figure 9-3 – Diagonal interpolation for one eighth sample luminance interpolation

#### 9.2.1.3 Chrominance interpolation

Motion compensated prediction fractional chrominance samples shall be obtained using Equation 8-1.

v=((s−dx)(s−dy)A+dx(s−dy)B+(s−dx)dyC+dxdyD+s22)s2	(9-1)

where A, B, C and D are the integer position reference picture samples surrounding the fractional sample location; dx and dy are the fractional parts of the sample position in units of one eighth samples for quarter sample interpolation or one sixteenth samples for one eighth sample interpolation; and s is 8 for quarter sample interpolation and is 16 for one eighth sample interpolation. The relationships between the variables in Equation 9-1 and reference picture positions are illustrated in Figure 9-4.

Figure 9-4 – Fractional sample position dependent variables in chrominance interpolation and surrounding integer position samples A, B, C, and D.

## 9.3	Transform Coefficient Decoding

This subclause defines aspects related to transform coefficient decoding.

### 9.3.1	Use of 2-dimensional model for coefficient coding.

The variable-length coding of quantised coefficient values is performed using a 2D model of run-length and coefficient value or an End Of Block (EOB) indication.  This means that an event to be coded (RUN, LEVEL) consists of:

–	RUN	the number of zero coefficients in coefficient scanning order since the last nonzero coefficient.

–	LEVEL	the size of the nonzero coefficient

–	EOB	signals that there are no more nonzero coefficients in the block

### 9.3.2	Coefficient scanning order

Two scan orders are specified for 4x4 block coefficient scanning.  They are the simple zig-zag scan and the double zig-zag scan, as specified in this section.  The zig-zag scan is used when the luminance QP is less than 24, and the double zig-zag scan is used otherwise.

In the case of 16x16 intra macroblocks, the coefficients of the 4x4 luminance DC transform are scanned in the same scan order as ordinary 4x4 coefficient blocks. Then for each 4x4 block of luminance coefficients with AC coefficients to scan, the 15 remaining coefficients are sent by starting the zig-zag or double zig-zag scan at its second position.

The coefficients of the 2x2 chroma DC transform are sent in raster order.  Then for each 4x4 block of chroma coefficients with AC coefficients to scan, the 15 remaining coefficients are sent by starting the zig-zag or double zig-zag scan at its second position.

#### 9.3.2.1	Simple zig-zag scan

Except for Intra coding of luminance with QP<24, simple scan is used.  This is basically zig-zag scanning similar to the one used in H.263.  The scanning pattern is:

Figure 9-5 – Simple zig-zag scan

#### 9.3.2.2	Double zig-zag scan

When using the VLC defined above, we use a one bit code for EOB.  For Inter blocks and Intra with high QP the probability of EOB is typically 50%, which is well matched with the VLC.  In other words this means that we on average have one non-zero coefficient per 4x4 block in addition to the EOB code (remember that a lot of 4x4 blocks only have EOB).  On the other hand, for Intra coding we typically have more than one non-zero coefficient per 4x4 block.  This means that the 1 bit EOB becomes inefficient.  To improve on this the 4x4 block is subdivided into two parts that are scanned separately and with one EOB each.  The two scanning parts are shown below – one of them in bold.

Figure 9-6 – Double zig-zag scan

### 9.3.3	Inverse quantisation and inverse transformation

As specified in subclause QQ 52 different QP values are used, from -12 to +39.  The inverse quantisation equations are defined such that the equivalent step size doubles for every increment of 6 in QP. Thus, there is an increase in step size of about 12% from one QP to the next. For the QP in the range [0…31] the corresponding step size range is about the same as that for H.263. With the range [-12…39], the smallest step size is about four times smaller than in H.263, allowing for very high fidelity reconstruction, and the largest is about 60% larger than in H.263, allowing for more rate control flexibility.

The QP signalled in the bitstream applies for luminance inverse quantisation.  This could be called QPluma.  For chrominance inverse quantisation a different value - QPchroma - is used.  The relation between the two is:

QPluma:	17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39

QPchroma:	17 17 18 19 20 20 21 22 22 23 23 24 24 25 25 25 26 26 26 27 27 27 27

with QPchroma = QPluma if QPluma < 17.

The coefficients R(m,i,j), used in the formulas below, are defined in pseudo code by:

R[m][i][j] = Sm,0 for (i,j) = {(0,0),(0,2),(2,0),(2,2)},

R[m][i][j] = Sm,1 for (i,j) = {(1,1),(1,3),(3,1),(3,3)},

R[m][i][j] = Sm,2 otherwise;

where

S=101613111814132016142318162520182923		(9-2)

#### 9.3.3.1	Luminance DC coefficients in Intra 16x16 macroblock

After decoding the quantised coefficient indices for a 4x4 block of luminance DC coefficients coded in 16x16 intra mode, the inverse transform process shall be applied in a manner mathematically equivalent to the following process.  The process uses application of an inverse transform before the inverse quantisation process.

The inverse transform for the 4x4 luminance DC coefficients in 16x16 intra macroblocks is defined by:

(9-3)

A bitstream conforming to this Specification shall not contain encoded data that results in a value of XQD(i,j) that exceeds the range of integer values from –215 to 215–1, inclusive.

After the inverse transform, inverse quantisation is performed according to the following:

a)	If QP is greater than or equal to zero, then the inverse quantised result shall be calculated as

XD(i,j)=[XQD(i,j)⋅R((QP+12)%6,0,0)]<<[(QP+12)6−2], i,j=0,...,3	(9-4)

b)	If QP is less than zero, then the inverse quantised results shall be calculated as

1)	First, computing

XFB(i,j)=XQD(i,j)⋅R((QP+12)%6,0,0), i,j=0,...,3	(9-5)

2)	Then, computing

Frank(i,j)=2 if XQD(i,j)≥01 otherwise		(9-6)

3)	Then, computing

XD(i,j)=[XFB(i,j)+(Frank(i,j)>>(QP+12)6)]>>(2−(QP+12)6).	(9-7)

A bitstream conforming to this Specification shall not contain encoded data that results in a value of XD(i,j) that exceeds the range of integer values from –215 to 215–1, inclusive.

#### 9.3.3.2	Chrominance DC coefficients

After decoding the quantised coefficient indices for a 2x2 block of chrominance DC coefficients, the inverse transform process is applied before the inverse quantisation process.

Definition of inverse transform:

XQD=111−1i00i01i10i11111−1		(9-8)

A bitstream conforming to this Specification shall not contain encoded data that results in a value of XQD(i,j) that exceeds the range of integer values from –215 to 215–1, inclusive.

After the inverse transform, inverse quantisation is performed according to the following.

a)	If QP is greater than or equal to –6, then the inverse quantised result shall be calculated as

XD(i,j)=[XQD(i,j)⋅R((QP+12)%6,0,0)]<<[(QP+12)6−1], i,j=0,...,3	(9-9)

b)	If QP is less than –6, then the inverse quantised results shall be calculated by

1)	First, computing

XFB(i,j)=XQD(i,j)⋅R((QP+12)%6,0,0), i,j=0,...,3	(9-10)

2)	Then, computing

Frank(i,j)=1 if XQD(i,j)≥00 otherwise		(9-11)

3)	Then, computing

XD(i,j)=(XFB(i,j)+Frank(i,j))>>1	(9-12)

A bitstream conforming to this Specification shall not contain encoded data that results in a value of XD(i,j) that exceeds the range of integer values from –215 to 215–1, inclusive.

#### 9.3.3.3	Residual 4x4 blocks

Inverse quantisation of coefficients other than those as specified in subclauses 9.3.2.1 and 9.3.2.2 is performed according to the following equation:

(9-13)

where the R(m,i,j) are the inverse quantisation coefficients listed below.

A bitstream conforming to this Specification shall not contain encoded data that results in a value of Y'(i,j) that exceeds the range of integer values from –215 to 215–1, inclusive.

After constructing the entire 4x4 block of reconstructed transform coefficients expressed in matrix form as

Y'=y'00y'01y'02y'03y'10y'11y'12y'13y'20y'21y'22y'23y'30y'31y'32y'33 ,		(9-14)

The inverse transform process shall convert the block of reconstructed transform coefficients to a block of output samples in a manner mathematically equivalent to the following process:

a)	First, each row of reconstructed transform coefficients is transformed using a one-dimensional transform, and

b)	Second, each column of the resulting matrix is transformed using the same one-dimensional transform.

The one-dimensional transform is defined as follows for four input samples y0, y1, y2, y3.

a)	First, a set of intermediate values is computed:

z0 = y0 + y2		(9-15)

z1 = y0 – y2		(9-16)

z2 = (y1 >> 1) – y3		(9-17)

z3 = y1 + (y3 >> 1) 		(9-18)

b)	Then the transformed result is computed from these intermediate values

x0 = z0 + z3		(9-19)

x1 = z1 + z2		(9-20)

x2 = z1 – z2		(9-21)

x3 = z0 – z3		(9-22)

A bitstream conforming to this Specification shall not contain encoded data that results in a value of z0, z1, z2, z3, x0, x1, x2, or x3 that exceeds the range of integer values from –215 to 215–1, inclusive, in either the first (horizontal) or second (vertical) stage of application of this transformation process. A bitstream conforming to this Specification shall not contain encoded data that results in a value of  x0, x1, x2, or x3 that exceeds the range of integer values from –215 to 215–33, inclusive, in the second (vertical) stage of application of this transformation process.

After performing the inverse transform in both the horizontal and vertical directions to produce a block of inverse-transformed samples,

X'=x'00x'01x'02x'03x'10x'11x'12x'13x'20x'21x'22x'23x'30x'31x'32x'33		(9-23)

the final reconstructed sample residual values shall be obtained as

X''(i,j)=[X'(i,j)+25]>>6		(9-24)

Finally, the reconstructed sample residual values X''(i,j) are added to the prediction values P(i,j) from motion compensated prediction or spatial prediction and clipped to the range of 0 to 255 to form the final decoded sample result:

S'(i,j) = clip1( P(i,j) + X''(i,j) )		(9-25)

## 9.4	Deblocking Filter

After the reconstruction of a macroblock, a conditional filtering of this macroblock that affects the boundaries of the 4x4 block structure shall be applied. Filtering is done across slice boundaries on a macroblock basis, as soon as all the macroblocks required for filtering are reconstructed.  As the first step, the 16 pels of each of the 4 vertical edges of the 4x4 raster shall be filtered as shown in Figure QQ (horizontal filtering).  Filtering of the 4 horizontal edges follows (vertical filtering). This process also affects the boundaries of the already reconstructed macroblocks above and to the left of the current macroblock (which hence have to be available).  Picture edges are not filtered.

Figure 9-7 – Boundaries in a macroblock to be filtered (luminance boundaries shown with solid lines and chrominance boundaries shown with dotted lines)

Note 1: The intra prediction of a macroblock takes place based on the unfiltered content of the neighbouring macroblocks. Depending on the implementation, the values necessary for intra prediction may need to be stored before filtering in order to be used in the intra prediction of the macroblocks to the right and below the current macroblock.

Note 2: When PSTRUCT indicates a field picture, then all calculations for the deblocking filter are based solely on within the current field.

### 9.4.1	Content dependent boundary filtering strength

For each boundary between neighbouring 4x4 luma blocks, a “Boundary Strength” Bs is assigned as shown in Figure 9-8 that influences the strength of filtering for this particular piece of boundary. As indicated in Figure 9-7, every block boundary of a chroma block corresponds to a specific boundary of a luma block. Bs values for chroma are not calculated, but simply copied from the corresponding luma Bs.

Figure 9-8 – Flow chart for determining the boundary strength (Bs), for the block boundary between two neighbouring blocks p and q, where Ref(p) is the reference frame or field of block p and V(p) is the motion vector of block p [Ed. Note: Figure should be replaced with Visio-format figure – text placement got distorted during copy.]

### 9.4.2	Thresholds for each block boundary

| p3 | p2 | p1 | p0 | q0 | q1 | q2 | q3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 9-9 – Convention for describing samples across 4x4 block horizontal or vertical boundary

In the following description, the set of eight samples across a 4x4 block horizontal or vertical boundary is denoted as shown in Figure 9-9 with the actual boundary lying between p0 and q0. In the default mode, up to two samples on both sides of the boundary can be updated as a result of the filtering process (that is at most p1, p0, q0, q1). Filtering across a certain 4x4 block boundary is skipped altogether if the corresponding Bs is equal to zero. Sets of samples across this edge are only filtered if the condition

Bs ≠ 0  &&  |p0 – q0| <   &&  |p1 – p0| <   &&  |q1 – q0| < 	(9-26)

with the actual boundary lying between p0 and q0. In the default mode up to two samples on both sides of the boundary shall be updated as a result of the filtering process (that is at most p1, p0, q0, and q1). Filtering across a certain 4x4 block boundary is skipped all together if the corresponding Bs is equal to zero. Sets of samples across this edge shall only be filtered if the condition

Bs ≠ 0  &&  |p0 – q0| <   &&  |p1 – p0| <   &&  |q1 – q0| < 	(9-27)

is true.  The values of the thresholds  and  are dependent on the average value of QP for the two macroblocks, as determined by QPav = (QPp+QPq)>>1 is used to determine and . The values for the thresholds are shown in Table 9-1.

Table 9-1 – QPav dependent threshold parameters  and 

|  | QPav |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -12 | -11 | -10 | -9 | -8 | -7 | -6 | -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 4 | 4 | 5 | 6 | 7 | 9 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 3 | 3 | 4 | 4 | 4 |

Table 9-1 (concluded)

|  | QPav |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
|  | 10 | 12 | 14 | 17 | 20 | 24 | 28 | 33 | 39 | 46 | 55 | 65 | 76 | 90 | 106 | 126 | 148 | 175 | 207 | 245 | 255 | 255 | 255 | 255 | 255 | 255 |
|  | 6 | 6 | 7 | 7 | 8 | 8 | 9 | 9 | 10 | 10 | 11 | 11 | 12 | 12 | 13 | 13 | 14 | 14 | 15 | 15 | 16 | 16 | 17 | 17 | 18 | 18 |

### 9.4.3	Default filtering

Two types of filtering are defined. In the default case the equations below are used to filter  p0 and q0.  Here, uppercase letters indicate filtered and lower case letters indicate unfiltered samples with regard to the current edge filtering operation. However, p1 and p2  may indicate samples that have been modified by the filtering of a previous block edge.

 = clip3( -C, C, (((q0 – p0) << 2 + (p1 – q1) + 4) >> 3) )	(9-28)

P0 = clip1( p0+ ) 		(9-29)

Q0 = clip1(q0- )		(9-30)

where C is determined as specified below.

Two intermediate threshold variables

ap = |p2 – p0|		(9-31)

aq = |q2 – q0|		(9-32)

shall be used to determine whether luminance samples p1 and q1 are filtered. These samples are only processed for luminance.

If ap <  for a luminance edge, a filtered sample P1 shall be produced as specified by

P1 = p1 + clip3( -C0, C0, (p2 + P0  – 2*p1)  >>  1)	(9-33)

If aq <  for a luminance edge, a filtered sample Q1 shall be produced as specified by

Q1 = q1 + clip3( -C0, C0, (q2 + Q0  – 2*q1)  >>  1)	(9-34)

where C0 is specified in Table 9-2.

C is determined by setting C equal to C0 and then incrementing C by one if ap<, and again by one if  aq<.

Table 9-2 – Value of filter clipping parameter C as a function of QPav and Bs

|  | QPav |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -12 | -11 | -10 | -9 | -8 | -7 | -6 | -5 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| Bs = 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| Bs = 2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| Bs = 3 or 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Table 9-2 (concluded)

|  | QPav |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| Bs = 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 4 | 5 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 13 |
| Bs = 2 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 7 | 8 | 8 | 10 | 11 | 12 | 13 | 15 | 17 |
| Bs = 3 or 4 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 4 | 5 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 13 | 14 | 16 | 18 | 20 | 23 | 25 |

### 9.4.4	Stronger filtering for some intra coded blocks

In order to improve subjective picture quality in intra coded areas of the picture [Ed. Note: What is an intra-coded area of the picture? Note that intra can happen at an 8x8 level while this is discussing macroblock boundary filtering – is this only for intra macroblocks or is it also for 8x8 intra areas in inter macroblocks?  If one macroblock is intra and the other is not, is that an intra area?] that exhibit small amounts of texture, stronger filtering across macroblock boundaries is applied when the following condition holds:

( Bs == 4 ) && ( ap <  ) && ( aq <  ) && ( 1  <  |p0  – q0|  <  (QPav >> 2))	(9-35)

In this case default [Ed. Note: This "default" versus "non-default" terminology is not appropriate language or structure.  First we get a description of some filtering, then another section saying that in some cases something else is done.  The conditions for each type of filtering should be put up front.] filtering is replaced by a filter defined by the equations:

P0 = ( p2 + 2*p1 + 2*p0 + 2*q0 + q1 + 4) >> 3	(9-36)

P1 = ( p3 + 2*p2 + 2*p1 + 2*p0 + q0 + 4) >> 3	(9-37)

Q0 = ( p1 + 2*p0 + 2*q0 + 2*q1 + q2 + 4) >> 3	(9-38)

Q1 = ( p0 + 2*q0 + 2*q1 + 2*q2 + q3 + 4) >> 3	(9-39)

For luma only, p2 and q2 shall also be filtered as specified by:

P2 = ( 2*p3 + 3*p2 + p1 + p0 + q0 + 4) >> 3	(9-40)

Q2 = ( 2*q3 + 3*q2 + q1 + q0 + p0 + 4) >> 3	(9-41)

NOTE – Q1(blockn) might be used and be overwritten as P2 during the calculation of blockn+1. The same is true for Q2(blockn) which might become P1( blockn+1 ).

## 9.5	Entropy Coding

In the default entropy coding mode, a universal VLC is used to code all syntax. The table of codewords are written in the following compressed form.

1

0 1 x0

0 0 1 x1 x0

0 0 0 1 x2 x1 x0

0 0 0 0 1 x3 x2 x1 x0

.................

where xn take values 0 or 1.  We will sometimes refer to a codeword with its length in bits (L = 2n-1) and INFO = xn,…x1, x0. Notice that the number of bits in INFO is n-1 bits. The codewords are numbered from 0 and upwards.  The definition of the numbering is:

Code_number = 2^L/2 + INFO -1    (L/2 use division with truncation.  INFO = 0 when L = 1) Some of the first code numbers and codewords are written explicitly in Table 9-3.  As an example, for the code number 5, L = 5 and INFO = 10 (binary) = 2 (decimal).

Table 9-3 – UVLC code number and codewords in explicit form

| Code_number | Code word |
| --- | --- |
| 0 | 1 |
| 1 | 0 1 0 |
| 2 | 0 1 1 |
| 3 | 0 0 1 0 0 |
| 4 | 0 0 1 0 1 |
| 5 | 0 0 1 1 0 |
| 6 | 0 0 1 1 1 |
| 7 | 0 0 0 1 0 0 0 |
| 8 | 0 0 0 1 0 0 1 |
| 9 | 0 0 0 1 0 1 0 |
| 10 | 0 0 0 1 0 1 1 |
| ...... | . . . . . . . |

When L (L = 2N-1) and INFO is known, the regular structure of the table makes it easy to create a codeword.  Similarly, a decoder may easily decode a codeword by reading in N bit prefix followed by N-1 INFO.  L and INFO is then readily available.  For each parameter to be coded, there is a conversion rule from the parameter value to the code number (or L and INFO).  Table 9-4 lists the connection between code number and most of the parameters used in the present coding method.

Table 9-4 – Connection between codeword number and parameter values

| Code_ number | RUN | MB_Type |  | 8x8 mode | MVD D-QUANT | CBP |  | Tcoeff_chroma_DC1 |  | Tcoeff_chroma_AC1 Tcoeff_luma1 Simple scan |  | Tcoeff_luma1 Double scan |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Intra | Inter |  |  | Intra | Inter | Level | Run | Level | Run | Level | Run |
| 0 | 0 | Intra4x4 | 16x16 | 8x8 | 0 | 47 | 0 | EOB | - | EOB | - | EOB | - |
| 1 | 1 | 0,0,02 | 16x8 | 8x4 | 1 | 31 | 16 | 1 | 0 | 1 | 0 | 1 | 0 |
| 2 | 2 | 1,0,0 | 8x16 | 4x8 | -1 | 15 | 1 | -1 | 0 | -1 | 0 | -1 | 0 |
| 3 | 3 | 2,0,0 | 8x8 | 4x4 | 2 | 0 | 2 | 2 | 0 | 1 | 1 | 1 | 1 |
| 4 | 4 | 3,0,0 | 8x8 (ref=0) | Intra | -2 | 23 | 4 | -2 | 0 | -1 | 1 | -1 | 1 |
| 5 | 5 | 0,1,0 | Intra4x4 |  | 3 | 27 | 8 | 1 | 1 | 1 | 2 | 2 | 0 |
| 6 | 6 | 1,1,0 | 0,0,02 |  | -3 | 29 | 32 | -1 | 1 | -1 | 2 | -2 | 0 |
| 7 | 7 | 2,1,0 | 1,0,0 |  | 4 | 30 | 3 | 3 | 0 | 2 | 0 | 1 | 2 |
| 8 | 8 | 3,1,0 | 2,0,0 |  | -4 | 7 | 5 | -3 | 0 | -2 | 0 | -1 | 2 |
| 9 | 9 | 0,2,0 | 3,0,0 |  | 5 | 11 | 10 | 2 | 1 | 1 | 3 | 3 | 0 |
| 10 | 10 | 1,2,0 | 0,1,0 |  | -5 | 13 | 12 | -2 | 1 | -1 | 3 | -3 | 0 |
| 11 | 11 | 2,2,0 | 1,1,0 |  | 6 | 14 | 15 | 1 | 2 | 1 | 4 | 4 | 0 |
| 12 | 12 | 3,2,0 | 2,1,0 |  | -6 | 39 | 47 | -1 | 2 | -1 | 4 | -4 | 0 |
| 13 | 13 | 0,0,1 | 3,1,0 |  | 7 | 43 | 7 | 1 | 3 | 1 | 5 | 5 | 0 |
| 14 | 14 | 1,0,1 | 0,2,0 |  | -7 | 45 | 11 | -1 | 3 | -1 | 5 | -5 | 0 |
| 15 | 15 | 2,0,1 | 1,2,0 |  | 8 | 46 | 13 | 4 | 0 | 3 | 0 | 1 | 3 |
| 16 | 16 | 3,0,1 | 2,2,0 |  | -8 | 16 | 14 | -4 | 0 | -3 | 0 | -1 | 3 |
| 17 | 17 | 0,1,1 | 3,2,0 |  | 9 | 3 | 6 | 3 | 1 | 2 | 1 | 1 | 4 |
| 18 | 18 | 1,1,1 | 0,0,1 |  | -9 | 5 | 9 | -3 | 1 | -2 | 1 | -1 | 4 |
| 19 | 19 | 2,1,1 | 1,0,1 |  | 10 | 10 | 31 | 2 | 2 | 2 | 2 | 2 | 1 |
| 20 | 20 | 3,1,1 | 2,0,1 |  | -10 | 12 | 35 | -2 | 2 | -2 | 2 | -2 | 1 |
| 21 | 21 | 0,2,1 | 3,0,1 |  | 11 | 19 | 37 | 2 | 3 | 1 | 6 | 3 | 1 |
| 22 | 22 | 1,2,1 | 0,1,1 |  | -11 | 21 | 42 | -2 | 3 | -1 | 6 | -3 | 1 |
| 23 | 23 | 2,2,1 | 1,1,1 |  | 12 | 26 | 44 | 5 | 0 | 1 | 7 | 6 | 0 |
| 24 | 24 | 3,2,1 | 2,1,1 |  | -12 | 28 | 33 | -5 | 0 | -1 | 7 | -6 | 0 |
| 25 | 25 |  | 3,1,1 |  | 13 | 35 | 34 | 4 | 1 | 1 | 8 | 7 | 0 |
| 26 | 26 |  | 0,2,1 |  | -13 | 37 | 36 | -4 | 1 | -1 | 8 | -7 | 0 |
| 27 | 27 |  | 1,2,1 |  | 14 | 42 | 40 | 3 | 2 | 1 | 9 | 8 | 0 |
| 28 | 28 |  | 2,2,1 |  | -14 | 44 | 39 | -3 | 2 | -1 | 9 | -8 | 0 |
| 29 | 29 |  | 3,2,1 |  | 15 | 1 | 43 | 3 | 3 | 4 | 0 | 9 | 0 |
| 30 | 30 |  |  |  | -15 | 2 | 45 | -3 | 3 | -4 | 0 | -9 | 0 |
| 31 | 31 |  |  |  | 16 | 4 | 46 | 6 | 0 | 5 | 0 | 10 | 0 |
| 32 | 32 |  |  |  | -16 | 8 | 17 | -6 | 0 | -5 | 0 | -10 | 0 |
| 33 | 33 |  |  |  | 17 | 17 | 18 | 5 | 1 | 3 | 1 | 4 | 1 |
| 34 | 34 |  |  |  | -17 | 18 | 20 | -5 | 1 | -3 | 1 | -4 | 1 |
| 35 | 35 |  |  |  | 18 | 20 | 24 | 4 | 2 | 3 | 2 | 2 | 2 |
| 36 | 36 |  |  |  | -18 | 24 | 19 | -4 | 2 | -3 | 2 | -2 | 2 |
| 37 | 37 |  |  |  | 19 | 6 | 21 | 4 | 3 | 2 | 3 | 2 | 3 |
| 38 | 38 |  |  |  | -19 | 9 | 26 | -4 | 3 | -2 | 3 | -2 | 3 |
| 39 | 39 |  |  |  | 20 | 22 | 28 | 7 | 0 | 2 | 4 | 2 | 4 |
| 40 | 40 |  |  |  | -20 | 25 | 23 | -7 | 0 | -2 | 4 | -2 | 4 |
| 41 | 41 |  |  |  | 21 | 32 | 27 | 6 | 1 | 2 | 5 | 2 | 5 |
| 42 | 42 |  |  |  | -21 | 33 | 29 | -6 | 1 | -2 | 5 | -2 | 5 |
| 43 | 43 |  |  |  | 22 | 34 | 30 | 5 | 2 | 2 | 6 | 2 | 6 |
| 44 | 44 |  |  |  | -22 | 36 | 22 | -5 | 2 | -2 | 6 | -2 | 6 |
| 45 | 45 |  |  |  | 23 | 40 | 25 | 5 | 3 | 2 | 7 | 2 | 7 |
| 46 | 46 |  |  |  | -23 | 38 | 38 | -5 | 3 | -2 | 7 | -2 | 7 |
| 47 | 47 |  |  |  | 24 | 41 | 41 | 8 | 0 | 2 | 8 | 11 | 0 |
|  | .. |  |  |  | .. |  |  | .. | .. | .. | .. | .. | .. |

For the entries above the horizontal line, the table is needed for relation between code number and Level/Run/EOB.  For the remaining Level/Run combination there is a simple rule.  The Level/Run combinations are assigned a code number according to the following priority: 1) sign of Level (+ -) 2) Run (ascending) 3) absolute value of Level (ascending).

For 16x16 based intra mode. The 3 numbers refer to values for (Imode,AC,nc) - see qq.

Table 9-5 – Connection between codeword number and Intra Prediction Mode Probability

| Code_ number | Prob0, Prob13 |  | Code_ number | Prob0, Prob13 |  | Code_ number | Prob0, Prob13 |  | Code_ number | Prob0, Prob13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0,0 |  | 21 | 2,3 |  | 41 | 2,6 |  | 61 | 6,5 |
| 1 | 0,1 |  | 22 | 3,2 |  | 42 | 6,2 |  | 62 | 4,7 |
| 2 | 1,0 |  | 23 | 1,5 |  | 43 | 3,5 |  | 63 | 7,4 |
| 3 | 1,1 |  | 24 | 5,1 |  | 44 | 5,3 |  | 64 | 3,8 |
| 4 | 0,2 |  | 25 | 2,4 |  | 45 | 1,8 |  | 65 | 8,3 |
| 5 | 2,0 |  | 26 | 4,2 |  | 46 | 8,1 |  | 66 | 4,8 |
| 6 | 0,3 |  | 27 | 3,3 |  | 47 | 2,7 |  | 67 | 8,4 |
| 7 | 3,0 |  | 28 | 0,7 |  | 48 | 7,2 |  | 68 | 5,7 |
| 8 | 1,2 |  | 29 | 7,0 |  | 49 | 4,5 |  | 69 | 7,5 |
| 9 | 2,1 |  | 30 | 1,6 |  | 50 | 5,4 |  | 70 | 6,6 |
| 10 | 0,4 |  | 31 | 6,1 |  | 51 | 3,6 |  | 71 | 6,7 |
| 11 | 4,0 |  | 32 | 2,5 |  | 52 | 6,3 |  | 72 | 6,7 |
| 12 | 3,1 |  | 33 | 5,2 |  | 53 | 2,8 |  | 73 | 5,8 |
| 13 | 1,3 |  | 34 | 3,4 |  | 54 | 8,2 |  | 74 | 8,5 |
| 14 | 0,5 |  | 35 | 4,3 |  | 55 | 4,6 |  | 75 | 6,8 |
| 15 | 5,0 |  | 36 | 0,8 |  | 56 | 6,4 |  | 76 | 8,6 |
| 16 | 2,2 |  | 37 | 8,0 |  | 57 | 5,5 |  | 77 | 7,7 |
| 17 | 1,4 |  | 38 | 1,7 |  | 58 | 3,7 |  | 78 | 7,8 |
| 18 | 4,1 |  | 39 | 7,1 |  | 59 | 7,3 |  | 79 | 8,7 |
| 19 | 0,6 |  | 40 | 4,4 |  | 60 | 5,6 |  | 80 | 8,8 |
| 20 | 6,0 |  |  |  |  |  |  |  |  |  |

Prob0 and Prob1 defines the Intra prediction modes of two blocks relative to the prediction of prediction modes (see details in the section for Intra coding).

# 10	Context-based Adaptive Binary Arithmetic Coding (CABAC)

## 10.1	Overview

The entropy coding method of context-based adaptive binary arithmetic coding (CABAC) has three distinct elements compared to the default entropy coding method using a fixed, universal table of variable length codes (UVLC):

–	Context modeling provides estimates of conditional probabilities of the coding symbols. Utilizing suitable context models, given inter-symbol redundancy can be exploited by switching between different probability models according to already coded symbols in the neighbourhood of the current symbol to encode.

–	Arithmetic codes permit non-integer number of bits to be assigned to each symbol of the alphabet. Thus the symbols can be coded almost at their entropy rate. This is extremely beneficial for symbol probabilities much greater than 0.5, which often occur with efficient context modeling. In this case, a variable length code has to spend at least one bit in contrast to arithmetic codes, which may use a fraction of one bit.

–	Adaptive arithmetic codes permit the entropy coder to adapt itself to non-stationary symbol statistics. For instance, the statistics of motion vector magnitudes vary over space and time as well as for different sequences and bit rates. Hence, an adaptive model taking into account the cumulative probabilities of already coded motion vectors leads to a better fit of the arithmetic codes to the current symbol statistics.

Figure 10-1 – Generic block diagram of CABAC entropy coding scheme

Next we give a short overview of the main coding elements of the CABAC entropy coding scheme as depicted in Figure 10-1. Suppose a symbol related to an arbitrary syntax element is given, then, in a first step, a suitable model is chosen according to a set of past observations. This process of constructing a model conditioned on neighbouring symbols is commonly referred to as context modeling and is the first step in the entropy coding scheme. The particular context models that are designed for each given syntax model are described in detail in Section 6.2 and Section 6.3. If a given symbol is non-binary valued, it will be mapped onto a sequence of binary decisions, so-called bins, in a second step. The actual binarization is done according to a given binary tree, as specified in Section 6.6. Finally, each binary decision is encoded with the adaptive binary arithmetic coding (AC) engine using the probability estimates, which have been provided either by the context modeling stage or by the binarization process itself. The provided models serve as a probability estimation of the related bins. After encoding of each bin, the related model will be updated with the encoded binary symbol. Hence, the model keeps track of the actual statistics.

## 10.2	Context Modeling for Coding of Motion and Mode Information

In this section we describe in detail the context modeling of our adaptive coding method for the syntax elements macroblock type (MB_type), motion vector data (MVD) and reference frame parameter (ref_idx_fwd).

[Ed. Note: Update to cover 8x8 partitioning of macroblocks]

### 10.2.1	Context Models for Macroblock Type

We distinguish between MB_type for intra and inter frames. In the following, we give a description of the context models which have been designed for coding of the MB_type information in both cases. The subsequent process of mapping a non-binary valued MB_type symbol to a binary sequence in the case of inter frames will be given in detail in section QQ.

#### 10.2.1.1	Intra Pictures

For intra pictures, there are two possible modes for each macroblock, i.e. Intra4x4 and Intra16x16, so that signalling the mode information is reduced to transmitting a binary decision. Coding of this binary decision for a given macroblock is performed by means of context-based arithmetic coding, where the context of a current MB_type C is build by using the MB_types A and B  of neighbouring macroblocks (as depicted in Figure 10-2) which are located in the causal past of the current coding event C. Since A and B are binary decisions, we define the actual context number ctx_mb_type_intra(C) of C by ctx_mb_type_intra(C) = A + B, which results in three different contexts according to the 4 possible combinations of MB_type states for A and B.

In the case of MB_type Intra16x16, there are three additional parameters related to the chosen intra prediction mode, the occurrence of significant AC-coefficients and the coded block pattern for the chrominance coefficients, which have to be signalled. In contrast to the current test model, this information is not included in the mode information, but is coded separately by using distinct models as described in Section 6.6.

Figure 10-2 – Neighbouring symbols A and B used for conditional coding of a current symbol C

#### 10.2.1.2	P and B pictures

[Ed. Note: Frank Bossen reports software may be ahead of text in this section with regard to coding MB type for P and B pictures]

Currently there a 10 different macroblock types for P frames and 18 different macroblock types for B frames, provided that the additional information of the 16x16 Intra mode is not considered as part of the mode information. Coding of a given MB_type information C is done similar to the case of intra frames by using a context model which involves the MB_type information A and B of previously encoded (or decoded) macroblocks (see Figure 10-2). However, here we only use the information whether the neighbouring macroblocks of the given macroblock are of type Skip (P frame) or Direct (B frame), such that the actual context number ctx_mb_type_inter(C) is given in C-style notation for P-frame coding by ctx_mb_type_inter(C) = ((A==Skip)?0:1) + ((B==Skip)?0:1) and by  ctx_mb_type_inter(C) = ((A==Direct)?0:1) + ((B==Direct)?0:1) for B-frame coding. Thus, we obtain 3 different contexts, which, however, are only used for coding of the first bin of the binarization b(C) of C, where the actual binarization of C will be performed as outlined in Section 6.6. For coding the second bin, a separate model is provided and for all remaining bins of b(C) two additional models are used as further explained in Section 6.6. Thus, a total number of 6 different models are supplied for coding of macroblock type information relating to P-and B frames.

(a)								(b)

Figure 10-3 – Illustration of the encoding process for a given residual motion vector component mvdk(C) of a block C: (a) Context selection rule. (b) Separation of mvdk(C) into sign and magnitude, binarization of the magnitude and assignment of context models to bin_nos.

### 10.2.2	Context Models for Motion Vector Data

Motion vector data consists of residual vectors obtained by applying motion vector prediction. Thus, it is a reasonable approach to build a model conditioned on the local prediction error. A simple measure of the local prediction error at a given block C is given by evaluating the L1-norm   of two neighbouring motion vector prediction residues  and   for each component of a motion vector residue   of a given block, where A and B are neighbouring blocks of block C, as shown in  Figure 17(a). If one of the neighbouring blocks belongs to an adjacent macroblock, we take the residual vector component of the leftmost neighbouring block in the case of the upper block B, and in the case of the left neighbouring block A we use the topmost neighbouring block. If one of the neighbouring blocks is not available, because, for instance, the current block is at the picture boundary, we discard the corresponding part of  . By using  , we now define a context model ctx_mvd(C,k) for the residual motion vector component   consisting of three different context models:

For the actual coding process, we separate   in sign and modulus, where only the first bin of the binarization of the modulus   is coded using the context models ctx_mvd(C,k). For the remaining bins of the modulus, we have 4 additional models: one for the second, one for the third bin, one for the fourth, and one model for all remaining bins. In addition, the sign coding routine is provided with a separate model. This results in a total sum of 8 different models for each vector component.

In the case of B-frame coding, an additional syntax element has to be signalled when the bi-directional mode is chosen. This element represents the block size (Blk_size), which is chosen for forward or backward motion prediction. The related code number value ranges between 0 and 6 according to the 7 possible block shapes in Figure 2 [Ed. Note: Are there still seven possible macroblock partitions?]. Coding of Blk_size is done by using the binarization of the P_MB_type as described in Section 6.6.

### 10.2.3	Context Models for Reference Frame Parameter

If the option of temporal prediction from more than one reference frame is enabled, the chosen reference frame for each macroblock must be signalled. Given a macroblock and its reference frame parameter as a symbol C according to the definition in Section QQ, a context model is built by using symbols A and B of the reference frame parameter belonging to the two neighbouring macroblocks (see Figure 16). The actual context number of C is then defined by ctx_ref_frame(C) = ((A==0)?0:1) + 2*((B==0)?0:1), such that ctx_ref_frame(C) indicates one of four models  used for coding of the first bin of the binary equivalent b(C) of C. Two additional models are given for the second bin and all remaining bins of b(C), which sums up to a total number of six different models for the reference frame information.

## 10.3	Context Modeling for Coding of Texture Information

This section provides detailed information about the context models used for the syntax elements of coded block pattern (CBP), intra prediction mode (IPRED) and (RUN, LEVEL) information.

### 10.3.1	Context Models for Coded Block Pattern

Except for MB_type Intra16x16, the context modeling for the coded block pattern is treated as follows. There are 4 luminance CBP bits belonging to 4 8x8 blocks in a given macroblock. Let C denote such a Y-CBP bit, then we define ctx_cbp_luma(C) = A + 2*B, where A and B are Y-CBP bits of the neighbouring 8x8 blocks, as depicted in Figure 16. The remaining 2 bits of CBP are related to the chrominance coefficients. In our coding approach, these bits are translated into two dependant binary decisions, such that, in a first step, we send a bit cbp_chroma_sig which signals whether there are significant chrominance coefficients at all. The related context model is of the same kind as that of the Y-CBP bits, i.e. ctx_cbp_chroma_sig(C) = A + 2*B, where A and B are now notations for the corresponding cbp_chroma_sig bits of neighbouring macroblocks. If cbp_chroma_sig = 1 (non-zero chroma coefficients exist), a second bit cbp_chroma_ac related to the significance of AC chrominance coefficients has to be signalled. This is done by using a context model conditioned on the cbp_chroma_ac decisions A and B of neighbouring macroblocks, such that ctx_cbp_chroma_AC(C) = A + 2*B. Note, that due to the different statistics there are different models for Intra and Inter macroblocks, so that the total number of different models for CBP amounts to 2*3*4=24. For the case of MB_type Intra16x16, there are three additional models, one for the binary AC decision and two models for each of the two chrominance CBP bits.

### 10.3.2	Context Models for Intra Prediction Mode

In Intra4x4 mode, coding of the intra prediction mode C of a given block is conditioned on the intra prediction mode of the previous block A to the left of C (see Figure 16). In fact, it is not the prediction mode number itself which is signalled and which is used for conditioning but rather its predicted order similar as it is described in subclause 4.4.4. There are six different prediction modes and for each mode, two different models are supplied: one for the first bin of the binary equivalent of C and the other for all remaining bins. Together with two additional models for the two bits of the prediction modes of MB_type Intra16x16 (in binary representation), a total number of 14 different models for coding of intra prediction modes is given.

### 10.3.3	Context Models for Run/Level and Coeff_count

Coding of RUN and LEVEL is conditioned primarily on the scanning mode, the DC/AC block type, the luminance/chrominance, and the intra/inter macroblock decision. Thus, a total number of 8 different block types are given according to Table 10-1. In contrast to the UVLC coding mode, RUN and LEVEL are coded separately in the CABAC mode, and furthermore an additional coding element called COEFF_COUNT is introduced, which denotes the number of non-zero coefficients in a given block. The coefficients corresponding to a scanning mode are processed in the following way: First, the number of non-zero coefficients COEFF_COUNT is encoded. Then, in a second step, RUN, LEVEL and SIGN of all non-zero coefficients are encoded. Figure 18 illustrates this encoding scheme that is described in more detail in the following sections.

Table 10-1 –  Numbering of the different block types used for coding of RUN, LEVEL and COEFF_COUNT

| Ctx_run_level_coeff_count | Block Type |
| --- | --- |
| 0 | Double Scan (Intra Luma only) |
| 1 | Single Scan (Inter Luma only) |
| 2 | Intra Luma 16x16, DC |

Figure 10-4 – Coding scheme for transform coefficients

#### 10.3.3.1	Context-based Coding of COEFF_COUNT Information

For capturing the correlations between COEFF_COUNTs of neighbouring blocks appropriate context models are designed, which are only applied to the first binary decision (bin) in the unary binarization tree. More specifically, first the COEFF_COUNT is classified according the block type and then a local context for the first bin is built according to the rules specified in Table 10. The remaining bins are coded with two separate models; one for the second and the other for all remaining bins.

Table 10-2 – Description of the context formation for the first bin for coding of COEFF_COUNT
(CC denotes the COEFF_COUNT of the corresponding blocks)

| Block Type | Number of ctx’s | Description of the context formation for the first bin | Comments |
| --- | --- | --- | --- |
| 0 | 3 | ctx_coeff_count(A) =0; ctx_coeff_count(B) = (CC(A)==0) ? 1 : 2; | A: upper scan path; B: lower scan path |
| 1 | 4 | ctx_coeff_count(C) = ((CC(A)==0) ? 0 : 1) + 2*(( CC(B)==0)?0:1); | A, B, C as in QQ |
| 2 | 1 | Only one ctx |  |
| 3 | 3 | ctx_coeff_count(0) =0; ctx_coeff_count(i) = (CC(i-1)==0) ? 1 : 2; i={1,..,15} | i denotes the number of the 4x4 block as in Figure QQ |
| 4, 5, 6, 7 | 2 | ctx_coeff_count(U) == 0; ctx_coeff_count(V) == 1; | U, V : the different chroma parts |

#### 10.3.3.2	Context-based Coding of RUN Information

For encoding the RUN, the information of the initially encoded COEFF_COUNT and all previously encoded RUN elements is exploited, as shown in the pseudo-C code of Figure 18. Here, we make use of the fact that, if the maximum number of a RUN range is known in advance, the binarization can be restricted to a truncated code tree, which is explained in section 4.5.2.3.5. However, by using the information given by previously encoded RUNs the MaxRun counter can further be adapted on the fly, which may lead to potentially shorter binarized codewords of subsequent RUN elements to encode. In some cases, where, for instance, coefficients are aggregated at the end of the scan path, signaling of zero-valued RUN elements is completely omitted.

Context models for coding of RUN information depend on a threshold decision involving mostly the coefficient counter, e.g. in 4x4 single scan inter coding mode, the context model ctx_run for luma is given by ctx_run = ((COEFF_COUNT) >= 4) ? 1 : 0. For a more detailed description refer to Table 11. The idea behind this context design is that the COEFF_COUNT represents the activity of the given block, and that the probability distribution of the RUN symbol somehow depends on the activity of the block. First, the RUN is classified according the given block type, then a context ctx_run is chosen according to Table 11. For each context ctx_run two separate models are provided for the coding of RUN; one model for the first bin and the second model for all remaining bins of the binary codeword related to RUN.

Table 10-2 – Description of the context formation for RUNs
(the variable i denotes the coefficient counter (0 ≤ i ≤COEFF_COUNT-1)

| Block Type | Description of the context formation for the runs) |
| --- | --- |
| 0,3,6,7 | ctx_run = ((COEFF_COUNT-i) >= 3) ? 1 : 0; |
| 1 | ctx_run = (COEFF_COUNT >= 4) ? 1 : 0; |
| 2 | ctx_run = ((COEFF_COUNT-i) >= 4) ? 1 : 0; |
| 4,5 | ctx_run = ((COEFF_COUNT-i) >= 2) ? 1 : 0; |

#### 10.3.3.3	Context-based Coding of LEVEL Information

LEVEL information is first separated into sign and magnitude. Then the magnitude and sign of LEVEL, are first classified based on the block type. For coding of ABS_LEVEL=abs(LEVEL) context models depending on the previously encoded or decoded ABS_LEVEL within a given block are applied. More specifically, the following context model is defined for bins of the magnitude ABS_LEVEL:

if (bin_nr>3)

bin_nr=3;

end

if (Prev_level>MAX_LEVEL)

Prev_level=MAX_LEVEL;

end

context_nr = (bin_nr-1)*MAX_LEVEL+Prev_level,

where MAX_LEVEL=3, and where the Prev_level is initialized to zero at the beginning of each block and updated after each en-/decoding step by the most recently en-/decoded ABS_LEVEL value. Note that for double-scan, Prev_level is initialized at the beginning of each scan, i.e., twice per block.

## 10.4	Double Scan Always for CABAC Intra Mode

In the UVLC mode, the scan mode for intra coding depends on the QP value: for QP < 24 the double scan is used, whereas for QP24 the single scan mode is employed. In the contrast to that, CABAC entropy coding always uses the double scan mode, independently of the given QP value.

## 10.5	Context Modeling for Coding of Dquant

For a given macroblock, the value of Dquant is first mapped to a positive value using the arithmetic wrap as described in subclause QQ. This wrapped value C is then coded conditioned on the corresponding Dquant A of the left neighbouring macroblock, such that that ctx_dquant(C)=(A !=0);  This results in 2 context models for the first bin. Two additional models are used for the second and all remaining bins of the binarized value C.  Thus, a total sum of four context models are used for Dquant.

Table 10-3 – Binarization by means of the unary code tree

| Code symbol | Binarization |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 |  |  |  |  |  |  |  |
| 1 | 0 | 1 |  |  |  |  |  |  |
| 2 | 0 | 0 | 1 |  |  |  |  |  |
| 3 | 0 | 0 | 0 | 1 |  |  |  |  |
| 4 | 0 | 0 | 0 | 0 | 1 |  |  |  |
| 5 | 0 | 0 | 0 | 0 | 0 | 1 |  |  |
| 6 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |
| . . . | . | . | . | . | . | . | . | . |
| Bin_no. | 1 | 2 | 3 | 4 | 5 | 6 | 7 | … |

## 10.6	Binarization of Non-Binary Valued Symbols

A non-binary valued symbol will be decomposed into a sequence of binary decisions. Except for the MB_type syntax element we use the binarization given by the unary code tree in Table 12.

For the binary decomposition of the MB_type symbols of P or B frames, which are of limited range (0 … 9) or (0 … 17) respectively, an alternative binarization is used, which is shown in Table 10-4.

Table 10-4 – (a) Binarization for P-frame MB_type and (b) for B-frame MB_type

| P_MB_type | Binarization |  | B_MB_type | Binarization |
| --- | --- | --- | --- | --- |
| 0 | 0 |  | 0 | 0 |
| 1 | 1 0 0 |  | 1 | 1 0 0 |
| 2 | 1 0 1 |  | 2 | 1 0 1 |
| 3 | 1 1 0 0 0 |  | 3 | 1 1 0 0 0 |
| 4 | 1 1 0 0 1 |  | 4 | 1 1 0 0 1 |
| 5 | 1 1 0 1 0 |  | 5 | 1 1 0 1 0 |
| 6 | 1 1 0 1 1 |  | 6 | 1 1 0 1 1 |
| 7 | 1 1 1 0 0 |  | 7 | 1 1 1 0 0 0 0 |
| 8 | 1 1 1 0 1 |  | . | . |
| 9 | 1 1 1 1 0 |  | 17 | 1 1 1 1 0 1 0 |
| Bin_no | 1 2 3 4 5 |  | Bin_no | 1 2 3 4 5 6 7 |

### 10.6.1	Truncated Binarization for RUN, COEFF_COUNT and Intra Prediction Mode

If the maximum number of a given syntax element is known in advance, the binarization can be restricted to a truncated code tree. Suppose the alphabet size of a specific syntax element is fixed, then for the unary binarization of the maximum value of this syntax element the terminating “1” can be omitted. This mechanism applies for RUN, COEFF_COUNT and Intra Prediction Mode.

## 10.7	Adaptive Binary Arithmetic Coding

At the beginning of the overall encoding of a given frame the probability models associated with all 126 different contexts are initialized with a pre-computed start distribution. For each symbol to encode the frequency count of the related binary decision is updated, thus providing a new probability estimate for the next coding decision. However, when the total number of occurrences of a given model exceeds a pre-defined threshold, the frequency counts will be scaled down. This periodical rescaling exponentially weighs down past observations and helps to adapt to the non-stationarity of a source.

The binary arithmetic coding engine used in our presented approach is a straightforward implementation similar to that given in "Arithmetic Coding Revisited", Moffat, Neal, Witten. ACM Transactions on Information Systems, 16(3):256-294, July 1998.

[Editor: This must be improved by the area assistant editor.]

# 11	B-pictures

[Ed. Note: Consider moving into main body of document.  Still needs considerable work to complete "generalized ERPS" incorporation.]

## 11.1	Introduction

The use of B pictures is indicated in PTYPE.  The B pictures are predicted from either or both the previous and subsequent reconstructed pictures to achieve improved coding efficiency as compared to that of P pictures.  Figure 11-1 illustrates the predictive structure with two B pictures inserted between I and P pictures.

Figure 11-1 – Illustration of B picture concept

[Ed. Note: Picture should be replaced to use zero-based indexing for consistency.  Should be in Visio format for editing.]

The location of pictures in the bitstream is in a data-dependence order rather than in temporal order.  Pictures that are dependent on other pictures shall be located in the bitstream after the pictures on which they depend.  For example, for the picture prediction structure illustrated Figure 11-1, pictures B2 and B3 are dependent on I1 and P4, and B5 and B6 are dependent on P4 and P7.  Therefore the bitstream syntax order of the encoded pictures could be I1, P4, B2, B3, P7, B5, B6, … for the display order of I1, B2, B3, P4, B5, B6, P7, …

## 11.2	Macroblock modes and 8x8 sub-partition modes

There are five different prediction types supported by B pictures.  They are the direct, forward, backward, bi-directional and intra prediction modes.  Both direct mode and bi-directional mode are bi-directional prediction. The only difference is that the bi-directional mode uses separate motion vectors for forward and backward prediction, whereas the forward and backward motion vectors of the direct mode are derived from the motion vectors used in the corresponding macroblocks of the subsequent reference frame.  In the direct mode, the same number of motion vectors as in the co-located macroblock of the temporal following reference picture is used. To calculate prediction blocks for the direct and bi-directional prediction mode, the forward and backward motion vectors are used to obtain appropriate blocks from reference frames and then these blocks are averaged by dividing the sum of the two prediction blocks by two.

Forward prediction means prediction from a previous reference picture, and backward prediction means prediction from a temporally subsequent reference picture. [Ed. Note: That statement is not true in this codec.]

The intra prediction means to encode the macroblock by using intra coding.

For B frames, a similar tree-structured macroblock partitioning as in P-pictures is used. As in P frames, one reference frame parameter is transmitted for each 16x16, 16x8, and 8x16 block as well as for each 8x8 sub-partition. Additionally, for each 16x16, 16x8, 8x16 block, and each 8x8 sub-partition, the prediction direction (forward, backward, bi-directional) can be chosen separately. For avoiding a separate code word to specify the prediction direction, the indication of the prediction direction is incorporated into the codewords for macroblock modes and 8x8 partitioning modes, respectively, as shown in the table Table 12 and Table 13. Moreover, an 8x8 sub-partition of a B-frame macroblock can also be coded in Direct mode.

Table 11-1 – Macroblock modes for B frames

| Code number | Macroblock mode | 1. block | 2. block |
| --- | --- | --- | --- |
| 0 | Direct |  |  |
| 1 | 16x16 | Forw. |  |
| 2 | 16x16 | Backw. |  |
| 3 | 16x16 | Bidirect. |  |
| 4 | 16x8 | Forw. | Forw. |
| 5 | 8x16 | Forw. | Forw. |
| 6 | 16x8 | Backw. | Backw. |
| 7 | 8x16 | Backw. | Backw. |
| 8 | 16x8 | Forw. | Backw. |
| 9 | 8x16 | Forw. | Backw. |
| 10 | 16x8 | Backw. | Forw. |
| 11 | 8x16 | Backw. | Forw. |
| 12 | 16x8 | Forw. | Bidirect. |
| 13 | 8x16 | Forw. | Bidirect. |
| 14 | 16x8 | Backw. | Bidirect. |
| 15 | 8x16 | Backw. | Bidirect. |
| 16 | 16x8 | Bidirect. | Forw. |
| 17 | 8x16 | Bidirect. | Forw. |
| 18 | 16x8 | Bidirect. | Backw. |
| 19 | 8x16 | Bidirect. | Backw. |
| 20 | 16x8 | Bidirect. | Bidirect. |
| 21 | 8x16 | Bidirect. | Bidirect. |
| 22 | 8x8(split) |  |  |
| 23 | Intra4x4 |  |  |
| 24 … | Intra16x16 |  |  |

Table 11-2 – Modes for 8x8 sub-partitions in B frames

| Code number | 8x8 partition mode | Prediction |
| --- | --- | --- |
| 0 | Direct |  |
| 1 | 8x8 | Forw. |
| 2 | 8x8 | Backw. |
| 3 | 8x8 | Bidirect. |
| 4 | 8x4 | Forw. |
| 5 | 4x8 | Forw. |
| 6 | 8x4 | Backw. |
| 7 | 4x8 | Backw. |
| 8 | 8x4 | Bidirect. |
| 9 | 4x8 | Bidirect. |
| 10 | 4x4 | Forw. |
| 11 | 4x4 | Backw. |
| 12 | 4x4 | Bidirect. |
| 13 | Intra |  |

## 11.3	B-Picture Syntax

Some additional syntax elements are needed for B pictures. The structure of B picture related fields is shown in Figure 20.  On the Ptype, two picture types shall be added to include B pictures with and without multiple reference frame prediction.  On the MB_type, different macroblock types are defined to indicate the different prediction types for B pictures.  The fields of MVDFW, and MVDBW are inserted to enable bi-directional prediction. These fields replace the syntax element MVD.

Figure 11-2 – Syntax diagram for B pictures [Ed. Note: Add PSTRUCT]

### 11.3.1	Picture type (Ptype) ), Picture Structure (PSTRUCT) and RUN

See subclause QQ for definition.

### 11.3.2	Macroblock type (MB_type) and 8x8 sub-partition type

Table QQ shows the macroblock modes for B pictures.

In “Direct” prediction type, no motion vector data is transmitted.

In NxM mode, each NxM block of a macroblock is predicted by using different motion vectors, reference frames, and prediction directions. As indicated in Table 12, three different macroblock modes that differ in their prediction directions exist for the 16x16 mode. For the 16x8 and 8x16 macroblock modi, 9 different combinations of the prediction directions are possible. If a macroblock is transmitted in 8x8 mode, an additional codeword for each 8x8 sub-partition indicates the decomposition of the 8x8 block as well as the chosen prediction direction (see Table 13).

The MBintra4x4” and MBintra16x16” prediction type indicates that the macroblock is encoded by intra coding with different intra prediction modes which are defined in the same manner as in subclause qq and Table 9-4.  No motion vector data is transmitted for the intra macroblocks or 8x8 sub-partition coded in intra mode.

### 11.3.3	Intra prediction mode (Intra_pred_mode)

As present, Intra_pred_mode indicates which intra prediction mode is used.  Intra_pred_mode is present when MBintra4x4 prediction type is indicated in the MB_type or BlockIntra4x4 is indicated in region8x8_type.  The code_number is same as that described in the Intra_pred_mode entry of Table 9-4.

### 11.3.4	Reference picture parameters (ref_idx_fwd and ref_idx_bwd)

The ref_idx_fwd parameter indicates the position of the reference picture in the forward reference set to be used for forward motion-compensated prediction of the current macroblock. The ref_idx_fwd parameter is transmitted for each 16x16, 16x8, 8x16 block, and each 8x8 sub-partition if forward or bi-directional prediction is used and if multi-frame forward prediction is in use. ref_idx_fwd is present only when the Ptype signals the use of multiple forward reference frames. The code_number and interpretation for ref_idx_fwd differs for PSTRUCT indicating field picture or frame picture and is the same as the definition for ref_idx_fwd in subclause QQ.

Similarly, ref_idx_bwd is the index into the backward reference set to determine which frame or field is used for backward prediction when multi-frame backward prediction is in use.  ref_idx_bwd is present only when Ptype indicates the use of multiple backward reference frames.  ref_idx_bwd is used in the same manner as ref_idx_fwd, but is an index into the backward reference set rather than the forward reference set.

### 11.3.5	Motion vector data (MVDFW, MVDBW)

MVDFW is the motion vector data for the forward vector, if present.  MVDBW is the motion vector data for the backward vector, if present.  If so indicated by MB_type and/or 8x8 sub-partition type, vector data for 1-16 blocks are transmitted.  The order of transmitted motion vector data is the same as that indicated in Figure 2. For the code_number of motion vector data, please refer to Table 9-4.

## 11.4	Decoder Process for motion vector

### 11.4.1	Differential motion vectors

Motion vectors for forward, backward, or bi-directionally predicted blocks are differentially encoded.  A prediction has to be added to the motion vector differences to get the motion vectors for the macroblock.  The predictions are formed in way similar to that described in section 4.4.6.  The only difference is that forward motion vectors are predicted only from forward motion vectors in surrounding macroblocks, and backward motion vectors are predicted only from backward motion vectors in surrounding macroblocks.

If a neighbouring macroblock does not have a motion vector of the same type, the candidate predictor for that macroblock is set to zero for that motion vector type.

### 11.4.2	Motion vectors in direct mode

In direct mode the same block structure as for the co-located macroblock in the temporally subsequent picture is used. For each of the sub-blocks the forward and backward motion vectors are computed as scaled versions of the corresponding vector components of the co-located macroblock in the temporally subsequent picture as described below.

If the multiple reference frame prediction is used, the forward reference frame for the direct mode is the same as the one used for the corresponding macroblock in the temporally subsequent reference picture. The forward and backward motion vectors for direct mode macroblocks are calculated differently depending on whether PSTRUCT and the reference are fields or frames. Also note that if the subsequent reference picture is an intra-coded frame or the reference macroblock is an intra-coded block, the motion vectors are set to zero. With possible adaptive switch of frame/field coding at picture level, a B frame or its future reference frame can be coded in either frame structure or field structure. Hence, there can be four different combinations of frame or field coding for a pair of a MB in B and its collocated MB in the future reference. Calculations of the two MVs in direct mode are slightly different for the four cases.

Case 1: Both the current MB and its collocated are in frame mode

Both the current B and its future reference are in frame structure, as shown in Figure 4. The forward reference is the frame pointed by the forward MV of the collocated MB and the backward reference is the immediate future reference frame of I or P. Two MVs ( ) are calculated by (see Figure 4)

MVF=TRB⋅MVTRDMVB=(TRB−TRD)⋅MVTRD

where  is the temporal distance between the current B frame and the reference frame pointed by the forward MV of the collocated MB, and   is the temporal distance between the future reference frame and the reference frame pointed by the forward MV of the collocated MB.

Figure 11-3 – Both the current MB in B and its collocated MB in the future reference of I or P are in frame mode. Solid-line is for frames and dot-line for fields. f1 stands for field 1 and f2 for field 2 [Ed. Note: Strange title for a Figure]

Case 2: Both the current MB and its collocated MB are in field mode

Both the current B frame and its future reference are in field structure. Two MVs of direct mode MB in B field are calculated from the forward MV of the collocated MB in the future reference field of the same parity. For field 1, the forward MV of the collocated MB will always point to one of the previously coded I or P fields, as shown in Figure 5. The forward reference field for the direct mode MB will be the same as pointed by the forward MV of the collocated MB, and the backward reference field will be field 1 of the future reference frame. The forward and backward MVs   for the direct mode MB are calculated as follows,

MVF,i=TRB,i⋅MViTRD,iMVB,i=(TRB,i−TRD,i)⋅MViTRD,i

where the subscript,  , is the field index (1 for the 1st field and 2 for the 2nd field), and   is the forward MV of the collocated MB in field   of the future reference frame.   is the temporal distance between the current B field ( ) and the reference field pointed by the forward MV of the collocated MB in field interval,  is the temporal distance between the future reference field ( ) and the reference field pointed by the forward MV of the collocated MB in field interval.

Figure 11-4 – Both the current MB in B and its collocated MB in the future reference of I or P are in field mode. Solid-line is for frames and dot-line for fields. f1 stands for field 1 and f2 for field 2. The forward MV of the collocated MB in field 1 of the future reference always points to one of the previously coded I or P fields.

For field 2, the forward MV of the collocated MB may point to one of the previously coded I or P fields. Calculation of the forward and backward MVs therefore follows the same equation (2) above. However, it is also possible that the forward MV of the collocated MB in field 2 of the future reference frame points to field 1 of the same frame, as shown in Figure 6. In this case, the two MVs   of the direct mode MB are calculated as follows:

MVF,2=−TRB,2⋅MV2TRD,2MVB,2=−(TRB,2+TRD,2)⋅MV2TRD,2

Note that both MVs   are now backward MVs, pointing to field 1 and field 2 of the future reference frame respectively.

Figure 11-5 – Both the current MB in B and its collocated MB in the future reference of I or P are in field mode. Solid-line is for frames and dot-line for fields. f1 stands for field 1 and f2 for field 2.  The forward MV of the collocated MB in field 2 of the future reference frame may point to field 1 of the same frame.

Case 3:The current MB is field mode and its collocated MB in frame mode

The current B is in field structure while its future reference is in frame structure, as shown in Figure 7. Let   and   be the notations of the vertical indexes of the current MB and its collocated MB respectively. Then,  QQ. Two MVs of direct mode MB are calculated from the forward MV of the collocated MB in the future reference frame as follows

MVF,i=TRB,i⋅MVTRDMVB,i=(TRB,i−TRD)⋅MVTRD

Since the collocated MB is frame coded,   in (4) is derived by dividing the associated frame-based MV by 2 in vertical direction. Forward reference field of direct mode MB is the same parity field in the reference frame used by its collocated MB. Backward reference field is the same parity field of the future reference frame.

Figure 11-6 – The current MB in B is in field mode while its collocated MB in the future reference of I or P in frame mode. Solid-line is for frames and dot-line for fields. f1 stands for field 1 and f2 for field 2

Case 4: The current MB is in frame mode while its collocated MB in fieldmode

The current B is in frame structure while its future reference is in field structure, as shown in Figure 8. Let   and   be the notations of the vertical indexes of the current MB and its collocated MB respectively. Then,  . The two fields of the collocated MB in the future reference may be coded in different modes. Since field 1 of the future reference is temporally close to the current B, the collocated MB in field 1 of the future reference provides certain basis for the current MB. Hence, the collocated MB in field 1 of the future reference is used in calculating the MVs and determining the references for direct mode. Two frame-based MVs   of direct mode MB are calculated as follows

MVF=TRB⋅MV1TRD,1MVB=(TRB−TRD,1)⋅MV1TRD,1

where   is derived by doubling the field-based MV of the collocated MB in field 1 of the future reference in vertical direction,   is the temporal distance between the reference frame of one of whose fields is pointed by the forward MV of the collocated MB in field 1 of the future reference and the current B frame in field interval, and   is the temporal distance between field 1 of the future reference and the reference field pointed by the forward MV of the collocated MB in field 1 of future reference. Forward reference frame of the direct mode MB is the frame of one of whose fields is pointed by the forward field MV of the collocated MB in field 1 of the future reference frame. Backward reference frame is the future reference frame.

Figure 11-7 – The current MB in B is in frame mode while its collocated MB in the future reference of I or P is in field mode. Solid-line is for frames and dot-line for fields. f1 stands for field 1 and f2 for field 2.

# 12	SP-pictures

There are two types of S-pictures, namely SP-pictures and SI-pictures. SP-pictures make use of motion-compensated predictive coding to exploit temporal redundancy in the sequence similar to P-pictures and SI-pictures make use of spatial prediction similar to I-pictures. Unlike P-pictures, however, SP-picture coding allows identical reconstruction of a frame even when different reference frames are being used. SI-picture, on the other hand, can identically reconstruct a corresponding SP-picture. These properties of S-pictures provide functionalities for bitstream switching, splicing, random access, VCR functionalities such as fast-forward and error resilience/recovery.

## 12.1	Syntax

The use of S-pictures is indicated by PTYPE. If PTYPE indicates an S-picture, quantisation parameter PQP is followed by an additional quantisation parameter SPQP in the slice header, see Section QQ. The rest of the syntax elements for SP-pictures are the same as those in Inter-frames. Similarly, the rest of the syntax elements for SI-pictures are the same as those in I-pictures.

### 12.1.1	Picture type (Ptype) and RUN

See Section QQ for definition.

### 12.1.2	Macro block type (MB_Type)

MB_Type indicates the prediction mode and the block size used to encode each macroblock. There are different MB types for SP and SI-pictures.

### 12.1.3	Macroblock modes for SI-pictures

The MB types for SI-pictures are similar to those of I-pictures with an additional MB type, called SIntra 4x4. Table 12-1 depicts the relationship between the code numbers and MB_Type for SI-pictures.

SIntra 4x4	SI Mode.

Intra 4x4	4x4 Intra coding.

Imode, nc, AC	See definition in Section QQ. These modes refer to 16x16 intra coding.

Table 12-1 – MB Type for SI-Pictures

| Code_number | MB_Type (SI-pictures) |
| --- | --- |
| 0 | SIntra 4x4 |
| 1 | Intra 4x4 |
| 2 | 0,0,0 |
| 3 | 1,0,0 |
| 4 | 2,0,0 |
| 5 | 3,0,0 |
| 6 | 0,1,0 |
| 7 | 1,1,0 |
| 8 | 2,1,0 |
| 9 | 3,1,0 |
| 10 | 0,2,0 |
| 11 | 1,2,0 |
| 12 | 2,2,0 |
| 13 | 3,2,0 |
| 14 | 0,0,1 |
| 15 | 1,0,1 |
| 16 | 2,0,1 |
| 17 | 3,0,1 |
| 18 | 0,1,1 |
| 19 | 1,1,1 |
| 20 | 2,1,1 |
| 21 | 3,1,1 |
| 22 | 0,2,1 |
| 23 | 1,2,1 |
| 24 | 2,2,1 |
| 25 | 3,2,1 |

1 16x16 based intra mode. The 3 numbers refer to values for (Imode,AC,nc) - see QQ.

### 12.1.4	Macroblock Modes for SP-pictures

The MB types for SP-pictures are identical to those of P-pictures, see Section QQ and Table 9-4. However, all of the inter mode macroblocks, i.e., Skip and NxM, in SP frames refer to SP mode.

### 12.1.5	Intra Prediction Mode (Intra_pred_mode)

Intra_pred_mode is present when Intra 4x4 or SIntra 4x4 prediction types are indicated by the MB_Type. Intra_pred_mode indicates which intra prediction mode is used for a 4x4 block in the macroblock. The code_number for SIntra 4x4 is the same as that described in the Intra_pred_mode entry of Table 9-4.

Coding of Intra 4x4 prediction modes

Coding of the intra prediction modes for SIntra 4x4 blocks in SI-pictures is identical to that in I-pictures, i.e., the intra prediction modes of the neighbouring blocks are taken into account as described in Section 3.4.3.1.1. Coding of the intra prediction modes for Intra 4x4 blocks in SI-pictures differs from I-picture coding if the neighbouring block is coded in SIntra 4x4 mode. In this case the prediction mode of the neighbouring Sintra 4x4 block is treated to be “mode 0: DC_prediction”.

Figure 12-1 – A generic block diagram of S-picture decoder

## 12.2	S-frame decoding

A video frame in SP-picture format consists of blocks encoded in either Intra mode (Intra 4x4, Intra 8x8 or Intra 16x16 Modes) or in SP mode (Skip or NxM). Similarly, SI-pictures consist of macroblocks encoded in either Intra mode or in SI mode (SIntra 4x4). Intra macroblocks are decoded identically to those in I and P pictures. Figure 26 depicts a generic S-picture decoding for SI and SP modes. In SP mode, the prediction P(x,y) for the current macroblock of the frame being decoded is formed by the motion compensated prediction block in the identical way to that used in P-picture decoding. In SI mode, the prediction P(x,y) is formed by intra prediction block in the identical way to that used in I-picture decoding. After forming the predicted block P(x,y), the decoding of SI and SP-macroblocks follows the same steps. Firstly, the received prediction error coefficients denoted by Lerr are inverse quantised using quantisation parameter PQP and the results are added to the transform coefficients Kpred of predicted block which are found by applying forward transform, see Section QQ, to the predicted block. The resulting sum is quantised with the quantisation parameter SPQP. These steps are combined in a single step as follows:

(11-1)

where

(11-2)

and the values of f and the coefficients Q(m,i,j) are defined in Subsection 5.3.3.3. Here QP1 is signalled by PQP value and the QP2 is signalled by the additional QP parameter SPQP. Notice that when QP1=QP2, then the calculation of YQREC reduces simply to the sum of the received level and the level found by quantising the predicted block coefficient. The coefficients YQREC are then inverse quantised using QP=QP2 and inverse transform is performed for these inverse quantised levels, as defined in Subsections 0 and 5.3.1, respectively. Finally, after the reconstruction of a macroblock, filtering of this macroblock takes place as described in subclause 8.2.2.

### 12.2.1	Decoding of DC values of Chrominance

The decoding of chrominance components for SP- and SI-macroblocks is similar to the decoding of luminance components described in subclause 8.2. AC coefficients of the chrominance blocks are decoded following the steps described in the earlier subclause 8.2 where the quantisation parameters PQP and SPQP are changed according to the relation between the quantisation values of luminance and chrominance, as specified in Section QQ. As described in Section QQ, the coding of DC coefficients of chrominance components includes an additional 2x2 transform. Similarly, for SP and SI-macroblocks, an additional 2x2 transform is applied to the DC coefficients of the predicted 4x4 chrominance blocks and the steps described in subclause 8.2 are applied to the 2x2 transform coefficients.

### 12.2.2	Deblocking Filter

When applying deblocking filter for macroblocks in S-frames, all macroblocks are treated as Intra macroblocks as described in Section QQ.

# 13	Hypothetical Reference Decoder

[Ed. Note: Lots of font trouble in this section that needs repair.]

## 13.1	Purpose

The hypothetical reference decoder (HRD) is a mathematical model for the decoder, its input buffer, and the channel.  The HRD is characterized by the channel’s peak rate R (in bits per second) [Ed. Note: Is peak rate really what is intended?  Do we necessarily assume a “channel”  in the HRD definition?] , the buffer size B (in bits), and the initial decoder buffer fullness F (in bits).  These parameters represent levels of resources (transmission capacity, buffer capacity, and delay) used to decode a bitstream.

A closely related object is the leaky bucket (LB), which is a mathematical constraint on a bitstream.  A leaky bucket is characterized by the bucket’s leak rate R1 (in bits per second), the bucket size B1 (in bits), and the initial bucket fullness B1–F1 (in bits).  A given bitstream may be constrained by any number of leaky buckets (R1,B1,F1),…,(RN,BN,FN), N≥1.  The LB parameters for a bitstream, which are encoded in the bitstream header, precisely describe the minimum levels of the resources R, B, and F that are sufficient to guarantee that the bitstream can be decoded.

## 13.2	Operation of the HRD

The HRD input buffer has capacity B bits.  Initially, the buffer begins empty.  At time tstart it begins to receive bits, such that it receives S(t) bits through time t.  S(t) can be regarded as the integral of the instantaneous bit rate through time t.  The instant at which S(t) reaches the initial decoder buffer fullness F is identified as the decoding time t0 of the first picture in the bitstream.  Decoding times t1, t2, t3, …, for subsequent pictures (in bitstream order) are identified relative to t0, per subclause 13.3.  At each decoding time ti, the HRD instantaneously removes and decodes all di bits associated with picture i, thereby reducing the decoder buffer fullness from bi bits to bi – di bits.  Between time ti and ti+1, the decoder buffer fullness increases from bi – di bits to bi – di + [S(ti+1) – S(ti)] bits.  That is, for i ≥ 0,

b0 = F		(13-1)

bi+1= bi – di + [S(ti+1) – S(ti)].		(13-2)

The channel connected to the HRD buffer has peak rate R.  This means that unless the channel is idle (whereupon the instantaneous rate is zero), the channel delivers bits into the HRD buffer at instantaneous rate R bits per second.

[Ed. Note: Are we sure we want to mention “channel” at all in the HRD definition? How does an HRD determine from looking at the stream when the rate is 0 and when it is maximum?]

b0 = F		(13-3)

bi+1= bi – di + [S(ti+1) – S(ti)].		(13-4)

[Ed. Note: Are we sure we want to mention “channel” at all in the HRD definition? How does an HRD determine from looking at the stream when the rate is 0 and when it is maximum?]

## 13.3	Decoding Time of a Picture

The decoding time ti of picture i is equal to its presentation time i, if there are no B pictures in the sequence [Ed. Note: Oops, not true – that’s the old model of the world when P pictures always moved forward in time.].  If there are B pictures in the sequence, then ti = i – mi, where mi = 0 if picture i is a B picture; otherwise mi equals i – i', where i' is the presentation time of the I or P picture that immediately precedes picture i (in presentation order). If there is no preceding I or P picture (i.e., if i = 0), then mi = m0 = t1 – t0.  The presentation time of a picture is determinable from its temporal reference and the picture clock frequency.

## 13.4	Schedule of a Bitstream

The sequence (t0,d0), (t1,d1), (t2,d2), … is called the schedule of a bitstream.  The schedule of a bitstream is intrinsic to the bitstream, and completely characterizes the instantaneous coding rate of the bitstream over its lifetime.  A bitstream may be pre-encoded, stored to a file, and later transmitted over channels with different peak bit rates to decoders with different buffer sizes.  The schedule of the bitstream is invariant over such transmissions.

## 13.5	Containment in a Leaky Bucket

A leaky bucket with leak rate R1, bucket size B1, and initial bucket fullness B1–F1 is said to contain a bitstream with schedule (t0,d0), (t1,d1), (t2,d2), … if the bucket does not overflow under the following conditions.  At t0, d0 bits are inserted into the leaky bucket on top of the B1–F1 bits already in the bucket, and the bucket begins to drain at rate R1 bits per second.  If the bucket empties, it remains empty until the next insertion.  At time ti, i  ≥ 1, di, bits are inserted into the bucket, and the bucket continues to drain at rate R1 bits per second.  In other words, for i  ≥ 0, the state of the bucket just prior to time ti is

b0 = B1–F1		(13-5)

bi+1 = max{0, bi + di – R1(ti+1–ti)}.		(13-6)

The leaky bucket does not overflow if bi + di  B1 for all i ≥ 0.

Equivalently, the leaky bucket contains the bitstream if the graph of the schedule of the bitstream lies between two parallel lines with slope R1, separated vertically by B1 bits, possibly sheared horizontally, such that the upper line begins at F1 at time t0, as illustrated in the Figure below.  Note from the Figure that the same bitstream is containable in more than one leaky bucket.  Indeed, a bitstream is containable in an infinite number of leaky buckets.

t0

t1

t2

…

t3

t0

t1

t2

…

t3

B1

B2

F2

B2

0

F1

B1

0

bits

bits

time

time

slope R1

Q A B C D E F G H

I a b c d

J e f g h

K i j k l

L m n o p

M

N

O

P

Figure 13-1 – Illustration of the leaky bucket concept

If a bitstream is contained in a leaky bucket with parameters (R1,B1,F1), then when it is communicated over a channel with peak rate R1 to a hypothetical reference decoder with parameters R=R1, B=B1, and F=F1, then the HRD buffer does not overflow or underflow.

## 13.6	Bitstream Syntax

The header of each bitstream shall specify the parameters of a set of N ≥ 1 leaky buckets, (R1,B1,F1),…,(RN,BN,FN), each of which contains the bitstream.  In the current Test Model, these parameters are specified in the first 1+3N 32-bit integers of the Interim File Format, in network (big-endian) byte order:

N, R1, B1, F1, …, RN, BN, FN .		(13-7)

The Rn shall be in strictly increasing order, and both Bn and Fn shall be in strictly decreasing order.

These parameters shall not exceed the capability limits for the profile and level.

## 13.7	Minimum Buffer Size and Minimum Peak Rate

If a bitstream is contained in a set of leaky buckets with parameters (R1,B1,F1), …, (RN,BN,FN), then when it is communicated over a channel with peak rate R, it is decodable (i.e., the HRD buffer does not overflow or underflow) provided B ≥ Bmin(R) and F ≥ Fmin(R), where for Rn  R  Rn+1,

Bmin(R) = Bn + (1 – Bn+1		(13-8)

Fmin(R) = Fn + (1 – Fn+1		(13-9)

 = (Rn+1 – R) / (Rn+1 – Rn).		(13-10)

For R  R1,

Bmin(R) = B1 + (R1 – R)T		(13-11)

Fmin(R) = F1,		(13-12)

where T = tL-1 – t0 is the duration of the bitstream (i.e., the difference between the decoding times for the first and last pictures in the bitstream).  And for R ≥ RN,

Bmin(R) = BN		(13-13)

Fmin(R) = FN .		(13-14)

Thus, the leaky bucket parameters can be linearly interpolated and extrapolated.

Alternatively, when the bitstream is communicated to a decoder with buffer size B, it is decodable provided ≥ Rmin(B) and F ≥ Fmin(B), where for Bn ≥ B ≥ Bn+1,

Rmin(B) = Rn + (1 – Rn+1		(13-15)

Fmin(B) = Fn + (1 – Fn+1		(13-16)

 = (B – Bn+1) / (Bn – Bn+1).		(13-17)

For B ≥ B1,

Rmin(B) = R1 – (B – B1)/T		(13-18)

Fmin(B) = F1.		(13-19)

For B  BN, the stream may not be decodable.

In summary, the bitstream is guaranteed to be decodable in the sense that the HRD buffer does not overflow or underflow, provided that the point (R,B) lies on or above the lower convex hull of the set of points (0,B1+R1T), (R1,B1), …, (RN,BN), as illustrated in Figure 13-12.  The minimum start-up delay necessary to maintain this guarantee is Fmin(R) / R.

Figure 13-2 – Illustration of the leaky bucket concept [Ed. Note: Wasn’t that the name of the last Figure?]

A compliant decoder with buffer size B and initial decoder buffer fullness F that is served by a channel with peak rate R shall perform the tests B ≥ Bmin(R) and F ≥ Fmin(R), as defined above, for any compliant bitstream with LB parameters (R1,B1,F1),…,(RN,BN,FN), and shall decode the bitstream provided that B ≥ Bmin(R) and F ≥ Fmin(R).

## 13.8	Encoder Considerations (informative)

The encoder can create a bitstream that is contained by some given N leaky buckets, or it can simply compute N sets of leaky bucket parameters after the bitstream is generated, or a combination of these.  In the former, the encoder enforces the N leaky bucket constraints during rate control.  Conventional rate control algorithms enforce only a single leaky bucket constraint.  A rate control algorithm that simultaneously enforces N leaky bucket constraints can be obtained by running a conventional rate control algorithm for each of the N leaky bucket constraints, and using as the current quantisation parameter (QP) the maximum of the QP’s recommended by the N rate control algorithms.

Additional sets of leaky bucket parameters can always be computed after the fact (whether rate controlled or not), from the bitstream schedule for any given Rn, from the iteration specified in Section 13.5.