| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C098 Filename: JVT-C098.doc |
| --- | --- |

| Title: | A New Syntax for Error Resilient Transmission of CABAC Stream |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shun-ichi Sekiguchi, Yoshihisa Yamada and Kohtaro Asai Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, Kanagawa, Japan | Tel: Email: | +81-467-41-2463 shun@isl.melco.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

1. Summary

This document proposes a set of new additional syntax elements for error resilient transmission of CABAC stream. Currently, JM supports slice structure as the minimum unit of independent transmission data, where CABAC enforces initialization of all the conditions on arithmetic coding process, including the restriction of available macroblock(MB)s to decide context model [1]. Slice structure provides a simple solution for error resilient transmission of the CABAC coded VCL stream, however, there should be the cases that frequent termination of the arithmetic coding process loses coding efficiency when there is strong correlation around the region where termination happens. Thus, it will be preferable to have adaptive syntax mechanism to allow CABAC coder to exploit the coding states of the surrounding regions even at slice boundary.

For this purpose, we propose a new syntax to allow the encoder to convey side information to restore context model at slice boundary. The proposed adaptive syntax will provide a flexible adaptation mechanism to the local statistics while maintaining error resilience performance and existing slice coding scheme for CABAC.

2. Proposed Syntax Elements

A slice terminates all the dependencies with surrounding MBs to enable the decoder to reconstruct it without any dependency information as shown in Figure 1. Basic property is that the first MB of slice cannot use the data of left and upper MBs for coding and the other MBs cannot use the data of upper MB. For these cases, CABAC coding process cannot exploit the correlation with unavailable adjacent MBs, which sacrifices coding efficiency. If it is possible to

Figure 1  Coding dependency at slice boundary

send side information to restore context models defined across slice boundary at the decoder, this loss of coding efficiency can be avoided while maintaining error resilience performance.

Considering the current NAL concept, codec system will be designed such that slice headers are delivered to the decoder through reliable channel such as TCP [2]. Thus, we can assume complete availability of slice headers at the decoder. Furthermore, slice header for CABAC stream contains the address information of first and last MBs of that slice. This enables the decoder to identify correct spatial position of the slice within a frame. Hence, the decoder can completely identify which MB of current slice is located at slice boundary.

Based on this assumption, we specify the following syntax elements for restoring context models for MBs at slice boundary. Using them, the CABAC encoder can fully exploit the correlation around slice boundary without degrading error resilience performance. Note that the proposed side information is not necessary at all for the context models decided based only on the coded elements within a slice data.

CM_reset_flag: indicates whether current slice uses all the dependencies to be required to build context model (0) or not (1). If this flag is set to one, no more side information is transmitted and the existing slice coding is employed. If this flag is set to zero, the following set of side information is transmitted to convey the context model to be used to the decoder.

CM_type: transmitted only if CM_reset_flag is set to zero. This data element indicates which CM is restored for current slice (MB type, Motion Vector, etc).

CM_init_value: transmitted only if both CM_reset_flag is set to zero. This data element indicates the frequency counts for the context model represented by CM_type, which should be used at the beginning of slice decoding.

CM_side_info: transmitted only if CM_reset_flag is set to zero. This data element indicates side information in MB unit to restore context model represented by CM_type for the decoding of current slice. Data representation may depend on the type of context model. There are two ways to code this information. One is to transmit required data to decide context models for current slice (e.g., not_coded, mvd etc), and the other is to send the context value itself (i.e., ctx_mb_type, ctx_mvd etc). Maximum number of multiplexed CM_side_info is given by (MBA_last – MBA_first + 1).

Note that CM_side_info is not necessary for the MBs where all surrounding MBs for context model decision are available. Since slice header can convey the information on which MB requires CM_side_info in that slice, the encoder can skip to code such CM_side_info.

If it is effective to maintain multiple context models between successive slices, (CM_type, CM_init_value, sequence_of_CM_side_info) can be duplicated in a slice header.

3. Discussion

Table 1 is shown to estimate the coding efficiency obtained by using statistics dependency across slice boundary for CABAC coding. Test sequences are encoded under the common test conditions for coding efficiency test [3] and the slice is defined as one MB line (i.e., 9 Slices in a frame). B-frame coding is not used. Estimate gain is obtained by subtracting 9 x 10 [bytes] from diff bits, where the bit budget for slice header is regarded as 10 bytes in average.

Estimate gain represents the performance gain when no CABAC initialization is performed at slice boundary. Based on this observation, the proposed syntax with efficient representation (to be discussed further) can provide the CABAC encoder a chance for obtaining more coding gain in spite that the slice coding is employed.

4. Conclusion

In this contribution, we discussed the issue on coding efficiency of CABAC when using slice coding, and proposed a potential syntax to allow the encoder to use context models across slice boundary while maintaining error resilience performance. At this moment, expected gain employing the proposed syntax discussed in Section 4 has not been fully evaluated, however, we recommend the group to consider further study on this proposal especially on efficient data representation of syntax elements, since it gives encoder a chance of flexible adaptation to the local statistics without sacrificing error resilience performance.

Table 1  Estimation of CABAC coding gain using local statistics access slice boundary

|  | bits for slice coding | bits for no slice coding | diff. [bits] | estimate gain [bits] | estimate gain [%] |
| --- | --- | --- | --- | --- | --- |
| Foreman, QCIF, Qp=16 | 864,984 | 743,448 | 121,536 | 49,536 | 5.73 |
| News, QCIF, Qp=16 | 526,040 | 435,664 | 90,376 | 18,376 | 3.49 |

Reference

[1] JM software implementation Ver2.0

[2] Working Draft of JVT Standard Number 2, Revision 3 (WD-2r3), JVT-B118r3, March 2002.

[3] Gary Sullivan and Gisle Bjontegaard, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan source Material,” document VCEG-N81, ITU- T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Mitsubishi Electric Corporation |  |  |
| Mailing address | 5-1-1, Ofuna, Kamakura |  |  |
| Country | Japan |  |  |
| Contact person | Kohtaro Asai |  |  |
| Telephone | +81-467-41-2461 |  |  |
| Fax | +81-467-41-2486 |  |  |
| Email | asai@isl.melco.co.jp |  |  |
| Place and date of submission | Place: 3rd JVT meeting at FairFax, VA, USA Date: May 6th – 10th , 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X |  |  |  |  |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: