| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C101 Filename: JVT-C101.doc |
| --- | --- |

| Title: | High Complexity Mode Decision for Error Prone Environment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul-Woo Kim McubeWorks 14 F, SK Telecom Bldg, Sunae-Dong, Bundang-Gu, Sungnam Seoul, 463-784, Korea Dong-Wook Kang Kookmin University 861-1, Chongnung-dong Songbuk-gu, Seoul, 136-702, Korea In Seong Hwang 13F, Seoul Finance Center, 84, Taepyungro 1-ga, Chung-gu, Seoul 100-768, Korea | Tel: Fax Email: Tel: Fax Email Tel: Email | +82-31-788-7652 +82-31-788-7655 charlie@mcubeworks.com +82-2-910-4641 +82-2-910-4641 dwkang@kookmin.ac.kr +82-2-6323-3934 billy@sktelecom.com |
| Source: | McubeWorks Inc. & Kookmin University |  |  |

_____________________________

- Introduction

This contribution proposes an improvement of the high complexity mode decision for error-prone channel presented in VCEG-N43. It was stated that the drift noise, which is defined as the encoder-decoder mismatch of reference frame, is the major factor that causes degradation of video quality when transmitting compressed video bitstream through error prone channel. Though intra-refreshment is known to be the most effective method of reducing the drift noise, it generally requires sacrifice of compression efficiency. This contribution proposes an improved R-D optimization method considering estimated drift noise to perform optimized intra-refreshment.

- High Complexity Mode Decision

2.1    Distortion Model with Error-Prone Channel

Reference frames at the encoder are reconstructed in sequel as follows:

xn=xn−xn−1mv+qr+xn−1mvINTERxn+qaINTRA

Decoding process is basically the inverse of encoding process (normal condition). But if errors are detected, decoder should be changed to the error concealment mode (abnormal condition). Assuming simple temporal error concealment, the decoded pixel, yn, can be modeled as follows:

Fig. 1. Block diagram of encoder / decoder behavior

For pixels in the INTRA MBs

yn=xn+qanormalyn−1abnormal

For pixels in the INTER MBs

yn=xn−xn−1mv+qr+yn−1mvnormalyn−1abnormal

It can be assumed that there exists the difference between xn−1 and yn−1 which are the reference frames for motion compensation in the encoder and the decoder, respectively. We define the difference as the drift noise:

dn−1=yn−1−xn−1.

In error-prone channel, the overall distortion between the decoded and the original macroblocks can be expressed as

where p means the probability that errors occurs during transmission of this macroblock, and D1 denote the distortion calculated assuming that the corresponding macroblock is correctly received and D2 the distortion assuming that the corresponding macroblock is corrupted by channel errors and as consequence concealed by the decoder.

D2is identical regardless of the encoding mode.

D2=Eyn−1−xn2

But the first term can be different according to the encoding mode and current status of drift noise.

For an INTRA MB

D1=Exn+qa−xn2=Eqa2

For an INTER MB, assuming that the drift and quantization noise are uncorrelated,

D1=Exn−xn−1mv+qr+yn−1mv−xn2Eyn−1mv−xn−1mv+qr2Eyn−1mv−xn−1mv2+Eqr2Edn−12mv+Eqr2

2. 2   RD-optimized mode decision

Select the mode minimizing the cost defined as follows,

Cost=1−pD1+λm⋅R

# Estimation and Update of Drift Noise

The cause of drift noise is assumed mainly due to packet loss. The power of newly incurred drift noise is proportional to the probability of error concealment and the variability of the encoded pictures. Assuming that the packet loss occurs with iid distribution for every frame (packet loss probability isp),, the expected value of newly incurred drift noise is

Ednew2=pExn−1−xn2

Drift noise propagates by motion compensate decoding and temporal error concealment. If the current inter-coded macroblock is normally, i.e. with out error, compensated and decoded, the macroblock will inherit the drift noise of the macroblock from which the current macroblock performed the motion compensation. And if a macroblock is intra-coded and transmitted normally, there exists no drift noise for those pixels. If a macroblock is error concealed, then drift noise of the same position of the previous frame is inherited. The probability of the error concealment is the same as the probability of packet loss.

According to the above inferences, we can conclude that the propagation of drift noise can be specified as following:

Edsuccessive2=pEdn−12INTRAMB1−pEdn−12mv+pEdn−12INTERMB

Successive inter-coding of frames can produce locally greater drift noise than the value of Edsuccessive2 in the above modeling. This can be modeled in a similar manner to the noise power increase due to the Brownian motion. To handle this matter, we modified the drift noise by multiplying a cost function as following:

Edprop2=pexpPerrorEdn−12INTRAMB1−pexpPerrormvEdn−12mv+pexpPerrorEdn−12INTERMB

where Perror and Perrormv represents the probability that channel error occurred in the pixels of the same position in the previous frame and in the pixels used for motion compensation  respectively. Assuming that n successive inter-coding following the latest intra refreshment, the probability can be obtained as follows:

Perror=1−1−pn

The overall drift noise can be obtained as following:

Edn2=Ednew2+Edprop2

When error occurs in the I-picture area, especially in the macoblocks with zero motion in the successive P-pictures, the effects of error in the first I-picture propagates through following P-pictures. This can be regarded that the drift noise which occured in transmitting the I-picture is not estimated properly. To handle this matter, forced-intra-refreshment as well as proposed R-D optimization is performed on the successive n frames following the I-picture. In the following simulation result, to block the error propagation which might have happened in I-picture,  every macroblock in the first 5 frames is intra-updated at least once by adopting intra refreshment on GOB unit,

- Simulation Results

The proposed mode decision algorithm was tested according to the conditions in VCEG-M77 in addition to the extra sequences. The overall simulation results are shown in JVT-C101.xls. Simulation results for the sequences in the common conditions, i.e. hall_monitor, foreman and paris are presented in this document. Results for extra sequences are presented in JVT-C101.xls. The performance of proposed mode decision method was compared with the forced-intra refreshment using MbLineIntraUpdate, which is implemented in JM 1.7. Optimal MbLineIntraUpdate values were searched depending on the channel characteristics to be used for the forced-intra refreshment. First frame was intra-coded, while remaining 99 frames were inter-coded The proposed mode decision method shows better performance above 1 dB over the current high complexity mode decision for error robustness.

Additional advantage of the proposed algorithm is that the total encoding time can be significantly decreased by the adoption of drift noise modeling rather than the direct implementation of multiple decoder which is in the current JM. Approximate reduction of decoding time is about 40 ~ 60% when compared with multiple decoder implementation. PSNR results comparing the performance of each method can be found in JVT-C101.xls and Fig. 2 ~ Fig.7.

- Summary

The proposed algorithm shows effective R-D optimized mode decision performance for error prone channel. This mode decision method can be refined when it is combined with data partitioning.

- Reference

[1] Gisle Bjontegaard, “H.26L Test Model Long Term Number 8 (TML-8) draft 0,” June. 2001.

[2] Y. Wang, et. al., "Error resilient video coding techniques," IEEE Signal Processing Magazine, pp.61-82, July 2000.

[3] Chul-Woo Kim et. al, “High complexity mode decision for error prone channel,” VCEG-O43, Nov. 2001.

## Fig. 2. Hall_Monitor at 3 kmph

- Fig. 3. Hall_Monitor at 50 kmph

- Fig.4. Foreman at 3 kmph

- Fig. 5. Foreman at 50 kmph

- Fig.6. Paris at 3 kmph

- Fig. 7. Paris at 50 kmph

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | McubeWorks Inc. & SK Telecom |  |  |
| Mailing address | 406 Korea Design Center Yatap 344-1, Bundang, Seongnam, Kyunggi, 463-848 |  |  |
| Country | Korea |  |  |
| Contact person | Chul-Woo Kim |  |  |
| Telephone | +1 82 31 788 7652 |  |  |
| Fax | +1 82 31 788 7655 |  |  |
| Email | charlie@mcubeworks.com |  |  |
| Place and date of submission | 30/04/2002 Korea |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | High Complexity Mode Decision for Error-Prone Channel |  |
| Contribution number |  | JVT-C |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | McubeWorks Inc. & SK Telecom |  |  |
| Mailing address | 406 Korea Design Center Yatap 344-1, Bundang, Seongnam, Kyunggi, 463-848 |  |  |
| Country | Korea |  |  |
| Contact person | Chul-Woo Kim |  |  |
| Telephone | +1 82 31 788 7652 |  |  |
| Fax | +1 82 31 788 7655 |  |  |
| Email | charlie@mcubeworks.com |  |  |
| Place and date of submission | 30/04/2002 Korea |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Threshold Adjustment for Adaptive Use of Double Scan |  |
| Contribution number |  | JVT-C |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: