| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 March, 2002 | Document: JVT-C103 Filename: JVT-C103.doc |
| --- | --- |

| Title: | Interpolation coefficient adaptation in multi-frame interpolative prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshihiro Kikuchi Takeshi Chujoh | Tel: Email: | +81 44 549 2288 yoshihiro.kikuchi@toshiba.co.jp |
| Source: | Toshiba |  |  |

_____________________________

# Introduction

The multi-frame interpolative prediction (MFIP) was proposed in JVT-B075. Detailed description of MFIP with modified syntax is in JVT-C066. It is regarded as the generalized form of MH-pictures, employing a temporal-interpolation on multiple frames in the reference frame buffer. The prediction signal is calculated by the following equation:

pred = r1 * w1 + r2 * w2 + d

where r1 and r2 correspond to the two reference frames, and w1, w2 and d are the linear interpolation coefficients.

In JVT-B075, two coefficient sets (w1, w2, d)=(1/2,1/2,0) and (2,-1,0) are used for the simulation. The prediction signal calculation for these coefficients is simple (one shift and one addition/subtraction) while the improvement is significant as shown in JVT-B075. There is no need to calculate fading gain at the encoder.

However, more coding gain should be derived by adaptively determining the MFIP coefficients frame by frames. This contribution explains an example algorithm for such coefficients detection and evaluates the coding performance.

# MFIP coefficient detection algorithm (Informative)

The algorithm is to determine MFIP coefficients frame by frame to minimize the following mean square prediction error J:

J=h,vo(h,v)−w1r1(h,v)−w2r2(h,v)−d2

where o(h,v), r1(h,v) and r2(h,v) are the pixels at (h,v)-th position in the current frame and the reference frames r1 and r2, respectively.

For simplicity, the following restrictions are applied to the MFIP coefficients:

- Number of coefficient sets is pre-determined as three; c(i)=(w1(i), w2(i), d(i)) where i=0 to 2.

- Weighting factor for the reference frame r2 in c(0) is set to 0; w2(0)=0.

- Weighting factor for the reference frame r1 in c(1) is set to 0; w1(1)=0.

- Weighting factors w1(0) and w2(1) are calculated by the following simplified form;

w1(0)=E(abs(o(h,v)-E(o(h,v))) / E(abs(r1(h,v)-E(r1(h,v)))), d(0)=E(o(h,v))- w1(0)*E(r1(h,v))

w2(1)=E(abs(o(h,v)-E(o(h,v))) / E(abs(r2(h,v)-E(r2(h,v)))), d(1)=E(o(h,v))- w2(1)*E(r2(h,v))

where E() is the mean value;

Ex(h,v)=1HVh=0H−1v=0V−1x(h,v),

abs(x) is the absolute value of x.

- c(2) is determined to minimize J defined above.

The coefficients in this algorithm are determined by pixel order calculations.

Further improvements are expected by e.g. introducing motion estimation, or detecting the optimal coefficients for image regions in a frame. However, such algorithms are not used in this contribution for simplicity.

# Test condition

Computer simulations were conducted following the common test condition (VCEG-N81). The following tables summarize the simulation condition.

Common test condition

| Base program | JM1.7 |
| --- | --- |
| R-D optimization | used |
| MV search range | 32 |
| Bi-directional/Backward prediction | not used(forward prediction only) |
| CABAC | not use(UVLC) |
| 1/8 pel MC | not used(up to 1/4 pel accuracy) |
| Slice structure, Data partitioning | not used |
| Hadamard | used |
| Number of reference pictures | 5 |
| QP | 4, 8, 12, 16, 20, 24, 28 |

The MFIP coefficients are detected frame by frame. The precision (number of bits) of the coefficient is controlled so that the prediction signal is calculated within 16-bit precision.

To evaluate the performance for fade sequences, additional video sequences with fade were generated. The following table lists the fade patterns and test sequences used for the simulation. Only linear fade-in from black was used in JVT-B057. This time, other several fade patterns (quadratic curve, sigmoid curve, cross-fade, fade-in from white) are additionally used. The detailed fade sequence generation formula is explained in Annex A.

Fade patterns and test sequences

| Fade type | Video sequence | Fade duration |
| --- | --- | --- |
| Linear fade from black | Container(QCIF 10Hz) News(QCIF 10Hz) Foreman(QCIF 10Hz) | 2 sec. |
| Quadratic curve fade from black |  |  |
| Sigmoid (S-curve) fade from black |  |  |
| Linear fade from white |  |  |
| Quadratic curve fade from white |  |  |
| Sigmoid (S-curve) fade from white |  |  |
| Cross fade | Container to News (QCIF 10Hz) | 2 sec. |

# Simulation result

Simulation results are show in the attached excel file (JVT-C103.xls). Average SNR improvements and bitrate savings calculated by avsnr4 are summarized in the table bellow.

The results show extremely high coding gains for overall fade types and video sequences. The average SNR improvements are 3.47-5.21dB for Container, 3.11-5.78dB for News, 2.32-4.34dB for Foreman, and 1.57-1.65dB for cross-fade.

Average SNR improvement and bitrate saving

| ###### Sequence | Effect |  | avsnr4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | QP=16,20,24,28 |  | QP=4,8,12,16 |  |
|  |  |  | ΔPSNR(dB) | Δbitrate(%) | ΔPSNR(dB) | Δbitrate(%) |
| Container | Fade-in from black | Linear | 5.21 | -55.22 | 4.50 | -56.46 |
|  |  | Quadratic | 4.33 | -48.88 | 3.82 | -55.94 |
|  |  | S-curve | 4.48 | -51.23 | 3.66 | -52.95 |
| News |  | Linear | 3.45 | -46.64 | 4.46 | -50.16 |
|  |  | Quadratic | 3.11 | -44.42 | 3.80 | -50.78 |
|  |  | S-curve | 3.24 | -42.51 | 3.69 | -46.61 |
| Foreman |  | Linear | 4.34 | -52.33 | 3.52 | -49.71 |
|  |  | Quadratic | 3.98 | -49.09 | 3.13 | -50.98 |
|  |  | S-curve | 3.59 | -45.39 | 2.74 | -44.19 |
| Container | Fade-in white | Linear | 4.54 | -50.52 | 4.27 | -54.53 |
|  |  | Quadratic | 3.48 | -44.59 | 3.65 | -53.60 |
|  |  | S-curve | 4.17 | -46.78 | 3.47 | -50.95 |
| News |  | Linear | 5.59 | -60.36 | 5.78 | -59.24 |
|  |  | Quadratic | 4.76 | -55.20 | 4.79 | -58.67 |
|  |  | S-curve | 4.96 | -54.38 | 4.61 | -54.26 |
| Foreman |  | Linear | 3.22 | -44.75 | 2.99 | -44.11 |
|  |  | Quadratic | 2.86 | -42.60 | 2.86 | -47.75 |
|  |  | S-curve | 2.78 | -39.27 | 2.32 | -39.30 |
| Container -> News | Cross-fade | Linear | 1.57 | -24.18 | 1.65 | -20.58 |

# Conclusion

An interpolation coefficient detection algorithm for the multi-frame interpolative prediction (MFIP) is proposed. Extremely high coding gains are observed in the simulation. The SNR improvement from the JM is up to 5.8dB. We propose to adopt MFIP in the JVC CD, with MFIP coefficient loading syntax proposed in JVT-C066[2] to enable coefficient adaptation.

# References

[1] Y. Kikuchi, T. Chujoh, “Improved multiple frame motion compensation using frame interpolation”, JVT-B075, Jan. 2002.

[2] Y. Kikuchi, T. Chujoh, S. Kadono, “Multi-frame interpolative prediction with modified syntax”, JVT-C066, May 2002.

[3] G. Sullivan, G. Bjontegaard, "Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material", VCEG-N81, Sep. 2001.

Annex A

The fade curve functionf(t) is represented as follows,

- Linear	f(t)=tT

- Quadratic	f(t)=tT2

- S-curve	f(t)=2tT2,0≤t≤T2

where T is the fading duration. Fading image g(t) is created by the following formula,

g(t)=a(t)(1−f(t))+b(t)f(t)

a(t) and b(t) are set as the following table.

|  | a(t) | b(t) |
| --- | --- | --- |
| fade-in from black/white | black/white | original image |
| fade-out to black/white | original image | black/white |
| cross-fade | original image A | original image B |

Black and white images are created by adding random value 0 or 1 toY,Cb,Cr=16,128,128 and Y,Cb,Cr=239,128,128, respectively.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | TOSHIBA CORPORATION |  |  |
| Mailing address | 1, Komukai Toshiba-Cho, Saiwai-Ku, Kawasaki 212-8582, Japan |  |  |
| Country | Japan |  |  |
| Contact person | Yoshihiro Kikuchi |  |  |
| Telephone | +81-44-549-2288 |  |  |
| Fax | +81-44-520-1267 |  |  |
| Email | yoshihiro.kikuchi@toshiba.co.jp |  |  |
| Place and date of submission | Fairfax, VA, USA, 6-10 May, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  |  |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: