| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C107 Filename: JVT-C107.doc |
| --- | --- |

| Title: | 16 bit Adaptive Block size Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mathias Wien, Achim Dahlhoff Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52056 Aachen, Germany | Tel: Email: | ++49-241-8027681 wien@ient.rwth-aachen.de |
| Source: | RWTH Aachen |  |  |

_____________________________

# Introduction

In this paper presents the revision of the ABT proposal according to the current working draft WD2. The transform design is modified to fit into 16 bits. The new JM 4x4 transform and an 8x8 transform proposed by Gisle Bjontegaard are used. The new design exhibits less ringing artifacts and superior subjective quality compared to the TML9 proposal.

The CABAC encoding mechanism is redesigned and a VLC solution is proposed. The intra prediction is extended to 9 prediction modes as adopted at the last meeting. The accompanying contribution JVT-C108 provides the integration of the ABT proposal into the JM20 software. In this paper, the coding elements needed for ABT coding are defined.

The paper is organized as follows: First, the modifications to the bit stream syntax are described. Then the ABT decoding process is described in more detail. A set of simulation results and RD plots is given. Subjective demos will be given at the meeting.

# ABT Syntax Elements

## Slice Header Syntax

[MWI: not described in JWD2r8,. Include useABT symbol, (useABT=0: no ABT, useABT=1:inter ABT, useABT=2:inter&intraABT)]

| slice_header( ) { | Category | Mnemonic |
| --- | --- | --- |
| Qq |  |  |
| } |  |  |

## Prediction data for 8x8 modes

[MWI: BlockIntra4x4 might be renamed to BlockIntra8x8]

| prediction8x8( mb_type ) { | Category | Mnemonic |
| --- | --- | --- |
| region8x8_type | 4 | ecselbf |
| if ( region8x8_type = = BlockIntra4x4 ) { |  |  |
| if ( useABT > 1 ) |  |  |
| intra_block_modeABT /* determines num_ipred */ | 4 | ecselbf |
| for ( i4x4=0; i4x4< num_ipred; i4x4++ ) |  |  |
| intra_pred_mode | 4 | ecselbf |
| } else { |  |  |
| …. |  |  |

## Prediction data for 16x16 modes

| prediction16x16( mb_type ) { | Category | Mnemonic |
| --- | --- | --- |
| if ( mb_type = = MBintra4x4 ) |  |  |
| if ( useABT > 1 ) |  |  |
| intra_block_modeABT /* determines num_ipred */ | 4 | ecselbf |
| for ( i4x4=0; i4x4< num_ipred; i4x4++ ) |  |  |
| intra_pred_mode | 4 | ecselbf |
| else if ( mb_type != MBintra16x16 ) { |  |  |
| … |  |  |

## Residual Data Syntax

| residual( mb_type ) { | Category | Mnemonic |
| --- | --- | --- |
| if ( mb_type = = MBintra16x16 ) |  |  |
| do |  |  |
| luma_dc_zigzag_token | 5 ¦ 6 | ecselbf |
| while ( lum_dc_zigzag_token != EOB ) |  |  |
| for ( i8x8=0; i8x8<4; i8x8++ ) /* each luma 8x8 region */ |  |  |
| /* ABT number of sub-blocks num_blks depends on block mode non-ABT: num_blks=4 */ |  |  |
| for (i4x4=0; i4x4< num_blks; i4x4++) /* each sub-block of region */ |  |  |
| if (cbp & (1<< i8x8) |  |  |
| if ( double_scan() ) { |  |  |
| do |  |  |
| luma_dblscan0_token | 5 ¦ 6 | ecselbf |
| while ( luma_dblscan0_token != EOB ) |  |  |
| do |  |  |
| luma_dblscan1_token | 5 ¦ 6 | ecselbf |
| while ( luma_dblscan1_token != EOB ) |  |  |
| } else |  |  |
| do |  |  |
| luma_zigzag_token | 5 ¦ 6 | ecselbf |
| while ( luma_zigzag_token != EOB ) |  |  |
| if (cbp & 0x30) { /* chroma DC residual coded */ |  |  |
| … |  |  |

# ABT Semantics

## Luminance intra coding

If useABT = = 2, ABT intra coding is used. ABT intra coding is very similar to base line 4x4 intra coding. The block size used for prediction is extended to 4x4, 8x4, 4x8, or 8x8 pixels. The transform block sizes is chosen accordingly. MBintra16x16 is not used with ABT intra.

### Prediction modes for ABT blocks

The 9 intra prediction modes of base line are modified to be used for ABT intra blocks larger than 4x4. ABT 4x4 intra blocks are predicted using the base line technique.

### Prefiltered prediction

Before employed for prediction, the edge pixels of (available) neighboring blocks are gathered in a prediction vector EP. Let P denote the predicted block of N pixels in M lines. P[m,n] is the pixel at horizontal position n, 0<=n<N, and vertical position m, 0<=m<M. The pixels used for prediction shall be identified by R[m,n], with n=-1 & -1<=m and –1<n & m=-1. Only decoded pixels of the same slice are valid for prediction.

The prediction vector EP contains the reconstructed luminance at valid positions of R. EP is constructed according to the following pseudo code:

offset = 0;

valid_left_down = valid(R[M:M+N-1,-1]);

valid_left      = valid(R[0:M-1,-1]);

valid_up        = valid(R[-1,0:N-1]);

valid_up_right  = valid(R[-1,N:N+M-1]);

if valid_left_down

{

offset += N;

for (k=0; k<N; k++)

EP[offset-k] = R[k,-1];

}

if valid_left

{

offset += M;

for (k=0; k<M; k++)

EP[offset-k] = R[k,-1];

EP[offset+1] = R[0,-1];

}

offset+=1;

if valid_up

{

for (k=0; k<N; k++)

EP[offset+k+1] = R[-1,k];

EP[offset] = R[-1,0];

}

if valid_up_right

{

for (k=N; k<N+M; k++)

EP[offset+k+1] = R[-1,k];

}

if (valid_left & valid_up)

EP[offset] = R[-1,-1];

To enhance prediction quality, this vector EP is filtered using the filter

[1, 2, 1] / 4

with constant extension at the boundaries.

#### Mode 0: DC-Prediction

All positions of P are predicted by sum(EP), i.e. the sum of all elements of EP. If there are no valid edge pixels, the block is predicted by P[m,n]=128, 0<=n,m<N,M.

#### Mode 1: Vertical Prediction

if(valid_up)

for(y=0;y<M;y++)

for(x=0;x<N;x++)

P[y,x]=EP[offset+1+x];

#### Mode 2: Horizontal Prediction

if(valid_left)

for(y=0;y<M;y++)

for(x=0;x<N;x++)

P[y,x]=EP[offset-1-y];

#### Mode 3: Down-Right Prediction

if(valid_left&&valid_up)

for(y=0;y<M;y++)

for(x=0;x<N;x++)

P[y,x]=EP[offset+x-y];

#### Mode 4: Up-Right Prediction (Bidirectional)

if(valid_left&&valid_up)

for(y=0;y<M;y++)

for(x=0;x<N;x++)

P[y,x]=(EP[offset+2+x+y]+EP[offset-2-(x+y)])>>1;

#### Mode 5: Down-Right-Down Prediction

if(valid_left&&valid_up)

{

for(y=0;y<M;y+=2)//even lines

for(x=0;x<N;x++)

if( (i=x-(y>>1)) >= 0 )

P[y,x]=(EP[offset+i]+EP[offset+1+i])>>1;

else

P[y,x]=EP[offset+1+2*x-y];

for(y=1;y<M;y+=2)//odd lines

for(x=0;x<N;x++)

if( (i=x-(y>>1)) >= 0 )

P[y,x]=EP[offset+i];

else

P[y,x]=EP[offset+1+2*x-y];

}

#### Mode 6: Down-Left-Down Prediction

if(valid_left&&valid_up)

{

for(y=0;y<M;y+=2)//even lines

for(x=0;x<N;x++)

P[y,x]=(EP[offset+1+x+(y>>1)]+EP[offset+2+x+(y>>1)])>>1;

for(y=1;y<M;y+=2)//odd lines

for(x=0;x<N;x++)

P[y,x]=EP[offset+2+x+(y>>1)];

}

#### Mode 7: Right-Up-Right Prediction

if(valid_left&&valid_up)

{

for(y=0;y<M;y++)//even columns

for(x=0;x<N;x+=2)

P[y,x]=(EP[offset+-1-(y+(x>>1))]+EP[offset+-2-(y+(x>>1))])>>1;

for(y=0;y<M;y++)//odd columns

for(x=1;x<N;x+=2)

P[y,x]=EP[offset+-2-(y+(x>>1))];

}

#### Mode 8: Right-Down-Right Prediction

if(valid_left&&valid_up)

{

for(y=0;y<M;y++)//even columns

for(x=0;x<N;x+=2)

if( (i=-y+(x>>1)) <= 0 )

P[y,x]=(EP[offset+i]+EP[offset+i-1])>>1;

else

P[y,x]=EP[offset-1-2*y+x];

for(y=0;y<M;y++)//odd columns

for(x=1;x<N;x+=2)

if( (i=-y+(x>>1)) <= 0 )

P[y,x]=EP[offset+i];

else

P[y,x]=EP[offset-1-2*y+x];

}

### Coding of ABT intra prediction modes

For encoding, the ABT intra prediction modes of the subblocks of a macroblock are grouped in pairs. Since Intra blocks of variable block size might be encoded in one macroblock, a rule has to be defined for grouping the prediction modes: The prediction modes are grouped left to right, top to bottom.

1

2

3

4

5

##### Figure 1: Macroblock in 8x8 Mode with 4 ABT intra subblocks

If the number of intra coded blocks is odd, the prediction mode of the last intra subblock is grouped with a '0' for encoding. E.g. in Figure 1, a macroblock in 8x8 mode containing subblocks with tilings 8x8, 8x8, 8x8, 8x4, is given. The 8x8 prediction modes of subblocks 1,2 are grouped together for encoding. The prediction mode of block 3 is grouped with prediction mode of block 4. The prediction mode of block 5 is grouped with a prediction mode '0' for encoding.

For encoding of the intra prediction modes, the base line encoding method is used.

## ABT Transform Coefficient Decoding

Two transforms, one for blocks of length 8 and one for blocks of length 4, are used for ABT coding. In the following, matrix notation is used to describe the inverse transform process. The 4x4 transform matrix and inverse transform matrix are the ones used for transform when ABT is not employed. The 4x4 transform features different norms in the even and odd basis functions, leading to a set of 3 different norms for a 2-dimensional transform. These norms have to be regarded during the quantization and dequantization processes. The 8x8 transform matrix proposed  for ABT was originally proposed by Gisle Bjontegaard in [Q15-I-39]. This transform matrix has one norm for all basis functions. Transform and inverse transform matrix are identical. The transform matrices are given below.

T8 = [

13, 13, 13, 13, 13, 13, 13, 13

19, 15,  9,  3, -3, -9,-15,-19

17,  7, -7,-17,-17, -7,  7, 17

9,  3,-19,-15, 15, 19, -3, -9

13,-13,-13, 13, 13,-13,-13, 13

15,-19, -3,  9, -9,  3, 19,-15

7,-17, 17, -7, -7, 17,-17,  7

3, -9, 15,-19, 19,-15,  9, -3 ];

In contrast to the T8 matrix proposed earlier, this transform allows for a 16 bit implementation.

### Scanning method

ABT transform coefficients are encoded using the (RUN,LEVEL) model. For each transform block size, a 'simple' zig-zag scan is defined. The scans are given in Figure 2.

4x4

4x8

8x8

8x4

Figure 2 : Zig-Zag scan for all transform block sizes

### Inverse Quantization and transform

The inverse quantization of the ABT transform coefficients for each mode uses the dequantization values given in the table below. Please note that the current design is given for quantization parameters QP=0,…, QP=31. The extension of this range is possible.

According to the base line dequantization procedure, the quantizer step size doubles every 6 QP. The coefficient R(k,i,j) used in the following is mode dependend and chosen from the table below. Additionally, a bit shift parameter are defined.

k  S8x8  S8x4,4x8      S4x4

0   15      9 11     40  64 51

1   17     10 12     45  72 57

2   19     11 14     50  81 64

3   22     12 16     57  91 72

4   24     14 17     63 102 80

5   27     15 20     71 114 90

Mode 8x8:

R[k][i][j] = S8x8[k] for all i,j

Bshift = 7;

Mode 8x4:

R[k][i][j] = S8x4,4x8[k][0] for all even j

R[k][i][j] = S8x4,4x8[k][1] for all odd j

Bshift = 2;

Mode 4x8:

R[k][i][j] = S8x4,4x8[k][0] for all even i

R[k][i][j] = S8x4,4x8[k][1] for all odd i

Bshift = 2;

Mode 4x4:

R[k][i][j] = S4,4[k][0] for (i,j) = {(0,0),(0,2),(2,0),(2,2)},

R[k][i][j] = S4,4[k][1] for (i,j) = {(1,1),(1,3),(3,1),(3,3)},

R[k][i][j] = S4,4[k][2]  otherwise;

Bshift = 0;

The reconstructed coefficients are multiplied with the dequantization value R

YD(i,j) = YQ(i,j) * R(QP%6,i,j);   i=0,..,N; j=0,...,M

The resulting block YD of size NxM is then inverse transformed horizontally. If N=4, the one-dimensional inverse transform is performed according to WD2, section 9.3.3.3 [JVT-B118r7].  If N=8, a matrix multiply is performed and the result is rounded,

Z = (YD * TN)//Bshift,

where (YD * TN) denotes the appropriate inverse transform operation. The notation (YD * TN)//Bshift means that the resulting value for each element Z(i,j) is down-shifted by Bshift bits with proper rounding.

Z(i,j) = sign(Z'(i,j) )*[abs(Z'(i,j)) + 2Bshift-1]>>Bshift.

This operation is performed to stay within 16 bit precision. After the vertical inverse transform step the result is finally rounded

X' = (TTM* Z),

X''(i,j) = [X'(i,j) + 26-QP/6-1]>>(6-QP/6).

Finally, the reconstructed sample residual values X''(i,j) are added to the prediction values P(i,j) from motion compensated prediction or spatial prediction and clipped to the range of 0 to 255 to form the final decoded sample result:

S'(i,j) = clip1( P(i,j) + X''(i,j) ).

## Deblocking Filter

Deblocking is employed on all subblock and macroblock boundaries. No modifications are introduced to the deblocking filter besides a modification of the determination of  and .

No filtering is performed at 4x4 block boundaries inside transformed blocks. I.e., the number of block boundaries that have to be filtered is reduced by 1/2 for blocks of size 8x8 and larger and by a factor of 1/4 for 4x8 and 8x4 blocks.

Without ABT, the threshold parameters  and  are driven by the quantization parameter QP. For ABT blocks, the index to the threshold table is increased by IQP. IQP depends on the depth and the width of the neighboring blocks. The depth is the distance to the next block boundary on each side, while the width of the block is the number of pixels at the current boundary.

IQP=0;

if (depth(Block p)==8)

IQP++;

if (depth(Block q)==8)

IQP++;

if (width(Block p)==8)

IQP++;

if (width(Block q)==8)

IQP++;

clip3(0,3, IQP);

## Entropy Coding

For encoding of the ABT coefficients CABAC is employed. New contexts and binarizations are introduced for encoding of the coefficients of ABT blocks. Additionally; a VLC encoding method is introduced to enable encoding of ABT coefficients in VLC mode.

### CABAC

For encoding of the ABT coefficients, the base line methods are used. 4x4 blocks are encoded using exactly the base line routines. The context definition of the coefficients of the other modes is described in the following. There is one set of contexts defined for 8x8 blocks and one for 4x8/8x4 blocks. For all modes, the Levels are encoded using the base line routines.

Coeff_Count is used for encoding of the ABT coefficients. The unary binarization is applied for Coeff_Count <16. For Coeff_Count >=16, the 16 zeros are followed by the binary represenation of (Coeff_Count-16). The sync bit is omitted here. The number of bits used for the binary representation is determined by the maximum possible Coeff_Count value in the current mode. The bits of the representation are encoded from low to high, i.e. 20,21,…

Four contexts are used for the first bin. The context is determined by

ctx_coeff_count(C) = ((CC(A)==0) ? 0 : 1) + 2*(( CC(B)==0)?0:1);

where CC(X) , X=A,B, is the Coeff_Count of the left and top neighboring decoded blocks A and B. The remaining bins are encoded using a context for the 2nd bin, a joint context for the 3rd and 4th bin, a context for the remaining bins of the unary binarization, and one context for the bins of the binary representation.

Run is encoded exploiting the information of Coeff_Count and the previously coded Runs. The binarization of Coeff_Count is used for Runs as well. Here, only one context is used for the first bin. The remaining contexts are defined as with Coeff_Count. Two sets of Run contexts can be chosen, depending on the activity of the block, which is measured by Coeff_Count

ctx_run = ((COEFF_COUNT) >= 4) ? 1 : 0.

### VLC

In VLC mode, all coefficients are encoded as (Level,Run) symbols. Inter blocks use the EOB symbol. For Intra coding, a Coeff_Count symbol is encoded.

Golomb codes of several degrees are employed for encoding of the symbols. The Golomb-0 code is used for encoding of all non-luminance symbols. For ABT, additional Golomb-1,2, and 3 codes are employed. The general structure of these codes is depicted in the table below

| Layer | Golomb-0 | Golomb-1 | Golomb-2 | Golomb-3 |
| --- | --- | --- | --- | --- |
| 0 1 2 3 … | 1 01x 001xx 0001xxx … | 1x 01xx 001xxx 0001xxxx … | 1xx 01xxx 001xxxx 0001xxxxx … | 1xxx 01xxxx 001xxxxx 0001xxxxxx … |

Generally, a Golomb code word consists of a sync word 0…01 and an info word x0x1…xn. For ABT coding, a finite number of symbols is encoded using Golomb codes. Symbols that are not in the finite set are encoded using an Escape symbol an separate coding of Level and Run. The Escape symbol is 59 for all used codes.

Since finite alphabets are encoded, the '1'-bit of the sync word is omitted in the layer containing the Escape symbol.

#### Intra Coeff_Count

Intra Coeff_Count is encoded using the Golomb-2 code:

| Coeff_Count | Golomb-2 symbol |
| --- | --- |
| 0 | 100 |
| 1 | 101 |
| 2 | 110 |
| 3 | 111 |
| 4 | 01000 |
| … | … |

#### 2D (Level,Run) Symbols

The Golomb code used for encoding the (Level,Run) symbols is block mode dependend:

|  | Inter |  | Intra |  |
| --- | --- | --- | --- | --- |
| Block Mode | Degree | Layers | Degree | Layers |
| 8x8 | 0 | 6 | 2 | 4 |
| 8x4,4x8 | 1 | 5 | 2 | 4 |
| 8x8 | 2 | 4 | 2 | 4 |

#### Code Tables

Four tables are used to adapt the set of Golomb codes to the symbol statistics. The intra tables are chosen QP-dependent. The tables contain the (odd) code word numbers for Level>0. Level<0 is encoded using the even numbers. EOB is encoded as code word 0. In case of Intra coding, no EOB symbol is encoded and the symbols are shifted –1 in the table.

Symbols that are not in the table are encoded by sending the Escape Symbol 59 first and then Level using Golomb-3 and Run using Golomb-2. Here, no finite alphabet is assumed. The Levels are arranged as follows

level  code word

1      0

-1      1

2      2

-2      3

3      4

-3      5

4      6

...

The code tables are given below:

Inter

level

0  1  5 13 21 31 39 47

*  3 15 33 51  *  *  *

*  7 25 53  *  *  *  *

*  9 35  *  *  *  *  *

* 11 45  *  *  *  *  *

run* 17 55  *  *  *  *  *

| * 19  *  *  *  *  *  *

V * 23  *  *  *  *  *  *

* 27  *  *  *  *  *  *

* 29  *  *  *  *  *  *

* 37  *  *  *  *  *  *

* 41  *  *  *  *  *  *

* 43  *  *  *  *  *  *

* 49  *  *  *  *  *  *

* 57  *  *  *  *  *  *

*  *  *  *  *  *  *  *

Intra table 1 (QP < 14)

level

0  1  3  7  9 13 19 21

*  5 15 25 31 39 45 49

* 11 29 41 51  *  *  *

* 17 35 55  *  *  *  *

* 23 43  *  *  *  *  *

run* 27 57  *  *  *  *  *

| * 33  *  *  *  *  *  *

V * 37  *  *  *  *  *  *

* 47  *  *  *  *  *  *

* 53  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

Intra table 2 (14 <= QP < 22)

level

0  1  3  9 13 19 23 31

*  5 15 27 37 49 57  *

*  7 25 43  *  *  *  *

* 11 35  *  *  *  *  *

* 17 45  *  *  *  *  *

run* 21 51  *  *  *  *  *

| * 29  *  *  *  *  *  *

V * 33  *  *  *  *  *  *

* 39  *  *  *  *  *  *

* 41  *  *  *  *  *  *

* 47  *  *  *  *  *  *

* 53  *  *  *  *  *  *

* 55  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

Intra table 3 (QP>=22)

level

0  1  5 13 21 33 43 57

*  3 17 31 49  *  *  *

*  7 25 47  *  *  *  *

*  9 35  *  *  *  *  *

* 11 41  *  *  *  *  *

run* 15 51  *  *  *  *  *

| * 19  *  *  *  *  *  *

V * 23  *  *  *  *  *  *

* 27  *  *  *  *  *  *

* 29  *  *  *  *  *  *

* 37  *  *  *  *  *  *

* 39  *  *  *  *  *  *

* 45  *  *  *  *  *  *

* 53  *  *  *  *  *  *

* 55  *  *  *  *  *  *

*  *  *  *  *  *  *  *

# References

[Q15-I-39]	Gisle Bjontegaard, 'Addition of 8x8 transform to H.26L', ITU-T SG16/Q15, Doc Q15-I-39, Red Bank, 1999

[JVT-B118r7]	Thomas Wiegand, Ed., 'Working Draft Number 2', JVT-B118r7, Geneva, 2002

[VCEG-M33]	Gisle Bjontegaard, 'Calculation of average PSNR differences between RD 
curves,' Document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

# Simulation Results

## Simulation Conditions Interlaced

The following conditions were used for the interlaced simulations. Since Interlaced coding was not available in JM20, the IP BB BB PP mode could not be used in these simulations. Both JM20 and ABT are encoded with the same configuration.

| Seq. Type | - CCIR - HHR |
| --- | --- |
| Picture Structure | - II II II - IB BB BB PB BB BB PB |
| Motion Resolution | ¼ pel |
| Quantization Parameters | 16,20,24,28 [QP(B) = QP(I/P)+2] |
| Search Range | 16 pixel |
| Hadamard (SATD) | ON |
| Reference Frames | 2 |
| Entropy Coding | CABAC |

The following test sequences were used

| Sequence Name | Frame Size | No. of Frames |
| --- | --- | --- |
| F1_car | 720x576 | 220 |
| Canoa | 720x576 | 220 |
| Rugby | 720x576 | 220 |
| Tempete | 720x480 | 260 |
| Football | 720x480 | 260 |
| Bus | 720x480 | 150 |

The sequences (except for Bus) are available from the VQEG ftp site (www.vqeg.org). They were converted from 422 to 420 using the converter tool provided by Peter Borgwardt [VCEG-N85]. Only field based coding was used for the simulations.

## Results

|     CCIR I5BP    |      CCIR III     |     HHR I5BP   |      HHR III

Seq     |   dSNR,    dRate |    dSNR,    dRate |  dSNR,    dRate|   dSNR,    dRate

Bus     |  0.280 dB  -6.46 |  0.134 dB   -2.13 | 0.237 dB  -5.83| -0.015 dB   0.20

Canoa   |  0.161 dB  -4.87 |  0.267 dB   -5.54 | 0.175 dB  -4.86|  0.225 dB  -3.95

F1 Car  |  0.066 dB  -1.61 |  0.179 dB   -2.89 | 0.053 dB  -1.49|  0.053 dB  -0.89

Football|  0.284 dB  -6.81 |  0.279 dB   -5.70 | 0.134 dB  -3.35|  0.063 dB  -1.34

Rugby   |  0.258 dB  -5.68 |  0.359 dB   -6.09 | 0.093 dB  -2.19|  0.108 dB  -2.01

Tempete |  0.327 dB  -8.81 |  0.235 dB   -3.18 | 0.182 dB  -4.96|  0.046 dB

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Institut und Lehrstuhl für Nachrichtentechnik, RWTH Aachen |  |  |
| Mailing address | Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 53056 Aachen, Germany |  |  |
| Country | Germany |  |  |
| Contact person | Mathias Wien |  |  |
| Telephone | +49-241-8027681 |  |  |
| Fax | +49-241-8022196 |  |  |
| Email | wien@ient.rwth-aachen.de |  |  |
| Place and date of submission | Fairfax, VA, USA, May 6, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | 16bit Adaptive Block size Transforms |  |
| Contribution number |  | JVT-C107 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  | Robert Bosch GmbH |  |  |
| Mailing address |  | PO Box 106050 70049 Stuttgart |  |  |
| Country |  | Germany |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: