| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C107r1 Filename: JVT-C107r1.doc |
| --- | --- |

| Title: | 16 bit Adaptive Block size Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mathias Wien, Achim Dahlhoff Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52056 Aachen, Germany | Tel: Email: | ++49-241-8027681 wien@ient.rwth-aachen.de |
| Source: | RWTH Aachen |  |  |

_____________________________

# Introduction

In this paper we present the revision of the ABT proposal according to the current working draft WD2. The transform design is modified to fit into 16 bits. The new JM 4x4 transform and an 8x8 transform proposed by Gisle Bjontegaard are used. The new design exhibits less ringing artifacts and superior subjective quality compared to the TML9 proposal.

The CABAC encoding mechanism is redesigned and a VLC solution is proposed. The intra prediction is extended to 9 prediction modes as adopted at the last meeting. The accompanying contribution JVT-C108 provides the integration of the ABT proposal into the JM20 software. In this paper, the coding elements needed for ABT coding are defined.

The paper is organized as follows: First, the modifications to the bit stream syntax are described. Then the ABT decoding process is described in more detail. A set of simulation results and RD plots is given. Subjective demos will be given at the meeting.

# ABT Syntax Elements

## Slice Header Syntax

[MWI: not described in JWD2r8,. Include useABT symbol, (useABT=0: no ABT, useABT=1:inter ABT, useABT=2:inter&intraABT)]

| slice_header( ) { | Category | Mnemonic |
| --- | --- | --- |
| Qq |  |  |
| } |  |  |

## Prediction data for 8x8 modes

[MWI: BlockIntra4x4 might be renamed to BlockIntra8x8]

| prediction8x8( mb_type ) { | Category | Mnemonic |
| --- | --- | --- |
| region8x8_type | 4 | ecselbf |
| if ( region8x8_type = = BlockIntra4x4 ) { |  |  |
| if ( useABT > 1 ) |  |  |
| intra_block_modeABT /* determines num_ipred */ | 4 | ecselbf |
| for ( i4x4=0; i4x4< num_ipred; i4x4++ ) |  |  |
| intra_pred_mode | 4 | ecselbf |
| } else { |  |  |
| …. |  |  |

## Prediction data for 16x16 modes

| prediction16x16( mb_type ) { | Category | Mnemonic |
| --- | --- | --- |
| if ( mb_type = = MBintra4x4 ) |  |  |
| if ( useABT > 1 ) |  |  |
| intra_block_modeABT /* determines num_ipred */ | 4 | ecselbf |
| for ( i4x4=0; i4x4< num_ipred; i4x4++ ) |  |  |
| intra_pred_mode | 4 | ecselbf |
| else if ( mb_type != MBintra16x16 ) { |  |  |
| … |  |  |

## Residual Data Syntax

| residual( mb_type ) { | Category | Mnemonic |
| --- | --- | --- |
| if ( mb_type = = MBintra16x16 ) |  |  |
| do |  |  |
| luma_dc_zigzag_token | 5 ¦ 6 | ecselbf |
| while ( lum_dc_zigzag_token != EOB ) |  |  |
| for ( i8x8=0; i8x8<4; i8x8++ ) /* each luma 8x8 region */ |  |  |
| /* ABT number of sub-blocks num_blks depends on block mode non-ABT: num_blks=4 */ |  |  |
| for (i4x4=0; i4x4< num_blks; i4x4++) /* each sub-block of region */ |  |  |
| if (cbp & (1<< i8x8) |  |  |
| if ( double_scan() ) { |  |  |
| do |  |  |
| luma_dblscan0_token | 5 ¦ 6 | ecselbf |
| while ( luma_dblscan0_token != EOB ) |  |  |
| do |  |  |
| luma_dblscan1_token | 5 ¦ 6 | ecselbf |
| while ( luma_dblscan1_token != EOB ) |  |  |
| } else |  |  |
| do |  |  |
| luma_zigzag_token | 5 ¦ 6 | ecselbf |
| while ( luma_zigzag_token != EOB ) |  |  |
| if (cbp & 0x30) { /* chroma DC residual coded */ |  |  |
| … |  |  |

# ABT Semantics

## Luminance intra coding

If useABT = = 2, ABT intra coding is used. ABT intra coding is very similar to base line 4x4 intra coding. The block size used for prediction is extended to 4x4, 8x4, 4x8, or 8x8 pixels. The transform block sizes is chosen accordingly. MBintra16x16 is not used with ABT intra.

### Prediction modes for ABT blocks

The 9 intra prediction modes of base line are modified to be used for ABT intra blocks larger than 4x4. ABT 4x4 intra blocks are predicted using the base line technique.

### Prefiltered prediction

Before employed for prediction, the edge pixels of (available) neighboring blocks are gathered in a prediction vector EP. Let P denote the predicted block of N pixels in M lines. P[m,n] is the pixel at horizontal position n, 0<=n<N, and vertical position m, 0<=m<M. The pixels used for prediction shall be identified by R[m,n], with n=-1 & -1<=m and –1<n & m=-1. Only decoded pixels of the same slice are valid for prediction.

The prediction vector EP contains the reconstructed luminance at valid positions of R. EP is constructed according to the following pseudo code:

offset = 0;

valid_left_down = valid(R[M:M+N-1,-1]);

valid_left      = valid(R[0:M-1,-1]);

valid_up        = valid(R[-1,0:N-1]);

valid_up_right  = valid(R[-1,N:N+M-1]);

if valid_left_down

{

offset += N;

for (k=0; k<N; k++)

EP[offset-k] = R[k,-1];

}

if valid_left

{

offset += M;

for (k=0; k<M; k++)

EP[offset-k] = R[k,-1];

EP[offset+1] = R[0,-1];

}

offset+=1;

if valid_up

{

for (k=0; k<N; k++)

EP[offset+k+1] = R[-1,k];

EP[offset] = R[-1,0];

}

if valid_up_right

{

for (k=N; k<N+M; k++)

EP[offset+k+1] = R[-1,k];

}

if (valid_left & valid_up)

EP[offset] = R[-1,-1];

To enhance prediction quality, this vector EP is filtered using the filter

[1, 2, 1] / 4

with constant extension at the boundaries.

#### Mode 0: DC-Prediction

All positions of P are predicted by sum(EP), i.e. the sum of all elements of EP. If there are no valid edge pixels, the block is predicted by P[m,n]=128, 0<=n,m<N,M.

#### Mode 1: Vertical Prediction

if(valid_up)

for(y=0;y<M;y++)

for(x=0;x<N;x++)

P[y,x]=EP[offset+1+x];

#### Mode 2: Horizontal Prediction

if(valid_left)

for(y=0;y<M;y++)

for(x=0;x<N;x++)

P[y,x]=EP[offset-1-y];

#### Mode 3: Down-Right Prediction

if(valid_left&&valid_up)

for(y=0;y<M;y++)

for(x=0;x<N;x++)

P[y,x]=EP[offset+x-y];

#### Mode 4: Up-Right Prediction (Bidirectional)

if(valid_left&&valid_up)

for(y=0;y<M;y++)

for(x=0;x<N;x++)

P[y,x]=(EP[offset+2+x+y]+EP[offset-2-(x+y)])>>1;

#### Mode 5: Down-Right-Down Prediction

if(valid_left&&valid_up)

{

for(y=0;y<M;y+=2)//even lines

for(x=0;x<N;x++)

if( (i=x-(y>>1)) >= 0 )

P[y,x]=(EP[offset+i]+EP[offset+1+i])>>1;

else

P[y,x]=EP[offset+1+2*x-y];

for(y=1;y<M;y+=2)//odd lines

for(x=0;x<N;x++)

if( (i=x-(y>>1)) >= 0 )

P[y,x]=EP[offset+i];

else

P[y,x]=EP[offset+1+2*x-y];

}

#### Mode 6: Down-Left-Down Prediction

if(valid_left&&valid_up)

{

for(y=0;y<M;y+=2)//even lines

for(x=0;x<N;x++)

P[y,x]=(EP[offset+1+x+(y>>1)]+EP[offset+2+x+(y>>1)])>>1;

for(y=1;y<M;y+=2)//odd lines

for(x=0;x<N;x++)

P[y,x]=EP[offset+2+x+(y>>1)];

}

#### Mode 7: Right-Up-Right Prediction

if(valid_left&&valid_up)

{

for(y=0;y<M;y++)//even columns

for(x=0;x<N;x+=2)

P[y,x]=(EP[offset+-1-(y+(x>>1))]+EP[offset+-2-(y+(x>>1))])>>1;

for(y=0;y<M;y++)//odd columns

for(x=1;x<N;x+=2)

P[y,x]=EP[offset+-2-(y+(x>>1))];

}

#### Mode 8: Right-Down-Right Prediction

if(valid_left&&valid_up)

{

for(y=0;y<M;y++)//even columns

for(x=0;x<N;x+=2)

if( (i=-y+(x>>1)) <= 0 )

P[y,x]=(EP[offset+i]+EP[offset+i-1])>>1;

else

P[y,x]=EP[offset-1-2*y+x];

for(y=0;y<M;y++)//odd columns

for(x=1;x<N;x+=2)

if( (i=-y+(x>>1)) <= 0 )

P[y,x]=EP[offset+i];

else

P[y,x]=EP[offset-1-2*y+x];

}

### Coding of ABT intra prediction modes

For encoding, the ABT intra prediction modes of the subblocks of a macroblock are grouped in pairs. Since Intra blocks of variable block size might be encoded in one macroblock, a rule has to be defined for grouping the prediction modes: The prediction modes are grouped left to right, top to bottom.

1

2

3

4

5

##### Figure 1: Macroblock in 8x8 Mode with 4 ABT intra subblocks

If the number of intra coded blocks is odd, the prediction mode of the last intra subblock is grouped with a '0' for encoding. E.g. in Figure 1, a macroblock in 8x8 mode containing subblocks with tilings 8x8, 8x8, 8x8, 8x4, is given. The 8x8 prediction modes of subblocks 1,2 are grouped together for encoding. The prediction mode of block 3 is grouped with prediction mode of block 4. The prediction mode of block 5 is grouped with a prediction mode '0' for encoding.

For encoding of the intra prediction modes, the base line encoding method is used.

## ABT Transform Coefficient Decoding

Two transforms, one for blocks of length 8 and one for blocks of length 4, are used for ABT coding. In the following, matrix notation is used to describe the inverse transform process. The 4x4 transform matrix and inverse transform matrix are the ones used for transform when ABT is not employed. The 4x4 transform features different norms in the even and odd basis functions, leading to a set of 3 different norms for a 2-dimensional transform. These norms have to be regarded during the quantization and dequantization processes. The 8x8 transform matrix proposed  for ABT was originally proposed by Gisle Bjontegaard in [Q15-I-39]. This transform matrix has one norm for all basis functions. Transform and inverse transform matrix are identical. The transform matrices are given below.

T8 = [

13, 13, 13, 13, 13, 13, 13, 13

19, 15,  9,  3, -3, -9,-15,-19

17,  7, -7,-17,-17, -7,  7, 17

9,  3,-19,-15, 15, 19, -3, -9

13,-13,-13, 13, 13,-13,-13, 13

15,-19, -3,  9, -9,  3, 19,-15

7,-17, 17, -7, -7, 17,-17,  7

3, -9, 15,-19, 19,-15,  9, -3 ];

In contrast to the T8 matrix proposed earlier, this transform allows for a 16 bit implementation.

### Scanning method

ABT transform coefficients are encoded using the (RUN,LEVEL) model. For each transform block size, a 'simple' zig-zag scan is defined. The scans are given in Figure 2.

4x4

4x8

8x8

8x4

Figure 2 : Zig-Zag scan for all transform block sizes

### Inverse Quantization and transform

The inverse quantization of the ABT transform coefficients for each mode uses the dequantization values given in the table below. Please note that the current design is given for quantization parameters QP=0,…, QP=31. The extension of this range is possible.

According to the base line dequantization procedure, the quantizer step size doubles every 6 QP. The coefficient R(k,i,j) used in the following is mode dependend and chosen from the table below. Additionally, a bit shift parameter are defined.

k  S8x8  S8x4,4x8      S4x4

0   15      9 11     40  64 51

1   17     10 12     45  72 57

2   19     11 14     50  81 64

3   22     12 16     57  91 72

4   24     14 17     63 102 80

5   27     15 20     71 114 90

Mode 8x8:

R[k][i][j] = S8x8[k] for all i,j

Bshift = 7;

Mode 8x4:

R[k][i][j] = S8x4,4x8[k][0] for all even j

R[k][i][j] = S8x4,4x8[k][1] for all odd j

Bshift = 2;

Mode 4x8:

R[k][i][j] = S8x4,4x8[k][0] for all even i

R[k][i][j] = S8x4,4x8[k][1] for all odd i

Bshift = 2;

Mode 4x4:

R[k][i][j] = S4,4[k][0] for (i,j) = {(0,0),(0,2),(2,0),(2,2)},

R[k][i][j] = S4,4[k][1] for (i,j) = {(1,1),(1,3),(3,1),(3,3)},

R[k][i][j] = S4,4[k][2]  otherwise;

Bshift = 0;

The reconstructed coefficients are multiplied with the dequantization value R

YD(i,j) = YQ(i,j) * R(QP%6,i,j);   i=0,..,N; j=0,...,M

The resulting block YD of size NxM is then inverse transformed horizontally. If N=4, the one-dimensional inverse transform is performed according to WD2, section 9.3.3.3 [JVT-B118r7].  If N=8, a matrix multiply is performed and the result is rounded,

Z = (YD * TN)//Bshift,

where (YD * TN) denotes the appropriate inverse transform operation. The notation (YD * TN)//Bshift means that the resulting value for each element Z(i,j) is down-shifted by Bshift bits with proper rounding.

Z(i,j) = sign(Z'(i,j) )*[abs(Z'(i,j)) + 2Bshift-1]>>Bshift.

This operation is performed to stay within 16 bit precision. After the vertical inverse transform step the result is finally rounded

X' = (TTM* Z),

X''(i,j) = [X'(i,j) + 26-QP/6-1]>>(6-QP/6).

Finally, the reconstructed sample residual values X''(i,j) are added to the prediction values P(i,j) from motion compensated prediction or spatial prediction and clipped to the range of 0 to 255 to form the final decoded sample result:

S'(i,j) = clip1( P(i,j) + X''(i,j) ).

## Deblocking Filter

Deblocking is employed on all subblock and macroblock boundaries. No modifications are introduced to the deblocking filter besides a modification of the determination of  and .

No filtering is performed at 4x4 block boundaries inside transformed blocks. I.e., the number of block boundaries that have to be filtered is reduced by 1/2 for blocks of size 8x8 and larger and by a factor of 1/4 for 4x8 and 8x4 blocks.

Without ABT, the threshold parameters  and  are driven by the quantization parameter QP. For ABT blocks, the index to the threshold table is increased by IQP. IQP depends on the depth and the width of the neighboring blocks. The depth is the distance to the next block boundary on each side, while the width of the block is the number of pixels at the current boundary.

IQP=0;

if (depth(Block p)==8)

IQP++;

if (depth(Block q)==8)

IQP++;

if (width(Block p)==8)

IQP++;

if (width(Block q)==8)

IQP++;

clip3(0,3, IQP);

## Entropy Coding

For encoding of the ABT coefficients CABAC is employed. New contexts and binarizations are introduced for encoding of the coefficients of ABT blocks. Additionally; a VLC encoding method is introduced to enable encoding of ABT coefficients in VLC mode.

### CABAC

For encoding of the ABT coefficients, the base line methods are used. 4x4 blocks are encoded using exactly the base line routines. The context definition of the coefficients of the other modes is described in the following. There is one set of contexts defined for 8x8 blocks and one for 4x8/8x4 blocks. For all modes, the Levels are encoded using the base line routines.

Coeff_Count is used for encoding of the ABT coefficients. The unary binarization is applied for Coeff_Count <16. For Coeff_Count >=16, the 16 zeros are followed by the binary represenation of (Coeff_Count-16). The sync bit is omitted here. The number of bits used for the binary representation is determined by the maximum possible Coeff_Count value in the current mode. The bits of the representation are encoded from low to high, i.e. 20,21,…

Four contexts are used for the first bin. The context is determined by

ctx_coeff_count(C) = ((CC(A)==0) ? 0 : 1) + 2*(( CC(B)==0)?0:1);

where CC(X) , X=A,B, is the Coeff_Count of the left and top neighboring decoded blocks A and B. The remaining bins are encoded using a context for the 2nd bin, a joint context for the 3rd and 4th bin, a context for the remaining bins of the unary binarization, and one context for the bins of the binary representation.

Run is encoded exploiting the information of Coeff_Count and the previously coded Runs. The binarization of Coeff_Count is used for Runs as well. Here, only one context is used for the first bin. The remaining contexts are defined as with Coeff_Count. Two sets of Run contexts can be chosen, depending on the activity of the block, which is measured by Coeff_Count

ctx_run = ((COEFF_COUNT) >= 4) ? 1 : 0.

### VLC

In VLC mode, all coefficients are encoded as (Level,Run) symbols. Inter blocks use the EOB symbol. For Intra coding, a Coeff_Count symbol is encoded.

Golomb codes of several degrees are employed for encoding of the symbols. The Golomb-0 code is used for encoding of all non-luminance symbols. For ABT, additional Golomb-1,2, and 3 codes are employed. The general structure of these codes is depicted in the table below

| Layer | Golomb-0 | Golomb-1 | Golomb-2 | Golomb-3 |
| --- | --- | --- | --- | --- |
| 0 1 2 3 … | 1 01x 001xx 0001xxx … | 1x 01xx 001xxx 0001xxxx … | 1xx 01xxx 001xxxx 0001xxxxx … | 1xxx 01xxxx 001xxxxx 0001xxxxxx … |

Generally, a Golomb code word consists of a sync word 0…01 and an info word x0x1…xn. For ABT coding, a finite number of symbols is encoded using Golomb codes. Symbols that are not in the finite set are encoded using an Escape symbol and separate coding of Level and Run. The Escape symbol is 59 for all used codes.

Since finite alphabets are encoded, the '1'-bit of the sync word is omitted in the layer containing the Escape symbol.

#### Intra Coeff_Count

Intra Coeff_Count is encoded using the Golomb-2 code:

| Coeff_Count | Golomb-2 symbol |
| --- | --- |
| 0 | 100 |
| 1 | 101 |
| 2 | 110 |
| 3 | 111 |
| 4 | 01000 |
| … | … |

#### 2D (Level,Run) Symbols

The Golomb code used for encoding the (Level,Run) symbols is block mode dependend:

|  | Inter |  | Intra |  |
| --- | --- | --- | --- | --- |
| Block Mode | Degree | Layers | Degree | Layers |
| 8x8 | 0 | 6 | 2 | 4 |
| 8x4,4x8 | 1 | 5 | 2 | 4 |
| 8x8 | 2 | 4 | 2 | 4 |

#### Code Tables

Four tables are used to adapt the set of Golomb codes to the symbol statistics. The intra tables are chosen QP-dependent. The tables contain the (odd) code word numbers for Level>0. Level<0 is encoded using the even numbers. EOB is encoded as code word 0. In case of Intra coding, no EOB symbol is encoded and the symbols are shifted –1 in the table.

Symbols that are not in the table are encoded by sending the Escape Symbol 59 first and then Level using Golomb-3 and Run using Golomb-2. Here, no finite alphabet is assumed. The Levels are arranged as follows

level  code word

1      0

-1      1

2      2

-2      3

3      4

-3      5

4      6

...

The code tables are given below:

Inter

level

0  1  5 13 21 31 39 47

*  3 15 33 51  *  *  *

*  7 25 53  *  *  *  *

*  9 35  *  *  *  *  *

* 11 45  *  *  *  *  *

run* 17 55  *  *  *  *  *

| * 19  *  *  *  *  *  *

V * 23  *  *  *  *  *  *

* 27  *  *  *  *  *  *

* 29  *  *  *  *  *  *

* 37  *  *  *  *  *  *

* 41  *  *  *  *  *  *

* 43  *  *  *  *  *  *

* 49  *  *  *  *  *  *

* 57  *  *  *  *  *  *

*  *  *  *  *  *  *  *

Intra table 1 (QP < 14)

level

0  1  3  7  9 13 19 21

*  5 15 25 31 39 45 49

* 11 29 41 51  *  *  *

* 17 35 55  *  *  *  *

* 23 43  *  *  *  *  *

run* 27 57  *  *  *  *  *

| * 33  *  *  *  *  *  *

V * 37  *  *  *  *  *  *

* 47  *  *  *  *  *  *

* 53  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

Intra table 2 (14 <= QP < 22)

level

0  1  3  9 13 19 23 31

*  5 15 27 37 49 57  *

*  7 25 43  *  *  *  *

* 11 35  *  *  *  *  *

* 17 45  *  *  *  *  *

run* 21 51  *  *  *  *  *

| * 29  *  *  *  *  *  *

V * 33  *  *  *  *  *  *

* 39  *  *  *  *  *  *

* 41  *  *  *  *  *  *

* 47  *  *  *  *  *  *

* 53  *  *  *  *  *  *

* 55  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

*  *  *  *  *  *  *  *

Intra table 3 (QP>=22)

level

0  1  5 13 21 33 43 57

*  3 17 31 49  *  *  *

*  7 25 47  *  *  *  *

*  9 35  *  *  *  *  *

* 11 41  *  *  *  *  *

run* 15 51  *  *  *  *  *

| * 19  *  *  *  *  *  *

V * 23  *  *  *  *  *  *

* 27  *  *  *  *  *  *

* 29  *  *  *  *  *  *

* 37  *  *  *  *  *  *

* 39  *  *  *  *  *  *

* 45  *  *  *  *  *  *

* 53  *  *  *  *  *  *

* 55  *  *  *  *  *  *

*  *  *  *  *  *  *  *

# References

[Q15-I-39]	Gisle Bjontegaard, 'Addition of 8x8 transform to H.26L', ITU-T SG16/Q15, Doc Q15-I-39, Red Bank, 1999

[JVT-B118r7]	Thomas Wiegand, Ed., 'Working Draft Number 2', JVT-B118r7, Geneva, 2002

[VCEG-M33]	Gisle Bjontegaard, 'Calculation of average PSNR differences between RD 
curves,' Document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

# Simulation Results

## Simulation Conditions Interlaced

The following conditions were used for the interlaced simulations. Since Interlaced coding was not available in JM20, the IP BB BB PP mode could not be used in these simulations. Both JM20 and ABT are encoded with the same configuration.

| Seq. Type | - CCIR - HHR |
| --- | --- |
| Picture Structure | - II II II - IB BB BB PB BB BB PB |
| Motion Resolution | ¼ pel |
| Quantization Parameters | 16,20,24,28 [QP(B) = QP(I/P)+2] |
| Search Range | 16 pixel |
| Hadamard (SATD) | ON |
| Reference Frames | 2 |
| Entropy Coding | CABAC |

The following test sequences were used

| Sequence Name | Frame Size | No. of Frames |
| --- | --- | --- |
| F1_car | 720x576 | 220 |
| Canoa | 720x576 | 220 |
| Rugby | 720x576 | 220 |
| Tempete | 720x480 | 260 |
| Football | 720x480 | 260 |
| Bus | 720x480 | 150 |

The sequences (except for Bus) are available from the VQEG ftp site (www.vqeg.org). They were converted from 422 to 420 using the converter tool provided by Peter Borgwardt [VCEG-N85]. Only field based coding was used for the simulations.

## Results Interlaced

|     CCIR I5BP    |      CCIR III     |     HHR I5BP   |      HHR III

Seq     |   dSNR,    dRate |    dSNR,    dRate |  dSNR,    dRate|   dSNR,    dRate

Bus     |  0.280 dB  -6.46 |  0.134 dB   -2.13 | 0.237 dB  -5.83| -0.015 dB   0.20

Canoa   |  0.161 dB  -4.87 |  0.267 dB   -5.54 | 0.175 dB  -4.86|  0.225 dB  -3.95

F1 Car  |  0.066 dB  -1.61 |  0.179 dB   -2.89 | 0.053 dB  -1.49|  0.053 dB  -0.89

Football|  0.284 dB  -6.81 |  0.279 dB   -5.70 | 0.134 dB  -3.35|  0.063 dB  -1.34

Rugby   |  0.258 dB  -5.68 |  0.359 dB   -6.09 | 0.093 dB  -2.19|  0.108 dB  -2.01

Tempete |  0.327 dB  -8.81 |  0.235 dB   -3.18 | 0.182 dB  -4.96|  0.046 dB

## Simulation Conditions VLC

In this simulation, the common testing set was encoded using the settings stated below.

| Picture Structure | - III - IBBP |
| --- | --- |
| Motion Resolution | ¼ pel |
| Quantization Parameters | 16,20,24,28 [QP(B) = QP(I/P)] |
| Search Range | 16 pixel |
| Hadamard (SATD) | ON |
| Reference Frames | 2 |
| Entropy Coding | VLC |

## Results VLC

IBBP              III

sequence            |   dSNR    dRate |   dSNR    dRate |

container qcif 10hz | 0.021 dB  -0.42 | -0.148 dB  2.36 |

foreman   qcif 10hz | 0.330 dB  -5.97 |  0.201 dB -3.25 |

news      qcif 10hz | 0.070 dB  -1.10 | -0.014 dB  0.38 |

silent    qcif 15hz | 0.284 dB  -6.02 |  0.069 dB -1.26 |

paris      cif 15hz | 0.204 dB  -3.97 | -0.029 dB  0.51 |

mobile     cif 30hz | 0.150 dB  -3.58 |  0.161 dB -1.68 |

tempete    cif 30hz | 0.180 dB  -4.63 |  0.141 dB -1.97 |

# ANNEX A – CCIR RD Plots

bus ccir field jm20 ccir I5BP

QP     Rate	PSNR

28   138290	25.95

24   253465	28.32

20   471393	30.89

16   879404	33.65

bus ccir field abt2 ccir I5BP

QP     Rate	PSNR

28   138311	26.25

24   254468	28.61

20   474645	31.18

16   858797	33.88

canoa ccir field jm20 ccir I5BP

QP     Rate	PSNR

28   202144	26.07

24   412896	28.32

20   815089	30.61

16  1525691	32.79

canoa ccir field abt2 ccir I5BP

QP     Rate	PSNR

28   187958	26.12

24   386141	28.29

20   817723	30.75

16  1578413	32.98

f1 car ccir field jm20 ccir I5BP

QP     Rate	PSNR

28   130787	26.33

24   359104	28.63

20   829393	31.33

16  1587298	34.19

f1 car ccir field abt2 ccir I5BP

QP     Rate	PSNR

28   136359	26.35

24   338014	28.53

20   836612	31.41

16  1548262	34.37

football ccir field jm20 ccir I5BP

QP     Rate	PSNR

28   142631	28.43

24   248412	30.37

20   441240	32.58

16   765174	35.03

football ccir field abt2 ccir I5BP

QP     Rate	PSNR

28   134053	28.34

24   234212	30.41

20   424407	32.74

16   740216	35.26

rugby ccir field jm20 ccir I5BP

QP     Rate	PSNR

28   243766	26.69

24   449195	28.97

20   803451	31.46

16  1384155	34.14

rugby ccir field abt2 ccir I5BP

QP     Rate	PSNR

28   230050	26.63

24   427113	28.99

20   781168	31.61

16  1349182	34.43

tempete ccir field jm20 ccir I5BP

QP     Rate	PSNR

28    95723	25.37

24   200040	27.85

20   451454	30.57

16   947095	33.48

tempete ccir field abt2 ccir I5BP

QP     Rate	PSNR

28   103859	26.08

24   200957	28.27

20   438507	30.73

16   902768	33.49

bus ccir field jm20 ccir III

QP     Rate	PSNR

28   831759	27.59

24  1370333	30.30

20  2156839	33.11

16  3206397	36.11

bus ccir field abt2 ccir III

QP     Rate	PSNR

28   823303	27.70

24  1332845	30.25

20  2130629	33.17

16  3213375	36.30

canoa ccir field jm20 ccir III

QP     Rate	PSNR

28   798385	28.14

24  1317148	30.55

20  2124698	32.82

16  3417129	35.16

canoa ccir field abt2 ccir III

QP     Rate	PSNR

28   765236	28.42

24  1252425	30.63

20  2058980	32.89

16  3441982	35.28

f1 car ccir field jm20 ccir III

QP     Rate	PSNR

28   889662	28.29

24  1539561	31.07

20  2437883	33.92

16  3573925	36.83

f1 car ccir field abt2 ccir III

QP     Rate	PSNR

28   885903	28.36

24  1483145	31.02

20  2371022	33.96

16  3541709	36.98

football ccir field jm20 ccir III

QP     Rate	PSNR

28   472852	30.21

24   802111	32.40

20  1329590	34.71

16  2063007	37.23

football ccir field abt2 ccir III

QP     Rate	PSNR

28   462326	30.30

24   761662	32.42

20  1274429	34.83

16  2015199	37.42

rugby ccir field jm20 ccir III

QP     Rate	PSNR

28   790602	28.81

24  1320784	31.30

20  2118538	33.95

16  3200304	36.80

rugby ccir field abt2 ccir III

QP     Rate	PSNR

28   763439	28.97

24  1249006	31.35

20  2035948	34.08

16  3136282	37.08

tempete ccir field jm20 ccir III

QP     Rate	PSNR

28  1203177	27.00

24  1878837	29.98

20  2812857	32.92

16  4021817	35.93

tempete ccir field abt2 ccir III

QP     Rate	PSNR

28  1168530	27.17

24  1802676	29.93

20  2750741	32.96

16  4002581	36.07

# ANNEX B – HHR RD Plots

bus hhr field jm20 hhr I5BP

QP     Rate	PSNR

28    62794	24.78

24   124255	27.15

20   246172	29.79

16   474812	32.71

bus hhr field abt2 hhr I5BP

QP     Rate	PSNR

28    62855	25.12

24   124037	27.40

20   247101	30.00

16   466494	32.82

canoa hhr field jm20 hhr I5BP

QP     Rate	PSNR

28   101186	25.34

24   221812	27.62

20   460761	30.17

16   844651	32.88

canoa hhr field abt2 hhr I5BP

QP     Rate	PSNR

28    95594	25.43

24   209355	27.63

20   460221	30.31

16   857872	33.10

f1 car hhr field jm20 hhr I5BP

QP     Rate	PSNR

28    60494	25.40

24   174057	27.62

20   436618	30.34

16   887723	33.33

f1 car hhr field abt2 hhr I5BP

QP     Rate	PSNR

28    63462	25.46

24   171573	27.64

20   442397	30.43

16   867354	33.39

football hhr field jm20 hhr I5BP

QP     Rate	PSNR

28    83490	27.39

24   150218	29.34

20   273775	31.61

16   482889	34.19

football hhr field abt2 hhr I5BP

QP     Rate	PSNR

28    78044	27.25

24   142326	29.26

20   267957	31.67

16   476047	34.34

rugby hhr field jm20 hhr I5BP

QP     Rate	PSNR

28   145693	25.39

24   281528	27.58

20   521187	30.06

16   911553	32.83

rugby hhr field abt2 hhr I5BP

QP     Rate	PSNR

28   136441	25.26

24   269265	27.50

20   516978	30.11

16   910478	33.00

tempete hhr field jm20 hhr I5BP

QP     Rate	PSNR

28    52199	24.20

24   114662	26.70

20   271193	29.52

16   587141	32.69

tempete hhr field abt2 hhr I5BP

QP     Rate	PSNR

28    56512	24.77

24   116445	27.01

20   272081	29.64

16   578480	32.67

bus hhr field jm20 hhr III

QP     Rate	PSNR

28   543224	26.35

24   908315	29.06

20  1435410	32.00

16  2117445	35.23

bus hhr field abt2 hhr III

QP     Rate	PSNR

28   536950	26.34

24   886369	28.90

20  1424876	31.92

16  2128291	35.25

canoa hhr field jm20 hhr III

QP     Rate	PSNR

28   497078	27.30

24   851140	29.90

20  1373939	32.60

16  2069796	35.45

canoa hhr field abt2 hhr III

QP     Rate	PSNR

28   476406	27.42

24   806682	29.85

20  1329459	32.61

16  2048400	35.55

f1 car hhr field jm20 hhr III

QP     Rate	PSNR

28   538201	27.03

24   989680	29.82

20  1610534	32.90

16  2351655	36.13

f1 car hhr field abt2 hhr III

QP     Rate	PSNR

28   539830	27.01

24   963912	29.71

20  1574397	32.84

16  2318167	36.12

football hhr field jm20 hhr III

QP     Rate	PSNR

28   311383	29.12

24   527511	31.31

20   886904	33.69

16  1392696	36.43

football hhr field abt2 hhr III

QP     Rate	PSNR

28   305012	29.09

24   509884	31.18

20   867817	33.67

16  1383016	36.54

rugby hhr field jm20 hhr III

QP     Rate	PSNR

28   512236	27.36

24   877991	29.78

20  1447665	32.44

16  2210876	35.46

rugby hhr field abt2 hhr III

QP     Rate	PSNR

28   498556	27.39

24   842505	29.67

20  1417588	32.43

16  2201848	35.56

tempete hhr field jm20 hhr III

QP     Rate	PSNR

28   776125	25.65

24  1243617	28.82

20  1856189	32.11

16  2593607	35.56

tempete hhr field abt2 hhr III

QP     Rate	PSNR

28   756005	25.67

24  1198694	28.61

20  1829184	31.98

16  2599385	35.56

# ANNEX C – VLC RD Plots

container qcif jm20 com vlc red IBBP 10Hz

QP     Rate	PSNR

28     4010	27.93

24     6321	30.70

20    10485	33.43

16    18341	36.06

container qcif abt2 com vlc red IBBP 10Hz

QP     Rate	PSNR

28     4061	28.15

24     6480	30.94

20    10899	33.59

16    19416	36.20

foreman qcif jm20 com vlc red IBBP 10Hz

QP     Rate	PSNR

28    16278	27.97

24    26438	30.50

20    43356	33.12

16    69448	35.80

foreman qcif abt2 com vlc red IBBP 10Hz

QP     Rate	PSNR

28    16312	28.27

24    26015	30.80

20    42819	33.36

16    69250	36.06

news qcif jm20 com vlc red IBBP 10Hz

QP     Rate	PSNR

28     8765	28.14

24    14856	30.75

20    24822	33.76

16    40643	36.68

news qcif abt2 com vlc red IBBP 10Hz

QP     Rate	PSNR

28     8851	28.12

24    14965	30.99

20    25592	33.90

16    41478	36.90

silent qcif jm20 com vlc red IBBP 15Hz

QP     Rate	PSNR

28    11095	27.84

24    20133	30.43

20    35269	32.94

16    58312	35.74

silent qcif abt2 com vlc red IBBP 15Hz

QP     Rate	PSNR

28    10690	28.11

24    19352	30.49

20    33901	33.07

16    57258	35.85

paris cif jm20 com vlc red IBBP 15Hz

QP     Rate	PSNR

28    62596	26.76

24   110156	29.52

20   200308	32.39

16   338723	35.38

paris cif abt2 com vlc red IBBP 15Hz

QP     Rate	PSNR

28    62056	26.94

24   107748	29.66

20   198116	32.52

16   340165	35.52

mobile cif jm20 com vlc red IBBP 30Hz

QP     Rate	PSNR

28   159919	24.85

24   309810	27.56

20   683297	30.53

16  1424133	33.78

mobile cif abt2 com vlc red IBBP 30Hz

QP     Rate	PSNR

28   179713	25.65

24   340514	28.17

20   723919	30.83

16  1484619	33.95

tempete cif jm20 com vlc red IBBP 30Hz

QP     Rate	PSNR

28   125165	26.53

24   238234	29.03

20   509018	31.73

16  1088956	34.72

tempete cif abt2 com vlc red IBBP 30Hz

QP     Rate	PSNR

28   132193	27.15

24   249609	29.51

20   525854	31.94

16  1135707	34.81

container qcif jm20 com vlc red III 10Hz

QP     Rate	PSNR

28    78251	28.81

24   124249	31.55

20   184606	34.35

16   274336	37.10

container qcif abt2 com vlc red III 10Hz

QP     Rate	PSNR

28    82064	28.72

24   123036	31.42

20   187942	34.31

16   282078	37.11

foreman qcif jm20 com vlc red III 10Hz

QP     Rate	PSNR

28    72398	28.64

24   113420	31.26

20   179697	33.99

16   269763	36.79

foreman qcif abt2 com vlc red III 10Hz

QP     Rate	PSNR

28    71052	28.85

24   109593	31.27

20   174754	34.00

16   268498	36.88

news qcif jm20 com vlc red III 10Hz

QP     Rate	PSNR

28    93825	28.61

24   144657	31.52

20   208806	34.63

16   294410	37.66

news qcif abt2 com vlc red III 10Hz

QP     Rate	PSNR

28    95511	28.63

24   140550	31.47

20   210602	34.58

16   307060	37.72

silent qcif jm20 com vlc red III 15Hz

QP     Rate	PSNR

28   106817	28.49

24   171213	31.01

20   278858	33.55

16   429794	36.34

silent qcif abt2 com vlc red III 15Hz

QP     Rate	PSNR

28   107969	28.58

24   169023	30.97

20   274018	33.57

16   433162	36.49

paris cif jm20 com vlc red III 15Hz

QP     Rate	PSNR

28   693910	27.56

24  1090822	30.49

20  1612255	33.54

16  2313934	36.58

paris cif abt2 com vlc red III 15Hz

QP     Rate	PSNR

28   718204	27.60

24  1069984	30.43

20  1612100	33.47

16  2362770	36.61

mobile cif jm20 com vlc red III 30Hz

QP     Rate	PSNR

28  2498605	25.38

24  3970523	28.45

20  5748062	31.77

16  8119252	35.22

mobile cif abt2 com vlc red III 30Hz

QP     Rate	PSNR

28  2491726	25.45

24  3773626	28.40

20  5718642	31.76

16  8254530	35.27

tempete cif jm20 com vlc red III 30Hz

QP     Rate	PSNR

28  1333403	26.95

24  2212207	29.77

20  3423846	32.83

16  5050170	36.04

tempete cif abt2 com vlc red III 30Hz

QP     Rate	PSNR

28  1335320	27.02

24  2119354	29.71

20  3354786	32.80

16  5061904	36.09

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Institut und Lehrstuhl für Nachrichtentechnik, RWTH Aachen |  |  |
| Mailing address | Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 53056 Aachen, Germany |  |  |
| Country | Germany |  |  |
| Contact person | Mathias Wien |  |  |
| Telephone | +49-241-8027681 |  |  |
| Fax | +49-241-8022196 |  |  |
| Email | wien@ient.rwth-aachen.de |  |  |
| Place and date of submission | Fairfax, VA, USA, May 6, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | 16bit Adaptive Block size Transforms |  |
| Contribution number |  | JVT-C107 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  | Robert Bosch GmbH |  |  |
| Mailing address |  | PO Box 106050 70049 Stuttgart |  |  |
| Country |  | Germany |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: