| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C111 Filename: JVT-C111.doc |
| --- | --- |

| Title: | Revised Downloadable Threshold Tables for Loop-Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teck-Wee Foo Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Sheng-Mei Shen Block 1022 Tai Seng Avenue #04-3530 Singapore 534415 Shinya Kadono 1006 Kadomo, Kadomo, Osaka 573-8501 Japan | Tel: Email: Tel: Email: Tel: Email: | (65) 6550-5474 twfoo@psl.com.sg (65) 6550-5466 shen@psl.com.sg +81-6-6900-9689 kadono@drl.mei.co.jp |
| Source: | Panasonic Singapore Laboratories Pte. Ltd. |  |  |

_____________________________

# Summary

Downloadable threshold table has been proposed in [2]. The recommendation for this tool to be included into the JVT codec [1] is because the current threshold tables in the loop-filter is optimized to cover all picture sequences and picture sizes. If using the same threshold tables to encode all video sequences, some sequences would not be encoded with the best subjective quality. With the ability to change the threshold tables, the encoder is able to tweak the values to maximize picture quality for certain sequences. The same tool could also make the codec future-proof. Evidences are provided here to support the need for this tool

# Introduction

Loop-filter is divided into two parts, the detection of block edge and the filtering part. The detection of block edge is dependent on a set of threshold values, in the case of JVT codec, the ALPHA and BETA tables. The quantization parameter (QP) of the macroblock will determine which ALPHA and BETA values to be used for detecting the block edge.

# Method

In [2], we proposed 4 bits (1 sign bit, and 3 bits for value) to code the difference for each QP.

After performing more experiments, we would like to revise the proposal to the values below (Table 1).

Table 1: New coding values for storing threshold value differences.

| Coding | Value |
| --- | --- |
| 0000 | 0 |
| 0001 | +1 |
| 0010 | +2 |
| 0011 | +3 |
| 0100 | +4 |
| 0101 | +5 |
| 0110 | +6 |
| 0111 | +7 |
| 1000 | +8 |
| 1001 | +9 |
| 1010 | +10 |
| 1011 | +11 |
| 1100 | +12 |
| 1101 | +13 |
| 1110 | +14 |
| 1111 | +15 |

# Experiments

The results below are derived from testing selected threshold values.

We have run the test on QCIF, CIF and CCIR601 video sequences to show the results of default tables and the tweaked tables for the loop-filter. Due to time constraint, only few sequences have been tested.

# Results

The pictures on the left are from the default tables.

The pictures on the right are from the tweaked tables.

Figure 1: Silent_QCIF (QP16) ALPHA_delta = +2, BETA_delta = +4

Figure 2: Foreman_QCIF (QP16), ALPHA_delta = +4, BETA_delta = +2

Figure 3: Paris_CIF (QP16), ALPHA_delta = +5, BETA_delta = 0

Figure 4: Akiyo_CCIR (QP20), ALPHA_delta = +10, BETA_delta = +10

Figure 5: Akiyo_CCIR (QP24), ALPHA_delta = +12, BETA_delta = +14

# Conclusion

The results of the experiments have shown that changing the values in the threshold tables could improve the picture quality. We therefore, recommend that the JVT adopt the proposed tool into the next JVT draft.

# References

[1]	Thomas Wiegand, Gary Sullivan, “Joint Video Team Working Draft Number 2, Revision 4”, Geneva, Switzerland, February 2002.

[2]	Teck-Wee Foo, Sheng-Mei Shen, Shinya Kadono, “Downloadable Threshold Tables for Loop-filter”, JVT-B084, JVT Meeting (ITU-T/ISO), Geneva, Switzerland, February 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma, Osaka, 573-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | Fairfax, VA, USA, 6-10 May, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Revised Downloadable Threshold Tables for Loop-Filter |  |
| Contribution number |  |  |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: