| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C115 Filename: JVT-C115.doc |
| --- | --- |

| Title: | Reduced Decoder Peak Bus Bandwidth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Contacts and Sources: | Lowell Winger Videolocus, Inc. 97 Randall Dr Waterloo, Ontario, CANADA, N2V1C5 | Tel: Email: | +1 519 725 9797 lwinger@videolocus.com |

# Introduction

The current decoder can be quite demanding in terms of the worst-case bus bandwidth required to be compliant in all cases. This proposal suggests a solution to reduce the system cost of a compliant decoder by reducing the worst-case bus bandwidth.

Physical memory performs best when accessed in a regular fashion. To reduce bus bandwidth, accesses to non-local (reference-frame) memory for motion compensation should be as regular as possible.   In the current JM, the main concern is the maximum number of small blocks that must be accessed in order to motion compensate a given macroblock.  Physical memory may be structured to perform efficiently for something as regular as an individual block access; however, as the total number of block accesses increases and block size decreases, overhead (such as wastage due to minimum access unit sizes – eg. SDRAM burst lengths) increases unavoidably.  The total number of pixels moved across the bus is increased by the additional overhead of the extra “border” reference memory that must accompany each block in order to do 6-tap sub-pixel interpolation for each referenced block.  Our proposal is a solution to reduce the maximum total number of block accesses without affecting the rate-distortion performance of the current JM.

# Method

The worst case motion compensation for the current JM is sixteen 4x4 motion-compensated sub-blocks in direct or bi-direct mode in a B-frame, each requiring 6-tap interpolation. We propose to reduce the worst-case number of block accesses in half by restricting bi-directional mode to only use sub-blocks larger than 4x4, and by using half the number of direct mode motion vectors when motion compensation of 4x4 blocks is used in the temporally subsequent co-located macroblock. (the worst case number of motion vectors per MB is halved).

Under the common conditions of VCEG-N81, restricting the bi-directional mode to not use 4x4 blocks causes virtually no change in bitrate or distortion, as this mode is almost never used. Even when direct mode is restricted to also use at most two motion vectors from each sub-block of the temporally subsequent MB to derive the direct mode vectors, the total change is less than .08% delta Bjontengaard change in bitrate (.004 dB Bjontengaard delta PSNR) for the worst case sequence used in the common conditions.

In our proposed method direct mode is only modified when the temporally subsequent co-located macroblock contains 8x8 sub-partitions with 4 motion vectors (compensating 4x4 blocks). For each of these sub-partitions direct mode motion vectors for two 8x4 sub-blocks are derived from only two motion vectors – those of the 4x4 sub-blocks that are contained within the 8x4 sub-blocks that border either the top or bottom edge of the macroblock. In effect, the motion vectors of the 4x4 sub-blocks that are “interior” (do not border the top or bottom macroblock edge) are replaced with the other 4x4 sub-block that is within the same 8x4 sub-block for the purpose of deriving the motion vectors of the direct mode macroblock. This is demonstrated diagrammatically in Figure 1.

Figure 1: Replacement of “vertically interior” 4x4 sub-block motion vectors from the temporally subsequent co-located (16x16 pixel) macroblock for the purpose of derivation of direct mode motion vectors for 8x4 sub-blocks in the derived direct mode block.

### Note that other derivations are possible. For example, the rate-distortion performance is not significantly affected if replacement is horizontal, rather than verticle (the verticle case is slightly better on the common conditions, though).  However, if the entire bi-directional or direct 8x8 subblock must use only one forward and one backward motion vector, then RD performance is decreased noticeably (by about 1%). For this reason, we have not proposed this solution here, despite the potential for an additional factor of two reduction in the worst case number of memory block accesses.

# Results & Conclusion

Complete results are present in the accompanying document JVT-C115.xls.

In summary, our proposed solution has virtually no impact on the rate-distortion performance of the JM1.9 codec for the common test conditions, while reducing the worst case total number of block accesses by half.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | VideoLocus, Inc 97 Randall DrWaterloo, Ontario, CANADA, N2V1C5 |  |  |
| Contact person | Lowell Winger |  |  |
| Telephone | See contribution header |  |  |
| Fax | See contribution header |  |  |
| Email | See contribution header |  |  |
| Place and date of submission | 3rd JVT Meeting, Fairfax VA USA, May 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT standard (aka H.26L, H.264) |  |
| Title |  | Reduced Decoder Peak Bus Bandwidth |  |
| Contribution number |  | JVT-C115 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | In application |  |
| Inventor(s)/Assignee(s) |  | Lowell Winger, Videolocus |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: