| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C119r1 Filename: JVT-C119r1.doc |
| --- | --- |

| Title: | Complexity Analysis of Improved MB Prediction Modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S.Adachi, S.Kato, F.Moschetti, and M.Etoh 3-5, Hikari-no-oka, Yokosuka, Japan | Tel: Email: | +81-468-40-3519 adachi@spg.yrp.nttdocomo.co.jp |
| Source: | NTT DoCoMo, Inc. |  |  |

_____________________________

1. Introduction

This document describes the results of complexity analysis of the MB prediction modes proposed in VCEG-O22 [1]. In the Geneva meeting, core experiment results on improved macroblock (MB) prediction modes, as a comparison between VCEG-O22 and VCEG-O17 [2] were reported [3] [4] [5] [6]; the coding efficiency improvement of the VCEG-O22 proposal has been proved. However, concerns were raised about the complexity of the VCEG-O22 proposal, and further investigation was requested.

This document reports the results of the complexity analysis at the decoder and encoder side and discusses the adaptation of the proposal as a tool for higher profiles.

2. Complexity analysis

In this complexity analysis comparisons between VCEG-O22 (implemented on TML9.0) and VCEG-O17 (implemented on TML8.5) have been carried out for decoder and encoder.

Tests have been carried out using a MS based PC equipped with an Intel Xeon Processor 1.7 GHz and 1024MB of memory.

As suggested in the Common Test Condition document [7], since we are targeting the adoption of the advanced macroblock prediction modes for higher complexity profiles, we have used a CABAC enabled encoder in our simulations.

2.1  Decoder

VCEG-O22 (implemented on TML9.0) does not add any computational complexity at the decoder when compared to the relevant TML9.0 version (see Fig.1).

The same can be said for VCEG-O17 (implemented on TML8.5) when compared to its relevant TML8.5 version (see Fig.2).

The two decoders also show similar decoding time, we can therefore clearly conclude that at decoder side both VCEG-O22 and VCEG-O17 do not add any computational complexity to the relevant TML core implementation.

Tables with Decoding time, PSNR for quality measure and Bitrate for compression efficiency are shown for all the sequences in the enclosed Excel file, JVT-C119r1_dectime_CABAC_P1.xls.

Fig. 2 Comparison of VCEG-O17 versus the relative TML8.5 implementation (Foreman)

Fig. 1 Comparison of VCEG-O22 versus the relative TML8.5 implementation (Foreman)

2.2 Encoder                                                                       For what concerns the complexity analysis at the encoder we have measured the elapsed execution time for the various sequences. We underline in this report the fundamental role played in complexity by the number of reference frames. We demonstrate here that the wider selection choice of MB modes introduced in VCEG-O22 is a better choice than considering more reference frames. More reference frames constitutes in fact an important concern for the implementation of decoders for handheld devices, particularly when multicast streaming applications are targeted.

- Tables with Encoding time, PSNR for quality measure and Bitrate for compression efficiency are shown for all the sequences in the enclosed Excel file, JVT-C119r1_enctime_CABAC_P.xls and JVT-C119r1_enctime_CABAC_B.xls.

- 2.2.1 Computational complexity:

- Results show that:

- when only one reference frame is considered VCEG-O22 is between 5% and 14% slower than VCEG-O17.

Therefore the overall increase in computational complexity of VCEG-O22 over VCEG-O17 is quite limited.

- 2.2.2 Compression efficiency

- When the same number of reference frames is considered, VCEG-O22 shows higher compression efficiency than VCEG-O17, both when compared to the relevant TML implementation and in absolute terms.

- Note that there are differences of coding efficiency between original TML8.5 and TML9.0. Because of that, some of the results may show that VCEG-O17 surpasses VCEG-O22, but VCEG-O22 always surpasses VCEG-O17 in terms of improvement on the relative TML, as clearly shown in the core experiment report [3].

- 2.2.3 Analysis of results

- VCEG-O22 provides the possibility to select among 19 different coding modes for the macroblocks, while VCEG-O17 considers only 7 different coding modes.

- This has as a consequence that VCEG-O22 is providing higher coding efficiency at the price of a marginal increase in complexity.

- VCEG-O22 using only 3 reference frames shows a coding performance comparable to that of VCEG-O17 using 5 reference frames.

- In this case the computational complexity of the two approaches is the same, but the memory requirement of VCEG-O17 is 5/3 that of VCEG-O22.

- For comparable complexity VCEG-O22 shows comparable and sometimes better coding efficiency at reduced memory requirements.

- Therefore results show that increasing the number of coding modes for the macroblock is a better strategy than increasing the number of reference frames.

- Memory requirements are clearly reduced. Memory size and memory bandwidth indeed play a fundamental role for complexity, particularly when implementations for handheld devices are targeted (particularly for CIF images which require 152 KB per frame).

- 3. Proposal of Profile scalable mode selection

- Higher profiles may be set for streaming/multicasting video transmission.

- In this respect the computational asymmetry of the codec plays an important role when it boils down to the sole complexity of the decoder.

- Therefore reducing the number of frames to be stored in the memory of the decoder appears to be as a fundamental concern together with the absolute need to achieve the highest possible quality at the lowest bitrates.

- In this framework of needs and requirements the computational complexity at encoder side indeed plays a secondary role when compared to the need for higher coding efficiency.

- For those higher profiles, a scalable MB coding mode set can be an approach to achieve coding efficiency improvement without adding any burden to the decoder. MB coding modes of VCEG-O22 can be determined as a super set of the baseline mode sets, and a higher profile can adopt them. Figure 3 shows MB mode set determinations. This approach can improve the coding efficiency without increasing the decoder complexity both in computational and memory aspects, as discussed above.

Fig. 3 MB coding modes for different profiles

- 4. Comments

In general using more coding modes allows to achieve higher compression efficiency at the limited price of marginal increase in the number of operations needed for the motion estimation.

- For the sake of realistic computational complexity analysis, it should be remarked that full search motion estimation approaches are not used in practical implementations; this reduces the overall impact of motion estimation on the total computational burden of the encoder and therefore also of a more detailed mode selection scheme.

- 5. Conclusions

- From the analysis of the data shown in the tables we can draw the following conclusions:

- - Introducing more coding modes defined in VCEG-O22 does not add computational complexity at the decoder

- - Using more coding modes is a better policy than considering more reference frames.

- In fact for reduced memory requirements and the same computational complexity, VCEG-O22 shows a coding performance comparable and sometimes better than VCEG-O17.

- The factor for memory saving is 40%

Therefore:

we propose the adoption of the improved prediction modes described in VCEG-O22 for higher profiles particularly targeting streaming applications.

# References

[1] Satoru Adachi, Shun-ichi Sekiguchi, Sadaatsu Kato, Mitsuru Kobayashi and Minoru Etoh, “Refined Results on the Low-overhead Prediction Modes”, document VCEG-O22, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, TH, 4-6 December 2001.

[2] Heiko Schwarz and Thomas Wiegand, “Tree-structured Macroblock Partition”, document VCEG-O17, ITU-T Video Coding Experts Group (VCEG) Meeting, Pattaya, TH, 4-6 December 2001.

[3] S.Adachi, S.Kato, M.Kobayashi, and M.Etoh, “Core Experiment Results on Improved Macroblock Prediction Modes”, document JVT-B090, JVT meeting, Geneva, Switzerland, January 2002

[4] Heiko Schwarz and Thomas Wiegand, “Core Experiment Results On Improved Macroblock Prediction Modes”, document JVT-B054, JVT meeting, Geneva, Switzerland, January 2002

[5] Shun-ichi Sekiguchi, Yoshihisa Yamada and Kohtaro Asai, “Core Experiment Result on Improved Macroblock Prediction Modes, document JVT-B052, JVT meeting, Geneva, Switzerland, January 2002

[6] Makoto Hagai and Shinya Kadono, “Results of Core Experiment on Improved Macroblock Prediction Modes (VPM2)”, document JVT-B058, JVT meeting, Geneva, Switzerland, January 2002

[7] Gary Sullivan and Gisle Bjontegaard, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan source Material,” document VCEG-N81, ITU- T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-27 September 2001.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | NTT DoCoMo, Inc. |  |  |
| Mailing address | 3-5, Hikari-no-oka, Yokosuka, Kanagawa |  |  |
| Country | Japan |  |  |
| Contact person | Minoru Etoh |  |  |
| Telephone | +81 468 40 3515 |  |  |
| Fax | +81 468 40 3788 |  |  |
| Email | etoh@mml.yrp.nttdocomo.co.jp |  |  |
| Place and date of submission | Fairfax, May.3-May.10, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  |  |  |
| Contribution number |  | JVT-C |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  | S.Adachi, S.Kato, M.Kobayashi, and M.Etoh |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: