| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C122 Filename: JVT-C122.doc |
| --- | --- |

| Title: | JTV transport over RTP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT. |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jan van der Meer |  |  |
| Source: | Philips |  |  |

JVT Transport over RTP

This document suggests some requirements for transport of JVT video over RTP, in particular in relation to transport over MPEG-2 System streams. This document assumes that the MPEG-2 NAL of JVT will be derived from the proposals made by Broadcom to MPEG and JVT. For the Broadcom proposal to the Fairfax meeting see JVT-C046. Amongst others, the following requirements are important:

- Simple transcoding between the MPEG-2 NAL and the RTP NAL

- Easy slice parsing at the RTP NAL

- Simple transport mechanisms over RTP and MPEG-2 with a high level of commonality

- Usage of RTP and MPEG-2 specific mechanisms to convey JVT video information

- Keep transported JVT video stream self-contained

To meet the above requirements, the following approach is proposed;

  - To support easy slice parsing at the RTP NAL, replace slice start codes with a two byte length field that specifies the number of bytes in the immediately following slice, while maintaining all fields that immediately follow the slice startcode.

  - To carry picture information across RTP, remove the picture start code by a two byte length field that specifies the number of bytes in the immediately following picture header and any extensions, preceding the first slice.

  - Remove the sequence start code and define the remaining fields a decoder_configuration_data() that is transported as a MIME format parameter

The transport of JVT video can be based on the internet draft draft-ietf-avt-mpeg4-simple-02.txt, a copy of which is contained in this document for convenience. This draft can be used as follows:

    - Each Access Unit corresponds to the data of one picture

    - The AU-header contains the AU-size field that specifies the length of the contained Aus

    - Any parsing on slice level is JVT defined within the AU.

    - Required MIME format parameters:

      - MPEG-4 stream type, as assigned to JVT-video

      - Profile and level id, as defined by JVT

      - Config, the decoder_configuration_data() as defined by JVT, as described above.

Note that the use of draft-ietf-avt-mpeg4-simple-02.txt may require a new internet draft that extends draft-ietf-avt-mpeg4-simple-02.txt.

- - - - - - -

Internet Engineering Task Force                         J. van der Meer

Internet Draft                                      Philips Electronics

D. Mackie

Cisco Systems Inc.

V. Swaminathan

Sun Microsystems Inc.

D. Singer

Apple Computer

P. Gentric

Philips Electronics

April 2002

Expires October 2002

Document: draft-ietf-avt-mpeg4-simple-02.txt

Transport of MPEG-4 Elementary Streams

Status of this Memo

This document is an Internet-Draft and is in full conformance with

all provisions of Section 10 of RFC2026.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF), its areas, and its working groups. Note that

other groups may also distribute working documents as Internet-

Drafts. Internet-Drafts are draft documents valid for a maximum of

six months and may be updated, replaced, or obsoleted by other

documents at any time. It is inappropriate to use Internet- Drafts

as reference material or to cite them other than as "work in

progress."

The list of current Internet-Drafts can be accessed at

http://www.ietf.org/ietf/1id-abstracts.txt

The list of Internet-Draft Shadow Directories can be accessed at

http://www.ietf.org/shadow.html.

This specification is a product of the Audio/Video Transport working

group within the Internet Engineering Task Force. Comments are

solicited and should be addressed to the working group's mailing

list at avt@ietf.org and/or the authors.

<< Note for the RFC editor: xxxx should be replaced with the RFC

number that will be assigned. >>

Abstract

The MPEG Committee (ISO/IEC JTC1/SC29 WG11) is a working group in

ISO that produced the MPEG-4 standard. MPEG defines tools to

compress content such as audio-visual information into elementary

streams. This specification defines a simple, but generic RTP

payload format for transport of any non-multiplexed MPEG-4

elementary stream.

van der Meer et al.        Expires October 2002                [Page 1]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

Table of Contents

- Introduction . . . . . . . . . . . . . . . . . . . . . . . . .  3

2.  Carriage of MPEG-4 elementary streams over RTP . . . . . . . .  4

2.1.  Introduction . . . . . . . . . . . . . . . . . . . . . . . .  4

2.2.  MPEG Access Units  . . . . . . . . . . . . . . . . . . . . .  4

2.3.  Concatenation of Access Units  . . . . . . . . . . . . . . .  4

2.4.  Fragmentation of Access Units  . . . . . . . . . . . . . . .  5

2.5.  Interleaving . . . . . . . . . . . . . . . . . . . . . . . .  5

2.6.  Time stamp information . . . . . . . . . . . . . . . . . . .  6

2.7.  Carriage of auxiliary information  . . . . . . . . . . . . .  6

2.8.  MIME format parameters and configuring conditional field . .  6

2.9.  Global structure of payload format . . . . . . . . . . . . .  7

2.10. Modes to transport MPEG-4 streams  . . . . . . . . . . . . .  7

2.11. Alignment with RFC 3016  . . . . . . . . . . . . . . . . . .  8

3.  Payload format . . . . . . . . . . . . . . . . . . . . . . . .  8

3.1.  RTP header field usage . . . . . . . . . . . . . . . . . . .  8

3.2.  RTP payload structure  . . . . . . . . . . . . . . . . . . . 10

3.2.1.  The AU Header Section  . . . . . . . . . . . . . . . . . . 10

3.2.1.1.  The AU-header  . . . . . . . . . . . . . . . . . . . . . 10

3.2.2.  The Auxiliary Section  . . . . . . . . . . . . . . . . . . 12

3.2.3.  The Access Unit Data Section . . . . . . . . . . . . . . . 13

3.2.3.1.  Fragmentation  . . . . . . . . . . . . . . . . . . . . . 14

3.2.3.2.  Interleaving . . . . . . . . . . . . . . . . . . . . . . 14

3.2.3.3.  Constraints for interleaving . . . . . . . . . . . . . . 15

3.3.  Usage of this specification  . . . . . . . . . . . . . . . . 15

3.3.1.  General  . . . . . . . . . . . . . . . . . . . . . . . . . 15

3.3.2.  The generic mode . . . . . . . . . . . . . . . . . . . . . 16

3.3.3.  Constant bit rate CELP . . . . . . . . . . . . . . . . . . 16

3.3.4.  Variable bit rate CELP . . . . . . . . . . . . . . . . . . 17

3.3.5.  Low bit rate AAC . . . . . . . . . . . . . . . . . . . . . 18

3.3.6.  High bit rate AAC  . . . . . . . . . . . . . . . . . . . . 18

3.3.7.  Additional modes . . . . . . . . . . . . . . . . . . . . . 19

4.  IANA considerations  . . . . . . . . . . . . . . . . . . . . . 19

4.1.  MIME type registration . . . . . . . . . . . . . . . . . . . 20

4.2.  Concatenation of parameters  . . . . . . . . . . . . . . . . 24

4.3.  Usage of SDP . . . . . . . . . . . . . . . . . . . . . . . . 24

4.3.1.  The a=fmtp keyword . . . . . . . . . . . . . . . . . . . . 24

5.  Security considerations  . . . . . . . . . . . . . . . . . . . 25

6.  Acknowledgements . . . . . . . . . . . . . . . . . . . . . . . 26

7.  References . . . . . . . . . . . . . . . . . . . . . . . . . . 26

8.  Author addresses . . . . . . . . . . . . . . . . . . . . . . . 26

APPENDIX: Usage of this payload format . . . . . . . . . . . . 27

A.1. Examples of delay analysis with interleave  . . . . . . . 27

A.1.1 Group interleave . . . . . . . . . . . . . . . . . . . . 27

A.1.2 Continuous interleave  . . . . . . . . . . . . . . . . . 28

van der Meer et al.        Expires October 2002                [Page 2]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

1. Introduction

The MPEG Committee is Working Group 11 (WG11) in ISO/IEC JTC1 SC29

that specified the MPEG-1, MPEG-2 and, more recently, the MPEG-4

standards [1]. The MPEG-4 standard specifies compression of

audio-visual data into for example an audio or video elementary

stream. In the MPEG-4 standard, these streams take the form of

audiovisual objects that may be arranged into an audio-visual scene

by means of a scene description. Each MPEG-4 elementary stream

consists of a sequence of Access Units; examples of an Access Unit

(AU) are an audio frame and a video picture.

The MPEG-4 system specification is a rather abstract specification

in the sense that no transport format for MPEG-4 elementary streams

is defined. Instead, a conceptual synchronization layer (SL) has

been specified to store transport specific information such as time

stamps and random access point information. When transporting an

MPEG-4 elementary stream, transport information from the SL is

typically mapped to the actual transport layer. Note that the SL is

conceptual and may not exist in practice.

This specification defines a general and configurable payload

structure to transport MPEG-4 elementary streams such as audio,

speech, video and BIFS streams. The RTP payload defined in this

document is simple to implement and reasonably efficient. It allows

for optional interleaving of Access Units (such as audio frames) to

increase error resiliency in packet loss.

Configuration of the payload is provided to accommodate transport

of any MPEG-4 stream at any possible bit rate. However, for a

specific MPEG-4 elementary stream typically only very few

configurations are needed. So as to allow for the design of

simplified, but dedicated receivers, this specification requires

that specific modes are defined for transport of MPEG-4 streams.

This document defines modes for MPEG-4 CELP and AAC streams, as

well as a generic mode that can be used to transport any MPEG-4

stream. In the future new RFCs are expected to specify additional

modes for transport of MPEG-4 streams.

The RTP payload format defined in this document specifies carriage

of system-related information that is often equivalent to the

information that may be contained in the MPEG-4 SL. This

document does not prescribe how to transcode or map information

from the SL to fields defined in the RTP payload format. Such

processing, if any, is left to the discretion of the application.

However, to anticipate the need for transport of any additional

system-related information in future, an auxiliary field can be

configured that may carry any such data.

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED",  "MAY", and "OPTIONAL" in

this document are to be interpreted as described in RFC 2119 [3].

van der Meer et al.        Expires October 2002                [Page 3]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

2. Carriage of MPEG-4 elementary streams over RTP

2.1 Introduction

With this payload format a single MPEG-4 elementary stream can be

transported. Information on the type of MPEG-4 stream carried in

the payload is conveyed by MIME format parameters, for example in

an SDP [6] message or by other means. These MIME format parameters

specify the configuration of the payload. To allow for simplified

and dedicated receivers, a MIME format parameter is available

to signal a specific mode of using this payload. A mode definition

MAY include the type of MPEG-4 elementary stream as well as the

applied configuration, so as to avoid the need in receivers

to parse all MIME format parameters. The applied mode MUST be

signalled.

2.2 MPEG Access Units

For carriage of compressed audio-visual data MPEG defines Access

Units. An MPEG Access Unit (AU) is the smallest data entity to

which timing information is attributed. In case of audio an Access

Unit may represent an audio frame and in case of video a picture.

MPEG Access Units are by definition byte aligned. If for example an

audio frame is not byte aligned, up to 7 zero-padding bits MUST be

inserted at the end of the frame to achieve a byte-aligned Access

Unit. MPEG-4 decoders MUST be able to decode AUs in which such

padding is applied.

Consistent with the MPEG-4 specification, this document requires

that each MPEG-4 part 2 video Access Unit includes all the coded

data of a picture, any video stream headers that may precede the

coded picture data, and any video stream stuffing that may follow

it, up to, but not including the startcode indicating the start of

a new video stream or the next Access Unit.

2.3 Concatenation of Access Units

Frequently it is possible to carry multiple Access Units in one RTP

packet. This is particularly useful for audio; for example, when

AAC is used for encoding of a stereo signal at 64 kbits/sec, AAC

frames contain on average approximately 200 octets. On a LAN with a

1500 octet MTU this would allow on average 7 complete AAC frames to

be carried per AAC packet.

Access Units may have a fixed size in octets, but a variable size

is also possible. To facilitate parsing in case of multiple

concatenated AUs in one RTP packet, the size of each AU is made

known to the receiver. When concatenating in case of a constant AU

size, this size is communicated "out of band" through a MIME format

van der Meer et al.        Expires October 2002                [Page 4]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

parameter. When concatenating in case of variable size AUs, the RTP

payload carries "in band" an AU size field for each contained AU.

In combination with the RTP payload length the size information

allows the RTP payload to be split by the receiver back into the

individual AUs.

To simplify the implementation of RTP receivers, it is required

that when multiple AUs are carried in an RTP packet, each AU MUST

be complete, i.e. the number of AUs in an RTP packet MUST be

integral.

2.4 Fragmentation of Access Units

MPEG allows for very large Access Units. Since most IP networks

have significantly smaller MTU sizes, this payload format allows

for the fragmentation of an Access Unit over multiple RTP packets

so as to avoid IP layer fragmentation. To simplify the

implementation of RTP receivers, an RTP packet SHALL either carry

one or more complete Access Units or a single fragment of one

Access Unit.

2.5 Interleaving

When an RTP packet carries a contiguous sequence of Access Units,

the loss of such a packet can result in a "decoding gap" for the

user. One method to alleviate this problem is to allow for the

Access Units to be interleaved in the RTP packets. For a modest

cost in latency and implementation complexity, significant error

resiliency to packet loss can be achieved.

To support optional interleaving of Access Units, this payload

format allows for index information to be sent for each Access Unit.

The RTP sender is free to choose the interleaving pattern without

propagating this information to the receiver(s). Indeed the sender

could dynamically adjust the interleaving pattern based on the

Access Unit size, error rates, etc. The RTP receiver does not need

to know the interleaving pattern used, it only needs to extract the

index information of the Access Unit and insert the Access Unit

into the appropriate sequence in the rendering queue. An example of

interleaving is given below.

Assume that an RTP packet contains 3 AUs, and that the AUs are

numbered 1, 2, 3, 4, etc. If an interleaving group length of 9 is

chosen, then RTP packet(i) contains the following AU(n):

RTP packet(1):  AU(1),  AU(4),  AU(7)

RTP packet(2):  AU(2),  AU(5),  AU(8)

RTP packet(3):  AU(3),  AU(6),  AU(9)

RTP packet(4):  AU(10), AU(13), AU(16)

RTP packet(5):  AU(11), AU(14), AU(17)

Etc.

van der Meer et al.        Expires October 2002                [Page 5]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

2.6 Time stamp information

The RTP time stamp MUST carry the sampling instance of the first AU

(fragment) in the RTP packet. When multiple AUs are carried within

an RTP packet, the time stamps of subsequent AUs can be calculated

if the frame period of each AU is known. For audio and video this

is possible if the frame rate is constant. However, in some cases

it is not possible to make such calculation, for example for

variable frame rate video and for MPEG-4 BIFS streams carrying

composition information. To support such cases, this payload format

can be configured to carry a time stamp in the RTP payload for each

contained Access Unit. A time stamp MAY be conveyed in the RTP

payload only for non-first AUs in the RTP packet, and SHALL NOT be

conveyed for the first AU (fragment), as the time stamp for the

latter is carried by the RTP time stamp.

MPEG-4 defines two type of time stamps, the composition time stamp

(CTS) and the decoding time stamp (DTS). The CTS represents the

sampling instance of an AU, and hence the CTS is equivalent to the

RTP time stamp. The DTS may be used only in MPEG-4 video streams

that use bi-directional coding, i.e. when pictures are predicted in

both forward and backward direction by using either a reference

picture in the past, or a reference picture in the future. The DTS

cannot be carried in the RTP header. In some cases the DTS can be

derived from the RTP time stamp using frame rate information; this

requires deep parsing in the video stream, which may be considered

objectionable. But if the video frame rate is variable, the required

information may not even be present in the video stream. For both

reasons, the capability has been defined to optionally carry the

DTS in the RTP payload for each contained Access Unit.

Since RTP time stamps may be re-stamped by RTP devices, each time

stamp contained in the RTP payload is coded differentially from the

RTP time stamp, so as to avoid extensive parsing by re-stamping

devices.

2.7 Carriage of auxiliary information.

This payload format defines a specific field to carry auxiliary

data. The auxiliary data field is preceded by a field that specifies

the length of the auxiliary data, so as to facilitate skipping of

the data without parsing it. The coding of the auxiliary data is not

defined in this document, but is left to the discretion of

applications. Receivers that have knowledge of the auxiliary data

MAY decode the auxiliary data, but receivers without knowledge of

such data MUST skip the auxiliary data field.

2.8 MIME format parameters and configuring conditional fields

To support the features described in the previous sections several

fields are defined for carriage in the RTP payload. However, their

use strongly depends on the type of MPEG-4 elementary stream that

van der Meer et al.        Expires October 2002                [Page 6]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

is carried. Sometimes a specific field is needed with a certain

length, while in other cases such field is not needed at all. To be

efficient in either case, the fields to support these features are

configurable by means of MIME format parameters. In general, a MIME

format parameter defines the presence and length of the associated

field. A length of zero indicates absence of the field. As a

consequence, parsing of the payload requires knowledge of MIME

format parameters. The MIME format parameters are conveyed to the

receiver via SDP [6] messages or through other means.

2.9 Global structure of payload format

The RTP payload following the RTP header, contains three byte

aligned data sections, of which the first two MAY be empty. See

figure 1.

+---------+-----------+-----------+---------------+

| RTP     | AU Header | Auxiliary | Access Unit   |

| Header  | Section   | Section   | Data Section  |

+---------+-----------+-----------+---------------+

<----------RTP Packet Payload----------->

Figure 1: Data sections within an RTP packet

The first data section is the AU (Access Unit) Header Section, that

contains one or more AU-headers; however, each AU-header MAY be

empty, in which case the entire AU Header Section is empty. The

second section is the Auxiliary Section, containing auxiliary data;

this section MAY also be configured empty. The third section is the

Access Unit Data Section, containing either a single fragment of

one Access Unit or one or more complete Access Units. The Access

Unit Data Section is never empty.

2.10 Modes to transport MPEG-4 streams

While it is possible to build fully configurable receivers capable

of receiving any MPEG-4 stream, this specification also allows for

the design of simplified, but dedicated receivers, that are capable

for example of receiving only one type of MPEG-4 stream. This

is achieved by requiring that specific modes be defined for using

this specification. Each mode may define constraints for transport

of one or more type of MPEG-4 streams, for instance on the payload

configuration.

The applied mode MUST be signalled. Signalling the mode is

particularly important for receivers that are only capable of

decoding one or more specific modes. Such receivers need to

determine whether the applied mode is supported, so as to avoid

problems with processing of payloads that are beyond the

capabilities of the receiver.

van der Meer et al.        Expires October 2002                [Page 7]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

In this document several modes are defined for transport of MPEG-4

CELP and AAC streams, as well as a generic mode that can be used

for any MPEG-4 stream. In future, new RFCs are expected to specify

additional modes of using this specification. New modes can be

defined as deemed appropriate, typically by specifications that are

hierarchically higher than this payload format. However, each mode

MUST be in full compliance with this specification.

2.11 Alignment with RFC 3016

This payload can be configured to be nearly identical to the

payload format defined in RFC 3016 [5] for the MPEG-4 video

configurations recommended in RFC 3016. Hence, receivers that

comply with RFC 3016 can decode such RTP payload, providing that

additional packets containing video decoder configuration (VO,

VOL, VOSH) are inserted in the stream, as required by RFC 3016.

Conversely, receivers that comply with the specification in this

document SHOULD be able to decode payloads, names and parameters

defined for MPEG-4 video in RFC 3016. In this respect it is

strongly recommended to implement the ability to ignore "in band"

video decoder configuration packets in the RFC 3016 payload.

For interoperability reasons, applications that transport MPEG-4

video part 2 over RTP SHOULD use the payload format and associated

names and parameters defined in RFC 3016 if the functionality

provided by RFC 3016 can meet the requirements of that application.

On the other hand, if applications wish to use a single RTP payload

format for transport of all type of MPEG-4 streams, then the RTP

payload defined in this document provides a suitable solution, also

for transport of MPEG-4 video part 2 streams.

Note that since the "out of band" availability of the video decoder

configuration as a MIME parameter is optional in RFC 3016, for

obvious interoperability reasons with this specification it is

recommended to systematically implement this optional feature.

3 Payload Format

3.1 RTP Header Fields Usage

Payload Type (PT): The assignment of an RTP payload type for this

RTP packet format is outside the scope of this document, and will

not be specified here. It is expected that the RTP profile for a

particular class of applications will assign a payload type for

this encoding, or if that is not done, then a payload type in the

dynamic range shall be chosen.

van der Meer et al.        Expires October 2002                [Page 8]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

Marker (M) bit: The M bit is set to 1 to indicate that the RTP

packet payload includes the end of each Access Unit of which data

is contained in this RTP packet. As the payload either carries one

or more complete Access Units or a single fragment of an Access

Unit, the M bit is always set to 1, except when the packet carries

a single fragment of an Access Unit that is not the last one.

Extension (X) bit: Defined by the RTP profile used.

Sequence Number: The RTP sequence number SHOULD be generated by

the sender with a constant random offset.

Timestamp: Indicates the sampling instance of the first AU

contained in the RTP payload. This sampling instance is equivalent

to the CTS in the MPEG-4 time domain. When using SDP the clock rate

of the RTP time stamp MUST be expressed using the "rtpmap"

attribute. If an MPEG-4 audio stream is transported, the rate SHOULD

be set to the same value as the sampling rate of the audio stream.

If an MPEG-4 video stream is transported, it is RECOMMENDED to set

the rate to 90 kHz.

In all cases, the sender SHALL make sure that RTP time stamps

are identical only if the RTP time stamp refers to fragments of the

same Access Unit.

According to RFC 1889 [2] (section 5.1), RTP time stamps are

recommended to start at a random value for security reasons. This

is not an issue for synchronization of multiple RTP streams.

However, in applications where streams from multiple sources are to

be synchronized (for example one stream from local storage, another

from a RTP streaming server), synchronization may become impossible.

To also enable synchronization in such cases, it may be necessary to

provide the required relationship between time stamps for obtaining

synchronization by out of band means. The format of such information

as well as methods to convey such information are beyond the scope

of this specification.

SSRC: set as described in RFC1889 [2].

CC and CSRC fields are used as described in RFC 1889 [2].

RTCP SHOULD be used as defined in RFC 1889 [2].

van der Meer et al.        Expires October 2002                [Page 9]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

3.2 RTP Payload Structure

3.2.1 The AU Header Section

When present, the AU Header Section consists of the AU-header-length

field, followed by a number of AU-headers. See figure 2.

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+- .. -+-+-+-+-+-+-+-+-+-+

|AU-headers-length|AU-header|AU-header|      |AU-header|padding|

|                 |   (1)   |   (2)   |      |   (n)   | bits  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+- .. -+-+-+-+-+-+-+-+-+-+

Figure 2: The AU Header Section

The AU-headers are configured using MIME format parameters and MAY

be empty. If the AU-header is configured empty, the

AU-headers-length field SHALL not be present and consequently the

AU Header Section is empty. If the AU-header is not configured

empty, then the AU-headers-length is a two octet field that

specifies the length in bits of the immediately following

AU-headers, excluding the padding bits.

Each AU-header is associated with a single Access Unit (fragment)

contained in the Access Unit Data Section in the same RTP packet.

For each contained Access Unit (fragment) there is exactly one

AU-header. Within the AU Header Section, the AU-headers are

bit-wise concatenated in the order in which the Access Units are

contained in the Access Unit Data Section. Hence, the n-th

AU-header refers to the n-th AU (fragment). If the concatenated

AU-headers consume a non-integer number of octets, up to 7

zero-padding bits MUST be inserted at the end in order to achieve

byte-alignment of the AU Header Section.

3.2.1.1 The AU-header

The AU-header contains the fields given in figure 3. The length in

bits of the above fields with the exception of the CTS-flag and

the DTS-flag fields is defined by MIME format parameters; see

section 4.1. If a MIME format parameter has the default value of

zero, then the associated field is not present.

van der Meer et al.        Expires October 2002               [Page 10]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

+---------------------------------------+

|     AU-size                           |

+---------------------------------------+

|     AU-Index / AU-Index-delta         |

+---------------------------------------+

|     CTS-flag                          |

+---------------------------------------+

|     CTS-delta                         |

+---------------------------------------+

|     DTS-flag                          |

+---------------------------------------+

|     DTS-delta                         |

+---------------------------------------+

Figure 3: The fields in the AU-header. If used, the AU-Index field

only occurs in the first AU-header within an AU Header

Section; in any other AU-header the AU-Index-delta field

occurs instead.

AU-size: indicates the size in octets of the associated Access Unit

in the Access Unit Data Section in the same RTP packet. When

the AU-size is associated with an AU fragment, the AU size

indicates the size of the entire AU and not the size of the

fragment. This can be exploited to determine whether a packet

contains an entire AU or a fragment, which is particularly

useful after losing a packet carrying the last fragment of an

AU.

AU-Index: indicates the serial number of the associated Access Unit

(fragment). For each (in decoding order) consecutive AU or AU

fragment, the serial number is incremented with 1. When

present, the AU-Index field occurs in the first AU-header in

the AU Header Section, but MUST NOT occur in any subsequent

(non-first) AU-header in that Section. To encode the serial

number in any such non-first AU-header, the AU-Index-delta

field is used. If each AU-Index field is coded with the value

0, the serial number of the AU (fragment) is not specified,

and in that case receivers MAY ignore the AU-Index field.

AU-Index-delta: The AU-Index-delta field is an unsigned integer

that specifies the serial number of the associated AU as the

difference with respect to the serial number of the previous

Access Unit. Hence, for the n-th (n>1) AU the serial number

is found from:

AU-Index(n) = AU-Index(n-1) + AU-Index-delta(n) + 1

If the AU-Index field is present in the first AU-header in

the AU Header Section, then the AU-Index-delta field MUST be

present in any subsequent (non-first) AU-header. When the

AU-Index-delta is coded with the value 0, it indicates that

the Access Units are consecutive in decoding order. An

AU-Index-delta value larger than 0 signals that interleaving

is applied.

van der Meer et al.        Expires October 2002               [Page 11]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

CTS-flag: Indicates whether the CTS-delta field is present.

A value of 1 indicates that the field is present, a value

of 0 that it is not present.

The CTS-flag field MUST be present in each AU-header if the

length of the CTS-delta field is signalled to be larger than

zero. In that case, the CTS-flag field MUST have the value 0

in the first AU-header and MAY have the value 1 in all

non-first AU-headers. The CTS-flag field SHOULD be 0 for

any non-first fragment of an Access Unit.

CTS-delta: Encodes the CTS by specifying the value of CTS as a 2's

complement offset (delta) from the time stamp in the RTP

header of this RTP packet. The CTS MUST use the same clock

rate as the time stamp in the RTP header.

DTS-flag: Indicates whether the DTS-delta field is present. A value

of 1 indicates that DTS-delta is present, a value of 0 that

it is not present.

The DTS-flag field MUST be present in each AU-header if the

length of the DTS-delta field is signalled to be larger than

zero. The DTS-flag field SHOULD be 0 for any non-first

fragment of an Access Unit.

DTS-delta: specifies the value of the DTS as a 2's complement

offset (delta) from the CTS. The DTS MUST use the

same clock rate as the time stamp in the RTP header.

If present, the fields MUST occur in the mutual order given in

figure 3. In the general case a receiver can only discover the size

of an AU-header by parsing it since the presence of the CTS-delta

and DTS-delta fields is signalled by the value of the CTS-flag and

DTS-flag, respectively.

3.2.2 The Auxiliary Section

The Auxiliary Section consists of the auxiliary-data-size field

followed by the auxiliary-data field. Receivers MAY (but are not

required to) parse the auxiliary-data field; to facilitate skipping

of the auxiliary-data field by receivers, the auxiliary-data-size

field indicates the length in bits of the auxiliary-data. If the

concatenation of the auxiliary-data-size and the auxiliary-data

fields consume a non-integer number of octets, up to 7 zero padding

bits MUST be inserted immediately after the auxiliary data in order

to achieve byte-alignment. See figure 4.

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+- .. -+-+-+-+-+-+-+-+-+

| auxiliary-data-size   | auxiliary-data       |padding bits |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+- .. -+-+-+-+-+-+-+-+-+

Figure 4: The fields in the Auxiliary Section

van der Meer et al.        Expires October 2002               [Page 12]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

The length in bits of the auxiliary-data-size field is configurable

by a MIME format parameter; see section 4.1. The default length of

zero indicates that the entire Auxiliary Section is absent.

auxiliary-data-size: specifies the length in bits of the immediately

following auxiliary-data field;

auxiliary-data: the auxiliary-data field contains data of a format

not defined by this specification.

3.2.3 The Access Unit Data Section

The Access Unit Data Section contains an integer number of complete

Access Units or a single fragment of one AU. The Access Unit Data

Section is never empty. If data of more than one Access Unit is

present, then the AUs are concatenated into a contiguous string

of octets. See figure 5. The AUs inside the Access Unit Data

Section MUST be in decoding order.

The size and number of Access Units SHOULD be adjusted such that

the resulting RTP packet is not larger than the path MTU. To handle

larger packets, this payload format relies on lower layers for

fragmentation, which may not be desirable.

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|AU(1)                                                              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-                                |

|                                                                   |

|     +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|               |AU(2)                                              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+                                   |

|                                                                   |

|                            -+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               | AU(n)                             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|               |

|-+-+-+-+-+-+-+-+

Figure 5: Access Unit Data Section; each AU is byte aligned.

When multiple Access Units are carried, the size of each AU MUST be

made available to the receiver. If the AU size is variable then the

size of each AU MUST be indicated in the AU-size field of the

corresponding AU-header. However, if the AU size is constant for a

stream, this mechanism SHOULD NOT be used, but instead the fixed

size SHOULD be signalled by the MIME format parameter

"ConstantSize", see section 4.1.

The absence of both AU-size in the AU-header and the ConstantSize

MIME format parameter indicates carriage of a single AU (fragment),

i.e. that a single Access Unit (fragment) is transported in each

RTP packet for that stream.

van der Meer et al.        Expires October 2002               [Page 13]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

3.2.3.1 Fragmentation

A packet SHALL carry either one or more Access Units, or a single

fragment of an Access Unit.  Fragments of the same Access Unit have

the same time stamp but different RTP sequence numbers. The marker

bit in the RTP header is 1 on the last fragment of an Access Unit,

and 0 on all other fragments.

3.2.3.2 Interleaving

Access Units MAY be interleaved. Senders MAY perform interleaving.

Receivers MUST support interleaving. When interleaving of Access

Units is used it SHALL be implemented using the AU-Index and

AU-Index-delta fields in the AU-header.

Based on the RTP sequence number, the RTP time stamp, the AU-Index

and the AU-Index-delta, a receiver can unambiguously reconstruct

the original order even in case of out-of-order packets, packet

loss or duplication. Note that for this purpose the AU-Index is

redundant when the RTP time stamp and the AU-Index-delta values are

sufficient for placing the AUs correctly in time. In such cases

receivers MAY ignore the AU-Index value and senders MAY code the

AU-Index field with the value 0, but only if they code each AU-Index

field with that value.

When interleaving is applied, a de-interleave buffer is needed in

receivers to put the Access Units in their correct logical

consecutive decoding order. This requires the computation of the

time stamp for each Access Unit. In case of a fixed time duration

per Access Unit, the time stamp of the i-th access unit in an RTP

packet with RTP time stamp T is calculated as follows:

Timestamp[0] = T

Timestamp[i, i > 0] = T +(Sum(for k=1 to i of (AU-Index-delta[k]

+ 1))) * access-unit-duration

When AU-Index-delta is always 0, this reduces to T + i * (access-

unit-duration). This is the non-interleaved case, where the frames

are consecutive in decoding order. Note that the AU-Index field

(present for the first Access Unit) is not needed in this

calculation. Hence in cases where the Access-unit-duration has a

fixed and known value, the AU-Index does not need to provide index

information and can be coded with the value 0. See also the

semantics of the AU-Index field in 3.2.1.1.

When an RTP packet arrives (after any reordering has been done),

receivers may 'flush' all Access Units from the interleave buffer

which have a time stamp strictly less than the time stamp of the

arriving packet. Similarly the first Access Unit of every arriving

packet can always be flushed (as no following packet can provide

van der Meer et al.        Expires October 2002               [Page 14]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

an earlier Access Unit), and any Access Units which are consecutive

with it which have already been received. Access Units should also

be flushed in time to be played; this can be important if there is

loss before end-of-stream, before a silence interval, or before a

large drop-out.

3.2.3.3 Constraints for interleaving

The size of the packets should be suitably chosen to be appropriate

to both the path MTU and the duration and capacity of the receiver's

de-interleave buffer. The maximum packet size for a session should

be chosen not to exceed the path MTU.

In order to control receiver latency and mitigate the effects of

loss, there are profile-based limits on the size of the packet.

This is expressed as a duration: it is calculated from the duration

of the Access Units contained within a packet. Note that this

duration is NOT the difference between the time stamps of the first

and last Access Unit in a packet.

No matter what interleaving scheme is used, the scheme must be

analyzed to calculate the minimum number of frames a receiver has

to buffer in order to de-interleave.

Three profiles are defined to constrain the latency when interlea-

ving. The applied profile is signalled by the MIME format parameter

"Profile", indicating the decimal number of the profile. The maximum

de-interleave buffer required at the receiver can be determined if

the maximum packet duration is known. The maximum packet duration

in milliseconds for the three profiles, shall not exceed:

Profile 0 --  200 milliseconds

Profile 1 --  500 milliseconds

Profile 2 –- 1500 milliseconds

When interleaving is applied, the applied RTP transport profile

MUST be signalled by the MIME format parameter "Profile"; see

section 4.1.

Note that for low bit-rate material, this duration limit may make

packets shorter than the MTU size.

3.3 Usage of this specification

3.3.1 General

Usage of this specification requires definition of a mode. A mode

defines how to use this specification, as deemed appropriate.

Senders MUST signal the applied mode via the MIME format parameter

"Mode". This specification defines a generic mode that can be used

for any MPEG-4 stream, as well as specific modes for transport of

MPEG-4 CELP and MPEG-4 AAC streams.

van der Meer et al.        Expires October 2002               [Page 15]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

In any mode compliant to this specification the same requirements

apply for the rtpmap attributes. The general form of an rtpmap

attribute is:

a=rtpmap:<payload type> <encoding name>/<clock rate>[/<encoding

parameters>]

For audio streams, <encoding parameters> specifies the number of

audio channels. This parameter may be omitted if the number of

channels is one, provided no additional parameters are needed.

In any mode, the following attributes are REQUIRED:

a) The encoding name

b) The RTP clock rate MUST be expressed.

c) The number of audio channels MUST be specified, for example as

2 for  stereo material (see RFC 2327) and MAY be specified as 1

for mono material; 1 is the default.

3.3.2 The generic mode

The generic mode can be used for any MPEG-4 stream. In this mode

no mode-specific constraints are applied; hence, the generic mode

exploits the full flexibility of this specification. The generic

mode is signalled by mode=generic.

An example is given below for transport of a BIFS stream. In this

example carriage of multiple BIFS Access Units is allowed in one

RTP packet. The AU-header section contains the AU-size field, the

CTS-flag and, if the CTS flag is set to 1, the CTS-delta field.

The number of bits of the AU-size and the CTS-delta fields is 15

and 16 respectively, which results in an AU-header of two or four

octets per BIFS AU. The RTP time stamp uses a 1 kHz clock.

In detail:

m=video 49230 RTP/AVP 96

a=rtpmap:96 mpeg4-generic/1000

a=fmtp:96 streamtype=3; profile-level-id=257; mode=generic;

ObjectType=2; config=BIFSConfiguration(); SizeLength=15;

CTSDeltaLength=16

3.3.3 Constant bit-rate CELP

This mode is signalled by mode=CELP-cbr. In this mode one or more

fixed size CELP frames can be transported in one RTP packet; there

is no support for interleaving. The RTP payload consist of one or

more concatenated CELP frames, each of the same size. Both the AU

Header Section and the Auxiliary Section are empty.

The MIME format parameter ConstantSize MUST be provided to specify

the length of each CELP frame.

van der Meer et al.        Expires October 2002               [Page 16]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

For example:

m=audio 49230 RTP/AVP 96

a=rtpmap:96 mpeg4-generic/44100/2

a=fmtp:96 streamtype=5; profile-level-id=15; mode=CELP-cbr; config=

AudioSpecificConfig(); ConstantSize=xxx;

The AudioSpecificConfig() specifies that the audio stream type is

CELP.

3.3.4 Variable bit-rate CELP

This mode is signalled by mode=CELP-vbr. With this mode one or

more variable size CELP frames can be transported in one RTP packet

with optional interleaving. As the largest possible frame size in

this mode is greater than the maximum CELP frame size, there is no

support for fragmentation of CELP frames.

In this mode the RTP payload consists of the AU Header Section,

followed by one or more concatenated CELP frames. The Auxiliary

Section is empty. For each CELP frame contained in the payload

there is a one octet AU-header in the AU Header Section to

provide:

(a) the size of each CELP frame in the payload and

(b) index information for computing the sequence (and hence timing)

of each CELP frame.

Transport of CELP frames requires that the AU-size field is coded

with 6 bits. In this mode therefore 6 bits are allocated to the

AU-size field, and 2 bits to the AU-Index(-delta) field. Each

AU-Index field MUST be coded with the value 0. In the AU Header

Section, the concatenated AU-headers are preceded by the 16-bit

AU-headers-length field, as specified in 3.2.1.

In addition to the required MIME format parameters, the following

parameters MUST be present: SizeLength, IndexLength, and

IndexDeltaLength.

When interleaving is applied (AU-Index-delta coded with a value

larger than 0), the parameter Profile MUST also be present.

For example:

m=audio 49230 RTP/AVP 96

a=rtpmap:96 mpeg4-generic/44100/2

a=fmtp:96 streamtype=5; profile-level-id=15; mode=CELP-vbr; config=

AudioSpecificConfig(); SizeLength=6; IndexLength=2;

IndexDeltaLength=2; Profile=1

The AudioSpecificConfig() specifies that the audio stream type is

CELP.

van der Meer et al.        Expires October 2002               [Page 17]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

3.3.5 Low bit-rate AAC

This mode is signalled by mode=AAC-lbr. This mode supports transport

of one or more variable size AAC frames with optional support for

interleaving and fragmenting. The maximum size of an AAC frame

(fragment) in this mode is 63 octets.

The payload configuration in this mode is the same as in the

variable bit-rate CELP mode as defined in 3.3.4. The RTP payload

consists of the AU Header Section, followed by concatenated AAC

frames. The Auxiliary Section is empty. For each AAC frame contained

in the payload the one octet AU-header provides:

(a) the size of each AAC frame in the payload and

(b) index information for computing the sequence (and hence timing)

of each AAC frame.

In the AU-header, the AU-size is coded with 6 bits and the

AU-Index(-delta) with 2 bits; the AU-Index field MUST have the

value 0 in each AU-header.

In the AU-header Section, the concatenated AU-headers are preceded

by the 16-bit AU-headers-length field, as specified in 3.2.1.

In addition to the required MIME format parameters, the following

parameters MUST be present: SizeLength, IndexLength, and

IndexDeltaLength.

When interleaving is applied (AU-Index-delta coded with a value

larger than 0), also the parameter Profile MUST be present.

For example:

m=audio 49230 RTP/AVP 96

a=rtpmap:96 mpeg4-generic/44100/2

a=fmtp:96 streamtype=5; profile-level-id=15; mode=AAC-lbr; config=

AudioSpecificConfig(); SizeLength=6; IndexLength=2;

IndexDeltaLength=2; Profile=1

The AudioSpecificConfig() specifies that the audio stream type is

AAC.

3.3.6 High bit-rate AAC

This mode is signalled by mode=AAC-hbr. This mode supports transport

of one or more large variable size AAC frames in one RTP packet with

optional support for interleaving and fragmenting. The maximum size

of an AAC frame (fragment) in this mode is 8191 octets.

In this mode the RTP payload consists of the AU Header Section,

followed by one or more concatenated AAC frames. The Auxiliary

Section is empty. For each AAC frame contained in the payload there

is an AU-header in the AU Header Section to provide:

(a) the size of each AAC frame in the payload and

(b) index information for computing the sequence (and hence timing)

of each AAC frame.

van der Meer et al.        Expires October 2002               [Page 18]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

To code the maximum size of an AAC frame requires 13 bits. Therefore

in this configuration 13 bits are allocated to the AU-size, and

3 bits to the AU-Index(-delta) field. Thus each AU-header has a size

of 2 octets. Each AU-Index field MUST be coded with the value 0. In

the AU Header Section, the concatenated AU-headers are preceded by

the 16-bit AU-headers-length field, as specified in 3.2.1.

In addition to the required MIME format parameters, the following

parameters MUST be present: SizeLength, IndexLength, and

IndexDeltaLength.

When interleaving is applied (AU-Index-delta coded with a value

larger than 0), also the parameter Profile MUST be present.

For example:

m=audio 49230 RTP/AVP 96

a=rtpmap:96 mpeg4-generic/44100/2

a=fmtp:96 streamtype=5; profile-level-id=15; mode=AAC-hbr;

config=AudioSpecificConfig(); SizeLength=13; IndexLength=3;

IndexDeltaLength=3; Profile=1

The AudioSpecificConfig() specifies that the audio stream type is

AAC.

3.3.7 Additional modes

This specification only defines the modes specified in sections

3.3.2 up to 3.3.6. Additional modes are expected to be defined in

future RFCs. Each additional mode MUST be in full compliance with

this specification.

When defining a new mode care MUST be taken that an implementation

of all features of this specification can decode the payload format

corresponding to this new mode. For this reason a mode MUST NOT

specify new default values for MIME parameters. In particular, MIME

parameters that configure the RTP payload MUST be present (unless

they have the default value), even if its presence is redundant in

case the mode assigns a fixed value to a parameter. A mode may

define additionally that some MIME parameters are required instead

of optional, that some MIME parameters have fixed values (or

ranges), and that there are rules restricting the usage.

4. IANA considerations

This section describes the MIME types and names associated with

this payload format. Section 4.1 registers the MIME types, as per

RFC 2048.

This format may require additional information about the mapping to

be made available to the receiver. This is done using parameters

also described in the next section.

van der Meer et al.        Expires October 2002               [Page 19]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

4.1 MIME type registration

MIME media type name: "video" or "audio" or "application"

"video" MUST be used for MPEG-4 Visual streams (ISO/IEC 14496-2)

or MPEG-4 Systems streams (ISO/IEC 14496-1) that convey information

needed for an audio/visual presentation.

"audio" MUST be used for MPEG-4 Audio streams (ISO/IEC 14496-3)

or MPEG-4 Systems streams that convey information needed for an

audio only presentation.

"application" MUST be used for MPEG-4 Systems streams (ISO/IEC

14496-1) that serve purposes other than audio/visual presentation,

e.g. in some cases when MPEG-J streams are transmitted.

Depending on the required payload configuration, MIME format

parameters need to be available to the receiver. This is done using

the parameters described in the next section. There are required

and optional parameters.

Optional parameters are of two types: general parameters and

configuration parameters. The configuration parameters are used to

configure the fields in the AU Header section and in the auxiliary

section. The absence of any configuration parameter is equivalent to

the associated field set to its default value, which is always zero.

The absence of all configuration parameters resolves into a default

"basic" configuration with an empty AU-header section and an empty

auxiliary section in each RTP packet.

MIME subtype name: mpeg4-generic

Required parameters:

MIME format parameters are not case dependent; however for clarity

both upper and lower case are used in the names of the parameters

described in this specification.

StreamType:

The integer value that indicates the type of MPEG-4 stream that

is carried; its coding corresponds to the values of the

streamType as defined for the DecoderConfigDescriptor in

ISO/IEC 14496-1. The value 6, indicating an MPEG-7 stream, MUST

NOT be used, as this payload format is not intended for transport

of MPEG-7 streams.

Profile-level-id:

A decimal representation of the MPEG-4 Profile Level indication.

This parameter MUST be used in the capability exchange or

session set-up procedure to indicate the MPEG-4 Profile and Level

combination of which the relevant MPEG-4 media codec is capable

of.

van der Meer et al.        Expires October 2002               [Page 20]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

For MPEG-4 Audio streams, this parameter is the decimal value

from Table 5 (audioProfileLevelIndication Values) in ISO/IEC

14496-1, indicating which MPEG-4 Audio tool subsets are

required to decode the audio stream.

For MPEG-4 Visual streams, this parameter is the decimal value

from Table G-1 (FLC table for profile and level indication of

ISO/IEC 14496-2), indicating which MPEG-4 Visual tool subsets

are required to decode the visual stream.

For BIFS streams, this parameter is the decimal value that is

obtained from (SPLI + 256*GPLI), where:

SPLI is the decimal value from Table 4 in ISO/IEC 14496-1 with

the applied sceneProfileLevelIndication;

GPLI is the decimal value from Table 7 in ISO/IEC 14496-1 with

the applied graphicsProfileLevelIndication.

For MPEG-J streams, this parameter is the decimal value from

table 13 (MPEGJProfileLevelIndication) in ISO/IEC 14496-1,

indicating the profile and level of the MPERG-J stream.

For OD streams, this parameter is the decimal value from table 3

(ODProfileLevelIndication) in ISO/IEC 14496-1, indicating the

profile and level of the OD stream.

For IPMP streams, this parameter has either the decimal value 0,

indicating an unspecified profile and level, or a value larger

than zero, indicating an MPEG-4 IPMP profile and level as

defined in a future MPEG-4 specification.

For Clock Reference streams and Object Content Info streams, this

parameter has the decimal value zero, indicating that profile

and level information is conveyed through the OD framework.

Config:

A hexadecimal representation of an octet string that expresses

the media payload configuration. Configuration data is mapped

onto the octet string in an MSB-first basis. The first bit of

the configuration data SHALL be located at the MSB of the first

octet. In the last octet, if necessary to achieve byte alignment,

up to 7 zero-valued padding bits shall follow the configuration

data.

For MPEG-4 Audio streams, config is the audio object type

specific decoder configuration data AudioSpecificConfig() as

defined in ISO/IEC 14496-3.

For MPEG-4 Visual streams, config is the MPEG-4 Visual

configuration information as defined in subclause 6.2.1 Start

codes of ISO/IEC 14496-2. The configuration information

indicated by this parameter SHALL be the same as the

configuration information in the corresponding MPEG-4 Visual

stream, except for first-half-vbv-occupancy and

latter-half-vbv-occupancy, if it exists, which may vary in

the repeated configuration information inside an MPEG-4

Visual stream (See 6.2.1 Start codes of ISO/IEC 14496-2).

For BIFS streams, this the BIFSConfig() information as defined

in ISO/IEC 14496-1. For version 1, BIFSConfig is defined in

section 9.3.2.4, and for version 2 in section 9.3.5. The MIME

format parameter ObjectType signals the version of BIFSConfig.

van der Meer et al.        Expires October 2002               [Page 21]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

For IPMP streams, this is either the decimal value 0, indicating

the absence of any decoder configuration information, or the

decimal value 1, followed by IPMPConfiguration() as defined

in a future MPEG-4 IPMP specification.

For Object Content Info (OCI) streams, this is the

OCIDecoderConfiguration() information of the OCI stream, as

defined in section 8.4.2.4 in ISO/IEC 14496-1.

For OD streams, Clock Reference streams and MPEG-J streams, this

is the decimal value 0, indicating that no information on the

decoder configuration is required.

Mode:

The mode in which this specification is used. The following modes

can be signalled:

mode=generic,

mode=CELP-cbr,

mode=CELP-vbr,

mode=AAC-lbr and

mode=AAC-hbr.

Other modes are expected to be defined in future RFCs. See also

section 3.3.7.

Optional general parameters:

ObjectType:

The decimal value from Table 8 in ISO/IEC 14496-1, indicating

the value of the objectTypeIndication of the transported stream.

For BIFS streams this parameter MUST be present to signal the

type of BIFSConfiguration(). The ObjectType SHALL not signal a

non-MPEG-4 stream.

ConstantSize:

The constant size in octets of each Access Unit for this stream.

Simultaneous presence of ConstantSize and the SizeLength

parameters is not permitted.

Profile:

The decimal representation of the applied profile to constrain

the latency when interleaving; see section 3.2.3.3. Absence of

this parameter signals that the profile is not specified.

Optional configuration parameters:

SizeLength:

The number of bits on which the AU-size field is encoded in the

AU-header. Simultaneous presence of SizeLength and the

ConstantSize parameter is not permitted.

van der Meer et al.        Expires October 2002               [Page 22]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

IndexLength:

The number of bits on which the AU-Index is encoded in the first

AU-header. The default value of zero indicates the absence of

the AU-Index and AU-Index-delta fields in each AU-header.

IndexDeltaLength:

The number of bits on which the AU-Index-delta field is encoded

in any non-first AU-header.

CTSDeltaLength:

The number of bits on which the CTS-delta field is encoded in

the AU-header.

DTSDeltaLength:

The number of bits on which the DTS-delta field is encoded in

the AU-header.

AuxiliaryDataSizeLength:

The number of bits that is used to encode the auxiliary-data-size

field.

Applications MAY use more parameters, in addition to those defined

above. Receivers MUST tolerate the presence of such additional

parameters, but these parameters SHALL not impact the decoding of

receivers that comply to this specification.

Encoding considerations:

System bitstreams MUST be generated according to MPEG-4 Systems

specifications (ISO/IEC 14496-1). Video bitstreams MUST be generated

according to MPEG-4 Visual specifications (ISO/IEC 14496-2). Audio

bitstreams MUST be generated according to MPEG-4 Visual

specifications (ISO/IEC 14496-3). The RTP packets MUST be packetized

according to the RTP payload format defined in RFC xxxx.

Security considerations:

As defined in section 5 of RFC xxxx.

Interoperability considerations:

MPEG-4 provides a large and rich set of tools for the coding of

visual objects.  For effective implementation of the standard,

subsets of the MPEG-4 tool sets have been provided for use in

specific applications. These subsets, called 'Profiles', limit the

size of the tool set a decoder is required to implement. In order to

restrict computational complexity, one or more 'Levels' are set for

each Profile. A Profile@Level combination allows:

. a codec builder to implement only the subset of the standard he

needs, while maintaining interworking with other MPEG-4 devices

that implement the same combination, and

. checking whether MPEG-4 devices comply with the standard

('conformance testing').

van der Meer et al.        Expires October 2002               [Page 23]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

A stream SHALL be compliant with the MPEG-4 Profile@Level specified

by the parameter "profile-level-id". Interoperability between a

sender and a receiver is achieved by specifying the parameter

"profile-level-id" in MIME content. In the capability exchange /

announcement procedure this parameter may mutually be set to the

same value.

Published specification:

The specifications for MPEG-4 streams are presented in ISO/IEC

14469-1, 14469-2, and 14469-3. The RTP payload format is described

in RFC xxxx.

Applications which use this media type:

Multimedia streaming and conferencing tools, Internet messaging and

Email applications.

Additional information: none

Magic number(s): none

File extension(s):

None. A file format with the extension .mp4 has been defined for

MPEG-4 content but is not directly correlated with this MIME type

for which the sole purpose is RTP transport.

Macintosh File Type Code(s): none

Person & email address to contact for further information:

Authors of RFC xxxx, IETF Audio/Video Transport working group.

Intended usage: COMMON

Author/Change controller:

Authors of RFC xxxx, IETF Audio/Video Transport working group.

4.2 Concatenation of parameters

Multiple parameters SHOULD be expressed as a MIME media type string,

in the form of a semicolon-separated list of parameter=value pairs

(for parameter usage examples see sections 3.3.2 up to 3.3.6).

4.3 Usage of SDP

4.3.1 The a=fmtp keyword

It is assumed that one typical way to transport the above-described

parameters associated with this payload format is via a SDP message

[6] for example transported to the client in reply to a RTSP

DESCRIBE or via SAP. In that case the (a=fmtp) keyword MUST be used

as described in RFC 2327 [7], section 6, the syntax being then:

a=fmtp:<format> <parameter name>=<value>[; <parameter name>=<value>]

van der Meer et al.        Expires October 2002               [Page 24]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

5. Security Considerations

RTP packets using the payload format defined in this specification

are subject to the security considerations discussed in the RTP

specification [5]. This implies that confidentiality of the media

streams is achieved by encryption. Because the data compression used

with this payload format is applied end-to-end, encryption may be

performed on the compressed data so there is no conflict between the

two operations. The packet processing complexity of this payload

type (i.e. excluding media data processing) does not exhibit any

significant non-uniformity in the receiver side to cause a denial-

of-service threat.

However, it is possible to inject non-compliant MPEG streams (Audio,

Video, and Systems) to overload the receiver/decoder's buffers,

which might compromise the functionality of the receiver or even

crash it. This is especially true for end-to-end systems like MPEG

where the buffer models are precisely defined.

MPEG-4 Systems supports stream types including commands that are

executed on the terminal like OD commands, BIFS commands, etc. and

programmatic content like MPEG-J (Java(TM) Byte Code) and

ECMAScript. It is possible to use one or more of the above in a

manner non-compliant to MPEG to crash or temporarily make the

receiver unavailable.

Senders SHOULD ensure that packet loss does not cause severe

problems in application execution when the packet carries OD

commands, BIFS commands, or programmatic content such as MPEG-J and

ECMAScript. When such measures cannot be taken, instead of this

payload format applications SHOULD use more reliable means to

transport the information.

Authentication mechanisms can be used to validate the sender and

the data to prevent security problems due to non-compliant malignant

MPEG-4 streams.

In ISO/IEC 14469-1 a security model is defined for MPEG-4 Systems

streams carrying MPEG-J access units which comprise Java(TM) classes

and objects. MPEG-J defines a set of Java APIs and a secure

execution model. MPEG-J content can call this set of APIs and

Java(TM) methods from a set of Java packages supported in the

receiver within the defined security model. According to this

security model, downloaded byte code is forbidden to load libraries,

define native methods, start programs, read or write files, or read

system properties.

Receivers can implement intelligent filters to validate the buffer

requirements or parametric (OD, BIFS, etc.) or programmatic (MPEG-J,

ECMAScript) commands in the streams. However, this can increase the

complexity significantly.

van der Meer et al.        Expires October 2002               [Page 25]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

6. Acknowledgements

This document evolved through several revisions thanks to

contributions by people from the ISMA forum, from the IETF AVT

Working Group and from the 4-on-IP ad-hoc group within MPEG. The

authors wish to thank all involved people, and in particular Colin

Perkins, Stephan Wenger and Dorairaj V for their valuable comments

and support.

7. References

[1] ISO/IEC International Standard 14496 (MPEG-4); "Information

technology - Coding of audio-visual objects", January 2000

[2] Schulzrinne, Casner, Frederick, Jacobson RTP: A Transport

Protocol for Real Time Applications  RFC 1889, Internet Engineering

Task Force, January 1996.

[3] S. Bradner, Key words for use in RFCs to Indicate Requirement

Levels, RFC 2119, March 1997.

[4] D. Hoffman, G. Fernando, V. Goyal, M. Civanlar, RTP payload

format for MPEG1/MPEG2 Video, RFC 2250, January 1998.

[5] Y. Kikuchi, T. Nomura, S. Fukunaga, Y. Matsui, H. Kimata, RTP

payload format for MPEG-4 Audio/Visual streams, RFC 3016.

[6] Handley, Jacobson, SDP: Session Description Protocol, RFC 2327,

Internet Engineering Task Force, April 1998.

7. Author Adresses

Jan van der Meer

Philips Digital Networks

Cederlaan 4

5600 JB Eindhoven

Netherlands

Email : jan.vandermeer@philips.com

David Mackie

Cisco Systems Inc.

170 West Tasman Dr.

San Jose, CA 95034

Email: dmackie@cisco.com

Viswanathan Swaminathan

Sun Microsystems Inc.

901 San Antonio Road, M/S UMPK15-214

Palo Alto, CA 94303

Email: viswanathan.swaminathan@sun.com

van der Meer et al.        Expires October 2002               [Page 26]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

David Singer

Apple Computer, Inc.

One Infinite Loop, MS:302-3MT

Cupertino  CA 95014

Email: singer@apple.com

Philippe Gentric

Philips Digital Networks, MP4Net

51 rue Carnot

92156 Suresnes

France

e-mail: philippe.gentric@philips.com

Full Copyright Statement

"Copyright (C) The Internet Society (date). All Rights Reserved.

This document and translations of it may be copied and furnished to

others, and derivative works that comment on or otherwise explain

it or assist in its implementation may be prepared, copied,

published and distributed, in whole or in part, without restriction

of any kind, provided that the above copyright notice and this

paragraph are included on all such copies and derivative works.

However, this document itself may not be modified in any way, such

as by removing the copyright notice or references to the Internet

Society or other Internet organizations, except as needed for the

purpose of developing Internet standards in which case the

procedures for copyrights defined in the Internet Standards process

MUST be followed, or as required to translate it into.

APPENDIX: Usage of this payload format

Appendix A. Examples

A.1 Examples of delay analysis with interleave

A.1.1 Group interleave

An example of regular interleave is when packets are formed into

groups.  If the number of packets in a group is N, packet 0 contains

frame 0, frame N, frame 2N, and so on;  packet 1 contains frame 1,

frame 1+N, 1+2N, and so on.  The AU-Index field is used to document

the sequence of the packet within the group (or the first frame in

the packet, which is the same thing in this scheme), and all the

AU-Index-delta fields contain N-1.

van der Meer et al.        Expires October 2002               [Page 27]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

Receivers can tell when a new interleave group is starting, by

noting that the computed time stamp of the first frame in a packet

is later than any previously computed time stamp. This is because no

following packet can contain an earlier RTP time stamp (RTP rules),

and the second and subsequent frames in a packet have larger

time stamps (the frames in a packet are also in time-order).

If the group size is 3, then packets are formed as follows:

Packet   Time stamp   Frame Numbers       AU-Index, AU-Index-delta

0        T[0]         0, 3, 6             0, 2, 2

1        T[1]         1, 4, 7             0, 2, 2

2        T[2]         2, 5, 8             0, 2, 2

3        T[9]         9,12,15             0, 2, 2

In this case, the receiver would have to buffer 4 frames at least

from packets 0 and 1, and can flush all frames when packet 2

arrives. (Frame 0 can be flushed as packet 0 arrives, since it is

the earliest frame we hold, and likewise frame 1 from packet 1; we

are therefore holding 3,4,6,7 until packet 2 arrives).

If there is loss, then the receiver may wait longer than is strictly

necessary before it emits frames.  For example, say packet 1 is lost

from the above example.  Packet 0 allows frame 0 to be emitted, and

then packet 2 arrives, allowing us to notice the loss of frame 1,

and emit frame 2 and 3. Then it is not until the arrival of packet 3

(which has a time-stamp beyond the times of all the frames seen so

far), that we can finish dealing with the loss, even though the

first group has, in fact, ended. (This is in contrast to schemes

which signal the group size explicitly;  if the receiver knows that

this is packet 3 of 3, then even if 2 of 3 is missing, it can

de-interleave this group without waiting for the next one to start).

In the above example the AU-Index is coded with the value 0, as

required for the modes defined in this document. To reconstruct the

original order, the RTP time stamp and the AU-Index-delta are used.

See also 3.2.3.2.

A.1.2 Continuous interleave

In continuous interleave, once the scheme is 'primed', the number of

frames in a packet exceeds the 'stride' (the distance between them).

This shortens the buffering needed, smooths the data-flow, and gives

slightly larger packets -- and thus lower overhead -- for the same

interleave.  For example, here is a continuous interleave also over

a stride of 3 frames, but with 4 frames per packet, for a run of 20

frames.  This shows both how the scheme 'starts up' and how it

finishes.

van der Meer et al.        Expires October 2002               [Page 28]

RFC xxxx        Transport of MPEG-4 Elementary Streams       April 2002

Packet   Time-stamp   Frame Numbers       AU-Index, AU-Index-delta

0        T[0]                     0       0

1        T[1]                 1   4       0  2

2        T[2]             2   5   8       0  2  2

3        T[3]          3   6   9  12      0  2  2  2

4        T[7]          7  10  13  16      0  2  2  2

5        T[11]        11  14  17  20      0 2  2  2

6        T[15]        15  18              0 2

7        T[19]        19                  0

In this case, the receiver has to buffer only 3 frames, not 4. Say

we are waiting for packet 4.  We can flush frames 0, 1, 2, 3, 4, 5,

6;  we are holding therefore 8, 9, 12.   Packet 4 arrives, allowing

us to emit 7,8,9,10, and we are holding 12,13,16.  Each arriving

packet contains 4 frames, and allows 4 frames to be flushed.

In the above example the AU-Index is coded with the value 0, as

required for the modes defined in this document. To reconstruct the

original order, the RTP time stamp and the AU-Index-delta are used.

See also 3.2.3.2.

If there is loss, again the receiver has to wait to emit the erasure

frames.  In this case, say packet 3 is lost.  We were holding frames

4, 5, and 8.  On the arrival of packet 4, (time-stamp of frame 7),

we now know frame 3 was lost, we can emit frames 4,5, and we know 6

must be lost, and emit 7, which is in the packet that arrived. Then

on the arrival of packet 5 (time-stamp 11) we can emit 8, indicate

loss of 9, and emit 10 and 11. Finally, the arrival of packet 6

(time-stamp 15) indicates that 12 must be lost;  we have now

detected all the lost frames.

van der Meer et al.        Expires October 2002               [Page 29]

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Philips |  |  |
| Mailing address |  |  |  |
| Country | Netherlands |  |  |
| Contact person | Jan van der Meer, |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email | jan.vandermeer@philips.com |  |  |
| Place and date of submission | May 1, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | JVT Transport over RTP |  |
| Contribution number |  | JVT-C122 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: