| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C127 Filename: JVT-C127.doc |
| --- | --- |

| Title: | Motion Vector Prediction in Bidirectionally Predictive (B) frames with regards to Direct Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis‡, Jizheng Xu, Feng Wu, and Shipeng Li Microsoft Corporation 3F Sigma Center, No.49 Zhichun Road, Haidian District, Beijing, 100080, China | Tel: Fax Email: | ‡86-10-62617711-3108 86-10-88097306 ‡alexismt@ieee.org |
| Source: | Microsoft |  |  |

_____________________________

Abstract

The current JVT standard appears to be quite unclear on how a Direct Mode coded Macroblock or block should be considered in the motion vector prediction within Bidirectionally Predicted (B) frames. Instead, it appears that the current software considers a Direct Mode Macroblock or subblock as having a “different reference frame” and thus not used in the prediction. Unfortunately, considering that there might still be high correlation between the motion vectors of a Direct predicted block with its neighbors such a condition could considerably hinder the performance of B frames and reduce their efficiency. This could also reduce the efficiency of error concealment algorithms when applied to B frames.

In this document, we propose an alternative approach, which could improve the coding efficiency increase the correlation of motion vectors within B frames. This is done by considering a Direct Mode coded block essentially equivalent to a Bidirectionally predicted block within the motion prediction phase. The modifications necessary are basically negligible.

Proposed Method

Direct Mode Macroblocks or blocks (in the case of 8x8 sub-partitions) could considerably improve the efficacy of Bidirectionally Predicted (B) frames since they can effectively exploit temporal correlations of motion vector information of adjacent pictures. The idea is essentially derived from temporal interpolation techniques where the assumption is made that if a block has moved from a position (x+dx,y+dy)at time t to a position (x,y)at time t+2, then, by using temporal interpolation, at time t+1 the same block must have essentially been at position (x+dx2,y+dy2) (Figure 1). The approach though most often used in current encoding standards instead assumes that the block at position (x,y) of frame at time t+1 most likely can be found at positions (x+dx2,y+dy2) at time t and (x−dx2,y−dy2)at time t+2 (Figure 2).  Since the number Direct Mode coded blocks within a sequence can be significant, whereas no residue and motion information are transmitted for such a case, efficiency of B frames is considerably increased. Run length coding (for the case of UVLC) could also be used to improve performance even further.

Figure 1: Direct Pixel/Block Projection

Figure 2: Direct Motion Projection as is used in B frame coding

Unfortunately, the current JVT standard does not clarify how the motion vector prediction of blocks adjacent to direct mode blocks should be performed. As it appears from the current software, Direct Mode blocks are currently considered as having a “different reference frame” thus no spatial correlation is exploited in such a case. This could considerably reduce the efficiency of the prediction, but could also potentially affect the performance of error concealment algorithms applied on B frames in case such is needed. As an example, if we would like to predict the motion vector of E in the current codec, if A, B, C, and D were all Direct Mode coded, then the predictor will be set as (0,0) which could obviously not be a good decision.

| D B C A E | - E is predicted from A, B, C, D. - If A, B, C, or D are Direct then they are not currently used in prediction. - Modification - If A, B, C, or D are direct, then use actual values of Motion Vectors and reference frames in prediction. Two options: - If collocated macroblock/block in subsequent P frame is intra coded then reference frame is set to -1 - If collocated macroblock/block in subsequent P frame is intra coded then assume reference frame is 0. |
| --- | --- |

Instead, we propose of using the actual Motion information available from the Direct Mode coded blocks, for performing the motion vector prediction. This will enable a higher correlation of the motion vectors within a B frame sequence, and thus can lead to improved efficiency, while only doing negligible modifications in the standard and the reference software.

A possible issue would be how to handle Direct Mode Macroblocks for which, the collocated block/macroblock in the subsequent frame was intra coded. We may select from two possible options.

- Consider this macroblock/block as having a different reference frame, thus not using it in the motion vector prediction.

- Consider this macroblock as having (0,0) motion vector and reference frame 0.

Only the first case was evaluated in our simulation. It should also be noted that a modification is probably necessary in the deblocking filter process. Currently for the direct mode case the software only compares stored motion vectors taken from direct coded blocks (otherwise these are considered as zero). Instead, we should compare the exact motion vectors regardless of the block type used. It is though the author’s opinion that since for Direct Coded blocks no residue is transmitted, a stronger deblocking filter might give better performance. Furthermore, the Rate Distortion Decision for B pictures should probably be reexamined since it is quite likely that under the current motion vector prediction scheme, a different lambda (possibly higher) could lead to better coding efficiency. In addition, it was noted that the current rate distortion optimization scheme, does not properly consider the direct mode during the process (direct mode is considered as a residual transmission macroblock) [4]. We believe that such should be properly investigated and the rate distortion and lambda parameters used should be corrected.

Simulation Results

- The testing conditions described in [2] were selected for the evaluation of this proposal. Due to time constrains though, some of the sequences were not used (especially for the 5 reference simulation). Additional experiments using 1/8th pel accuracy were also performed for a subset of the CIF resolution sequences using 1 reference frame. Our results, presented in more detail in the attached excel document, demonstrate a rather non insignificant PSNR gain, especially on CIF sequences (Table 1).  On the average, using 1 reference frame, prediction using direct MVs could lead to 1.95% bit savings or 0.073dB gain (best case 0.155dB), and for 5 reference frames 1.75% bit savings or 0.066dB PSNR gain (best case 0.150dB). The implementation is also quite straightforward and simple. Note that an implementation of this proposal is also included with document JVT-C128 [5].

Table 1: Average Bit savings and PSNR gain using Direct MV prediction

| Sequence | Res. | Framerate | Ref. | Avg.Bit Saving % (AVSNR) | Avg. PSNR gain (AVSNR) | Avg.Bit Saving % (excel) | Avg. PSNR gain (excel) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Foreman | QCIF | 10 | 1 | 0.22% | 0.011 | 0.22% | 0.008 |
|  | CIF | 30 | 1 | 1.88% | 0.073 | 1.89% | 0.061 |
| Container | QCIF | 10 | 1 | 0.18% | 0.020 | 0.18% | 0.015 |
| Coastguard | CIF | 30 | 1 | 2.86% | 0.072 | 2.86% | 0.097 |
| Mobile | CIF | 30 | 1 | 3.93% | 0.135 | 3.92% | 0.172 |
| Stefan | CIF | 30 | 1 | 1.01% | 0.047 | 1.01% | 0.044 |
|  | QCIF | 15 | 1 | 0.61% | 0.031 | 0.61% | 0.027 |
| Bus | CIF | 30 | 1 | 3.98% | 0.155 | 3.98% | 0.153 |
| Flowergarden | CIF | 30 | 1 | 2.91% | 0.115 | 2.91% | 0.143 |
| Average |  |  | 1 | 1.95% | 0.073 | 1.95% | 0.080 |
| Foreman | QCIF | 10 | 5 | 0.22% | 0.012 | 0.22% | 0.008 |
|  | CIF | 30 | 5 | 1.58% | 0.065 | 1.60% | 0.051 |
| Coastguard | CIF | 30 | 5 | 2.55% | 0.066 | 2.53% | 0.087 |
| Stefan | CIF | 30 | 5 | 1.02% | 0.047 | 1.02% | 0.045 |
|  | QCIF | 15 | 5 | 0.23% | 0.012 | 0.23% | 0.009 |
| Bus | CIF | 30 | 5 | 3.85% | 0.150 | 3.85% | 0.149 |
| Flowergarden | CIF | 30 | 5 | 2.79% | 0.112 | 2.81% | 0.139 |
| Average |  |  | 5 | 1.75% | 0.066 | 1.75% | 0.070 |

References

- ITU-T, Video Coding Expert Group (VCEG), “JVT Coding – (ITU-T H.26L & ISO/IEC JTC1 Standard) – Working Draft Number 2 (WD-2)”, ITU-T JVT-B118, Mar. 2002.

- G. Sullivan, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material”, Doc. VCEG-N81, Sep. 2001.

- G. Bjontegaard, “Calculation of average PSNR differences between RD-Curves”, Doc. VCEG-M33, Mar. 2001

- Hideaki Kimata, “GMVC and GMC with B-picture“, Doc. JVT-C043, May. 2002

- Alexis Michael Tourapis, Feng Wu, and Shipeng Li, “Direct Prediction for Predictive (P) and Bidirectionally Predictive (B) frames in Video Coding“, Doc. JVT-C128, May. 2002

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft |  |  |
| Mailing address | 3F Sigma, No49 ZhiChun Rd. Haidian, Beijing, 100080, China |  |  |
| Country | China |  |  |
| Contact person | Alexis Michael Tourapis |  |  |
| Telephone | (86-10) 6261-7711 ext 3108 |  |  |
| Fax |  |  |  |
| Email | alexismt@ieee.org |  |  |
| Place and date of submission | 5/1/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Motion Vector Prediction in Bidirectionally Predictive (B) frames with regards to Direct Mode |  |
| Contribution number |  | C127 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: