| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3nd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C130 Filename: JVT-C130.doc |
| --- | --- |

| Title: | Simplification of the JVT deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Cristina Gomila and Jill Boyce 2 Independence Way Princeton, NJ 08540 USA | Tel: Email: | + 1 609 734 9505 (9859) GomilaC@tce.com BoyceJ@tce.com |
| Source: | Thomson Multimedia |  |  |

_____________________________

## 1.  Overview

In this contribution we propose the simplification of the deblocking filter currently implemented in the standard [1].  Within the whole process, we aim at reducing the number of line-based operations, which are by far the most demanding term when evaluating the computational cost of filtering.  On average, the proposed method uses on average 47.81% fewer operations than the JM 1.9 deblocking filter, with minimal impact on visual quality and PSNR.

Towards this goal, we have analyzed the deblocking algorithm in order to figure out those parts with the lowest quality gain versus computational cost ratio. Hence the simplification we propose affects the following parts of the algorithm:

- Transition filtering may be skipped for certain intensity levels.  Given that the human visual system is less sensitive to contrast in dark and bright regions, deblocking is not required.  This simplification can strongly reduce the computational cost depending on the sequence.

- Suppression of strong filters. Such a simplification greatly reduces the complexity of the worst case as well as the number of comparisons involved in the filter selection step.

- Simplification of the chrominance filter on the basis of the lower resolution of the chrominance components and the small length of filters.

- Chrominance filtering is linked to luminance filtering.  The suppression of the decision criteria for chrominance filtering reduces the cost of deblocking to the cost of filtering since no decision or selection operations are required.

Hereunder we describe the details of this approach and present some experimental results using JM 1.9, which proves the effectiveness of our proposal. Furthermore, being the core of the technique preserved, the quality of the results does not suffer any significant degradation.

2. Simplification of the deblocking filter

As stated in the overview, our simplification applies only to the line-based filtering steps of the JM 1.9 deblocking filter.  Therefore, the block-based computation of the boundary strength (Bs) has not been modified.  Actually we continue using Bs to decide the intensity of the filter applied on each transition.

The deblocking algorithm proceeds in three steps when it applies to a transition. In a first step it decides if the transition must be filtered or not.  This decision relies on line-based criteria as well as on other block-based parameters previously evaluated.  For those cases it decides to filter, there is a second step in which the filter to apply has to be selected.  Finally, the last step consists in the filtering process itself, where the value of a certain number of pixels surrounding the block edge is modified in order to smooth this transition.  Following these three steps we describe our approach for filtering, first the luminance and then the chrominance components.   The set of eight pixels involved in the filtering of a transition will be denoted by:

| p3 | p2 | p1 | p0 | q0 | q1 | q2 | q3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

where the block edge is located between p0 and q0.

2.1 Inclusion of a new condition in the filtering decision process

In order to reduce the number of filtered transitions, and hence the computational cost, we have added an additional condition based on the intensity of the luminance component. We aim at skipping the deblocking of those transitions that fall in a range of values too dark or too bright to be differentiated by the human eye.  For simplicity, in our current implementation this criterion is only evaluated on one pixel around the transition:

p0 > Ymin   AND   p0 < Ymax

Only those transitions that verify both the basic and these additional conditions will be filtered.  Based on our simulations, the thresholds have been fixed to Ymin=48 and Ymax=232.

This new criterion increases the number of operations involved in the decision process from 10 (in JM 1.9) to 12 (see Table 2.1).  Even with the introduction of two extra comparisons, experiments have shown that the overall number of operations is reduced, because of the computations saved when the decision is made not to filter the line transition. Experiments also have shown that the reduction in operations comes without a significant loss of quality, even though the actual reduction depends on the sequence.

2.2 Suppression of strong filtering

Once the algorithm makes the decision of whether or not to filter the line transition, the filter strength has to be chosen in order to fit the characteristics of the transition.  Filtering aims at: 1) smoothing the gap as much as possible; without 2) blurring the real contours of the objects.

Towards this goal, the JM 1.9 algorithm considers two possibilities: strong and normal filtering.  Strong filters are based on a 5-tap approach that modifies (p2, p1, p0, q0, q1, q2) when filtering luminance transitions (F6*) and only (p1, p0, q0, q1) when filtering chrominance (F4*). Strong filters apply only to some INTRA coded macroblocks within large homogeneous zones (see [1] for more details).

Such constraints strongly limit the use of the strong filters and lead us to question their real effectiveness: 1 condition has to be added in all cases to differentiate INTER from INTRA coded macroblocks, and for these last ones 11 extra operations are needed before deciding if the use of an strong filter applies.

According to our experiments, the cost of making this differentiation seems not to compensate for gain it provides in terms of visual quality.  For this reason, in our approach the largest normal filter (F4 for luminance and F2 for chrominance) has occupied the place of the stronger ones.

| Luminance: | F6* (p2, p1, p0, q0, q1, q2) |  | F4 (p1, p0, q0, q1) |
| --- | --- | --- | --- |
| Chrominance: | F4* (p1, p0, q0, q1) |  | F2 (p0, q0) |

Note how this change 1) simplifies the choice of the filter (for chrominance filtering the same filter is applied in all cases and no selection is required); 2) reduces the complexity of the filtering step (both filters are smaller); and as the most important point, 3) greatly reduces the complexity of the worst case (see Table 2.1).

2.3 Simplification of small filters

The filtering technique actually implemented in JM 1.9 performs a very good job when deblocking strongly deteriorated images.  Used filters used are expensive in terms of computational cost, but effective in terms of the quality achieved.

To tackle this trade off between cost and quality, in our approach we have adopted the original filtering technique for the larger filters (F4 and F3) while proposing a simpler one when only two pixels are involved in the deblocking process (F2).  Depending on if we are filtering luminance or chrominance, our approach proceeds as follows:

- Luminance: F2 (p0, q0) is simply suppressed on the basis of the low impact of such changes in the luminance image.

- Chrominance: because the resolution of these components is smaller, the importance of F2(p0, q0) is larger.  However, as only two pixels are considered for deblocking, we propose a simpler technique that provides similar results while strongly reducing the computational cost of this filtering:

P0   =  (    q1  + 3*p1 )  >>  2

Q0  = ( 3*q1  +     p1 )  >>  2

The reduction in the number of operations per transition filtered can be observed in Table 2.1. Note that in the case of the chrominance filtering, such a simplification has a deep impact in the overall complexity of the algorithm given that the F2 is the only filter defined for deblocking the U and V components, thereby eliminating the comparisons needed to determine the filter choice for chroma blocks.

2.4 Chrominance filtering is linked to luminance filtering

In the JM 1.9 implementation of the deblocking filter, all the color components of the YUV signal are filtered independently.  Such a procedure ensures the best quality when deblocking color, however it forces both the filtering decision and the filter selection to be recomputed for the chrominance, even if these components have lower resolution and a smaller impact in the subjective evaluation of the visual quality.

In the algorithm we propose the decision of filtering the chrominance transitions to be linked to the decision of filtering already taken for the luminance transition.  As this simplification affects to all the filtered transitions for the chrominance components, it has a deep impact on the final complexity of the algorithm.   This fact can be easily observed when analyzing the experimental results in the following section.

To summarize all the simplifications proposed in this contribution, Table 2.1 gives the number of operations incurred when deblocking a transition with both the original and the simplified version of the algorithm.  Note that in our approach the strong filtering is not differentiated any more from normal filtering.

| ORIGINAL VERSION |  | comp | shift | sum | mult | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| Luma |  |  |  |  |  |  |
|  | Filtering decision | 7 | 0 | 3 | 0 | 10 |
|  | Filter selection (intra) | 9 | 1 | 2 | 0 | 12 |
|  | Filter selection (inter) | 6 | 0 | 2 | 0 | 8 |
|  | F3 (p2, p0, q0) | 8 | 4 | 10 | 0 | 22 |
|  | F3 (p0, q0, q1) | 8 | 4 | 10 | 0 | 22 |
|  | F4 (p1, p0, q0, q1) | 10 | 6 | 13 | 0 | 29 |
|  | F6* (p2, p1, p0, q0, q1, q2) | 0 | 12 | 29 | 0 | 41 |
| Chroma |  |  |  |  |  |  |
|  | Filtering decision | 7 | 0 | 3 | 0 | 10 |
|  | Filter selection (intra) | 9 | 1 | 2 | 0 | 12 |
|  | Filter selection (inter) | 5 | 0 | 2 | 0 | 7 |
|  | F2 (p0, q0) | 6 | 2 | 7 | 0 | 15 |
|  | F4* (p1, p0, q0, q1) | 0 | 8 | 19 | 0 | 27 |

| OUR APPROACH |  | comp | shift | sum | mult | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| Luma |  |  |  |  |  |  |
|  | Filtering decision | 9 | 0 | 3 | 0 | 12 |
|  | Filter selection | 5 | 0 | 2 | 0 | 7 |
|  | F3 (p1, p0, q0) | 8 | 4 | 10 | 0 | 22 |
|  | F3 (p0, q0, q1) | 8 | 4 | 10 | 0 | 22 |
|  | F4 (p1, p0, q0, q1) | 10 | 6 | 13 | 0 | 29 |
| Chroma |  |  |  |  |  |  |
|  | Filtering decision | 0 | 0 | 0 | 0 | 0 |
|  | Filter selection | 0 | 0 | 0 | 0 | 0 |
|  | F2 (p0, q0) | 0 | 2 | 2 | 2 | 6 |

Table 2.1.  Number of operations involved in the deblocking process of one transition.

3. Experimental results

Experimental results are provided on six test sequences using the JM 1.9 implementation of the standard.  The following parameters have been selected to configure the encoder:

Intra period:			0 and 15

QP:				24

Motion vector resolution: 	1/4 pel

Inter block search:		all modes enabled

Number of reference frames: 	2

Number of B frames:		0

Entropy coding method:		CABAC

Tables 3.1 and 3.2 allow comparing the gain achieved by our technique on two different sequences: Foreman (qcif) and Tempete (cif).  The cost of deciding, selecting and filtering is expressed as the total number of operations required (see Table 2.1 presented before).  Besides, for each filter, its percentage of use is provided. Bold numbers at the bottom of tables represent the total number of operations needed to filter one transition.

ORIGINAL VERSION

| Luma | Cost | % use | Chroma | Cost | % use |
| --- | --- | --- | --- | --- | --- |
| F2 (p0, q0) | 0 | 0.036 | F2 (p0, q0) | 15 | 0.984 |
| F3 (p1, p0, q0) | 22 | 0.100 | F4* (p1, p0, q0, q1) | 27 | 0.016 |
| F3 (p0, q0, q1) | 22 | 0.097 |  |  |  |
| F4 (p1, p0, q0, q1) | 29 | 0.748 |  |  |  |
| F6* (p2, p1, p0, q0, q1, q2) | 29 | 0.020 |  |  |  |
| Cost of deciding | 10 |  | Cost of deciding | 10 |  |
| Cost of selecting (intra) | 12 |  | Cost of selecting (intra) | 12 |  |
| Cost of selecting (inter) | 8 |  | Cost of selecting (inter) | 7 |  |
| Cost of filtering |  | 27.355 | Cost of filtering |  | 15.192 |
| TOTAL |  | 45.435 |  |  | 32.192 |

OUR APPROACH

| Luma | Cost | % use | Chroma | Cost | % use |
| --- | --- | --- | --- | --- | --- |
| F0 | 0 | 0.065 | F0 | 0 | 0.103 |
| F2 (p0, q0) | 0 | 0.036 | F2 (p0, q0) | 6 | 0.897 |
| F3 (p1, p0, q0) | 22 | 0.096 |  |  |  |
| F3 (p0, q0, q1) | 22 | 0.093 |  |  |  |
| F4 (p1, p0, q0, q1) | 29 | 0.710 |  |  |  |
| Cost of deciding F0 | 2 |  | Cost of deciding | 0 |  |
| Cost of deciding F2, F3, F4 | 12 |  |  |  |  |
| Cost of selecting F0 | 0 |  | Cost of selecting | 0 |  |
| Cost of selecting F2, F3, F4 | 7 |  |  |  |  |
| Cost of filtering |  | 24.733 | Cost of filtering |  | 5.381 |
| TOTAL |  | 42.626 |  |  | 5.381 |

Table 3.1.  Complexity comparison based on the number of operations required to deblock one transition.  Results are presented on the Foreman sequence.

ORIGINAL VERSION

| Luma | Cost | % use | Chroma | Cost | % use |
| --- | --- | --- | --- | --- | --- |
| F2 (p0, q0) | 0 | 0.071 | F2 (p0, q0) | 15 | 0.976 |
| F3 (p1, p0, q0) | 22 | 0.136 | F4* (p1, p0, q0, q1) | 27 | 0.024 |
| F3 (p0, q0, q1) | 22 | 0.127 |  |  |  |
| F4 (p1, p0, q0, q1) | 29 | 0.642 |  |  |  |
| F6* (p2, p1, p0, q0, q1, q2) | 29 | 0.023 |  |  |  |
| Cost of deciding | 10 |  | Cost of deciding | 10 |  |
| Cost of selecting (intra) | 12 |  | Cost of selecting (intra) | 12 |  |
| Cost of selecting (inter) | 8 |  | Cost of selecting (inter) | 7 |  |
| Cost of filtering |  | 26.440 | Cost of filtering |  | 15.283 |
| TOTAL |  | 44.532 |  |  | 32.401 |

OUR APPROACH

| Luma | Cost | % use | Chroma | Cost | % use |
| --- | --- | --- | --- | --- | --- |
| F0 | 0 | 0.430 | F0 | 0 | 0.493 |
| F2 (p0, q0) | 0 | 0.052 | F2 (p0, q0) | 6 | 0.507 |
| F3 (p1, p0, q0) | 22 | 0.080 |  |  |  |
| F3 (p0, q0, q1) | 22 | 0.077 |  |  |  |
| F4 (p1, p0, q0, q1) | 29 | 0.361 |  |  |  |
| Cost of deciding F0 | 2 |  | Cost of deciding | 0 |  |
| Cost of deciding F2, F3, F4 | 12 |  |  |  |  |
| Cost of selecting F0 | 0 |  | Cost of selecting | 0 |  |
| Cost of selecting F2, F3, F4 | 7 |  |  |  |  |
| Cost of filtering |  | 13.921 | Cost of filtering |  | 3.044 |
| TOTAL |  | 25.609 |  |  | 3.044 |

Table 3.2.  Complexity comparison based on the number of operations required to deblock one transition.  Results are presented on the Tempete sequence.

In both cases we can observe that linking the chrominance filtering to the luminance decision of filtering provides the most reduction in terms of computational cost.  The simplification of F2 for chrominance is also of great importance.

Furthermore, an important reduction can also be obtained depending on the sequence when skipping the filtering of those transitions too dark or too bright to be visually distinguished.  The number of all these unfiltered transitions is associated to a dummy filter F0.  Note also that in our approach all the luminance transitions classified as F2 are not filtered.

Table 3.3 provide the gains in terms of cost reduction, PSNR and bitrate for a set of test sequences. For each sequence, the reduction associated to the filtering of the luminance (Y) and chrominance components (UV) is given independently.

When comparing the proposed method to JM 1.9, there is a reduction in operations for deblocking of 39.53% on average for the QCIF sequences and of 56.09% on average for the CIF sequences.  The overall average reduction was 47.81%.  Furthermore there is not a significant loss in terms of PSNR.

The visual quality of the initial Intra frame for the proposed method as compared to JM1.9 can be seen in Figure 3.1 for the Foreman sequence and Figure 3.2 for the Mobile sequence,

| Sequence | Format |  cost/luma |  cost /chroma |  cost /frame |
| --- | --- | --- | --- | --- |
| Foreman | qcif | -4.63% | -87.17% | -39.92% |
| Container | qcif | -5.74% | -89.78% | -42.21% |
| Silent | qcif | -2.36% | -86.87% | -36.45% |
| QCIF average | qcif | -9.01% | -88.70% | -39.53% |
| Mobile | cif | -14.91% | -92.98% | -58.29% |
| Paris | cif | -6.79% | -90.50% | -47.56% |
| Tempete | cif | -51.30% | -93.73% | -62.41% |
| CIF average | cif | -34.50% | -94.26% | -56.09% |
| Overall average | qcif/cif | -21.76% | -91.48% | -47.81% |

| Sequence | # Format |  PSNRY |  PSNRU |  PSNRV |  Bitrate |
| --- | --- | --- | --- | --- | --- |
| Foreman | qcif | 0.00dB | -0.06dB | -0.01dB | 0.00% |
| Container | qcif | +0.04dB | -0.53dB | -0.14dB | 0.00% |
| Silent | qcif | -0.01dB | -0.24dB | -0.03dB | 0.00% |
| QCIF average | qcif | 0.00 dB | -0.22 dB | -0.07 dB | 0.00% |
| Mobile | cif | -0.01dB | -0.17dB | -0.27dB | +2.29% |
| Paris | cif | -0.05dB | -0.37dB | -0.37dB | 0.00% |
| Tempete | cif | -0.03dB | -0.25dB | -0.26dB | +1.38% |
| CIF average | cif | -0.02 dB | -0.32 dB | -0.28 dB | +1.22% |
| Overall average | qcif/cif | -0.01 dB | -0.27 dB | -0.17 dB | +0.62% |

Table 3.3.  Comparison between the simplified proposal and original JM1.9 software. Configuration: IPPP coding

Figure 3.1(a) Foreman with JM 1.9

Figure 3.1 (b) Foreman with Proposed Method

Figure 3.2 (a) Mobile with JM 1.9

Figure 3.2(b) Mobile with Proposed Method

4. References

[1] Thomas Wiegand and Gary Sullivan, "Working Draft Number 2, Revision 4 (WD-2 rev 4)", jwd2r4.doc, Geneva, January 29-February 1, 2002.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Thomson multimedia |  |  |
| Mailing address | 2 Independence Way Princeton, NJ 08540 |  |  |
| Country | USA |  |  |
| Contact person | Cristina Gomila |  |  |
| Telephone | +1 609 734 9505 |  |  |
| Fax | +1 609 734 9870 |  |  |
| Email | gomilac@tce.com |  |  |
| Place and date of submission | Fairfax, VA, USA May 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Simplification of JVT Deblocking Filter |  |
| Contribution number |  | JVT-C130 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| x | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

According to our notation Fn denotes a filter changing n pixel values, while the mark * is used to differentiate strong filters from normal filters.