| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C131 Filename: JVT-C131.doc |
| --- | --- |

| Title: | NAL Packet Segmentation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thomas Stockhammer Institute for Communications Engineering Munich University of Technology D-80290 Munich Germany | Tel: Email: | +49-89-28923474 stockhammer@ei.tum.de |
|  | Stephan Wenger Teles AG / TU Berlin Franklinstr. 28-29 D-10587 Berlin Germany | Tel: Email: | +49-172-3000813 stewe@cs.tu-berlin.de |
| Source: | Munich University of Technology, TELES AG / ITU Sector Member |  |  |

_____________________________

# Summary

A new NAL packet type and an additional NAL header flag are proposed to allow segmentation of long NAL packets in shorter packets. Syntax and Semantics are included in this contribution.

# Motivation

A new NAL packet type and an additional NAL header flag are proposed to allow segmentation of NAL packets. Several useful scenarios of this segmentation functionality can be listed. Long single slice packets or data partition packets are beneficial as all inter macroblock predictions (MB skip, intra prediction, MV prediction, CABAC context, etc.) are allowed over the entire slice and compression efficiency is increased. However, for some scenarios long NAL packets can either not be transmitted due to limited maximum transfer unit (MTU) sizes or it is beneficial for the underlying network protocol to transmit shorter packets. Some examples for applications are

- Limited MTU size in RTP/IP applications (RTP segmentation would be possible),

- If the loss probability of shorter RTP packets is smaller than for longer (e.g., bit error prone transmission), then retransmission protocols are more efficient for shorter RTP packets,

- Same arguments hold for H.324 based transmission.

Although on the JVT transport reflector possible solutions in the VCL have been discussed, this adaptation of the packet size is an NAL feature. Therefore, the introduction of the segmentation functionality is proposed based on an NAL packet type and an additional segmentation flag in the NAL packet header. The segmentation functionality is the somehow complementary to the compound packet functionality.

# Syntax

To signal the segmentation functionality we propose to change the NAL packet header table in the working draft by introducing a Segmented NAL Sub-Packet (SNSP) and an open packet flag (OPF) as follows.

| NALP header | NALP packet type | OPF | EI-Flag |
| --- | --- | --- | --- |
| 0x10 | SSP | 0 | 0 |
| 0x11 | SSP | 0 | 1 |
| 0x12 | SSP | 1 | 0 |
| 0x13 | SSP | 1 | 1 |
| 0x20 | DPA | 0 | 0 |
| 0x21 | DPA | 0 | 1 |
| 0x22 | DPA | 1 | 0 |
| 0x23 | DPA | 1 | 1 |
| 0x30 | DPB | 0 | 0 |
| 0x31 | DPB | 0 | 1 |
| 0x32 | DPB | 1 | 0 |
| 0x33 | DPB | 1 | 1 |
| 0x40 | DPC | 0 | 0 |
| 0x41 | DPC | 0 | 1 |
| 0x42 | DPC | 1 | 0 |
| 0x43 | DPC | 1 | 1 |
| 0x50 | SEIP | 0 | 0 |
| 0x51 | SEIP | 0 | 1 |
| 0x52 | SEIP | 1 | 0 |
| 0x53 | SEIP | 1 | 1 |
| 0x60 | PUP | 0 | 0 |
| 0x61 | PUP | 0 | 1 |
| 0x62 | PUP | 1 | 0 |
| 0x63 | PUP | 1 | 1 |
| 0x70 | CP | 0 | 0 |
| 0x71 | CP | 0 | 1 |
| 0x72 | CP | 1 | 0 |
| 0x73 | CP | 1 | 1 |
| 0x80 | IDERP | 0 | 0 |
| 0x81 | IDERP | 0 | 1 |
| 0x82 | IDERP | 1 | 0 |
| 0x83 | IDERP | 1 | 1 |
| 0x90 | SNSP | 0 | 0 |
| 0x91 | SNSP | 0 | 1 |
| 0x92 | SNSP | 1 | 0 |
| 0x93 | SNSP | 1 | 1 |

# Semantics

For the semantic description let us define a regular NAL packet, segmented NAL sub-packets (SNSPs) and a transport NAL packet. Regular NAL packets (SSP, DPA, DPB, DPC, IDERP, PUP, SEIP, CP) are either directly transmitted in one transport packet or segmented into multiple NAL sub-packets which are then individually transported in transport packets. The following packetization rules apply using the SNSP and OPF:

- For all regular NAL packets which are transported in one transport packet the OPF is set to 0.

- For all regular NAL packets which are transported in multiple transport packets, the following rules apply
- the first transport packet contains the NAL packet header with OPF set to 1 and the initial bytes of the regular NAL packet
- the next bytes of the regular NAL packet are appended to a SNSP header. If the entire remainder of the regular NAL packet is contained in the transport packet the OPF is set to 1. Otherwise, this procedure is repeated until bytes of the regular packets are transported in SNSPs. For the final SNSP the OPF is set to 1.

- The segmented packets resulting from a regular packet shall be transmitted consecutively on the underlying network (this has to be defined somewhere what this means).

- Segmentation of SNSP is not allowed.

- Segmentation of CPs should be avoided.

- The emulation prevention is applied to all regular NAL packets.

The NAL decoder expects to receive NAL packets in transmission order. The miss of one or more packets shall be indicated to the NAL receiver.

When receiving a regular NAL packet with OPF set to 1, the decoder shall store this packet. If no missing packet is signaled the payload of all received SNSPs should be appended to the stored packet until an SNSP with an OPF set to 0 is received. If any missing packet is signaled while an NAL packet is open, the entire NAL packet shall be discarded. Any SNSP received without having stored an open NAL packet should be discarded.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Munich University of Technology |  |  |
| Mailing address | Institute for Communications Engineering Munich University of Technology 80290 Munich |  |  |
| Country | Germany |  |  |
| Contact person | Thomas Stockhammer |  |  |
| Telephone | +49 89 28923474 |  |  |
| Fax | +49 89 28923490 |  |  |
| Email | stockhammer@ei.tum.de |  |  |
| Place and date of submission | Fairfax, USA, May 1st, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | NAL Packet Segmentation |  |
| Contribution number |  | JVT-C131 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: