| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C139 Filename: JVT-C139.doc |
| --- | --- |

| Title: | Macroblock Adaptive Frame/Field Coding for Interlace Sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Limin Wang, Rajeev Gandhi, Krit Panusopone, Yue Yu and Ajay Luthra Motorola Inc. Broadband Communications Sector 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 858-404-3893 liwang@gi.com |
| Source: | Motorola Inc. |  |  |

_____________________________

## Abstract

This document presents the results of MB level adaptive frame/field coding for interlaced video materials, and the performance comparisons with picture level adaptive frame/field coding [3]. The simulation results show that MB level adaptive coding can provide additional gain over picture level adaptive coding for sequences that favor frame coding. MB level adaptive frame/field coding can also be integrated in picture level adaptive coding. MB/picture level adaptive coding provides the best performance (up to about 18% savings in the bit rate over current technology in WD2r8 [6]).

1. Introduction

A frame of an interlace video sequence consists of two fields, scanned at different time instants. The two fields of a frame can be coded jointly (e.g. frame-based coding) or separately (e.g. field-based coding). The decision on frame or field coding can be made at either picture level [3] or MB level [4].

Picture level adaptive frame/field coding demonstrates a significant improvement over fixed frame or field coding [3]. In this document, the decision on frame/field coding is moved to MB level. A MB can now be coded in either frame or field mode. This MB level adaptive frame/field coding is aligned with core experiment for interlace coding defined in [2]. The simulation results show that MB level adaptive coding gives an additional gain over picture level adaptive coding for the sequences that favor frame coding. MB level coding can be integrated into picture level adaptive coding as well.

2. Adaptive Frame/Field Coding at MB level

In MB level adaptive coding, the selection of frame or field coding is at MB level. A frame/field flag of one bit may therefore be required at MB level to indicate if the MB is coded in frame or field mode, as shown in Fig. 1. “0” can be used to indicate frame coding and “1” field coding. For detailed information on MB level coding, refer to [5].

Fig. 1. A frame/field flag of one bit may be required to indicate if the MB is coded in frame or field mode.

2.1 Modes for Regular MB

A MB of 16x16 can be divided in one of seven patterns (modes) [1], as shown in Fig. 2. The associated block size for the seven modes are 16x16, 16x8, 8x16, 8x8, 8x4, 4x8, and 4x4. A MB can be coded in either intra or inter. For intra mode, MB can be only in either mode 1 (block size of 16x16) or mode 7 (block size of 4x4). For inter mode, MB can be in any of seven modes. Motion estimation and compensation (ME/MC) is performed for these blocks separately. In other words, each block within a MB has a separate MV, and hence, a MB can have up to 16 MVs, depending upon the MB mode.

Fig. 2. Seven block patterns for a MB.

2.2 Modes for Super MB

The concept of super MB was introduced for interlace coding in [4]. A super MB consists of 2 MBs of 16x16, as shown in Fig. 3. A super MB of 32x16 can be coded as two frame MBs of 16x16, or one top-field MB of 16x16 and one bottom-field MB of 16x16. For frame coding, a super MB is coded as two frame MBs and each of two MBs can be further divided into one of seven modes, as shown in Fig. 2. For field coding, a super MB is first split into one top-field MB and one bottom-field MB, as shown in Fig. 4. The top-field, or the bottom-field, MB is further divided into one of seven block patterns (modes 1a – 7a), as shown in Fig. 4. The block size in the seven modes can be 16x16, 16x8, 8x16, 8x8, 8x4, 4x8 or 4x4, -- the same as for frame MB (Fig. 2).

Motion compensation for super MB is performed in the same way as described in [1]. Each MB in a super MB has its own reference frame. When a super MB employs field mode, one field MB may use any field of any picture in the reference frame buffer. Reference frame information (ref_frame) of super MB in field mode determines which field is used for motion compensation. The field notation is set in favor to the same field; i.e., the lower number to the same field parity. For example, the top field MB of a super MB with ref_frame = 0 points to the top field of the previous reference picture. Similarly, the bottom field MB of a Super MB with ref_frame = 0 points to the bottom field of the previous reference picture.

The coding rules for super MB, such as intra prediction, reference frame/field and the assignment of code numbers for reference fields in the reference frame (field) buffer, follows the same as described in [5]. For the skipped MB (copy mode), if in field, it is reconstructed by copying the co-located MB in the most recently coded (past) I or P field of the same field parity.

Fig. 3. Input frame is divided into super MB of 2 16x16 pixels.

Fig. 4. For field-based coding, a super MB is split into a top-field MB and a bottom-field MB. The top- and the bottom-field MB is further divided into one of seven block patterns (modes).

Super MBs of a frame are coded from left to right, and top to bottom. For frame-based coding, the top MB of a super Mb is coded first, followed by the bottom MB. For field-based coding, the top-field MB of a super MB is coded first followed by the bottom-field MB. PMV of the current block follows the same rules described in [1].

3. Encoder Architectures with MB level Adaptive coding

Fig. 5 and 6 shows two possible encoder architectures with MB level adaptive coding. In Fig. 5, the input frame is always coded as one frame picture. Adaptation of frame/field coding is made on each MB (or super MB). The decision on frame or field coding for a MB (or super MB) can be RD-based. In Fig. 6, the input frame can be coded in either frame or field structure. For frame picture, a MB (or super MB) can be coded in frame or field mode. The decision on frame or field coding for a MB (or super MB) can be RD-based. For field picture, there is no MB level adaptive frame/field coding and two fields of a frame are coded sequentially. Note that the 1st coded field of a frame, if I or P, can be used as reference field for the 2nd field of the same frame.

The decoder will learn the picture structure of the incoming frame from PSTRUCT in picture header [6]. For PSTRUT=0 (frame coding), the decoder can expect that some of MB in the picture may be coded in frame mode and others in field mode.

Fig. 5. A MB can be coded in either frame or field mode.

Fig. 6. Input frame can be coded in either frame or field structure. For frame coding, MB can be in either frame or field mode. For field coding, the first coded field, if I or P, can be used as reference field for the second field of the same frame.

4. Computer Simulations

Simulations were carried out for six video sequences, as shown in Table 1, where the first three sequences are the common test sequences selected for interlace coding core experiment [2]. For comparison purpose, three encoder architectures, picture level adaptive coding, MB level adaptive coding (Fig. 5) and MB/picture level adaptive coding (Fig. 6), were performed for these sequences separately. A set of four quantization parameters (QP) was used per test per sequence, where QP for I and P are 16, 20, 24, and 28 and QP for B is equal to QP for P plus 2. Other coding parameters are listed in Table 2. Two picture structures are used in simulations. They are IPPPPP… and IBBPBBP….

Table 1. Test Sequences

|  | Format 4:2:0 | Length | Frame/Second |
| --- | --- | --- | --- |
| Canoa | 720x576 | 220 | 25 |
| Rugby | 720x576 | 220 | 25 |
| Mobile | 720x480 | 260 | 30 |
| Tempete | 720x480 | 260 | 30 |
| Coastguard | 720x480 | 260 | 30 |
| Ice Hockey | 528x480 | 300 | 30 |

Table 2. Test conditions

| Entropy C. | MC | Hadamard | Ref. Frames | Search Res. | RD Opt. | Search R. |
| --- | --- | --- | --- | --- | --- | --- |
| UVLC | 1/4 pel | Yes | 2 | 2 | Yes | 16 |

The results are presented in terms of bit rate saving (%) calculated by using the SW provided by Real Network (Bjontegaard Delta bitrate saving in % [7]). Table 3 shows the bit rate saving (%) by using MB level adaptive coding over picture level adaptive coding. For the sequences favoring frame coding [3], such as Mobile and Tempete, MB level adaptive coding provide a significant gain over picture level adaptive coding (18~24% for Mobile and 3~4%for Tempete). For the sequences favoring field coding, such as Canoa and Rugby, picture level adaptive coding has some advantage (-3 % for Canoa and -6~-9% for Rugby).

Table 4 shows the bit rate saving (%) by using MB/picture level adaptive coding over picture level adaptive coding. The performance of MB/picture level adaptive coding is guaranteed over picture level coding.

Table 3. Bit Rate savings (%) by using MB level adaptive coding over picture level adaptive coding

|  | Canoa | Rugby | Mobile | Tempete | Coastguard | Ice Hockey |
| --- | --- | --- | --- | --- | --- | --- |
| IPPPPPPP… | -2.67 | -6.79 | 18.02 | 4.24 | 3.82 | 1.35 |
| IBBPBBP… | -7.20 | -9.70 | 14.00 | 3.00 | TBD | 3.27 |

Table 4. Bit Rate savings (%) by using MB/picture level adaptive coding over picture level adaptive coding

|  | Canoa | Rugby | Mobile | Tempete | Coastguard | Ice Hockey |
| --- | --- | --- | --- | --- | --- | --- |
| IPPPPPPP… | 0.14 | 0.02 | 18.02 | 4.24 | 4.05 | 1.35 |
| IBBPBBP… | 0.40 | 0.23 | 14.00 | 3.00 | TBD | 3.27 |

5. Conclusions

Computer simulations were carried out for the test sequences. Test conditions and picture structures of simulations follow the same as defined in core experiment [2]. The simulation results show that

- MB level adaptive coding can provide significant improvement over picture level adaptive coding for sequence that favor frame coding.

- Picture level adaptive coding has some advantage for sequences that favor field coding.

- MB/picture level adaptive coding provides the best performance (up to about 18% savings in the bit rate over current technology in WD2r8 [6]).

References:

- ITU-T, Video Coding Expert Group (VCEG), “H.26L test model long term number 9 (TML-9) draft 0”, 12/21/2001.

- P. Borgwrdt “Core Experiment on Macroblock Adaptive Frame/Field Encoding” Geneva, JVT-B117, 01/29/02.

- L. Wang, K. Panusopone, R. Gnadhi, Y. Yu, and A. Luthra, “Adaptive frame/field coding for JVT video coding”, JVT-B-071, Geneva, Jan. 2002.

- L. Wang, R. Gandhi, K. Panusopone, Y. Yu, and A. Luthra, “MB-level adaptive frame/field coding for JVT”, JVT-B-106, Geneva, Jan. 2002.

- L. Wang, K. Panusopone, R. Gandhi, Y. Yu, and A. Luthra, “Interlace coding tools for H.26L video coding”, VCEG-O37, Pattaya, Dec. 2001.

- T. Wiegand  and G. Sullivan “Working Draft Number 2, Revision 8 (WD-2 rev 8)”, 4/23/02.

- G. Bjontegaard, “Calculation of average PSNR differences between RD-curves”, VCEG-M33, Austin, 4/2/01.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Motorola Inc. |  |  |
| Mailing address | # 6450 Sequence Drive San Diego, CA 92121 |  |  |
| Country | U.S.A. |  |  |
| Contact person | Limin Wang |  |  |
| Telephone | 858-404-3893 |  |  |
| Fax | 858-404-2501 |  |  |
| Email | liwang@gi.com |  |  |
| Place and date of submission | San Diego, CA., USA, 05/01/02 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Macroblock adaptive frame/field coding for interlace sequences |  |
| Contribution number |  | JVT-C139 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| Without having conducted a search or investigation, the submitter/author(s) individually believe(s) they or General Instrument Corporation (Patent Holder) may have granted, pending or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. see comments below regarding Baseline Profile |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | Comment to Section 2.2. To support the proposed royalty free JVT Baseline Profile, and in addition to the statement of 2.2 above, Patent Holder is also willing to grant a royalty free license to patents essential to JVT Baseline Profile decoder applications on the condition that any and all other patent holders do the same. |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: