| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, May 6-10, 2002 | Document: JVT-C151 Filename: JVT-C151r1.doc Generated: 2002-04-30 |
| --- | --- |

| Title: | New Intra Prediction using Intra-Macroblock Motion Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Siu-Leong Yu and Christos Chrysafis Divio Inc. 441 DeGuigne Drive Sunnyvale, CA 94085, U.S.A. | Tel: Fax: Email: | +1 408 328 3321 +1 408 328 3399 siu@divio.com |
| Source: | Divio Inc. |  |  |

# Introduction

This document proposes a new approach to improve Intra macroblock coding by using motion compensation technique for video coding. This approach can also be applied to image coding.

Motion compensation with motion estimation has provided significant improvements for coding inter frames by forming predictions from reference frames other than the frame itself. This concept has been used successfully to remove temporal redundancy for video coding standards including MPEG-1, MPEG-2, MPEG-4, H261, H.263 and H.26L. Multi-frames have been proposed and used in the H.26L standard as well as the JVT standard. Coding gain is achieved by coding the difference between the frame itself and the prediction.  Macroblocks that do not have good prediction, i.e. the residue is high, are coded as intra-macroblocks similar to those in the intra-frames.

Although intra-frames use significantly more bits to encode them comparing to the inter-frames, intra frames are needed in order to avoid error propagation, achieve random access and support trick play modes, such as seeking, fast forward and fast backward. To reduce the number of bits coding macroblocks in intra-frames as well as intra-macroblocks in inter-frames, we propose to use a motion compensation technique by forming the prediction from the frame itself. All techniques for motion compensation including different motion modes, motion vector coding as well as the bitstream syntax used in the JVT standard can be used for this new approach. Regular motion estimation can also be applied.

To successfully decode the encoded bitstreams, we need to address the issue of causality. We cannot use the macroblock information in the future since it is not available during decoding. For the raster scanning order of marcroblocks depictured in Figure 1, assume the macroblok A is the current marcroblock used for encoding or decoding, we can not use the information for macroblocks in the shading area C. The non-shaded area B consists for the reconstructed macroblocks before the current macroblock, all pixels in this area are available for prediction since they have already been decoded.

To avoid limiting the search area in the causal areas only we fill in the non causal part of the image by a predefined value. In all our experiments we assume that pixel values that have not yet been encoded or decoded are zero. Other techniques such as pixel replication and/or symmetric extension can also be used to improve performance.

One can also argue that limiting the search area to the causal part only can save in terms of computational complexity, that is indeed the case. It is unlikely that we can find a match in a non causal area even after we have performed some type of image extension. It is also possible to save in terms of bit rate when we transmit the motion vectors, since in out case there are only vectors with positive sign. In our current work we haven’t done so, we just chose to keep the motion estimation algorithm the same as for the P frame types, requiring thus only a limited amount of modifications to the JVT verification model.

# ¼,1/8 of a pixel interpolation and memory requirements

From an implementation point of view the only difficulty in our algorithm appears when we want to use fractional motion vectors. In this case we need to have a higher resolution version of our image to be able to perform the motion estimation in software. We chose to use a frame buffer with size 4x the image size in the case of quarter pixel motion estimation and 8x in the case of 1/8 of pixel motion estimation in the exact same way as in the reference implementation for P frame motion estimation. The reference frame is set to zero when we start encoding. Every time we encode a new macroblock we update a portion of our reference frame that corresponds to the given macroblock as well as the area around it. The algorithm to do that is fairly simple to implement, it turns out that each 16x16(x4x4 or x8x8) macroblock only affects a 24x24(x4x4 or x8x8) area in the interpolated reference frame, and this is the area to be updated. This approach only affects the software implementation of the encoder, the decoder works in the exact same way as in the decoding of P frames. On the decoder side the reference frame is kept in the original image resolution, after decoding each macroblock we just update a 16x16 area of the reference frame. Hardware implementations of the encoder can work either way depending of the requirements.

There is no extra memory needed for buffering our reference frames, since we can reuse the memory allocated for P frame motion estimation.

B

A

C

Figure 1 Depiction of the current macroblock to be encoded along with the causal past (Area B), and the future (area C).

# The Results

TABLE 1: PSNR and bitrate improvement of Intra macroblock motion compensation

| Sequence | Frame Size | Proposed Improvement |  |
| --- | --- | --- | --- |
|  |  | psnr (dB) | bits (%) |
| akiyo | CIF | 0.35 | 6.28 |
| MaD900 | CIF | 0.10 | 2.13 |
| coastgurad | CIF | 0.02 | 1.21 |
| foreman | CIF | 0.52 | 12.44 |
| container | CIF | 0.09 | 2.40 |
| deadline | CIF | 0.09 | 1.81 |
| hall | CIF | 0.09 | 2.34 |
| news | CIF | 0.28 | 4.54 |
| paris | CIF | 0.13 | 2.10 |
| students | CIF | 0.11 | 2.55 |
| akiyo | QCIF | 0.20 | 3.41 |
| MaD900 | QCIF | 0.04 | 0.72 |
| coastgurad | QCIF | 0.01 | 1.00 |
| foreman | QCIF | 0.87 | 13.88 |
| container | QCIF | 0.09 | 2.01 |
| deadline | QCIF | 0.02 | 0.49 |
| hall | QCIF | 0.06 | 0.90 |
| news | QCIF | 0.12 | 1.57 |
| paris | QCIF | 0.03 | 0.49 |
| students | QCIF | 0.04 | 0.83 |

# Conclusion

We propose to use the motion estimation module of the JVT algorithm when we encode intra frames. This motion estimation algorithm remains idle during and I frame encoding. By using the motion estimation module during the encoding of an I frame we do not increase the total cost of a JVT system, since we are reusing an existing module. On top of that we do not even increase the worse case timing requirements for the encoding of any given frame, since the motion estimation algorithm was idle during the encoding of an I frame.

The proposed modification can also be applied to the encoding of intra macroblocks in a P or B frame if complexity is not a concern, thus increasing the compression ratio even for P and B frames.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Divio, Inc. |  |  |
| Mailing address | Divio Inc. 441 DeGuigne Drive Sunnyvale, CA 94085, U.S.A. |  |  |
| Country | U.S.A. |  |  |
| Contact person | Dr. Siu-Leong Yu |  |  |
| Telephone | +1 408 328 3321 |  |  |
| Fax | +1 408 328 3399 |  |  |
| Email | siu@divio.com |  |  |
| Place and date of submission | Sunnyvale, CA, U.S.A., April 30 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | New Intra Prediction using Intra-Macroblock Motion Compensation |  |
| Contribution number |  | JVT-C151 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: