| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C152 Filename: JVT-C152.doc |
| --- | --- |

| Title: | JVT/H.26L Decoder Complexity Analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Michael Horowitz Polycom Inc. 5000 Plaza on the Lake Austin, Texas 78746 USA Anthony Joch UB Video Inc. Suite 400,1788 West 5th Ave Vancouver, British Columbia Canada V6J 1P2 Faouzi Kossentini UB Video Inc. Suite 400,1788 West 5th Ave Vancouver, British Columbia Canada V6J 1P2 Antti Hallapuro Nokia Corporation Visiokatu 1 33720 Tampere, Finland | Tel: Email: Tel: Email: Tel: Email: Tel: Email: | +1 512.372.7091 mhorowitz@austin.polycom.com +1 604.737.2294 anthony@ubvideo.com +1 604.737.2294 faouzik@ubvideo.com +358718035390 antti.hallapuro@nokia.com |
| Source: | Polycom Inc, UB Video Inc. and Nokia Corporation, ITU-T Sector Members |  |  |

_____________________________

Authors’ note: The data presented in the following contribution was generated with respect to a decoder implementation consistent with JWD2.

Abstract-- In this paper, we will study the complexity of the JVT/H.26L decoder, which is an important part of the overall cost effectiveness of a JVT/H.26L-based video system. We take a systematic approach to quantifying the computational complexity of a JVT/H.26L decoder. Our method is based on determining the number of basic computational operations required by a decoder to perform each of the key decoding sub-functions. The frequency of use of the required decoding sub-functions is measured using bit streams generated with two encoders with different characteristics for different content, resolutions and bit rates. Therefore, using the measured frequencies, an estimate of the decoder computational complexity for various hardware platforms can be determined. A detailed example is provided to assist readers in deriving their own computational complexity estimates. We compare the resulting estimates to actual numbers measured for a fairly optimized decoder on the Pentium 3 platform. We then use the actual numbers to evaluate the dependence of the computational complexity of the major decoder sub-functions on encoder characteristics, content, format and bit rate. Finally, we compare a JVT/H.26L-compliant decoder to a decoder that is compliant with the H.263 standard, which is currently dominant in many video applications. Our results indicate that the JVT/H.26L decoder may be 2-3:1 higher in terms of computational complexity than an H.263 baseline decoder.

JVT/H.26L/JVT Decoder Complexity Analysis

Michael HOROWITZ, Anthony JOCH, Faouzi KOSSENTINI, and Antti HALLAPURO

Index Terms—Complexity, Decoder, JVT/H.26L/JVT, Video

# INTRODUCTION

THE ITU-T/ISO draft JVT/H.26L video coding standard promises improved coding efficiency over existing video coding standards. Although its coding algorithms are based on the same block-based motion compensation and transform-based spatial coding framework of current standards, JVT/H.26L provides higher coding efficiency through added features and functionality. However, such features and functionality also entail additional complexity in encoding and decoding. The complexity of the coding algorithms directly affects the cost effectiveness of developing a JVT/H.26L-based video solution.

In this paper, we will study the complexity of the JVT/H.26L decoder, which we describe in the next section. Unlike encoder implementations, which can vary by orders of magnitude in terms of complexity (yet similar coding efficiency can be attained), decoder implementations usually entail the same level of complexity. This is because the behavior of the decoder is fully specified by the standard that it supports and the characteristics (e.g. bit rate, resolution) of the coded content that it must process. Therefore, it will be easier and arguably more useful to study and analyze the general complexity of the JVT/H.26L decoder.

The objective of this paper is two-fold. First, we will develop and validate a novel methodology for estimating decoder computational complexity. The methodology will allow the reader, using data presented in this paper, to derive computational complexity estimates for a variety of operating conditions and hardware platforms. Second, we will study the relationship between decoder computational complexity and encoder characteristics, source content, format and bit rate.

To estimate the overall complexity of a JVT/H.26L decoder implementation, it is important to understand that there are at least three major contributors to complexity: 1) Algorithmic complexity, 2) Storage and 3) Computational complexity. Both the algorithmic complexity and storage determine to a large part the cost of hardware implementation, mainly because they affect the size of the circuits used in the implementation. The computational complexity determines mainly the execution speed of the algorithms on the hardware system. In this work, the complexity of the JVT/H.26L decoder is studied and analyzed in the context of a software implementation on general and DSP/media processors.

In a software implementation, algorithmic complexity usually translates onto additional computational complexity. However, while storage and computational complexity can be traded for each other, a minimum amount of storage is often required for a reasonably efficient software implementation. Knowledge of such minimum storage requirements for an efficient software implementation of a JVT/H.26L decoder is important, particularly in the case of DSP/media processors. The storage requirements of a JVT/H.26L decoder are presented in Section III. In the rest of the paper, we focus on the evaluation of the computational complexity of a JVT/H.26L decoder software implementation.

To derive an accurate estimate of the computational complexity of a JVT/H.26L decoder, we should first address the memory bandwidth. Memory bandwidth is loosely defined as the bandwidth that is required to move data from memory to the processor and back. A viable software architecture will strike a balance between the time spent operating on data and the time spent loading/storing the same data. A typical JVT/H.26L video decoder requires large memory bandwidth and computational resources, making this balancing act all the more difficult. Consider an example implementation of an algorithm in which a simple set of operations require a total of 20 machine cycles to process a set of data on a particular hardware platform. Further, assume that loading of that data from memory requires 50 cycles.  It is clear that, in this example, the implementation is memory I/O bound. In other words, the implementation will spend more time loading data than performing the operations. In this case, an estimate of the number of operations will not provide an accurate estimate of the overall computational complexity of this implementation. Optimization techniques such as pre-loading data to a data cache, an intermediate memory location that provides very fast data access to the processor, would alleviate the memory I/O bottleneck in this example. In this work, we will assume that the software implementation of the JVT/H.26L video decoder has been designed to achieve the best possible balance between memory bandwidth and operations, and so only the number of cycles required for the operations will be used to develop the lower-bound estimates for the computational complexity.

We take a systematic approach to quantifying the computational complexity of a JVT/H.26L decoder. The basis of our method is to determine the number of basic operations required by a decoder to perform each of the key decoding sub-functions. The number of times that these sub-functions must be performed is measured using bit streams generated with two encoders with different characteristics for different content, resolutions and bit rates. By mapping these computational requirements to the processing capabilities of several hardware platforms, an estimate of the decoder’s computational complexity can be determined for each platform. Our methodology is described in Section IV. In Section V, we will compare our computational complexity estimates to empirical results generated for an optimized decoder on the Pentium 3 (P3) platform. In Section VI, we will study (using empirical results) the behavior of the JVT/H.26L decoder as a function of content, resolution and bit rate. Section VII presents results that quantify the difference in computational complexity between a decoder compliant with the next-generation JVT/H.26L standard, and a decoder compliant with the H.263 standard, which is currently dominant in video conferencing applications. Conclusions are presented in Section VIII.

# JVT/H.26L DECODER: OVERVIEW

As with all popular video standards, the operation of a compliant JVT/H.26L decoder is fully specified by the standard. The decoding process consists of interpreting the coded symbols of a compliant bit stream and processing this data according to the standard specification in order to generate a reconstructed video sequence. JVT/H.26L defines a hybrid block-based video codec, similar to earlier standards like MPEG-2 and H.263. Such codecs combine inter-picture prediction to exploit temporal redundancy with transform-based coding of the prediction error to exploit the video spatial redundancy. Thus, the decoding process consists generally of two primary paths: the generation of the predicted video blocks and the decoding of the coded residual blocks. The pixel values of the output blocks resulting from these two paths are summed and clipped to the valid range for pixel data to form the reconstructed blocks. While it is based on the same hybrid-coding framework, JVT/H.26L features a number of significant components that distinguish this new standard from its predecessors. This section provides a description of the data flow and processes of a JVT/H.26L decoder. For a detailed description of the standard, see another paper in this special issue [BSW].

A generalized block diagram of a JVT/H.26L decoder is given in Figure 1. The JVT/H.26L decoder shares several common features with all other hybrid video decoders. This includes generally the buffering of the input bitstream and entropy decoding, motion compensated prediction, inverse scanning, quantization and transformation, and the addition of the prediction and the transform coded residual video blocks.

The incoming video bitstream is stored in a buffer at the input to the decoder. The first stage in the decoding process is the parsing and decoding of the entropy coded bitstream symbols that are stored in the buffer. In JVT/H.26L, two different entropy coding modes are supported at this time: a simple universal variable length coding (UVLC) method, which makes use of a single code-table for all syntax elements; and a more complex and efficient context-adaptive binary arithmetic coding (CABAC) method. The complexity of the CABAC method is derived from the need to continually update a large set of context models throughout the decoding process, and the arithmetic decoding of symbols. The UVLC method can be implemented with a relatively small number of operations, requiring parsing of each symbol and table lookups. For the analysis presented in this work, only the UVLC method will be addressed.

The various syntax elements in the bitstream are de-multiplexed for use in different processes within the decoder. High-level syntax elements include temporal information for each frame, frame coding types and frame dimensions. JVT/H.26L coding, as with earlier standards, is based primarily on macroblocks consisting of 16x16 luminance-pixel blocks and 2 8x8 chrominance pixel blocks. On the macroblock level, syntax elements include the coding mode of the macroblock, information required for forming the prediction, such as motion vectors and spatial prediction modes, and the coded information of the residual (difference) blocks, such as the coded block pattern (CBP) for each macroblock and quantized transform coefficients for each of the underlying blocks.

Depending on the coding mode of each macroblock, the predicted macroblock can be generated either temporally (inter coding) or spatially (intra coding). The prediction for an inter coded macroblock is determined by the motion vectors that are associated with that macroblock. The motion vectors indicate the position within the set of previously decoded frames from which each block of pixels will be predicted. Each inter coded macroblock can be partitioned in one of seven ways, with luminance block sizes ranging from 16x16 pixels to 4x4 pixels. Also, a special SKIP mode exists in which no motion vectors (or coded residual blocks) are transmitted and the prediction is taken from the spatially corresponding macroblock in the previous picture. Thus, 0 to 16 motion vectors can be transmitted for each inter coded macroblock. Finally, note that additional predictive modes are supported when B-pictures are employed, however, only I- and P-pictures are considered in this work.

Motion vectors are coded using either median or directional prediction, depending on the partition that is used for the luminance component of the macroblock. For each motion vector, a predicted block must be computed by the decoder and then arranged with other blocks to form the predicted macroblock. Motion vectors in JVT/H.26L are specified generally with quarter-pixel accuracy (an optional mode allows for the use of eighth-pixel accurate motion vectors, but this mode is not addressed in this work). Interpolation of the reference video frames is necessary to determine the predicted macroblock using sub-pixel accurate motion vectors. The complexity of the required interpolation filter varies as follows. To generate a predicted macroblock using half-pixel accurate motion vectors, an interpolation filter that is based on a 6-tap windowed sinc function is employed. In the case of prediction using quarter-pixel accurate motion vectors, filtering consists simply of averaging two integer- or half-pixel values (i.e., two pixels from the interpolated reference frame that is used to generate the half-pixel accurate motion vectors), although one of every 12 quarter-pixel values (a.k.a. the “funny position”) is replaced by the average the four surrounding integer-pixel values, providing more low-pass filtering than the remaining positions. A bilinear filter is used to interpolate the chrominance frames when sub-pixel motion vectors are used to predict the underlying chrominance blocks.

JVT/H.26L also supports the use of multiple (previous for P-pictures) reference frames for prediction. Selection of a particular reference frame is made on a macroblock basis. This feature can improve both coding efficiency and error resilience. However, this feature also requires that the decoder buffer store several previously decoded and reconstructed frames, rather than just the most recent reconstructed frame, increasing substantially the decoder’s memory requirements.

Two different modes are supported in intra coding of macroblocks. In the 4x4 Intra mode, each 4x4 block within a macroblock can use a different prediction mode. There are 9 possible modes, one DC and 8 directional prediction modes. The complexity of the prediction mode varies, with the DC, vertical and horizontal modes being the least complex, and the diagonal modes being the most complex. In the 16x16 Intra mode, which is generally used in smooth areas, there are 4 modes available: DC, vertical, horizontal and planar, with the latter being the most complex.  The prediction of intra-coded blocks is always based on neighboring pixel values that have already been decoded and reconstructed.

The decoding of a residual (difference) macroblock requires that a number of inverse transforms be performed, along with associated inverse scanning and quantization operations. The encoding of a difference macroblock is based primarily on the transformation of 4x4 blocks of both the luminance and chrominance pixels, although in some circumstances, a second-level transform must be performed on the DC coefficients of a group of 4x4 blocks. More specifically, a special 2x2 transform is applied to the 4 DC coefficients of the blocks of the chrominance pixels. For macroblocks that are being coded in the 16x16 Intra mode, an additional scan and transform are applied to the DC values of each of the 16 4x4 luminance blocks of a macroblock.

The inverse transforms that are required for each macroblock are determined based on the coding mode and the coded block pattern (CBP) of the macroblock. The input data are the run-level codes that are parsed by the entropy decoder. These are put into their correct order based on the run values through the inverse scanning process and then the levels, which represent quantized transform coefficients, are inverse quantized via multiplication by a scaling factor. Finally, the necessary integer-specified inverse transform is performed on the inverse quantized coefficients. The inverse transformed result for each macroblock is added to the predicted macroblock and stored in the reconstructed frame buffer.

In the final stage of the decoding process, a JVT/H.26L decoder must apply the normative de-blocking filtering process, which reduces blocking artifacts that are introduced by the coding process. Since the standard specifies that the filter be applied within the motion compensation loop, any compliant decoder must perform this filtering exactly. The filtering is based on the 4x4 block edges of both luminance and chrominance components. The type of filter used, the length of the filter and its strength are dependent on several coding parameters as well as picture content on both sides of each edge. A stronger filter is used if either side of the edge is a macroblock boundary where one or both sides of the edge are intra coded. The length of the filtering is also determined by the pixel values over the edge, which determine the so-called “activity parameters”. These parameters determine whether 0, 1, or 2 pixels on either side of the edge are modified by the standard filter.

# JVT/H.26L DECODER: STORAGE REQUIREMENTS

The storage required by a JVT/H.26L decoder can be divided into four memory classes:

- Memory that is needed for the whole frame: This memory includes reconstruction frame memory and reference frame memory. Assuming a 4:2:0 picture format with 8 bits per color component, one pixel takes 1.5 bytes.

- Memory that is needed only for one line of macroblocks: This memory includes tables that are used for loop filtering and intra prediction. The loop filter and intra predictor need some values from upper macroblocks so the whole line of values has to be stored.

- Memory that is needed for a macroblock: This memory includes temporary buffers that are used for storing transform coefficients, prediction values and pixel values. It is possible to reduce this memory to less than 1 kilobyte by copying prediction and reconstruction values directly to the frame buffer.

- Memory that is needed for constant data: This includes VLD tables, intra prediction probability tables and some other small constant tables.

Table 1 presents a summary of the storage requirements of a JVT/H.26L decoder. In the table, w is width of the picture, h is the height of the picture and n is the number of reference frames. As expected, frames buffers dominate the storage requirements, particularly for high-resolution video (95% for QCIF, 98% for CIF). It is also clear from the table that less than 0.5 megabytes (1 megabyte for 3 reference frames) is required even when CIF resolution is used. This can be easily afforded by most general purpose processors and most DSP/media processors.

# JVT/H.26L DECODER COMPUTATIONAL COMPLEXITY: THEORETICAL ANALYSIS

As mentioned in the previous section, JVT/H.26L contains a number of optional features that can have a significant effect on decoder computational complexity. The features used in our analysis represent the current understanding of the features that will compose the baseline profile of JVT/H.26L. This profile would contain the simplest set of features and is most likely to be used in the first generation of JVT/H.26L-based video products, particularly those that require real-time encoding. The feature set includes UVLC entropy coding rather than CABAC, quarter-pixel accurate motion compensation, I- and P-pictures only and as many as five previous reference frames would be supported for motion compensated prediction.

To develop both our theoretical and experimental results, we have generated bitstreams using two JVT/H.26L encoders with differing characteristics. The use of two different encoders helps to establish the amount of variation that might be experienced in decoder complexity as a function of the encoder’s characteristics. The first encoder is the public JM 1.9 encoder, which performs an exhaustive search of all possible motion vectors and coding modes in order to attain the best possible rate-distortion performance. The second encoder is a real-time JVT/H.26L encoder developed by UB Video Inc. This encoder differs from the JM encoder in that it implements sub-optimal motion search and mode decision algorithms in order to reduce the encoder’s computational complexity and thereby enable real-time encoding. The UB Video encoder is highly optimized algorithmically, yet its objective and subjective performance levels are close to those of the JM encoder [VCEG-O42].

To generate the bitstreams, we have used a set of 3 QCIF- and 3 CIF-resolution video sequences. The QCIF video sequences are Container and Foreman at 10 Hz, and Silent Voice at 15 Hz. The CIF video sequences are Paris at 15 Hz, and Foreman and Mobile & Calendar at 30 Hz. Each video sequence was encoded by each of the two encoders at 3 different bit rates. The selected rates represent typical video data rates used in video conferencing systems. Neither of the two encoders employed a rate control method, and so the target rates were approximated by choosing, in each case, a constant quantization parameter that resulted in an average bit rate nearest the target rate.

Estimating computational complexity of a video decoder is a tedious and time-consuming task.  Besides its dependence on factors such as the source video content, format, frame rate, data rate, and type of encoder, this task depends on the intended use of the estimate. For hardware design, often a worst-case estimate is warranted, other times an average-case estimate is of interest, still other times the worst-likely-case estimate is considered.  Next, we describe a methodology and provide tools in the form of data tables that together will be used to derive JVT/H.26L video decoder computational estimates over a wide variety of operational scenarios.

## Table Descriptions

In this subsection, we describe the contents of the data tables that lie at the heart of the complexity analysis methodology. As detailed later, the Decoder Sub-function, Operation Count and Operation Cost tables may be used to estimate the computational complexity of the JVT/H.26L video decoder.

### Decoder Sub-Function Tables

The Decoder Sub-function tables contain data that describe the frequency of use of the decoder sub-functions. Such tables provide a means of evaluating the effects of the content, bit rate and encoder characteristics on the decoder computational complexity. Although we have generated several tables containing frequencies as a function of content, resolution, bit rate and type of encoder, we only present Table 2, which contains frequencies corresponding to the video sequences Mobile and Calendar encoded with a fixed quantization parameter (QP) of 21, and Forman encoded with a QP of 28, respectively.

### Operation Count Table

The Operation Count table contains a detailed description of the number of operations required to perform different decoder sub-functions. Each operation in the table is listed in a separate column. In a given decoder implementation, there are two basic types of operations. One type is strongly dependent on the hardware platform and varies widely with implementation. For example, on some hardware platforms, function calls are computationally expensive. Consequently, a good software implementation may replace frequently used calls with the source code of the referenced functions. On other platforms, this strategy may be inappropriate. Other examples are operations related to loop overhead, arithmetic related to computing memory load and store locations and special-case codes for boundary conditions. The other type of operations includes those operations that every JVT/H.26L compliant video decoder must perform. Examples are the computation of the loop filter output values and the 6-tap half-pel and bilinear quarter-pel interpolation values. The Operation Count table accounts for the operations in this second type only.

The Operation Count table (which can be obtained from http://spmg.ece.ubc.ca) is very long, and it can be very difficult to understand. Therefore, we only present in Table 3 a summary of the number of required basic operations for the three main sub-functions of the decoder: 1) Inverse transform and reconstruction, 2) Interpolation and 3) Loop filtering. These sub-functions account for 70-80% of the decoder computational complexity. Note that there is a close (not one-to-one) correspondence of the rows in the operation count table to the rows in the sub-function tables to facilitate the process of combining data to compute a complexity estimate.

### Operation Cost Table

The Operation Cost table (Table 4) contains data that describes the number of execution sub-units capable of performing each of the basic operations found in the Operation Count table for a variety of hardware platforms. It is apparent from the data in the table that most platforms have several execution sub-units operating in parallel to enhance computational efficiency.

## Analysis Methodology

In this subsection, we describe a method using the tables described above to derive lower-bound computational complexity estimates for JVT/H.26L decoder implementations on a variety of hardware platforms. The method consists of two basic steps. In the first step, we compute the number of cycles required to execute a particular sub-function on a chosen hardware platform. This intermediate result may be derived by mapping the sub-function operations onto the hardware, applying SIMD parallelism and distributing the remaining operations as efficiently as possible among the execution sub-units on the hardware. In the second step, the cycle count estimate is derived by multiplying the result from the first step by the frequency that the sub-function was used. The sub-function frequency data is available in the Sub-Function Tables (see Table 2 for an example).

We illustrate the estimation of the computational complexity in detail with an example in which the number of cycles required to execute the 4x4 inverse transform and reconstruction is estimated for the TriMedia TM-1300 for the video sequence Mobile and Calendar encoded with a QP of 21. There are two cases to consider when analyzing the 4x4 inverse transform and reconstruction. The first is the inverse transform and reconstruction, and the second is the reconstruction only case where no inverse transform is necessary (i.e. no non-zero coefficients were decoded).  An estimate for the number of cycles required in each case is derived below. The estimates are then combined with the sub-function frequency data to yield the computational complexity estimate.

Case 1: 4x4 Inverse Transform and Reconstruction

The operations used in the 4x4 inverse transform and reconstruction are mapped onto the TriMedia hardware architecture using data extracted from the Operation Count (Table 3) and Operation Cost (Table 4) tables as shown in Table 5. Next, the parallelism provided by the SIMD feature of the architecture is taken into account. For the inverse transform sub-function, no SIMD parallelism is exploited. Therefore, the cycle count is estimated by distributing the required operations among the execution sub-units so as to minimize the total number of cycles. More specifically, the minimum number of cycles, C, is computed as follows:

C = max [(A+S+ST+L)/5, (A+S+ST)/5, (A+S+L)/5, (A+L+ST)/5, (A+ST+L)/4, (A+S)/5, (A+ST)/5, (A+L)/5, (S+ST)/4, (S+L)/4, (ST+L)/2, A/5, S/2, ST/2, L/2]

= (A+S+ST+L) / 5  = 38.4 cycles,								   (1)

where A, S, ST and L represent the number of adds, shifts, stores and loads, respectively.  Rounding to the next higher integer, the 4x4 inverse transform and reconstruction requires 39 cycles on the TriMedia TM-1300 architecture. For completeness, we show all operation combinations in Equation (1), however, many may have been easily eliminated. For example, (A+S+ST+L)/5 seems to be always greater than or equal to  (A+S+ST)/5, and so the latter may be safely ignored.

Case 2: 4x4 Reconstruction Only

As in Case 1, the operations are mapped onto the hardware as shown in Table 6.  Also in this case, SIMD parallelism is not applicable. Therefore, the minimum number of cycles, C, is computed as follows:

C = max [(ST+L)/2, ST/2, L/2] = (ST+L)/2 = 16 cycles.

After estimating the number of cycles required to execute the sub-function, the overall computational cost is derived from the sub-function frequency data provided in the Sub-Function table. More specifically, from Table 2 we see that an average of 42165.7 4x4 inverse transforms are required per second to decode Mobile and Calendar encoded using a QP of 21. Using the cycle estimate derived in the first step, the computational cost due to the 4x4 inverse transform and reconstruction is 42165.7 * 39 = 1644462.3 cycles per second. Likewise, noting that, on average, 242954.3 blocks per second require reconstruction only, it follows that the computation burden for reconstruction only is 242954.3*16 = 3887268.8 cycles per second. The estimated computational cost is computed by summing the cycle estimates for the two cases. That is, the estimated cost of the 4x4 inverse transform and reconstruction for Mobile at a QP of 21 is 1644462.3 + 3887268.8 = 5531731.1 TM-1300 cycles per second.

Finally, we note that it is possible to compute alternative lower-bound computational complexity estimates using variations on the method described above.  For example, to derive a benchmark for the computational complexity, it is necessary to identify the most computationally complex decoding mode for a particular hardware platform using the Operation Count and Operation Cost tables.  Once identified, a lower bound for a benchmark computational complexity estimate may be derived under the assumption that every macroblock is to be decoded in that mode.

# JVT/H.26L DECODER COMPUTATIONAL COMPLEXITY: COMPARATIVE ANALYSIS

In the previous section, two factors lead to the lower-bound estimates, or more precisely the under estimation of the computational complexity. First, recall that the Operation Count table contains data for only those operations that are fundamental to the decoding process. Overhead operations such as loop and flow control and boundary condition handling are not included. Second, we assume that the software is designed so that the overhead due to instruction cache misses is negligible, hardware register counts are not exceeded and operation latency is hidden. Therefore, it is usual practice to multiply an estimate derived using similar assumptions by a factor of 2-3 to achieve a realistic computational complexity target for highly optimized machine specific code. For optimized code that is not machine specific (i.e. “C” only code), factors of 4-5 are more typical. In fact, we will show in this section through a couple of examples and through a summary for the Pentium 3 that the theoretical results are approximately 5:1 away from the experimental results.

Let’s first re-visit the same example used in the previous section, except that we now develop estimates for the Pentium 3 (P3) platform. More specifically, let’s estimate the number of cycles required to execute the 4x4 inverse transformation and reconstruction for the P3 for the video sequence Mobile and Calendar encoded with a QP of 21. The same two cases are considered next.

Case 1: 4x4 Inverse Transform and Reconstruction

The operations used in the 4x4 inverse transform and reconstruction are mapped onto the P3, as shown in Table 7. Next, the parallelism provided by the SIMD feature of the architecture is taken into account. Therefore, the minimum cycle count, C, is computed as follows:

C = max [(A+S+2*ST+L)/3, (A+S)/2, A/2, S, ST, L]

= (A+S+2*ST+L)/3 = 69.3 cycles,

where A, S, ST and L represent the number of adds, shifts, stores and loads, respectively. Therefore, the cost of the 4x4 inverse transform + reconstruction is 69.3 cycles on the P3.

Case 2: 4x4 Reconstruction Only

As in the first case, the operations are mapped onto the Pentium hardware as shown in Table 8. Assuming load/store parallelism is not exploited, C can be computed as follows:

C = max [(2*ST+L)/3,ST, L] = 16 cycles.

Next, from the Sub-Function table, we see that 42165.7 total 4x4 transforms per second, or 28266.6 luminance and 13899.1 chrominance 4x4 transforms per second, are required on average to decode Mobile and Calendar encoded using a QP of 21. Using the cycle estimate derived in the first step, the computational cost due to the 4x4 inverse transform and reconstruction is 42165.7 * 69.3 = 2922083.01 cycles per second. Similarly, noting 242954.3 4x4 luminance and chrominance blocks per second need to be reconstructed only to decode Mobile and Calendar encoded using a QP of 21, it follows that the computational cost for reconstruction only is 242954.3*16 = 3887268.8 cycles per second. Therefore, the estimated cost of the 4x4 inverse transform and reconstruction for Mobile at a QP of 21 is 2922083.01 + 3887268.8 = 6809351.81 Pentium 3 cycles per second.

To see how the above result compares with profile results generate using Intel’s VTune performance analyzer, we first note that 600 million cycles per second * (30 frames per second) / (55 frames per second) = 327.27 million cycles per second are needed to decode one second of Mobile at QP=21. The VTune analysis shows that 10.73% of the decoder computations are used for transforms and reconstruction for this sequence, and 10.73% of 327.27 million cycles gives 35.11 million cycles per second for this process. Therefore, the theoretical results are off by a factor of (35.11 million) / (6809352) = 5.16.

As a second example, let’s estimate the overall computational complexity that is required to decode Forman encoded using a QP of 28. From the Sub-Function table, we see that 4509.8 total 4x4 transforms per second, or 2591.8 luminance and 1918.0 chrominance 4x4 transforms per second, are required to decode Forman. Using the cycle estimate derived in the first step, the computational cost due to the 4x4 inverse transform and reconstruction is 4509.9 * 69.3 = 312536.07 cycles per second. Similarly, noting that (30 * 396 * 24) – 4509.8 = 280610.2 4x4 luminance and chrominance blocks per second need to be reconstructed only to decode Forman encoded using a QP of 28, it follows that the computational cost for reconstruction only is 280610.2 * 16 = 4489763.2 cycles per second. Therefore, the estimated cost of the 4x4 inverse transform and reconstruction for Forman at a QP of 28 is 312536.07 + 4489763.2 = 4802299.27 P3 cycles per second.

To see how the above result compares with the VTune results, we first note that 600M cycles per second * (30 frames per second) / (86 frames per second) = 209.30 million cycles per second are needed to decode one second of Foreman at QP=28. Inverse transforms and reconstruction compose 13.7% of the decoder complexity, or 29.0 million cycles per second, based on VTune results. This is a factor of 6.04 larger than the theoretically derived result of 4.8 million cycles per second.

The above two examples indicate that, as expected, the theoretical estimates are approximately 5:1 lower than the experimental measurements. On average, across all sources and data rates, we found the theoretical estimates to be 4.2 times lower than the measured values. Table 9 indeed confirms this result. The same table, which displays the (theoretical/experimental) estimate ratios for several sequences, resolutions and bit rates, also demonstrate that our theoretical analysis can be used effectively to predict accurately the computational complexity of a JVT/H.26L decoder implementation on a specific general-purpose platform.

# JVT/H.26L DECODER COMPUTATIONAL COMPLEXITY: EXPERIMENTAL ANALYSIS

In this section, we investigate the relationship between the video decoder’s computational complexity and the source content, format, bit rate and encoder characteristics. Our experimental analysis reveals that the computational complexity of the decoder and the important sub-functions that form the decoder operation is primarily dependent upon the bit rate of the coded bitstream. The characteristics of the source content is also found to affect decoder complexity, but to a lesser degree. However, a significant observation from our experiments is that the characteristics of the source encoder do not have a significant impact on decoder complexity for the two encoders that were tested in this study. We next present an analysis for each of the most computationally intensive sub-functions of the decoder: 1) Loop filtering, 2) Interpolation and 3) Inverse transform and reconstruction. The analysis is based on the VTune profile data generated using the UB Video codec.

## Loop Filtering

The complexity of the loop filter can be separated into two parts: the computation of the strength for each 4-pixel edge and the actual filtering. Since the computation of the strength is generally performed in the same way for every macroblock, the time required for this operation remains relatively constant per macroblock over various types of content and bit rates. At lower bit rates, the complexity of this operation is slightly reduced, since some of the strength computations can be skipped when there are a larger number of macroblocks coded in the SKIP mode. The presence of INTRA-coded macroblocks can also reduce the complexity of the strength measurement, since block edges that neighbor INTRA-coded blocks have a constant strength, regardless of other coding parameters. However, the amount of INTRA coding is limited in our test set, and so this is not a significant factor.

The complexity of the filtering process varies significantly with the content characteristics, the bit rate, and also the quantization parameter. Block edges that have been copied directly or simply motion compensated from a previous frame, without having different motion vectors or non-zero residual coded blocks are not filtered, since blocking artifacts are not generated by such coding modes. Therefore, sequences coded at low bit rates that generally contain a large number of zero-valued residual blocks require less filtering operations. The decision to filter each row or column of a block edge is also strongly dependent on content and the quantization parameter. Sequences that are high in spatial detail (Mobile & Calendar, for example) require less filtering since true edges are not filtered in order to preserve detail. When the quantization parameter at a block edge is small, the maximum pixel intensity difference across the edge that can be the result of a coding artifact, and therefore not a true edge, is also small. This characteristic of the loop filter has the tendency to reduce the time spent filtering when the quantization parameter is small. In fact, for a quantization parameter with a value that is less than 8, no filtering takes place. Still, these content-dependent decisions require a number of calculations to be performed on each row or column of pixels to determine whether to filter or not.

## Interpolation

Interpolation of both luminance and chrominance samples is generally performed for each macroblock that is coded in the INTER mode (i.e. not SKIP or INTRA coded). Thus, we expect that the average time required for interpolation in the decoder is a direct factor of the number of INTER-coded macroblocks. Our experimental results meet this expectation, as illustrated in Figure 1, which plots the percentage of INTER-coded macroblocks against the average number of cycles per macroblock spent performing interpolation of luminance and chrominance pixels for each sequence in the test set. The relationship is approximately linear. INTER macroblocks are used most frequent when the content exhibits temporal redundancy so that INTRA coding is inefficient, and the SKIP mode is also not suitable for coding the content with the desired fidelity, determined in our experiments with the fixed quantization parameter. We can also observe from this figure that the complexity of chrominance interpolation is generally just less than half that of luminance. This approximate relationship is expected, since there are half as many chrominance samples as there are luminance sample in the input data. While fewer basic operations are required for the chrominance interpolation, there is also a larger proportion of setup and looping overhead for chrominance, since the basis for interpolation is sub-sampled by a factor of 2 in each direction.

## Inverse Transforms and Reconstruction

The complexity of the inverse transform and reconstruction is directly affected by the number of blocks and macroblocks that contain non-zero coefficients. While macroblocks coded in the SKIP mode or INTER mode with no coefficients require only simple and regular copying of data from the reference frame or predicted macroblock to the current frame, inverse transforms, addition of the coded residual and clipping must be performed to reconstruct macroblocks with non-zero coefficients. Thus, the number of inverse transforms is a key factor in determining the complexity of this decoder sub-function. Again, the bit rate is the most important factor that determines the number of non-zero blocks that require inverse transforms, with larger numbers occurring at higher bit rates.

## Intra Prediction

Intra prediction is not a large factor in our tests, since the generated bit streams only used a single I-picture per sequence, there was no Intra MB refresh, and there are no scene changes in the input content. Thus, although it is not addressed by our results, the complexity of the intra prediction is still important. Such complexity is largely a factor of the application, and how much intra coding it requires, either for random access or error resilience, and (of course) content.

# COMPUTATIONAL COMPLEXITY: JVT/H.26L VERSUS H.263 BASELINE

The Baseline profile of ITU-T Recommendation H.263 is the most widely implemented standard for use in real-time video communications. Since decoder cost is often the most critical factor in the cost effectiveness of deploying a JVT/H.26L-based video solution, a comparison of the decoding cost of these two standards is valuable. As discussed previously in this paper, the storage requirements of a JVT/H.26L decoder are dominated by frame buffers, the number of which is determined by the number of reference frames supported. In this section, we describe an experiment that we have performed to estimate the computational complexity of JVT/H.26L decoding compared to that of H.263 Baseline.

Our comparison is based on two similarly optimized “C” only decoder implementations running on a 600 MHz P3 PC. The encoders also used similar motion estimation and mode decision processes while generating bit streams compliant with their respective standards. The JVT/H.26L encoder used the same feature set as in our previous experiments, as described in Section IV. Three CIF-resolution sequences were encoded by both encoders at three different bit rates each, using different constant quantization parameters to control the rate. Decoding speed was measured for each coded sequence and bit rate. These results, along with the speed ratio between the two decoders for each sequence and matching bit rate, are presented in Table 10.

For all 9 coded bit streams, the ratio of decoding speed between H.263 Baseline and JVT/H.26L lies in the range from 2.1 to 2.9, with an average of 2.4.  Therefore, our conclusion is that the computational complexity of JVT/H.26L decoding is 2 to 3 times that of H.263 Baseline decoding for the same sequence encoded at the same bit rate. We also note that at the same bit rate, JVT/H.26L has a clear quality advantage, and if the goal in an application was to achieve similar visual quality, the JVT/H.26L bit streams could have at a bit rate that is 35 - 50% lower than the H.263 baseline bit streams [WSJKS, CSVT]. Since lowering the bit rate reduces the decoding complexity, as discussed in the previous section, the ratio of complexity between H.263 and JVT would also be reduced in this scenario.

# CONCLUSIONS

In this paper, we study the complexity of the JVT/H.26L decoder using both theoretical and experimental methods. We derive our theoretical estimates using bit streams generated with two encoders with different characteristics for different content, format and bit rates. Together with hardware-specific cycle counts for each basic operation (e.g., add, shift) as well as measured numbers of computational operations for each sub-function (e.g., interpolation), the resulting frequencies can be used to estimate the JVT/H.26L decoder complexity for various hardware platforms. The resulting estimates are on average 4.2:1 lower than measured values, and this is expected since we ignore major bottlenecks (computational overhead, memory transfer, etc.) in our theoretical analysis. We also study the behavior of the JVT/H.26L decoder as a function of encoder characteristics, content, resolution and bit rate. Moreover, we demonstrate, through the use of fairly optimized decoders on the P3 platform, that the JVT/H.26L decoder is 2-3:1 higher in terms of complexity than an H.263 baseline decoder. Finally, we note that, at the time of the writing of this paper, JVT/H.26L was still an evolving standard, and the final decoder may differ from the decoder analyzed in this paper.

| Buffer name | Formula | QCIF w=176, h=144,n=1 | CIF w=176,h=144,n=1 |
| --- | --- | --- | --- |
| Reconstruction frame | 1.5*w*h | 38016 | 152064 |
| Reference frames | n*1.5*w*h | 38016 | 152064 |
| Reference indices | w/16 | 11 | 22 |
| Motion vectors | w/16*64 | 704 | 1408 |
| Intra prediction modes | w/16*16 | 176 | 352 |
| CBP values | w/16*4 | 44 | 88 |
| MB types | w/16 | 11 | 22 |
| MB temp data | ~2048 | 2048 | 2048 |
| Constants | ~1024 | 1024 | 1024 |
| TOTAL | (n+1)*1.5*w*h + w/16*84 + 3072 | 80050 | 309092 |

Table 1. Storage requirements of the JVT/H.26L decoder.

|  | Foreman, QP = 28 | Mobile, QP = 21 |
| --- | --- | --- |
| SKIP MBs | 4322.8 | 276.7 |
| INTER MBs | 7222.3 | 11545.7 |
| INTRA MBs | 334.9 | 57.6 |
| 4x4 Luma Transforms | 2591.8 | 28266.6 |
| 4x4 Chroma Transforms | 1918 | 13899.1 |
| "b" 4x4 blocks | 30669.4 | 37481.2 |
| "c" 4x4 blocks | 8713.8 | 6134.2 |
| "d" + "e" 4x4 blocks | 24141 | 59797.4 |
| "f" + "g" 4x4 blocks | 8945.4 | 22023.6 |
| "h" 4x4 blocks | 13634.2 | 13634.2 |
| "i" 4x4 blocks | 6307.2 | 6307.2 |
| Edges with Non-zero Strength | 343608.4 | 746964.8 |
| Filter Type Decision (L+C) | 322985.2 | 332858.8 |
| Strong Luma Filter | 5770.7 | 269.1 |
| Strong Chroma Filter | 3882 | 493.5 |
| Standard Filter (P0 + Q0) | 313332.5 | 332096.2 |
| Filter P1 or Q1 | 304341 | 160411.8 |

Table 2. Sub-Function Operation Counts (Data specified as average number of operations per second).

|  | Add8 | Add16 | Mult8 | Mult16 | MAC8 | MAC16 | Branch | Shift | Load | Store | AND | OR | ? : |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4x4 Inverse Transform + Reconstruction | 0 | 96 | 0 | 0 | 0 | 0 | 0 | 32 | 48 | 16 | 0 | 0 | 0 |
| 4x4 Null Inv. Transform (reconstruct only) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 16 | 16 | 0 | 0 | 0 |
| Luma Interpolation (ops per sample) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| "b" (MAC support) | 1 | 2 | 0 | 0 | 1 | 0 | 0 | 1 | 7 | 1 | 0 | 0 | 0 |
| "b" | 0 | 6 | 0 | 2 | 0 | 0 | 0 | 1 | 7 | 1 | 0 | 0 | 0 |
| "c" (MAC support) | 2 | 5 | 0 | 0 | 2 | 3 | 0 | 1 | 20 | 3 | 0 | 0 | 0 |
| "c" | 0 | 17 | 0 | 6 | 0 | 0 | 0 | 1 | 20 | 3 | 0 | 0 | 0 |
| "d", "e" (MAC support) | 2 | 2 | 0 | 0 | 1 | 0 | 0 | 2 | 9 | 2 | 0 | 0 | 0 |
| "d", "e" | 1 | 6 | 0 | 2 | 0 | 0 | 0 | 2 | 9 | 2 | 0 | 0 | 0 |
| "f", "g" (MAC support) | 3 | 6 | 0 | 0 | 2 | 3 | 0 | 3 | 24 | 5 | 0 | 0 | 0 |
| "f", "g" | 1 | 17 | 0 | 6 | 0 | 0 | 0 | 3 | 24 | 5 | 0 | 0 | 0 |
| "h" (MAC support) | 1 | 2 | 0 | 0 | 0 | 12 | 0 | 3 | 16 | 3 | 0 | 0 | 0 |
| "h" | 1 | 12 | 0 | 4 | 0 | 0 | 0 | 3 | 16 | 3 | 0 | 0 | 0 |
| "i" - "funny position" | 4 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 2 | 0 | 0 | 0 |
| Chroma Interpolation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| One Chroma Sample (MAC support) | 0 | 1 | 0 | 0 | 0 | 2 | 0 | 1 | 4 | 1 | 0 | 0 | 0 |
| One Chroma Sample | 0 | 4 | 0 | 4 | 0 | 0 | 0 | 1 | 4 | 1 | 0 | 0 | 0 |
| Deblocking Filter (ops per block edge) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Strength Measure Inter | 0 | 5 | 0 | 0 | 0 | 0 | 1 | 8 | 11 | 1 | 4 | 3 | 7 |
| Strength Measure Intra | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 0 | 0 | 0 |
| Edges With Non-Zero Strength | 7 | 0 | 0 | 0 | 0 | 0 | 1.25 | 0.25 | 5.25 | 0 | 2.5 | 0 | 3 |
| Filtered Edges |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Filter Type Decision (luma+chroma) | 2 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 2 | 0 | 3 | 0 | 2 |
| Strong Luma Filter (MAC support) | 4 | 10 | 0 | 0 | 6 | 0 | 0 | 6 | 2 | 6 | 0 | 0 | 0 |
| Strong Luma Filter | 28 | 0 | 2 | 0 | 0 | 0 | 0 | 12 | 2 | 6 | 0 | 0 | 0 |
| Strong Chroma Filter (MAC support) | 0 | 8 | 0 | 0 | 4 | 0 | 0 | 4 | 2 | 4 | 0 | 0 | 0 |
| Strong Chroma Filter | 20 | 0 | 0 | 0 | 0 | 0 | 0 | 8 | 2 | 4 | 0 | 0 | 0 |
| Standard Filter p0 + q0 | 8 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 0 | 0 | 2 |
| Standard Filter p1, q1 (each) | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 0 | 1 | 0 | 0 | 2 |

Table 3. Summary of Sub-Function Operation Count table.

|  | Add8 | Add16 | Add32 | Mult8 | Mult16 | MAC8 | MAC16 | Branch | Shift | Load | Store | OR | AND | ? : |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TriMedia | 1,3 | 1,3 | 1,2,3,4,5 | 2,3 | 2,3 | 2,3 | 2,3 | 2,3,4 | 1,2 | 4,5 | 4,5 | 1,2,3,4,5 | 1,2,3,4,5 | 1,2,3,4,5 |
| Pentium III | 1,2 | 1,2 | 1,2 | N/A | 1 | N/A | 1 | 2 | 2 | 3 | 4 | 1,2 | 1,2 | 1,2 |
| C64 | 1,2 | 1,2,5,6,7,8 | 1,2,5,6,7,8 | 3,4 | 3,4 | N/A | N/A | 5,6 | 3,4,5,6 | 7,8 | 7,8 | 1,2,5,6,7,8 | 1,2,5,6,7,8 | 1,2 |
| MAP-CA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4. Operation Cost for Several Hardware Platforms.

| Operation | Operation Count | Sub-units Capable of Performing Operation |
| --- | --- | --- |
| Add | 96 | 1, 2, 3, 4, 5 |
| Shift | 32 | 2, 3 |
| Store | 16 | 4, 5 |
| Load | 48 | 4, 5 |

Table 5. Mapping of 4x4 Inverse Transform operations onto the TriMedia TM-1300.

| Operation | Operation Count | Sub-units Capable of Performing Operation |
| --- | --- | --- |
| Store | 16 | 4, 5 |
| Load | 16 | 4, 5 |

Table 6. Mapping of 4x4 Reconstruction Only operation onto TriMedia TM-1300.

| Operation | Operation Count | Sub-units Capable of Performing Operation |
| --- | --- | --- |
| Add | 96 | 1, 2 |
| Shift | 32 | 2 |
| Store | 16 | 4 |
| Load | 48 | 3 |

Table 7. Mapping of 4x4 Inverse Transform operations onto the P3.

| Operation | Operation Count | Sub-units Capable of Performing Operation |
| --- | --- | --- |
| Store | 16 | 4 |
| Load | 16 | 3 |

Table 8. Mapping of 4x4 Reconstruction Only operation onto the P3.

|  | Container (QCIF, QP=15) | Foreman (QCIF, QP=24) | Silent (QCIF, QP=22) | Paris (CIF, QP=17) | Foreman (CIF, QP=28) | Mobile (CIF, QP=21) |
| --- | --- | --- | --- | --- | --- | --- |
| Loop filtering | 3.66 | 3.24 | 3.77 | 4.12 | 3.64 | 4.10 |
| Reconstruction | 4.66 | 5.66 | 4.94 | 5.01 | 6.04 | 5.16 |
| Luma interpolation | 2.62 | 2.35 | 2.63 | 2.97 | 2.49 | 2.57 |
| Chroma interpolation | 5.38 | 4.59 | 5.24 | 5.97 | 4.88 | 5.40 |

Table 9. Experimental (VTune)/Theoretical estimate ratios for the P3.

|  | H.263 Baseline |  |  | H.26L |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP | Bit Rate (Kbps) | Decoding Speed (fps) | QP | Bit Rate (Kbps) | Decoding Speed (fps) | Speed Ratio |
| Paris, 15 Hz | 5 | 672 | 140 | 11 | 677 | 67 | 2.09 |
|  | 10 | 298 | 183 | 17 | 313 | 82 | 2.23 |
|  | 21 | 111 | 220 | 24 | 116 | 100 | 2.20 |
| Foreman, 30 Hz | 9 | 652 | 149 | 14 | 680 | 56 | 2.66 |
|  | 14 | 342 | 192 | 19 | 326 | 67 | 2.87 |
|  | 31 | 149 | 225 | 28 | 120 | 85 | 2.65 |
| Mobile & Calendar, 30 Hz | 12 | 1842 | 96 | 16 | 1723 | 44 | 2.18 |
|  | 23 | 699 | 130 | 21 | 704 | 55 | 2.36 |
|  | 31 | 443 | 155 | 26 | 307 | 65 | 2.38 |
| AVERAGE |  |  |  |  |  |  | 2.40 |

Table 10. Decoding speed ratios for H.263 Baseline and H.26L measured on 600 MHz Pentium 3.

Figure 1. Block diagram of a JVT decoder.

Figure 2. Interpolation complexity as a function of Inter MB frequency.

The TriMedia TM-1300 data bus is 32 bits wide. Consequently, if the data were packed appropriately, two sixteen-bit loads or four eight-bit stores could be performed per operation, thereby reducing the total operation count.  To simplify our methodology, we assume that for most operations the cost of packing and merging the data into the proper format to exploit SIMD loads and stores is balanced by the gains that the parallel operations would yield.

The P3 can load and store 64 bit words. Since the data consists of 8-bit pixels, we can load and store the data for 8 pixels at a time. Since such amounts to a simple data move, it would be easy to parallelize the loads and stores. However, data has to often be packed/merged to be placed into the correct format to perform parallel load/stores, reducing the gains realized by the parallelism. Since parallelization of loads and stores is beneficial sometimes and not others we have opted not to try to exploit this kind of parallelism to simplify our methodology.