| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D011 Filename: JVT-D011.doc |
| --- | --- |

| Title: | Timing and Timing Information Ad-Hoc Group Report |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Sherman Chen | Tel: Email: Tel: Email: | 1-949-585-6185 schen@broadcom.com |
| Source: | Microsoft and Broadcom |  |  |

_____________________________

## 1. Activities

Most of activities on discussion of timing related issues were through mail exchanges. Discussion were focused on the following issues:

- Temporal reference (TR).

Do we need to use Temporal Reference in the decoding of sample values?  Discussions on the reflector address at least these issues: (a) using TR to assist coding of the direct mode; (b) introducing TR for determining decoding and display timing relations; (c) using display order for determining default buffer index order (which could be done equivalently in conventional cases using picture numbers instead), (d) using display order for implicit ABP weighting selection.  There are several contributions submitted to this meeting.  The group needs to review these contributions.

- Timing related information for HRD.

Should we introduce time-stamps in the JVT video elementary stream?  Introducing timestamps in the video stream can help multiplexing/re-multiplexing for some applications.  However, these time-stamps might not be useful for other applications.  No contribution is submitted to this meeting.

Is the current coding method sufficient for supporting the film mode (3:2 pull-down)?  The film mode can be supported explicitly by the current JVT coding algorithm.  However, the description on interlaced coding is not very clear for this application.

- Fixed picture rate.

Should we introduce picture rate to support fixed-picture rate applications?  Fixed picture rate is required by many applications.  Introducing a flag in the parameter set will be helpful.

- Picture delay due to re-ordering of decoded pictures.

How can we characterize the picture delay in a JVT stream?  There are several proposals submitted to this meeting to address this issue.  The group needs to review these contributions.

## 2.  Contributions registered for this meeting

Contributions registered for this meeting, which are related to timing and timing information, include at least the following.

JVT-D044     Prop.      [Wang+]        	Temporal Reference for AFF

JVT-D055     Info.       [Jeon]		Usage of related timing information in VCL

JVT-D075     Prop.      [Kimata] 		A proposal on B-picture related issue

JVT-D111     Prop.      [Lim+]        		Post decoder buffer for HRD

JVT-D124     Prop.      [Haskell+]           	Variable-Accuracy Inter-picture timing for video

JVT-D125     Prop.      [Chen]         		An issue regarding to B-picture coding