| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D016r2 Filename: JVT-D016r2.doc |
| --- | --- |

| Title: | Complexity Reduction of Adaptive Interpolation Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lifeng Song, Gang Wei College of Electronic & Information Engineering, South China University of Technology, Guangzhou 510641, China | Tel: Email: | 86-20-87113038 slf@21cn.com ecgwei@scut.edu.cn |
| Source: | South China University of Technology |  |  |

_____________________________

- Introduction

The Adaptive Interpolation Filter (AIF) for motion compensation of fractional pixel resolution proposed by T. Wedi [1-2] improves compression efficiency especially for the video sequences with unknown and/or non-stationary statistical properties. But the complexity of this AIF encoder is at least twice as much as that of the JVT encoder. So it is unsuitable for real-time applications [3]. In order to reduce the complexity of the AIF codec, we propose to amend Wedi’s AIF software (available at http://www.tnt.uni-hannover.de/~wedi/jvt/tml900-af.zip) on motion estimation and mode decision, interpolation at the decoder, iterated interpolation at the encoder. These amendments are incorporated into the JVT JM2.0 software codec together with Wedi’s AIF and available as JVT-D016-Software.zip at the JVT FTP site. The experimental results show our AIF codec achieves up to 4.55% bit rate savings or 0.21dB gain in comparison to JM2.0 with the same memory size and the almost same running speed. So our AIF codec is suitable for real-time applications.

- DESCRIPTION

- Estimation Procedure of AIF Coefficients

This procedure presented in [2] consists of the three steps below:

- Motion vectors of 1/4 pixel resolution are estimated with the initial interpolation filter.

- The motion vectors from Step 1 are used to perform the motion compensated prediction hereinafter. Applying a numerical iterated optimization algorithm, the AIF coefficients are estimated by minimizing the energy of the motion compensated prediction error.

- The motion compensated prediction for the current frame is performed with the AIF coefficients from Step 2 and the motion vectors from Step 1.

- Motion Estimation and Mode Decision

The encoding at frame level is run once per frame for ordinary video encoders and twice per frame for the AIF encoder: the motion vectors of all the macroblocks in a frame are obtained in the first encoding, code words are generated and then output in the second encoding. Motion estimation and mode decision has to be performed to obtain block shapes and corresponding motion vectors including temporal and spatial displacement. There are two methods of motion estimation and mode decision in JVT WD-2 [4]: one is the low complexity method based on a Hadamard transform, the other one is the high complexity method (R-D optimized method) based on a Lagrangian function . The later method makes the encoding reach the best rate-distortion performance at macroblock level at the expense of high complexity, as the encoding is run 5 times per 8x8 sub-partition and 7 times per 16x16 macroblock. So both the coding efficiency and the complexity of the AIF encoder depend on the usage of the motion estimation and mode decision method.

For Wedi’s AIF encoder, the low complexity method of motion estimation and mode decision is used in the first encoding at frame level while the high complexity one is applied in the second encoding. So its complexity is at least twice as much as that of the JM2.0 encoder. For our proposed AIF encoder, the motion estimation and mode decision is performed only once per frame. The high complexity method is adopted in the first encoding to obtain macroblock types, sub-partition types, intra prediction modes, reference frame numbers, motion vectors. In the second encoding, these elements are used to finish the remained encoding at macroblock level and then code words are written into the bit stream buffer. During these two times of encoding, the different interpolation filters only cause the differences of the syntax elements about transform coefficients, including coded block pattern, delta quantization parameters and run-level-EOB symbols.

The whole CABAC bit stream must be generated so as to get the code length for the motion estimation and mode decision at every macroblock. If CABAC is selected as the entropy coding, all the code words including headers are written into the bit stream buffer even in the first encoding. The second encoding is started with the reset bit stream states and the re-initialized arithmetic coding engine. The bit stream is output only at the end of the second encoding. If UVLC is selected, code words don’t need writing into the buffer in the first encoding.

- Interpolation at the Decoder

The whole reference pictures with the expanding size by a factor of 4x4 are obtained for the motion compensation of 1/4 pixel resolution at Wedi’s AIF decoder. So the frame memory size for one reference frame is 4x4 times as large as that of the JM2.0 decoder. Different from the interpolation at the encoder where the completely interpolated images must be generated for motion search, the direct interpolation at the decoder is required for the part of fractional pixel positions indicated by the known motion vectors. For example, to generate a prediction block of 4x4 with the motion vector of 1/4 pixel horizontal displacement and 3/4 pixel vertical displacement, firstly the horizontal interpolation is performed at the 1/4 pixel positions of the reference frame. The intermediate results are saved in a 2 dimensional array of 9x4 (6-tap filter) or 11x4 (8-tap filter). Secondly the vertical interpolation is performed at the 3/4 pixel positions. The interpolation procedure of our proposed AIF decoder is somehow similar to that of the JM2.0 decoder without generation of any completely interpolated image. Both the computational complexity and the frame memory sizes of our proposed AIF decoder are the same as that of the JM2.0 decoder and much lower than that of Wedi’s AIF decoder. When multiple reference frames are used, the AIF coefficients for each reference frame are saved in the corresponding array. These arrays are arranged in the way similar to that for the frame memory.

- Iterated Interpolation at the Encoder

The above situation also exists at the AIF encoder. The interpolation for motion compensation of 1/4 pixel resolution included in the numerical iterated optimization process (the downhill simplex method), which is Step 2, is replaced by the interpolation at the decoder as described in 2.3. Because this interpolation is iterated, huge computational complexity is saved. The interpolated image of the previous frame is generated completely with the obtained optimized filter coefficients and also saved into the frame memory of the AIF encoder at the end of the numerical iterated optimization process for the estimation of AIF coefficients.

- Experimental Results

| Test Sequence | QP | JM2.0 |  | AIF |  |  | Distance between two R-D curves |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bit rate (Kbps) | Y PSNR (dB) | Bit rate (Kbps) | AIF bit rate (bps) | Y PSNR (dB) |  |
| Foreman QCIF 300 frames 30 fps | 28 | 29.51 | 28.31 | 30.40 | 1473.60 | 28.71 | 1.66% bit rate savings or 0.09dB gain |
|  | 24 | 46.73 | 30.99 | 47.39 | 1395.80 | 31.12 |  |
|  | 20 | 77.27 | 33.43 | 77.04 | 1347.80 | 33.50 |  |
|  | 16 | 133.02 | 36.01 | 131.74 | 1290.20 | 36.04 |  |
| News QCIF 300 frames 30 fps | 28 | 15.17 | 28.04 | 16.93 | 1589.80 | 28.08 | -4.08% bit rate savings or -0.22dB gain |
|  | 24 | 25.56 | 30.77 | 27.22 | 1583.80 | 30.84 |  |
|  | 20 | 42.82 | 33.68 | 44.11 | 1614.90 | 33.72 |  |
|  | 16 | 70.79 | 36.71 | 71.81 | 1478.20 | 36.76 |  |
| Mobile & Calendar CIF 300 frames, 30 fps | 28 | 194.00 | 24.91 | 188.65 | 1426.20 | 25.06 | 4.55% bit rate savings or 0.21dB gain |
|  | 24 | 351.88 | 27.80 | 341.79 | 1353.10 | 27.89 |  |
|  | 20 | 718.58 | 30.82 | 695.77 | 1313.40 | 30.88 |  |
|  | 16 | 1406.65 | 34.08 | 1369.20 | 1298.60 | 34.11 |  |
| Paris CIF 300 frames 30 fps | 28 | 74.54 | 26.71 | 76.21 | 1511.60 | 26.77 | -0.09% bit rate savings or 0.00dB gain |
|  | 24 | 138.86 | 29.56 | 139.82 | 1440.60 | 29.57 |  |
|  | 20 | 256.45 | 32.48 | 256.37 | 1380.10 | 32.49 |  |
|  | 16 | 449.56 | 35.46 | 448.23 | 1256.00 | 35.49 |  |
| Tempete CIF 260 frames 30 fps | 28 | 143.56 | 26.49 | 141.08 | 1342.73 | 26.64 | 2.71% bit rate savings or 0.11dB gain |
|  | 24 | 266.42 | 29.18 | 261.93 | 1227.35 | 29.23 |  |
|  | 20 | 538.75 | 32.00 | 528.79 | 1177.73 | 32.02 |  |
|  | 16 | 1077.52 | 35.06 | 1060.75 | 1155.58 | 35.06 |  |

Table 1 Experimental Results

We implement the above amendments combined with Wedi’s AIF on the JVT JM2.0 software. The obtained AIF codec only contains 6-tap adaptive interpolation filters. The experiment is carried out using 5 test sequences under the common test conditions. The encoding settings for all the tests are 5 reference frames, CABAC, 1/4 pixel motion vector resolution, R-D optimized motion estimation and mode decision, IPP…P picture type sequence, no B-frame, no slice, H.26L VCL bit stream without encapsulation. The distance between the two R-D curves of JM2.0 and our AIF for each sequence is calculated by the software avsnr4. The results are in Table 1.

- Conclusions

In comparison to JM2.0, our AIF codec achieves 0%~4.55% bit rate savings or 0.0dB~0.21dB gain on CIF sequences, where the maximum is 4.55% or 0.21dB (Mobile & Calendar) and the minimum is –0.09% or 0.0dB(Paris). The running of our AIF codec on a 700MHz Pentium Ⅲ show the added complexity of our AIF encoder is justified by the moderate increase in compression efficiency, and our AIF codec is suitable for real-time applications. The adaptive filters show advantages in the specific situations that the statistical properties of signals are unknown and/or non-stationary. We hope that JVT will re-evaluate Wedi’s adaptive interpolation filter with our software.

- References

- T. Wedi, "Adaptive Interpolation Filter for Motion Compensated Hybrid Video Coding," Picture Coding Symposium (PCS 2001), Seoul, Korea, April 2001.
(downloadable via ftp://ftp.tnt.uni-hannover.de/pub/papers/2001/PCS2001-TW.pdf )

- T. Wedi, “Adaptive Interpolation Filter for H.26L”, ITU-T SG16/Q6, doc. VCEG-N28, Santa Barbara, CA, USA, Sep. 2001.

- A. Fuldseth, “Evaluation of Adaptive Interpolation Filter for Real-time Applications”, JVT of ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, doc. JVT-C057, Fairfax, Virginia, USA, May 2002.

- JVT Working Draft Number 2 Revision 8, JVT of ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, doc. JVT-B118r8, Apr. 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | South China University of Technology |  |  |
| Mailing address | ## College of Electronic & Information Engineering, South China University of Technology, Guangzhou 510641, P. R. China |  |  |
| Country | P. R. China |  |  |
| Contact person | Lifeng Song |  |  |
| Telephone | 86-20-87113038 |  |  |
| Fax | 86-20-87113541 |  |  |
| Email | slf@21cn.com |  |  |
| Place and date of submission | Guangzhou, China, July 8, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Complexity Reduction of Adaptive Interpolation Filter |  |
| Contribution number |  | JVT-D016 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: