| Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 |  | Document JVT-D018 File: JVT-D018.doc Generated: 2002-07-10 |
| --- | --- | --- |
| Title: | Multi-Picture Handling |  |
| Status: | Input to JVT |  |
| Contact: | Thomas Wiegand Heinrich Hertz Institute (HHI), Einsteinufer 37, D-10587 Berlin, Germany Tel: +49 - 30 - 31002 617, Fax: +49 - 030 - 392 72 00, wiegand@hhi.de |  |
| Purpose: | Report of Ad-Hoc Group Activity |  |

Summary

The current text of the CD does not properly handle multiple reference pictures. Especially the frame/field handling concept is not clear. This document provides a concept for multi-picture handling.

# 1. Picture Number Assignment

A frame consists of two fields, one being the top field and the other one being the bottom field. The first field is defined as being the field-structured picture sent earlier then the second field. Within a frame, the first field is the one as indicated by the frame picture_struct (top field first or bottom field first). To each field, a picture number is assigned that is incremented by one against the previous sent and stored field.

Each field-structure picture has a second codeword assigned to it to indicate whether it is the first field of a frame (use code_number = 0) or the second one. If it is the second field of a frame, it contains the value of the negative picture number difference between its picture number and the picture number of the first field (use code_number = negative difference) (this is how we identify frames in the video signal, otherwise all first fields are treated as dangling fields). Each field-structure picture has a third codeword assigned to it to indicate whether it is a top or bottom field. Each field is assigned a disposable flag.

To each frame, a codeword (picture_struct) is assigned to indicate whether the top field comes first or the bottom field comes first. The frame receives a picture number that is incremented by two against the previous stored field with the first field being assigned a picture number that is incremented by one and the second field being assigned a picture number that is incremented by two. Each field of the frame is assigned a disposable flag with that having to transmit 2 disposable flags.

# 2. Reference picture reordering

## 2.1 Coding of progressive pictures

In the case of coding of progressive pictures, each picture is identified as a separate reference picture with a unique index via its picture number. The default index order is as specified in the CD for P frames as well as for B frames. Reordering is conducted on a picture basis.

## 2.2 Coding of interlace pictures

### 2.2.1 Field-based coding

Identical to coding of progressive pictures with pictures being fields. Please note the parity dependent default index ordering in the CD applies.

### 2.2.2 Frame-based coding

Identical to coding of progressive pictures with pictures being frames. Only complete frames are utilized and with that dangling fields cannot be referenced.

# 3. Picture buffering

## 3.1 Buffering of progressive pictures

The buffering process as specified in the CD applies.

## 3.2 Buffering of interlace pictures

### 3.2.1 Buffering of fields

Identical to buffering of progressive pictures with pictures being fields.

### 3.2.2 Buffering of frames

The memory_management_control_operation loop is transmitted for each of the two fields of the frame with buffering for each field as being identical to buffering of fields.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | HHI |  |  |
| Mailing address |  |  |  |
| Country |  |  |  |
| Contact person |  |  |  |
| Telephone |  |  |  |
| Fax |  |  |  |
| Email |  |  |  |
| Place and date of submission |  |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  | Multi-Picture Handling |  |
| Contribution number |  | JVT-D018 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| x | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: