| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 March, 2002 | Document: JVT-D026 Filename: JVT-D026.doc |
| --- | --- |

| Title: | Intra Prediction with Simplified Prediction Modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Incorporated 12500 TI Boulevard, MS 8649 Dallas, Texas 75243, USA | Tel: Email: | +1 214 480-3816 zhou@ti.com |
| Source: | Texas Instruments |  |  |

# Introduction

The current 4x4 Intra prediction is based on the nine prediction modes, which require 1K bytes table and 17 reference pixels from top and left neighboring blocks for prediction.  In JVT-C033 concerns were raised regarding the table size as well as the computational complexity.

Significant progress was made in the reduction of the table size. Nokia has proposed a mode coding method that reduces the table size from 981 bytes to roughly 40 bytes. To reduce the implementation complexity, an extension could be made to this proposal by simply replacing the current nine 17-reference pixels based prediction modes with the nine  9-reference pixels based prediction modes described in this document. This simplification can cut the computational complexity by about half without quality loss vs. the current intra prediction approach. On the top of that, it enables the concurrent processing of the 4x4 intra prediction as illustrated below, in which the 4x4 block pairs marked with the same color in a 16x16 luma block can be processed concurrently.

- Prediction Mode Coding Simplifications

The prediction mode coding method proposed by Nokia is used without any changes. The description below is copied from the Nokia document.

The same label is assigned to different prediction modes of blocks A and B before using them to specify the prediction mode for block C.  Diagonal modes 3, 5 and 8 (modes 4, 6 and 7) are grouped together and labeled as 3 (4). After this grouping each of blocks A and B can have one of the 5 modes labeled as 0, 1, 2, 3 and 4. Therefore instead of 9x9 possible combination of prediction modes of A and B there are only 5x5 such combinations.

To each combination only one most probable mode is assigned. The assignment is given in Table 1.

Table 1:  Prediction mode as a function of ordering signalled in the bitstream

| B/A | outside | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- |
| outside | 0 | 0 | 1 |  |  |  |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 |
| 1 |  | 0 | 1 | 0 | 1 | 1 |
| 2 | 2 | 2 | 1 | 2 | 2 | 2 |
| 3 |  | 0 | 1 | 2 | 3 | 4 |
| 4 |  | 0 | 1 | 2 | 4 | 4 |

To the decoder first information is send whether the most probable mode is used. This information is coded jointly for 2 4x4 blocks as given in table below.

| Codeword | Block 1 | Block 2 |
| --- | --- | --- |
| 0 | x | x |
| 10 |  |  |
| 110 | x |  |
| 111 |  | x |

By X we denote that the most probable mode should be used. If for the block C the most probable mode m is not selected, a code number from 0 to 7 is sent to indicate which of the remaining 8 modes should be used. The code number is sent as 3 bit codeword, which is its binary representation. When the code number n is received the used prediction mode is equal to:

n, if n<m

n+1, otherwise.

- Prediction Mode Definition Simplifications

We propose to use the following simlipfied prediction modes to replace the current ones. Only 9 reference pixels from neigboring blocks are used in prediction. Note that those modes defined below already existed in the previous H.26L versions.

For the luminance signal, there are 9 intra prediction modes labeled 0 to 8.  Mode 0 is ‘DC-prediction’ (see below).  The other modes represent directions of predictions as indicated below.

1

2

3

4

6

7

5

8

- FIGURE 1

Mode 0: DC prediction

Generally all pixels are predicted by (A+B+C+D+E+F+G+H)//8.  If four of the pixels are outside the picture, the average of the remaining four is used for prediction.  If all 8 pixels are outside the picture the prediction for all pixels in the block is 128.  A block may therefore always be predicted in this mode.

Mode 1: Vertical prediction

If A,B,C,D are inside the picture,  a,e,i,m are predicted by A,  b,f,j,n by B etc.

Mode 2: Horizontal prediction

If E,F,G,H are inside the picture,  a,b,c,d are predicted by E,  e,f,g,h by F etc.

Mode 3: Diagonal prediction

This mode is used only if all A,B,C,D,E,F,G,H,I are inside the picture.  This is a 'diagonal' prediction.

m is predicted by: 			(H+2G+F)//4

i,n are predicted by 			(G+2F+E)//4

e,j,o are predicted by 		(F+2E+I)//4

a,f,k,p are predicted by 		(E+2I+A)//4

b,g,l are predicted by 		(I+2A+B)//4

c,h are predicted by 		(A+2B+C)//4

d is predicted by 			(B+2C+D)//4

Mode 4: Vertical/Diagonal Prediction

This mode is used only if all A,B,C,D are inside the picture.

a is predicted by: 			(A+B)/2

e is predicted by 			B

b,i are predicted by 		(B+C)/2

f,m are predicted by 		C

c,j are predicted by 		(C+D)/2

d,g,h,k,l,n,o,p are predicted by 	D

Mode 5: Horizontal/Diagonal prediction

This mode is used only if all E,F,G,H are inside the picture.

a is predicted by: 			(E+F)/2

b is predicted by 			F

c,e are predicted by 		(F+G)/2

f,d are predicted by 		G

i,g are predicted by 		(G+H)/2

h,j,k,l,m,n,o,p are predicted by 	H

Mode 6: Vertical-Left prediction

This mode is used only if all A,B,C,D,E,F,G,H,I are inside the picture.  This is a 'diagonal' prediction.

a, j are predicted by 	(I + A + 1)>>1;

b, k are predicted by 	 (A + B + 1)>>1;

c, l are predicted by 	 (B + C + 1)>>1;

d  is predicted by 	 (C + D + 1)>>1;

e, n are predicted by 	 (E + 2*I + A + 2)>>2;

f, o are predicted by 	 (I + 2*A + B + 2)>>2;

g, p are predicted by 	 (A + 2*B + C + 2)>>2;

h  is predicted by 	 (B + 2*C + D + 2)>>2;

I is predicted by	 	 (I + 2*E + F + 2)>>2;

m is predicted by 	(E + 2*F + G + 2)>>2;

Mode 7: Vertical-Right prediction

This mode is used only if all A,B,C,D,E,F,G,H,I are inside the picture.  This is a 'diagonal' prediction.

a  is predicted by 	 ( 2*(A+B+G)+F+H+4)>>3;

b, i are predicted by 	 (B + C + 1)>>1;

c, j are predicted by 	 (C + D + 1)>>1;

d, k are predicted by 	 (D + E + 1)>>1;

l  is predicted by 	 (E + F + 1)>>1;

e  is predicted by 	 (2*(B+H) + A + C + G + H + 4)>>3;

f, m are predicted by 	 (B + 2*C + D + 2)>>2;

g, n are predicted by 	 (C + 2*D + E + 2)>>2;

h, o are predicted by 	 (D + 2*E + F + 2)>>2;

p is  predicted by 	 (E + 2*F + G + 2)>>2;

Mode 8: Horizontal-Down prediction

This mode is used only if all A,B,C,D,E,F,G,H,I are inside the picture.  This is a 'diagonal' prediction.

a, g are predicted by 	 (I + E + 1)>>1;

b, h are predicted by 	 (E + 2*I + A + 2)>>2;

c is predicted by 	 (I + 2*A + B +2)>>2;

d is predicted by 	 (A + 2*B + C +2)>>2;

e, k are predicted by 	 (E + F + 1)>>1;

f, l are predicted by 	 (I + 2*E + F + 2)>>2;

i, o are predicted by 	 (F + G + 1)>>1;

j, p are predicted by 	 (E + 2*F + G + 2)>>2;

m is predicted by 	 (G + H + 1)>>1;

n is predicted by 	 (F + 2*G + H +2)>>2;

- Experimental Results

The proposed prediction modes were implemented in the JM 2.0 based software provided by Nokia. Two sets of test were carried out. One is the comparison between JM2.0 intra prediction and the Nokia proposal (JM2.0 vs. Nokia), the other is the comparison between JM2.0 and the Nokia proposal with the proposed simplified prediction modes (JM2.0 vs. Nokia + TI). The simulation results are summarized in Table 2 and in Error: Reference source not found in terms of Bjontegaard Delta bitrate savings in % with respect to JM2.0. The common test conditions from VCEG-N81 were followed with the exception that all frames were encoded as Intra. The detailed results are provided in Appendix A.

Table 2: Performance Comparison (Bjontegaard Delta bitrate savings in %) between JM2.0 and Nokia + TI proposal .QP = 0, 4, 8, 12 is used for low QP and QP = 16, 20, 24, 28 for high QP.

| QP | container | foreman | mobile | news | paris | silent | tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -0.141% | -0.431% | -0.377% | -0.195% | 0.035% | 0.271% | -0.245% | -0.158% |
| High | 0.157% | -1.887% | -0.271% | 0.684% | 0.897% | 0.766% | 0.356% | -0.095% |

Table 3: Performance Comparison (Bjontegaard Delta bitrate savings in %) between JM2.0 and Nokia  proposal .QP = 0, 4, 8, 12 is used for low QP and QP = 16, 20, 24, 28 for high QP.

| QP | container | foreman | mobile | news | paris | silent | tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -0.292% | -0.977% | -0.691% | -0.157% | -0.231% | -1.135% | -0.753% | -0.585% |
| High | 0.175% | -2.348% | -1.210% | 0.570% | 0.328% | -2.197% | -0.787% | -0.795% |

Table 2 revealed that the simplified intra prediction approach (i.e. the Nokia prediction mode coding approach PLUS the simplified prediction modes) described in this document (Nokia + TI) performs as well as the current JM2.0 intra prediction approach. The coding loss resulted from the prediction mode simplification is negligible. The comparison between Table 3 and Table 2 showed that the proposed simplified prediction modes led to about 0.42% and 0.70% coding loss for low and high QPs, respectively, when compared to 17-reference pixels based ones in JM2.0. Therefore, it is recommended to use the simplified prediction modes for the complexity reduction purpose.

Appendix A:

Table 4: Performance Comparison (Bjontegaard Delta bitrate savings in %, dB) between JM2.0 and Nokia + TI proposal .QP = 0, 4, 8, 12

| Sequence | frames | QP | PSNRY JM2.0 | Bitrate JM2.0 | PSNRY Nokia+TI | Bitrate Nokia+TI | Average Bit saving | Average dB gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 0 | 50.07 | 1138.06 | 50.10 | 1136.42 | -0.141% | 0.013 |
|  |  | 4 | 46.57 | 826.87 | 46.60 | 826.98 |  |  |
|  |  | 8 | 43.19 | 590.99 | 43.19 | 591.33 |  |  |
|  |  | 12 | 40.12 | 409.95 | 40.13 | 410.58 |  |  |
| foreman | 100 | 0 | 49.90 | 1189.39 | 49.92 | 1186.62 | -0.431% | 0.039 |
|  |  | 4 | 46.33 | 848.88 | 46.35 | 846.22 |  |  |
|  |  | 8 | 42.94 | 591.76 | 42.94 | 589.85 |  |  |
|  |  | 12 | 39.80 | 399.48 | 39.80 | 397.55 |  |  |
| mobile | 300 | 0 | 49.88 | 26032.21 | 49.90 | 25989.66 | -0.377% | 0.051 |
|  |  | 4 | 46.21 | 20155.63 | 46.23 | 20120.34 |  |  |
|  |  | 8 | 42.55 | 15386.69 | 42.58 | 15356.02 |  |  |
|  |  | 12 | 38.90 | 11422.86 | 38.93 | 11398.23 |  |  |
| news | 100 | 0 | 50.24 | 1088.66 | 50.25 | 1085.66 | -0.195% | 0.020 |
|  |  | 4 | 47.13 | 803.36 | 47.15 | 802.86 |  |  |
|  |  | 8 | 43.98 | 591.93 | 44.01 | 592.41 |  |  |
|  |  | 12 | 40.86 | 425.95 | 40.85 | 426.43 |  |  |
| paris | 150 | 0 | 49.81 | 8520.52 | 49.83 | 8523.02 | 0.035% | -0.004 |
|  |  | 4 | 46.35 | 6283.71 | 46.37 | 6293.45 |  |  |
|  |  | 8 | 43.08 | 4612.66 | 43.09 | 4623.23 |  |  |
|  |  | 12 | 39.86 | 3327.46 | 39.88 | 3339.31 |  |  |
| silent | 150 | 0 | 50.04 | 1956.31 | 50.07 | 1961.76 | 0.271% | -0.027 |
|  |  | 4 | 46.44 | 1422.12 | 46.47 | 1428.44 |  |  |
|  |  | 8 | 42.93 | 1001.72 | 42.96 | 1008.38 |  |  |
|  |  | 12 | 39.57 | 676.55 | 39.58 | 681.88 |  |  |
| tempete | 260 | 0 | 49.94 | 18904.73 | 49.96 | 18884.67 | -0.245% | 0.027 |
|  |  | 4 | 46.44 | 14070.98 | 46.46 | 14060.20 |  |  |
|  |  | 8 | 42.99 | 10363.59 | 43.01 | 10356.55 |  |  |
|  |  | 12 | 39.53 | 7411.42 | 39.54 | 7405.11 |  |  |
| Average |  |  |  |  |  |  | -0.158%, | 0.015 |

Table 5: Performance Comparison (Bjontegaard Delta bitrate savings in %, dB) between JM2.0 and Nokia + TI proposal .QP = 16, 20, 24, 28

| Sequence | frames | QP | PSNRY JM2.0 | Bitrate JM2.0 | PSNRY Nokia+TI | Bitrate Nokia+TI | Average Bit saving | Average dB gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 16 | 37.10 | 273.47 | 37.11 | 273.81 | 0.157% | -0.012 |
|  |  | 20 | 34.35 | 183.75 | 34.38 | 184.57 |  |  |
|  |  | 24 | 31.55 | 123.39 | 31.54 | 123.75 |  |  |
|  |  | 28 | 28.81 | 77.41 | 28.84 | 77.76 |  |  |
| foreman | 100 | 16 | 36.83 | 263.61 | 36.83 | 261.62 | -1.887% | 0.120 |
|  |  | 20 | 34.10 | 177.39 | 34.12 | 175.57 |  |  |
|  |  | 24 | 31.35 | 112.86 | 31.41 | 111.44 |  |  |
|  |  | 28 | 28.72 | 73.42 | 28.82 | 71.95 |  |  |
| mobile | 300 | 16 | 35.22 | 8116.53 | 35.24 | 8097.18 | -0.271% | 0.021 |
|  |  | 20 | 31.77 | 5745.35 | 31.79 | 5735.20 |  |  |
|  |  | 24 | 28.45 | 3967.82 | 28.46 | 3967.71 |  |  |
|  |  | 28 | 25.38 | 2495.96 | 25.39 | 2497.25 |  |  |
| news | 100 | 16 | 37.66 | 293.55 | 37.67 | 295.18 | 0.684% | -0.047 |
|  |  | 20 | 34.63 | 207.94 | 34.62 | 210.24 |  |  |
|  |  | 24 | 31.52 | 143.80 | 31.59 | 145.09 |  |  |
|  |  | 28 | 28.61 | 92.98 | 28.62 | 94.42 |  |  |
| paris | 150 | 16 | 36.58 | 2312.58 | 36.60 | 2326.26 | 0.897% | -0.067 |
|  |  | 20 | 33.54 | 1610.90 | 33.56 | 1626.15 |  |  |
|  |  | 24 | 30.49 | 1089.46 | 30.51 | 1103.56 |  |  |
|  |  | 28 | 27.56 | 692.59 | 27.58 | 706.83 |  |  |
| silent | 150 | 16 | 36.34 | 428.51 | 36.34 | 433.05 | 0.766% | -0.044 |
|  |  | 20 | 33.54 | 277.56 | 33.54 | 279.02 |  |  |
|  |  | 24 | 31.01 | 169.92 | 31.01 | 171.63 |  |  |
|  |  | 28 | 28.49 | 105.55 | 28.50 | 106.35 |  |  |
| tempete | 260 | 16 | 36.04 | 5047.44 | 36.06 | 5047.28 | 0.356% | -0.025 |
|  |  | 20 | 32.83 | 3421.12 | 32.85 | 3430.13 |  |  |
|  |  | 24 | 29.77 | 2209.48 | 29.79 | 2229.35 |  |  |
|  |  | 28 | 26.95 | 1330.73 | 26.97 | 1356.83 |  |  |
| Average |  |  |  |  |  |  | -0.095% | 0.005 |

Table 6: Performance Comparison (Bjontegaard Delta bitrate savings in %, dB) between JM2.0 and Nokia  proposal .QP = 0, 4, 8, 12

| Sequence | frames | QP | PSNRY JM2.0 | Bitrate JM2.0 | PSNRY Nokia | Bitrate Nokia | Average Bit saving | Average dB gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 0 | 50.07 | 1138.06 | 50.11 | 1136.21 | -0.292% | 0.028 |
|  |  | 4 | 46.57 | 826.87 | 46.61 | 826.29 |  |  |
|  |  | 8 | 43.19 | 590.99 | 43.20 | 590.85 |  |  |
|  |  | 12 | 40.12 | 409.95 | 40.13 | 410.56 |  |  |
| foreman | 100 | 0 | 49.90 | 1189.39 | 49.93 | 1182.70 | -0.977% | 0.090 |
|  |  | 4 | 46.33 | 848.88 | 46.35 | 842.66 |  |  |
|  |  | 8 | 42.94 | 591.76 | 42.95 | 586.66 |  |  |
|  |  | 12 | 39.80 | 399.48 | 39.82 | 395.41 |  |  |
| mobile | 300 | 0 | 49.88 | 26032.21 | 49.90 | 25926.05 | -0.691% | 0.092 |
|  |  | 4 | 46.21 | 20155.63 | 46.23 | 20062.91 |  |  |
|  |  | 8 | 42.55 | 15386.69 | 42.58 | 15304.28 |  |  |
|  |  | 12 | 38.90 | 11422.86 | 38.93 | 11351.94 |  |  |
| news | 100 | 0 | 50.24 | 1088.66 | 50.26 | 1085.76 | -0.157% | 0.016 |
|  |  | 4 | 47.13 | 803.36 | 47.15 | 803.72 |  |  |
|  |  | 8 | 43.98 | 591.93 | 43.99 | 592.18 |  |  |
|  |  | 12 | 40.86 | 425.95 | 40.88 | 426.14 |  |  |
| paris | 150 | 0 | 49.81 | 8520.52 | 49.83 | 8507.72 | -0.231% | 0.025 |
|  |  | 4 | 46.35 | 6283.71 | 46.37 | 6279.07 |  |  |
|  |  | 8 | 43.08 | 4612.66 | 43.10 | 4612.02 |  |  |
|  |  | 12 | 39.86 | 3327.46 | 39.88 | 3330.39 |  |  |
| silent | 150 | 0 | 50.04 | 1956.31 | 50.06 | 1944.00 | -1.135% | 0.112 |
|  |  | 4 | 46.44 | 1422.12 | 46.46 | 1410.70 |  |  |
|  |  | 8 | 42.93 | 1001.72 | 42.95 | 991.26 |  |  |
|  |  | 12 | 39.57 | 676.55 | 39.59 | 667.45 |  |  |
| tempete | 260 | 0 | 49.94 | 18904.73 | 49.97 | 18825.79 | -0.753% | 0.084 |
|  |  | 4 | 46.44 | 14070.98 | 46.47 | 14008.71 |  |  |
|  |  | 8 | 42.99 | 10363.59 | 43.02 | 10311.10 |  |  |
|  |  | 12 | 39.53 | 7411.42 | 39.56 | 7366.23 |  |  |
| Average |  |  |  |  |  |  | -0.585% | 0.057 |

Table 7: Performance Comparison (Bjontegaard Delta bitrate savings in %, dB) between JM2.0 and Nokia  proposal .QP = 16, 20, 24, 28

| Sequence | frames | QP | PSNRY JM2.0 | Bitrate JM2.0 | PSNRY Nokia | Bitrate Nokia | Average Bit saving | Average dB gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| container | 100 | 16 | 37.10 | 273.47 | 37.11 | 273.84 | 0.175% | -0.011 |
|  |  | 20 | 34.35 | 183.75 | 34.39 | 184.70 |  |  |
|  |  | 24 | 31.55 | 123.39 | 31.55 | 123.80 |  |  |
|  |  | 28 | 28.81 | 77.41 | 28.84 | 78.28 |  |  |
| foreman | 100 | 16 | 36.83 | 263.61 | 36.86 | 260.60 | -2.348% | 0.151 |
|  |  | 20 | 34.10 | 177.39 | 34.11 | 174.46 |  |  |
|  |  | 24 | 31.35 | 112.86 | 31.38 | 110.35 |  |  |
|  |  | 28 | 28.72 | 73.42 | 28.77 | 71.22 |  |  |
| mobile | 300 | 16 | 35.22 | 8116.53 | 35.25 | 8055.91 | -1.210% | 0.101 |
|  |  | 20 | 31.77 | 5745.35 | 31.80 | 5697.53 |  |  |
|  |  | 24 | 28.45 | 3967.82 | 28.47 | 3927.70 |  |  |
|  |  | 28 | 25.38 | 2495.96 | 25.41 | 2472.13 |  |  |
| news | 100 | 16 | 37.66 | 293.55 | 37.66 | 294.37 | 0.570% | -0.041 |
|  |  | 20 | 34.63 | 207.94 | 34.64 | 210.05 |  |  |
|  |  | 24 | 31.52 | 143.80 | 31.55 | 144.77 |  |  |
|  |  | 28 | 28.61 | 92.98 | 28.64 | 94.10 |  |  |
| paris | 150 | 16 | 36.58 | 2312.58 | 36.60 | 2317.80 | 0.328% | -0.024 |
|  |  | 20 | 33.54 | 1610.90 | 33.57 | 1619.54 |  |  |
|  |  | 24 | 30.49 | 1089.46 | 30.51 | 1096.56 |  |  |
|  |  | 28 | 27.56 | 692.59 | 27.59 | 703.26 |  |  |
| silent | 150 | 16 | 36.34 | 428.51 | 36.34 | 420.09 | -2.197% | 0.124 |
|  |  | 20 | 33.54 | 277.56 | 33.55 | 271.54 |  |  |
|  |  | 24 | 31.01 | 169.92 | 31.01 | 166.06 |  |  |
|  |  | 28 | 28.49 | 105.55 | 28.50 | 103.88 |  |  |
| tempete | 260 | 16 | 36.04 | 5047.44 | 36.07 | 5014.28 | -0.787% | 0.055 |
|  |  | 20 | 32.83 | 3421.12 | 32.86 | 3401.18 |  |  |
|  |  | 24 | 29.77 | 2209.48 | 29.80 | 2201.31 |  |  |
|  |  | 28 | 26.95 | 1330.73 | 26.98 | 1341.55 |  |  |
| Average |  |  |  |  |  |  | -0.795% | 0.047 |

- Reference

[1]  H.26L Joint Video Team Committee Draft (JVT-CD), 3rd JVT meeting, Fairfax, VA, May 4 – May 11, 2002

[2] Recommended Simulation Conditions for H.26L, ITUT-T VCEG-N81

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Texas Instruments |  |  |
| Mailing address | 12500 TI Blvd., Dallas, TX-75243 |  |  |
| Country | USA |  |  |
| Contact person | Minhua Zhou |  |  |
| Telephone | +1 214 480 3816 |  |  |
| Fax | +1 972 671 6969 |  |  |
| Email | zhou@ti.com |  |  |
| Place and date of submission | Klangenfurt, Austria, July 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Intra prediction with simplified prediction modes |  |
| Contribution number |  | JVT-D026 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| x | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  | Nokia, RealNetwork |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: