| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D027 Filename: JVT-D027.doc |
| --- | --- |

| Title: | Intra-Prediction-Mode Ordering and Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 USA Louis Kerofsky Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: Tel: Email: | 360-817-7531 ssun@sharplabs.com 360-817-7644 lkerofsky@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

1. Introduction

In the Joint Committee Draft (CD), a Look-Up Table (LUT) of 10x10x9 is used as the 9-intra-prediction-mode ordering function (Table 8-9 in CD). The memory requirement of this LUT for low-end applications has caused concern in the Fairfax meeting (JVT-C033). Another new problem with the LUT is that the table has become incomplete because Flexible Macroblock Order (FMO) has been adopted into the CD.

In this contribution, we propose a simple intra-prediction-mode ordering function to remove the 10x10x9 table.

2. Proposed Intra-Prediction-Mode Ordering Function

The semantics of the intra-prediction modes are same as those of CD. However, the 9 intra-prediction modes shall be named according to their probability order in intra coding:

Mode 0: Horizontal prediction

Mode 1: Vertical prediction

Mode 2: DC prediction

Mode 3: Diagonal Down/Left prediction

Mode 4: Horizontal Down prediction

Mode 5: Diagonal Down/Right prediction

Mode 6: Vertical Right prediction

Mode 7: Vertical Left prediction

Mode 8: Horizontal Up prediction

The reordering is independent from prediction definition as long as the prediction directions are the same.

Figure 1. Prediction mode of block C to be established, where A and B are adjacent blocks.

As described in CD, the intra-prediction-mode order of a block C is highly correlated with prediction modes of adjacent blocks A and B. For mode ordering of block C, when an adjacent block is coded by 16x16 intra mode, the intra prediction mode (imode) of this adjacent block is viewed as “mode 2: DC prediction”; when an adjacent block is coded in inter mode, its imode is viewed as “mode 2: DC prediction” in the usual case and “outside” in the case of constraint intra update; when an adjacent block is outside of the current slice, its imode is viewed as “outside”. Based on the intra prediction modes of block A and block B, the intra-prediction-mode order for block C shall be defined as the following:

First we define pred_mode = min(imode(A), imode(B)), and define a symbol OUTSIDE = 9 (or any value larger than 8).

If (pred_mode != OUTSIDE)

Intra-prediction-mode-order(C) = {pred_mode, other modes in ascending order};

else   Intra-prediction-mode-order(C) = {0,1,2,3,4,5,6,7,8};

E.g., if imode(A) = 3 and imode(B) = 1, Intra-prediction-mode-order(C) = {1,0,2,3,4,5,6,7,8}; and    if imode(A) = OUTSIDE and imode(B) = 2, Intra-prediction-mode-order(C) = {2,0,1,3,4,5,6,7,8}.

Note that the definition here stands no matter FMO is in the profiles or not. And it is up to implementers as how to implement this intra-prediction-mode ordering function, either by direct calculation or by pre-calculated LUT (10x10x9).

3. Experiments

Experiments have been conducted based on JM2.0 using a 13-sequence data set. All frames were coded as I frames in the experiments. The results are summarized in Table 1. There is no significant impact in coding efficiency by the proposed intra-prediction-mode ordering function. The simplified intra-prediction-mode ordering function has a 0.17% bit penalty on average.

4. Conclusion

We have proposed an intra-prediction-mode ordering function for the 9-intra-prediction-mode coding. The proposed method enables a way to reduce the memory requirement and therefore allows more flexible implementations. No syntax change is required by this method. It is proposed that JVT adopt the new method into the standard.

- Proposed changes to CD

- 1. Section 8.5.4.1 shall be modified and Table 8-9 shall be removed.

#### 8.5.4.1	Coding of Intra 4x4 prediction modes

Since each of the 4x4 luma blocks is assigned a prediction mode, this will require a considerable number of bits if coded independently.  The chosen prediction of a block is highly correlated with the prediction modes of adjacent blocks.  This is illustrated in Figure 8-3.  When the prediction modes of A and B are known (including the case that A or B or both are outside the slice) an ordering of the most probable, next most probable etc. of C is given.  When an adjacent block is coded by 16x16 intra mode, prediction mode is “mode 2: DC_prediction”; when it is coded in inter mode, prediction mode is “mode 2: DC_prediction” in the usual case and “outside” in the case of constrained intra update. This ordering is defined as the following.

First let us define pred_imode = min(imode(A), imode(B)), and a symbol OUTSIDE=9 (or any value larger than 8).

If (pred_imode != OUTSIDE)

imode-Prob(C) = {pred_imode, other modes in ascending order};

else

imode-Prob(C) = {0,1,2,3,4,5,6,7,8};

For example, if imode(A) = 3 and imode(B) = 1, imode-Prob(C) = {1,0,2,3,4,5,6,7,8}; and

if imode(A) = OUTSIDE and imode(B) = 2, imode-Prob(C) = {2,0,1,3,4,5,6,7,8}.

Note that it is up to implementers as how to implement this intra-prediction-mode ordering function, either by direct calculation or by pre-calculated a LUT (10x10x9).

For more efficient coding, information on intra prediction of two 4x4 luma blocks is coded in one codeword (Prob0 and Prob1 in Table 9-5).  The order of the resulting eight codewords is indicated in Figure 8-3.

2. Section 9.3.1 shall be modified to reorder the intra modes. There is no change to the definition of each mode.

## 9.3.1	Intra Prediction for 4x4 mode for luma

For the luma signal, there are nine intra prediction modes labeled 0 to 8.  Mode 2 is ‘DC-prediction’ (see below).  The other modes represent directions of predictions as indicated below.

### 9.3.1.1	Mode 0: Horizontal prediction

If I, J, K, L are inside the slice, then

–	a, b, c, d are predicted by I,

–	e, f, g, h are predicted by J,

–	i, j, k, l are predicted by K,

–	m, n, o, p are predicted by L.

### 9.3.1.2	Mode 1: Vertical Prediction

If A, B, C, D are inside the slice, then

–	a, e, i, m are predicted by A,

–	b, f, j, n are predicted by B,

–	c, g, k, o are predicted by C,

–	d, h, l, p are predicted by D.

### 9.3.1.3	Mode 2: DC prediction

If all samples A, B, C, D, I, J, K, L, are within the slice, all samples are predicted by (A+B+C+D+I+J+K+L+4)>>3.  If A, B, C, and D are outside the slice and I, J, K, and L are not, all samples are predicted by (I+J+K+L+2)>>2.  If I, J, K, and L are outside the slice and A, B, C, and D are not, all samples are predicted by (A+B+C+D+2)>>2.  If all eight samples are outside the slice, the prediction for all samples in the block is 128.  A block may therefore always be predicted in this mode.

### 9.3.1.4	Mode 3: Diagonal Down/Left prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	a is predicted by			(A + 2B + C + I + 2J + K + 4) >> 3

–	b, e are predicted by 			(B + 2C + D + J + 2K + L + 4) >> 3

–	c, f, i are predicted by 		(C + 2D + E + K + 2L + M + 4) >> 3

–	d, g, j, m are predicted by 		(D + 2E + F + L + 2M + N + 4) >> 3

–	h, k, n are predicted by 		(E + 2F + G + M + 2N + O + 4) >> 3

–	l, o are predicted by 			(F + 2G + H + N + 2O + P + 4) >> 3

–	p is predicted by 			(G + H + O + P + 2) >> 3

### 9.3.1.5	Mode 4: Horizontal-Down prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	a, g are predicted by 			(Q + I + 1) >> 1

–	b, h are predicted by 			(I + 2Q + A+ 2) >> 2

–	c is predicted by 			(Q + 2A + B+ 2) >> 2

–	d is predicted by 			(A + 2B + C+ 2) >> 2

–	e, k are predicted by 			(I + J + 1) >> 1

–	f, l are predicted by 			(Q + 2I + J+ 2) >> 2

–	i, o are predicted by 			(J + K + 1) >> 1

–	j, p are predicted by 			(I + 2J + K+ 2) >> 2

–	m is predicted by 			(K + L + 1) >> 1

–	n is predicted by 			(J + 2K + L + 2) >> 2

### 9.3.1.6	Mode 5: Diagonal Down/Right prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	m is predicted by: 			(J + 2K + L + 2) >> 2

–	i, n are predicted by 			(I + 2J + K + 2) >> 2

–	e, j, o are predicted by 		(Q + 2I + J + 2) >> 2

–	a, f, k, p are predicted by 		(A + 2Q + I + 2) >> 2

–	b, g, l are predicted by 		(Q + 2A + B + 2) >> 2

–	c, h are predicted by 			(A + 2B + C + 2) >> 2

–	d is predicted by 			(B + 2C + D + 2) >> 2

### 9.3.1.7	Mode 6: Vertical-Right prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	a is predicted by 			(2A + 2B + J + 2K + L + 4) >> 3

–	b, i are predicted by			(B + C + 1) >> 1

–	c, j are predicted by 			(C + D + 1) >> 1

–	d, k are predicted by 			(D + E + 1) >> 1

–	l is predicted by 			(E + F + 1) >> 1

–	e is predicted by 			(A + 2B + C + K + 2L + M + 4) >> 3

–	f, m are predicted by 		(B + 2C + D + 2) >> 2

–	g, n are predicted by 			(C + 2D + E + 2) >> 2

–	h, o are predicted by 			(D + 2E + F + 2) >> 2

–	p is predicted by 			(E + 2F + G + 2) >> 2

### 9.3.1.8	Mode 7: Vertical-Left prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	a, j are predicted by 			(Q + A + 1) >> 1

–	b, k are predicted by			(A + B + 1) >> 1

–	c, l are predicted by 			(B + C + 1) >> 1

–	d is predicted by 			(C + D + 1) >> 1

–	e, n are predicted by 			(I + 2Q + A + 2) >> 2

–	f, o are predicted by 			(Q + 2A + B + 2) >> 2

–	g, p are predicted by 			(A + 2B + C + 2) >> 2

–	h is predicted by 			(B + 2C + D + 2)  >> 2

–	i is predicted by 			(Q + 2I + J + 2) >> 2

–	m is predicted by 			(I + 2J + K + 2) >> 2

### 9.3.1.9	Mode 8: Horizontal-Up prediction

This mode is used only if all A, B, C, D, I, J, K, L, Q are inside the slice.  This is a 'diagonal' prediction.

–	a is predicted by 			(B + 2C + D + 2I + 2J + 4) >> 3

–	b is predicted by 			(C + 2D + E + I + 2J + K + 4) >> 3

–	c, e are predicted by 			(D + 2E + F + 2J + 2K + 4) >> 3

–	d, f are predicted by 			(E + 2F + G + J + 2K + L + 4) >> 3

–	g, i are predicted by 			(F + 2G + H + 2K + 2L + 4) >> 3

–	h, j are predicted by 			(G + 3H + K + 3L + 4) >> 3

–	l, n are predicted by 			(L + 2M + N + 2) >> 3

–	k, m are predicted by 		(G + H + L + M + 2) >> 2

–	o is predicted by 			(M + N + 1) >> 1

–	p is predicted by 			(M + 2N + O + 2) >> 2

Table 1. Comparison of JM2.0 and the proposed intra-prediction-mode ordering

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sharp Labs of America |  |  |
| Mailing address | 5750 Pacific Rim Blvd, Camas, WA 98607 |  |  |
| Country | USA |  |  |
| Contact person | Shijun Sun |  |  |
| Telephone | 360 817 7531 |  |  |
| Fax | 360 817 8436 |  |  |
| Email | ssun@sharplabs.com |  |  |
| Place and date of submission | Klagenfurt, July 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Intra Prediction Mode Ordering and Coding |  |
| Contribution number |  | JVT-D27 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Provisional |  |
| Inventor(s)/Assignee(s) |  | Shijun Sun and Louis Kerofsky |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | To support a royalty-free baseline we follow ITU-T 2.2.1 for Baseline Profile decoder applications agreeing to a free license on condition that all other patent holders do the same. |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: