| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D028 Filename: JVT-D028.doc |
| --- | --- |

| Title: | New Results of Lossless Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shijun Sun Sharp Labs of America 5750 NW Pacific Rim Blvd Camas, WA 98607 USA | Tel: Email: | 360-817-7531 ssun@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

A lossless video coding result based on the JM syntax (JVT-C023) was reported in the Fairfax meeting. The compression ratio of the lossless coding was better than the JM1.7 lossy coding result at QP of (–10), which is equivalent to QP of 2 in the CD. This contribution provides new experimental result based on JM2.0. It is proposed that JVT adopt this lossless coding feature into the standard.

# 1. Introduction

Since the current transforms and quantization could cause data loss by rounding or shifting operation, the transform and quantization steps are skipped in order to achieve a simple lossless coding. To make this easy to fit into the current codec, the existing entropy coding tools are used to code the image residuals after Intra or Inter predictions. Therefore, no syntax change is needed. Since the lossless coding approach still uses all the possible Intra and Inter predictions, it fits naturally into I, P, and B frames. Since MIN_QP is defined as (-8) in JM2.0, QP of (-9) is used to signal the lossless coding option. When QP of a slice is (-9), the slice is to be coded losslessly, in the meanwhile, Dquant is forced to be zero for all MB’s in this slice, and there is no double scan when UVLC is used.

# 2. Experimental results

2.1 Lossless Coding

The Experiments were based on JM2.0. For the lossless coding, RDOPT is on, search range is 32, number of reference frames is 5, all MB modes are enabled. Each sequence is coded as one I frame followed by all P frames; no B frame is inserted. The loopfilter is turned off for the lossless coding. Both CABAC and UVLC were tested in the experiments.

Table 1. Compression Ratio of the Lossless Video Coding

The results are listed in Table 1. Among thirteen tested sequences, the compression ratio ranges from 2.06 to 4.68 for CABAC, from 1.24 to 3.35 for UVLC; the average is 2.80 for CABAC and 1.87 for UVLC.

2.2 Comparison with JM Lossy Coding

Lossy coding results of JM2.0 were obtained to compare with the lossless coding. The coding condition is same as stated in Section 2.1 with RDOPT, 5 reference frames, search range of 32, all MB modes, and IPPP. QP values of (-7) and (–8) were chosen for the comparison. The compression ratios are shown in Table 2. Since MIN_QP is defined as (-8) in JM2.0, lossy coding result with QP less than (-8) is not available for comparison. The compression ratio for the lossless coding is quite comparable with the JM2.0 lossy coding; and we shall expect that the JM2.0 compression ratio at QP of (-12) or even (-10) be not better than that of the lossless coding.

Table 2. Comparison of the Compression Ratio of JM2.0 coding and Lossless Coding

3. Alternative UVLC for coefficient coding

In Fairfax, a CAVLC proposal has been adopted into the CD for coefficients coding, however, it has not been implemented in JM2.0. Since the 2D level-run coding in JM2.0 using UVLC is not very efficient, we have tested separate level/run coding based on UVLC. A (level, run) pair is coded as a “level” using the same code table as MVD, followed by a “run” using the same code table as MB run. A zero “level” signals EOB. As summarized in Table 3, the lossless compression ratio increases from 1.87 (Table 1) to 2.04 on average, which means a relative 8.3% bit reduction. Since both level and run tables are existing, there is no any extra complexity increase by doing the simple 1D level/run coding. The CAVLC, even though more complex, might be able to improve the compression ratio even further.

Table 3. Compression ratio with the separate level/run coding

# Summary

We have demonstrated that a decent lossless coding performance can be achieved based on the existing JM framework by simply skipping transform and quantization steps. It is proposed that JVT adopt this feature into the standard. Since the CD has redefined the QP as [0, 51] instead of [-12, 39] in WD, zero QP could be used to signal the lossless coding.

# Proposed changes to CD

The following paragraph shall be added to section 8.3.4.

“When the value of QPY is zero, all transform and quantization processes shall be skipped. The zero QPY therefore enables a lossless video coding feature for the standard.”

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sharp Labs of America |  |  |
| Mailing address | 5750 Pacific Rim Blvd, Camas, WA 98607 |  |  |
| Country | USA |  |  |
| Contact person | Shijun Sun |  |  |
| Telephone | 360 817 7531 |  |  |
| Fax | 360 817 8436 |  |  |
| Email | ssun@sharplabs.com |  |  |
| Place and date of submission | Fairfax, May 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | ## Lossless Coding and QP Range Selection |  |
| Contribution number |  | JVT-C023 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  | Provisional |  |
| Inventor(s)/Assignee(s) |  | Shijun Sun |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | To support a royalty-free baseline we follow ITU-T 2.2.1 for Baseline Profile decoder applications agreeing to a free license on condition that all other patent holders do the same. |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: