| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D029 Filename: JVT-D029.doc |
| --- | --- |

| Title: | 4-tap filter for bi-predicted macroblocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Antti Hallapuro, Jani Lainema, Marta Karczewicz Nokia Research Center 6000 Connection Drive Irving, TX 75039, USA | Tel: Email: | +1 972 894 4045 jani.lainema@nokia.com |
| Source: | Nokia Inc. |  |  |

_____________________________

# Introduction

Bi-predictive interpolation is twice as complex as P-frame interpolation since both forward and backward predictions have to be computed. There have been proposals such as [1] to reduce interpolation complexity by using 4-tap filter, but the proposals cause some loss in quality.

In this proposal 4-tap filter is proposed only for bi-predicted macroblocks. The worst-case complexity of the decoder is significantly reduced while the quality is not affected.

# Method description

In B-frames, a macroblock can be predicted from a single frame in forward or backward direction or it can be predicted from two frames in forward and/or backward directions (bi-predicted macroblock). In the latter case, final prediction is average of the two predictions.

When the decoder sees forward or backward predicted macroblock, the normal 6-tap filtering is used possibly followed by simple linear filter as described in the current JVT specification [2]. When the decoder encounters a bi-predicted Macroblock, the decoder will switch to a selected 4-tap filter. Linear filtering is not used in this case. The following 4-tap filter coefficients are used for full-pixel, 1/4-pixel, 1/2-pixel and 3/4-pixel positions, respectively:

[ 0, 16,  0,  0], [-2, 14,  5, -1], [-2, 10, 10, -2], [-1,  5, 14, -2]

The worst-case complexity of interpolation occurs when 4x4 prediction mode is chosen and motion vector points to one of the five middle sub-pixel positions. In this case he number of multiplications when using 6-tap filter is (9*4+4*4)*6 = 312. That is, interpolation of one 4x4 block requires 9*4 horizontal filter operations and 4x4 vertical filter operations and each filter operation requires 6 multiplications (and 6 loads and 6 additions). For 4-tap filter, the number of multiplications is reduced to (7*4+4*4)*4 = 176. Thus, the complexity reduction compared to the 6-tap filter is 44%.

At first glance the encoder implementation may look problematic since two different filters are used. It seems that, for the best performance, motion estimation for bi-predicted macroblocks should be performed using sub-pixels from 4-tap filter. Simulations in the JM3.6 environment show, however, that using 6-tap filter always in motion estimation and obtaining only final prediction using 4-tap filter will not degrade performance. Therefore, no additional motion estimation is required.

# PSNR results

Attached spreadsheet JVT-D029.xls contains RD-curves for the test set and QP values of the common conditions. One B-frame per one P-frame was used. Table 1 summarizes the Bjontegaard Delta PSNR provided by the proposed method. JM 3.6 was used in the tests with RD-optimization and UVLC coding. It can be seen that the proposed method does not introduce any loss in performance.

Table 1: Performance difference (Bjontegaard Delta PSNR) of the proposed method relative to the JM 3.6. QP = 12, 16, 20, 24 is used for low QP and QP = 28, 32, 36, 40 for high QP.

| QP | News | Foreman | Container | Silent | Mobile | Paris | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | 0.0139 | -0.0225 | 0.0002 | 0.0057 | -0.0169 | 0.0108 | 0.0258 | 0.0024 |
| High | 0.0139 | 0.0789 | 0.0151 | 0.0097 | 0.0746 | 0.0281 | 0.0513 | 0.0388 |

# Conclusions

The compression efficiency of the proposed method is on average very close to the current method. The worst-case complexity of luma interpolation kernel is reduced by 44%. The reduction is achieved with negligible increase in encoder complexity. We propose that the method is adopted to the next version of the JVT draft standard.

# References

[1] Jill Boyce and Cristina Gomila, "4-Tap Motion Interpolation Filter ", document VCEG-C104, Joint Video Team (JVT) Meeting, Fairfax, VA, USA, 6-10, May 2002.

[2] Thomas Wiegand, "Joint Committee Draft (CD)", document JVT-C167, Joint Video Team (JVT) Meeting, Fairfax, VA, USA, 6-10, May 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Inc. |  |  |
| Mailing address | 6000 Connection Drive, Irving, TX 75039 USA |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 972 894 4188 |  |  |
| Fax |  |  |  |
| Email | marta.karczewicz@nokia.com |  |  |
| Place and date of submission | Klagenfurt, Austria, 22/7/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | 4-tap filter for bi-predicted macroblocks |  |
| Contribution number |  | JVT-D029 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: