| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D031 Filename: JVT-D031.doc |
| --- | --- |

| Title: | Error robustness memory management control operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Kadono, Satoshi Kondo and Kiyofumi Abe 1006 Kadoma, Kadoma Osaka 573-8501 Japan | Tel: Email: | +81-6-6900-9689 kadono@drl.mei.co.jp |
| Source: | Matsushita Electric Industrial Co., Ltd. |  |  |

_____________________________

# Introduction

Reference picture buffer is controlled by using memory management control operation (MMCO). As the mismatch of reference picture buffer cause the significant picture degradation, MMCO commands is one of the most important information of the header.

In JVT CD, some MMCO commands can be sent repeatedly to protect from an unintentional data loss. However, MMCO “reset” commands cannot be sent repeatedly. Further more, MMCO commands always executed after decoding of the current picture.

In this contribution, we propose additional MMCO commands to improve the error robustness, which includes a new “reset” which can be sent repeatedly and new MMCO commands executable before decoding of the current picture.

# Problems of MMCO

## MMCO “reset” command

MMCO “reset” command does not carry any information to indicate target frame number. Therefore, “reset” should be applied only to the current picture. Figure 1 shows a first bit stream example. All B-pictures are not stored pictures.

If I10 picture includes “reset”, all stored picture except the current decoded picture I10 are removed from the reference picture buffer. Therefore, the reference picture buffer contains the decoded picture as shown in Table 1.

It is clear that “reset” cannot be sent repeatedly for error robustness, because the repeated “reset” following I10 picture remove I10 picture from the reference picture buffer.

Figure 1 First bit stream example.

Table 1 Decoded pictures in the reference picture buffer of first bit stream.

| Current decoding picture | Decoded pictures of the reference picture buffer |
| --- | --- |
| B11 | I10 |
| B12 | I10 |
| P13 | I10 |
| B14 | I10, P13 |
| B15 | I10, P13 |

## Repeat MMCO command

Figure 2 shows another bit stream example, which includes stored B-pictures. Table 2 shows the contents in a reference picture buffer and MMCO commands without MMCO commands repetition. As B3 picture and B4 picture are non-stored pictures, they should be marked as “unused” by MMCO commands. B2 picture should be marked as “unused” at P5 picture, because CD prohibits remove the stored pictures except the current picture if the current picture is non-stored and the stored pictures have not been marked as “unused”.

If MMCO sent with P5 picture is lost by an unintentional data loss, B2 picture has stayed in the reference picture buffer while the following pictures are decoded. To prevent this error propagation, CD allows using the same MMCO commands repeatedly as shown in Table 3. “unused” B2 picture can be sent at B6 picture. Even if P5 picture may be lost, as B2 picture is marked as “unused” at B6 picture, the error propagation caused by mismatch of number of stored picture can be stopped.

However, as MMCO commands are executed after decoding the current picture, B2 picture is still in the reference picture buffer during B6 picture decoding.

Figure 2 Second bit stream example.

Table 2 Contents in a reference picture buffer and MMCO commands (without repetition).

| Current decoding picture | Decoded pictures of the reference picture buffer | MMCO command |
| --- | --- | --- |
| I0 |  |  |
| P1 | I0 |  |
| B2 | I0, P1 |  |
| B3 | I0, P1, B2 | unused B3 |
| B4 | I0, P1, B2, B3 | unused B4 |
| P5 | I0, P1, B2, B3, B4 | unused B2 |
| B6 | I0, P1, P5 |  |
| B7 | I0, P1, P5, B6 | unused B7 |

Table 3 Contents in a reference picture buffer and MMCO commands (with repetition).

| Current decoding picture | Decoded pictures of the reference picture buffer | MMCO command |
| --- | --- | --- |
| I0 |  |  |
| P1 | I0 |  |
| B2 | I0, P1 |  |
| B3 | I0, P1, B2 | unused B3 |
| B4 | I0, P1, B2, B3 | unused B4 |
| P5 | I0, P1, B2, B3, B4 | unused B2 |
| B6 | I0, P1, P5 | unused B2 |
| B7 | I0, P1, P5, B6 | unused B7 |

# Proposed solutions

To solve the problems described in section 2, “reset” MMCO command is modified and MMCO commands executable before decoding the current picture are newly introduced.

If decoder detects the frame number that “reset” command was sent, the decoder can calculate how many pictures should stay in the reference picture buffer and decide which picture should be marked as “unused”. So, by sending “reset” command repeatedly, the mismatch of number of stored picture in the reference picture buffer can be corrected.

Proposed modification to “reset” commands is only adding an associated data filed, difference_of_pic_nums. If difference_of_pic_nums is equal to 0, semantics of “reset” command is the same as CD. If difference_of_pic_nums is greater than 0, it means that “reset” command was sent at the picture whose frame number was smaller than the current frame number by difference_of_pic_nums.

When the first sent command is lost by an unintentional data loss, the picture with the repeated command can be decode by referring the reference picture buffer which includes correct number of stored picture if repeated MMCO command is executed before decoding. That is, the damage of decoded picture can be reduced. For example, assume that MMCO command is sent repeatedly as shown in Table 3. If P5 picture is lost, B2 picture is marked as “unused” at B6 picture after decoding B6 picture. If “repeat unused” B2 picture is sent at B6 picture as shown in Table 4, the error of B6 picture of this case is smaller than that of B6 picture of CD case as B2 picture is marked as “unused” before decoding B6 picture.

Table 4 Contents in a reference picture buffer and MMCO commands (with repetition).

| Current decoding picture | Decoded pictures of the reference picture buffer | MMCO command |
| --- | --- | --- |
| I0 |  |  |
| P1 | I0 |  |
| B2 | I0, P1 |  |
| B3 | I0, P1, B2 | unused B3 |
| B4 | I0, P1, B2, B3 | unused B4 |
| P5 | I0, P1, B2, B3, B4 | unused B2 |
| B6 | I0, P1, P5 | repeat unused B2 |
| B7 | I0, P1, P5, B6 | unused B7 |

# Proposed revised text

Table 8-9 – Memory management control operation (memory_management_control_operation) values

| Code Number | Memory Management Control Operation | Associated Data Fields Following |
| --- | --- | --- |
| 0 | End memory_management_control_operation Loop | None (end of RPS layer) |
| 1 | Mark a Short-Term Frame as “Unused” | difference_of_pic_nums |
| 2 | Mark a Long-Term Frame as “Unused” | long_term_pic_index |
| 3 | Assign a Long-Term Index to a Frame | difference_of_pic_nums and long_term_pic_index |
| 4 | Specify the Maximum Long-Term Frame Index | max_long_term_pic_index_plus1 |
| 5 | Reset | difference_of_pic_nums |
| 6 | Mark a Short-Term Frame as “Unused” (repeated) | difference_of_pic_nums |
| 7 | Mark a Long-Term Frame as “Unused” (repeated) | long_term_pic_index |
| 8 | Assign a Long-Term Index to a Frame (repeated) | difference_of_pic_nums and long_term_pic_index |

8.4.4.1 Memory management control operation (memory_management_control_operation)

If memory_management_control_operation is Reset and it follows zero value in the associated data field, all frames in the multi-frame buffer (but not the current frame unless specified separately) shall be marked “unused” (including both short-term and long-term frames). Moreover, the maximum long-term frame index shall be reset to zero. If memory_management_control_operation is Reset and it follows non-zero value in the associated data field, it indicates that Reset was sent at the previous picture whose frame number is specified by difference_of_pic_num.

If memory_management_control_operation is repeated command, the same operation shall be indicated in the previous picture. Therefore, if the same operation has not indicated yet, decoder should treat as the previous operation was lost by an unintentional data loss. Decoder may apply memory_management_control_operation before decoding the current picture in this case.

# Conclusion

By introducing a new “reset” which can be send repeatedly and new MMCO commands which indicates these commands were sent repeatedly, decoder can improve the ability of error concealment. Therefore we propose to incorporate this modification into FCD.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma, Osaka, 573-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Error robustness memory management control operation |  |
| Contribution number |  | JVT-D031 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: