| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D033 Filename: JVT-D033.doc |
| --- | --- |

| Title: | Clarification and improvement of direct mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe, Satoshi Kondo and Shinya Kadono 1006 Kadoma, Kadoma, Osaka, Japan. | Tel: Email: | +81-6-6900-9689 abe@drl.mei.co.jp |
| Source: | Matsushita Electric Industrial Co., Ltd. |  |  |

_____________________________

1. Introduction

According to the CD[1], B picture can refer to the previous B-pictures in addition to I- and P-pictures, however, when the co-located blocks in the backward reference picture does not refer to a B-pictures, a B-picture cannot be referred to as the forward reference picture in direct mode even if it is the most recent previous picture.

In this document, we propose a method to clean up these problems. In the proposed method, the forward reference picture used in direct mode is set to index 0 of the forward reference set. According to the simulation results, the proposed method improves the coding efficiency about 1-7 %.

2. Background

At the Pattaya meeting and the Geneva meeting, we proposed a new additional prediction method for B-pictures [2] [3], which improved coding efficiency significantly. However, our proposal was not adopted, because the consideration for the bi-predictive prediction was not enough. Furthermore at the Fairfax meeting, the scaling factors in direct mode (direct_mv_scale_fwd, direct_mv_scale_bwd, direct_mv_divisor) were introduced as substitution for temporal references.

In this document, we propose a revised method considering above changes. The proposed method is simpler than the previous one and needs only one change to CD.

3. Proposed Method

Figure 1 shows a example of picture sequence with temporal order. The coding order of this example is  “P1”,”P4”,”B2”,”B3”. Thiat is,  “P4” can refer only “P1”, “B2” and.”B3” can refer “P1” and “P4”.

According to the CD[1], the forward reference picture in direct mode of the current block “bl3” is the same as the forward reference picture of the co-located block “bl4” in the backward reference picture “P4”. Therefore, “B2” cannot be reference picture in direct mode even if it may the most recent previous picture.

In the proposed method, the forward reference picture in direct mode is set to the picture with index 0 of the forward reference set. Therefore the reference picture of the current picture is independent of the referencepicture of the co-located block “bl4”. Because index 0 is usually assigned to the most recent temporally previous picture “B2”, this proposal is the best relationship for “bl3”.

Fortunately, this simple modification to CD can also solve an ambiguousness of the reference picture whena co-located block has no forward references. According to this proposal, if the co-located block “bl4” has no forward references, the forward reference picture of “bl3” is determined as “B2” instantly.

Figure 1: The reference relationship in direct mode

4. Simulation Results

4.1. Conditions of Simulation

Simulation conditions follow VCEG-N81 (recommended simulation common condition):

- Based software version: JM2.0.

- RD optimization: on.

- QP: 16, 20, 24, 28, and fixed for all over sequence.

- Picture coding type:  IBBPBBP for all simulations.

- Number of reference frames: 5 frames.

- The motion vector search range: 32 pels.

- Motion compensation: 1/4-pel accuracy for QCIF, 1/8-pel accuracy for CIF.

- Other coding options: using Hadamard, using CABAC, and no slice structure.

- Test sequences: see Table 1.

Table 1: Test sequences

| Frame Skip | 2 | 1 | 0 |
| --- | --- | --- | --- |
| Sequences | Container, QCIF,300 News, QCIF,300 Foreman, QCIF,300 | Paris, CIF,300 Silent Voice, QCIF,300 | Mobile CIF 300 Tempete CIF 260 |

To show that improvement of our proposed methods is independent of temporal reference, we simulated in two conditions. One is the result of using the temporal references for the motion vector scaling “With TR”, and the other is the result of  the scaling factors used substituting for temporal references “Without TR”. The value of the scaling factors are shown in Table 3. Since the picture coding type is “IBBPBBP” in this simulation, the scaling factors depend on the position of B-picture whether it is “1st B-picture” or “2nd B-picture”.

Table 2: The value of the scaling factors

|  |  | Direct_mv_scale_fwd | Direct_mv_scale_bwd | Direct_mv_divisor |
| --- | --- | --- | --- | --- |
| 1st B-picture | Proposed | 1 | 2 | 3 |
|  | Anchor | 1 | 2 | 3 |
| 2nd B-picture | Proposed | 1 | 1 | 3 |
|  | Anchor | 2 | 1 | 3 |

4.2. Simulation Results

Table 3 shows the bit savings and PSNR differences for B-pictures of the proposed method compared to CD. These values are calculated by using avsnr software. And R-D curves for B-pictures of each sequence are shown from Figure 2 to Figure 5. These results show that the proposed method improves the coding efficiency for every sequence. The improvement is achieved at any QP and independent whether temporal reference is used or not.

Table 3: Improved avsnr of B-pictures by proposed method.

|  | With TR |  | Without TR |  |
| --- | --- | --- | --- | --- |
|  | Bit savings (%) | PSNR (dB) | Bit savings (%) | PSNR (dB) |
| Foreman | 1.16 | 0.054 | 0.92 | 0.043 |
| Container | 6.80 | 0.400 | 6.72 | 0.390 |
| News | 2.56 | 0.146 | 2.56 | 0.146 |
| Silent voice | 3.70 | 0.174 | 3.58 | 0.167 |
| Paris | 4.67 | 0.229 | 4.48 | 0.220 |
| Mobile | 4.30 | 0.161 | 1.93 | 0.074 |
| Tempete | 2.26 | 0.077 | 3.31 | 0.119 |

5. Proposed Revised Text

11.4.3	Motion vectors in direct mode

If the multiple reference frame prediction is used, the forward reference picture for direct mode is set to the picture with index 0 of the forward reference set. Also note that if the picture with index 0 of the backward reference set is an intra-coded picture or the reference macroblock is an intra-coded block or it doesn’t use forward prediction, the motion vectors are set to zero. With possible adaptive switch of frame/field coding at picture level, a B-picture can be coded in either frame structure or field structure.

6. Conclusions

In this contribution, we propose the method that the reference picture in direct mode is set to the picture whit index 0 of the forward reference set. The modification to CD is very simple. According to the simulation results, the proposed method improves the coding efficiency about 1-7 % independent whether temporal reference is used or not. And it also clarifies the reference picture in direct mode when the co-located block has no forward references. Therefore we propose to incorporate this modification into FCD.

##### References

[1] Thomas Wiegand, “ Joint Committee Draft (CD)”, JVT-C167, May 2002

[2] S.Kondo, S.Kadono, and M.Schlockermann, “New Prediction Method to Improve B-picture Coding Efficiency”,VCEG-O26, November 2001

[3] S.Kondo, S.Kadono, and M.Schlockermann, “Proposal of Minor Changes to Multi-Frame Buffering Syntax for Improving Coding Efficiency of B-pictures”, JVT-B057, January 2002

[4] M.Schlockermann, “AHG Report: Multiframe Buffering Syntax“, JVT-C010, May 2002

Figure 2: R-D curve for B-picture of “Container” (With TR)

Figure 3: R-D curve for B-picture of “Paris” (With TR)

Figure 4: R-D curve for B-picture of “Container” (Without TR)

Figure 5: R-D curve for B-picture of “Paris” (Without TR)

###### JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma, Osaka, 573-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Kiyofumi Abe |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | abe@drl.mei.co.jp |  |  |
| Place and date of submission | 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Clarification and improvement of direct mode |  |
| Contribution number |  | JVT-D033 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: