| Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 |  | Document JVT-D036 File: JVT-D036.doc Generated: 2002-07-12 |
| --- | --- | --- |
| Title: | Proposed updates to CAVLC |  |
| Status: | Input to JVT |  |
| Contact: | Karl Lillevold RealNetworks, 2601 Elliott Ave, Seattle, WA 98121, USA karll@real.com +1.206.892.6286 |  |
| Source: | RealNetworks |  |
| Purpose: | Proposal |  |

## Corrections

The following are proposed corrections to JVT-D017d0.doc.  The text is copied from the document, then the corrections are marked with ‘track changes’ enabled

#### 7.3.6.3	 Residual data syntax

| residual( mb_mode ) { | Category | Descriptor |
| --- | --- | --- |
| if( mb_mode = = MBintra16x16 ) |  |  |
| residual_4x4block_cavlc( intra16x16DC, 16 ) |  |  |
| for( i8x8=0; i8x8<4; i8x8++ ) /* each luma 8x8 block */ |  |  |
| for( i4x4=0; i4x4<4; i4x4++ ) /* each 4x4 sub-block of block */ |  |  |
| if( coded_block_pattern & ( 1 << i8x8 ) ) |  |  |
| if( mb_mode = = MBintra16x16 ) |  |  |
| residual_4x4block_cavlc( intra16x16AC, 16 ) |  |  |
| else |  |  |
| residual_4x4block_cavlc( luma, 16 ) |  |  |
| if( coded_block_pattern & 0x30 ) /* chroma DC residual coded */ |  |  |
| for( iCbCr=0; iCbCr<2; iCbCr++ ) |  |  |
| residual_4x4block_cavlc( chromaDC, 4 ) |  |  |
| if( coded_block_pattern & 0x20 ) /* chroma AC residual coded */ |  |  |
| for( iCbCr=0; iCbCr<2; iCbCr++ ) |  |  |
| for( i4x4=0; i4x4<4; i4x4++ ) |  |  |
| residual_4x4block_cavlc( chromaAC, 16 ) |  |  |
| } |  |  |

#### 7.3.6.4	Residual 4x4 block CAVLC syntax

| residual_4x4block_cavlc( block_mode, max_numcoeff ) { |  |  |
| --- | --- | --- |
| numcoeff_trailingones | 5 ¦ 6 | ce(v) |
| if( trailingones ) |  |  |
| for( i=trailingones-1; i>=0; i-- ) |  |  |
| trailingones_sign[ i ] | 5 ¦ 6 | u(1) |
| if( numcoeff_trailingones ) { |  |  |
| for( i=numcoeff-1; i>=0; i-- ) { |  |  |
| luma_level[ i ] | 5 ¦ 6 | ce(v) |
| } |  |  |
| if( numcoeff_trailingones < max_numcoeff ) { |  |  |
| totalzeros | 5 ¦ 6 | ce(v) |
| i = numcoeff_trailingones - 1 |  |  |
| zerosleft = totalzeros |  |  |
| if( i > 0 && zerosleft > 0 ) { |  |  |
| do { |  |  |
| run_before[ i ] | 5 ¦ 6 | ce(v) |
| zerosleft -= runbefore[ i ] |  |  |
| i-- |  |  |
| } while ( zerosleft != 0 && i != 0 ) |  |  |
| runbefore[ i ] = zerosleft |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

##### 9.7.3.1.1	Table selection

Table 10-10 – Calculation of N for Num-VLCN

| Upper block (NU) | Left block (NL) | N |
| --- | --- | --- |
| X | X | (NL+NU)/2 |
| X |  | NU |
|  | X | NL |
|  |  | 0 |

0 <= N < 2 : Num-VLC0

2 <= N < 4 : Num-VLC1

4 <= N < 8 : Num-VLC2

N >= 8 : 6 bit FLC xxxxyy, as follows:

#### 10.1.6.2	Decoding of level information

| Lev-VLC4 |  |  |
| --- | --- | --- |
| Code no | Code no | Code no |
| 0-15 | 1xxxx | ±1 to ±8 |
| 16-31 | 01xxxx | ±9 to ±16 |
| .. | .. | .. |
| 224-239 | 000000000000001xxxx | 113 to 120 |
| 240 -> | 0000000000000001xxxxxxxxxxxx | ±121 -> |

Normally levels to be coded take values ± 1, ± 2 etc. However, for the first coefficient to be decoded (after T1s)  if T1s < 3, levels to be decoded take values ± 2, ± 3 etc (Level’).  Since the first coefficient is always decoded with Lev-VLC0 or Lev-VLC1, these levels starting at ± 2 are only applicable to Lev-VLC0 and Lev-VLC1.

## Clarifications

The following is a proposed clarification to the table selection in the “Num-trail” section (Number of coefficients and trailing ones).

##### 9.7.3.1.1	Table selection

For all elements, except chroma DC, a choice between three tables and one FLC is made.  Selection is done as follows:  N is calculated based on the number of coefficients in the block above and to the left of the current block: NU and NL. In the table below, X means that the block is available in the same slice.  The block’s coding mode is not taken into account when determining availability.  When finding the block above and to the left for a block of Intra16x16 DC coefficients, the location of the block is assumed to be (0,0), i.e. the upper left corner of the macroblock.

## Improvements

The tables used for CAVLC have been updated in a semi-structured manner such that each entry can be stored as a 4 bit codeword + 4 bit for length (or less). This means that even the simplest implementation need only 410 bytes to implement CAVLC.  The tables were also re-trained with focus on the QP range 16-28 instead of 0-28. This provided up to 1% compression improvements in this range, with little change for the low QP range.  See JVT-D036.xls for detailed results. All tables were updated, except the structured tables for level coding.  In JVT-D034 UBVideo proposes an improvement and simplification to the level coding.  JVT-D036.xls also contains cross-verification results obtained with this modification

These tables also correct the following errors in JVT-D017d0:

- NumVLC-ChromaDC table: missing row for NumCoeff = 0

- TotalZeros: missing row for TotalZeros = 0

The suggested new tables are as follows.  The format is code followed by length.

Table 1: Num-VLC0 (4+4 bit code/length)

| NumCoef\T1s | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| 0 | 1 1 | - | - | - |
| 1 | 0101 6 | 01 2 | - | - |
| 2 | 0111 8 | 0100 6 | 0001 3 | - |
| 3 | 0111 9 | 0110 8 | 0101 7 | 0011 5 |
| 4 | 0111 10 | 0110 9 | 0101 8 | 0011 6 |
| 5 | 0111 11 | 0110 10 | 0101 9 | 0100 7 |
| 6 | 1111 13 | 0110 11 | 0101 10 | 0100 8 |
| 7 | 1011 13 | 1110 13 | 0101 11 | 0100 9 |
| 8 | 1000 13 | 1010 13 | 1101 13 | 0100 10 |
| 9 | 1111 14 | 1110 14 | 1001 13 | 0100 11 |
| 10 | 1011 14 | 1010 14 | 1101 14 | 1100 13 |
| 11 | 1111 15 | 1110 15 | 1001 14 | 1100 14 |
| 12 | 1011 15 | 1010 15 | 1101 15 | 1000 14 |
| 13 | 1111 16 | 0001 15 | 1001 15 | 1100 15 |
| 14 | 1011 16 | 1110 16 | 1101 16 | 1000 15 |
| 15 | 0111 16 | 1010 16 | 1001 16 | 1100 16 |
| 16 | 0100 16 | 0110 16 | 0101 16 | 1000 16 |

Table 2: Num-VLC1 (4+4 bit code/length)

| NumCoeff\T1s | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| 0 | 11 2 | - | - | - |
| 1 | 1011 6 | 10 2 | - | - |
| 2 | 0111 6 | 0111 5 | 011 3 | - |
| 3 | 0111 7 | 1010 6 | 1001 6 | 0101 4 |
| 4 | 0111 8 | 0110 6 | 0101 6 | 0100 4 |
| 5 | 0100 8 | 0110 7 | 0101 7 | 0110 5 |
| 6 | 0111 9 | 0110 8 | 0101 8 | 1000 6 |
| 7 | 1111 11 | 0110 9 | 0101 9 | 0100 6 |
| 8 | 1011 11 | 1110 11 | 1101 11 | 0100 7 |
| 9 | 1111 12 | 1010 11 | 1001 11 | 0100 9 |
| 10 | 1011 12 | 1110 12 | 1101 12 | 1100 11 |
| 11 | 1000 12 | 1010 12 | 1001 12 | 1000 11 |
| 12 | 1111 13 | 1110 13 | 1101 13 | 1100 12 |
| 13 | 1011 13 | 1010 13 | 1001 13 | 1100 13 |
| 14 | 0111 13 | 1011 14 | 0110 13 | 1000 13 |
| 15 | 1001 14 | 1000 14 | 1010 14 | 0001 13 |
| 16 | 0111 14 | 0110 14 | 0101 14 | 0100 14 |

Table 3: Num-VLC2 (4+4 bit code/length)

| NumCoeff\T1s | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| 0 | 1111 4 | - | - | - |
| 1 | 1111 6 | 1110 4 | - | - |
| 2 | 1011 6 | 1111 5 | 1101 4 | - |
| 3 | 1000 6 | 1100 5 | 1110 5 | 1100 4 |
| 4 | 1111 7 | 1010 5 | 1011 5 | 1011 4 |
| 5 | 1011 7 | 1000 5 | 1001 5 | 1010 4 |
| 6 | 1001 7 | 1110 6 | 1101 6 | 1001 4 |
| 7 | 1000 7 | 1010 6 | 1001 6 | 1000 4 |
| 8 | 1111 8 | 1110 7 | 1101 7 | 1101 5 |
| 9 | 1011 8 | 1110 8 | 1010 7 | 1100 6 |
| 10 | 1111 9 | 1010 8 | 1101 8 | 1100 7 |
| 11 | 1011 9 | 1110 9 | 1001 8 | 1100 8 |
| 12 | 1000 9 | 1010 9 | 1101 9 | 1000 8 |
| 13 | 1101 10 | 0111 9 | 1001 9 | 1100 9 |
| 14 | 1001 10 | 1100 10 | 1011 10 | 1010 10 |
| 15 | 0101 10 | 1000 10 | 0111 10 | 0110 10 |
| 16 | 0001 10 | 0100 10 | 0011 10 | 0010 10 |

Table 4: Num-VLC_Chroma_DC (3+3 bit code/length)

| NumCoeff\T1s | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| 0 | 01 2 | - | - | - |
| 1 | 111 6 | 1 1 | - | - |
| 2 | 100 6 | 110 6 | 001 3 | - |
| 3 | 011 6 | 011 7 | 010 7 | 101 6 |
| 4 | 010 6 | 011 8 | 010 8 | 000 7 |

### TotalZeros

Table 5: TotalZeros tables for all 4x4 blocks (3+4 bit)

| NumCoeff TotZeros | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 1 | 111 3 | 101 4 | 011 5 | 101 4 | 001 6 | 001 6 |  |
| 1 | 011 3 | 110 3 | 111 3 | 111 3 | 100 4 | 001 5 | 001 5 |  |
| 2 | 010 3 | 101 3 | 110 3 | 101 4 | 011 4 | 111 3 | 101 3 |  |
| 3 | 011 4 | 100 3 | 101 3 | 100 4 | 111 3 | 110 3 | 100 3 |  |
| 4 | 010 4 | 011 3 | 100 4 | 110 3 | 110 3 | 101 3 | 011 3 |  |
| 5 | 011 5 | 101 4 | 011 4 | 101 3 | 101 3 | 100 3 | 11 2 |  |
| 6 | 010 5 | 100 4 | 100 3 | 100 3 | 100 3 | 011 3 | 010 3 |  |
| 7 | 011 6 | 011 4 | 011 3 | 011 4 | 011 3 | 010 3 | 001 4 |  |
| 8 | 010 6 | 010 4 | 010 4 | 011 3 | 010 4 | 001 4 | 001 3 |  |
| 9 | 011 7 | 011 5 | 011 5 | 010 4 | 001 5 | 001 3 | 000 6 |  |
| 10 | 010 7 | 010 5 | 010 5 | 010 5 | 001 4 | 000 6 | - |  |
| 11 | 011 8 | 011 6 | 001 6 | 001 5 | 000 5 | - | - |  |
| 12 | 010 8 | 010 6 | 001 5 | 000 5 | - | - | - |  |
| 13 | 011 9 | 001 6 | 000 6 | - | - | - | - |  |
| 14 | 010 9 | 000 6 | - | - | - | - | - |  |
| 15 | 001 9 | - | - | - | - | - | - |  |
| NumCoeff TotZeros | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 | 001 6 | 001 6 | 001 5 | 000 4 | 000 4 | 000 3 | 00 2 | 0 1 |
| 1 | 001 4 | 000 6 | 000 5 | 001 4 | 001 4 | 001 3 | 01 2 | 1 1 |
| 2 | 001 5 | 001 4 | 001 3 | 001 3 | 01 2 | 1 1 | 1 1 | - |
| 3 | 011 3 | 11 2 | 11 2 | 010 3 | 1 1 | 01 2 | - | - |
| 4 | 11 2 | 10 2 | 10 2 | 1 1 | 001 3 | - | - | - |
| 5 | 10 2 | 001 3 | 01 2 | 011 3 | - | - | - | - |
| 6 | 010 3 | 01 2 | 001 4 | - | - | - | - | - |
| 7 | 001 3 | 001 5 | - | - | - | - | - | - |
| 8 | 000 6 | - | - | - | - | - | - | - |
| 9 | - | - | - | - | - | - | - | - |
| 10 | - | - | - | - | - | - | - | - |
| 11 | - | - | - | - | - | - | - | - |
| 12 | - | - | - | - | - | - | - | - |
| 13 | - | - | - | - | - | - | - | - |
| 14 | - | - | - | - | - | - | - | - |
| 15 | - | - | - | - | - | - | - | - |

Table 6: TotalZeros table for chroma DC 2x2 blocks (no change) (2+2 bit)

| NumCoeff TotZeros | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 0 | 1 1 | 1 1 | 1 1 |
| 1 | 01 2 | 01 2 | 0 1 |
| 2 | 01 3 | 00 2 | - |
| 3 | 00 3 | - | - |

### Run before each coefficient

Table 7 Tables for Run before each coefficient  (3+4 bit)

| TotZeros Run Before | 1 | 2 | 3 | 4 | 5 | 6 | >6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 11 | 11 | 11 | 11 | 111 3 |
| 1 | 0 | 01 | 10 | 10 | 10 | 000 | 110 3 |
| 2 | - | 00 | 01 | 01 | 011 | 001 | 101 3 |
| 3 | - | - | 00 | 001 | 010 | 011 | 100 3 |
| 4 | - | - | - | 000 | 001 | 010 | 011 3 |
| 5 | - | - | - | - | 000 | 101 | 010 3 |
| 6 | - | - | - | - | - | 100 | 001 3 |
| 7 | - | - | - | - | - | - | 001 4 |
| 8 |  | - | - | - | - | - | 001 5 |
| 9 | - | - | - | - | - | - | 001 6 |
| 10 | - | - | - | - | - | - | 001 7 |
| 11 | - | - | - | - | - | - | 001 8 |
| 12 | - | - | - | - | - | - | 001 9 |
| 13 | - | - | - | - | - | - | 001 10 |
| 14 | - | - | - | - | - | - | 001 11 |

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | RealNetworks, Inc. |  |  |
| Mailing address | RealNetworks, Inc. 2601 Elliott Ave Seattle, WA 98121 |  |  |
| Country | USA |  |  |
| Contact person | Gary Greenbaum |  |  |
| Telephone | +1 206 674 2339 |  |  |
| Fax | +1 206 674 3580 |  |  |
| Email | garyg@real.com |  |  |
| Place and date of submission | 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  |  |  |
| Title |  |  |  |
| Contribution number |  | JVT-D036 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
| X | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: For purposes of this contribution, the terms of 2.2.1 above [and as detailed in document VCEG-O54 submitted by Gary Sullivan] are agreed to only as necessary to comply with the required portions of a “baseline” JVT video profile.