| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D039 Filename: JVT-D039.doc |
| --- | --- |

| Title: | Performance comparison: H.26L intra coding vs. JPEG2000 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information/Report |  |  |
| Author(s) or Contact(s): | Till Halbach | Tel: Fax: Email: | +47 - 73 59 44 88 +47 - 73 59 26 40 halbach@tele.ntnu.no |
| Source: | Norwegian University of Science and Technology (NTNU) (Department of Telecommunications) Trondheim, Norway |  |  |

_____________________________

# Abstract

This contribution reports the performance comparison of H.26L’s JM-2.0 in intra-frame mode to the well known JPEG2000 still image compression standard for error-free transmission. The results show that H.26L partly outperforms its competitor at very low bit rates and for small picture sizes.

# Intellectual Property Rights

The author is not aware of any IPRs that are connected to the proposed techniques. For more information, see the JVT Patent Disclosure Form attached to this document.

# Introduction

There are a number of performance comparisons to evaluate the coding performance of H.26L in terms of PSNR and a subjective evaluation, see e.g. [VCEG-N18]. All comparisons, however, are undertaken with regard to the overall performance of H.26L, i.e. considering both I-, P- and B-frames. In contrast to that, here it is investigated if H.26L can be treated as a serious alternative to still-image compression standards if operated in intra-frame mode, i.e. without exploiting temporal video sequence correlation.

# Performance evaluation: H.26L – JPEG2000

Testing is carried out under the following conditions. H.26L’s JM-2.0 Main Profile without ABT has to compete with VM-9.0 of JPEG2000. Since H.26L lacks a rate distortion control, the QPs of JM have been chosen beforehand, and then VM-9.0 is adjusted to the achieved rate of JM-2.0 in several iterations with a tolerance of 0.001 bpp. The following Table shows the results in terms of luminance PSNR [dB] and bit rate [bpp].

| Sequence (Size) | Q | QP | Bit rate | PSNR |  |  | Difference JM to - |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | JPEG | H.26L |  | JPEG | JPEG2000 | H.26L | JPEG | JPEG2000 |
| Container (QCIF) | 93 | 4 | 2.711 | 42.64 | 48.22 | 46.75 | 4.11 | -1.47 |
|  | 76 | 12 | 1.446 | 35.58 | 40.54 | 40.19 | 4.61 | -0.35 |
|  | 25 | 20 | 0.674 | 29.74 | 33.16 | 34.41 | 4.67 | 1.25 |
|  | 3 | 28 | 0.288 | 22.03 | 27.02 | 28.83 | 6.80 | 1.81 |
| News (QCIF) | 92 | 4 | 2.651 | 42.61 | 49.38 | 47.24 | 4.63 | -2.14 |
|  | 75 | 12 | 1.508 | 36.11 | 41.38 | 40.89 | 4.78 | -0.49 |
|  | 25 | 20 | 0.771 | 30.19 | 33.47 | 34.62 | 4.43 | 1.15 |
|  | 4 | 28 | 0.347 | 23.52 | 27.31 | 28.62 | 5.10 | 1.31 |
| Tempete (CIF) | 95 | 4 | 3.707 | 43.52 | 50.91 | 46.68 | 3.16 | -4.23 |
|  | 84 | 12 | 2.123 | 36.86 | 41.87 | 39.69 | 2.83 | -2.18 |
|  | 43 | 20 | 1.031 | 30.81 | 33.67 | 32.95 | 2.14 | -0.72 |
|  | 9 | 28 | 0.407 | 25.49 | 27.52 | 27.01 | 1.52 | -0.51 |
| Mobile (PAL) | 95 | 4 | 4.310 | 43.13 | 50.78 | 46.38 | 3.25 | -4.40 |
|  | 85 | 12 | 2.518 | 35.52 | 41.40 | 39.30 | 3.78 | -2.10 |
|  | 52 | 20 | 1.306 | 28.93 | 33.23 | 32.57 | 3.64 | -0.66 |
|  | 13 | 28 | 0.555 | 24.54 | 26.48 | 26.37 | 1.83 | -0.11 |
| Total |  |  |  |  |  |  | 3.83 | -0.87 |

The achieved bit rate corresponds to the QPs 4, 12, 20, and 28. All values are averaged over 20 pictures of equispaced frame indices from the respective sequence. The last column contains the performance differences, i.e. (JM minus VM) PSNR values. Negative values here means that JPEG2000 outperforms H.26L.

H.26L performs very well in intra mode. It is observed that the difference between both standards becomes smaller with decreasing picture size. Moreover, H.26L approaches JPEG2000 in performance as the rate becomes smaller. For small images and at very low rates, the video coding standard even outperforms JPEG2000 which is specialized in coding still images! This may be explained by the various excellent intra-frame prediction modes of H.26L. In the JPEG2000 standard, prediction is not employed. The average gain over all sequences and all bit rates is -0.87 dB, i.e. JPEG2000 outperforms H.26L slightly (by approximately 4%).

Considering the objective results in the last row of the Table, i.e. a very low bit rate, the JPEG2000 images look generally blurred; they contain further ringing artifacts and lack high-frequency information. This is due to the wavelet transform employed. The H.26L images do not show these artifacts, even though high-frequency information was removed under the coding process at low bit rates as well. Blocking can successfully be avoided by the build-in loop filter. At higher rates however, JPEG2000 outperforms H.26L also subjectively. Examples are given in presentation accompanying this paper.

# Performance evaluation: H.26L – JPEG

In addition to JPEG2000, this contribution also records the coding efficiency of JPEG, as the mentioned Table shows. The software implementation version 6b of the IJG was used. The PSNR values given must be interpreted with caution since it is not possible to put a rate constraint on the software directly. Instead, the quality parameter of the JPEG algorithm has been adjusted such that the resulting rate is closest possible to the target bit rate.

JPEG consists, like H.26L, of an integer DCT with subsequent coefficient scan, quantization, and entropy encoding, but lacks a prediction mechanism. Mainly because of this difference, it can be seen that H.26L outperforms JPEG significantly for all image size at all rates. The average gain over all sequences and all bit rates is 3.83 dB, i.e. H.26L performs superior to JPEG (by more than 12%).

A useful side result is the PSNR gain of JPEG2000 over the old JPEG standard; it is on the average 4.7 dB, spanning a range from approximately 2 dB to over 7 dB. This is an improvement of roughly 14%.

# Conclusions

In addition to the exploitation of temporal redundancies in video sequences by H.26L, as quantified by other tests, also its intra-frame coding abilities, which reduce the picture’s spatial redundancies, perform very well. Compared to the JPEG standard, H.26L operates significantly better, both objectively and subjectively. Considering JPEG2000, H.26L is outperformed for large picture sizes and at moderate to high bit rates. For small image dimensions and at very low rates, however, H.26L’s intra mode achieves PSNR results superior to JPEG2000.

# Abbreviations

ABT		Adaptive Block Transform
B		Bi-directionally predicted
I		Intra (-frame predicted)
IJG		Independent JPEG group
IPR		Intellectual Property Right
JM		Joint Model
JPEG		Joint Photographic Experts Group
P		Predicted (forwardly)
PSNR		Peak-signal-to-noise ratio
QP		Quantization parameter
VM		Verification Model

# References

[VCEG-N18]	Pankaj Topiwala, Gary Sullivan, Anthony Joch and Faouzi Kossentini. Performance Evaluation of H.26L, TML 8 vs. H.263++ and MPEG-4. Santa Barbara (CA; USA), September 2001

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Norwegian University of Science and Technology (NTNU) |  |  |
| Mailing address | Department of Telecommunications O.S. Bragstads veg 2b 7491 Trondheim |  |  |
| Country | Norway |  |  |
| Contact person | Till Halbach |  |  |
| Telephone | +47 - 73 59 44 88 |  |  |
| Fax | +47 - 73 59 26 40 |  |  |
| Email | halbach@tele.ntnu.no |  |  |
| Place and date of submission | Trondheim; July 15, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | H.26L/H.264 |  |
| Title |  | Joint Committee Draft (C167) |  |
| Contribution number |  | D039 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks:

The implementation of ABT in the JM is due to July 9, 2002, and was therefore not taken into account under testing which was undertaken prior to that date. JM-2.0 with ABT included would change the results presented here only insignificantly.

This is the Verification Model of the final standard.