| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D040 Filename: JVT-D040.doc |
| --- | --- |

| Title: | Performance Analysis of Lagrangian Parameter Selection in JVT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information and Proposal |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis, Feng Wu, and Shipeng Li 3F Sigma Center, No.49 Zhichun Road, Haidian District, Beijing, 100080, China | Tel: Email: | (86-10) 6261-7711 ext 3108 alexismt@ieee.org |
| Source: | Microsoft Corporation |  |  |

_____________________________

# Introduction

Rate Distortion Optimization (RDO) with the usage of Lagrangian Parameters () [1] is a method that can potentially increase coding efficiency of video coding systems. This method is based on the principle of jointly minimizing both Distortion D and Rate R using an equation of the form:

J=D+λ⋅R												(1)

The JVT standard [2] has also adopted RDO as the encoding method of choice, even though this is not considered as normative, whereas all testing conditions [3] of new proposals and evaluations are based on such a method. This is though highly dependent on the selection of  which is in the current software selected, for I and P frame, as λI,P=0.85×2QP3, where QP is the quantizer used for the current Macroblock, and λB=4×λI,P for B frames. We have though discovered that these numbers were not selected wisely, especially since these could inadvertently affect considerably the efficiency of B frame coding. In this document we analyze the performance mainly with regards to B frames, and propose an interim value for , which agrees with the value proposed in [6],until further investigation is done, and a better method proposed.

# Rate Distortion Optimization in JM3.X

The current JVT reference software [5] includes two different complexity modes used for the encoding of a sequence, a high complexity mode and a lower complexity mode. As discussed above, the high complexity mode is based on a RDO scheme with the usage of Lagrangian parameters which try to optimize separately several aspects of the encoding. This includes motion estimation, intra block decision, subblock decision of the tree macroblock structure, and the final mode decision of a macroblock. Complete analysis can be found in [4]. As it was also mentioned in [6], this method depends highly on the values of  which though have been changed several times in the past. In particular, after the January meeting the value of  change from

λI,P=5×QP+534−QP×expQP10											(2)

to:

λI,P=0.85×2QP3													(3)

or basically

λI,P=A1000×2QP3													(4)

where A=850, mainly since the previous function could not accommodate the new QP range adopted by the standard. Apparently though the decision of changing the value of  appears to most likely have been solely based on P frame performance, and probably was not carefully tested. In particular we have observed that, especially for the testing conditions recommended by JVT [3], the two equations are considerably different.  Such a relationship can be seen in Table 1.

Table 1: Relationship between previous  and current one.

| QP | λI,PPrev=5×QP+534−QP×expQP10 | λI,PCurr=0.85×2QP3 | λI,PCurr−λI,PPrevλI,PCurr |
| --- | --- | --- | --- |
| 16 | 28.89 | 34.27 | 18.61% |
| 17 | 35.42 | 43.18 | 21.90% |
| 18 | 43.48 | 54.40 | 25.11% |
| 19 | 53.49 | 68.54 | 28.14% |
| 20 | 65.97 | 86.35 | 30.89% |
| 21 | 81.66 | 108.80 | 33.23% |
| 22 | 101.53 | 137.08 | 35.01% |
| 23 | 126.94 | 172.71 | 36.05% |
| 24 | 159.84 | 217.60 | 36.14% |
| 25 | 203.04 | 274.16 | 35.03% |
| 26 | 260.86 | 345.42 | 32.42% |
| 27 | 340.11 | 435.20 | 27.96% |
| 28 | 452.23 | 548.32 | 21.25% |

We observe that for the range {16,20,24,28} recommended in [3] the new  is, surprisingly, between 18% and 36% larger than the previous value. The increase in   can have several negative effects in the overall performance of the encoder, such as in reduced reference frame quality and at the efficiency of motion estimation/prediction. We should also point out that PSNR does not always imply a good visual quality, and we have observed that in several cases several blocking artifacts may appear even at higher bitrates. This could also be affected by the usage of the Non-residual skip mode, which in a sense bypasses the specified quantizer value and thus reduces the efficiency of the deblocking filter. This could be more visually understood when taking in consideration that this mode could in several cases require even zero bits to be encoded, thus minimizing the effect of the  ( depends on the original QP). Considering that the distortion of all other, more efficient, macroblock modes is penalized by the larger value of  it becomes apparent that quite possibly the actual coding efficiency of the current codec has reduced. Furthermore, as we have mentioned above, the new value was most likely not tested within B frames.

# Rate Distortion Optimization evaluation

In view of the fact that B frames rely even more on the quality of their references and use an even larger lagrangian parameter(λB=4×λI,P), we have evaluated in this document the performance of the current  when B frames are enabled. We have also compared performance with A=500 & A=700 especially since the later gives results very close to the previous, e-based . Since we have found that [8] can be possibly a better design for B frames for several reasons, we have used that scheme in our evaluation. The same concepts should also apply to the original scheme as well. We should also mention that in this design, the  for B frames is calculated as λB=max2,min(4,QP6)×λI,P, since the λB=4×λI,P was excessive. The testing conditions as described in [3], were used, using the UVLC entropy coder and 1 reference frame.

From our experiments, we observe that if the same QP is used for both B and P frames, A=500 outperforms considerably the current  (A=850). More specifically, encoding performance [7] can be up to 2.75% bit savings or 0.113dB higher for the sequences examined.

Table 2: Comparison of encoding performance for different values of .

| A | QPB | Sequence | Foreman | Container | News | Paris | Silence | Mobile | Tempete |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 500 | QPP | PSNR | 0.128 | -0.015 | 0.084 | 0.117 | 0.039 | 0.280 | 0.156 |
|  |  | Bitrate | 3.43% | -0.33% | 2.01% | 2.85% | 1.06% | 6.35% | 3.89% |
|  | QPP+1 | PSNR | 0.102 | -0.111 | 0.057 | 0.048 | 0.005 | 0.133 | 0.094 |
|  |  | Bitrate | 2.58% | -2.78% | 1.39% | 1.19% | 0.16% | 3.04% | 2.35% |
| 700 | QPP | PSNR | 0.070 | 0.034 | 0.061 | 0.079 | 0.009 | 0.176 | 0.086 |
|  |  | Bitrate | 1.87% | 0.79% | 1.43% | 1.97% | 0.19% | 4.08% | 2.19% |
|  | QPP+1 | PSNR | 0.060 | -0.012 | 0.043 | 0.046 | 0.002 | 0.090 | 0.057 |
|  |  | Bitrate | 1.60% | -0.35% | 0.97% | 1.16% | 0.03% | 2.19% | 1.52% |

Considering the above performance, and also the analysis in [6], we also agree that, even though a further analysis of the RDO is necessary, JVT should adopt a value of A between 500 and 700 (we agree with the value of 680 that was recommended in [6]). In particular, we observe the considerable improvement in performance in sequences Mobile and Tempete. Even though from the above results the value of 500 appears to give better performance in most cases (except container) since from [6] this could affect the performance of P frames as well a larger value is recommended. Usually also a larger QP is more commonly used for B frames than P frames, and we observe that the difference between the two different A reduces.

We actually feel that the calculation of , particularly for P (and I) frames, should actually be not only a function of QP, but a function of the QP to be used for B frames as well (λ=f(QPP,QPB)), if such is known and if single quantizer is applied on the entire frame, or even of other parameters not currently used such as, for example, P & B frame distance.

References

- Gary Sullivan, and Thomas Wiegand, , “Rate Distortion Optimization for Video Compression”, IEEE Signal Processing Magazine, pp. 74-90, Nov. 1998

- Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, “Joint Video Specification (ITU-T Rec. H.264 | ISO/IEC 14496-10 AVC) – Joint Committee Draft”, document JVT-D015d5.doc, July 2002.

- G. Sullivan, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material”, document JVT-N81, Sep. 2001.

- Gary Sullivan and Thomas Wiegand, Q15-E-26, “Rate-Distortion Optimization for Video Compression,” July 1998

- JVT Reference Software, unofficial Version 3.3, http://bs.hhi.de/~suehring/tml/download/Unofficial/

- Koichi Takagi, “Lagrange Multiplier and RD-characteristics”, document JVT-C027, JVT Meeting, Fairfax, May 2002.

- G. Bjontegaard, “Calculation of average PSNR differences between RD-Curves”, document JVT-M33, Mar. 2001

- Alexis Michael Tourapis, Gary Sullivan, Feng Wu, and Shipeng Li, “Timestamp Independent Motion Vector Prediction for P and B frames with Division Elimination”, Doc. JVT-D040, July 2002

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Microsoft |  |  |
| Mailing address | Microsoft Research Asia, 3F Sigma, No49 Zhichun Rd, Haidian Beijing 100080, China |  |  |
| Country | China/USA |  |  |
| Contact person | Alexis Michael Tourapis |  |  |
| Telephone | (86-10) 6261-7711 ext 3108 |  |  |
| Fax | (86-10) 8809 7306 |  |  |
| Email | alexismt@ieee.org |  |  |
| Place and date of submission | Klagenfurt, 16/7/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Performance Analysis of Lagrangian Parameter Selection in JVT |  |
| Contribution number |  | D041 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: